TECHNICAL EVALUATION

&

PRELIMINARY DETERMINATION

APPLICANT

Company Name

Address

City, State, Zip

Plant Name

Facility ID No. 9999999

PROJECT

Project No. 9999999-999-AC

Application for Minor Source Air Construction Permit

Project Name

New Source Review Reform Project

COUNTY

Name County, Florida

PERMITTING AUTHORITY

Florida Department of Environmental Protection

District/Local Office

Address

City, State  ZIP

Date

- General Project INFORMATION

Air Pollution Regulations

Projects at stationary sources with the potential to emit air pollution are subject to the applicable environmental laws specified in Section 403 of the Florida Statutes (F.S.).  The statutes authorize the Department of Environmental Protection (Department) to establish regulations regarding air quality as part of the Florida Administrative Code (F.A.C.), which includes the following applicable chapters:  62-4 (Permits); 62-204 (Air Pollution Control – General Provisions); 62-210 (Stationary Sources – General Requirements); 62-212 (Stationary Sources – Preconstruction Review); 62-213 (Operation Permits for Major Sources of Air Pollution); 62-296 (Stationary Sources - Emission Standards); and 62-297 (Stationary Sources – Emissions Monitoring).  Specifically, air construction permits are required pursuant to Chapters 62-4, 62-210 and 62-212, F.A.C.

In addition, the U. S. Environmental Protection Agency (EPA) establishes air quality regulations in Title 40 of the Code of Federal Regulations (CFR).  Part 60 specifies New Source Performance Standards (NSPS) for numerous industrial categories.  Part 61 specifies National Emission Standards for Hazardous Air Pollutants (NESHAP) based on specific pollutants.  Part 63 specifies NESHAP based on the Maximum Achievable Control Technology (MACT) for numerous industrial categories.  The Department adopts these federal regulations in Rule 62-204.800, F.A.C.

Glossary of Common Terms

Because of the technical nature of the project, the permit contains numerous acronyms and abbreviations, which are defined in Appendix A of this permit.

Facility Description and Location

Provide a facility description.

Plant Name will be/is a new/existing Source Type, which is categorized under Standard Industrial Classification Code No. 9999.  The new/existing Plant Name will be/is located in Name County at address in City, Florida.  The UTM coordinates of the existing/new facility are Zone 99, 999.9 km East, and 9999.9 km North.  This site is in an area that is in attainment (or designated as unclassifiable) for all air pollutants subject to Ambient Air Quality Standards (AAQS).

Facility Regulatory Categories

- The facility is/is not a major source of hazardous air pollutants (HAP).

- The facility is/is not a Title V major source of air pollution in accordance with Chapter 62-213, F.A.C.

- The facility is/is not a major stationary source in accordance with Rule 62-212.400, F.A.C. for the Prevention of Significant Deterioration (PSD) of Air Quality.

- The facility does operate units subject to the NSPS of 40 CFR 60.  {Only include if applicable to the facility.}

- The facility does operate units subject to the NESHAP of 40 CFR 63. {Only include if applicable to the facility.}

- Project Description

- Provide a concise description of the project.

- The following existing emissions units (EU) will be affected by this project.

| - EU No. | - Description |
| --- | --- |

- The following new emissions units will be added by this project.

| - EU No. | - Description |
| --- | --- |

- Processing Schedule

Date	Received the application for a minor source air pollution construction permit.

Date	Requested additional information.

Date	Received additional information; application complete.

- PSD Applicability

Leave in the following PSD applicability information and provide the potential emissions, baseline emissions (if applicable) and PSD applicability for the project if the project would require a PSD analysis or if the facility is a major PSD source.

General PSD Applicability

For areas currently in attainment with the AAQS or areas otherwise designated as unclassifiable, the Department regulates major stationary sources of air pollution in accordance with Florida’s PSD preconstruction review program as defined in Rule 62-212.400, F.A.C.  Under preconstruction review, the Department first must determine if a project is subject to the PSD requirements (“PSD applicability review”) and, if so, must conduct a PSD preconstruction review.  A PSD applicability review is required for projects at new and existing major stationary sources.  In addition, proposed projects at existing minor sources are subject to a PSD applicability review to determine whether potential emissions from the proposed project itself will exceed the PSD major stationary source thresholds.  A facility is considered a major stationary source with respect to PSD if it emits or has the potential to emit:

- 250 tons per year or more of any regulated air pollutant; or

- 100 tons per year or more of any regulated air pollutant and the facility belongs to one of the following 28 PSD-major facility categories:  fossil fuel-fired steam electric plants of more than 250 million British thermal units per hour heat input, coal cleaning plants (with thermal dryers), Kraft pulp mills, Portland cement plants, primary zinc smelters, iron and steel mill plants, primary aluminum ore reduction plants, primary copper smelters, municipal incinerators capable of charging more than 250 tons of refuse per day, hydrofluoric, sulfuric, and nitric acid plants, petroleum refineries, lime plants, phosphate rock processing plants, coke oven batteries, sulfur recovery plants, carbon black plants (furnace process), primary lead smelters, fuel conversion plants, sintering plants, secondary metal production plants, chemical process plants, fossil fuel boilers (or combinations thereof) totaling more than 250 million British thermal units per hour heat input, petroleum storage and transfer units with a total storage capacity exceeding 300,000 barrels, taconite ore processing plants, glass fiber processing plants and charcoal production plants.

Once it is determined that a project is subject to PSD preconstruction review, the project emissions are compared to the “significant emission rates” defined in Rule 62-210.200, F.A.C. for the following pollutants:  carbon monoxide (CO); nitrogen oxides (NOX); sulfur dioxide (SO2); particulate matter (PM); particulate matter with a mean particle diameter of 10 microns or less (PM10); particulate matter with a mean particle diameter of 2.5 microns or less (PM2.5); volatile organic compounds (VOC); lead (Pb); fluorides (F); sulfuric acid mist (SAM); hydrogen sulfide (H2S); total reduced sulfur (TRS), including H2S; reduced sulfur compounds, including H2S; municipal waste combustor organics measured as total tetra- through octa-chlorinated dibenzo-p-dioxins and dibenzofurans; municipal waste combustor metals measured as particulate matter; municipal waste combustor acid gases measured as SO2 and hydrogen chloride (HCl); municipal solid waste landfills emissions measured as non-methane organic compounds (NMOC); and mercury (Hg).  In addition, significant emissions rate also means any emissions rate or any net emissions increase associated with a major stationary source or major modification which would construct within 10 kilometers of a Class I area and have an impact on such area equal to or greater than 1 μg/m3, 24-hour average.

If the potential emission equals or exceeds the defined significant emissions rate of a PSD pollutant, the project is considered “significant” for the pollutant and the applicant must employ the Best Available Control Technology (BACT) to minimize the emissions and evaluate the air quality impacts.  Although a facility or project may be major with respect to PSD for only one regulated pollutant, it may be required to install BACT controls for several “significant” regulated pollutants.

PSD Applicability for Project

Describe the emissions from the project.  As provided in the application, the following table summarizes potential emissions and PSD applicability for the project.

Table 1.  Summary of the Applicant’s PSD Applicability Analysis

| Pollutant | Annual Emissions, Tons/Year |  |  |  |  | Subject to PSD? |
| --- | --- | --- | --- | --- | --- | --- |
|  | Baseline Actual | Could Have Accommodated/ Demand Growth | PAE/PTE | Increase | SER/ PSD Threshold |  |
| CO |  |  |  |  | 100/250 | No |
| NOX |  |  |  |  | 40/250/100 | No |
| PM |  |  |  |  | 25/250/100 | No |
| PM10 |  |  |  |  | 15/250/100 | No |
| PM2.5 |  |  |  |  | 10/250/100 | No |
| SO2 |  |  |  |  | 40/250/100 | No |
| VOC |  |  |  |  | 40/250/100 | No |
| other |  |  |  |  |  |  |
| other |  |  |  |  |  |  |
| Example: - Baseline actual emissions (BAE) were calculated based on the following highest consecutive 2-year average: year and year for list pollutants; and year and year for list pollutants. - Could have accommodated emissions/Demand Growth were calculated based on the highest 30-operating days during the 24-month baseline period (year – year), which was determined to be XXX of steam output/heat input/etc. in month year. - Projected actual emissions (PAE)/Potential to Emit (PTE) for each pollutant were based on a steam output/heat input/etc. of XXX and the following: Provide emission factors used in the application for each emission unit if multiple. Ex: A PSD avoidance limit of 39 TPY for NOX and SO2; site specific data of 0.45 lb/MMBtu for CO; AP-42, Chapter 1.6, 0.037 lb/MMBtu for PM; etc. - Add an additional column if an emission unit(s) was shut down. - The increase in emissions from the project is the difference between the PAE/PTE and the BAE, could have accommodated/demand growth and units shut down (if applicable). |  |  |  |  |  |  |

As shown in the above table, total project emissions will not exceed the PSD significant emissions rates; therefore, the project is not subject to PSD preconstruction review.

Or

For projects that do not require a PSD analysis provide the following statement:

The project is located in Name County which is in an area that is currently in attainment with the AAQS or is otherwise designated as unclassifiable.  The proposed project will not increase emissions for any PSD pollutant; therefore, the project is not subject to a PSD preconstruction review.

- Department review

Brief Discussion of proposed project, equipment, controls etc.

Text

Brief Discussion of Emissions

Text

State Requirements

Text

Federal NSPS Provisions

Text

Federal NESHAP Provisions

Text

Other Draft Permit Requirements

Provide a brief description of additional production and operation limitations, monitoring requirements, etc.

- Preliminary Determination

The Department makes a preliminary determination that the proposed project will comply with all applicable state and federal air pollution regulations as conditioned by the draft permit.  This determination is based on a technical review of the complete application, reasonable assurances provided by the applicant, and the conditions specified in the draft permit.  No air quality modeling analysis is required because the project does not result in a significant increase in emissions.  Name is the project engineer responsible for reviewing the application and drafting the permit.  Additional details of this analysis may be obtained by contacting the project engineer at the Department’s District/Local Office, Address, City, State  ZIP, phone (xxx) xxx-xxxx, or by email XXXXXX.