Non-Metallic Mineral Processing Plant Quick Reference Air Regulation Guide

(Subpart OOO – After April 22, 2008)

Compliance

- Maintain fuel usage records.

- Visible emissions testing must be conducted annually. One test per calendar year.

- Control fugitive dust.

- Notify the Department at least one day prior to relocating.

- Perform monthly inspections of wet suppression system.

Limitations

- Fuel Consumption Per Year:

- If you fire gasoline: Use less than 23,000 gallons

- If you fire diesel: Use less than 275,000 gallons

- If you fire propane: Use less than 1.3 million gallons

- If you fire natural gas: Use less than 44 million standard cubic feet

- If you fire more than one fuel type: Prorate

- Visible Emissions Standards for facilities subject to 40 CFR 60 Subpart OOO constructed on or after April 22, 2008:

| Fugitive emissions limit for crushers at which a capture system is not used | 12 percent opacity |
| --- | --- |
| Fugitive emissions limit for grinding mills, screening operations, bucket elevators, transfer points on belt conveyors, bagging operations, storage bins, enclosed truck or railcar loading stations or from any other affected facility | 7 percent opacity |

- Plants processing dry material and Relocatable Plants must use a water suppression system with spray bars located wherever unconfined emissions occur at the feeder(s), the entrance and exit of the crusher(s), the classifier screens, and the conveyor drop points.

- Unconfined emissions generated by vehicular traffic or wind shall be controlled by applying water or effective dust suppressants on a regular basis to all stockpiles, roadways and work yards where the nonmetallic mineral processing plant is located.

- Renew your permit online at: http://www.dep.state.fl.us/air/emission/agpers.htm

- Notify the department using the Facility Relocation Notification found at: http://www.dep.state.fl.us/air/rules/forms/relocation.htm. Give us a call at least one day prior to your move.

District Contact Information

| NORTHWEST DISTRICT 160 Governmental Center, Suite 308 Pensacola, FL 32502-5794 Phone: (850) 595-8300 Fax: (850) 595-8096 rick.bradburn@dep.state.fl.us | NORTHEAST DISTRICT 7777 Baymeadows Way West, Suite 100 Jacksonville, FL 32256-7590 Phone: (904) 256-1700 Fax: (904) 256-1590 vincent.clark@dep.state.fl.us | CENTRAL DISTRICT 3319 Maguire Boulevard, Suite 232 Orlando, FL 32803-3767 Phone: 407-897-2931 Fax: (407) 412-0455 garry.kuberski@dep.state.fl.us |
| --- | --- | --- |
| SOUTHWEST DISTRICT 13051 N. Telecom Parkway Temple Terrace, FL 33637-0926 Phone: (813) 632-7600 Fax: (813) 632-7665 rhonda.hughes@dep.state.fl.us | SOUTH DISTRICT 2295 Victoria Avenue, Suite 364 P.O. Box 2549 Fort Myers, FL 33902-2549 Phone: (239) 344-5600 Fax: (850) 412-0590 sherril.culliver@dep.state.fl.us | SOUTHEAST DISTRICT 400 North Congress Avenue, Suite 200 West Palm Beach, FL 33401 Phone: (561) 681-6600 Fax: (561) 681-6790 maria.serrano@dep.state.fl.us |

County Contact Information

| Orange County Environmental Protection Division Phone: 407-836-1445 Email: Jodi.Dittell@ocfl.net | Broward County Environmental Protection and Growth Management Phone: 954-519-1220 Email: pshelton@broward.org | City of Jacksonville/Duval County Neighborhoods Department Environmental Quality Division Phone: (904) 255-7100 Email: tilley@coj.net |
| --- | --- | --- |
| Hillsborough County Environmental Protection Commission Air Management Division Phone: 813-627-2600 watersj@epchc.org | Pinellas County Department of Environmental Management Air Quality Division Phone: 727-464-4422 grobbins@pinellascounty.org | Miami-Dade County Permitting, Environment and Regulatory Affairs Phone: 305-372-6925 Email: gordor@miamidade.gov |
| Sarasota County Sarasota County ESBC Air Quality/ Storage Tank Management Phone: 941-861-0676 Fax: 941-861-6267 mstorino@scgov.net | Palm Beach County Health Department Phone: 561-837-5900 Fax: 561-837-5295 Thomas_Tittle@doh.state.fl.us |  |
|  | DISCLAIMER: This Quick Reference Air Regulation Guide is for guidance purposes only. It is not official rule language and does not include a comprehensive listing of all environmental regulations that may be applicable to portable Nonmetallic Mineral Processing Plants operating under the Air General Permit. Please refer to Rule 62-210.310(5)(e), F.A.C. for complete and up-to-date rule language. |  |