Non-Metallic Mineral Processing Plant Quick Reference Air Regulation Guide

(Not Subject to Subpart OOO)

Compliance

- Maintain fuel usage records.

- Control fugitive dust.

- Notify the Department at least one day prior to relocating.

Limitations

- Fuel Consumption Per Year:

- If you fire gasoline: Use less than 23,000 gallons

- If you fire diesel: Use less than 275,000 gallons

- If you fire propane: Use less than 1.3 million gallons

- If you fire natural gas: Use less than 44 million standard cubic feet

- If you fire more than one fuel type: Prorate

- Visible Emissions Standard: 20 percent opacity. This means you can see 80% of the background through the dust plume.

- Stationary Plants processing dry material and Relocatable Plants must use a water suppression system with spray bars located wherever unconfined emissions occur at the feeder(s), the entrance and exit of the crusher(s), the classifier screens, and the conveyor drop points.

- Unconfined emissions generated by vehicular traffic or wind shall be controlled by applying water or effective dust suppressants on a regular basis to all stockpiles, roadways and work yards where the nonmetallic mineral processing plant is located.

- Renew your permit online at: http://www.dep.state.fl.us/air/emission/agpers.htm

- Notify the department of your relocation using the Facility Relocation Notification found at: http://www.dep.state.fl.us/air/rules/forms/relocation.htm. Give us a call at least one day prior to your move.

District Contact Information

| NORTHWEST DISTRICT 160 Governmental Center, Suite 308 Pensacola, FL 32502-5794 Phone: (850) 595-8300 Fax: (850) 595-8096 Email: rick.bradburn@dep.state.fl.us | NORTHEAST DISTRICT 7777 Baymeadows Way West, Suite 100 Jacksonville, FL 32256-7590 Phone: (904) 256-1700 Fax: (904) 256-1590 Email: vincent.clark@dep.state.fl.us | CENTRAL DISTRICT 3319 Maguire Boulevard, Suite 232 Orlando, FL 32803-3767 Phone: 407-897-2931 Fax: (407) 412-0455 Email: garry.kuberski@dep.state.fl.us |
| --- | --- | --- |
| SOUTHWEST DISTRICT 13051 N. Telecom Parkway Temple Terrace, FL 33637-0926 Phone: (813) 632-7600 Fax: (813) 632-7665 Email: rhonda.hughes@dep.state.fl.us | SOUTH DISTRICT 2295 Victoria Avenue, Suite 364 P.O. Box 2549 Fort Myers, FL 33902-2549 Phone: (239) 344-5600 Fax: (850) 412-0590 Email: sherril.culliver@dep.state.fl.us | SOUTHEAST DISTRICT 400 North Congress Avenue, Suite 200 West Palm Beach, FL 33401 Phone: (561) 681-6600 Fax: (561) 681-6790 Email: maria.serrano@dep.state.fl.us |

County Contact Information

| Orange County Environmental Protection Division Phone: 407-836-1445 Email: Jodi.Dittell@ocfl.net | Broward County Environmental Protection and Growth Management Phone: 954-519-1220 Email: pshelton@broward.org | City of Jacksonville/Duval County Neighborhoods Department Environmental Quality Division Phone: (904) 255-7100 Email: tilley@coj.net |
| --- | --- | --- |
| Hillsborough County Environmental Protection Commission Air Management Division Phone: 813-627-2600 Email: Jason Waters - watersj@epchc.org | Pinellas County Department of Environmental Management Air Quality Division Phone: 727-464-4422 Email: grobbins@pinellascounty.org | Miami-Dade County Permitting, Environment and Regulatory Affairs Phone: 305-372-6925 Email: gordor@miamidade.gov |
| Sarasota County Sarasota County ESBC Air Quality/ Storage Tank Management Phone: 941-861-0676 Fax: 941-861-6267 Email: mstorino@scgov.net | Palm Beach County Health Department Phone: 561-837-5900 Fax: 561-837-5295 Email: Thomas_Tittle@doh.state.fl.us |  |
|  | DISCLAIMER: This Quick Reference Air Regulation Guide is for guidance purposes only. It is not official rule language and does not include a comprehensive listing of all environmental regulations that may be applicable to portable Nonmetallic Mineral Processing Plants operating under the Air General Permit. Please refer to Rule 62-210.310(5)(e), F.A.C. for complete and up-to-date rule language. |  |