Below is the procedure of ArcNLET operations for splitting large number (more than around 5,000 numbers) of septic tanks:

- Split the septic tanks into several pieces; each piece should contain less than 5,000 numbers. The procedures of creating the pieces are:

- Create a new polygon shape file using a Catalog. This is a temporary file used to clip the septic tank pieces.

- Draw the shape file polygon area (editing and create features tool) for the chosen area of septic tanks.

- Use clip (from Analysis Tools>extraction>clip) to get the piece of septic tank. Remember to select the coordinate system of other ArcNLET files.

Repeat the procedures until you get the all pieces from the whole area

- There is no need to process other datasets (e.g., water bodies, DEM, porosity, and hydraulic conductivity).

- Run ArcNLET using the each of the septic tank file.

- After getting the results from the load estimation module of individual pieces, the .csv file will provide the each WBID loadings. Add the loadings of same WBID from the different pieces manually, if any.

- Make the spatial join of each pieces plume output (raster file) from the transport module to create one raster file for the whole area.

Example Run Using the Indian River County Data:

- This modeling area has a total of 12,741 septic tanks. The locations can be found in the file “AOI_SepticTankLocations_No_Sewer_Prj” in the data folder.

- This area has a total of 1474 surface waterbodies, and they are shown in the file “canal_WB_Lagoon”.

- It is our practice to use the locations of surface waterbodies to determine how to split the septic tanks into pieces.

Step 1: Open the shape files of septic tanks and surface waterbodies mentioned above.

Step 2: Create a polygon shape file called “PieceX”, where X is the index of the septic tank pieces that will be created.

Step 3: Draw the shape file polygon area (editing and create features tool) for the chosen area of septic tanks.

Step 4: Clip the septic tanks in the area determined in Step 3 and choose the right coordinate system.