Facility Management

Summary

DRP has a twofold need as it relates to facility management. First, a KPI (Key Performance Indicator) for facility conditions needs to be established for the upcoming 2018-2019 fiscal year. Second, a robust facility assessment program needs to be developed and implemented to ensure FCO funding is being applied to maximum effectiveness. This document outlines the initial concepts for meeting both needs.

KPI for FY18-19

The major FCO-funded projects for the upcoming fiscal year have been identified and confirmed over the past several weeks. From that entire list, those projects receiving funding for construction can be used as the basis for a KPI. These projects involve facilities which are in “poor” or “fair” condition. Conditions can be objectively defined by applying the definitions pioneered by NASA and later adopted by other federal agencies for establishing a Facility Condition Index (FCI):

5. Excellent. Only normal scheduled maintenance required.

4. Good. Some minor repairs needed. Normally functions as intended.

3. Fair. More minor repairs and some infrequent larger repairs required. System occasionally function as intended.

2. Poor. Significant repairs required. Excessive wear and tear clearly visible. Obsolete. Not fully functional as intended. Repair parts not easily obtainable. Does not meet all codes.

1. Non-functional. Major repair or replacement required to restore function. Unsafe to use.

The KPI can be formulated as the percentage of these projects moved into “good” to “excellent” condition.

Example KPI: The percentage of identified projects successfully upgrading facility conditions from fair/poor to excellent/good by the end of FY18/19.

Example goal: Complete 80% of the identified projects by the end of the fiscal year, with the remaining balance in active construction.

Proposed for FY 18/19 (48 Total Projects)

KPI for Funded Projects:  	Poor to Good = 22 of 48 = 46%

Fair to Good = 26 of 48 = 54%

Goal for Funded Projects:	Poor to Good: 18 Projects Complete by 6/30/19 (82% of Total Projects)

Fair to Good: 21 Projects Complete by 6/30/19 (81% of Total Projects)

Assessment Program and 10-Year Planning

Additional elements from the FCI program can be adapted to establish a facility assessment program capable of enabling long term planning and budgeting. Additional benefits of such a program include a better understanding of facility conditions, more objective project identification, more cost-efficient repair projects, increased accountability for maintenance, and more. Major steps in developing such a program will include (1) determining the parameters of the program, (2) development of the assessment tool and database, (3) conducting initial facility assessments, (4) performing identified repair needs, (5) conducting end-of-year assessments to measure outcomes, and (6) assessing the program to implement improvements for following years.

1. Determining the parameters of the program: Clear parameters need to be established by the division management team. Each facility type needs to be specifically included or excluded from this program. For example, should we only include structures at first or should we also include linear facilities? Do we want to include all facilities captured in FL-SOLARIS, or are there minor structures we should exclude? BDC recommendation is to only include all facilities captured in FL-SOLARIS, as linear facilities (i.e. roads) can be managed in a separate system or added at a future date.

2. Development of the assessment tool and database: Development of the assessment tool will be the responsibility of the DRP GIS team. The tool will need to be accessible on mobile devices with the ability to rate the condition each individual facility’s major systems using the same criteria outlined above for the KPI. Facility systems to be rated include the roof, structure, HVAC (mechanical), electrical, plumbing, and internal and external finishes. Additional information, such as roof installation year and cost of performed repairs, should also be captured as much as possible. Assessments entered in the field need to automatically populate a division-wide database.

3. Conducting initial facility assessments: Once the tool and database are ready, the next step is conducting the facility assessments. Assessments will be performed mostly by field staff located within the parks, with BDC project managers available to assist upon request for more complex facilities.

4. Performing identified repair needs: Facility repairs will be performed per the identified projects of division and district management.  Minor facility repairs will be performed by field staff or contractors hired locally by field staff. Repair needs rising to a higher level of cost and complexity will be managed as projects by BDC.

5. Conducting end-of-year assessments to measure outcomes: An end-of-year assessment will enable measurement of progress and KPI reporting. Each end-of-year assessment will also provide the benchmark for the following year.

6. Assessing the program to implement improvements for following years: In conjunction with end of year reporting, a program assessment should be led by a process improvement facilitator to collect program feedback from field staff and all central office program areas involved. This assessment will use a formalized issuediscussionrecommendationdecision format.

Example KPI: The division-wide facility condition index (FCI) for all facilities in DRP.

Example goal: Achieve a division-wide FCI of 3.0 (“fair”) or better.

Example outcome: The division-wide FCI at the end of this fiscal year is a 3.12, as compared to 2.85 at the end of the previous year.