| Analyte | GCTL | 1Z-16104 93rd Rd N 8/4/2009 | 1Z-16231 70th St N 8/4/2009 | 1Z-16441 87th Ln N 8/4/2009 | 1Z-16441 87th Ln N 8/27/2009 | 1Z-16525 Temple Blvd 8/6/2009 | 1Z-16758 67th Ct N 8/4/2009 | 1Z-17148 Hamlin Blvd 8/4/2009 | 1Z-17639 68th St N 8/4/2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Gross Alpha | 15 pCi/L | 1.8 | 4.8 | 7.1 | 8.4 | 3.5 | 4.6 | 6.6 | 7.3 |
| Aluminum | 200 ug/L (SSO) | 6.5 | ND | 58 | NA | 30 | 58 | ND | 8.8 |
| Arsenic | 10 ug/L | 0.51 | ND | ND | NA | ND | 0.55 | ND | 0.35 |
| Barium | 2000 ug/L | 22.5 | 25.5 | 13.8 | NA | 9.6 | 39.9 | 25.7 | 36.6 |
| Chromium | 100 ug/L | 1.0 | ND | 0.42 | NA | 0.74 | 0.32 | ND | ND |
| Copper | 1000 ug/L (SSO) | 1.93 | 2.61 | ND | NA | 2.32 | 26.6 | 1.48 | 4.54 |
| Lead | 15 ug/L | 0.30 | 0.26 | ND | NA | ND | 3.93 | ND | 0.85 |
| NO2/NO3 | 10,000 ug/L | 25 | 12 | 4 | NA | 6 | ND | 8 | ND |
| Radium 226/228 | 5 pCi/L | 1.0 | NA | NA | 0.8 | NA | NA | NA | 3.8 |
| Strontium | 4200 ug/L (MCST) | 594 | 1180 | 622 | NA | 1210 | 835 | 787 | 420 |
| 1,4-Dioxane | 3.2 ug/L (MCC) | ND | ND | 0.43 | NA | ND | ND | ND | ND |
| Titanium | Not Established | ND | ND | 0.50 | NA | 1.4 | 0.88 | ND | ND |

ND=Non Detect

NA=Not Analyzed

Yellow Infill=Exceedance of Ground Water Cleanup Target Level

ug/L= micrograms per Liter

pCi/L=picocuries per Liter

SSO=Secondary Standard Organoleptic

MCST=Minimum Criteria Systemic Toxicant

MCC=Minimum Criteria Carcinogen

Primary Standard Unless Otherwise Noted

| Analyte | GCTL | 1Z-17724 60th Ln N 8/4/2009 | 1Z-17724 60th Ln N 8/27/2009 | 1Z-17855 83rd Pl N 8/4/2009 | 1Z-17895 80th St N 8/4/2009 | 1Z-18801 Hamlin Blvd 8/4/2009 | 2Z-14072 78th Pl N 8/5/2009 | 2Z-14390 Orange Blvd 8/5/2009 | 2Z -14692 Tangerine Blvd 8/4/2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Gross Alpha | 15 pCi/L | ND | 3.4 | 2.5 | 2.9 | 4.7 | 2.3 | 1.6 | 15.6 |
| Aluminum | 200 ug/L (SSO) | ND | NA | ND | ND | ND | ND | ND | ND |
| Arsenic | 10 ug/L | ND | NA | ND | 0.79 | 1.10 | ND | ND | 1.01 |
| Barium | 2000 ug/L | 53.8 | NA | 30.1 | 31.0 | 49.3 | 29.1 | 20.8 | 65.1 |
| Chromium | 100 ug/L | ND | NA | ND | ND | ND | 0.34 | ND | 0.30 |
| Copper | 1000 ug/L (SSO) | 3.65 | NA | 3.22 | 0.89 | 13.1 | ND | ND | 1.02 |
| Lead | 15 ug/L | 0.30 | NA | 0.25 | ND | 6.37 | ND | ND | ND |
| NO2/NO3 | 10,000 ug/L | 9 | NA | ND | ND | ND | 5 | ND | 6 |
| Radium 226/228 | 5 pCi/L | NA | 1.3 | NA | NA | NA | NA | 1.0 | NA |
| Strontium | 4200 ug/L (MCST) | 988 | NA | 677 | 917 | 547 | 589 | 391 | 528 |

ND=Non Detect

NA=Not Analyzed

Yellow Infill=Exceedance of Ground Water Cleanup Target Level

ug/L= micrograms per Liter

pCi/L=picocuries per Liter

SSO=Secondary Standard Organoleptic

MCST=Minimum Criteria Systemic Toxicant

MCC=Minimum Criteria Carcinogen

Primary Standard Unless Otherwise Noted

| Analyte | GCTL | 2Z-14741 77th Pl N 8/5/2009 | 2Z-15163 82nd Ln N 8/5/2009 | 2Z-15285 95th Ln N 8/4/2009 | 2Z-15365 71st Pl N 8/4/2009 | 2Z-15553 63rd Pl N 8/5/2009 | 2Z-15670 78th Pl N 8/4/2009 | 2Z -9035 Hall Blvd 8/4/2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Gross Alpha | 15 pCi/L | ND | ND | ND | 2.2 | 2.8 | 3.8 | ND |
| Aluminum | 200 ug/L (SSO) | ND | ND | ND | ND | ND | ND | ND |
| Arsenic | 10 ug/L | ND | ND | ND | 0.29 | 0.27 | ND | ND |
| Barium | 2000 ug/L | 8.77 | 14.9 | 42.1 | 29.6 | 14.3 | 28 | 36.9 |
| Chromium | 100 ug/L | ND | ND | ND | ND | ND | 0.47 | ND |
| Copper | 1000 ug/L (SSO) | ND | 0.52 | ND | 1.03 | ND | ND | 4.49 |
| Lead | 15 ug/L | ND | ND | ND | ND | ND | ND | 0.61 |
| NO2/NO3 | 10,000 ug/L | ND | 4 | 28 | 9 | 7 | 7 | 16 |
| Radium 226/228 | 5 pCi/L | NA | NA | NA | NA | NA | NA | 2.2 |
| Strontium | 4200 ug/L (MCST) | 1080 | 795 | 748 | 515 | 848 | 786 | 1190 |
| MTBE | 20 ug/L (MCO) | ND | ND | ND | ND | 0.72 | ND | ND |

ND=Non Detect

NA=Not Analyzed

Yellow Infill=Exceedance of Ground Water Cleanup Target Level

ug/L= micrograms per Liter

pCi/L=picocuries per Liter

SSO=Secondary Standard Organoleptic

MCST=Minimum Criteria Systemic Toxicant

MCC=Minimum Criteria Carcinogen

MCO=Minimum Criteria Organoleptic

Primary Standard Unless Otherwise Noted

| Analyte | GCTL | 3Z-11546 60th St N 8/5/2009 | 3Z-11546 60th St N 8/27/2009 | 3Z-11843 Tangerine Blvd 8/5/2009 | 3Z-11843 Tangerine Blvd 8/27/2009 | 3Z-11855 Keswick Way 8/4/2009 | 3Z-12325 87th St N 8/5/2009 | 3Z-12754 77th Pl N 8/5/2009 | 3Z- 13048 82nd St N 8/5/2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Gross Alpha | 15 pCi/L | ND | 2.4 | 8.9 | 9.0 | 2.3 | 1.4 | ND | ND |
| Aluminum | 200 ug/L (SSO) | ND | NA | ND | NA | ND | 31 | 7.1 | ND |
| Arsenic | 10 ug/L | 0.36 | NA | 1.58 | NA | ND | ND | ND | ND |
| Barium | 2000 ug/L | 27.7 | NA | 92.9 | NA | 16.4 | 14.9 | 11.3 | 10.4 |
| Chromium | 100 ug/L | 0.47 | NA | ND | NA | 0.73 | 0.73 | 0.79 | 0.40 |
| Copper | 1000 ug/L (SSO) | ND | NA | 3.40 | NA | 1.01 | 2.09 | 7.77 | 8.48 |
| Lead | 15 ug/L | ND | NA | ND | NA | ND | 0.38 | 0.82 | 0.41 |
| NO2/NO3 | 10,000 ug/L | 4 | NA | 5 | NA | 6 | ND | 4 | ND |
| Radium 226/228 | 5 pCi/L | NA | 1.6 | NA | 5.6 | 0.3 | NA | NA | NA |
| Strontium | 4200 ug/L (MCST) | 393 | NA | 680 | NA | 730 | 436 | 335 | 889 |
| 1,4-Dioxane | 3.2 ug/L (MCC) | ND | NA | 0.34 | NA | ND | ND | ND | ND |

ND=Non Detect

NA=Not Analyzed

Yellow Infill=Exceedance of Ground Water Cleanup Target Level

ug/L= micrograms per Liter

pCi/L=picocuries per Liter

SSO=Secondary Standard Organoleptic

MCST=Minimum Criteria Systemic Toxicant

MCC=Minimum Criteria Carcinogen

Primary Standard Unless Otherwise Noted

| Analyte | GCTL | 3Z-13678 82nd St N 8/5/2009 | 3Z-13756 73rd St N 8/5/2009 | 3Z-13920 63rd Ln N 8/5/2009 | 3Z-7655 Woodmuir Dr 8/5/2009 | 4Z-11060 56th Pl N 8/5/2009 | 4Z-11499 51st Ct N 8/5/2009 | 4Z-11919 Orange Grove Blvd 8/5/2009 | 4Z-13355 54th Ln N 8/5/2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Gross Alpha | 15 pCi/L | ND | 2.2 | 6.2 | ND | ND | 2.7 | ND | 1.1 |
| Aluminum | 200 ug/L (SSO) | ND | 5.6 | 6.4 | ND | 7.7 | ND | ND | ND |
| Arsenic | 10 ug/L | ND | ND | ND | ND | 0.54 | 1.46 | ND | ND |
| Barium | 2000 ug/L | 10.2 | 14.9 | 33.1 | 12.6 | 18.8 | 27.8 | 11.6 | 13.0 |
| Chromium | 100 ug/L | 0.52 | 0.62 | 0.48 | 0.98 | ND | 0.31 | 1.7 | 1.3 |
| Copper | 1000 ug/L (SSO) | 0.56 | 5.63 | 4.61 | 2.71 | 1.85 | 1.58 | ND | 0.67 |
| Lead | 15 ug/L | 0.20 | 0.29 | ND | 0.38 | ND | ND | ND | ND |
| NO2/NO3 | 10,000 ug/L | 7 | 5 | 7 | ND | ND | 5 | 5 | 8 |
| Radium 226/228 | 5 pCi/L | NA | NA | 2.4 | NA | 0.6 | NA | NA | NA |
| Strontium | 4200 ug/L (MCST) | 466 | 675 | 972 | 648 | 222 | 422 | 958 | 672 |
| 1,1,2-TCA | 5 ug/L | ND | 0.22 | ND | ND | ND | ND | ND | ND |
| 1,2-DCA | 3 ug/L | ND | 0.21 | ND | ND | ND | ND | ND | ND |

ND=Non Detect

NA=Not Analyzed

Yellow Infill=Exceedance of Ground Water Cleanup Target Level

ug/L= micrograms per Liter

pCi/L=picocuries per Liter

SSO=Secondary Standard Organoleptic

MCST=Minimum Criteria Systemic Toxicant

MCC=Minimum Criteria Carcinogen

Primary Standard Unless Otherwise Noted

| Analyte | GCTL | 4Z-13445 44th PL N 8/5/2009 | 4Z-4331 Coconut Blvd 8/6/2009 | 4Z-5806 Coconut Blvd 8/5/2009 | 4Z-5924 Avocado Blvd 8/5/2009 | 4Z-5924 Avocado Blvd 8/26/2009 | 5Z-14155 43rd RD N 8/5/2009 | 5Z-14155 43rd Rd N 8/27/2009 | 5Z-14563 40th St N 8/5/2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Gross Alpha | 15 pCi/L | 3.1 | 2.7 | ND | 1.3 | ND | 6.2 | 4.2 | 4.5 |
| Aluminum | 200 ug/L (SSO) | 89 | ND | ND | ND | NA | 26 | NA | ND |
| Arsenic | 10 ug/L | ND | 4.83 | ND | ND | NA | 0.45 | NA | 0.39 |
| Barium | 2000 ug/L | 6.51 | 49.4 | 16.0 | 18.9 | NA | 44.9 | NA | 45.3 |
| Chromium | 100 ug/L | 1.2 | ND | 0.57 | 0.72 | NA | 0.83 | NA | ND |
| Copper | 1000 ug/L (SSO) | 1.50 | 6.01 | 2.51 | 0.83 | NA | 3.24 | NA | 0.80 |
| Lead | 15 ug/L | 0.57 | 0.31 | ND | 0.21 | NA | 0.23 | NA | ND |
| NO2/NO3 | 10,000 ug/L | 7 | 5 | 21 | 5 | NA | 9 | NA | 8 |
| Radium 226/228 | 5 pCi/L | NA | NA | NA | NA | 0.2 | NA | 2.8 | NA |
| Strontium | 4200 ug/L (MCST) | 813 | 320 | 1080 | 683 | NA | 1070 | NA | 562 |
| Titanium | Not Established | 5.1 | ND | ND | ND | NA | 1.2 | NA | ND |
| MTBE | 20 ug/L (MCO) | ND | ND | ND | ND | NA | 1.5 | NA | ND |

ND=Non Detect

NA=Not Analyzed

Yellow Infill=Exceedance of Ground Water Cleanup Target Level

ug/L= micrograms per Liter

pCi/L=picocuries per Liter

SSO=Secondary Standard Organoleptic

MCST=Minimum Criteria Systemic Toxicant

MCC=Minimum Criteria Carcinogen

MCO=Minimum Criteria Organoleptic

Primary Standard Unless Otherwise Noted

| Analyte | GCTL | 5Z-15530 42nd Rd N 8/27/2009 | 5Z-15530 42nd Rd N 9/14/2009 | 5Z-15655 42nd Rd N 8/5/2009 | 5Z-15962 42nd St N 8/6/2009 | 5Z-16280 Sycamore Dr E 8/5/2009 | 5Z-16280 Sycamore Dr E 8/27/2009 | 6Z-17368 49th St N 8/6/2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Gross Alpha | 15 pCi/L | 23.7 | NA | 2.3 | ND | 10.7 | 7.4 | 6.5 |
| Aluminum | 200 ug/L (SSO) | NA | ND | ND | ND | ND | NA | 6.0 |
| Arsenic | 10 ug/L | NA | 0.60 | 0.26 | ND | 1.43 | NA | 0.36 |
| Barium | 2000 ug/L | NA | 119 | 10.6 | 13.1 | 25.6 | NA | 48.7 |
| Chromium | 100 ug/L | NA | ND | ND | ND | ND | NA | ND |
| Copper | 1000 ug/L (SSO) | NA | 3.18 | 1.31 | 12.1 | 0.83 | NA | 5.17 |
| Lead | 15 ug/L | NA | 0.66 | ND | 3.49 | ND | NA | 0.87 |
| NO2/NO3 | 10,000 ug/L | NA | ND | 5 | ND | 5 | NA | 21 |
| Radium 226/228 | 5 pCi/L | 13.3 | NA | NA | NA | NA | 4.6 | NA |
| Strontium | 4200 ug/L (MCST) | NA | 1050 | 1070 | 806 | 789 | NA | 1040 |
| Hexazione | 230 ug/L (MCST) | NA | ND | 0.31 | ND | ND | NA | ND |
| Selenium | 50 ug/L | ND | 0.31 | ND | ND | ND | ND | ND |

ND=Non Detect

NA=Not Analyzed

Yellow Infill=Exceedance of Ground Water Cleanup Target Level

ug/L= micrograms per Liter

pCi/L=picocuries per Liter

SSO=Secondary Standard Organoleptic

MCST=Minimum Criteria Systemic Toxicant

MCC=Minimum Criteria Carcinogen

Primary Standard Unless Otherwise Noted

| Analyte | GCTL | 6Z-17413 44th Pl N 8/6/2009 | 6Z-18768 47th Ct N 8/6/2009 | 6Z-18768 47th Ct N 8/27/2009 | 6Z-18852 42nd Rd N 8/6/2009 | 6Z-18852 42nd Rd N 8/27/2009 | 6Z-4091 Mandarin Blvd 8/5/2009 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Gross Alpha | 15 pCi/L | 2.6 | 2.9 | 2.7 | 6.9 | 9.8 | 2.7 |
| Aluminum | 200 ug/L (SSO) | ND | ND | NA | ND | NA | ND |
| Arsenic | 10 ug/L | 0.50 | 0.73 | NA | 1.26 | NA | 0.46 |
| Barium | 2000 ug/L | 13.0 | 19.1 | NA | 38.5 | NA | 17.5 |
| Chromium | 100 ug/L | ND | ND | NA | ND | NA | ND |
| Copper | 1000 ug/L (SSO) | 5.37 | 0.62 | NA | 0.67 | NA | ND |
| Lead | 15 ug/L | 0.23 | ND | NA | ND | NA | ND |
| NO2/NO3 | 10,000 ug/L | 76 | ND | NA | ND | NA | 7 |
| Radium 226/228 | 5 pCi/L | NA | NA | 1.3 | NA | 5.2 | 1.7 |
| Strontium | 4200 ug/L (MCST) | 1220 | 487 | NA | 743 | NA | 1410 |
| Titanium | Not Established | 0.54 | ND | NA | ND | NA | ND |

ND=Non Detect

NA=Not Analyzed

Yellow Infill=Exceedance of Ground Water Cleanup Target Level

ug/L= micrograms per Liter

pCi/L=picocuries per Liter

SSO=Secondary Standard Organoleptic

MCST=Minimum Criteria Systemic Toxicant

MCC=Minimum Criteria Carcinogen

Primary Standard Unless Otherwise Noted

| Analyte | GCTL | 5Z-SID No. 1 (Untreated Water) 8/26/2009 | 5Z-SID No. 2 (Untreated Water) 8/26/2009 | 5Z-SID No. 3 (Untreated Water) 8/5/2009 | 5Z-SID No. 3 (Untreated Water) 8/26/2009 | 5Z-SID No. 3 Duplicate (Untreated Water) 8/26/2009 | 5Z-SID No. 4 (Untreated Water) 8/26/2009 | 5Z-SID No. 5 (Untreated Water) 8/26/2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Gross Alpha | 15 pCi/L | 5.2 | 14.7 | 15.3 | 19.6 | 18.5 | 17.1 | 11.4 |
| Aluminum | 200 ug/L (SSO) | NA | NA | ND | NA | NA | NA | NA |
| Arsenic | 10 ug/L | NA | NA | 0.59 | NA | NA | NA | NA |
| Barium | 2000 ug/L | NA | NA | 127 | NA | NA | NA | NA |
| Chromium | 100 ug/L | NA | NA | 0.34 | NA | NA | NA | NA |
| Copper | 1000 ug/L | NA | NA | ND | NA | NA | NA | NA |
| Lead | 15 ug/L | NA | NA | ND | NA | NA | NA | NA |
| NO2/NO3 | 10,000 ug/L | NA | NA | 6 | NA | NA | NA | NA |
| Radium 226/228 | 5 pCi/L | 3.6 | 8.5 | 11.1 | 10.8 | 11.8 | 8.9 | 2.3 |
| Strontium | 4200 ug/L | NA | NA | 1590 | NA | NA | NA | NA |
| Titanium | Not Established | NA | NA | 0.86 | NA | NA | NA | NA |

ND=Non Detect

NA=Not Analyzed

AR=Awaiting Results

Yellow Infill=Exceedance of Ground Water Cleanup Target Level

ug/L= micrograms per Liter

pCi/L=picocuries per Liter

SSO=Secondary Standard Organoleptic

MCST=Minimum Criteria Systemic Toxicant

MCC=Minimum Criteria Carcinogen

Primary Standard Unless Otherwise Noted

| Analyte | GCTL | 5Z-SID P.O.E. (Treated Water) 8/26/2009 | 5Z-SID Reject Water (Untreated Water) 8/26/2009 | 5Z-SID Repump Water (Treated Water) 8/26/2009 |
| --- | --- | --- | --- | --- |
| Gross Alpha | 15 pCi/L | ND | 68.0 | ND |
| Aluminum | 200 ug/L (SSO) | NA | NA | NA |
| Arsenic | 10 ug/L | NA | NA | NA |
| Barium | 2000 ug/L | NA | NA | NA |
| Chromium | 100 ug/L | NA | NA | NA |
| Copper | 1000 ug/L | NA | NA | NA |
| Lead | 15 ug/L | NA | NA | NA |
| NO2/NO3 | 10,000 ug/L | NA | NA | NA |
| Radium 226/228 | 5 pCi/L | ND | 31.4 | ND |
| Strontium | 4200 ug/L | NA | NA | NA |
| Titanium | Not Established | NA | NA | NA |

ND=Non Detect

NA=Not Analyzed

AR=Awaiting Results

Yellow Infill=Exceedance of Ground Water Cleanup Target Level

ug/L= micrograms per Liter

pCi/L=picocuries per Liter

SSO=Secondary Standard Organoleptic

MCST=Minimum Criteria Systemic Toxicant

MCC=Minimum Criteria Carcinogen

Primary Standard Unless Otherwise Noted

| Analyte | SWCTL | 5Z-Sid Outfall & Sycamore 8/26/2009 | L8-Tieback 8/26/2009 | M Canal-1 8/26/2009 | M Canal-2 8/26/2009 | M Canal-3 8/26/2009 |
| --- | --- | --- | --- | --- | --- | --- |
| Gross Alpha | 15 pCi/L | 1.2 | 1.8 | 1.8 | 1.3 | ND |
| Radium 226/228 | 5 pCi/L | 0.50 | 1.4 | 1.1 | 0.4 | 0.7 |

ND=Non Detect

NA=Not Analyzed

AR=Awaiting Results

Yellow Infill=Exceedance of Ground Water Cleanup Target Level

ug/L= micrograms per Liter

pCi/L=picocuries per Liter

SSO=Secondary Standard Organoleptic

MCST=Minimum Criteria Systemic Toxicant

MCC=Minimum Criteria Carcinogen

Primary Standard Unless Otherwise Noted