GEOFACT:  What Types of Rocks are Mined in Florida?

Limestone, clay, peat, phosphatic rock, sand and gravel, crushed stone, and heavy mineral sands are all mined in Florida. Periodically, the Florida Geological Survey (FGS) assists the U.S. Geological Survey (USGS) in compiling Florida’s mineral production information.  The USGS then includes this information in the U.S. Minerals Yearbook, a summary of national mineral production.  A combined volume of the yearbook, for the years 2012 and 2013, is set to be published in 2016.   In 2012 the value of Florida’s nonfuel mineral production ranked 4th out of the top producing 22 States. Here is a preview of Florida’s minerals production and rankings for 2012 and 2013 where the data were available:

- High-purity limestone: This was mined in Florida for use in the manufacture of clinker for cement.  Clinker is created by fusing limestone particles together in a cement kiln.  Clinker production in Florida in 2012 was 3.687 million metric tons.  Florida’s masonry cement production was ranked second in the U.S. in 2012 and rose to first in the nation in 2013.

- Clays: Common clay, fuller’s earth clay and kaolin are all mined in Florida.  Clay mined in Florida is most commonly used as fill and road base material.  Traditionally, fuller’s earth clay is used in the woolen cloth industry to absorb lanolin, oils, and other greasy impurities as part of the cloth finishing process.   More recent uses for fuller’s earth are as thickeners in drilling muds and paints, carriers in fertilizers, and as a method of absorbing water in packaged goods.  It is also used in the manufacturing of pet litter, oil and grease absorbents, and to decontaminate the clothing and equipment of servicemen and emergency responders who have been contaminated with chemical agents. The kaolin mined in Florida is used in ceramics manufacturing, as well as in the production of floor and wall tile and refractory materials.

Peat: Peat mined in Florida is used principally in garden cultivation and management. Florida, with 336,000 metric tons of production, accounted for 69 percent of U.S. peat production in 2012. In 2013 Florida, with 338,000 metric tons, accounted for 73 percent of U.S. peat production.

The first commercial phosphate mining in Florida occurred at Dunellon, Marion County, in 1889. Hard rock phosphate deposits were dredged in and near the Withlacoochee River.

Phosphate rock:  In Florida phosphate rock  is used in the production of phosphoric acid and phosphatic fertilizers.  The Florida phosphate industry is concentrated in   Hamilton, Hardee, Hillsborough, Manatee, and Polk counties. In 2012, phosphatic material was mined at six mines located in those counties and represented 65 percent of the US domestic annual production capacity.  In 2012 the US crude ore mine production was 150,000 thousand metric tons.

- Sand and gravel:  Silica sand and gravel mined in Florida is used in glassmaking, concrete, well construction, filters and as foundry sands for metal casing.  In2012 Florida’s silica sand and gravel production was 13.7 million metric tons and  was 15.7 million metric tons in 2013.

- Crushed stone: Florida’s crushed stone consists of limestone and dolostone (carbonate rock containing magnesium).  When mined, it is used in manufacturing cement andconcrete, as an agricultural additive, and as road base and construction material.  It is also used as land coverings, riprap (rock to armor shorelines against erosion), and for other  purposes.  The total crushed construction stone, used or sold in Florida in 2012 and 2013, was 47,100 metric tons and 52,600 metric tons respectively.  In 2012 and 2013, Florida ranked sixth and fourth respectively in crushed stone production nationally.

- Heavy mineral sands: Heavy mineral sands consisting primarily of ilmenite, leucoxene rutile, staurolite, and zirconium are mined from mineral concentrations known as placer deposits.  Placer deposits are created by the winnowing, under the action of wind and water, of heavy mineral grains from lighter weight mineral grains. Florida’s heavy minerals are mined from  ancient dune and beach ridges.  Ilmenite, leucoxene, and rutile areminerals that contain titanium, which is  used to manufacture titanium dioxide pigments.   Staurolite and zirconium are used as input material in the manufacture of industrial abrasives.  There were no production data publically available for heavy mineral sands mined in Florida during 2012 & 2013.

Contact:  Guy H. Means, P.G.