Section 4
Description of Proposed Management Actions

This section discusses the proposed management actions within the RAP Study Area.

4.1 Best Management Practices for Parameter of Concern Reduction

4.1.1 Septic to Sewer Projects

In Miami-Dade County, wastewater is handled under two broad categories: (1) onsite (decentralized) systems and (2) regional (centralized) systems. Onsite systems include OSTDSs and are used where centralized systems are not available. Centralized systems include private and public pipes, manholes, and pumps which move (convey) wastewater from buildings to locations where treatment and disposal occurs. Within the County, there are approximately 105,000 parcels served by OSTDSs, approximately 100,000 of which are within the Urban Development Boundary (UDB).  In 2018, the County produced a Final Report in support of Resolution No. R-911-16 entitled Septic Systems Vulnerable to Sea Level Rise (Miami Dade County, 2018).  Since 1994, sea levels have risen four inches and are expected to increase an additional two to six inches by 2030. This has, in turn, led to higher groundwater levels in certain areas. The County, in collaboration with the USGS, identified how sea level rise will increase groundwater levels over the longer term. During times of elevated groundwater levels, OSTDSs may not properly function as designed. The 2018 study identified areas where groundwater levels are close enough to the surface that the existing OSTDSs are likely compromised and may no longer be providing adequate treatment at least part of the year (refer back to Figure 2-11 for the OSTDS coverage in the study area). The report also specifically identified areas where those risks are most severe and systems are vulnerable to complete failure or are compromised during the average rainy season conditions each year, not just during particularly wet years or storms.

Beyond the impacts of flooding events, the volume of unsaturated soil underneath the drainfield impacts the OSTDS's ability to efficiently remove pollutants. It has been shown that aeration of effluent in the unsaturated soil is important in achieving decomposition of organic particles and compounds, in effectively removing phosphorus, in facilitating nitrification that serves as the basis for denitrification to remove nitrogen, and in decreasing bacteria and viruses (Bicki, et al. 1984). In addition, OSTDS effluent moves more slowly in unsaturated soil than in saturated soil, and, therefore, experiences longer treatment time and better opportunity for many pollutants to be removed (Bicki and Brown, 1990).  In many areas, the distance between the surface and the groundwater table is very small, and further increases in groundwater levels can result in hydraulic failure and cause the drainfield to be flooded, especially during the wet season. As Dr. Elmir notes, “a compromised treatment function may result in the relatively unimpeded movement of wastewater contaminants to ground and surface waters.” The 2018 Study concluded that to ultimately address the vulnerability of compromised or failed OSTDSs it is necessary to extend sanitary sewer service to certain areas within the County.

Due to the high density of OSTDSs within the RAP Study Area, it is a high priority for Miami-Dade County to extend sanitary sewer service as a means to reduce nutrient pollution (primarily nitrogen) to receiving water bodies. The County’s Water and Sewer Department (WASD) has identified several projects within the study area to expand sanitary sewer service to vulnerable areas near waterways connecting to Biscayne Bay. Additionally, WASD has also identified parcels served by OSTDSs that are highly vulnerable to being compromised and/or OSTDS failure due to rising groundwater. These projects are discussed in more detail under Section 4.2.

4.1.2 Stormwater Management

Since most of the project area is already developed with little to no vacant land, stormwater management is limited to practices that require minimal land space or prevention of debris entering the stormwater system. As a result, existing Best Management Practices (BMPs) for stormwater management in the study area predominantly consist of exfiltration trenches and drainage/recharge wells with some ongoing maintenance activities (i.e., street sweeping and structure cleaning) to reduce pollutant loading to the waterbodies and ultimately Biscayne Bay. Although, a few existing wet detention facilities were identified within the study area, no future projects have yet been proposed under this BMP for flood control or pollutant removal.

The proposed projects identified by the stakeholders located with the project area are described in further detail along with the general methodology used in quantifying the pollutant removal for each project.  Estimates of the pollutant removal are based on the removal efficiencies and methodologies outlined in the Statewide Best Management Practice (BMP) Efficiencies for Crediting Projects in Basin Management Action Plans (BMAPs) and Alternative Restoration Plans Draft – September 2021 (FDEP).  BMP Trains are used where applicable which generally follows, and in some cases is source-referenced, the efficiencies outlined in the FDEP document.

4.1.2.1 Exfiltration Trench

As presented in Section 2, exfiltration trenches are probably the predominant BMP used in the study area with approximately 500,000 linear feet installed to-date based on the GIS data provided by the County.  Proposed projects identified as eligible for nutrient load reduction credits include the addition of another 55,000 linear feet of exfiltration trench, with nearly 75 percent of the eligible projects including exfiltration trenches at the selected BMP. The quantity of TN and TP removed is calculated using the BMP Trains software for site-specific basin characteristics (DCIA, CN, EMC, etc.)  (described in Section 2), project drainage area, and exfiltration length. The standard County exfiltration trench detail is used when design information was not available.  The drainage area and trench length for each exfiltration project was identified by the stakeholder and/or estimated based on the project description provided by the stakeholder.

4.1.2.2 Drainage/Recharge Wells

Drainage or recharge wells are probably the second most predominant BMP in the study area with approximately 350 wells installed to date based on the GIS data provided by the County Recharge wells have additional pre-treatment and discharge east of the salinity line into groundwater zones greater than 10,000 mg/L of total dissolved solids (TDS).  There were four proposed projects identified as eligible that included the additional wells. The quantity of TN and TP removed is calculated using the BMPTRAINS software for the site-specific basin characteristics (DCIA, CN, EMC, etc.) (described in Section 2), project drainage area, and estimated well capacity.  BMPTRAINS does not include drainage/recharge well as a pre-defined BMP, however the software allows a user-defined BMP. FDEP documentation for drainage/recharge wells relies on a case-by-case basis for the percent reduction and is based on the loading permanently removed from the water body.  For purposes of this evaluation, 95% removal was used for both TN and TP. The drainage area was calculated based on the rational method using the anticipated capacity of the well(s) as described in Section 2.

4.1.2.3 Pollution Control Structures

Pollution control structures include baffle boxes, hydrodynamic separators, inlet filters, screens, etc. and may or may not include media filter materials to enhance the pollutant removal. These BMPs are included in the FDEP documentation for BMP efficiencies and generally available in BMP Trains. Eligible projects proposed by the stakeholders only included three projects which were identified as pilot project for demonstration and evaluation purposes.

4.1.2.4 Street Sweeping and Structure Cleaning

Street sweeping and debris removal through cleaning of inlet structures is an ongoing practice with total pollutants removed estimated using the MS4 BMP Toolkit based on the amount of debris collected prior to entering the stormwater system. Pollutant removal quantities from street sweeping activities were provided by the stakeholder(s). Quantities for entities that are not completely within the study area were pro-rated based on the area of development.

4.1.3. Other Efforts

4.1.3.1 Miami Dade County

4.1.3.1.1 Biscayne Bay Water Quality Characterization and Pollution Reduction Project

In 2021, Miami Dade County was awarded a grant under FDEP’s Coral Reef Protection and Restoration Grant Program which funded a number of activities that work to improve the health and water quality of Biscayne Bay. The projects included in the grant will directly support immediate and short-term recommendations identified in the Biscayne Bay Task Force Report, to improve water quality and address the health of Biscayne Bay. Specific recommendations from the report include:

- Undertake and secure funding for new pilot projects and research projects focused on reducing pollutant loads;

- Initiate and fund studies that illuminate specific knowledge gaps for application toward watershed restoration; and,

- Undertake immediate efforts to identify and eliminate root causes of SSOs including inflow and infiltration. Accelerate sewer infrastructure maintenance and upgrades.

The general description of the tasks in the grant agreement are listed below.

Water Quality Characterization – This project will identify and reduce or eliminate sources of water quality pollution in north Biscayne Bay watersheds, with a focus on the Miami River and Inlet Contributing Area (C-6), Little River (C-7), and Biscayne Canal (C-8) basins.

Innovative Technology – this project will explore the use of Smart Covers and Smart Rain Gauges to analyze priority areas with chronic sanitary sewer overflow (SSO) issues to both predict and prevent SSOs as well as identify and implement fixes for areas where infrastructure needs to be improved or replaced.

Education and Outreach: These efforts will focus on the proposed Little River Adaptation Action Area, which is undergoing a comprehensive climate adaptation planning effort under an FDEP Resilience Planning Grant. The County will conduct education and public outreach to residents as per FDEP’s Septic to Sewer guidance document and associated case studies in part to engage residents to confirm the presence of OSTDSs through registration, education on the program objectives, costs, and benefits, projected monthly utility fees following connection to the sewer system, and establish commitment to connect for parcels that will be converted through this project. The County will also provide outreach to property owners on best practices to maintain their systems.

Septic to Sewer Conversions - The County will prioritize the conversion of parcels based on the greatest likelihood and impact from OSTDS failure along with other environmental, social, and economic ranking criteria. The project will focus on the proposed Little River Adaptation Action Area. The project will have two main components: 1) connecting parcels with abutting sewer infrastructure; and, 2) expanding sewer service to connect the parcels most vulnerable to adverse impact or failure from rising groundwater. These failing systems are suspected of impacting water quality in the Little River and the bay. The project to connect parcels abutting sewer infrastructure will include the installation of public laterals, plumbing of private parcels, the removal of OSTDSs, and other items necessary to achieve connection. The projects expanding the sanitary sewer system to connect the most vulnerable parcels will include the installation of pump station(s) required to manage the new flow, main lines, plumbing of private parcels, the removal of OSTDSs, roadway work, and other items necessary to achieve connection.

Stormwater Treatment – This will build on recommendations from the Miami-Dade County Stormwater Management Program Master Plan Update and the Resilience Adaptation Area report to design and implement engineering strategies in priority areas, including innovative tools such as Green Infrastructure (GI) and Low Impact Development (LID) strategies, with the goal of improving the hydrologic functions and the mitigation of surface water impacts. This task will provide technical support to city and county officials in Miami-Dade County to provide information and technical assistance to promote and accelerate implementation, LID and GI to reduce, mitigate, and to the greatest extent possible, eliminate land-based sources of pollution.

Biological Restoration to Enhance Water Quality – This project supports creation of a living shorelines guidance document as well as a pilot project for a sponge propagation site that will lead to increased filtration of bay waters and reduction of nutrients and bacteria.

4.2 Projects and Load Reductions

Stakeholder coordination and outreach occurred during the development of this RAP to engage the local municipalities and agencies. Specific project data were requested from each municipality and agency that focuses on reducing nutrient loads within the study area and ultimately to the canals and the bay. A comprehensive listing of all project data submitted by stakeholders is included at the end of this section in Table 4-1. The verified list of impaired waters for the WBIDs within the RAP Study Area was adopted by a Secretarial Order signed by the FDEP Secretary on June 27, 2017. Therefore, any projects constructed after this date and/or the stakeholder have committed funding for future implementation were considered eligible for pollutant load reduction credit under this RAP. This is also consistent with the SFWMD land use year (2017-2019) used for the pollutant load analysis described in Section 2.5.

4.2.1 Cooperating Local Governments

Cooperating local governments within the RAP Study Area were previously identified in Section 2.4.1 and listed in Table 2-12.  These are municipalities that control and operate an MS4 within the study area and are permittees under the NPDES MS4 Program.  All of the municipalities were contacted to request project information within the study area. The following sub-sections provide a summary of those projects that were deemed eligible for nutrient load reduction credit.  The estimated load reduction is also shown and was calculated using either the ArcNLET (wastewater) or BMP TRAINS (stormwater) tool described in Section 2.5 of the RAP.

4.2.1.1 City of North Miami Beach

The City of North Miami Beach has the smallest area (81 acres) within the RAP Study Area. At this time, no projects were identified by the City for the current phase of the RAP.

4.2.1.2 City of North Miami

The City of North Miami provided a total of 14 projects within the city limits that provides septic to sewer conversions, stormwater best management practices, studies and monitoring, and/or education outreach. One additional project was identified from GIS data maintained by the County for exfiltration trenches, pollution control structures, and drainage/injection wells. These projects are listed in Table 4-2 with the project name, type of project, status, eligibility, and estimated load reduction.  Of the 15 projects, four were identified as eligible with four (001, 012-014) of the projects located outside of the RAP study area, two (010-011) are future projects, and four (002-004, 008) did not provide a reduction in nutrients.

Table 4-2 City of North Miami Projects

| ID | Name | Type | Status | Eligible | TN Load Removal (lbs/yr) | TP Load Removal (lbs/yr) |
| --- | --- | --- | --- | --- | --- | --- |
| - CNM-001 | Good Neighbor Stormwater Park | Wet Detention Pond | Completed | No | N/A | N/A |
| - CNM-002 | Flood Hazard Information Brochure | Education Efforts | Completed | No | N/A | N/A |
| - CNM-003 | Stormwater System Monitoring | Monitoring/Data Collection | Completed | No | N/A | N/A |
| - CNM-004 | Stormwater Master Plan Update | Study | Underway | No | N/A | N/A |
| - CNM-005 | Stormwater Filter Baskets | Catch Basin Inserts/Inlet Filter Cleanout | Completed | Yes | 0.3 | 0.2 |
| - CNM-006 | Septic to Sewer Conversions | OSTDS Phase Out | Planned | Yes | TBD | TBD |
| - CNM-007 | Street Sweeping | Street Sweeping | Ongoing | Yes | 608 | 395.5 |
| - CNM-008 | Sanitary Sewer Inspections | Sanitary Sewer Inspections | Ongoing | No | N/A | N/A |
| - CNM-009 | Structure Cleaning | BMP Cleanout | Ongoing | Yes | 32 | 15.4 |
| - CNM-010 | Biscayne Canal East | Exfiltration Trench | Future | No | N/A | N/A |
| - CNM-011 | Arch Creek South/Biscayne Canal East | Exfiltration Trench | Future | No | N/A | N/A |
| - CNM-012 | Tressler Street Improvements | Exfiltration Trench | Completed | No | N/A | N/A |
| - CNM-013 | Arch Creek North / South Improvements | Exfiltration Trench | Future | No | N/A | N/A |
| - CNM-014 | Arch Creek North / South Improvements "Basin C" | Exfiltration Trench | Completed | No | N/A | N/A |
| - CNM-015 | NE 18th Ave, NE 19th Dr and NE 122nd St | Exfiltration Trench | Completed | Yes | 35.7 | 6.1 |
| Total: |  |  |  |  | 676.0 | 417.2 |

4.2.1.2.1 Septic to Sewer

One of the identified projects for City of North Miami (CNM-006) provides for conversion and abandonment of 68 OSTDSs for connection to the public sewer system.  At this time, the details for the first phase of the project were not available and will be evaluated as part of a future phase.

4.2.1.2.2 Stormwater

The three eligible stormwater projects identified for City of North Miami included the installation of stormwater filter baskets and programs for street sweeping and stormwater structure cleaning.  These projects, and their estimated POC removal, are described as follows:

CNM-005: The City installed three catch basin filter baskets to keep debris from entering the collection system. The use of filter baskets is under evaluation and may result in additional baskets throughout the City if deemed effective.  Removal of nutrients is small for these three baskets and projected at 0.3 lbs/yr for TN and 0.2 lbs/yr for TP.

CNM-007: The City currently has a regular street sweeping program that includes two crews with separate routes. Frequency of street sweeping varies between daily and monthly based on location (problematic areas receive daily sweeping) and is dependent on various factors (operator schedule, weather, equipment maintenance, etc.).  The City tracks the amount of debris collected and calculates the total TN and TP removal at 950 lbs/yr and 618 lbs/yr, respectively. The developed area of the City is approximately 4,017 acres with 2,571 acres within the RAP study area (64%). For the RAP study area, the projected resulting TN and TP removal is 608 and 396 lbs/yr.

CNM-009: The City also performs cleanout of stormwater structures based on regions of the City. The City is divided into 10 geographical zones with structures within each zone cleaned over a period of 2 to 6 weeks.  The City tracks the amount of debris collected and estimates the TN and TP removal at 50 lbs/yr and 24 lbs/yr, respectively. For the RAP study area, the TN and TP removal is projected at 32 and 15 lbs/yr.  (The City occasionally contracts some structure cleaning services however the quantities are not tracked and thus not included in the POC removal numbers.)

4.2.1.3 Village of Biscayne Park

The Village has recently submitted for an EPA grant to develop a study which entails development of strategies for reducing the bacteria and nutrients from the village watershed. The project is designed to review village-owned sites for development of stormwater treatment areas, detention and retention ponds, and implementation of green infrastructure, and will provide analysis of the available space within vacant land, and existing natural areas within the village, for the purpose of flood protection and water quality improvement. As additional projects are identified, the project list will be updated to reflect those in future phases of the RAP.

4.2.1.4 Miami Shores Village

Miami Shores Village provided a total of 10 projects within the study area that included upgrades to the stormwater system, roadway improvements, and other repair projects. One additional project was identified from GIS data maintained by the County. These projects are listed in Table 4-3 with the project name, type of project, status, and eligibility. Of the 11 projects, two were identified as eligible, with the remaining slated as future projects and are not currently eligible.

Table 4-3 Miami Shores Village Projects

| ID | Name | Type | Status | Eligible | TN Load Removal (lbs/yr) | TP Load Removal (lbs/yr) |
| --- | --- | --- | --- | --- | --- | --- |
| MSV-001 | Shore Estates Storm Water Improvements | Stormwater System Upgrade | Underway | Yes | TBD | TBD |
| MSV-002 | Florida Inland Navigation District Assistance Program 2020 |  | Future | No | N/A | N/A |
| MSV-003 | Resilient Florida |  | Future | No | N/A | N/A |
| MSV-004 | Miami Dade County NE 96th Street between NE 2nd and NE 10th Avenue |  | Underway | No | N/A | N/A |
| MSV-005 | 93rd Street Pump Station |  | Future | No | N/A | N/A |
| MSV-006 | 8th Avenue Drainage Project | Stormwater System Upgrade | Future | No | N/A | N/A |
| MSV-007 | NE 104th Street Roadway Improvements (Phase 1) |  | Future | No | N/A | N/A |
| MSV-008 | NE 104th Street Roadway Improvements |  | Future | No | N/A | N/A |
| MSV-009 | Belvedere Drive Drainage Improvements | Stormwater System Upgrade | Future | No | N/A | N/A |
| MSV-010 | NE 94th Street and Bayshore Drive Drainage Improvements | Stormwater System Upgrade | Future | No | N/A | N/A |
| MSV-011 | NE 91st Terrace and NE 93rd St | Exfiltration Trench | Completed | Yes | 15.3 | 2.3 |
| Total: |  |  |  |  | 15.3 | 2.3 |

4.2.1.4.1 Stormwater

The Shore Estates Storm Water Improvements project (MSV-001) identified within Miami Shores Village provides for the upgrade of the existing stormwater system in the Shore Estates subdivision. The project design is currently underway. The load reduction credits will be quantified when further details on the project are available.

The second stormwater project identified within Miami Shores Village included 850 linear feet of exfiltration trench along NE 91st Terrace and NE 93rd Street. The project is a completed project per the County’s GIS data.  Removal of nutrients is estimated at 15.3 lbs/yr for TN and 2.3 lbs/yr for TP.

4.2.1.5 Village of El Portal

The Village of El Portal provided two projects within the village limits: one is a septic to sewer conversion project and the other is a stormwater improvements project.  Both projects, shown in Table 4-4, are currently underway and thus eligible projects.

Table 4-4 Village of El Portal Projects

| ID | Name | Type | Status | Eligible | TN Load Removal (lbs/yr) | TP Load Removal (lbs/yr) |
| --- | --- | --- | --- | --- | --- | --- |
| - VEP-001 | Village of El Portal Sanitary Sewer System Improvements Phase 1 | OSTDS Phase Out | Underway | Yes | 1,227.0 | N/A |
| - VEP-002 | Village of El Portal El Jardin Stormwater Improvements Phase 2 | Stormwater System Upgrades | Underway | Yes | 85.3 | 13.9 |
| Total: |  |  |  |  | 1,312.3 | 13.9 |

4.2.1.5.1 Septic to Sewer

The septic to sewer conversion project within the Village of El Portal is a Miami-Dade WASD project that will convert a residential neighborhood from OSTDSs to centralized sewer. The project includes new gravity system for the collection of wastewater for conveyance to the North and Central District Wastewater Treatment Plants. For this project, 117 vulnerable OSTDSs that are located close to Little River (C-7) Canal and prone to flooding are planned to be abandoned with the sources connected to sewer system during Phase I.  The locations of these OSTDSs are shown in the inset map on Figure 4-1. The ArcNLET tool previously described in Section 2.5.1 was used to estimate potential load reduction due to the anticipated septic to sewer conversion. The tool estimated a potential reduction of approximately 1,227 lbs/yr of nitrate as TN.

Figure 4-1 Septic to Sewer Projects Within Study Area

4.2.1.5.2 Stormwater

The stormwater project identified within the Village of El Portal provides for the upgrade of the existing stormwater system in the El Jardin and Sherwood Forest neighborhoods.  The project includes various upgrades to stormwater pipes, roadways, and outfalls.  The project also includes approximately 2,000 linear feet of exfiltration trench covering an area of 17 acres.  The project is currently underway with plans for bidding by the end of 2023. Using BMPTRAINS, removal of nutrients is estimated at 85.3 lbs/yr for TN and 13.9 lbs/yr for TP.

4.2.1.6 City of Miami

The City completed a Stormwater Master Plan (SWMP) Update and capital improvements plan (CIP) in 2021 that included stormwater, tidal surge, and sea level rise related flooding. A series of exfiltration and recharge well projects were identified. The portion of the City within the study area is 4,858 acres.

The City provided a total of 10 projects within the study area that include upgrades to their existing stormwater system.  These projects are listed in Table 4-5 with the project name, type of project, status, and eligibility. Of the 10 projects, three were identified as eligible as they are projects that are in planning stages and the City has committed to their implementation. The remaining seven projects are identified as future projects and are not currently eligible.

Table 4-5 City of Miami Projects

| ID | Name | Type | Status | Eligible | TN Load Removal (lbs/yr) | TP Load Removal (lbs/yr) |
| --- | --- | --- | --- | --- | --- | --- |
| - COM-001 | D5-Shorecrest North | Stormwater System Upgrade | Planned | Yes | 353.9 | 57.5 |
| - COM-002 | D5-Shorecrest South | Stormwater System Upgrade | Planned | Yes | 573.9 | 93.7 |
| - COM-003 | D5-NE 75th St Stormwater Improvements | Stormwater System Upgrade | Planned | Yes | 429.4 | 70.7 |
| - COM-004 | C7BN-01 – Little River Gardens / Oakland Grove | Stormwater System Upgrade | Future | No | N/A | N/A |
| - COM-005 | BN-04 – Belle Meade West | Stormwater System Upgrade | Future | No | N/A | N/A |
| - COM-006 | C7BN-03 – Little River Central West of Miami Ave | Stormwater System Upgrade | Future | No | N/A | N/A |
| - COM-007 | C7BN-05 – Northwestern Estates | Stormwater System Upgrade | Future | No | N/A | N/A |
| - COM-008 | BN-05 – Bayside | Stormwater System Upgrade | Future | No | N/A | N/A |
| - COM-009 | BN-06 – Palm Grove | Stormwater System Upgrade | Future | No | N/A | N/A |
| - COM-010 | BN-03 – Belle Meade | Stormwater System Upgrade | Future | No | N/A | N/A |
| Total: |  |  |  |  | 1,357.2 | 221.9 |

4.2.1.6.1 Stormwater

The three eligible stormwater projects identified for the City of Miami included stormwater system upgrades consisting of exfiltration trenches and drainage wells.  These projects and their estimated nutrient removal, are described as follows:

COM-001: This planned project, which is tributary to Biscayne Bay, covers an area of approximately 54 acres and includes 541 linear feet of exfiltration trench, a 140 cfs pump station, and 10 injection wells.  Each injection well has an estimated capacity of 8 cfs (80 cfs total). Removal of nutrients for this area is estimated at 354 lbs/yr for TN and 58 lbs/yr for TP.

COM-002: This planned project, also located in the Biscayne Bay basin, covers an area of approximately 64 acres and includes 1,016 linear feet of exfiltration trench, a 200 cfs pump station, and 12 injection wells (96 cfs total).  Removal of nutrients for this area is estimated at 574 lbs/yr for TN and 94 lbs/yr for TP.

COM-003: This planned project, located within the C-7 drainage basin, covers an area of approximately 47 acres and includes 7,773 linear feet of exfiltration trench, a 240 cfs pump station, and 28 injection wells (176 cfs total). Removal of nutrients for this area is estimated at 429 lbs/yr for TN and 71 lbs/yr for TP.

4.2.1.7 North Bay Village

North Bay Village provided a total of 19 projects within the study area that include upgrades to their existing stormwater system, education efforts, monitoring, studies, and maintenance activities. These projects are listed in Table 4-6 with the project name, type of project, status, and eligibility. Of the 19 projects, only two projects were identified as eligible as most are studies, sewer inspections, maintenance, education efforts, and monitoring/data collection.

Table 4-6 North Bay Village Projects

| ID | Name | Type | Status | Eligible | TN Load Removal (lbs/yr) | TP Load Removal (lbs/yr) |
| --- | --- | --- | --- | --- | --- | --- |
| - NBV-001 | Stormwater Outfall Rehab | Stormwater System Rehabilitation | Completed | No | N/A | N/A |
| - NBV-002 | Street Sweeping Services | Street Sweeping | Canceled | No | N/A | N/A |
| - NBV-003 | Civic Park & Boat Launch | Creating/ Enhancing Living Shoreline | Planned | No | N/A | N/A |
| - NBV-004 | NBI Stormwater Pump Stations | Stormwater System Upgrade | Planned | Yes | 114.2 | 20.6 |
| - NBV-005 | CRS Program | Enhanced Public Education | Underway | No | N/A | N/A |
| - NBV-006 | Fertilization Ordinance | Fertilizer Reduction | Ongoing | Yes | 13.0 | 2.3 |
| - NBV-007 | Drain Filtration Baskets | Catch Basin Inserts/Inlet Filter Cleanout | Planned | No | N/A | N/A |
| - NBV-008 | Wastewater Pump Station Improvements | Sanitary Sewer and (WWTF) Maintenance | Planned | No | N/A | N/A |
| - NBV-009 | CCTV Inspection | Sanitary Sewer Inspections | Ongoing | No | N/A | N/A |
| - NBV-010 | Stormwater Master Plan (link provided) | Study | Completed | No | N/A | N/A |
| - NBV-011 | Gas Leaf Blower Ordinance | Education Efforts | Ongoing | No | N/A | N/A |
| - NBV-012 | Sanitary Sewer Rehab Project | Sanitary Sewer and (WWTF) Maintenance | Ongoing | No | N/A | N/A |
| - NBV-013 | SSES Cycle 2 Eval | Study | Completed | No | N/A | N/A |
| - NBV-014 | SSES Cycle 3 Eval | Study | Planned | No | N/A | N/A |
| - NBV-015 | Plastic straws and styrofoam Ordinance | Enhanced Public Education | Ongoing | No | N/A | N/A |
| - NBV-016 | Ecopod Partnership | Education Efforts | Ongoing | No | N/A | N/A |
| - NBV-017 | Community Outreach | Education Efforts | Ongoing | No | N/A | N/A |
| - NBV-018 | SWPPP | Education Efforts | Ongoing | No | N/A | N/A |
| - NBV-019 | Waterkeepers Water Quality Testing | Monitoring/Data Collection | Ongoing | No | N/A | N/A |
| Total: |  |  |  |  | 127.2 | 23.9 |

4.2.1.7.1 Stormwater

The eligible stormwater project identified for North Bay Village included upgrades to a stormwater pump station that had operational issues since its construction in 1996.  The station has not been functional per design and not used on a regular basis. The pump station was evaluated in 2020-2021 by BCC Engineering (BCC 2021) which identified a number of issues/deficiencies in the drainage well and pump station design as the cause for a non-functional system.

4.2.1.7.2 Ordinances and Education Programs

In addition to the stormwater capital improvement project NBV-004, North Bay Village has adopted the FDEP’s model ordinance for Florida friendly fertilizer use covering all three islands of the city.  Per FDEP guidance for BMAP Credit for Education Activities, credit is available for 0.5% of the TN and TP loading for Urban land use. Based on TN and TP loadings of 2,596 lbs/yr and 467 lbs/yr respectively, credits of 13 and 2.3 lbs/yr is eligible.

4.2.1.8 Unincorporated Miami-Dade County

Over 200 capital improvement projects were provided by Miami-Dade County including both septic to sewer projects and stormwater improvement projects.  In addition, the County provides educational efforts and maintains ordinances that are eligible for pollutant loading credits.

4.2.1.8.1 Septic to Sewer

The Miami-Dade WASD identified a number of projects to extend sanitary sewer service within the RAP Study Area with the intent to convert OSTDSs to the centralized wastewater system.  The nutrient reductions associated with these projects were estimated using the ArcNLET model.  WASD provided a total of 15 projects within the study area to convert OSTDSs to sanitary sewer.  These projects are listed in Table 4-7 with the project PCTS (Project Control Tracking System) ID, project name, type of project, and status.

Table 4-7 Septic to Sewer Projects Within Unincorporated Miami Dade County

| PCTS ID | Project Name | Number of OSTDSs to be Phased Out | Type | Status |
| --- | --- | --- | --- | --- |
| - 15146 | GOB Basin 1262 Wastewater and Water system Improvements Green Tech Corridor‐S‐1 South | 246 | OSTDS Phase Out | Planned |
| - 15147 | WW Service to Commercial Properties‐Greentech Corridor S‐2 ‐Phase1, Gravity/Force/Watermains ‐ Eastside | 17 | OSTDS Phase Out | Planned |
| - 15149 | Water and Wastewater Service to Commercial Properties ‐Greentech Corridor ‐S‐4‐Gravity‐Force ‐Water Mains | 76 | OSTDS Phase Out | Planned |
| - 15156 | Wastewater Service to Commercial Properties ‐ Project D2‐A‐1 Gravity Mains Phase 2 | 44 | OSTDS Phase Out | Planned |
| - 15158 | WW Service to Commercial Properties‐Project D2‐C‐1‐Gravity/ Force and Water Mains | 62 | OSTDS Phase Out | Planned |
| - 15835 | Pump Station D-2 – C-2 | 109 | OSTDS Phase Out | Planned |
| - 15978 | Wastewater Service to Commercial Properties ‐ Greentech Corridors‐2 Phase 2 Gravity /Force Main /Water (Westside) | 31 | OSTDS Phase Out | Planned |
| - 16054 | Hialeah Transfer Area Phase I ‐ Proposed 16‐inch DI Water Main and Gravity System Along NW 37th Avenue, From NW 36th Street to NW 82nd Street | 5 | OSTDS Phase Out | Planned |
| - 16113 | GOB Basin 1262 North Green Tech Corridors‐1 North Wastewater and Water System Improvements | 59 | OSTDS Phase Out | Planned |
| - 16126 | GOB Basin 1204 Wastewater Improvements | 12 | OSTDS Phase Out | Planned |
| - 16139 | WW Service to Commercial Properties‐Green Tech Corridor Basin 0107 | 67 | OSTDS Phase Out | Planned |
| - 16155 | WW Service to Commercial Properties‐Greentech Corridor Basin 191 | 75 | OSTDS Phase Out | Planned |
| - 16245 | East And West Little River AAA Gravity/Lateral Septic to Sewer Project | 368 | OSTDS Phase Out | Planned |
| - 14882/14884 | NW 7th Ave Wastewater Collection System Expansion: Sub‐Basin D2‐D‐2 ‐ Gravity Sewers and Force Main | 116 | OSTDS Phase Out | Planned |
| Total |  | 1,287 | - | - |

The project information was provided in the form of plans, spreadsheets, and reports. Based on the review of information provided, a total of 1,287 OSTDSs were identified within WASD’s service area of the study area for the planned OSTDS phase out. The locations of these projects are shown on Figure 4-1. These 1,287 OSTDSs were included in the ArcNLET model to simulate the potential nitrate load reduction benefit to the receiving water bodies due to the planned septic to sewer projects.

Table 4-8 lists the septic to sewer projects within the study area and the corresponding nitrate load reduction benefit from these projects as well as the waterbodies that will benefit. Overall, the tool estimated a potential reduction of approximately 3,174 lbs/yr of TN. The projects that are not included in Table 4-8 are too far (beyond 1,500 ft) from the water bodies to provide any significant nitrate load reduction. This is illustrated by the nitrate plume map shown on Figure 4-2 which shows nitrate plumes originating from the OSTDSs located far from water bodies that are assumed for the purposes of this RAP not to reach the water bodies. Projects that are located within the 1,500 feet of surface water bodies are assumed to contribute to approximately 99% of the anticipated load reduction from these septic to sewer conversions.

Table 4-8 Simulated Nitrate Load Reduction Due to the Planned Septic to Sewer Projects in Unincorporated Miami Dade County

| PCTS ID | Type | Number of OSTDSs to be Phased Out | Status | Eligible | Water Body Benefitted | TN Load Removal (lbs/yr) |
| --- | --- | --- | --- | --- | --- | --- |
| - 162451 | OSTDS Phase Out | 368 | Planned | Yes | Little River / Upper North Biscayne Bay | 2,749.7 |
| - 151561 | OSTDS Phase Out | 44 | Planned | Yes | Little River | 140.7 |
| - 161131 | OSTDS Phase Out | 59 | Planned | Yes | Little River | 94.4 |
| - 14882/148841 | OSTDS Phase Out | 116 | Planned | Yes | Biscayne Canal | 85.3 |
| - 151581 | OSTDS Phase Out | 62 | Planned | Yes | Little River | 73.0 |
| - 161261 | OSTDS Phase Out | 12 | Planned | Yes | Little River | 19.9 |
| - 15835 | OSTDS Phase Out | 109 | Planned | Yes | Little River | 6.5 |
| - 16139 | OSTDS Phase Out | 67 | Planned | Yes | Little River | 2.0 |
| - 15146 | OSTDS Phase Out | 246 | Planned | Yes | Little River | 1.9 |
| - 15147 | OSTDS Phase Out | 17 | Planned | Yes | Little River | 0.3 |
| Total |  | 1,100 | - | - | - | 3,173.8 |

1  - Projects located within 1,500 feet from surface water bodies.

4.2.1.8.2 Stormwater

The Miami-Dade RER/DERM oversees stormwater management within the unincorporated areas of the County. DERM provided a total of 134 projects within the study area that include upgrades to their existing stormwater system.  Of the 134 projects, 35 was identified as eligible as they are projects that are in planning stages and the County has committed to their implementation.  The remaining 99 projects are not eligible due to various reasons (future projects, no nutrient reduction provided, completed prior to 2017, etc.). .  In addition to the 35 eligible projects, four exfiltration projects were identified in the County’s GIS data occurring after July 2017 and not listed in the projects provided.

Due to the number of projects provided by the County, only the eligible projects are shown in Table 4-9. The remaining County projects are provided in Table 4-1.

Figure 4-2 – Nitrate Plume Simulated by ArcNLET Model

Table 4-9 Miami Dade County Eligible Stormwater Projects

| ID | Name | Type | Status | TN Load Removal (lbs/yr) | TP Load Removal (lbs/yr) |
| --- | --- | --- | --- | --- | --- |
| MDC-021 | NW 72 Street from NW 8 Avenue to NW 10 Avenue Drainage Improvement | Exfiltration Trench | Completed | 14.6 | 2.4 |
| MDC-023 | NW 109 Street from NW 11 Avenue to NW 10 Avenue | Exfiltration Trench | Completed | 8.8 | 1.4 |
| MDC-024 | NPDES OUTFALL (53-41-12-NE001C) North Miami Ave from NW 71 St to Little River Canal [S:\ 4036, 4035, 4006] | Exfiltration Trench | Completed | 32.3 | 5.2 |
| MDC-025 | NW 96 ST FROM NW 13 AVE TO NW 14 AVE (1349 NW 96 St) | Exfiltration Trench | Completed | 10.1 | 1.6 |
| MDC-026 | NW 95 Terr Between NW 13 Ave to NW 14 Ave | Exfiltration Trench | Completed | 9.0 | 1.5 |
| MDC-027 | NE 88 Terr from Biscayne Blvd to east end of road | Exfiltration Trench | Completed | 3.9 | 0.6 |
| MDC-029 | Improvements to Five (5) Dirt Roads (PWD Project No. 20130232) (GOB) | Stormwater System Upgrade | Ongoing | 47.6 | 7.7 |
| MDC-031 | Drainage Improvement Project (PWD Project No. 20130013) (SWU) | Exfiltration Trench | Ongoing | 59.4 | 9.6 |
| MDC-032 | PWD Project No. 20040389 (PTP) (JPA with City of Miami) | Exfiltration Trench | Ongoing | 48.6 | 8.0 |
| MDC-033 | Drainage Conveyance Line and Emergency Overflow PWD Project No. 20130253 | Exfiltration Trench | Ongoing | 22.0 | 3.6 |
| MDC-034 | Drainage Improvement PWD Project No. 20120161) (QNIP) | Exfiltration Trench | Completed | 52.0 | 8.4 |
| MDC-035 | Drainage Improvement PWD Project No. 20130156) (GOB/SWU) | Exfiltration Trench | Ongoing | 59.9 | 10.4 |
| MDC-037 | 20130163 Drainage Improvement Project Multiple Sites | Exfiltration Trench | Completed | 28.9 | 4.7 |
| MDC-038 | NW 80 TER FROM NW 32 AVE TO NW 36 AVE Drainage Improvement Project | Exfiltration Trench | Completed | 43.5 | 7.0 |
| MDC-039 | NW 103 Street to NW 111 Street from NW 2 Court to NW 5 Avenue | Exfiltration Trench | Ongoing | 29.8 | 4.8 |
| MDC-040 | NW 11 Avenue to NW 8 Avenue from NW 107 Street to NW 111 Street | Exfiltration Trench | Ongoing | 9.3 | 1.5 |
| MDC-041 | NE 12 Avenue and NE 111 Street | Exfiltration Trench | Ongoing | 4.3 | 0.8 |
| MDC-042 | NW 100 Terrace and NW 6 Avenue | Exfiltration Trench | Ongoing | 24.2 | 3.9 |
| MDC-043 | NW 133 ST from NW 13 AVE to NW 16 AVE / NW 132 TER from NW 13 AVE to NW 15 AVE Drainage Improvements | Exfiltration Trench | Ongoing | 34.7 | 5.6 |
| MDC-044 | NW 1 AVE FROM NW 131 ST TO NW 135 ST | Exfiltration Trench | Ongoing | 3.7 | 0.6 |
| MDC-045 | NW 34 AVE RD & NW 87 TER DRAINAGE IMPROVEMENTS | Exfiltration Trench | Ongoing | 10.0 | 1.6 |
| MDC-046 | NW 24 Avenue from NW 79 Street to NW 78 Street | Exfiltration Trench | Ongoing | 0.9 | 0.2 |
| MDC-052 | NW 83 Street from NW 31 Avenue to NW 33 Avenue | Exfiltration Trench | Ongoing | 14.8 | 2.4 |
| MDC-053 | NW 24 Avenue and NW 87 Terrace | Exfiltration Trench | Ongoing | 44.5 | 7.2 |
| MDC-054 | NE 90 Street from NE 10 Court to N. Bayshore Drive | Exfiltration Trench | Ongoing | 82.8 | 14.4 |
| MDC-055 | NW 18 from NW 62 St to NW 71 St | Exfiltration Trench | Ongoing | 29.1 | 4.7 |
| MDC-056 | NW 133 ST TO 134 ST & 17 CT 13315 NW 17 CT | Exfiltration Trench | Ongoing | 64.2 | 10.4 |
| MDC-058 | NE MIAMI CT from NE 135 St to South Biscayne River Dr | Exfiltration Trench | Ongoing | 56.8 | 9.2 |
| MDC-059 | NE 164 St to Spur #4 Canal between N Biscayne Dr and NE 5 Ave (C8-N-17) | Exfiltration Trench | Ongoing | 165.5 | 26.7 |
| MDC-061 | General drainage improvements mitigation of flood complaints | Exfiltration Trench | Ongoing | 42.8 | 6.9 |
| MDC-062 | NW 12-17 ave from 75-79 st and additional sites. | Exfiltration Trench | Ongoing | 178.0 | 28.8 |
| MDC-063 | CRS North Mitigation of Repetitive Losses | Exfiltration Trench | Ongoing | 181.1 | 29.3 |
| MDC-064 | Pilot Project Site N.1 for Removal of TN, TP and Bacteria (NW 107 STREET FROM NW 19 AVE TO NW 18 AVE) | Stormwater - Biological/ Bacteria Treatment | Ongoing | 20.3 | 2.7 |
| MDC-065 | Pilot Project Site N.2 for Removal of TN, TP and Bacteria (NW 95 ST FROM NW 17 AVE TO NW 13 AVE) | Stormwater - Biological/ Bacteria Treatment | Ongoing | 22.0 | 2.9 |
| MDC-066 | Pilot Project Site N.3 for Removal of TN, TP and Bacteria (NW 83 ST TO NW 85 ST FROM NW 5 AVE TO N MIAMI AVE.) | Stormwater - Biological/ Bacteria Treatment | Ongoing | 10.1 | 1.3 |
| MDC-154 | NW 105th, NW 19th Ave and NW 104th St | Exfiltration Trench | Completed | 22.0 | 3.6 |
| MDC-155 | NW 29th Ct between NW 106th and NW 103rd | Exfiltration Trench | Completed | 10.5 | 1.7 |
| MDC-156 | Seaboard Industrial Area (NW119th St, NW27th Ave, NW135th St, | Exfiltration Trench | Completed | 23.1 | 3.7 |
| MDC-157 | NW 2nd Ave, NW 161St St | Exfiltration Trench | Completed | 22.0 | 3.6 |
| MDC-158 | Ordinances & Education Programs | Ordinances / Education | Ongoing | 4,871 | 792 |
| Total: |  |  |  | 6,418.1 | 1,042.7 |

4.2.1.8.3 Ordinances and Education Programs

Miami-Dade County has currently adopted seven of the nine activities identified in FDEP’s BMAP Credit for Education Activities.  These activities, along with their available credit, include:

Florida Yards and Neighborhoods Program (3.0%)

County ordinance for landscaping (0.5%)

County ordinance for fertilizer (0.5%)

County ordinance for pet waste management (0.50%)

Public service announcements (0.25%)

Informational packets (0.25%)

Website providing information of reducing nutrient loadings (0.25%)

Overall, a 4.25% credit is available for the TN and TP loading in unincorporated Miami-Dade County. The catchment area used in BMPTRAINS for unincorporated Miami-Dade County was 10,791 acres.  Urban land use within this area is 10,441 acres or roughly 97%.  With TN and TP loadings of 95,643 lbs/yr and 15,538 lbs/yr respectively, credits of 5,021 and 816 lbs/yr is eligible. Adjusted for the urban land use, the credits for TN and TP are 4,871 and 792 lbs/yr, respectively.

4.2.2 Florida Department of Transportation (FDOT)

One project was identified in the County’s GIS data that is maintained by FDOT and was constructed after July 2017 (January 2018).  Other FDOT-maintained exfiltration trench in the study area were constructed prior to 2017 and thus not eligible.  The project included 374 linear feet of exfiltration trench located in the C-8 Canal basin within the City of North Miami limits along SR 922 (NW 125th St) between I-95 and NW 2nd Ave.  The estimated pollutant removal for this project is 8.0 lbs/year for TN and 1.3 lbs/year for TP.

FDOT District 6 will be engaged by the County and the stakeholders as part of future phases of the RAP.

4.2.3 Florida Department of Agriculture and Consumer Services (FDACS)

There are currently no agricultural lands within the RAP Study Area and therefore none that fall under the jurisdiction of FDACS. However as future phases of the RAP are implemented, the County and the local stakeholders will coordinate with FDACS on applicable projects.

4.2.4 South Florida Water Management District (SFWMD)

The SFWMD will be engaged by the County and the stakeholders as part of future phases of the RAP and District projects that provide load reductions will be quantified.

4.3 Overall Program Plan

As part of the overall plan to reduce nutrients to North Biscayne Bay and to provide reasonable assurance, the eligible project load reductions were compared to the overall quantified reduction goals for the RAP based on the WASP analysis described in Section 3.3.1. It should be noted that the combined percent reduction goals for both TN and TP (37% and 41%, respectively) is based on loading from the entire C-7 and C-8 Canal systems and associated tributary areas. As the RAP Study Area comprises a sub-set of the overall C-7 and C-8 Canal watersheds, it is important to consider that the annual canal load and estimated percent reductions represent reductions from the entire watershed area, not just the RAP Study Area. As discussed in Section 2.5, calibrated models were not developed as part of this effort due to schedule limitations to quantify a source loading inventory for the study area. Therefore, in order to approximate the load associated with the RAP Study Area, a percentage based on the ratio of watershed area was applied to the watershed loading. Based on review of the Miami-Dade County FY2021 Stormwater Management Program Update, land use and soils in the upstream portions of the C-7 and C-8 Canal watersheds are also fairly homogenous throughout compared to the study area (the C-7 Canal watershed more so than the C-8 Canal watershed). The resulting ratio, based on watershed area, and approximated loading for the RAP Study Area, as well as required load reductions, are shown below in Tables 4-10 and 4-11.

Table 4-10 C-7 Canal Watershed Loading and Required Load Reductions

|  | Values for C-7 Canal at S-27 | Approximated Values for C-7 Watershed | Values for C-7 Canal Watershed in RAP Study Area | % Reduction Needed to meet Interim NNC Targets | Load Reduction from the RAP Study Area to meet Interim NNC Targets |
| --- | --- | --- | --- | --- | --- |
| - Tributary Area (sq mi) | 32 | 36 | 16.7 | N/A | N/A |
| - Flow (cfs) | 2081 | N/A | N/A | N/A | N/A |
| - Annual TN Load (lbs/yr) | 456,029 | 513,0332 | 239,4152 | 42% | 100,554 |
| - TN Average Concentration (mg/l) | 1.1 | N/A | N/A | N/A | N/A |
| - Annual TP Load (lbs/yr) | 10,982 | 12,3552 | 5,7662 | 44% | 2,537 |
| - TP Average Concentration (mg/l) | 0.027 | N/A | N/A | N/A | N/A |

1 – Excludes 2020 flow data

2 – Loading estimated based on ratio of total watershed area to area upstream of S-27. RAP Study Area is 47% of total 	watershed area.

Table 4-11 C-8 Canal Watershed Loading and Required Load Reductions

|  | Values for C-8 Canal at S-28 | Approximated Values for C-8 Watershed | Values for C-8 Canal in the RAP Study Area | % Reduction Needed to meet Interim NNC Targets | Load Reduction from the RAP Study Area to meet Interim NNC Targets |
| --- | --- | --- | --- | --- | --- |
| - Tributary Area (sq mi) | 28 | 36.9 | 7.1 | N/A | N/A |
| - Flow (cfs) | 110 | N/A | N/A | N/A | N/A |
| - Annual TN Load (lbs/yr) | 188,442 | 248,0031 | 47,4471 | 26% | 12,336 |
| - TN Average Concentration (mg/l) | 0.87 | N/A | N/A | N/A | N/A |
| - Annual TP Load (lbs/yr) | 4,982 | 6,5571 | 1,2541 | 35% | 439 |
| - TP Average Concentration (mg/l) | 0.023 | N/A | N/A | N/A | N/A |

1 – Loading estimated based on ratio of total watershed area to area upstream of S-28. RAP Study Area is 19% of total 	watershed area.

Based on the potential discrepancies in the 2020 flow data for the C-7 Canal as previously described in Section 2.5.3.1, further clarification will be needed on the validity of the 2020 flow data as it directly pertains to setting (and achieving) targets in the Biscayne Bay estuary.  The loads and required load reductions shown in Table 4-10 may be subject to change as more information becomes available in the future.

Project load reductions for eligible projects described in Section 4.2 are summarized below in Table 4-12 with the total load reduction by municipal stakeholder presented.

Table 4-12 Project Load Reduction by Stakeholder

| Stakeholder | Area (acres) | Project TN Load Reduction (lbs/yr) | Project TP Load Reduction (lbs/yr) |
| --- | --- | --- | --- |
| - City of North Miami | 2,559 | 676.0 | 417.2 |
| - City of North Miami Beach | 81 | N/A | N/A |
| - Miami Shores Village | 1,597 | 15.3 | 2.3 |
| - Village of El Portal | 266 | 1,312.3 | 13.9 |
| - Biscayne Park | 398 | N/A | N/A |
| - City of Miami | 2,429 | 1,357.2 | 221.9 |
| - North Bay Village | 235 | 127.2 | 23.9 |
| - Unincorporated Miami Dade County | 10,791 | 6,418.1 | 1,042.7 |
| - FDOT District 6 | -- | 8.0 | 1.3 |
| - Totals: | 18,356 | 9,914.1 | 1,723.2 |

Table 4-12 shows eligible projects are estimated to reduce approximately 9,914 lbs of TN and 1,723 lbs of TP on an annual basis.  Load reductions associated with projects will continue to be refined as additional project information and projects are provided in future phases.

4.3.1 Project Estimated Costs and Schedules

A summary of the eligible projects identified in this study is provided in Table 4-13. Where provided by the stakeholders, the estimated project construction costs and estimated completion dates are shown in the table.

Table 4-13 Summary of Eligible Project Estimated Cost and Completion Date

| Project ID | Project Type | Status | Estimated Construction Cost (million $) | Completion Date (Estimated or Actual) |
| --- | --- | --- | --- | --- |
| - CNM-005 | Catch Basin Inserts/Inlet Filter Cleanout | Completed | -- | -- |
| - CNM-006 | OSTDS Phase Out | Planned | -- | Mar 2024 |
| - CNM-007 | Street Sweeping | Ongoing | -- | -- |
| - CNM-009 | BMP Cleanout | Ongoing | -- | -- |
| - CNM-015 | Exfiltration Trench | Completed | -- | Dec 2019 |
| - COM-001 | Stormwater System Upgrade | Planned | $ 14.50 | Sep 2027 |
| - COM-002 | Stormwater System Upgrade | Planned | $ 20.00 | Mar 2028 |
| - COM-003 | Stormwater System Upgrade | Planned | $ 24.30 | Apr 2028 |
| - DOT-001 | Exfiltration Trench | Planned | -- | Jan 2018 |
| - MDC-001 | OSTDS Phase Out | Planned | -- | -- |
| - MDC-002 | OSTDS Phase Out | Planned | -- | -- |
| - MDC-003 | OSTDS Phase Out | Planned | -- | -- |
| - MDC-004 | OSTDS Phase Out | Planned | -- | -- |
| - MDC-005 | OSTDS Phase Out | Planned | -- | -- |
| - MDC-006 | OSTDS Phase Out | Planned | -- | -- |
| - MDC-007 | OSTDS Phase Out | Planned | -- | -- |
| - MDC-008 | OSTDS Phase Out | Planned | -- | -- |
| - MDC-009 | OSTDS Phase Out | Planned | -- | -- |
| - MDC-010 | OSTDS Phase Out | Planned | -- | -- |
| - MDC-011 | OSTDS Phase Out | Planned | -- | -- |
| - MDC-012 | OSTDS Phase Out | Planned | -- | -- |
| - MDC-013 | OSTDS Phase Out | Planned | -- | -- |
| - MDC-014 | OSTDS Phase Out | Planned | -- | -- |
| - MDC-015 | OSTDS Phase Out | Planned | -- | -- |
| - MDC-016 | OSTDS Phase Out | Planned | -- | -- |
| - MDC-017 | OSTDS Phase Out | Planned | -- | -- |
| - MDC-018 | OSTDS Phase Out | Planned | -- | -- |
| - MDC-019 | OSTDS Phase Out | Planned | -- | -- |
| - MDC-020 | OSTDS Phase Out | Planned | -- | -- |
| - MDC-021 | Exfiltration Trench | Completed | $ 0.30 | -- |
| - MDC-023 | Exfiltration Trench | Completed | $ 0.16 | -- |
| - MDC-024 | Exfiltration Trench | Completed | $ 0.35 | -- |
| - MDC-025 | Exfiltration Trench | Completed | $ 0.11 | -- |
| - MDC-026 | Exfiltration Trench | Completed | $ 0.05 | -- |
| - MDC-027 | Exfiltration Trench | Completed | $ 0.04 | -- |
| - MDC-029 | Stormwater System Upgrade | Planned | -- | -- |
| - MDC-031 | Exfiltration Trench | Planned | $ 0.11 | -- |
| - MDC-032 | Exfiltration Trench | Planned | -- | -- |
| - MDC-033 | Exfiltration Trench | Planned | $ 0.75 | -- |
| - MDC-034 | Exfiltration Trench | Completed | $ 0.19 | -- |
| - MDC-035 | Exfiltration Trench | Planned | $ 0.61 | -- |
| - MDC-037 | Exfiltration Trench | Completed | $ 0.13 | -- |
| - MDC-038 | Exfiltration Trench | Completed | $ 0.23 | -- |
| - MDC-039 | Exfiltration Trench | Planned | $ 0.79 | -- |
| - MDC-040 | Exfiltration Trench | Planned | $ 0.80 | -- |
| - MDC-041 | Exfiltration Trench | Planned | $ 0.11 | -- |
| - MDC-042 | Exfiltration Trench | Planned | $ 0.09 | -- |
| - MDC-043 | Exfiltration Trench | Planned | $ 0.29 | -- |
| - MDC-044 | Exfiltration Trench | Planned | $ 0.19 | -- |
| - MDC-045 | Exfiltration Trench | Planned | $ 0.14 | -- |
| - MDC-046 | Exfiltration Trench | Planned | $ 0.12 | -- |
| - MDC-052 | Exfiltration Trench | Planned | $ 0.92 | -- |
| - MDC-053 | Exfiltration Trench | Planned | $ 0.24 | -- |
| - MDC-054 | Exfiltration Trench | Planned | $ 0.84 | -- |
| - MDC-055 | Exfiltration Trench | Planned | $ 0.95 | -- |
| - MDC-056 | Exfiltration Trench | Planned | $ 0.26 | -- |
| - MDC-058 | Exfiltration Trench | Planned | $ 0.11 | -- |
| - MDC-059 | Exfiltration Trench | Planned | $ 0.59 | -- |
| - MDC-061 | Exfiltration Trench | Planned | $ 0.18 | -- |
| - MDC-062 | Exfiltration Trench | Planned | $ 0.71 | -- |
| - MDC-063 | Exfiltration Trench | Planned | $ 1.10 | -- |
| - MDC-064 | Stormwater - Biological/ Bacteria Treatment | Planned | $ 0.16 | -- |
| - MDC-065 | Stormwater - Biological/ Bacteria Treatment | Planned | $ 0.15 | -- |
| - MDC-066 | Stormwater - Biological/ Bacteria Treatment | Planned | $ 0.24 | -- |
| - MDC-154 | Exfiltration Trench | Completed | -- | Mar 2018 |
| - MDC-155 | Exfiltration Trench | Completed | -- | Dec 2017 |
| - MDC-156 | Exfiltration Trench | Completed | -- | Aug 2017 |
| - MDC-157 | Exfiltration Trench | Completed | -- | Jul 2018 |
| - MSV-001 | Stormwater System Upgrade | Underway | -- | -- |
| - MSV-011 | Exfiltration Trench | Completed | -- | Dec 2019 |
| - NBV-004 | Stormwater System Upgrade | Planned | $ 3.00 | 2025 |
| - VEP-001 | OSTDS Phase Out | Underway | $ 4.00 | Jun 2024 |
| - VEP-002 | Stormwater System Upgrade | Underway | $ 0.50 | Dec 2024 |
| Totals: |  |  | $82.55 |  |

Table 4-1 Project Data

This page intentionally left blank.

___________________________________