Documentation in Support of Category 4e

| Waterbody/Watershed Identification |  |  |  |
| --- | --- | --- | --- |
|  | Organization | Town of Lake Clarke Shores |  |
|  | Point of Contact | Daniel P Clark, PE Town Manager 1701 Barbados Road Lake Clarke Shores DClark@LakeClarke.org C: 561-722-8110 O: 561-964-1515 ext. 1112 |  |
|  | Waterbody(s) | The primary waterbody covered under this 4e Plan is Lake Clarke (Waterbody Identification [WBID] 3245B). However, the projects to be implemented under this plan serve areas that also contribute to the downstream receiving waterbody, the Central Segment of the Lake Worth Lagoon (WBID 3226F1). |  |
|  | No. Waterbody / Pollutant Combinations | Primary: Lake Clarke in the Lake Worth Lagoon – Palm Beach Coast, C-51 Planning Unit, Group 3. Verified for Fecal Coliform, Biology and Nutrients (Chlorophyll-a [Chl-a], and Total Phosphorus [TP]). |  |
|  | EPA Completed TMDL | The Environmental Protection Agency (EPA) has not completed a Total Maximum Daily Load (TMDL) for Lake Clarke or the Central Segment of Lake Worth Lagoon. |  |
| Description of Baseline Conditions |  |  |  |
|  | Watershed(s) | Group Name: Lake Worth Lagoon – Palm Beach Coast Group Number: 3 |
|  | Baseline Data | Lake Clarke Nutrient data are from IWR Run 60; nine stations; Chl-a corrected annual geometric mean (AGM) ≤ 20 micrograms per liter (µg/L) exceeded in 3 of 5 years in the verified period with data: Chl-a AGM 2013 (23 µg/L) 2014 (14 µg/L) 2017 (30 µg/L) 2018 (12 µg/L) 2019 (26 µg/L) TP exceed Chl-a-specific nutrient criteria in 3 of the 5 years in the verified period with data: TP AGM 2013 (0.070 mg/L) 2014 (0.090 mg/L) 2017 (0.060 mg/L) 2018 (0.080 mg/L) 2019 (0.070 mg/L) The Lake Vegetation Index was less than 43 for both sampling events: LVI (n=2) WBID Mean (26) Mean 1 (19), Mean 2 (32) AGMs for TN are also provided as a reference: TN AGM 2013 (1.07 mg/l) 2014 (0.91 mg/l) 2017 (1.13 mg/l) 2018 (1.25 mg/l) 2019 (0.98 mg/l) The data above are from the Biennial Assessment Draft Master List. |
|  | Map | The WBID for Lake Clarke (3245B) is drawn around the lake itself and Attachment 1 shows that it is contained within the C-51 East WBID (3245F). The C-51 East WBID contains an interconnected system of canals and control structures. Attachment 2, which was developed by the Lake Worth Drainage District, shows the primary direction of flow within its system. The information in Attachment 2 – combined with the Digital Elevation Model (DEM) based on the 2018 Florida Division of Emergency Management (FDEM) Light Detection and Ranging (LiDAR) and the watershed boundary developed for the Draft Lake Osborne TMDL – suggests the approximate watershed boundary shown in Attachment 1 based on predominant flow paths to Lake Clarke. The attachments also show the Lake Worth Lagoon (Central Segment) WBID (3226F1). |

| Evidence of Watershed Approach |  |  |
| --- | --- | --- |
|  | Area of Effort | Attachment 1 shows the approximate watershed area for Lake Clarke. This 4e Plan primarily covers the portion of the Town of Lake Clarke within that watershed with secondary coverage of Lake Worth Lagoon (Central Segment) based on the plan of improvements project area. |
|  | Key Stakeholders Involved and Their Roles | The Town of Lake Clarke Shores is the only stakeholder for this 4e Plan. The Village of Palm Springs and unincorporated Palm Beach County are also within the Lake Clarke watershed, but this Plan does not cover those areas. |
|  | Watershed Plan & Other Supporting Documentation | Attachment 3 shows that the watershed area in the 4e Plan includes the area within the septic-to-sewer area planned for the Town of Lake Clarke Shores that intersects the watershed boundary within WBID 3245B (Lake Clarke) and 3245F (C-51 East). Lake Clarke is included through the Town’s planned vegetation management, which comprises exotic vegetation harvesting and disposal from Lake Clarke. WBID 3245B is impaired for fecal coliform and nutrients based on the number of exceedances for the sample size. The objectives outlined by the MICA (Mediterranean Rd., Indian Rd., Caribbean Rd. and Arabian Rd.) West Neighborhood Infrastructure Improvement Plan (NIIP) Technical Memorandum (Attachment 4) and the Town’s planned vegetation management will provide benefit to these impairments. The WBID drainage area corresponds to the five phases of the following key project: Town of Lake Clarke Shores Water Quality Improvement Program. That Program includes septic-to-sewer conversions, creation of stormwater treatment swales along the roads where sewer is being installed, and the activities in the Town’s planned vegetation management. |
|  | Point Sources and Indirect Source Monitoring (Sites) | Attachment 5 from the Cycle 4 – Year 3 (National Pollutant Discharge Elimination System [NPDES] Municipal Separate Storm Sewer System [MS4]) Joint Annual Report supplemental information – as modified based on more recent data provided by the Town - shows that most of the Town of Lake Clarke Shores is served by septic systems. The Town’s stormwater management system is the only other point source discharge to Lake Clarke. No MS4 monitoring sites are in Lake Clarke. The closest MS4 monitoring station is Site C51S155 (Figure 5-1 in the Cycle 4 – Year 4 (NDPES MS4) Joint Annual Report [Attachment 6]). Most of the Town is considered to be in the MS4 area. Attachment 7, which is taken from the Cycle 4 – Year 3 (NPDES MS4) Joint Annual Report supplemental information, shows the portion of the Town in the MS4 area. The permit number is FLS000018-004, which is shown in the Cycle 4 – Year 4 Individual Annual Report (Attachment 8). |
|  | Nonpoint Sources | The Town’s stormwater management system is discussed here even though it is considered a point source because of the MS4 permit. The aerial imagery in Attachment 9 shows the land use in Lake Clarke Shores. Medium-density residential is the predominant land use (approximately 79 percent), with water (approximately 15 percent), commercial (approximately 5 percent), and major highways (approximately 1 percent) accounting for the remainder. Nonpoint source pollutant loading includes direct runoff, base flow, and atmospheric deposition. |
|  | Water Quality Criteria | This 4e plan is expected to achieve a measurable reduction in bacteria and nutrients and improve the Lake Vegetation Index (LVI) in Lake Clarke, with expectation to attain applicable water quality standards. However, it does not cover all the jurisdictions discharging pollutants to Lake Clarke. |
|  | Restoration Work | The Town of Lake Clarke Shores Water Quality Improvement Program consists of three elements that will improve water quality in Lake Clarke and, to a lesser degree, the Central Segment of the Lake Worth Lagoon: - A septic-to-sewer program that is in five phases (Attachment 3). - Stormwater retrofits – primarily through use of swales – in each of the septic-to-sewer areas. - Mechanical harvesting of invasive and nuisance aquatic plants meeting the 2021 Lake Worth Lagoon Management Plan Update Action WQ-3. The Town has completed a Sewer Master Plan, which defines a systematic and phased approach to removing 783 septic tanks from residential and commercial areas. The total project cost for all phases is approximately $21M. In 2016, the Town of Lake Clarke Shores began a phased approach to converting its businesses and homes from septic to a central sewer system. Phases 1A, 1B, and 1C are focused on commercial areas. All of Phase 1 construction removed several commercial septic systems equaling 81 homes. The total cost of Phase 1 is approximately $1,733,000. Lake Clarke Shores has paid $1,208,360 (70 percent) to date for Phase 1. Funding has been requested for the MICA West Water Quality Improvement Project, which is Phase 2 of the five-phase septic-to-sewer conversion project for Lake Clarke Shores that includes stormwater retrofits. MICA West includes constructing 7,200 feet of gravity sewer and a pump station for the MICA West area of Lake Clarke Shores and constructing 8,300 feet of grass-lined swales. The MICA West project will eliminate 135 on-site septic systems and convert them to central sewer. The design has been completed. Construction is expected to begin in July 2022 and will include constructing the sewer lines and pump station, re-establishing roadside swales, improving the drainage system, and upgrading asbestos-cement waterlines to polyvinyl chloride (PVC) waterlines. The Town intends to implement the final phases in less 7 years and will proceed as quickly as funding allows. Nutrient load reductions for the septic-to-sewer conversions are based on values used on the Caloosahatchee Estuary Basin Management Action Plan (January 2020): Total Nitrogen (TN) = 15 lb/year/onsite sewage treatment and disposal systems (OSTDS) and TP = 1.4 lb/yr/OSTDS. Converting the commercial septic systems to equivalent residential units results in a total load reduction of approximately 12,000 lb/yr of TN and 1,120 lb/yr of TP for all five phases. Significant reductions in fecal bacteria are also expected. Nutrient reductions for the stormwater retrofit element in the five phases are based on a 25-percent removal of a pro-rated share (55 percent) based on the area served of the pollutant load projections from the Cycle 4 – Year 3 (NPDES MS4) Joint Annual Report, with the septic loads removed before the 55 percent and 25 percent are applied. The 25-percent reduction is based on guidance from Statewide Best Management Practice (BMP) Efficiencies for Nonpoint Source Management of Surface Waters (Department of Environmental Protection, May 22, 2017). These values equate to 1,686 lb/yr for TN and 6.2 lb/yr for TP. |

| Critical Milestones/Monitoring |  |  |  |
| --- | --- | --- | --- |
|  | Anticipated Critical Milestone(s) and Completion Dates: |  | The expected completion dates for the septic-to-sewer conversions (and associated stormwater retrofits) and the exotic vegetation harvesting are: - Phase 1 – Complete. - Phase 2 – Second half of 2022. - Phases 3 through 5 – Before 2030. - Exotic vegetation harvesting: 2022 and as needed thereafter. |
|  | Monitoring Component |  | The Town proposes to coordinate with Palm Beach County to conduct monitoring for TN (TKN + NOx), TP (along with OP), and Chl-a. Monitoring will be performed quarterly and meet or exceed the minimum criteria specified in 62-302: Surface Water Quality Standards, Florida Administrative Code. Grab samples will be taken at the three stations throughout the lake, where previous monitoring has occurred (stations 21FLPBCH61, 62, and 69), and applicable sampling protocol will be followed. Attachment 10 shows the locations of the monitoring stations. The results of the monitoring efforts will be uploaded to DEP’s Watershed Integrated Network (WIN) through Palm Beach County’s Organization ID and will be used in subsequent biennial assessments. DEP has agreed to conduct the Lake Vegetation Index sampling as needed to meet data sufficiency requirements. |
| Other Key Dates |  |  |  |
|  | Estimated Date for Delisting from Verified List or Removal from Study List | WBID (3245B) is in the state’s Group 3 Lake Worth Lagoon – Palm Beach Coast basin. The current review and assessment cycle (the initial biennial assessment) is scheduled for completion in 2022. This waterbody is currently impaired for nutrients (chlorophyll-a and TP). Based on the 2022 draft assessment lists, Lake Clarke is being added to the State’s Verified List and federal 303(d) List. With the acceptance of this pollutant reduction plan by DEP and EPA, the assessment category will be updated from category 5 to 4e. The impaired parameters will then, instead be placed on to the Study List and submitted to EPA as an addition to the 303(d) list. Applicable data sufficiency will be required to fully assess the impaired parameters. If it’s determined that the parameters in question are no longer impaired, DEP is expected to request the WBID be delisted from the 303(d) List. |  |

| Financial Commitments |  |  |
| --- | --- | --- |
|  | Estimated Implementation Cost | The total project cost, including land acquisition (if applicable), is $21,000,000 for the septic-to-sewer and associated stormwater retrofits plus $700,000 for the initial exotic vegetation harvesting, totaling $21,700,000. The estimated 20-year operation and maintenance cost is $3,500,000, which includes two more exotic vegetation harvestings. |
|  | Land Acquisition (if applicable) | Funding Source: Total……………………………………………………………………...$ NA |
|  | Design and Construction (if applicable) | Funding Source: Total……………………………………………………………………...$ TBD. The Town intends to apply for grant funding and will apply for CWSRF funding if grant funding is not sufficient. The Town will co-fund the remainder. |

References:

(Attach any reports supporting the projects and restoration efforts)