Introduction:

After installing DEP Custom tools for a member of the Technical Services Team, I was asked if there was a quick way to divide the perimeter of a lake into 12 equal segments.  I immediately thought of the automated editing procedure available in ArcGIS whereby a line can be split into user defined segments, by number or length, and points can be constructed at each division. It was further mentioned that the process of dividing the perimeter into segments would need to be done for several more lakes.

Previous standard procedure in ArcGIS was for the lake perimeter to be manually measured to plot the 12 points which provide coordinates for the 12 equal line segments.  Additionally, lines are drawn between reference points to produce 12 polygons to be numbered in order to define the segments to be chosen for the Lake Vegetation Index sampling.

LVI Tool:

Being familiar with Python scripting in ArcGIS, I determined to investigate creation of a tool that would divide a lake perimeter into the necessary segments for LVI sampling purposes.

Review of information indicated that Python did not have a method to trace a given line, in ArcGIS 10.0 and previous versions.  Further investigation revealed that ArcGIS 10.1 does provide a method to trace along a given line using Python scripting.  During research, a function was found that could be easily adapted to the purpose that was needed for plotting equidistant points along a lake perimeter.  I immediately upgraded to ArcGIS10.1 which had just recently become available.

I proceeded to assemble a Python script which will do the following:

- Find the centroid of a lake polygon

  - Plot the centroid as a point shapefile

- Divide the lake perimeter into 12 equal segments

- Plot a point for each of the 12 segments of the perimeter

  - Find the coordinates of each point, in meters

- Draw a line using the centroid as a start point and the point coordinates  as an end point

  - Lines save as a shapefile

- Python script outputs consist of:

  - Point shapefile with the lake centroid

  - Line shapefile of lake perimeter comprised of 12 equal segments

  - Point shapefile containing 12 points, one for each segment

  - Line shapefile containing the 12 lines which create the “pie-shaped” reference segments

- Currently, all outputs are placed in a folder, on the C: drive, named “LVI_wbid”, ie. LVI_1647,  as the folder name.

To create a tool for use in ArcGIS, I created a custom Toolbox and add it to the ArcToolbox list.

The script is then added as a new tool in the custom toolbox.  The following user defined parameters are currently required after the tool has been opened:

- Input – the shapefile containing wbids, shapefile

- WBID  number - the wbid number as provided in the Input shapefile WBID column, text

- North Adjustment – after running the tool the first time, if the central most northerly directed line is not sufficiently aligned northward, a correction factor may be measured and added to the North Adjustment field and the tool can be run again, provided that outputs from the first run are not being used in a map, for the corrected output.

Comments:

Time to assemble and test the script was about 12 hours.  Additionally, research and initial documentation for the script required about 3 hours.

Before offering for broad use to others I would hope to do the following:

- Clean up the existing script and remove extraneous references/comments

- Add some Try and Except statements to improve error handling and user feedback

Snapshot of the tool interface:

Snapshot of outputs in folder in ArcCatalog:

Snapshot of optimal output in ArcMap:

Respectfully submitted,

Keith Reynolds