Caloosahatchee River and Estuary Basin Management Action Plan (BMAP)

Allocation Clipping Steps

August 2019

The Florida Department of Environmental Protection (DEP) created a geographic information system (GIS) base map for the Caloosahatchee River and Estuary watershed using output from the 2017 Hydrological Simulation Program – FORTRAN (HSPF) watershed model. Model output from the period of 2005 through 2014 were averaged to create this coverage. The GIS base map includes the fields summarized in Table 1.

DEP clipped and erased from the GIS base map to create separate shapefiles with each stakeholder's responsible area. This clipping was performed using the jurisdictional boundaries provided by each stakeholder. The individual stakeholder shapefiles were created by clipping out areas sequentially, as follows:

- Water management areas

- Natural lands associated with land cover codes 14 Upland Forest, 15 Wetland, 16 Water

- Agriculture lands associated with land cover codes 8 Sugar Cane, 9 Row and Field Crops, 10 Nurseries/Ornamentals/Vineyards, 11 Citrus Groves/Other Grove, 12 Improved Pastures, 13 Rangeland/Unimproved Pasture/Shrub

- Florida Department of Transportation (FDOT) roads and rights-of-way

- Water control districts (WCDs) and other Chapter 298 districts canals and rights-of-way

- Community development districts (CDDs)

- Municipalities

- Counties

After clipping each stakeholder area, the acres in the GIS shapefiles were recalculated and then the loads were recalculated using the following equations:

- Attenuated loads to the river and estuary:

  - TNLoadAtt = (GIS_Area* TN_UAL_Ave* TNAttenFac)+( TN_Septic* TNAttenFac)

  - TPLoadAtt = (GIS_Area* TP_UAL_Ave* TPAttenFac)+( TP_Septic* TPAttenFac)

- Unattenuated loads

  - TN_Load_Un = (GIS_Area* TN_UAL_Ave) + TN_Septic

  - TP_Load_Un = (GIS_Area* TP_UAL_Ave) + TP_Septic

These clipping steps are also be followed at the smaller scale of each of the impaired tributaries within the larger watershed: Townsend Canal, Long Hammock Creek, Lake Hicpochee, C-19 Canal, and S-4 Basin.

DEP used the results of the clipping to determine the starting load from each stakeholder and to calculate the required reductions to achieve the estuary and tributary total maximum daily loads (TMDLs).

Table 1. Field names and descriptions from the GIS base map shapefile

| Field Name | Description |
| --- | --- |
| OBJECTID | GIS identification number |
| NEXRAD | Next Generation Radar (NEXRAD) rainfall zone from the model |
| RAINGAGE | Rain gage data used in the model |
| FROM_S79 | Sub-basin upstream or downstream of S-79 |
| LU_Desc | Land use description |
| Subbasin | Sub-basin identification number |
| LU_Code | Land use code from the HSPF model |
| SubbasinLU | Combination of sub-basin and land use code numbers |
| GIS_Area | Polygon area in acres |
| TNAttenFac | TN attenuation factor based on the sub-basin and land use code combination |
| TPAttenFac | TP attenuation factor based on the sub-basin and land use code combination |
| P_LUFrac | Fraction of land use that is pervious |
| P_HSPFCode | Pervious land use code used to link with HSPF results |
| I_LUFrac | Fraction of land use that is impervious |
| I_HSPFCode | Impervious land use code used to link with HSPF results |
| P_TN_UAL | Pervious land TN average annual unit area load (lb/acre/year) |
| I_TN_UAL | Impervious land TN average annual unit area load (lb/acre/year) |
| P_TP_UAL | Pervious land TP average annual unit area load (lb/acre/year) |
| I_TP_UAL | Impervious land TP average annual unit area load (lb/acre/year) |
| TN_UAL_Ave | Pervious and impervious combined land TN average annual unit area load (lb/acre/year) |
| TP_UAL_Ave | Pervious and impervious combined TP average annual unit area load (lb/acre/year) |
| TN_Septic | TN septic system loads (lbs/yr) |
| TP_Septic | TP septic system loads (lbs/yr) |
| TNLoadAtt | Attenuated TN load to the river and estuary (lbs/yr) |
| TPLoadAtt | Attenuated TP load to the river and estuary (lbs/yr) |
| TNLoad_Un | Unattenuated TN load (lbs/yr) |
| TPLoad_Un | Unattenuated TP load (lbs/yr) |

*Download the zip file with your entity’s name and unzip it to view a shapefile of the area to be assigned to your entity in the allocations process.