Department recommend the following Buffer zone for using Average travel time. Also denitrification coefficient k=0.0011 /d can be used for NO3-NO2. Further detials will be provided in a separate email.

| Buffer zone | Travel Distance (m) | Average Velocity (m/day) | Average Travel Time (days) |
| --- | --- | --- | --- |
| 1 | <50 | 0.182 | 137.6 |
| 2 | 51-200 | 0.126 | 1385.7 |
| 3 | >200 | 0.060 | 9641.0 |

Some useful information can be used for this modeling:

Inflow mass was approximated as nitrogen release per person per day × people/household × 0.7 (the amount of nitrogen not lost in septic tanks). Instead of using national average, we can use the average total nitrogen (TN) load 11.2 g per person per day. Also, inflow mass is the source plane concentration (C0). It ranges between 0 mg/L and 80 mg/L (McCray et al., 1995), and the average of 40 mg/L can be used for this study.

Again this is for nitrogen only. For phosphorus and other pollutants you can use the typical literature value used for other Florida modeling (like the table you used).