9/7/16

So, the answer is basically “yes” BUT, DEP’s Assessment & TMDL team does not look at a single sample and say it’s a “violation” per se. It’s an evaluation over a longer period of time (7.5 years) using all the data from ambient samples from all the stations in a WBID if they are all monitoring the waterbody of focus. Find “Table 3: Verified List” in 62-303. It’s a table that provides a range of total number of samples and the number of samples that do not meet the criterion (410 or 130) which will trigger the “impaired” designation. Here’s a simplified example using part of table 3.

| Sample sizes |  | Are listed if they have at least this # of samples that do not meet a criterion |
| --- | --- | --- |
| From | To |  |
| 20 | 25 | 5 |
| 26 | 32 | 6 |
| 33 | 40 | 7 |
| 41 | 47 | 8 |
| 48 | 55 | 9 |

Scenario 1. If there are 22 ambient samples in a 3F WBID during the 7.5 year assessment period, and 4 are over the 410 cfu/100mL e-coli criterion, then the waterbody is not considered impaired for fecal indicator bacteria.

Scenario 2. If there are 22 ambient samples in a 3F WBID during the 7.5 year assessment period, and 5 are over the 410 e-coli criterion, then the waterbody is considered impaired for fecal indicator bacteria.

I hope this helps. 

You asked how to access the info about WBIDs so you can determine if a WBID will be assessed as fresh or marine.

Online access to the WBIDs and designations:

- Go to the Gallery of DEP featured Content maps.  http://fdep.maps.arcgis.com/home/gallery.html#c=organization&o=modified&page=2

- Find and open the map called NNC Tracker. http://ca.dep.state.fl.us/mapdirect/?focus=nutrientcriteria

- In the upper left corner, click the drop down arrows to show data layers.

- Click “More data”

- In the “Enter Key Word to Find Layers:” search box, type “WBID”

- Scroll down until you see the grouping called Watershed Assessment.

- Click “Waterbody IDs (WBIDs)” icon.

- It will tell you the WBIDs are added to the map. Close the “add more data to the map” pop-up.

- The WBID layer cannot draw correctly while the map is zoomed out. Don’t worry about this warning, WBIDs will be accessible once you zoom in.

- Click all the other layers to make them “not visible” so you can clearly select the WBIDs.

- Zoom to the area of interest like Wagner Creek/Seybold Canal. Click the map and the  WBID outline will appear. The info about the WBID will also appear on the right. It will tell you “This is a Class 3M ESTUARY”. Now you know DEP will assess this WBID using enterococci as the fecal indicator bacteria.

Please let me know if you have questions. 

Anita Nash 
O: 850-245-8545

C: 850-559-0436

Anita.nash@dep.state.fl.us

# From: Abdelrahman, Omar (RER) [mailto:AbdelO@miamidade.gov] 
Sent: Wednesday, September 07, 2016 2:08 PM
To: Nash, Anita <Anita.Nash@dep.state.fl.us>
Cc: Avila, Christian (RER) <AvilaC@miamidade.gov>; Muthiah P.E., Mallika (RER) <Mallika.Muthiah@miamidade.gov>
Subject: RE: Attached MRC Public Meetings

Ok thanks for the info. As for the highlighted section, we collect only 1 sample during a 30-day period, so does that mean any sample greater than 410 (130 for enterococci) is a violation, or does the State require the 10-sample minimum in order to apply the value?

Also, how can I access the WBID GIS Layer?

# From: Nash, Anita [mailto:Anita.Nash@dep.state.fl.us] 
Sent: Wednesday, September 07, 2016 1:13 PM
To: Abdelrahman, Omar (RER)
Subject: RE: Attached MRC Public Meetings

Right, and that’s the case for most of the state, so DEP typically assesses class 3 waterbodies using the highlighted part below. Please note the e-coli and entero criteria are different values. 

Do you know how to determine whether DEP considers a WBID to be fresh or marine? There is a designation in the WBID GIS layer, the class usually says either class 3F or 3M or another class like 1 or 2. If you want DEP to use your data for assessment purposes, be sure to use the class designation to determine whether to sample for enterococci, e-coli, or fecal coliform. 

Anita Nash 
O: 850-245-8545

C: 850-559-0436

Anita.nash@dep.state.fl.us

# From: Abdelrahman, Omar (RER) [mailto:AbdelO@miamidade.gov] 
Sent: Wednesday, September 07, 2016 11:43 AM
To: Nash, Anita <Anita.Nash@dep.state.fl.us>
Cc: Avila, Christian (RER) <AvilaC@miamidade.gov>
Subject: RE: Attached MRC Public Meetings

Thanks. How do we apply the following new rule: “MPN or MF counts shall not exceed a monthly geometric mean of 126 nor exceed the Ten Percent Threshold Value (TPTV) of 410 in 10% or more of the samples during any 30-day period.   Monthly  geometric means shall be based on a minimum of 10 samples taken over a 30-day period”, given that we collect only 1 sample at each site each month? Even if we pool samples by WBID, we will not meet the 10-sample minimum.

Regards,

Omar

# From: Nash, Anita [mailto:Anita.Nash@dep.state.fl.us] 
Sent: Wednesday, September 07, 2016 10:55 AM
To: Abdelrahman, Omar (RER); 'Brett Bibeau'
Cc: Muthiah P.E., Mallika (RER); Payne, Grover
Subject: RE: Attached MRC Public Meetings

Attached are the changed water quality standards, including the change in fecal indicator bacteria from fecal coliform to e-coli and enterococci. The DO changes are also included. I have copied Grover (Garry) who can talk with you, Omar, about the DO changes.

Anita Nash 
O: 850-245-8545

C: 850-559-0436

Anita.nash@dep.state.fl.us