# City of Cape Canaveral

Banana River Lagoon and North Indian River Lagoon BMAPs

Draft Allocations March 2020

# Summary:

- Used load estimation tool (LET) developed from SWIL model to calculate total nitrogen (TN) and total phosphorus (TP) starting loads for each TMDL “Segment Group”

- Applied percent reduction from TMDL for each Segment Group (see tables below) to calculate total TN and TP load reduction required for each Segment Group

- Used LET to calculate total anthropogenic starting load in each Segment Group and determine the percent each entity contributed to the total anthropogenic starting load.

- Multiplied the load reduction required in the Segment Group by the percent each entity contributed to the total anthropogenic starting load to calculate each entity’s required reductions within each Segment Group.

Note: Numbers have been rounded for display in this document but were not rounded during the full calculation process.

# Required Reductions Per TMDL Segment Group

| BMAP | Project Zone | TMDL Segment Group | % TN TMDL Total Reduction | % TP TMDL Total Reduction |
| --- | --- | --- | --- | --- |
| Banana River | Banana | BR1-2 | 67% | 72% |
| Banana River | Banana | BR3-5, BR7 | 59% | 64% |
| Banana River | Banana | BR6 | 66% | 70% |

# New Load Calculations from SWIL Model

| BMAP | Project Zone | TN (lbs/yr) |  |  | TP (lbs/yr) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Starting Load | Required Reductions (Adjusted) | Allocation | Starting Load | Required Reductions (Adjusted) | Allocation |
| Banana River | Banana | 6,843 | 4,946 | 1,896 | 1,073 | 811 | 262 |
| Total |  | 6,843 | 4,946 | 1,896 | 1,073 | 811 | 262 |

BR3-5, BR7

Note: City of Cape Canaveral has been denoted as “CCC” within shapefiles to distinguish project numbers between City of Cocoa and City of Cape Canaveral within the geodatabase.

# Draft Project Credit Recalculations

Using the LET and geographic information systems (GIS) tools, base loads for the project treatment areas were updated for all projects that require a base load for credit calculations. This allowed the project credits to be recalculated using the same foundation that was used for the updated allocations.

Current verified and completed projects:

Banana River Lagoon

| TN (lbs/yr) |  |  | TP (lbs/yr) |  |  |
| --- | --- | --- | --- | --- | --- |
| Required Reductions | Updated Project Credits | % Reductions Achieved | Required Reductions | Updated Project Credits | % Reductions Achieved |
| 4,946 | 5,875 | 119% | 1,073 | 300 | 28% |

These credits represent the City of Cape Canaveral’s status as of March 2020 – there will continue to be opportunities to submit new projects and obtain additional credits.

Notes:

  - There were large acreage differences between the shapefiles and project table acres for 10 projects: CC-02, CC-04, CC-06, CC-07, CC-09, CC-15, CC-19, CC-26, CC-29, CC-30

  - Project CC-18 and CC-20 are listed as a completed project, however we were unable to find a treatment area (TA) shapefile in our records. This project was left out of the reduction calculations but may be incorporated to include additional reduction credits once we have the TA.

Potential future reductions - [underway or not yet verified]:

| TN (lbs/yr) |  |  | TP (lbs/yr) |  |  |
| --- | --- | --- | --- | --- | --- |
| Required Reductions | Updated Project Credits | % Reductions Achieved | Required Reductions | Updated Project Credits | % Reductions Achieved |
| 4,946 | 7,041 | 142% | 1,073 | 537 | 50% |

  - Several projects (CC-08, CC-09, CC-18, CC-33 through CC-41) with load reductions listed as “TBD” can be calculated with the LET or other methods and added to total reductions once additional project information is received.