# Kennedy Space Center

Banana River Lagoon and North Indian River Lagoon BMAPs

Draft Allocations March 2020

# Summary:

- Used load estimation tool (LET) developed from SWIL model to calculate total nitrogen (TN) and total phosphorus (TP) starting loads for each TMDL “Segment Group”

- Applied percent reduction from TMDL for each Segment Group (see tables below) to calculate total TN and TP load reduction required for each Segment Group

- Used LET to calculate total anthropogenic starting load in each Segment Group and determine the percent each entity contributed to the total anthropogenic starting load.

- Multiplied the load reduction required in the Segment Group by the percent each entity contributed to the total anthropogenic starting load to calculate each entity’s required reductions within each Segment Group.

Note: Numbers have been rounded for display in this document but were not rounded during the full calculation process.

# Required Reductions Per TMDL Segment Group

| BMAP | Project Zone | TMDL Segment Group | % TN TMDL Total Reduction | % TP TMDL Total Reduction |
| --- | --- | --- | --- | --- |
| Banana River | Banana | BR1-2 | 67% | 72% |
| Banana River | Banana | BR3-5, BR7 | 59% | 64% |
| Banana River | Banana | BR6 | 66% | 70% |
| North IRL | A | IR1-5 | 35% | 47% |
| North IRL | B | IR6-7 | 36% | 53% |
| North IRL | B | IR8-11 | 36% | 48% |
| North IRL | B | EauGallie | 51% | 58% |

# New Load Calculations from SWIL Model

| BMAP | Project Zone | TN (lbs/yr) |  |  | TP (lbs/yr) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Starting Load | Required Reductions (Adjusted) | Allocation | Starting Load | Required Reductions (Adjusted) | Allocation |
| Banana River | Banana | 11,845 | 11,845 | 0 | 1,549 | 1,549 | 0 |
| Total (Adjusted) |  | 11,845 | 11,845 | 0 | 1,549 | 1,549 | 0 |

| BMAP | Project Zone | TN (lbs/yr) |  |  | TP (lbs/yr) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Starting Load | Required Reductions | Allocation | Starting Load | Required Reductions | Allocation |
| North IRL | A | 13,563 | 11,410 | 2,154 | 1,743 | 1,561 | 182 |
| North IRL | B | 1,761 | 883 | 878 | 228 | 154 | 74 |
| Total |  | 15,324 | 12,292 | 3,032 | 1,971 | 1,715 | 256 |

# Draft Project Credit Recalculations

Using the LET and geographic information systems (GIS) tools, base loads for the project treatment areas were updated for all projects that require a base load for credit calculations. This allowed the project credits to be recalculated using the same foundation that was used for the updated allocations.

Current verified and completed projects:

Banana River Lagoon Adjusted

| TN (lbs/yr) |  |  | TP (lbs/yr) |  |  |
| --- | --- | --- | --- | --- | --- |
| Required Reductions | Updated Project Credits | % Reductions Achieved | Required Reductions | Updated Project Credits | % Reductions Achieved |
| 11,845 | 17,495 | 148% | 1,549 | 1,832 | 118% |

North Indian River Lagoon Project Zone A

| TN (lbs/yr) |  |  | TP (lbs/yr) |  |  |
| --- | --- | --- | --- | --- | --- |
| Required Reductions | Updated Project Credits | % Reductions Achieved | Required Reductions | Updated Project Credits | % Reductions Achieved |
| 11,410 | 5,251 | 46% | 1,561 | 750 | 48% |

North Indian River Lagoon Project Zone B

| TN (lbs/yr) |  |  | TP (lbs/yr) |  |  |
| --- | --- | --- | --- | --- | --- |
| Required Reductions | Updated Project Credits | % Reductions Achieved | Required Reductions | Updated Project Credits | % Reductions Achieved |
| 883 | 375 | 42% | 154 | 74 | 48% |

These credits represent the Kennedy Space Center’s status as of March 2020 – there will continue to be opportunities to submit new projects and obtain additional credits.

Notes:

  - 3 treatment area shapefiles could not be located to update the project base loads: KSC-01 and KSC-11 in the NIRL and KSC-01 in the BRL. Once these are provided, the additional credits can be added to the total updated project credits.

  - There were large acreage differences between the shapefiles and project table acres for 7 projects: KSC-02, KSC-07, KSC-12, KSC-15, KSC-16 in the NIRL, and KSC-03, KSC-12 in the BRL