# City of Vero Beach

Central Indian River Lagoon BMAP Update

Draft Allocations March 2020

CIRL BMAP Project Zones and TMDL Segments

Summary:

- Used load estimation tool (LET) developed from SWIL model to calculate total nitrogen (TN) and total phosphorus (TP) starting loads for each TMDL “Segment Group”

- Applied percent reduction from TMDL for each Segment Group (see tables below) to calculate total TN and TP load reduction required for each Segment Group

- Used LET to calculate total anthropogenic starting load in each Segment Group and determine the percent each entity contributed to the total anthropogenic starting load.

- Multiplied the load reduction required in the Segment Group by the percent each entity contributed to the total anthropogenic starting load to calculate each entity’s required reductions within each Segment Group.

Note: Numbers have been rounded for display in this document but were not rounded during the full calculation process.

# Required Reductions per TMDL Segment Group

| BMAP | Project Zone | TMDL Segment Group | % TN TMDL Total Reduction | % TP TMDL Total Reduction |
| --- | --- | --- | --- | --- |
| Central IRL | A, B, SEB | IRL12-21 | 56% | 48% |
| Central IRL | SIRL | IR-SouthCentral | 36% | 58% |

# New Load Calculations from SWIL Model

| BMAP | Project Zone | TN (lbs/yr) |  |  | TP (lbs/yr) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Starting Load | Required Reductions | Allocation | Starting Load | Required Reductions | Allocation |
| Central IRL | B | 59,879 | 38,277 | 21,602 | 8,575 | 4,587 | 3,989 |
| Total Adjusted |  | 59,879 | 38,277 | 21,602 | 8,575 | 4,587 | 3,989 |

# Draft Project Credit Recalculations

Using the LET and geographic information systems (GIS) tools, base loads for the project treatment areas were updated for all projects that require a base load for credit calculations. This allowed the project credits to be recalculated using the same foundation that was used for the updated allocations.

Current verified and completed projects:

| TN (lbs/yr) |  |  | TP (lbs/yr) |  |  |
| --- | --- | --- | --- | --- | --- |
| Required Reductions | Updated Project Credits | % Reductions Achieved | Required Reductions | Updated Project Credits | % Reductions Achieved |
| 38,277 | 1,972 | 5% | 8,575 | 686 | 8% |

These credits represent the City’s status at the end of the BMAP’s first phase with the updated model – there will continue to be opportunity to submit new projects and obtain additional credits in the future phases of the BMAP.

Notes:

  - There were large acreage differences between the shapefiles and project table acres for 7 projects: VB-01, VB-03, VB-10, VB-11, VB-12, VB-13, VB-14

  - For VB-04, only 0.5% TN and TP reduction efficiency was applied for fertilizer ordinance; if there are additional components of an overall education/regulatory program, the efficiency would increase.

  - No project description was provided for many projects; project descriptions should be sent to DEP to be included in the updated BMAP.

Potential future reductions - [underway or not yet verified]:

| TN (lbs/yr) |  |  | TP (lbs/yr) |  |  |
| --- | --- | --- | --- | --- | --- |
| Required Reductions | Updated Project Credits | % Reductions Achieved | Required Reductions | Updated Project Credits | % Reductions Achieved |
| 38,277 | 4,218 | 11% | 8,575 | 686 | 8% |

Notes:

- Several projects (VB-07, VB-14, VB-16, VB-20) with load reductions listed as “TBD” can be calculated with the LET or other methods and added to total reductions once additional project information is received.