# St. Lucie County

- Updates to the Basin Management Action Plan (BMAP) were adopted February 2021 with an effective date of October 2021.

- Continued loss of the seagrass deep edge and updates to the loading model prompted the need to reevaluate stakeholder allocations to reduce nutrient loading in the Central Indian River Lagoon (CIRL).

  - Allocations are allowable loading targets mathematically calculated from the Total Maximum Daily Load (TMDL) based on each stakeholder’s jurisdictional area and anthropogenic sources based on land use.

  - Progress towards meeting the TMDL within the CIRL BMAP is measured by stakeholder projects implemented to reduce modeled starting loads for total nitrogen (TN) and total phosphorus (TP) to those allowable loading targets.

- An unusual mortality event (UME) for the Florida West Indian Manatee between 2020 and 2021 has put the Indian River Lagoon (IRL) under more scrutiny.

- A sense of fairness has stimulated stakeholders across the lagoon to call for assurances that all stakeholders participate towards achieving their allocations.

# BMAP Reductions

To date, St. Lucie County has submitted 14 projects to put towards tracking progress towards the allocated loading target. The project table is attached below. As of 2022, the County is approximately 17% and 13% of the way towards reducing the required model load reduction in meeting both the TN and TP allocation respectively.

Information required to verify creditable projects listed with a project status of completed:

| ProjID | ProjectNumber | ProjectName | ProjectType | Missing Information |
| --- | --- | --- | --- | --- |
| 2805 | SLC-05 | Taylor Creek Dredging | Muck Removal/Restoration Dredging | See guidance documentation for calculating nutrient reduction benefits. |
| 2804 | SLC-06 | Stan Blum Memorial Boat Launch | Wet Detention Pond | Treatment area shapefile if different from the one in record; detention time or permanent pool volume |

Information needed for creditable projects listed with a project status of Planned/Underway:

| ProjID | ProjectNumber | ProjectName | ProjectType | Missing Information |
| --- | --- | --- | --- | --- |
| 4382 | SLC-11 | Swales Material Collected | BMP Cleanout | MS4 tool. Permit number; additional retention achieved. |
| 6231 | SLC-14 | Harmony Heights Phase 3 | Dry Detention Pond | Treatment area shapefile. |
| 6234 | SLC-13 | Indrio Savannahs Wet Pond with Gabions | Hydrologic Restoration | Hydrologic review; design criteria; permit number. |

Possible projects to discuss:

- MS4 Permit - FLR04E029

  - Which areas of the county are still untreated?

  - Which areas of the county are only treated to basic permit requirements?

  - Could any of the first-generation baffle boxes be upgraded to second generation baffle boxes with media?

  - Ditch & Swale refurbish/retrofit/creation: Does the county already have a swale network? Would BAM, increased retention, swale blocks/raised culverts be feasible?

  - Are any of the privately owned COA/HOA's with ponds willing to partner for MAPs or Alum treatment to their ponds?

  - What actions are in place to encourage homeowners, particularly those along the canal and lagoon, to fertilize less, incorporate FL friendly landscaping techniques, and/or maintain riparian buffers?

    - 10 ft. wide no mow buffer strips along residential canals and seawalls/revetments?

    - Enforcement practices of fertilizer code/ordinance?

    - How is the relationship between the county and the IFAS extension agent regarding the Florida Friendly Yards and Neighborhoods Program?

  - Could any of the canceled projects be initiated if you had the resources? What would you need?

| Project ID | Project Status | Project Number | Project Name | Project Type |
| --- | --- | --- | --- | --- |
| 2802 | Canceled | SLC-08 | Georgia Ave. Basin WQ Improvement | Baffle Boxes- Second Generation |

- Septic to Sewer or OSTDS refurbish –

  - potentially 4,300 tanks with the jurisdictional boundary – are you already working to replace with advance treatment systems or connect these parcels to sewer?

    - The layer used to obtain these numbers is from a DOH study dated 2009 so we may not have as up to date information as your records.

- Wastewater –

  - Permitted Facility – FLA013945 – SLCU

    - Permitted Flow: 0.05 MGD

    - No reported discharge to the lagoon

    - Do you have a plan to reduce reuse TN and TP concentrations to meet the BMAP requirements upon permit renewal?

      - Discharge type sprayfield (SF) and slow rate restricted public access (RAA).

        - Annual average concentration for 1.958 mg/L NO3- for holding pond pulled from DMR between 2012 and 2022.

        - TN and TP are not currently analyzed in the DMR.

          - TN Limits: 10 mg/L TP Limits: 6 mg/L

      - Background well 0.322 mg/L NO3- between 2012 to 2022, biannual.

      - Compliance Wells:

        - MWC-1: 0.040 mg/L NO3- between 2012 to 2022, biannual.

        - MWC-2: 0.038 mg/L NO3- between 2012 to 2022, biannual.

        - MWC-3: 0.066 mg/L NO3- between 2012 to 2022, biannual.

  - Permitted Facility – FLA013969 – SLCU

    - Permitted Flow: 0.3 MGD

    - No reported discharge to the lagoon

    - Do you have a plan to reduce reuse TN and TP concentrations to meet the BMAP requirements upon permit renewal?

      - Annual average concentration for 1.958 mg/L NO3- for holding pond pulled from DMR between 2012 and 2022.

      - TN and TP are not currently analyzed in the DMR.

      - Discharge type rapid infiltration basin (RIB) and public access reuse (PAA).

        - PAA: 3.336 mg/L TN and 3.344 mg/L TP in 2022 quarterly.

          - Based on average flow rate from the same time, load estimates are 835 lb/yr TN and 837 lb/yr TP.

            - TN Limits: 10 mg/L TP Limits: 6 mg/L

        - RIBs: 3.832 mg/L TN and 3.758 mg/L TP between 2017 and 2022, quarterly.

          - Based on average flow rate from the same time, load estimates are 1,176 lb/yr TN and 1,153 lb/yr TP.

            - TN Limits: 6 mg/L TP Limits: 3 mg/L

      - Compliance Wells:

        - MWC-1:

          - 0.252 mg/L NO3- between 2013 to 2022, quarterly.

          - 3.553 mg/L TN between 2017 to 2022, quarterly.

          - 1.301 mg/L TP between 2017 to 2022, quarterly.

        - MWC-2:

          - 0.180 mg/L NO3- between 2013 to 2022, quarterly.

          - 4.745 mg/L TN between 2017 to 2022, quarterly.

          - 2.372 mg/L TP between 2017 to 2022, quarterly.

        - MWC-3:

          - 0.385 mg/L NO3- between 2013 to 2022, quarterly.

          - 4.280 mg/L TN between 2017 to 2022, quarterly.

          - 2.721 mg/L TP between 2017 to 2022, quarterly.

        - MWC-4:

          - 0.253 mg/L NO3- between 2013 to 2022, quarterly.

          - 4.316 mg/L TN between 2017 to 2022, quarterly.

          - 16.812 mg/L TP between 2017 to 2022, quarterly.

            - Was there a typo in December 2016? Should it be 0.441 mg/L instead of 441 mg/L?

        - MWC-5:

          - 0.120 mg/L NO3- between 2016 to 2022, quarterly.

          - 1.239 mg/L TN between 2017 to 2022, quarterly.

          - 0.373 mg/L TP between 2016 to 2022, quarterly.

  - Permitted Facility – FLA039586 – SLCU

    - Permitted Flow: 0.02 MGD

    - No reported discharge to the lagoon

    - Do you have a plan to reduce reuse TN and TP concentrations to meet the BMAP requirements upon permit renewal?

      - Annual average concentration for 2.102 mg/L NO3- for perc pond pulled from DMR between 2013 and 2022.

      - TN and TP are not currently analyzed in the DMR.

        - TN Limits: 10 mg/L TP Limits: 6 mg/L

      - Background well:

        - 0.474 mg/L NO3- between 2013 to 2022, quarterly.

        - 3.094 mg/L TN between 2013 to 2022, quarterly.

        - 0.335 mg/L TP between 2013 to 2022, quarterly

      - Compliance Well:

        - 0.775 mg/L NO3- between 2013 to 2022, quarterly.

        - 3.794 mg/L TN between 2013 to 2022, quarterly.

        - 1.780 mg/L TP between 2013 to 2022, quarterly.

  - Permitted Facility – FL0139475 – SLCUD

    - Permitted Flow: 1.6 MGD

    - No reported discharge to the lagoon, but does discharge to the Atlantic:

      - 9.837 mg/L TN and 2.393 mg/L TP between 2014 to 2022, monthly.

        - Based on average flow rate from the same time, load estimates are 962 lb/yr TN and 234 lb/yr TP.

    - Do you have a plan to reduce reuse TN and TP concentrations to meet the BMAP requirements upon permit renewal?

      - TN and TP are not currently analyzed in the DMR.

        - TN Limits: 10 mg/L TP Limits: 6 mg/L

      - Compliance Well:

        - MWC-01

          - 0.081 mg/L NO3- between 2013 to 2022, quarterly.

          - 1.138 mg/L TKN between 2013 to 2022, quarterly.

          - 1.219‬ mg/L TN between 2013 to 2022, quarterly.

          - 0.096 mg/L TP between 2013 to 2022, quarterly.

        - MWC-03

          - 0.027 mg/L NO3- between 2013 to 2022, quarterly.

          - 5.026 mg/L TKN between 2013 to 2022, quarterly.

          - 5.053‬‬ mg/L TN between 2013 to 2022, quarterly.

          - 0.345 mg/L TP between 2013 to 2022, quarterly.

        - MWC-04

          - 1.567mg/L NO3- between 2013 to 2022, quarterly.

          - 0.536 mg/L TKN between 2013 to 2022, quarterly.

          - 2.103‬ mg/L TN between 2013 to 2022, quarterly.

          - 0.072 mg/L TP between 2013 to 2022, quarterly.

        - MWC-05

          - 0.081 mg/L NO3- between 2013 to 2022, quarterly.

          - 15.220 mg/L TKN between 2013 to 2022, quarterly.

            - Was there a typo in June 2017? Should it be 0.594 mg/L instead of 594 mg/L?

          - 15.301‬‬ mg/L TN between 2013 to 2022, quarterly.

          - 0.070 mg/L TP between 2013 to 2022, quarterly.

Do you have any additional wish list projects that may have a benefit to water quality improvement?

Table 1. Allocation and Reduction Summary

| Entity | Project Zone | Model Starting Load TN (lb/yr) | Model Starting Load TP (lb/yr) | Allowable Load TN (lb/yr) | Allowable Load TP (lb/yr) | Required TN Reduction (lb/yr) | Required TP Reduction (lb/yr) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| St. Lucie County | SIRL | 104,021 | 16,773 | 54,241 | 4,809 | 49,780 | 11,964 |

Table 2. Water Quality at Closest BMAP Station

| Project Zone | Station | Estimated Target TN Concentration (mg/L) | Estimated Target TP Concentration (mg/L) | 5-yr Average TN Concentration (mg/L) | 5-yr Average TP Concentration (mg/L) | 15-yr TN Trend | 15-yr TP Trend |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SIRL | IRL39B | 0.72 | 0.07 | 0.216 | 0.055 | Insufficient Data | Insufficient Data |

Table 3. Project Table

| Proj ID | Partners | Project Number | Project Name | Project Description | Project Type | Project Status | Estimated Start Date | Estimated Completion Date | TN Reduction (lbs/yr) | TP Reduction (lbs/yr) | Crediting Location | Acres Treated | Cost Estimate | Cost Annual O&M | Funding Source | Funding Amount | DEP Contract Agreement Number | WMD Contract Agreement Number |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2638 | NA | SLC-01 | Education Efforts | FYN; landscaping, irrigation, fertilizer, and pet waste ordinances; PSAs, pamphlets, website, and illicit discharge program. | Education Efforts | Ongoing | Prior to 2013 | NA | 6241 | 1006 | SIRL | NA | $0.00 | $0.00 | NA | NA - $0.00 | NA | Not provided |
| 2808 | NA | SLC-02 | Street Sweeping | 470 tons/yr collected. | Street Sweeping | Ongoing | Prior to 2013 | NA | 664 | 299 | SIRL | NA | $150,000.00 | $50,000.00 | County | County - $50,000.00 | NA | Not provided |
| 2818 | DEP; SFWMD; IRL National Estuary Program (NEP) | SLC-03 | Paradise Park Stormwater Improvement | Construction of drainage system providing 75 % treatment of first 1-inch runoff. | Dry Detention Pond | Completed | 2007 | 2014 | 171 | 28 | SIRL | 168.1 | $10,000,000.00 | $0.00 | DEP; SFWMD; IRL NEP | DEP - $225,000; SFWMD - $304,448; IRL NEP - $125,000 | LP56020; NA; NA | Not provided; Not provided; Not provided |
| 2806 | NA | SLC-04 | Harmony Heights Stormwater Improvement | Construction of drainage system providing 75 % treatment of first 1-inch runoff. | Dry Detention Pond | Completed | 2009 | 2015 | 253 | 44 | SIRL | 238.743026 | $10,000,000.00 | $0.00 | TBD | TBD - $511,838.00 | NA | Not provided |
| 2805 | NA | SLC-05 | Taylor Creek Dredging | Three-phase sediment/muck removal project totaling approximately 200,000 cubic yards. | Muck Removal/Restoration Dredging | Completed | 2021 | 2021 | 0 | 0 | SIRL | 12.70 | $7,500,000.00 | $0.00 | FIND; DEP | FIND - $0.00; DEP - $3,573,390.00 | NA; NA | Not provided; Not provided |
| 2804 | NA | SLC-06 | Stan Blum Memorial Boat Launch | Not provided. | Wet Detention Pond | Completed | TBD | Prior to 2013 | 0 | 0 | SIRL | 6.129561 | $0.00 | $0.00 | Not provided | Not provided - $0.00 | NA | Not provided |
| 2803 | IRL NEP | SLC-07 | San Lucie Plaza Stormwater Master Plan | Construction of drainage system providing 75 % treatment of first 1-inch runoff. | On-line Retention BMPs | Completed | 2018 | 2018 | 1210 | 214 | SIRL | 157.080338 | $20,000,000.00 | $0.00 | TBD | TBD - $650,325.00 | NA | Not provided |
| 2802 | NA | SLC-08 | Georgia Ave. Basin WQ Improvement | Baffle box and control weir. | Baffle Boxes- Second Generation | Canceled | NA | NA | 0 | 0 | SIRL | NA | $0.00 | $0.00 | NA | NA - $0.00 | NA | Not provided |
| 2801 | NA | SLC-09 | North Hutchinson Island Septic to Sewer Project | 538 Homes converted to Central Sewer. | Wastewater Service Area Expansion | Underway | 2020 | 2024 | 0 | 0 | SIRL | NA | $1,846,465.00 | $0.00 | DEP | DEP - $990,000.00 | NA | NA |
| 2827 | NA | SLC-10 | Port of Fort Pierce | Stormwater management system. | Stormwater System Rehabilitation | Completed | TBD | 2018 | 0 | 0 | SIRL | TBD | $0.00 | $0.00 | County | County - $0.00 | NA | Not provided |
| 4382 | NA | SLC-11 | Swales Material Collected | Roadside swale cleanout and reprofiling. | BMP Cleanout | Underway | 2018 | TBD | 0 | 0 | SIRL | NA | $0.00 | $0.00 | TBD | TBD - $0.00 | NA | Not provided |
| 5238 | NA | SLC-12 | St. Lucie County Stormwater Needs Assessment Study | Report that will provide information on identified project opportunities to reduce nutrients, estimated benefits, and costs. | Study | Completed | 2020 | 2021 | 0 | 0 | SIRL | NA | $142,380.00 | $0.00 | TBD | TBD - $0.00 | NA | NA |
| 6234 | American Rescue Plan | SLC-13 | Indrio Savannahs Wet Pond with Gabions | Indrio Savannahs is a preserve that contains a natural flow-way that connects the northernmost lands in St. Lucie County to Taylor Creek. This site has some restrictions associated with state funding but an existing lake is currently bypassed and could be | Hydrologic Restoration | Planned | 2024 | 2026 | 0 | 0 | SIRL | 1,082.00 | $1,200,000.00 | $0.00 | American Rescue Plan | American Rescue Plan - $1,200,000.00 | NA | NA |
| 6231 | American Rescue Plan | SLC-14 | Harmony Heights Phase 3 | The goal of the project is to construct dry detention ponds and associated swales in the Harmony Heights subdivision, a 228-acre basin. Due to the age of the area, the subdivision currently lacks an overall stormwater management system. The project will p | Dry Detention Pond | Planned | 2024 | 2025 | 0 | 0 | SIRL | 228.00 | $0.00 | $0.00 | American Rescue Plan | American Rescue Plan - $3,600,000.00 | NA | NA |
| Totals for Projects Completed & Ongoing |  |  |  |  |  |  |  |  | 8,539 | 1,591 |  |  |  |  |  |  |  |  |

*Sums may differ from the Statewide Annual Report (STAR) 2022. STAR only reports on reductions from verified ongoing and completed projects while this table summarizes all projects with reduction calculations.