Banana River Lagoon Basin Management Action Plan

Allocations for Total Nitrogen and Total Phosphorus

--Revised for Two Project Zones--

April 2012

Background

The Banana River Lagoon (BRL) is now divided into two hydrologic project zones—BRL A and BRL B.  The BRL A project zone was determined to be compliant with the TMDL seagrass depth limit target; therefore, reductions are not required from the stakeholders in this project zone for the first iteration of the BMAP.  This paper outlines the process that FDEP applied to calculate allocations and presents the allocations for the BRL B project zone.

The stakeholders in the BRL B project zone are required to achieve 15% of the required reductions in the first five-year iteration of the BMAP.  Progress towards the seagrass depth limit targets will be evaluated to determine the necessary reductions in future BMAP iterations.  The allocation approach selected for the first iteration of the BMAP could be changed for future BMAP iterations, as needed.

Calculating Baseline Loads

Baseline loadings were generated from the Pollutant Load Screening Model (PLSM), which is a Geographic Information Systems (GIS) based model.  This model uses soil, rainfall, runoff data, and nutrient-land use relationships to generate total nitrogen (TN) and total phosphorus (TP) loadings for each basin land use. The baseline loadings were generated from year 2000 land use data.

To determine the BMAP allocations, the first step was to remove the areas associated with the land use land cover code 5400, the lagoon’s surface area, from the PLSM data file for the BRL.  The resulting file was then clipped to the BRL A and BRL B project zone boundaries.  For the BRL B project zone, the individual entity data files were created by clipping out each entity’s jurisdiction, as follows:

- Florida Department of Transportation (FDOT) roads and right-of-ways.

- Areas with agricultural land uses (2000 series of Florida land use codes).

- Municipalities and federal facilities, each to its own jurisdictional boundary.

- The remaining area was assigned to Brevard County.

Based on the clipped files, the PLSM output was used to determine the baseline loads (loads based on the 2000 land uses) for each entity.  The baseline load calculations were completed according to the procedures used by the St. Johns River Water Management District (SJRWMD).

The loading and acreages from the land use land cover code 5000 were removed from each entity’s baseline load and total acreages and set aside because this land use category is surface waters and is not considered an anthropogenic land use in the TMDL process.  FDEP predicts that internal loading in the surface waters will decrease with the flushing of cleaner watershed runoff once the pollutant loading from the watershed is controlled.

Tables 1 and 2 show the TN and TP loads, respectively, for the baseline loads, 5000 land use loads, and the net load (baseline loads minus 5000 land use loads) for each entity in BRL B.

Table 1.  Baseline and Net TN Loads for Each Entity in BRL B

| Entity | Area (Acres) | 5000 Land Use Acres | Net Acres | TN Baseline (lbs/yr) | TN 5000 Land Use Load (lbs/yr) | TN Net Load (lbs/yr) |
| --- | --- | --- | --- | --- | --- | --- |
| Agriculture | 126 | 0 | 126 | 702 | 0 | 702 |
| Brevard County | 9,988 | 1,401 | 8,587 | 72,540 | 6042 | 66,498 |
| Cape Canaveral | 857 | 1 | 857 | 8,976 | 1 | 8,975 |
| Cocoa Beach | 1,842 | 33 | 1,809 | 18,616 | 97 | 18,519 |
| FDOT District 5 | 518 | 1 | 517 | 5,161 | 8 | 5,153 |
| Indian Harbour Beach | 1,244 | 45 | 1,199 | 11,800 | 225 | 11,575 |
| Melbourne | 4 | 0 | 4 | 30 | 0 | 30 |
| Patrick Air Force Base (AFB) | 1,835 | 70 | 1,765 | 24,897 | 334 | 24,563 |
| Port Canaveral | 146 | 0 | 146 | 720 | 0 | 720 |
| Satellite Beach | 1,737 | 56 | 1,681 | 17,572 | 327 | 17,245 |
| Total | 18,299 | 1,607 | 16,692 | 161,014 | 7,034 | 153,980 |

Table 2.  Baseline and Net TP Loads for Each Entity in BRL B

| Entity | Area (Acres) | 5000 Land Use Acres | Net Acres | TP Baseline (lbs/yr) | TP 5000 Land Use Load (lbs/yr) | TP Net Load (lbs/yr) |
| --- | --- | --- | --- | --- | --- | --- |
| Agriculture | 126 | 0 | 126 | 174.5 | 0 | 174.5 |
| Brevard County | 9,988 | 1,401 | 8,587 | 14,525.1 | 565.3 | 13,959.8 |
| Cape Canaveral | 857 | 1 | 857 | 2,031.4 | 0.2 | 2,031.2 |
| Cocoa Beach | 1,842 | 33 | 1,809 | 3,845.7 | 22.2 | 3,823.5 |
| FDOT District 5 | 518 | 1 | 517 | 1,605.6 | 0.7 | 1,604.9 |
| Indian Harbour Beach | 1,244 | 45 | 1,199 | 2,143.3 | 22.1 | 2,121.2 |
| Melbourne | 4 | 0 | 4 | 5.5 | 0 | 5.5 |
| Patrick AFB | 1,835 | 70 | 1,765 | 6,508.6 | 41.4 | 6,467.2 |
| Port Canaveral | 146 | 0 | 146 | 132.3 | 0 | 132.3 |
| Satellite Beach | 1,737 | 56 | 1,681 | 3,342.6 | 29.1 | 3,313.5 |
| Total | 18,299 | 1,607 | 16,692 | 34,314.6 | 681 | 33,633.6 |

De Minimus Calculations

The net loads from Tables 1 and 2 were then sorted for TN and TP loads, from highest to lowest, to determine whether any entity had loads low enough that reductions from these areas would have essentially no significant impact on the required reductions in the first phase of the BMAP.  Tables 3 and 4 present the sorted data and the shaded cells signify the de minimus load contributions.

Table 3.  BRL B Entity Baseline TN Loads Sorted from Highest to Lowest

Note: de minimus stakeholders are marked with a “*.”

| Entity | TN Net Load (lbs/yr) | % of Total Net Load |
| --- | --- | --- |
| Brevard County | 66,498 | 43.2% |
| Patrick AFB | 24,563 | 16.0% |
| Cocoa Beach | 18,519 | 12.0% |
| Satellite Beach | 17,245 | 11.2% |
| Indian Harbour Beach | 11,575 | 7.5% |
| Cape Canaveral | 8,975 | 5.8% |
| FDOT District 5 | 5,153 | 3.3% |
| Port Canaveral* | 720 | 0.5% |
| Agriculture* | 702 | 0.5% |
| Melbourne* | 30 | 0.0% |

Table 4.  BRL B Entity Baseline TP Loads Sorted from Highest to Lowest

Note: de minimus stakeholders are marked with a “*.”

| Entity | TP Net Load (lbs/yr) | % of Total Net Load |
| --- | --- | --- |
| Brevard County | 13,959.8 | 41.5% |
| Patrick AFB | 6,467.2 | 19.2% |
| Cocoa Beach | 3,823.5 | 11.4% |
| Satellite Beach | 3,313.5 | 9.9% |
| Indian Harbour Beach | 2,121.2 | 6.3% |
| Cape Canaveral | 2,031.2 | 6.0% |
| FDOT District 5 | 1,604.9 | 4.8% |
| Agriculture* | 174.5 | 0.5% |
| Port Canaveral* | 132.3 | 0.4% |
| Melbourne* | 5.5 | 0.0% |

Three entities each contributed less than 1% of the total load: agriculture, Port Canaveral, and Melbourne.  Combined they contribute approximately 1% of the total load and they are considered to be de minimus.  Additionally, most acreage identified as agriculture in 2000 is now fallow or converted to urban land uses.  These entities will not be assigned an allocation for either TN or TP for the first phase of the BMAP.  This de minimus designation does not preclude these entities from implementing reductions in the near term future, or from being assigned allocations in future BMAP iterations.

Calculating the Target Loads per Acre and Allocations

The target loads per acre were calculated by dividing the TMDL target load by the total non-water area in the BRL B project zone, as shown in Table 5.

Table 5.  Calculated Target Loads per Acre for TN and TP in BRL B

| Category | TN | TP |
| --- | --- | --- |
| TMDL Target Load (lbs/yr) | 63,028 | 11,823 |
| Non-water acres | 16,692 | 16,692 |
| Target Load (lbs/acre/yr) | 3.78 | 0.71 |

The target load per acre is used to calculate the allowable loadings of TN and TP for each entity.  It is the loading allocation assigned to each entity.  The amount of loading that each entity must reduce is the difference between the allocated loading and the net loading.

BMAP Phase 1 Allocations

Using the target loads discussed above, the allocations for Phase 1 of the BMAP for the BRL B project zone are shown in Tables 6 and 7.

Table 6. BRL B BMAP Phase 1 Required TN Reductions

| Entity | Area (Acres) | TN Target (lbs/acre/yr) | TN Target (lbs/yr) | TN Net Load (lbs/yr) | TN Required Reduction (lbs/yr) | 15% BMAP 1 TN Required Reduction (lbs/yr) |
| --- | --- | --- | --- | --- | --- | --- |
| Brevard County | 8,587 | 3.78 | 32,460 | 66,498 | 34,038 | 5,105.7 |
| Cape Canaveral | 857 | 3.78 | 3,239 | 8,975 | 5,736 | 860.4 |
| Cocoa Beach | 1,809 | 3.78 | 6,839 | 18,519 | 11,680 | 1,752.0 |
| FDOT District 5 | 517 | 3.78 | 1,955 | 5,153 | 3,198 | 479.7 |
| Indian Harbour Beach | 1,199 | 3.78 | 4,531 | 11,575 | 7,044 | 1,056.6 |
| Patrick AFB | 1,765 | 3.78 | 6,673 | 24,563 | 17,890 | 2,683.5 |
| Satellite Beach | 1,681 | 3.78 | 6,354 | 17,245 | 10,891 | 1,633.7 |
| Agriculture - de minimus | 126 | 3.78 | - | 702 | - | - |
| Melbourne - de minimus | 4 | 3.78 | - | 30 | - | - |
| Port Canaveral - de minimus | 146 | 3.78 | - | 720 | - | - |
| Total | 16,692 | N/A | 62,051 | 153,980 | 90,477 | 13,571.6 |

Table 7. BRL B BMAP Phase 1 Required TP Reductions

| Entity | Area (Acres) | TP Target (lbs/acre/yr) | TP Target (lbs/yr) | TP Net Load (lbs/yr) | TP Required Reduction (lbs/yr) | 15% BMAP 1 TP Required Reduction (lbs/yr) |
| --- | --- | --- | --- | --- | --- | --- |
| Brevard County | 8,587 | 0.71 | 6,097.0 | 13,959.8 | 7,862.8 | 1,179.4 |
| Cape Canaveral | 857 | 0.71 | 608.0 | 2,031.2 | 1,423.2 | 213.5 |
| Cocoa Beach | 1,809 | 0.71 | 1,285.0 | 3,823.5 | 2,538.5 | 380.8 |
| FDOT District 5 | 517 | 0.71 | 367.0 | 1,604.9 | 1,237.9 | 185.7 |
| Indian Harbour Beach | 1,199 | 0.71 | 851.0 | 2,121.2 | 1,270.2 | 190.5 |
| Patrick AFB | 1,765 | 0.71 | 1,253.0 | 6,467.2 | 5,214.2 | 782.1 |
| Satellite Beach | 1,681 | 0.71 | 1,194.0 | 3,313.5 | 2,119.5 | 317.9 |
| Agriculture - de minimus | 126 | 0.71 | - | 174.5 | - | - |
| Melbourne - de minimus | 4 | 0.71 | - | 5.5 | - | - |
| Port Canaveral - de minimus | 146 | 0.71 | - | 132.3 | - | - |
| Total | 16,692 | N/A | 11,655.0 | 33,633.6 | 21,666.3 | 3,249.9 |

Alternate Target Load per Acre Without Natural Areas Allocations

The allocations for the stakeholders in the BRL B project zone were also calculated based on a target load per acre using only the anthropogenic loading and those associated acres.  The anthropogenic load is represented by land use codes 1000, 2000, 7000, and 8000.

Calculating the Baseline Loads

The same steps as listed above were followed to clip each entity’s jurisdiction to the PLSM output for BRL B.  Table 8 shows the TN and TP baseline loads for each entity’s anthropogenic land uses.

Table 8. Baseline TN and TP Anthropogenic Loads for Each Entity in BRL B

| Entity | Area (Acres) | TN Anthropogenic Baseline Load (lbs/yr) | TP Anthropogenic Baseline Load (lbs/yr) |
| --- | --- | --- | --- |
| Agriculture | 126 | 702 | 174.5 |
| Brevard County | 5,393 | 64,236 | 13,774.0 |
| Cape Canaveral | 610 | 8,393 | 1,987.4 |
| Cocoa Beach | 1,709 | 18,300 | 3,811.5 |
| FDOT District 5 | 474 | 5,076 | 1,598.3 |
| Indian Harbour Beach | 1,101 | 11,266 | 2,101.2 |
| Melbourne | 4 | 30 | 5.5 |
| Patrick AFB | 1,729 | 24,468 | 6,459.1 |
| Port Canaveral | 53 | 584 | 124.2 |
| Satellite Beach | 1,525 | 16,692 | 3,283.9 |
| Total | 12,724 | 149,747 | 33,319.6 |

De minimus Entities

The loads in Table 8 were sorted from highest to lowest to identify the de minimus stakeholders in BRL B.  The same three entities each contributed approximately 1% of the total load: agriculture, Port Canaveral, and Melbourne.  These entities will not be assigned an allocation for either TN or TP for the first phase of the BMAP.

Calculating the Target Loads per Acre and Allocations

The target loads per acre were calculated by dividing the TMDL anthropogenic target load by the total anthropogenic area in the BRL B project zone.  The anthropogenic TN and TP target loads were calculated by subtracting the load associated with the natural lands from the TMDL target loading, as shown in Table 9.

Table 9.  Anthropogenic Target Loads per Acre for TN and TP in BRL B

| BRL B | TN | TP |
| --- | --- | --- |
| TMDL Target (lbs/yr) | 63,028 | 11,823 |
| Natural Loading (lbs/yr) | 11,267 | 995 |
| Anthropogenic Target Loads (lbs/yr) | 51,761 | 10,828 |
| Anthropogenic Acres | 12,724 | 12,724 |
| Target Load (lbs/acre/yr) | 4.07 | 0.85 |

BMAP Phase 1 Allocations

Using the target loads discussed above, the allocations under the alternate approach for Phase 1 of the BMAP for the BRL B project zone are shown in Tables 10 and 11.

Table 10. BRL B Anthropogenic BMAP Phase 1 Required TN Reductions

| Entity | Area (Acres) | TN Target (lbs/acre/yr) | TN Allocation (lbs/yr) | TN Anthropogenic Baseline Load (lbs/yr) | TN Required Reduction (lbs/yr) | 15% of TN Required Reduction (lbs/yr) |
| --- | --- | --- | --- | --- | --- | --- |
| Brevard County | 5,393 | 4.07 | 21,949 | 64,236 | 42,287 | 6,343.1 |
| Cape Canaveral | 610 | 4.07 | 2,484 | 8,393 | 5,909 | 886.4 |
| Cocoa Beach | 1,709 | 4.07 | 6,954 | 18,300 | 11,346 | 1,701.9 |
| FDOT District 5 | 474 | 4.07 | 1,930 | 5,076 | 3,146 | 471.9 |
| Indian Harbour Beach | 1,101 | 4.07 | 4,482 | 11,266 | 6,784 | 1,017.6 |
| Patrick AFB | 1,729 | 4.07 | 7,037 | 24,468 | 17,431 | 2,614.7 |
| Satellite Beach | 1,525 | 4.07 | 6,206 | 16,692 | 10,486 | 1,572.9 |
| Agriculture – de minimus | 126 | 4.07 | - | 702 | - | - |
| Melbourne – de minimus | 4 | 4.07 | - | 30 | - | - |
| Port Canaveral – de minimus | 53 | 4.07 | - | 584 | - | - |
| Total | 12,724 | N/A | 51,042 | 149,747 | 97,389 | 14,608.4 |

Table 11. BRL B Anthropogenic BMAP Phase 1 Required TP Reductions

| Entity | Area (Acres) | TP Target (lbs/acre/yr) | TP Allocation (lbs/yr) | TP Anthropogenic Baseline Load (lbs/yr) | TP Required Reduction (lbs/yr) | 15% of TP Required Reduction (lbs/yr) |
| --- | --- | --- | --- | --- | --- | --- |
| Brevard County | 5,393 | 0.85 | 4,584.0 | 13,774.0 | 9,190.0 | 1,378.5 |
| Cape Canaveral | 610 | 0.85 | 519.0 | 1,987.4 | 1,468.4 | 220.3 |
| Cocoa Beach | 1,709 | 0.85 | 1,452.0 | 3,811.5 | 2,359.5 | 353.9 |
| FDOT District 5 | 474 | 0.85 | 403.0 | 1,598.3 | 1,195.3 | 179.3 |
| Indian Harbour Beach | 1,101 | 0.85 | 936.0 | 2,101.2 | 1,165.2 | 174.8 |
| Patrick AFB | 1,729 | 0.85 | 1,470.0 | 6,459.1 | 4,989.1 | 748.4 |
| Satellite Beach | 1,525 | 0.85 | 1,296.0 | 3,283.9 | 1,987.9 | 298.2 |
| Agriculture – de minimus | 126 | 0.85 | - | 174.5 | - | - |
| Melbourne – de minimus | 4 | 0.85 | - | 5.5 | - | - |
| Port Canaveral – de minimus | 53 | 0.85 | - | 124.2 | - | - |
| Total | 12,724 | N/A | 10,660.0 | 33,319.6 | 22,355.4 | 3,353.3 |

Comparison of the Allocations

The overall reductions required in the target load per acre without natural areas approach are higher than the target load per acre approach.  Therefore, the target load per acre approach (which includes natural areas) will be used in the first BMAP iteration for BRL B.  In future BMAP iterations, a different allocation approach could be used, if appropriate.  A comparison of the reductions required in each approach is shown in Table 12 and 13 for TN and TP, respectively.

Table 12. Comparison of BRL B BMAP Phase 1 Required TN Reductions for Each Approach

| Entity | Entire BRL Basin Target Load Per Acre Approach Reductions (lbs/yr) | Target Load per Acre Approach Reductions (lbs/yr) | Target Load per Acre without Natural Lands Approach Reductions (lbs/yr) |
| --- | --- | --- | --- |
| Brevard County | 7,017.5 | 5,105.7 | 6,343.1 |
| Cape Canaveral | 1,078.7 | 860.4 | 886.4 |
| Cocoa Beach | 2,202.8 | 1,752.0 | 1,701.9 |
| FDOT District 5 | 644.1 | 479.7 | 471.9 |
| Indian Harbour Beach | 1,348.1 | 1,056.6 | 1,017.6 |
| Patrick AFB | 3,111.6 | 2,683.5 | 2,614.7 |
| Satellite Beach | 2,044.7 | 1,633.7 | 1,572.9 |
| Agriculture – de minimus | - | - | - |
| Melbourne – de minimus | - | - | - |
| Port Canaveral – de minimus | - | - | - |
| Total | 17,447.5 | 13,571.6 | 14,608.4 |

Table 13. Comparison of BRL B BMAP Phase 1 Required TP Reductions for Each Approach

| Entity | Entire BRL Basin Target Load Per Acre Approach Reductions (lbs/yr) | Target Load per Acre Approach Reductions (lbs/yr) | Target Load per Acre without Natural Lands Approach Reductions (lbs/yr) |
| --- | --- | --- | --- |
| Brevard County | 1,595.0 | 1,179.4 | 1,378.5 |
| Cape Canaveral | 260.4 | 213.5 | 220.3 |
| Cocoa Beach | 478.4 | 380.8 | 353.9 |
| FDOT District 5 | 226.8 | 185.7 | 179.3 |
| Indian Harbour Beach | 254.0 | 190.5 | 174.8 |
| Patrick AFB | 875.3 | 782.1 | 748.4 |
| Satellite Beach | 407.3 | 317.9 | 298.2 |
| Agriculture – de minimus | - | - | - |
| Melbourne – de minimus | - | - | - |
| Port Canaveral – de minimus | - | - | - |
| Total | 4,097.2 | 3,249.9 | 3,353.3 |