Banana River Lagoon

Basin Management Action Plan

June 2010

Allocation Calculations for Total Nitrogen (TN) and Total Phosphorus (TP)

Background

In establishing the detailed allocations for the Banana River Lagoon (BRL) Basin Management Action Plan (BMAP) and the reductions required by the total maximum daily load (TMDL) for total nitrogen (TN) and total phosphorus (TP), the Florida Department of Environmental Protection (FDEP) considered the best approach to allocate to the appropriate stakeholders in a manner that is reasonable and equitable.  Section 403.067(7)(a)(1), Florida Statutes (F.S.), includes the following guidance for preparing BMAPs:

In developing and implementing the total maximum daily load for a water body, the department, or the department in conjunction with a water management district, may develop a basin management action plan that addresses some or all of the watersheds and basins tributary to the water body. Such a plan must integrate the appropriate management strategies available to the state through existing water quality protection programs to achieve the total maximum daily loads and may provide for phased implementation of these management strategies to promote timely, cost-effective actions as provided for in s. 403.151. The plan must establish a schedule for implementing the management strategies, establish a basis for evaluating the plan's effectiveness, and identify feasible funding strategies for implementing the plan's management strategies.

In addition, Section 403.067(6)(b) provides the following guidance for allocations:

Allocation of total maximum daily loads. The total maximum daily loads shall include establishment of reasonable and equitable allocations of the total maximum daily load between or among point and nonpoint sources that will alone, or in conjunction with other management and restoration activities, provide for the attainment of the pollutant reductions established pursuant to paragraph (a) to achieve water quality standards for the pollutant causing impairment. The allocations may establish the maximum amount of the water pollutant that may be discharged or released into the water body or water body segment in combination with other discharges or releases. Allocations may also be made to individual basins and sources or as a whole to all basins and sources or categories of sources of inflow to the water body or water body segments. An initial allocation of allowable pollutant loads among point and nonpoint sources may be developed as part of the total maximum daily load. However, in such cases, the detailed allocation to specific point sources and specific categories of nonpoint sources shall be established in the basin management action plan pursuant to subsection (7). The initial and detailed allocations shall be designed to attain the pollutant reductions established pursuant to paragraph (a) and shall be based on consideration of the following…

This paper outlines the process and considerations that FDEP used in calculating the allocations for the BRL BMAP.

Calculating Baseline Loads

To determine the allocations for the BRL BMAP, FDEP clipped and erased the Pollutant Loading Screening Model (PLSM) Geographic Information System (GIS) shapefile according to the procedure outlined in the document Detailed Allocation Procedure for the Banana River Lagoon, with one exception.  The zero event mean concentration (EMC) acres were included, except the land use land cover code 5400 (used to identify the BRL surface area), in the total allocated acres because this is the same way the TMDL was calculated.  Based on the clipped files, the PLSM used to establish the TMDL was run to determine the baseline loads (loads based on the 2000 land uses in the model) for each entity.  The model was run according to the procedures used by the St. Johns River Water Management District (SJRWMD) to calculate the baseline loads.  The calculations for TN and TP total loads agree with the adopted baseline loads.  The baseline loads for each entity and total loads compared with the current loads adopted in the TMDL Report are presented in Table 1.

Table 1.  Baseline TN and TP Loads for Each Entity in BRL

| Entity | Area (acres) | TN (lbs) | TN (lbs/acre) | TP (lbs) | TP (lbs/acre) |
| --- | --- | --- | --- | --- | --- |
| Agriculture | 231 | 1,363 | 5.90 | 340 | 1.472 |
| Brevard County | 10,470 | 75,489 | 7.21 | 14,741 | 1.408 |
| Cape Canaveral | 856 | 8,945 | 10.45 | 1,957 | 2.287 |
| Cape Canaveral Air Force Station (AFS) | 13,795 | 53,007 | 3.84 | 10,790 | 0.782 |
| Cocoa Beach | 1,857 | 18,759 | 10.10 | 3,781 | 2.036 |
| Florida Department of Transportation District 5 (FDOT 5) | 386 | 3,741 | 9.69 | 1,076 | 2.788 |
| Indian Harbour Beach | 1,251 | 11,908 | 9.52 | 2,092 | 1.673 |
| Kennedy Space Center | 18,540 | 70,816 | 3.82 | 8,576 | 0.463 |
| Melbourne | 4 | 29 | 8.23 | 5 | 1.533 |
| Patrick Air Force Base (AFB) | 2,134 | 28,994 | 13.59 | 7,500 | 3.515 |
| Port Canaveral | 344 | 1,474 | 4.29 | 199 | 0.580 |
| Satellite Beach | 1,488 | 13,952 | 9.38 | 2,462 | 1.655 |
| U.S. Army Corps of Engineers (USACE) | 30 | 167 | 5.49 | 31 | 1.655 |
| Total | 51,385 | 288,644 | 5.62 | 53,552.1 | 1.042 |
| TMDL Baseline (2000) Loads | - | 289,117 | - | 54,981.0 | - |
| % Difference | - | 0.0% | - | -0.7% | - |

Note:  TMDL loads taken from the Indian River Lagoon (IRL) TMDL Table 4.9, which does not include atmospheric deposition or point sources.

De Minimus Calculations

The data from Table 1 were then sorted for TN and TP loads, from highest to lowest, to determine whether any entity had loads low enough that reductions from these areas would have essentially no significant impact on the required reductions in the first phase of the BMAP.  Tables 2 and 3 present the sorted data.

Table 2.  Entity Baseline TN Loads Sorted from Highest to Lowest

| Entity | TN (lbs) | % of Total Load | Cumulative Load (lbs) | Cumulative % of Total Load |
| --- | --- | --- | --- | --- |
| Brevard County | 75,489 | 26.15% | 75,489 | 26.15% |
| Kennedy Space Center | 70,816 | 24.53% | 146,305 | 50.69% |
| Cape Canaveral AFS | 53,007 | 18.36% | 199,312 | 69.05% |
| Patrick AFB | 28,994 | 10.04% | 228,306 | 79.10% |
| Cocoa Beach | 18,759 | 6.50% | 247,065 | 85.60% |
| Satellite Beach | 13,952 | 4.83% | 261,017 | 90.43% |
| Indian Harbour Beach | 11,908 | 4.13% | 272,925 | 94.55% |
| Cape Canaveral | 8,945 | 3.10% | 281,870 | 97.65% |
| FDOT 5 | 3,741 | 1.30% | 285,611 | 98.95% |
| Port Canaveral | 1,474 | 0.51% | 287,085 | 99.46% |
| Agriculture | 1,363 | 0.47% | 288,448 | 99.93% |
| USACE | 167 | 0.06% | 288,615 | 99.99% |
| Melbourne | 29 | 0.01% | 288,644 | 100.0% |

Note:  Highlighted entities signify de minimus load contributions

Table 3.  Entity Baseline TP Loads Sorted from Highest to Lowest

| Entity | TP (lbs) | % of Total Load | Cumulative Load (lbs) | Cumulative % of Total Load |
| --- | --- | --- | --- | --- |
| Brevard County | 14,741 | 27.53% | 14,741 | 27.53% |
| Cape Canaveral AFS | 10,790 | 20.15% | 25,531 | 47.68% |
| Kennedy Space Center | 8,576 | 16.02% | 34,107 | 63.69% |
| Patrick AFB | 7,500 | 14.01% | 41,608 | 77.70% |
| Cocoa Beach | 3,781 | 7.06% | 45,389 | 84.76% |
| Satellite Beach | 2,462 | 4.60% | 47,851 | 89.35% |
| Indian Harbour Beach | 2,092 | 3.91% | 49,943 | 93.26% |
| Cape Canaveral | 1,957 | 3.65% | 51,900 | 96.92% |
| FDOT 5 | 1,076 | 2.01% | 52,976 | 98.92% |
| Agriculture | 340 | 0.64% | 53,317 | 99.56% |
| Port Canaveral | 193 | 0.37% | 53,516 | 99.93% |
| USACE | 31 | 0.06% | 53,547 | 99.99% |
| Melbourne | 5 | 0.01% | 53,552 | 100.0% |

Note:  Highlighted entities signify de minimus load contributions

Four entities each contributed less than 1% of the total load: Agriculture, Port Canaveral, USACE, and the City of Melbourne.  These entities are thus considered to be de minimus.  Combined they contribute approximately 1% of the total load and therefore will not be assigned an allocation for either TN or TP for the first phase of the BMAP.  It is important to note that the de minimus status is only for the first BMAP iteration and will be reviewed with each BMAP cycle.  In future phases of the BMAP, TN and TP reductions may be needed from the de minimus entities; therefore, although they do not currently have a reduction responsibility, this does not prevent these entities from having reduction requirements in future BMAPs.  Any actions taken by these entities during this first phase of the BMAP that result in TN or TP reductions should be documented by those entities for credit against any reduction requirements allocated in subsequent BMAP iterations.

BMAP Considerations

The BRL BMAP will be phased, meaning that 33% of the allocated reductions will need to be met in the first five years of implementation.  However, in the second phase of the BMAP, the TMDL will be updated with more current land use and water quality data.  These revisions may result in a change to required reductions in future BMAP iterations.  In addition, stakeholders have expressed concern over uncertainties they feel exist in the TMDL, such as the relationship between TN and TP reductions and the seagrass target, salinity and algae influences on the system, and basin-specific data for model calibration.  Recognizing these issues, FDEP is opting to use the flexibility available in the BMAP process to set allocations for Phase 1 of the BMAP by selecting an interim target for TN (as described below).  This phased approach helps to ensure that FDEP does not ask the stakeholders to reduce more than is necessary by using adaptive management principles to evaluate progress towards the TMDL seagrass target.  It must be noted that the TMDL requirements have not changed; however, for the purposes of the first phase of this BMAP, FDEP is selecting a higher interim target while certain issues are further examined.  Additional reductions may be required in subsequent iterations of the BMAP to meet the TMDL target.

Calculating the Target Loads per Acre

The target loads per acre were calculated by dividing the TMDL target load by the total non-lagoon area in the BRL system.  A large portion of the basin is comprised of water, and in the model most of those areas contribute no load.

Table 4.  Calculated Target Loads per Acre for TN and TP

|  | TN | TP |
| --- | --- | --- |
| Non-lagoon acres (from Table 1) | 51,385 | 51,385 |
| TMDL Target Load (lbs) | 104,642 | 17,697 |
| Target Load (lbs/acre) | 2.04 | 0.344 |

Note:  Target loads summed from TMDL Table 6.3a are for the WBIDs 3044A, 3057A and 3057B, and 3057C and do not include atmospheric deposition or point sources.

The target nonpoint source loads are thus 2.04 lbs/acre and 0.344 lbs/acre for TN and TP, respectively.  This target differs from the TMDL target of 2.18 lbs/acre for TN and 0.374 lbs/acre for TP because the TMDL includes atmospheric deposition and point sources.  The point sources were already assigned allocations in the TMDL and their permits.  Subtracting atmospheric deposition and point source loads from the allowable loading results in the calculated target loads per acre shown in Table 4.

Allocations

A preliminary calculation of the required load reductions for each entity, based on the targets in Table 4, is included in Tables 5 and 6.

Table 5.  Preliminary TN Load Allocations by Entity

| Entity | Area (acres) | TN Target (lbs/acre) | TN Target (lbs) | Base Load (lbs) | Required Reduction (lbs) | Required Reduction (%) | Resulting Load (lbs) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Brevard County | 10,470 | 2.04 | 21,322 | 75,489 | 54,167 | 71.8% | 21,322 |
| Cape Canaveral | 856 | 2.04 | 1,742 | 8,945 | 7,203 | 80.5% | 1,742 |
| Cape Canaveral AFS | 13,795 | 2.04 | 28,092 | 53,007 | 25,915 | 47.0% | 28,092 |
| Cocoa Beach | 1,857 | 2.04 | 3,781 | 18,759 | 14,978 | 79.8% | 3,781 |
| FDOT 5 | 386 | 2.04 | 786 | 3,741 | 2,955 | 79.0% | 786 |
| Indian Harbour Beach | 1,251 | 2.04 | 2,547 | 11,908 | 9,361 | 78.6% | 2,547 |
| Kennedy Space Center | 18,540 | 2.04 | 37,756 | 70,816 | 33,060 | 46.7% | 37,756 |
| Patrick AFB | 2,134 | 2.04 | 4,345 | 28,994 | 24,649 | 85.0% | 4,345 |
| Satellite Beach | 1,488 | 2.04 | 3,030 | 13,952 | 10,922 | 78.3% | 3,030 |
| de minimus | 609 | - |  | 3,033 | - | 0.0% | 3,033 |
| Total | 51,385 | - | 103,402 | 288,644 | 182,209 | 63.1% | 106,435 |

Note:  The resulting load is higher than the target load because the de minimus entities are not required to reduce loads in the first iteration of the BMAP.

Table 6.  Preliminary TP Load Allocations by Entity

| Entity | Area (acres) | TP Target (lbs/acre) | TP Target (lbs) | Base Load (lbs) | Required Reduction (lbs) | Required Reduction (%) | Resulting Load (lbs) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Brevard County | 10,470 | 0.344 | 3,606 | 14,741 | 11,135 | 75.5% | 3,606 |
| Cape Canaveral | 856 | 0.344 | 295 | 1,957 | 1,663 | 84.9% | 295 |
| Cape Canaveral AFS | 13,795 | 0.344 | 4,751 | 10,790 | 6,039 | 56.0% | 4,751 |
| Cocoa Beach | 1,857 | 0.344 | 640 | 3,781 | 3,141 | 83.1% | 640 |
| FDOT 5 | 386 | 0.344 | 133 | 1,076 | 943 | 87.6% | 133 |
| Indian Harbour Beach | 1,251 | 0.344 | 431 | 2,092 | 1,662 | 79.4% | 431 |
| Kennedy Space Center | 18,540 | 0.344 | 6,385 | 8,576 | 2,191 | 25.5% | 6,385 |
| Patrick AFB | 2,134 | 0.344 | 735 | 7,500 | 6,766 | 90.2% | 735 |
| Satellite Beach | 1,488 | 0.344 | 512 | 2,462 | 1,949 | 79.2% | 512 |
| de minimus | 609 | - |  | 576 | - | 0.0% | 576 |
| Total | 51,385 | - | 17,487 | 53,552 | 35,489 | 66.3% | 18,063 |

Note:  The resulting load is higher than the target load because the de minimus entities are not required to reduce loads in the first iteration of the BMAP.

In examining these tables, the required TN reduction for Kennedy Space Center appeared large given the high proportion of undeveloped lands within this facility.  To determine why this might be occurring, FDEP calculated the required reductions for TN per built-up acre for each entity.  As highlighted in Table 7, the required reduction for TN per built-up acre is significantly higher for Kennedy Space Center than the other entities.

Table 7.  TN Required Reduction per Built Up Acre by Entity

| Entity | TN Required Reduction (lbs) | Undeveloped Acres | Built-Up Acres | TN Required Reduction per Built-Up Acre | % Undeveloped |
| --- | --- | --- | --- | --- | --- |
| Brevard County | 54,167 | 4,793 | 5,677 | 9.5 | 45.8% |
| Cape Canaveral | 7,203 | 247 | 609 | 11.8 | 28.9% |
| Cape Canaveral AFS | 24,915 | 11,103 | 2,691 | 9.3 | 80.5% |
| Cocoa Beach | 14,978 | 134 | 1,723 | 8.7 | 7.2% |
| FDOT 5 | 2,955 | 44 | 342 | 8.6 | 11.4% |
| Indian Harbour Beach | 9,361 | 144 | 1,107 | 8.5 | 11.5% |
| Kennedy Space Center | 33,060 | 16,788 | 1,752 | 18.9 | 90.5% |
| Patrick AFB | 24,649 | 113 | 2,021 | 12.2 | 5.3% |
| Satellite Beach | 10,922 | 207 | 1,281 | 8.5 | 13.9% |
| de minimus | 0 | 295 | 314 | - | - |

As a comparison, FDEP also calculated the average TN loads for the undeveloped areas for the overall BRL Basin, as well as for Kennedy Space Center (Tables 8 and 9).  As shown in these tables, the average loading from the undeveloped land for Kennedy Space Center is higher than the basin average.

Table 8.  TN and TP Loads for Undeveloped Acres in Banana River Lagoon

| FLUCCS | Definition | Area (acres) | TN (lbs) | TP (lbs) | TN (lbs/acre) | TP (lbs/acre) |
| --- | --- | --- | --- | --- | --- | --- |
| 3100 | Herbaceous (Dry Prairie) | 3,799 | 20,305 | 1,079 | 5.34 | 0.284 |
| 3200 | Shrub and Brushland | 5,768 | 26,057 | 1,379 | 4.52 | 0.239 |
| 3300 | Mixed Rangeland | 346 | 1,489 | 78 | 4.30 | 0.224 |
| 4100 | Upland Coniferous Forests | 52 | 187 | 24 | 3.57 | 0.458 |
| 4110 | Pine Flatwoods | 1,985 | 6,928 | 890 | 3.49 | 0.448 |
| 4130 | Sand Pine | 15 | 18 | 2 | 1.23 | 0.157 |
| 4200 | Upland Hardwood Forests | 370 | 893 | 116 | 2.41 | 0.312 |
| 4210 | Xeric Oak | 6,343 | 12,299 | 1,580 | 1.94 | 0.249 |
| 4340 | Hardwood - Coniferous Mixed | 961 | 2,085 | 269 | 2.17 | 0.280 |
| 4370 | Australian Pines | 60 | 143 | 19 | 2.39 | 0.309 |
| 4430 | Forest Regeneration Areas | 24 | 74 | 10 | 3.12 | 0.400 |
| 5100 | Streams and Waterways | 1,008 | 6,777 | 562 | 6.72 | 0.557 |
| 5200 | Lakes | 27 | 90 | 16 | 3.39 | 0.618 |
| 5300 | Reservoirs | 387 | 1,062 | 182 | 2.74 | 0.470 |
| 5340 | Reservoirs less than 10 acres (4 hectares) which are dominant features | 53 | 144 | 28 | 2.74 | 0.527 |
| 5430 | Embayments not opening directly into the Atlantic Ocean | 2,171 | - | - | - | - |
| 6120 | Mangrove Swamps | 1,730 | - | - | - | - |
| 6150 | Streams and Lake Swamps (Bottomland) | 0 | - | - | - | - |
| 6170 | Mixed Wetland Hardwoods | 1,250 | - | - | - | - |
| 6181 | Willow and Elderberry | 72 | - | - | - | - |
| 6182 | Willow and Elderberry | 286 | - | - | - | - |
| 6210 | Cypress | 16 | - | - | - | - |
| 6300 | Wetland Forested Mixed | 427 | - | - | - | - |
| 6410 | Freshwater Marshes | 2,082 | - | - | - | - |
| 6420 | Saltwater Marshes | 1,909 | - | - | - | - |
| 6430 | Wet Prairies | 268 | - | - | - | - |
| 6440 | Emergent Aquatic Vegetation | 15 | - | - | - | - |
| 6460 | Treeless Hydric Savanna | 2,431 | - | - | - | - |
| 6500 | Non-Vegetated | 11 | - | - | - | - |
|  | Overall | 33,869 | 78,551 | 6,234 | 2.32 | 0.184 |

Note:  In the BRL, approximately 66% of the acres in the basin are undeveloped areas, and the majority of these areas (82%) occur on Kennedy Space Center and Cape Canaveral AFS.

Table 9.  TN and TP Loads for Undeveloped Acres on Kennedy Space Center

| FLUCCS | Definition | Area (acres) | TN (lbs) | TP (lbs) | TN (lbs/acre) | TP (lbs/acre) |
| --- | --- | --- | --- | --- | --- | --- |
| 3100 | Herbaceous (Dry Prairie) | 2,935 | 17,226 | 918.0 | 5.87 | 0.313 |
| 3200 | Shrub and Brushland | 3,835 | 20,845 | 1,108.3 | 5.44 | 0.289 |
| 3300 | Mixed Rangeland | 155 | 892 | 46.9 | 5.76 | 0.303 |
| 4100 | Upland Coniferous Forests | 52 | 187 | 24.0 | 3.57 | 0.458 |
| 4110 | Pine Flatwoods | 1,906 | 6,698 | 860.7 | 3.51 | 0.451 |
| 4200 | Upland Hardwood Forests | 280 | 732 | 94.9 | 2.61 | 0.339 |
| 4210 | Xeric Oak | 31 | 92 | 12.0 | 2.95 | 0.385 |
| 4340 | Hardwood - Coniferous Mixed | 379 | 802 | 103.1 | 2.12 | 0.272 |
| 4370 | Australian Pines | 7 | 9 | 1.2 | 1.24 | 0.165 |
| 4430 | Forest Regeneration Areas | 24 | 74 | 9.5 | 3.12 | 0.400 |
| 5100 | Streams and Waterways | 59 | 341 | 27.9 | 5.76 | 0.471 |
| 5200 | Lakes | 2 | 9 | 1.5 | 3.69 | 0.614 |
| 5300 | Reservoirs | 87 | 239 | 34.2 | 2.76 | 0.394 |
| 5340 | Reservoirs less than 10 acres (4 hectares) which are dominant features | 48 | 138 | 25.9 | 2.87 | 0.538 |
| 5430 | Embayments not opening directly into the Atlantic Ocean | 483 | - | - | - | 0.000 |
| 6120 | Mangrove Swamps | 623 | - | - | - | 0.000 |
| 6150 | Streams and Lake Swamps (Bottomland) | 0 | - | - | - | 0.000 |
| 6170 | Mixed Wetland Hardwoods | 1,145 | - | - | - | 0.000 |
| 6181 | Willow and Elderberry | 39 | - | - | - | 0.000 |
| 6182 | Willow and Elderberry | 286 | - | - | - | 0.000 |
| 6210 | Cypress | 16 | - | - | - | 0.000 |
| 6300 | Wetland Forested Mixed | 297 | - | - | - | 0.000 |
| 6410 | Freshwater Marshes | 1,948 | - | - | - | 0.000 |
| 6420 | Saltwater Marshes | 410 | - | - | - | 0.000 |
| 6430 | Wet Prairies | 197 | - | - | - | 0.000 |
| 6440 | Emergent Aquatic Vegetation | 2 | - | - | - | 0.000 |
| 6460 | Treeless Hydric Savanna | 1,534 | - | - | - | 0.000 |
| 6500 | Non-Vegetated | 8 | - | - | - | 0.000 |
|  | Overall | 16,788 | 48,284 | 3,268 | 2.88 | 0.195 |

From Table 8, the average loading (2.32 lbs/acre) from the undeveloped acres in the BRL Basin is greater than the nonpoint source TMDL target load (2.04 lbs/acre).  Due to the large percentage of undeveloped area in this basin, FDEP decided to use the average loading from the undeveloped acres as the interim target for TN in the first phase of the BMAP.  Therefore, a target load of 2.32 lbs/acre of TN will be used in this BMAP.  However, the average loading from undeveloped acres on Kennedy Space Center is greater than the basin-wide average.  Since Kennedy Space Center has such a large portion of their area (approximately 91%) as undeveloped land, FDEP determined that the average undeveloped area load per acre of 2.88 will be used as the target for Kennedy Space Center.  Using this target puts the required reductions for TN per built-up acre on Kennedy Space Center in line with the other entities (see Table 10).  The TMDL nonpoint target for TP of 0.344 lbs/acre will be applied for all stakeholders.  It is important to note that the TMDL requirements have not changed; however, FDEP is using an interim target for the purposes of allocating reduction requirements in this first phase of the BMAP.  Additional reductions may be required in later iterations of the BMAP to meet the TMDL targets.

Table 10.  TN Required Reduction per Built Up Acre by Entity Using the Interim Target

| Entity | TN Required Reduction (lbs) | Undeveloped Acres | Built-Up Acres | TN Required Reduction per Built-Up Acre | % Undeveloped |
| --- | --- | --- | --- | --- | --- |
| Brevard County | 51,206 | 4,793 | 5,677 | 9.0 | 45.8% |
| Cape Canaveral | 6,960 | 247 | 609 | 11.4 | 28.9% |
| Cape Canaveral AFS | 21,013 | 11,103 | 2,691 | 7.8 | 80.5% |
| Cocoa Beach | 14,452 | 134 | 1,723 | 8.4 | 7.2% |
| FDOT 5 | 2,846 | 44 | 342 | 8.3 | 11.4% |
| Indian Harbour Beach | 9,007 | 144 | 1,107 | 8.1 | 11.5% |
| Kennedy Space Center | 17,492 | 16,788 | 1,752 | 10.0 | 90.5% |
| Patrick AFB | 24,046 | 113 | 2,021 | 11.9 | 5.3% |
| Satellite Beach | 10,501 | 207 | 1,281 | 8.2 | 13.9% |
| de minimus | 0 | 295 | 314 | - | - |

BMAP Phase 1 Allocations

Using the target loads discussed above, the final allocations for Phase 1 of the BRL BMAP are shown in Tables 11 and 12.

Table 11. BRL BMAP Phase 1 Required TN Reductions for the BRL BMAP

| Entity | Area (acres) | TN Target (lbs/acre) | TN Target (lbs) | Base Load (lbs) | Required Reduction (lbs) | Required Reduction (%) | BMAP 1 Required Reduction (lbs) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Brevard County | 10,470 | 2.32 | 24,283 | 75,489 | 51,206 | 67.8% | 17,068.7 |
| Cape Canaveral | 856 | 2.32 | 1,985 | 8,945 | 6,960 | 77.8% | 2,320.2 |
| Cape Canaveral AFS | 13,795 | 2.32 | 31,994 | 53,007 | 21,013 | 39.6% | 7,004.4 |
| Cocoa Beach | 1,857 | 2.32 | 4,307 | 18,759 | 14,452 | 77.0% | 4,817.5 |
| FDOT 5 | 386 | 2.32 | 895 | 3,741 | 2,846 | 76.1% | 948.5 |
| Indian Harbour Beach | 1,251 | 2.32 | 2,901 | 11,908 | 9,007 | 75.6% | 3,002.4 |
| Kennedy Space Center | 18,540 | 2.88 | 53,324 | 70,816 | 17,492 | 24.7% | 5,830.7 |
| Patrick AFB | 2,134 | 2.32 | 4,948 | 28,994 | 24,046 | 82.9% | 8,015.3 |
| Satellite Beach | 1,488 | 2.32 | 3,451 | 13,952 | 10,501 | 75.3% | 3,500.3 |
| de minimus | 609 | - | 2,954 | 2,954 | 0 | 0.0% | 0.0 |
| Total | 51,385 | - | 128,087 | 288,565 | 157,524 | 54.6% | 52,508.0 |

Table 12. BRL BMAP Phase 1 Required TP Reductions for the BRL BMAP

| Entity | Area (acres) | TP Target (lbs/acre) | TP Target (lbs) | Base Load (lbs) | Required Reduction (lbs) | Required Reduction (%) | BMAP 1 Required Reduction (lbs) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Brevard County | 10,470 | 0.344 | 3,606 | 14,741 | 11,135 | 75.5% | 3,711.8 |
| Cape Canaveral | 856 | 0.344 | 295 | 1,957 | 1,663 | 84.9% | 554.2 |
| Cape Canaveral AFS | 13,795 | 0.344 | 4,751 | 10,790 | 6,039 | 56.0% | 2,013.0 |
| Cocoa Beach | 1,857 | 0.344 | 640 | 3,781 | 3,141 | 83.1% | 1,047.2 |
| FDOT 5 | 386 | 0.344 | 133 | 1,076 | 943 | 87.6% | 314.4 |
| Indian Harbour Beach | 1,251 | 0.344 | 431 | 2,092 | 1,662 | 79.4% | 553.8 |
| Kennedy Space Center | 18,540 | 0.344 | 6,385 | 8,576 | 2,191 | 25.5% | 730.4 |
| Patrick AFB | 2,134 | 0.344 | 735 | 7,500 | 6,766 | 90.2% | 2,255.2 |
| Satellite Beach | 1,488 | 0.344 | 512 | 2,462 | 1,949 | 79.2% | 649.8 |
| de minimus | 609 | - | 566 | 566 | 0 | 0.0% | 0.0 |
| Total | 51,385 | - | 18,053 | 53,543 | 35,489 | 66.3% | 11,829.7 |

Comparison to the TMDL

Tables 13 and 14 provide a summary of the total BMAP loading in the basin, using the target loads for nonpoint sources described above, compared to the TN and TP loading required by the TMDL.

Table 13.  Comparison of TN Loading in the BMAP Allocations to the TMDL

| Source | BMAP TN Loading (lbs/yr) | TMDL TN Loading (lbs/yr) | Difference in Loading (lbs/yr) | % Difference in Loading |
| --- | --- | --- | --- | --- |
| Nonpoint sources | 128,087 | 104,642 | 25,626 | 22.4% |
| Wastewater treatment facilities | 7,387 | 7,387 | 0 | 0.0% |
| Atmospheric Deposition | 179,727 | 179,727 | 0 | 0.0% |
| Total | 315,201 | 291,756 | 25,626 | 8.0% |

Table 14.  Comparison of TP Loading in the BMAP Allocations to the TMDL

| Source | BMAP TP Loading (lbs/yr) | TMDL TP Loading (lbs/yr) | Difference in Loading (lbs/yr) | % Difference in Loading |
| --- | --- | --- | --- | --- |
| Nonpoint sources | 18,053 | 17,697 | 356 | 2.0% |
| Wastewater treatment facilities | 1,523 | 1,523 | 0 | 0.0% |
| Atmospheric Deposition | 4,033 | 4,033 | 0 | 0.0% |
| Total | 23,609 | 23,253 | 356 | 1.5% |

Next Steps

During the July 9, 2010 BRL BMAP meeting, FDEP will discuss the next steps in the BMAP process with the stakeholders.  FDEP will present TN and TP removal efficiencies for standard stormwater best management practices (BMPs), the method for project collection for credit towards TN and TP reductions, and timelines for submitting projects.  Additionally, the monitoring meetings will continue to determine the objectives of the monitoring plan and monitoring station locations for data collection.  The information from the monitoring plan discussions will also be brought to the technical meetings for discussion to help identify key unknowns in the basin, which should be addressed in the TMDL update for the second phase of the BMAP.