North Indian River Lagoon Basin Management Action Plan

Allocation Options

March 2012

This paper outlines the approach proposed by the Florida Department of Environmental Protection (FDEP) for allocations in the North Indian River Lagoon (IRL) Basin Management Action Plan (BMAP).  Two other options are also presented for comparison.  The stakeholders are required to achieve 15% of the required reductions in the first five-year iteration of the North IRL BMAP.  Progress towards the seagrass depth limit targets will be evaluated to determine the necessary reductions in future BMAP iterations.  The allocation approach selected for the first iteration of the BMAP could be changed for future BMAP iterations, as needed.

Proposed Allocation Approach – Target Load per Acre

FDEP is proposing to use a target load per acre allocation approach for the North IRL BMAP to focus on those areas of the watershed with high nutrient loadings, which would have the most impact on the lagoon.  Those areas with higher loads from higher density development would be required to make greater reductions to achieve the target load per acre than those areas with low-density development or natural lands.  This approach fairly and equitably allocates the reductions by requiring more reductions from those entities that have greater development and limited stormwater treatment and requiring fewer reductions from those entities that have less development and more extensive stormwater treatment.

Calculating Baseline Loading

Baseline loadings were generated from the Pollutant Load Screening Model (PLSM), which is a Geographic Information Systems (GIS) based model.  This model uses soil, rainfall, runoff data, and nutrient-land use relationships to generate total nitrogen (TN) and total phosphorus (TP) loadings for each basin land use. The baseline loadings were generated from year 2000 land use data.

To determine the BMAP allocations, the first step was to remove the areas associated with the land use land cover code 5400, the lagoon’s surface area, from the PLSM data file for the North IRL.  The resulting file was then clipped to the North A and North B project zone boundaries, which include the additional drainage area for Titusville, Cocoa, and Rockledge.  For each project zone shapefile, the individual entity data files were created by clipping out each entity’s jurisdiction, as follows:

- Florida Department of Transportation (FDOT) roads and right-of-ways.

- Areas with agricultural land uses (2000 series of Florida land use codes).

- Municipalities and federal facilities, each to its own jurisdictional boundary.

- The remaining area was assigned to the counties using the jurisdictional boundaries.

The TP total maximum daily load (TMDL) for the North IRL was based on loadings from the Hydrologic Simulation Program – FORTRAN (HSPF) model, not the PLSM.  To account for the differences in loading between the two models, a factor was calculated based on the proportion of the HSPF model TP loads to the PLSM TP loads for each lagoon segment.  These factors were then multiplied by the TP baseline loads for each polygon in the PLSM.  The resulting total load by entity is the adjusted TP baseline load, which was used in the allocation calculations.

The loading and acreages from the land use land cover code 5000 were removed from each entity’s baseline load and total acreages and set aside because this land use category is surface waters and is not considered an anthropogenic land use in the TMDL process.  FDEP predicts that internal loading in the surface waters will decrease with the flushing of cleaner watershed runoff once the pollutant loading from the watershed is controlled.

Tables 1 and 2 show the TN and TP loads for the baseline loads, 5000 land use loads, and the net load (baseline loads minus 5000 land use loads) for each entity in North A and North B, respectively.

Table 1.  Net Loads by Entity in North A

| Entity | Area (Acres) | 5000 Land Use Acres | Net Acres | TN Baseline Load (lbs/yr) | 5000 Land Use TN Load (lbs/yr) | TN Net Load (lbs/yr) | TP Adjusted Baseline Load (lbs/yr) | 5000 Land Use TP Load (lbs/yr) | TP Net Load (lbs/yr) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Agriculture | 8,069 | 0 | 8,069 | 62,810 | 0 | 62,810 | 9611.3 | 0.0 | 9,611.3 |
| Brevard County | 8,305 | 108 | 8,197 | 37,051 | 194 | 36,857 | 4205.7 | 27.4 | 4,178.3 |
| Edgewater | 1,244 | 62 | 1,182 | 3,266 | 181 | 3,085 | 346 | 22.8 | 323.2 |
| FDOT D5 | 354 | 0 | 354 | 3,917 | 0 | 3,917 | 1130.9 | 0.0 | 1,130.9 |
| Kennedy Space Center (KSC) | 44,295 | 3,711 | 40,584 | 84,095 | 2,161 | 81,934 | 10277 | 224.8 | 10,052.2 |
| Oak Hill | 127 | 0 | 127 | 218 | 0 | 218 | 15.3 | 0.0 | 15.3 |
| Titusville | 5,776 | 34 | 5,742 | 64,967 | 113 | 64,854 | 12102.2 | 26.0 | 12,076.2 |
| Volusia County | 23,605 | 254 | 23,351 | 32,375 | 1,169 | 31,206 | 2632.4 | 110.8 | 2,521.6 |
| Total | 91,776 | 4,169 | 87,607 | 288,699 | 3,818 | 284,881 | 40,320.8 | 411.8 | 39,909.0 |

Table 2.  Net Loads by Entity in North B

| Entity | Area (Acres) | 5000 Land Use Acres | Net Acres | TN Baseline Load (lbs/yr) | 5000 Land Use TN Load (lbs/yr) | TN Net Load (lbs/yr) | TP Adjusted Baseline Load (lbs/yr) | 5000 Land Use TP Load (lbs/yr) | TP Net Load (lbs/yr) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Agriculture | 3,692 | 0 | 3,692 | 24,477 | 0 | 24,477 | 5,504.2 | 0.0 | 5,504.2 |
| Brevard County | 23,829 | 1,431 | 22,398 | 144,429 | 5,318 | 139,111 | 22,517.4 | 803.9 | 21,713.5 |
| Cocoa | 2,111 | 13 | 2,098 | 26,319 | 40 | 26,279 | 5,224.1 | 7.7 | 5,216.4 |
| FDOT D5 | 929 | 3 | 926 | 8,743 | 12 | 8,731 | 2,804.1 | 1.7 | 2,802.4 |
| Indialantic | 168 | 0 | 168 | 1,756 | 0 | 1,756 | 318.4 | 0.0 | 318.4 |
| Indian Harbour Beach | 1 | 0 | 1 | 11 | 0 | 11 | 1.7 | 0.0 | 1.7 |
| KSC | 4,136 | 159 | 3,977 | 5,707 | 366 | 5,341 | 975.3 | 46.4 | 928.9 |
| Melbourne | 9,920 | 258 | 9,662 | 91,888 | 1,045 | 90,843 | 20,074.0 | 151.2 | 19,922.8 |
| Palm Shores | 267 | 2 | 265 | 1,700 | 6 | 1,694 | 311.3 | 1.3 | 310.0 |
| Rockledge | 3,892 | 69 | 3,823 | 32,175 | 181 | 31,994 | 6,648.3 | 35.8 | 6,612.5 |
| Titusville | 588 | 0 | 588 | 3,013 | 0 | 3,013 | 279.6 | 0.0 | 279.6 |
| Total | 49,535 | 1,935.0 | 47,600 | 340,218 | 6,968 | 333,250 | 64,658.4 | 1,048.0 | 63,610.4 |

De Minimus Calculations

The net loads from Tables 1 and 2 were then sorted for TN and TP loads, from highest to lowest, to determine whether any entity had loads low enough that reductions from these areas would have essentially no significant impact on the required reductions in the first phase of the BMAP.  In the North A project zone, Oak Hill was determined to be de minimus because the TN and TP loads were less than 0.1% of the total loads in North A.  In the North B project zone, Indian Harbour Beach is de minimus because the TN and TP loads are approximately 0.0% of the total loads in North B.  These entities will not be assigned an allocation for either TN or TP for the first phase of the BMAP.  This de minimus designation does not preclude these entities from submitting project information, implementing reductions in the near term future, or from being assigned allocations in future BMAP iterations.

Calculating the Target Loads per Acre and Allocations

The target loads per acre were calculated by dividing the TMDL target load by the total non-water area (based on the acreage within the original PLSM boundary) in the North A and North B project zones, as shown in Tables 3 and 4, respectively.

Table 3.  Calculated Target Loads per Acre for TN and TP for North A

| Category | TN | TP |
| --- | --- | --- |
| Non-water acres | 86,667 | 86,667 |
| TMDL Target Load (lbs) | 184,254 | 20,349 |
| Target Load (lbs/acre) | 2.126 | 0.235 |

Table 4.  Calculated Target Loads per Acre for TN and TP for North B

| Category | TN | TP |
| --- | --- | --- |
| Non-water acres | 42,786 | 42,786 |
| TMDL Target Load (lbs) | 188,341 | 27,638 |
| Target Load (lbs/acre) | 4.402 | 0.646 |

The target load per acre is used to calculate the allowable or acceptable loadings of TN and TP for each entity.  It is the loading allocation assigned to each entity.  The amount of loading that each entity must reduce is the difference between the allocated loading and the baseline loading.

When using these target loads to determine loading for each entity, some of the stakeholders had a negative value for required reductions, which indicates that the net (existing) load from the PLSM is lower than the allocation based on the target load per acre.  Therefore, these entities are not required to make reductions.  In the North A project zone, both KSC and Volusia County had lower net TN loads than the allowable loads and Volusia County also had a lower net TP load than the allowable load.  In the North B project zone, KSC’s net TN and TP loads were lower than the allowable load, and Titusville’s net TP load was lower than the allowable load.

To ensure that greater reductions were not assigned than required by the TMDL, this difference between the allowable load and net load for KSC, Volusia County, and Titusville was spread out to the other entities based on their percentage of the net load.  This adjustment reduced the stakeholders’ required reductions from the initial reductions that were calculated.  For the adjustment in the North A project zone, both Edgewater and KSC were initially assigned more allocation than their TP required reduction.  Therefore, the adjustment for these two entities was made by assigning them the portion of the TP load they needed to meet their required reduction.  The rest of the stakeholders received a portion of the remaining TP load based on their percentage of the net load.

BMAP Phase 1 Allocations

The allocations for Phase 1 of the North IRL BMAP are shown in Tables 5 and 6 for North A and Tables 7 and 8 for North B.

Table 5. BMAP Phase 1 TN Required Reductions for North A

| Entity | Area (Acres) | TN Target Load per Acre (lbs/ac/yr) | TN Allocation (lbs/yr) | TN Net Load (lbs/yr) | TN Required Reduction (lbs/yr) | Portion of Volusia County and KSC Load Reduction (lbs/yr) | Revised TN Required Reduction (lbs/yr) | 15% of TN Required Reduction (lbs/yr) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Agriculture | 8,069 | 2.126 | 17,155.1 | 62,810 | 45,654.9 | 8,343.9 | 37,311.0 | 5,596.6 |
| Brevard County | 8,197 | 2.126 | 17,427.4 | 36,857 | 19,429.6 | 4,896.2 | 14,533.3 | 2,180.0 |
| Edgewater | 1,182 | 2.126 | 2,512.9 | 3,085 | 572.1 | 409.8 | 162.2 | 24.3 |
| FDOT D5 | 354 | 2.126 | 752.6 | 3,917 | 3,164.4 | 520.3 | 2,644.1 | 396.6 |
| KSC | 40,584 | 2.126 | 86,281.1 | 81,934 | -4,347.1 | - | 0.0 | 0.0 |
| Titusville | 5,742 | 2.126 | 12,207.8 | 64,854 | 52,646.2 | 8,615.4 | 44,030.7 | 6,604.6 |
| Volusia County | 23,351 | 2.126 | 49,644.6 | 31,206 | -18,438.6 | - | 0.0 | 0.0 |
| Oak Hill – de minimus | 127 | 2.126 | 270.5 | 218 | - | - | - | - |
| Total | 87,607 | N/A | 186,252.2 | 284,881 | 98,681.3 | 22,785.7 | 98,681.3 | 14,802.2 |

Table 6. BMAP Phase 1 TP Required Reductions for North A

| Entity | Area (Acres) | TP Target Load per Acre (lbs/ac/yr) | TP Allocation (lbs/yr) | TP Net Load (lbs/yr) | TP Required Reduction (lbs/yr) | Portion of Volusia County Load Reduction (lbs/yr) | Revised TP Required Reduction (lbs/yr) | 15% of TP Required Reduction (lbs/yr) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Agriculture | 8,069 | 0.235 | 1,896.3 | 9,611.3 | 7,715.0 | 856.4 | 6,858.6 | 1,028.8 |
| Brevard County | 8,197 | 0.235 | 1,926.4 | 4,178.3 | 2,251.9 | 372.3 | 1,879.6 | 281.9 |
| Edgewater | 1,182 | 0.235 | 277.8 | 323.2 | 45.4 | 45.4 | 0.0 | 0.0 |
| FDOT D5 | 354 | 0.235 | 83.2 | 1,130.9 | 1,047.7 | 100.8 | 946.9 | 142.0 |
| KSC | 40,584 | 0.235 | 9,537.2 | 10,052.2 | 515.0 | 515.0 | 0.0 | 0.0 |
| Titusville | 5,742 | 0.235 | 1,349.4 | 12,076.2 | 10,726.8 | 1,076.0 | 9,650.7 | 1,447.6 |
| Volusia County | 23,351 | 0.235 | 5,487.5 | 2,521.6 | -2,965.9 | 0.0 | 0.0 | 0.0 |
| Oak Hill – de minimus | 127 | 0.235 | 29.9 | 15.3 | - | - | - | - |
| Total | 87,607 | N/A | 20,587.6 | 39,909.0 | 19,336.0 | 2,965.9 | 19,336.0 | 2,900.4 |

Table 7. BMAP Phase 1 TN Required Reductions for North B

| Entity | Area (Acres) | TN Target Load per Acre (lbs/ac/yr) | TN Allocation (lbs/yr) | TN Net Load (lbs/yr) | TN Required Reduction (lbs/yr) | Portion of KSC Load Reduction (lbs/yr) | Revised TN Required Reduction (lbs/yr) | 15% of TN Required Reduction (lbs/yr) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Agriculture | 3,692 | 4.402 | 16,252.7 | 24,477 | 8,224.3 | 908.3 | 7,316.0 | 1,097.4 |
| Brevard County | 22,398 | 4.402 | 98,597.3 | 139,111 | 40,513.7 | 5,162.1 | 35,351.6 | 5,302.7 |
| Cocoa | 2,098 | 4.402 | 9,235.4 | 26,279 | 17,043.6 | 975.2 | 16,068.5 | 2,410.3 |
| FDOT D5 | 926 | 4.402 | 4,075.8 | 8,731 | 4,655.2 | 324.0 | 4,331.2 | 649.7 |
| Indialantic | 168 | 4.402 | 741.1 | 1,756 | 1,014.9 | 65.2 | 949.8 | 142.5 |
| KSC | 3,977 | 4.402 | 17,508.5 | 5,341 | -12,167.5 | - | 0.0 | 0.0 |
| Melbourne | 9,662 | 4.402 | 42,531.2 | 90,843 | 48,311.8 | 3,371.0 | 44,940.8 | 6,741.1 |
| Palm Shores | 265 | 4.402 | 1,165.7 | 1,694 | 528.3 | 62.9 | 465.4 | 69.8 |
| Rockledge | 3,823 | 4.402 | 16,831.0 | 31,994 | 15,163.0 | 1,187.2 | 13,975.8 | 2,096.4 |
| Titusville | 588 | 4.402 | 2,589.3 | 3,013 | 423.7 | 111.8 | 311.9 | 46.8 |
| Indian Harbour Beach - de minimus | 1 | 4.402 | 6.3 | 11 | - | - | - | - |
| Total | 47,600 | N/A | 209,534.4 | 333,250 | 123,710.9 | 12,167.5 | 123,710.9 | 18,556.6 |

Table 8. BMAP Phase 1 TP Required Reductions for North B

| Entity | Area (Acres) | TP Target Load per Acre (lbs/ac/yr) | TP Allocation (lbs/yr) | TP Net Load (lbs/yr) | TP Required Reduction (lbs/yr) | Portion of KSC and Titusville Load Reduction (lbs/yr) | Revised TP Required Reduction (lbs/yr) | 15% of TP Required Reduction (lbs/yr) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Agriculture | 3,692 | 0.646 | 2,385.1 | 5,504.2 | 3,119.1 | 153.6 | 2,965.5 | 444.8 |
| Brevard County | 22,398 | 0.646 | 14,469.3 | 21,713.5 | 7,244.2 | 605.8 | 6,638.4 | 995.8 |
| Cocoa | 2,098 | 0.646 | 1,355.3 | 5,216.4 | 3,861.1 | 145.5 | 3,715.6 | 557.3 |
| FDOT D5 | 926 | 0.646 | 598.1 | 2,802.4 | 2,204.3 | 78.2 | 2,126.1 | 318.9 |
| Indialantic | 168 | 0.646 | 108.8 | 318.4 | 209.6 | 8.9 | 200.8 | 30.1 |
| KSC | 3,977 | 0.646 | 2,569.4 | 928.9 | -1,640.5 | - | 0.0 | 0.0 |
| Melbourne | 9,662 | 0.646 | 6,241.5 | 19,922.8 | 13,681.3 | 555.8 | 13,125.5 | 1,968.8 |
| Palm Shores | 265 | 0.646 | 171.1 | 310.0 | 138.9 | 8.6 | 130.3 | 19.5 |
| Rockledge | 3,823 | 0.646 | 2,470.0 | 6,612.5 | 4,142.5 | 184.5 | 3,958.0 | 593.7 |
| Titusville | 588 | 0.646 | 380.0 | 279.6 | -100.4 | - | 0.0 | 0.0 |
| Indian Harbour Beach - de minimus | 1 | 0.646 | 0.9 | 1.7 | - | - | - | - |
| Total | 47,600 | N/A | 30,749.5 | 63,610.4 | 32,860.1 | 1,740.9 | 32,860.1 | 4,929.0 |

Alternate Allocation Approach Option 1 – Percent of Net Load

The first alternate allocation approach assigns the required reduction to each entity based on the entity’s percentage of the total net load.  For instance, if an entity is 10% of the total net load for TN, the entity will receive 10% of the required reductions for TN.  This approach results in all the stakeholders receiving the same percent required reduction.  FDEP does not believe this is the most fair and equitable allocation approach because a jurisdiction that is mainly natural lands will be required to make the same percent reduction as a jurisdiction that is highly developed.

Calculating Baseline Loading

The baseline loading was calculated following the same steps as outlined in the target load per acre approach.  Tables 1 and 2 show the TN and TP loads for the baseline loads, 5000 land use loads, and the net load (baseline loads minus 5000 land use loads) for each entity in North A and North B, respectively.

De Minimus Calculations

The de minimus entities were identified following the same steps as outlined in the target load per acre allocation approach.  Oak Hill in the North A project zone and Indian Harbour Beach in the North B project zone were identified as de minimus stakeholders.  These entities will not be assigned an allocation for either TN or TP for the first phase of the BMAP.

Calculating the Allocations

Each entity’s net load was divided by the total net load to determine their percentage of the net load from the model.  This percentage was multiplied by the TN and TP required reductions from the TMDL for the North A and North B project zones to determine the entity’s required reduction.  Table 9 shows the required reductions from the TMDL.

Table 9. TMDL TN and TP Required Reductions by Project Zone

| Project Zone | TN Required Reductions (lbs/yr) | TP Required Reductions (lbs/yr) |
| --- | --- | --- |
| North A | 104,445.0 | 19,560.0 |
| North B | 144,898.0 | 35,970.7 |

BMAP Phase 1 Allocations

The allocations for Phase 1 of the North IRL BMAP are shown in Tables 10 and 11 for North A and Tables 12 and 13 for North B.

Table 10. BMAP Phase 1 TN Required Reductions for North A

| Entity | TN Net Load (lbs/yr) | TN Required Reduction (lbs/yr) | % TN Reduction | 15% of TN Required Reduction (lbs/yr) |
| --- | --- | --- | --- | --- |
| Agriculture | 62,810 | 22,723.3 | 36.2% | 3,408.5 |
| Brevard County | 37,051 | 13,404.2 | 36.2% | 2,010.6 |
| Edgewater | 3,266 | 1,181.6 | 36.2% | 177.2 |
| FDOT D5 | 3,917 | 1,417.1 | 36.2% | 212.6 |
| Kennedy Space Center | 84,095 | 30,423.7 | 36.2% | 4,563.6 |
| Titusville | 64,967 | 23,503.6 | 36.2% | 3,525.5 |
| Volusia County | 32,375 | 11,712.6 | 36.2% | 1,756.9 |
| Oak Hill - de minimus | 218 | - | - | - |
| Total | 288,699 | 104,366.1 | N/A | 15,654.9 |

Table 11. BMAP Phase 1 TP Required Reductions for North A

| Entity | TP Net Load (lbs/yr) | TP Required Reduction (lbs/yr) | % TP Reduction | 15% of TP Required Reduction (lbs/yr) |
| --- | --- | --- | --- | --- |
| Agriculture | 9,611.3 | 4,710.6 | 49.0% | 706.6 |
| Brevard County | 4,178.3 | 2,047.8 | 49.0% | 307.2 |
| Edgewater | 323.2 | 158.4 | 49.0% | 23.8 |
| FDOT D5 | 1,130.9 | 554.3 | 49.0% | 83.1 |
| Kennedy Space Center | 10,052.2 | 4,926.7 | 49.0% | 739.0 |
| Titusville | 12,076.2 | 5,918.7 | 49.0% | 887.8 |
| Volusia County | 2,521.6 | 1,235.9 | 49.0% | 185.4 |
| Oak Hill - de minimus | 15.3 | - | - | - |
| Total | 39,909.0 | 19,552.5 | N/A | 2,932.9 |

Table 12. BMAP Phase 1 TN Required Reductions for North B

| Entity | TN Net Load (lbs/yr) | TN Required Reduction (lbs/yr) | % TN Required Reduction | 15% of TN Required Reduction (lbs/yr) |
| --- | --- | --- | --- | --- |
| Agriculture | 24,477 | 10,643.0 | 43.5% | 1,596.5 |
| Brevard County | 139,111 | 60,487.8 | 43.5% | 9,073.2 |
| Cocoa | 26,279 | 11,426.6 | 43.5% | 1,714.0 |
| FDOT D5 | 8,731 | 3,796.4 | 43.5% | 569.5 |
| Indialantic | 1,756 | 763.5 | 43.5% | 114.5 |
| Kennedy Space Center | 5,341 | 2,322.4 | 43.5% | 348.4 |
| Melbourne | 90,843 | 39,500.1 | 43.5% | 5,925.0 |
| Palm Shores | 1,694 | 736.6 | 43.5% | 110.5 |
| Rockledge | 31,994 | 13,911.5 | 43.5% | 2,086.7 |
| Titusville | 3,013 | 1,310.1 | 43.5% | 196.5 |
| Indian Harbour Beach - de minimus | - | - | - | - |
| Total | 333,239 | 144,898.0 | 43.5% | 21,734.7 |

Table 13. BMAP Phase 1 TP Required Reductions for North B

| Entity | TP Net Load (lbs/yr) | TP Required Reduction (lbs/yr) | % TP Required Reduction | 15% of TP Required Reduction (lbs/yr) |
| --- | --- | --- | --- | --- |
| Agriculture | 5,504.2 | 3,112.6 | 56.5% | 466.9 |
| Brevard County | 21,713.5 | 12,279.0 | 56.5% | 1,841.8 |
| Cocoa | 5,216.4 | 2,949.9 | 56.5% | 442.5 |
| FDOT D5 | 2,802.4 | 1,584.8 | 56.5% | 237.7 |
| Indialantic | 318.4 | 180.1 | 56.5% | 27.0 |
| Kennedy Space Center | 928.9 | 525.3 | 56.5% | 78.8 |
| Melbourne | 19,922.8 | 11,266.3 | 56.5% | 1,690.0 |
| Palm Shores | 310.0 | 175.3 | 56.5% | 26.3 |
| Rockledge | 6,612.5 | 3,739.4 | 56.5% | 560.9 |
| Titusville | 279.6 | 158.1 | 56.5% | 23.7 |
| Indian Harbour Beach - de minimus | - | - | - | - |
| Total | 63,608.7 | 35,970.7 | 56.5% | 5,395.6 |

Alternate Allocation Approach Option 2 – Percent of Net Load without Natural Lands

The second alternate allocation approach assigns the required reduction to each entity based on the entity’s percentage of the anthropogenic baseline loading.  The anthropogenic starting load is represented by land use codes 1000, 2000, 7000, and 8000.  This approach only requires reductions from stakeholders based on the anthropogenic land uses and the loads from natural land uses are not addressed.  This approach also results in all the stakeholders receiving the same percent required reduction.  FDEP does not believe this is the most fair and equitable allocation approach because a jurisdiction that is mainly natural lands will be required to make the same percent reduction as a jurisdiction that is highly developed.

Calculating Baseline Loading

The baseline loading was calculated following the same steps as outlined in the target load per acre approach.  However, the acreage and loading were only calculated for the anthropogenic areas.  Tables 14 and 15 show the TN and TP loads for the baseline loads for the anthropogenic land uses.

Table 14.  Anthropogenic Baseline Loads by Entity in North A

| Entity | Area (Acres) | TN Baseline (lbs/yr) | TP Baseline (lbs/yr) | Adjusted TP (lbs/yr) |
| --- | --- | --- | --- | --- |
| Agriculture | 8,069 | 62,810 | 13,734.5 | 9,611.3 |
| Brevard County | 2,795 | 27,667 | 4,262.6 | 3,568.1 |
| Edgewater | 200 | 1,405 | 279.4 | 187.5 |
| FDOT D5 | 324 | 3,859 | 1,178.9 | 1,126.6 |
| Kennedy Space Center | 2,616 | 27,991 | 7,425.3 | 7,237.7 |
| Oak Hill | 12 | 85 | 10.3 | 7.1 |
| Titusville | 3,745 | 60,874 | 11,334.7 | 11,716.1 |
| Volusia County | 1,414 | 9,805 | 1,683.1 | 1,122.8 |
| Total | 19,176 | 194,496 | 39,908.8 | 34,577.2 |

Table 15.  Anthropogenic Baseline Loads by Entity in North B

| Entity | Area (Acres) | TN Baseline (lbs/yr) | TP Baseline (lbs/yr) | Adjusted TP (lbs/yr) |
| --- | --- | --- | --- | --- |
| Agriculture | 3,692 | 24,477 | 6,157.6 | 5,504.2 |
| Brevard County | 12,886 | 126,781 | 21,566.2 | 20,700.3 |
| Cocoa | 1,682 | 25,665 | 5,680.5 | 5,165.3 |
| FDOT D5 | 856 | 8,636 | 2,922.6 | 2,795.6 |
| Indialantic | 168 | 1,756 | 309.7 | 318.4 |
| Indian Harbour Beach | 1 | 11 | 1.7 | 1.7 |
| Kennedy Space Center | 318 | 3,989 | 951.1 | 842.4 |
| Melbourne | 8,002 | 86,106 | 19,035.5 | 19,572.8 |
| Palm Shores | 145 | 1,426 | 271.6 | 278.9 |
| Rockledge | 2,467 | 28,011 | 5,954.8 | 6,300.6 |
| Titusville | 174 | 2,254 | 259.9 | 230.3 |
| Total | 30,391 | 309,112 | 63,111.2 | 61,710.5 |

De Minimus Calculations

The de minimus entities were identified following the same steps as outlined in the target load per acre allocation approach.  Oak Hill and Edgewater in the North A project zone and Indian Harbour Beach in the North B project zone were identified as de minimus stakeholders.  These entities will not be assigned an allocation for either TN or TP for the first phase of the BMAP.

Calculating the Allocations

Each entity’s baseline load was divided by the total anthropogenic baseline load to determine their percentage of the load from the model.  This percentage was multiplied by the TN and TP anthropogenic required reductions from the TMDL for the North A and North B project zones to determine the entity’s required reduction.  The anthropogenic TN and TP reductions were calculated by subtracting the load associated with the natural lands from the TMDL allowable loading to determine the anthropogenic allowable loading.  The difference between the anthropogenic baseline load from the model and the allowable load from the TMDL is the anthropogenic required reduction.  Table 16 and 17 show the required reductions for North A and North B, respectively.

Table 16. Anthropogenic TN and TP Required Reductions for North A

| Category | TN Required Reductions (lbs/yr) | TP Required Reductions (lbs/yr) |
| --- | --- | --- |
| TMDL Target | 184,254 | 20,349.0 |
| Natural Loads | 94,203 | 5,743.6 |
| Anthropogenic Target | 90,051 | 14,605.4 |
| Anthropogenic Baseline Load | 194,496 | 34,577.2 |
| Anthropogenic Reduction | 104,445 | 19,971.8 |

Table 17. Anthropogenic TN and TP Required Reductions for North B

| Category | TN Required Reductions (lbs/yr) | TP Required Reductions (lbs/yr) |
| --- | --- | --- |
| TMDL Target | 188,341 | 27,638.0 |
| Natural Loads | 31,106 | 2,947.9 |
| Anthropogenic Target | 157,235 | 24,690.1 |
| Anthropogenic Baseline Load | 309,112 | 61,710.5 |
| Anthropogenic Reduction | 151,877 | 37,020.4 |

BMAP Phase 1 Allocations

The allocations for Phase 1 of the North IRL BMAP are shown in Tables 18 and 19 for North A and Tables 20 and 21 for North B.

Table 18. BMAP Phase 1 TN Required Reductions for North A

| Entity | TN Anthropogenic Baseline Load (lbs/yr) | TN Required Reduction (lbs/yr) | % TN Reduction | 15% of TN Required Reduction (lbs/yr) |
| --- | --- | --- | --- | --- |
| Agriculture | 62,810 | 33,729.2 | 53.7% | 5,059.4 |
| Brevard County | 27,667 | 14,857.3 | 53.7% | 2,228.6 |
| FDOT D5 | 3,859 | 2,072.3 | 53.7% | 310.8 |
| Kennedy Space Center | 27,991 | 15,031.3 | 53.7% | 2,254.7 |
| Titusville | 60,874 | 32,689.5 | 53.7% | 4,903.4 |
| Volusia County | 9,805 | 5,265.3 | 53.7% | 789.8 |
| Edgewater - de minimus | 1,405 | - | - | - |
| Oak Hill - de minimus | 85 | - | - | - |
| Total | 194,496 | 103,644.9 | N/A | 15,546.7 |

Table 19. BMAP Phase 1 TP Required Reductions for North A

| Entity | TP Anthropogenic Adjusted Load (lbs/yr) | TP Required Reduction (lbs/yr) | % TP Reduction | 15% of TP Required Reduction (lbs/yr) |
| --- | --- | --- | --- | --- |
| Agriculture | 9,611.3 | 5,551.5 | 57.8% | 832.7 |
| Brevard County | 3,568.1 | 2,060.9 | 57.8% | 309.1 |
| FDOT D5 | 1,126.6 | 650.7 | 57.8% | 97.6 |
| Kennedy Space Center | 7,237.7 | 4,180.5 | 57.8% | 627.1 |
| Titusville | 11,716.1 | 6,767.2 | 57.8% | 1,015.1 |
| Volusia County | 1,122.8 | 648.5 | 57.8% | 97.3 |
| Edgewater - de minimus | 187.5 | - | - | - |
| Oak Hill - de minimus | 7.1 | - | - | - |
| Total | 34,577.2 | 19,859.4 | N/A | 2,978.9 |

Table 20. BMAP Phase 1 TN Required Reductions for North B

| Entity | TN Anthropogenic Baseline Load (lbs/yr) | TN Required Reduction (lbs/yr) | % TN Required Reduction | 15% of TN Required Reduction (lbs/yr) |
| --- | --- | --- | --- | --- |
| Agriculture | 24,477 | 12,026.4 | 49.1% | 1,804.0 |
| Brevard County | 126,781 | 62,291.7 | 49.1% | 9,343.8 |
| Cocoa | 25,665 | 12,610.1 | 49.1% | 1,891.5 |
| FDOT D5 | 8,636 | 4,243.2 | 49.1% | 636.5 |
| Indialantic | 1,756 | 862.8 | 49.1% | 129.4 |
| Kennedy Space Center | 3,989 | 1,959.9 | 49.1% | 294.0 |
| Melbourne | 86,106 | 42,306.7 | 49.1% | 6,346.0 |
| Palm Shores | 1,426 | 700.6 | 49.1% | 105.1 |
| Rockledge | 28,011 | 13,762.7 | 49.1% | 2,064.4 |
| Titusville | 2,254 | 1,107.5 | 49.1% | 166.1 |
| Indian Harbour Beach - de minimus | 11 | - | - | - |
| Total | 309,112 | 151,871.6 | 49.1% | 22,780.7 |

Table 21. BMAP Phase 1 TP Required Reductions for North B

| Entity | TP Anthropogenic Adjusted Load (lbs/yr) | TP Required Reduction (lbs/yr) | % TP Required Reduction | 15% of TP Required Reduction (lbs/yr) |
| --- | --- | --- | --- | --- |
| Agriculture | 5,504.2 | 3,302.0 | 60.0% | 495.3 |
| Brevard County | 20,700.3 | 12,418.2 | 60.0% | 1,862.7 |
| Cocoa | 5,165.3 | 3,098.7 | 60.0% | 464.8 |
| FDOT D5 | 2,795.6 | 1,677.1 | 60.0% | 251.6 |
| Indialantic | 318.4 | 191.0 | 60.0% | 28.7 |
| Kennedy Space Center | 842.4 | 505.4 | 60.0% | 75.8 |
| Melbourne | 19,572.8 | 11,741.8 | 60.0% | 1,761.3 |
| Palm Shores | 278.9 | 167.3 | 60.0% | 25.1 |
| Rockledge | 6,300.6 | 3,779.8 | 60.0% | 567.0 |
| Titusville | 230.3 | 138.2 | 60.0% | 20.7 |
| Indian Harbour Beach - de minimus | 1.7 | - | - | - |
| Total | 61,710.5 | 37,019.4 | 60.0% | 5,552.9 |

Comparison of the Allocation Approaches

Tables 22 and 23 for North A and Tables 24 and 25 for North B compare the TN and TP required reductions for the first BMAP iteration.

Table 22. Comparison of BMAP Phase 1 TN Required Reductions for North A

| Entity | Target Load Per Acre Approach (lbs/yr) | Alternative Approach Option 1 (lbs/yr) | Alternative Approach Option 2 (lbs/yr) |
| --- | --- | --- | --- |
| Agriculture | 5,596.6 | 3,408.5 | 5,059.4 |
| Brevard County | 2,180.0 | 2,010.6 | 2,228.6 |
| Edgewater | 24.3 | 177.2 | - |
| FDOT D5 | 396.6 | 212.6 | 310.8 |
| KSC | 0.0 | 4,563.6 | 2,254.7 |
| Titusville | 6,604.6 | 3,525.5 | 4,903.4 |
| Volusia County | 0.0 | 1,756.9 | 789.8 |
| Oak Hill – de minimus | - | - | - |
| Total | 14,802.2 | 15,654.9 | 15,546.7 |

Table 23. Comparison of BMAP Phase 1 TP Required Reductions for North A

| Entity | Target Load Per Acre Approach (lbs/yr) | Alternative Approach Option 1 (lbs/yr) | Alternative Approach Option 2 (lbs/yr) |
| --- | --- | --- | --- |
| Agriculture | 1,028.8 | 706.6 | 832.7 |
| Brevard County | 281.9 | 307.2 | 309.1 |
| Edgewater | 0.0 | 23.8 | - |
| FDOT D5 | 142.0 | 83.1 | 97.6 |
| KSC | 0.0 | 739.0 | 627.1 |
| Titusville | 1,447.6 | 887.8 | 1,015.1 |
| Volusia County | 0.0 | 185.4 | 97.3 |
| Oak Hill – de minimus | - | - | - |
| Total | 2,900.4 | 2,932.9 | 2,978.9 |

Table 24. Comparison of BMAP Phase 1 TN Required Reductions for North B

| Entity | Target Load Per Acre Approach (lbs/yr) | Alternative Approach Option 1 (lbs/yr) | Alternative Approach Option 2 (lbs/yr) |
| --- | --- | --- | --- |
| Agriculture | 1,097.4 | 1,596.5 | 1,804.0 |
| Brevard County | 5,302.7 | 9,073.2 | 9,343.8 |
| Cocoa | 2,410.3 | 1,714.0 | 1,891.5 |
| FDOT D5 | 649.7 | 569.5 | 636.5 |
| Indialantic | 142.5 | 114.5 | 129.4 |
| KSC | 0.0 | 348.4 | 294.0 |
| Melbourne | 6,741.1 | 5,925.0 | 6,346.0 |
| Palm Shores | 69.8 | 110.5 | 105.1 |
| Rockledge | 2,096.4 | 2,086.7 | 2,064.4 |
| Titusville | 46.8 | 196.5 | 166.1 |
| Indian Harbour Beach - de minimus | - | - | - |
| Total | 18,556.6 | 21,734.7 | 22,780.7 |

Table 25. Comparison of BMAP Phase 1 TP Required Reductions for North B

| Entity | Target Load Per Acre Approach (lbs/yr) | Alternative Approach Option 1 (lbs/yr) | Alternative Approach Option 2 (lbs/yr) |
| --- | --- | --- | --- |
| Agriculture | 444.8 | 466.9 | 495.3 |
| Brevard County | 995.8 | 1,841.8 | 1,862.7 |
| Cocoa | 557.3 | 442.5 | 464.8 |
| FDOT D5 | 318.9 | 237.7 | 251.6 |
| Indialantic | 30.1 | 27.0 | 28.7 |
| KSC | 0.0 | 78.8 | 75.8 |
| Melbourne | 1,968.8 | 1,690.0 | 1,761.3 |
| Palm Shores | 19.5 | 26.3 | 25.1 |
| Rockledge | 593.7 | 560.9 | 567.0 |
| Titusville | 0.0 | 23.7 | 20.7 |
| Indian Harbour Beach - de minimus | - | - | - |
| Total | 4,929.0 | 5,395.6 | 5,552.9 |