| Category | Definition | Examples |
| --- | --- | --- |
| Studies | Project will not impact the watershed and/or springshed to help meet the TMDL | Studies |
| Low Impact | Project has low potential impact on the watershed and/or springshed to help meet the TMDL. | Retrofit projects using BMPs in Group A New development projects |
| Medium Impact | Project has medium potential impact on the watershed and/or springshed to help meet the TMDL. | Retrofit projects using BMPs in Group B Street sweeping projects Land conversation projects (non-Ag) |
| High Impact | Project has high potential impact on the watershed and/or springshed to help meet the TMDL. | Retrofit projects using BMPs in Group C Land conservation projects (Ag) |

BMP Project Groupings

BMPs in Group A include: dry detention ponds, inlet baskets, baffle boxes, CDS units, erosion protection/bank stabilization, and sewer pipe repair/reline.

BMPs in Group B include: wet detention ponds, wetland rehabilitation, exfiltration systems, dry retention ponds, stormwater system inspections & maintenance, and sewering areas on septic systems.

BMPs in Group C include wetlands and reclaimed water storage ponds.

Ranking of Street Sweeping and Land Conservation Projects

Street sweeping projects are ranked based on the length of roads swept per year (compared on a miles  per year basis).  Ten entities within the basin sweep roads.  The road length swept by each entity varies from a low of 572 miles per year to a high of 6,380 miles per year; thus the maximum difference in mileage swept is roughly within one order of magnitude, which is ranked as a ‘medium impact’.

Land Conservation projects are ranked based on the size of the land acquisition and the previous land use.  Projects that purchase agricultural land for conservation purposes are given a ‘high impact’ ranking due to the high potential for lowering nutrient yields.   Conservation projects are given a ‘medium impact’ ranking if the previous land use is anything other than agricultural land.  All conservation projects over 1,000 acres are given a ‘high impact’ ranking, regardless of previous land use type.

Other Considerations for Project Rankings

In addition to general ranking of criteria, project-specific information was also considered.  The following types of information that may have changed the impact rankings are:

- Surface water projects located within the springshed (where surface water drains to the Ocklawaha Basin) – Where there is no benefit to the Wekiva springshed and only benefit to the Ocklawaha surface watershed, the project was removed from consideration in the Wekiva BMAP project list but should be considered for inclusion in the Ocklawaha BMAP.

- Size of project and treatment area (e.g. large drainage area treated, extensive drainage features, extensive linear feet of pipe replaced or repaired) yielded a larger project impact classification.

- Additional components to a project (such as Bold and GoldTM) yielded a larger project impact classification