Ichetucknee River State Park & Wekiva Alternative Drainfield Studies

Design and installation considerations

- Passive alternative system installed during initial construction or required retrofit

- Use of a Bio-reactive media layer (hardwood mulch, etc. approximately 12 - 24” in depth) overlain with clean packed sand approximately 18-24”

- Drainfield underlined with a water barrier to maintain wetted (anaerobic) conditions of the media layer will optimize denitrification (plastic liner, clay layer, etc.)

- Calculate appropriate dimensions of alternative drainfield based on septic tank and household size

- Installation of a flow meter to calculate water volume to septic tank

- Determine appropriate number of monitoring devices and locations (lysimeters, piezometers, horizontal wells, & monitoring wells) based on size of drainfield

Operation, monitoring, maintenance

- Monthly sampling for the first year, reduced to quarterly sampling following first year of operation to allow for acclimation of the system

- Monitoring devices require annual maintenance for optimal data collection

- Unlined alternative drainfield without a water barrier has begun to subside after 4 years of operation due to decomposition and settling of media layer

- Decomposition of media layer is increased when wet or saturated conditions are not maintained

- Clean sand fill will be required for subsiding alternative drainfield

Results

- Infiltration of septic tank effluent to groundwater occurs greatest closer to the header (where the effluent enters the drainfield) of a lined or unlined system

- Ichetucknee alternative drainfield (4.5 years) TN% reduction:  approximately 60%

- Wekiva lined alternative drainfield (2 years) (TN% reduction:  approximately 70%

- Household water use, irrigation, rainfall, and fertilizer/pesticide application all affect results

- Life expectancy to be determined for both lined or unlined alternative drainfield systems

- Additional research required to obtain more data