FDEP Approved Removal Efficiencies for Stormwater Best Management Practices (BMPs)

| Standard BMPs | TP % Reduction | TN % Reduction | Data Source |
| --- | --- | --- | --- |
| Off-line Retention 0.25 treatment volume Off-line Retention 0.50 treatment volume Off-line Retention 0.75 treatment volume Off-line Retention 1.00 treatment volume On-line Retention 0.25 treatment volume On-line Retention 0.50 treatment volume On-line Retention 0.75 treatment volume On-line Retention 1.00 treatment volume | 40 62 75 84 30 52 65 74 | 40 62 75 84 30 52 65 74 | Harper, H. & D. Baker, 2007. Evaluation of Current Stormwater Design Criteria within the State of Florida. Note: These numbers are provisional and may change based on new information. |
| Wet detention ponds | Reduction from Figure 13.2 given the project’s residence time, which is based on the flow from the model | Reduction from Figure 13.3 given the project’s residence time, which is based on the flow from the model | Figures 13.2 and 13.3 in Draft Stormwater Treatment Applicant’s Handbook |
| BMP treatment trains using a combination of BMPs | Use BMP Treatment Train (TT) equation: BMP TT Efficiency = Eff1 +((1-Eff1)*Eff2) | Use BMP Treatment Train (TT) equation: BMP TT Efficiency = Eff1 +((1-Eff1)*Eff2) | Draft Stormwater Treatment Applicant’s Handbook |
| Dry detention | 10 | 10 | Harper, H. & D. Baker. 2007. Evaluation of Current Stormwater Design Criteria within the State of Florida. |
| Baffle box | 2.3 | 0.5 | Final Report Contract S0236 Effectiveness of Baffle Boxes |
| Nutrient baffle box (2nd generation) | 15.5 | 19.05 | Final Report Contract S0236 Effectiveness of Baffle Boxes |
| Catch basin inserts/inlet filters | Evaluated on a case-by case basis | Evaluated on a case-by case basis | Case-by-case |
| Grass swales with swale blocks or raised culverts | Use retention BMPs above | Use retention BMPs above | Draft Stormwater Treatment Applicant’s Handbook |
| Grass swales without swale blocks or raised culverts | 50% of value for grass swales with swale blocks or raised culverts | 50% of value for grass swales with swale blocks or raised culverts | Draft Stormwater Treatment Applicant’s Handbook |
| Alum injection | 90 | 50 | Evaluation of Harper data |
| Stormwater reuse | Detention/retention project - estimate amount water not discharged annually because used for irrigation. | Detention/retention project - estimate amount water not discharged annually because used for irrigation. | Evaluated on a case-by-case basis |

Note: The Draft Stormwater Treatment Applicant’s Handbook is located at: http://www.dep.state.fl.us/water/wetlands/erp/rules/stormwater/docs/ah_rule_draft_031710.pdf

| Provisional BMPs | TP % Reduction | TN % Reduction | Data Source |
| --- | --- | --- | --- |
| Stormceptor | 13 | 2 | Final Report Contract S0095 Sanford Stormceptor project |
| Continuous deflective separation (CDS) units | 10 | Not applicable | Final Report Contract WM793 Broadway Outfall Project |
| Floating islands | 20 | 20 | Chapter 14 Draft Stormwater Treatment Applicant’s Handbook |
| Street sweeping | Determine pounds of materials removed and multiply by values to be provided by the Florida Stormwater Association (FSA) University of Florida (UF) municipal separate storm sewer system (MS4) BMP project | Determine pounds of materials removed and multiply by values to be provided by FSA UF MS4 BMP project | Final Report of FSA UF MS4 BMP Project |
| Public education | 1-6, depending on extent of program | 1-6, depending on extent of program | Evaluation of Center for Watershed Protection (CWP). 2002. Watershed Treatment Model Version 3.1. See separate calculation spreadsheet. |