Table 1: Provisional Stormwater BMPs

| Standard BMPs | TP % Reduction | TN % Reduction | Data Source |
| --- | --- | --- | --- |
| Off-line Retention 0.25” treatment volume Off-line Retention 0.50” treatment volume Off-line Retention 0.75” treatment volume Off-line Retention 1.00” treatment volume On-line Retention 0.25” treatment volume On-line Retention 0.50” treatment volume On-line Retention 0.75” treatment volume On-line Retention 1.00” treatment volume | 40 62 75 84 30 52 65 74 | 40 62 75 84 30 52 65 74 | Harper, H. & D. Baker. 2007. Evaluation of Current Stormwater Design Criteria within the State of Florida. |
| Wet detention ponds | Reduction from Figure 13.2 given the project’s residence time | Reduction from Figure 13.3 given the project’s residence time | Figures 13.2 and 13.3 in Draft Stormwater Treatment Applicant’s Handbook |
| BMP treatment trains using a combination of BMPs | Use BMP Treatment Train (TT) equation: BMP TT Efficiency = Eff1 +((1-Eff1)*Eff2) | Use BMP Treatment Train (TT) equation: BMP TT Efficiency = Eff1 +((1-Eff1)*Eff2) | Draft Stormwater Treatment Applicant’s Handbook |
| Dry detention | 10 | 10 | Harper, H. & D. Baker. 2007. Evaluation of Current Stormwater Design Criteria within the State of Florida. |
| Baffle box | 2.3 | 0.5 | Final Report Contract S0236 Effectiveness of Baffle Boxes |
| Nutrient baffle box (2nd generation) | 15.5 | 19.05 | Final Report Contract S0236 Effectiveness of Baffle Boxes |
| Grass swales with swale blocks or raised culverts | Use on-line retention BMPs above | Use on-line retention BMPs above | Evaluation of Harper data |
| Grass swales without swale blocks or raised culverts | 50% of value for grass swales with swale blocks or raised culverts | 50% of value for grass swales with swale blocks or raised culverts | Evaluation of Harper data |
| Alum injection | 90 | 50 | Evaluation of Harper data |
| Stormwater reuse | Estimate amount water not discharged annually because used for irrigation. | Estimate amount water not discharged annually because used for irrigation. | Evaluated on a case-by-case basis |
| Stormceptor | 13 | 2 | Final Report Contract S0095 Sanford Stormceptor project |
| Continuous deflective separation (CDS) units | 10 | Not applicable | Final Report Contract WM793 Broadway Outfall Project |
| Street sweeping | Determine the dry weight/volume of material collected annually and multiply by values provided by the Florida Stormwater Association (FSA) University of Florida (UF) municipal separate storm sewer system (MS4) BMP project | Determine the dry weight/volume of material collected annually and multiply by values to be provided by FSA UF MS4 BMP project | Final Report of FSA UF MS4 BMP Project |
| Catch basin inserts/inlet filters | Determine the dry weight/volume of material collected annually and multiply by values to be provided by FSA UF MS4 BMP project | Determine the dry weight/volume of material collected annually and multiply by values to be provided by FSA UF MS4 BMP project | Final Report of FSA UF MS4 BMP Project |
| Septic tank phase out | Based on values from ArcNLET model | Based on values from ArcNLET model | http://people.sc.fsu.edu/~mye/ArcNLET/index.html |

Note: The Draft Stormwater Treatment Applicant’s Handbook is located at http://publicfiles.dep.state.fl.us/dwrm/stormwater/stormwater_rule_development/docs/ah_rule_draft_031710.pdf

Table 2: Provisional Stormwater BMPs

| Provisional BMPs | TP % Reduction | TN % Reduction | Data Source |
| --- | --- | --- | --- |
| Public education | 1-6, depending on extent of program | 1-6, depending on extent of program | Evaluation of Center for Watershed Protection. 2002. Watershed Treatment Model Version 3.1. See separate calculation spreadsheet. |
| Floating islands/ Managed aquatic plant systems (MAPS) | 20 | 20 | Chapter 14 Draft Stormwater Treatment Applicant’s Handbook |
| Muck removal/ restoration dredging | Case-by-case depending on the nutrient flux of the muck | Case-by-case depending on the nutrient flux of the muck | FDEP Muck Removal Credit Guidance (developed for the Indian River Lagoon BMAPs) |
| Aquatic vegetation harvesting | Based on the total mass of material collected, type of plant(s), and associated nutrient content in the dry material | Based on the total mass of material collected, type of plant(s), and associated nutrient content in the dry material | FDEP Removal of Aquatic Vegetation for Nutrient Credits (developed for the Indian River Lagoon BMAPs) |