St. Lucie Basin River and Estuary Management Action Plan (BMAP)

Allocation Process and Calculations

January 16, 2013

Background

The Florida Department of Environmental Protection (FDEP) created a spreadsheet model for the St. Lucie BMAP to calculate the starting loads, allocations, and required reductions for each responsible entity in the basin needed to achieve water quality standards.  FDEP worked with the South Florida Water Management District (SFWMD) to align the factors in the BMAP spreadsheet model with those factors used in the model for the 2009 St. Lucie River Watershed Protection Plan (SLRWPP) and the 2012 SLRWPP Update.

The overall BMAP area is divided into six sub-watersheds:  North Fork, South Fork, Basins 4 5 6, C-23, C-24, and C-44 S-153.  The loading in each of these sub-watersheds is estimated in the spreadsheet model using the 2004 land use coverage, event mean concentrations (EMCs) for total nitrogen (TN) and total phosphorus (TP) associated with each land use type, runoff coefficients (ROCs) for each land use type, and rainfall values for each sub-watershed.

The steps to calculate the allocations and required reductions for the stakeholders in the St. Lucie BMAP are outlined in this paper.

Starting Loads

To determine the starting loads, FDEP clipped and erased from the BMAP model Geographic Information Systems (GIS) shapefile, known as the base map, to create separate shapefiles with each entity’s responsible acres by sub-watershed.  This clipping was done using the jurisdictional boundaries provided by each entity.  The individual entity shapefiles were created by clipping out areas sequentially as follows:

- Natural lands, associated with land cover codes 3000 (upland nonforested; not including 3300), 4000 (upland forests), 5000 (water), and 6000 (wetlands);

- Water control districts (WCDs) and other special districts;

- Agriculture, associated with land cover codes 2000 (agriculture) and 3300 (rangeland);

- Florida Department of Transportation (FDOT) and Turnpike road and right-of-ways;

- Community development districts (CDDs);

- Municipal separate storm sewer system (MS4s) areas for municipalities and counties;

- Florida Power & Light (FPL) pond;

- Non-MS4 areas for municipalities; and

- Remaining area assigned to each county.

Using the model factors associated with each land cover type and each sub-watershed, the TN and TP starting loads were calculated.  Table 1 and Table 2 show the TN and TP starting loads, respectively, for each entity.  Table 3 shows the acres for each entity within each sub-watershed.

Table 1:  TN Starting Loads by Entity

| Entity | Basins 4 5 6 (lbs/yr) | C-23 (lbs/yr) | C-24 (lbs/yr) | C-44 S-153 (lbs/yr) | North Fork (lbs/yr) | South Fork (lbs/yr) | Total (lbs/yr) | Total (MT/yr) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Agriculture | 17,051 | 470,081 | 574,852 | 458,432 | 24,355 | 126,080 | 1,670,852 | 757.89 |
| Copper Creek CDD | - | - | 14 | - | - | - | 14 | 0.01 |
| FDOT District 4 | 952 | 1,510 | 950 | 1,537 | 4,277 | 3,649 | 12,874 | 5.84 |
| Fort Pierce MS4 | - | - | - | - | 17,041 | - | 17,041 | 7.73 |
| Hobe St. Lucie Conservancy District | - | - | - | 17,482 | - | 10,819 | 28,301 | 12.84 |
| Martin County MS4 | 26,394 | 5,947 | - | 10,775 | 19,806 | 40,423 | 103,345 | 46.88 |
| North St. Lucie River WCD | - | - | 37,251 | - | 160,152 | - | 197,404 | 89.54 |
| Okeechobee County MS4 | - | 3,184 | 121 | - | - | - | 3,305 | 1.50 |
| Pal Mar WCD | - | - | - | 8,834 | - | 22 | 8,856 | 4.02 |
| Port St. Lucie MS4 | - | 396 | 8,217 | - | 127,998 | - | 136,611 | 61.97 |
| Port St. Lucie Non-MS4 | - | 1,119 | 58 | - | 18,693 | - | 19,871 | 9.01 |
| Sewall’s Point MS4 | - | - | - | - | 1,771 | - | 1,771 | 0.80 |
| St. Lucie County MS4 | - | - | - | - | 18,114 | - | 18,114 | 8.22 |
| St. Lucie County Non-MS4 | - | 1,594 | 16,757 | - | 5,409 | - | 23,761 | 10.78 |
| Stuart MS4 | - | - | - | - | 1,614 | 12,384 | 13,998 | 6.35 |
| Tradition CDD | - | 1 | 7,057 | - | 31 |  | 7,089 | 3.22 |
| Troup-Indiantown WCD | - | - | - | 81,331 | - | - | 81,331 | 36.89 |
| Turnpike | 789 | 51 | - | - | 2,651 | 1,286 | 4,776 | 2.17 |
| Verano CDD | - | - | 257 | - | - | - | 257 | 0.12 |
| Total | 45,185 | 483,883 | 645,534 | 578,391 | 401,912 | 194,662 | 2,349,567 | 1,066 |

Table 2:  TP Starting Loads by Entity

| Entity | Basins 4 5 6 (lbs/yr) | C-23 (lbs/yr) | C-24 (lbs/yr) | C-44 S-153 (lbs/yr) | North Fork (lbs/yr) | South Fork (lbs/yr) | Total (lbs/yr) | Total (MT/yr) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Agriculture | 3,920 | 150,255 | 136,471 | 87,337 | 5,988 | 26,869 | 410,840 | 186.35 |
| Copper Creek CDD | - | - | 3 | - | - | - | 3 | 0.00 |
| FDOT District 4 | 200 | 464 | 226 | 229 | 818 | 659 | 2,596 | 1.18 |
| Fort Pierce MS4 | - | - | - | - | 3,879 | - | 3,879 | 1.76 |
| Hobe St. Lucie Conservancy District | - | - | - | 3,515 | - | 2,563 | 6,078 | 2.76 |
| Martin County MS4 | 5,930 | 2,250 | - | 1,871 | 4,339 | 8,419 | 22,809 | 10.35 |
| North St. Lucie River WCD | - | - | 9,063 | - | 36,821 | - | 45,884 | 20.81 |
| Okeechobee County MS4 | - | 937 | 38 | - | - | - | 974 | 0.44 |
| Pal Mar WCD | - | - | - | 1,318 | - | 4 | 1,322 | 0.60 |
| Port St. Lucie MS4 | - | 139 | 2,193 | - | 28,271 | - | 30,602 | 13.88 |
| Port St. Lucie Non-MS4 | - | 379 | 13 | - | 4,021 | - | 4,413 | 2.00 |
| Sewall’s Point MS4 | - | - | - | - | 384 | - | 384 | 0.17 |
| St. Lucie County MS4 | - | - | - | - | 4,127 | - | 4,127 | 1.87 |
| St. Lucie County Non-MS4 | - | 838 | 3,961 | - | 1,273 | - | 6,072 | 2.75 |
| Stuart MS4 | - | - | - | - | 379 | 2,727 | 3,107 | 1.41 |
| Tradition CDD | - | - | 1,903 | - | 7 | - | 1,910 | 0.87 |
| Troup-Indiantown WCD | - | - | - | 16,502 | - | - | 16,502 | 7.49 |
| Turnpike | 170 | 16 | - | - | 506 | 233 | 925 | 0.42 |
| Verano CDD | - | - | 63 | - | - | - | 63 | 0.03 |
| Total | 10,221 | 155,278 | 159,933 | 110,772 | 90,812 | 41,473 | 562,489 | 255 |

Table 3:  Acres by Entity

| Entity | Basins 4 5 6 (acres) | C-23 (acres) | C-24 (acres) | C-44 S-153 (acres) | North Fork (acres) | South Fork (acres) | Total (acres) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Agriculture | 2,445 | 84,744 | 63,488 | 65,937 | 3,967 | 18,176 | 238,757 |
| Copper Creek CDD | - | - | 2 | - | - | - | 2 |
| FDOT District 4 | 171 | 306 | 137 | 270 | 864 | 636 | 2,383 |
| Fort Pierce MS4 | - | - | - | - | 3,706 | - | 3,706 |
| FPL Pond | - | - | - | 6,501 | - | - | 6,501 |
| Hobe St. Lucie Conservancy District | - | - | - | 2,949 | - | 1,945 | 4,894 |
| Martin County MS4 | 4,989 | 1,738 | - | 2,231 | 4,378 | 7,763 | 21,100 |
| Natural Lands | 7,830 | 23,706 | 15,701 | 37,163 | 33,129 | 18,987 | 136,516 |
| North St. Lucie River WCD | - | - | 4,028 | - | 32,491 | - | 36,519 |
| Okeechobee County MS4 | - | 574 | 30 | - | - | - | 604 |
| Pal Mar WCD | - | - | - | 1,161 | - | 4 | 1,165 |
| Port St. Lucie MS4 | - | 89 | 1,252 | - | 30,374 | - | 31,715 |
| Port St. Lucie Non-MS4 | - | 237 | 6 | - | 3,744 | - | 3,988 |
| Sewall’s Point MS4 | - | - | - | - | 457 | - | 457 |
| St. Lucie County MS4 | - | - | - | - | 3,995 | - | 3,995 |
| St. Lucie County Non-MS4 | - | 763 | 2,172 | - | 1,146 | - | 4,081 |
| Stuart MS4 | - | - | - | - | 353 | 2,386 | 2,739 |
| Tradition CDD | - | - | 923 | - | 6 | - | 929 |
| Troup-Indiantown WCD | - | - | - | 13,649 | - | - | 13,649 |
| Turnpike | 147 | 10 | - | - | 528 | 226 | 912 |
| Verano CDD | - | - | 36 | - | - | - | 36 |
| Total | 15,582 | 112,167 | 87,777 | 129,862 | 119,139 | 50,122 | 514,649 |

Natural Land Uses and FPL Pond

The stakeholders are not expected to make reductions in areas that have natural land uses.  Therefore, the acres and loadings associated with the natural land uses are set aside in the allocations.  The target loads per acre, allocations, and required reductions described below are based on the anthropogenic (human) land uses.

In addition, the acreage and loading associated with the FPL pond were also set aside in the allocations.  FPL draws water from the C-44 canal for cooling during their process, which does not add nutrients.  The water is discharged to the cooling pond before it is returned to the C-44 canal.  Since FPL does not add nutrients to the water, they are considered a non-contributor in the basin, and additional nutrient reductions are not required in the pond.

De Minimus Entities

The starting loads were sorted for TN and TP loads, from highest to lowest, to determine whether any entity had loads low enough that reductions from these areas would not have a significant impact on the required reductions in the first BMAP iteration.  Those entities whose total TN and TP loads were low enough are considered de minimus.  Table 4 and Table 5 present the sorted data, and the shaded cells signify the de minimus load contributions.

Table 4:  TN Starting Loads Sorted from Highest to Lowest

| Entity | TN Starting Load (lbs/yr) | % of Total TN Starting Load |
| --- | --- | --- |
| Agriculture | 1,670,852 | 71.1% |
| North St. Lucie River WCD | 197,404 | 8.4% |
| Port St. Lucie MS4 | 136,611 | 5.8% |
| Martin County MS4 | 103,345 | 4.4% |
| Troup-Indiantown WCD | 81,331 | 3.5% |
| Hobe St. Lucie Conservancy District | 28,301 | 1.2% |
| St. Lucie County Non-MS4 | 23,761 | 1.0% |
| Port St. Lucie Non-MS4 | 19,871 | 0.8% |
| St. Lucie County MS4 | 18,114 | 0.8% |
| Fort Pierce MS4 | 17,041 | 0.7% |
| Stuart MS4 | 13,998 | 0.6% |
| FDOT District 4 MS4 | 12,874 | 0.5% |
| Pal Mar WCD* | 8,856 | 0.4% |
| Tradition CDD* | 7,089 | 0.3% |
| Turnpike* | 4,776 | 0.2% |
| Okeechobee County* | 3,305 | 0.1% |
| Sewall's Point MS4* | 1,771 | 0.1% |
| Verano CDD* | 257 | 0.0% |
| Copper Creek CDD* | 14 | 0.0% |

Note: de minimus stakeholders are marked with a “*.”

Table 5:  TP Starting Loads Sorted from Highest to Lowest

| Entity | TP Starting Load (lbs/yr) | % of Total TP Starting Load |
| --- | --- | --- |
| Agriculture | 410,840 | 73.0% |
| North St. Lucie River WCD | 45,884 | 8.2% |
| Port St. Lucie MS4 | 30,602 | 5.4% |
| Martin County MS4 | 22,809 | 4.1% |
| Troup-Indiantown WCD | 16,502 | 2.9% |
| Hobe St. Lucie Conservancy District | 6,078 | 1.1% |
| St. Lucie County Non-MS4 | 6,072 | 1.1% |
| Port St. Lucie Non-MS4 | 4,413 | 0.8% |
| St. Lucie County MS4 | 4,127 | 0.7% |
| Fort Pierce MS4 | 3,879 | 0.7% |
| Stuart MS4 | 3,107 | 0.6% |
| FDOT District 4 MS4 | 2,596 | 0.5% |
| Tradition CDD* | 1,910 | 0.3% |
| Pal Mar WCD* | 1,322 | 0.2% |
| Okeechobee County* | 974 | 0.2% |
| Turnpike* | 925 | 0.2% |
| Sewall's Point MS4* | 384 | 0.1% |
| Verano CDD* | 63 | 0.0% |
| Copper Creek CDD* | 3 | 0.0% |

Note: de minimus stakeholders are marked with a “*.”

Seven entities each contributed less than 0.5% of the total load for TN and less than 0.5% of the load for TP are considered to be de minimus.  The de minimus entities are Copper Creek CDD, Okeechobee County, Pal Mar WCD, Tradition CDD, Sewall’s Point, Turnpike, and Verano CCD, and combined they contribute approximately 1% of the total load.  The de minimus entities will not be assigned an allocation for either TN or TP for the first phase of the BMAP, and the loads associated with these entities will not be reallocated to the other stakeholders in the basin.

The de minimus status is only for the first BMAP iteration and will be reviewed with each subsequent BMAP cycle.  In future BMAP iterations, TN and TP reductions may be needed from the de minimus entities; therefore, although they do not currently have a reduction responsibility, this does not prevent these entities from having reduction requirements in future BMAPs.  Any actions taken by these entities during this first BMAP iteration that result in TN and/or TP reductions should be documented by those entities for credit against any reduction requirements allocated in subsequent BMAP iterations.

Target Loads per Acre

Allocations to each entity were assigned using a target load per acre for both TN and TP.  These target loads per acre were calculated by determining the percent reduction needed in each sub-watershed to achieve the target nutrient concentrations.  This percent reduction was multiplied by the current anthropogenic load per acre in each sub-watershed, which is the total anthropogenic loading for the sub-watershed from the model divided by the total anthropogenic acres in the sub-watershed.  The current anthropogenic load per acre minus the needed reduction in loading equals the target load per acre for each sub-watershed, as shown in Table 6 and Table 7.

Table 6:  TN Target Loads per Acre

| Category | Basins 4 5 6 | C-23 | C-24 | C-44 S-153 | North Fork | South Fork |
| --- | --- | --- | --- | --- | --- | --- |
| Estimated Concentration (mg/L) | 1.11 | 1.51 | 1.53 | 1.36 | 1.18 | 1.30 |
| Target Concentration (mg/L) | 0.72 | 0.72 | 0.72 | 0.72 | 0.72 | 0.72 |
| Reduction Required (%) | 35% | 52% | 53% | 47% | 39% | 45% |
| Estimated Load Per Acre (lbs/acre) | 5.83 | 5.47 | 8.96 | 6.71 | 4.67 | 6.25 |
| Target Load Per Acre (lbs/acre) | 3.77 | 2.60 | 4.21 | 3.56 | 2.85 | 3.45 |

Table 7:  TP Target Loads per Acre

| Category | Basins 4 5 6 | C-23 | C-24 | C-44 S-153 | North Fork | South Fork |
| --- | --- | --- | --- | --- | --- | --- |
| Estimated Concentration (mg/L) | 0.25 | 0.53 | 0.38 | 0.24 | 0.27 | 0.27 |
| Target Concentration (mg/L) | 0.081 | 0.081 | 0.081 | 0.081 | 0.081 | 0.081 |
| Reduction Required (%) | 68% | 85% | 79% | 66% | 70% | 70% |
| Estimated Load Per Acre (lbs/acre) | 1.32 | 1.76 | 2.14 | 1.29 | 1.06 | 1.33 |
| Target Load Per Acre (lbs/acre) | 0.43 | 0.27 | 0.46 | 0.44 | 0.32 | 0.39 |

Allocations

The TN and TP targets load per acre were then multiplied by each entity’s acreage in each sub-watershed to calculate the allocations.  The TN and TP allocations are shown in Table 8 and Table 9, respectively.

Required Reductions

The difference between the starting loads and allocations for each entity is the required reduction.  The TN and TP required reductions are shown in Table 10 and Table 11, respectively.

Table 8:  TN Allocations by Entity

| Entity | Basins 4 5 6 (lbs/yr) | C-23 (lbs/yr) | C-24 (lbs/yr) | C-44 S-153 (lbs/yr) | North Fork (lbs/yr) | South Fork (lbs/yr) | Total (lbs/yr) | Total (MT/yr) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Agriculture | 9,217 | 220,738 | 267,229 | 234,701 | 11,316 | 62,776 | 805,977 | 365.59 |
| FDOT District 4 | 644 | 796 | 578 | 960 | 2,463 | 2,195 | 7,636 | 3.46 |
| Fort Pierce MS4 | - | - | - | - | 10,572 | - | 10,752 | 4.88 |
| Hobe St. Lucie Conservancy District | - | - | - | 10,497 | - | 6,717 | 17,215 | 7.81 |
| Martin County MS4 | 18,812 | 4,527 | - | 7,941 | 12,488 | 26,812 | 70,581 | 32.02 |
| North St. Lucie River WCD | - | - | 16,953 | - | 92,674 | - | 109,627 | 49.73 |
| Port St. Lucie MS4 | - | 231 | 5,270 | - | 86,636 | - | 92,137 | 41.79 |
| Port St. Lucie Non-MS4 | - | 617 | 26 | - | 10,680 | - | 11,323 | 5.14 |
| St. Lucie County MS4 | - | - | - | - | 11,396 | - | 11,396 | 5.17 |
| St. Lucie County Non-MS4 | - | 1,988 | 9,143 | - | 3,269 | - | 14,399 | 6.53 |
| Stuart MS4 | - | - | - | - | 1,006 | 8,241 | 9,247 | 4.19 |
| Troup-Indiantown WCD | - | - | - | 48,584 | - | - | 48,584 | 22.04 |
| Copper Creek CDD - de minimus | - | - | - | - | - | - | - | - |
| Okeechobee County MS4 - de minimus | - | - | - | - | - | - | - | - |
| Pal Mar WCD - de minimus | - | - | - | - | - | - | - | - |
| Sewall’s Point MS4 - de minimus | - | - | - | - | - | - | - | - |
| Tradition CDD - de minimus | - | - | - | - | - | - | - | - |
| Turnpike - de minimus | - | - | - | - | - | - | - | - |
| Verano CDD - de minimus | - | - | - | - | - | - | - | - |
| Total | 28,673 | 228,898 | 299,199 | 302,684 | 242,500 | 106,741 | 1,208,694 | 548.25 |

Table 9:  TP Allocations by Entity

| Entity | Basins 4 5 6 (lbs/yr) | C-23 (lbs/yr) | C-24 (lbs/yr) | C-44 S-153 (lbs/yr) | North Fork (lbs/yr) | South Fork (lbs/yr) | Total (lbs/yr) | Total (MT/yr) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Agriculture | 1,040 | 22,708 | 29,076 | 28,747 | 1,275 | 7,168 | 90,013 | 40.83 |
| FDOT District 4 | 73 | 82 | 63 | 118 | 278 | 251 | 863 | 0.39 |
| Fort Pierce MS4 | - | - | - | - | 1,191 | - | 1,191 | 0.54 |
| Hobe St. Lucie Conservancy District | - | - | - | 1,286 | - | 767 | 2,053 | 0.93 |
| Martin County MS4 | 2,122 | 466 | - | 973 | 1,407 | 3,061 | 8,029 | 3.64 |
| North St. Lucie River WCD | - | - | 1,845 | - | 10,441 | - | 12,286 | 5.57 |
| Port St. Lucie MS4 | - | 24 | 573 | - | 9,761 | - | 10,358 | 4.70 |
| Port St. Lucie Non-MS4 | - | 63 | 3 | - | 1,203 | - | 1,270 | 0.58 |
| St. Lucie County MS4 | - | - | - | - | 1,284 | - | 1,284 | 0.58 |
| St. Lucie County Non-MS4 | - | 205 | 995 | - | 368 | - | 1,568 | 0.71 |
| Stuart MS4 | - | - | - | - | 113 | 941 | 1,054 | 0.48 |
| Troup-Indiantown WCD | - | - | - | 5,951 | - | - | 5,951 | 2.70 |
| Copper Creek CDD - de minimus | - | - | - | - | - | - | - | - |
| Okeechobee County MS4 - de minimus | - | - | - | - | - | - | - | - |
| Pal Mar WCD - de minimus | - | - | - | - | - | - | - | - |
| Sewall’s Point MS4 - de minimus | - | - | - | - | - | - | - | - |
| Tradition CDD - de minimus | - | - | - | - | - | - | - | - |
| Turnpike - de minimus | - | - | - | - | - | - | - | - |
| Verano CDD - de minimus | - | - | - | - | - | - | - | - |
| Total | 3,235 | 23,547 | 32,571 | 37,074 | 27,322 | 12,187 | 135,920 | 61.65 |

Table 10:  TN Required Reductions by Entity

| Entity | Basins 4 5 6 (lbs/yr) | C-23 (lbs/yr) | C-24 (lbs/yr) | C-44 S-153 (lbs/yr) | North Fork (lbs/yr) | South Fork (lbs/yr) | Total (lbs/yr) | Total (MT/yr) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Agriculture | 7,834 | 249,344 | 307,623 | 223,731 | 13,038 | 63,305 | 864,874 | 392.30 |
| FDOT District 4 | 308 | 713 | 372 | 576 | 1,814 | 1,454 | 5,238 | 2.38 |
| Fort Pierce MS4 | - | - | - | - | 6,469 | - | 6,469 | 2.93 |
| Hobe St. Lucie Conservancy District | - | - | - | 6,984 | - | 4,101 | 11,086 | 5.03 |
| Martin County MS4 | 7,581 | 1,419 | - | 2,834 | 7,318 | 13,611 | 32,763 | 14.85 |
| North St. Lucie River WCD | - | - | 20,298 | - | 67,478 | - | 87,776 | 39.81 |
| Port St. Lucie MS4 | - | 165 | 2,947 | - | 41,362 | - | 44,474 | 20.17 |
| Port St. Lucie Non-MS4 | - | 502 | 32 | - | 8,013 | - | 8,547 | 3.88 |
| St. Lucie County MS4 | - | - | - | - | 6,718 | - | 6,718 | 3.05 |
| St. Lucie County Non-MS4 | - | - | 7,615 | - | 2,141 | - | 9,756 | 4.43 |
| Stuart MS4 | - | - | - | - | 607 | 4,143 | 4,751 | 2.15 |
| Troup-Indiantown WCD | - | - | - | 32,747 | - | - | 32,747 | 14.86 |
| Copper Creek CDD - de minimus | - | - | - | - | - | - | - | - |
| Okeechobee County MS4 - de minimus | - | - | - | - | - | - | - | - |
| Pal Mar WCD - de minimus | - | - | - | - | - | - | - | - |
| Sewall’s Point MS4 - de minimus | - | - | - | - | - | - | - | - |
| Tradition CDD - de minimus | - | - | - | - | - | - | - | - |
| Turnpike - de minimus | - | - | - | - | - | - | - | - |
| Verano CDD - de minimus | - | - | - | - | - | - | - | - |
| Total | 15,723 | 252,143 | 338,887 | 266,873 | 154,960 | 86,614 | 1,115,199 | 505.85 |

Table 11:  TP Required Reductions by Entity

| Entity | Basins 4 5 6 (lbs/yr) | C-23 (lbs/yr) | C-24 (lbs/yr) | C-44 S-153 (lbs/yr) | North Fork (lbs/yr) | South Fork (lbs/yr) | Total (lbs/yr) | Total (MT/yr) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Agriculture | 2,881 | 127,548 | 107,395 | 58,590 | 4,713 | 19,701 | 320,827 | 145.52 |
| FDOT District 4 | 128 | 382 | 163 | 111 | 540 | 408 | 1,732 | 0.79 |
| Fort Pierce MS4 | - | - | - | - | 2,688 | - | 2,688 | 1.22 |
| Hobe St. Lucie Conservancy District | - | - | - | 2,230 | - | 1,796 | 4,025 | 1.83 |
| Martin County MS4 | 3,808 | 1,784 | - | 898 | 2,932 | 5,357 | 14,780 | 6.70 |
| North St. Lucie River WCD | - | - | 7,218 | - | 26,379 | - | 33,598 | 15.24 |
| Port St. Lucie MS4 | - | 115 | 1,620 | - | 18,510 | - | 20,244 | 9.18 |
| Port St. Lucie Non-MS4 | - | 316 | 10 | - | 2,817 | - | 3,143 | 1.43 |
| St. Lucie County MS4 | - | - | - | - | 2,843 | - | 2,843 | 1.29 |
| St. Lucie County Non-MS4 | - | 634 | 2,966 | - | 905 | - | 4,505 | 2.04 |
| Stuart MS4 | - | - | - | - | 266 | 1,786 | 2,052 | 0.93 |
| Troup-Indiantown WCD | - | - | - | 10,551 | - | - | 10,551 | 4.79 |
| Copper Creek CDD - de minimus | - | - | - | - | - | - | - | - |
| Okeechobee County MS4 - de minimus | - | - | - | - | - | - | - | - |
| Pal Mar WCD - de minimus | - | - | - | - | - | - | - | - |
| Sewall’s Point MS4 - de minimus | - | - | - | - | - | - | - | - |
| Tradition CDD - de minimus | - | - | - | - | - | - | - | - |
| Turnpike - de minimus | - | - | - | - | - | - | - | - |
| Verano CDD - de minimus | - | - | - | - | - | - | - | - |
| Total | 6,816 | 130,779 | 119,732 | 72,380 | 62,593 | 29,049 | 420,988 | 190.96 |

Proposed Reductions for the First BMAP Iteration

The BMAP required reductions will be phased over a 15-year period, through three, five-year BMAP iterations.  This phased approach allows reductions to occur to benefit the watershed while using adaptive management principles to evaluate progress towards the total maximum daily load (TMDL).  Based on reductions achieved through projects completed since January 1, 2000, the proposed reductions for this BMAP iteration are 30% of the total required reductions.  The proposed required reductions for the first BMAP iteration for each entity are shown in Table 12 and Table 13.

Table 12:  Proposed TN Reductions for the First BMAP Iteration

| Entity | Total TN Reductions (lbs/yr) | 30% TN Reductions (lbs/yr) | 30% TN Reductions (MT/yr) |
| --- | --- | --- | --- |
| Agriculture | 864,874 | 259,462.29 | 117.69 |
| FDOT District 4 | 5,238 | 1,571.48 | 0.71 |
| Fort Pierce MS4 | 6,469 | 1,940.71 | 0.88 |
| Hobe St. Lucie Conservancy District | 11,086 | 3,325.75 | 1.51 |
| Martin County MS4 | 32,763 | 9,829.05 | 4.46 |
| North St. Lucie River WCD | 87,776 | 26,332.85 | 11.94 |
| Port St. Lucie MS4 | 44,474 | 13,342.26 | 6.05 |
| Port St. Lucie Non-MS4 | 8,547 | 2,564.12 | 1.16 |
| St. Lucie County MS4 | 6,718 | 2,015.47 | 0.91 |
| St. Lucie County Non-MS4 | 9,756 | 2,926.69 | 1.33 |
| Stuart MS4 | 4,751 | 1,425.15 | 0.65 |
| Troup-Indiantown WCD | 32,747 |  |  |
| Copper Creek CDD - de minimus | - | - | - |
| Okeechobee County MS4 - de minimus | - | - | - |
| Pal Mar WCD - de minimus | - | - | - |
| Sewall’s Point MS4 - de minimus | - | - | - |
| Tradition CDD - de minimus | - | - | - |
| Turnpike - de minimus | - | - | - |
| Verano CDD - de minimus | - | - | - |
| Total | 1,384,999 | 334,559.85 | 151.75 |

Table 13:  Proposed TP Reductions for the First BMAP Iteration

| Entity | Total TP Reductions (lbs/yr) | 30% TP Reductions (lbs/yr) | 30% TP Reductions (MT/yr) |
| --- | --- | --- | --- |
| Agriculture | 320,827 | 96,248.10 | 43.66 |
| FDOT District 4 | 1,732 | 519.74 | 0.24 |
| Fort Pierce MS4 | 2,688 | 806.37 | 0.37 |
| Hobe St. Lucie Conservancy District | 4,025 | 1,207.58 | 0.55 |
| Martin County MS4 | 14,780 | 4,433.92 | 2.01 |
| North St. Lucie River WCD | 33,598 | 10,079.30 | 4.57 |
| Port St. Lucie MS4 | 20,244 | 6,073.26 | 2.75 |
| Port St. Lucie Non-MS4 | 3,143 | 943.04 | 0.43 |
| St. Lucie County MS4 | 2,843 | 852.92 | 0.39 |
| St. Lucie County Non-MS4 | 4,505 | 1,351.37 | 0.61 |
| Stuart MS4 | 2,052 | 615.65 | 0.28 |
| Troup-Indiantown WCD | 10,551 | 3,165.26 | 1.44 |
| Copper Creek CDD - de minimus | - | - | - |
| Okeechobee County MS4 - de minimus | - | - | - |
| Pal Mar WCD - de minimus | - | - | - |
| Sewall’s Point MS4 - de minimus | - | - | - |
| Tradition CDD - de minimus | - | - | - |
| Turnpike - de minimus | - | - | - |
| Verano CDD - de minimus | - | - | - |
| Total | 420,988 | 126,296.50 | 57.29 |