#### Septic System Removal

The estimated nutrient loads from the septic systems that will travel through the groundwater and intersect with a surface waterbody (tributaries, canals, and the lagoon itself) were estimated using typical septic system effluent concentrations and decay rates from USEPA (2002) (Table 12). The estimated travel times based on the distance from the septic system to a waterbody are shown in Table 13, and is based on an interpretation of the results from a recent study in the City of Port St. Lucie by Sayemuzzaman and Ye 2015. The concentration of each parameter for each buffer zone was calculated using the effluent concentration and decay rates in Table 12 and the travel times in Table 13. The concentrations used in the estimates for this plan are shown in Table 14.

Table 12: Septic System Effluent Concentrations and Decay Rates

| Parameter | Effluent Concentration (mg/L) | Decay Rate (1/day) |
| --- | --- | --- |
| TN | 70 | 0.1 |
| Organic N | 0.458 | 0.1 |
| Ammonia | 10.5 | 0.1 |
| Nitrate + Nitrite | 59.3 | 0.0011 |
| Organic P* | 0.3 | 0.014 |
| Orthophosphate* | 0 | 0.014 |

* Assumes that 90% of phosphorus is sorbed to sediment.

Table 13: Travel Time Based on Distance from Septic System to Waterbody

| Buffer Zone | Travel Distance (yards) | Average Velocity (yards/day) | Average Travel Time (days) | Average Travel Time (years) |
| --- | --- | --- | --- | --- |
| 1 | <55 | 0.199 | 137.6 | 0.4 |
| 2 | 55-219 | 0.138 | 1,385.7 | 3.8 |
| 3 | >219 | 0.066 | 9,641.0 | 26.4 |

Table 14: Parameter Concentrations from Each Buffer Zone

| Parameter | Buffer Zone 1 Concentration (mg/L) | Buffer Zone 2 Concentration (mg/L) | Buffer Zone 3 Concentration (mg/L) |
| --- | --- | --- | --- |
| Organic N | 0.000 | 0.000 | 0.000 |
| Ammonia | 0.000 | 0.000 | 0.000 |
| Nitrate + Nitrite | 50.971 | 12.914 | 0.001 |
| Organic P | 0.044 | 0.000 | 0.000 |
| Orthophosphate | 0.000 | 0.000 | 0.000 |

The cost for connection of all the septic systems in the County within the IRL watershed would be approximately $1.2 billion (see Table 15). Therefore, this plan focuses on the locations where reductions through septic system removal are the most cost-effective.

Table 15: Cost to Remove Septic Systems Based on Distance from a Surface Waterbody

| Septic System Distance from Surface Water | Number of Septic Systems | TN (lbs/yr/system) | TN (lbs/yr) | Cost/System to Connect | Total Cost | Cost/lb/yr of TN |
| --- | --- | --- | --- | --- | --- | --- |
| Less than 55 yards | 15,090 | 27.095 | 408,863 | $20,000 | $301,800,000 | $738 |
| Between 55 and 219 yards | 25,987 | 6.865 | 178,395 | $20,000 | $519,740,000 | $2,913 |
| Greater than 219 yards | 18,361 | 0.001 | 10 | $20,000 | $367,220,000 | $37,624,010 |
| Total in IRL Basin | 59,438 | N/A | 587,268 | $20,000 | $1,188,760,000 | $2,024 (average) |