Upper Ocklawaha Basin Management Action Plan

Field Descriptions for UOB_final_allocate GIS file.

Data in this file covers the area of the Harris Chain of Lakes including Lake Harris, Lake Carlton, Lake Beauclair, Lake Dora, Lake Eustis, Trout Lake, Lake Yale, and Lake Griffin.  It was created from 1995 and 2009 land use data and data supplied by St. Johns River Water Management District for location of restoration or management areas, sprayfields, and sub-basin delineations. Florida Department of Transportation provided the locations of roadways and stormwater treatment areas managed by them.  City boundary data were supplied by Lake County Public Works and are current as of December 2015.

- F_1995: FLUCC code for the 1995 land use.

- F_95_Descr: General Description of 1995 land use based on landuse categories used in the Harris Chain of Lakes TMDL.

- F_2009: FLUCC code for the 2009 land use.

- F_09_Descr: General Description of 2009 land use based on landuse categories used in the Harris Chain of Lakes TMDL.

- Change_95: Change in land use comparing 1995:2009.

- Name: Wastewater treatment facility or sprayfield name.

- Facility_T: Type of wastewater treatment facility.

- Contrib_su: Lake sub-basin where sprayfield or wastewater treatment plant is located.

- Contribute: Name of lake basin where treatment plant or sprayfield is located.

- Contribute_1: Numbering for major lake watersheds.

  - 0=lake

  - 1=Lake Beauclair watershed

  - 2=Lake Dora watershed

  - 3=Lake Harris and Little Lake Harris watershed

  - 4=Lake Eustis watershed

  - 5=Lake Griffin

  - 6=Lake Yale watershed

- Cont_Lake: Name of major to which loading is contributed.  Does not distinguish Trout Lake and Lake Carlton.

- Basin_ID: Lake Sub-basin number. Numbers followed by L designate only the lake surface area.  Drainage directly around the lake is subdivided as indicated by letters following numbers.

- Subbas_ID: Lake subbasin ID number.

- MGMT_Area: Name of restoration or management area or muck farm.

- Source1995: Land use as a source category used to track major shifts in land use. Similar to F_95_Descr, but identifies land included as muck farms and sprayfields.

- Source2009: Land use as a source category used to track major shifts in land use. Similar F_09_Descr, but identifies land included as muck farms and sprayfields.

- Urban1995: Aggregated categories of land use to allow large scale comparison in land use changes.

  - Wetlands and Water= Open water or wetlands

  - Forest/Rangeland=Forest, range, or open land

  - Pasture, Cropland, Other agriculture, Tree crops, Feeding operations=Agriculture activities

  - High density commercial, low density commercial, Institutional, Low density residential, Medium density residential, High density residential, mining=Urban

  - Industrial=Industrial

  - Open land/recreation=Forest, range, or open land

  - Muck farm=Muck farm or restoration area

  - Sprayfield=Sprayfield

- Urban2009: Aggregated categories of landuse to allow large scale comparison in landuse changes.

  - Wetlands and Water= Open water or wetlands

  - Forest/Rangeland=Forest, range, or open land

  - Pasture, Cropland, Other agriculture, Tree crops, Feeding operations=Agriculture activities

  - High density commercial, low density commercial, Institutional, Low density residential, Medium density residential, High density residential, mining=Urban

  - Industrial=Industrial

  - Open land/recreation=Forest, range, or open land

  - Muck farm=Muck farm or restoration area

  - Sprayfield=Sprayfield

- Model_2015: Aggregated category of landuse used for calculating reductions.

| Developed Land Use-High | High density residential, high density commercial, and industrial. |
| --- | --- |
| Developed Land Use-Medium | Medium density residential and low density commercial |
| Developed Land Use- Low | Low density residential, mining, sprayfields, and open land/recreation |

- Name: FDOT name of parcel. May be same as route number.

- Route: Major highway name listed by FDOT or Turnpike Authority.

- Lake_two: Name of lake watershed, including Trout and Carlton.

- City: Name of jurisdiction.

- County: County name

- Acreage: Area in acres.

- SQMILE: Area in square miles.

- Hectare: Area in hectares.