Exhibit D

Department of Environmental Protection

Quality Assurance Requirements for Grants

Standard Field & Lab Services

- General Requirements and Definitions

  - As applicable to the Scope of Services (i.e., scope of work, or grant work plan) described in the grant, the sampling, field testing and laboratory analyses performed under this Grant shall conform to the requirements set forth in Chapter 62-160, Florida Administrative Code (F.A.C.), and “Requirements for Field and Analytical Work Performed for the Department of Environmental Protection under Contract” (DEP-QA-002/02), February 2002.

  - Hereinafter, “DEP” or “Department” refers to the Florida Department of Environmental Protection.

  - “Grantee” shall refer to the grantee, subcontractors, subgrantees, or any entity procured to conduct work under the Grant.

  - “Sample” and “sampling” refers to samples that shall be either collected or analyzed under the terms of this Grant.

- Requirements for Laboratories

  - All applicable laboratory testing activities shall be performed by laboratories certified by the Florida Department of Health Environmental Laboratory Certification Program (DoH ELCP) for all applicable matrix/method/analyte combinations to be measured for this Grant.  Laboratory certification requirements are described in Rule 62-160.300, F.A.C. Certification is not required for laboratory tests outside of the scope of DoH ELCP accreditation as determined according to Paragraph 62-160.300(5)(c), F.A.C.

  - For samples collected from a non-potable water matrix, the certification requirement is met if the laboratory is certified for the contracted analyte(s) in at least one method utilizing an analytical technology appropriate for the Grant, as determined by the Department according to Paragraph 62-160.300(1)(c), F.A.C.

  - If the laboratory is not certified for some or all proposed test measurements, the laboratory shall apply for certification within one month of Grant execution.  The laboratory shall attempt to become fully certified for all applicable matrix/method/analyte combinations to be performed for the Grant by maintaining active coordination with the DoH ELCP throughout the application process.  Regardless of when the laboratory receives certification, the laboratory shall implement all applicable standards of the National Environmental Laboratory Accreditation Conference (NELAC 2003 Quality Systems standards, as adopted) upon Grant execution.

  - Laboratories shall maintain certification as specified in item 2.a above during the life of the Grant.  Should certification for an analyte or test method be lost, all affected tests shall be immediately sub-contracted to a laboratory with current DoH ELCP certification in the appropriate matrix/method/analyte combination(s).  The Grantee shall notify the DEP Grant Manager in writing before any change to a sub-contracted laboratory is made.

  - The DoH ELCP certificate number (certified laboratory identification number) for each contracted (and sub-contracted) laboratory shall be listed in the required Grant Quality Assurance (QA) Plan (see Section 6 below) in association with the analytical tests to be performed by each laboratory analyzing samples for the Grant.

  - Each certified laboratory analyzing contracted samples shall ensure that an acceptable demonstration of capability (DOC) is performed as described in the 2003 NELAC Quality Systems standards (NELAC 2003, Section 5.5.4.2.2 and Appendix C).  In addition, each certified laboratory that performs any of the proposed matrix/method/analyte combination(s) approved for the Grant shall have the requisite DOC documentation and supporting laboratory records on file for the applicable combinations.  The DOCs performed shall meet the requirements for precision, accuracy, method detection limit (MDL) and/or practical quantitation limit (PQL), as specified in each applicable laboratory test method, Standard Operating Procedure (SOP) or Quality Manual, or as listed in the Grant QA Plan (Section 6, below).  Alternative limits for detection and quantitation other than MDL and PQL shall be determined, if applicable to the laboratory. DOCs performed for the contracted analytes shall include any modifications to the test method or SOP that have been approved by DEP according to Subsection 62-160.330(3), F.A.C., if applicable. If requested by the Department, documentation that supports the DOC for a specified analyte and test method shall be made available for review.

  - The contracted (and/or subcontracted) laboratory shall report PQLs and MDLs or other specified limits of detection and quantitation with the results of sample analyses. MDLs and/or PQLs shall only be required for test methods that are technically amenable to the determination of MDLs and/or PQLs. For those test methods where the determination of MDLs and/or PQLs are not technically feasible, the laboratory shall report a value or increment representing the lower limit of the working range of the test method, however determined by the laboratory. The laboratory shall indicate whether the reported limit represents a limit of detection or quantitation. In all cases, limits of detection and quantitation other than MDLs and PQLs shall be explicitly defined and evaluated by the laboratory. All limits shall be as listed in the applicable laboratory test method, SOP or Quality Manual, or as listed in the Grant QA Plan (Section 6, below). The reported MDLs and PQLs (or other limits per above) shall meet the analytical sensitivity and quantitation objectives for the Grant.

  - Additional laboratory quality control expectations:

    - The selected laboratory test methods listed in the QA Plan shall provide results that meet applicable Grant data quality objectives.

    - All laboratory testing procedures shall follow the analytical methods as approved in the Grant QA Plan (see Section 6).

    - The laboratory shall adhere to the quality control requirements specified in the laboratory test methods and this Exhibit.

    - The laboratory shall calculate all sample results according to the procedures specified in the analytical test methods approved in the Grant QA Plan.

- Field Activities

  - All sample collection and field testing activities shall be performed in accordance with the Department’s “Standard Operating Procedures for Field Activities” (DEP-SOP-001/01, January, 2017).  The specific standard operating procedures (SOPs) to be used for this Grant shall be cited in the Grant QA Plan (see Section 6).

  - Field-Generated Quality Control (QC) Blanks are defined in DEP SOP FQ 1000 (subparts FQ 1211 – FQ 1214) and shall be composed and analyzed for sample collection activities associated with this Grant according to the requirements of part FQ 1230 (sections 1. – 2.3.1), DEP SOP FS 2100 (Part FS 2110, sec. 2.1.1.2) and/or DEP SOP FS 2400 (Part FS 2430, sec. 2.1.1.2), as applicable to the analytes and matrices to be collected using the sampling equipment specified in the Grant QA Plan (see Section 6 below).

    - If an analyte detected in the sample is also found in any field-generated QC blank that is associated with the sample, the Grantee shall investigate and attempt to determine the cause of the QC blank contamination.  If any contracted sample results are qualified as in (ii) below, the outcome of this investigation shall be reported to the DEP Grant Manager and shall include a discussion of the corrective measures taken to minimize future occurrences of QC blank contamination associated with the collection of samples for this Grant.

    - If an analyte detected in the sample is also found in any field-generated QC blank that is associated with the sample, the analytical result reported for the affected sample shall be qualified as an estimated value, unless the analyte concentration in the blank is less than or equal to 10% of the reported sample concentration. The “G” data qualifier code shall be reported with the sample result for any blank concentration exceeding the above “10%” criterion for the affected analyte (see Table 1, Chapter 62-160, F.A.C.).

- Reporting, Documentation and Records Retention

  - Reporting, Documentation and Records Retention shall be in compliance with the provisions specified in the DEP Grant.

  - Deliverable requirements for Reporting are further specified in DEP Grant Scope of Services.

  - All laboratory and field records described or listed in Rules 62-160.240 and 62-160.340, F.A.C., shall be retained for a minimum of five years after the generation (or completion) of the records applicable to the Grant. Longer retention times as specified in the Grant shall supersede.

  - All field and laboratory data and supporting information shall be reported for this Grant according to applicable requirements in Subsections 62-160.340(3) – (8), F.A.C.

  - Any other documentation and reports associated with work performed for this Grant shall be likewise retained and shall include relevant information for the procedures described in Sections 2 and 3, above.

  - Any documentation or reports specifically identified in this Grant as deliverable work products shall be retained as in 4.a., above.

  - All field and laboratory records that are associated with work performed under this Grant shall be organized so that any information can be quickly and easily retrieved for inspection, copying or distribution.

  - The Department reserves the right to request some or all of the laboratory or field information in an electronic format specified by the Department, as specified in the Grant and/or Scope of Services, and/or as described in the approved Grant QA Plan (see Section 6). Also, see Subsection k., below.

  - Any certified laboratory reports issued for contracted sample analyses using certified methods shall be generated in accordance with NELAC Quality Systems requirements (NELAC 2003, section 5.5.10).

  - Upon request by the Department Grant Manager or as required by the Grant, copies of the original laboratory reports shall be submitted to the Department Grant Manager.

  - In addition to any reports of sample results provided per Grant deliverable requirements and Subsections b., e., f. and g., above, the Grantee shall submit any of the laboratory information and/or records associated with the contracted analyses as described in this section (Section 4) upon request by DEP, including any of the following:

#### Laboratory sample identification (ID) and associated Field ID

- Analytical/test method

- Parameter/analyte name

- Analytical result (including dilution factor)

- Result unit

- Applicable DEP Data Qualifier Codes per Table 1 of Rule 62-160.700, F.A.C.

- Result comment(s) to include corrective/preventive actions taken for any failed QC measure (e.g., QC sample result, calibration failure) or other problem related to the analysis of the samples

- Date and time of sample preparation (if applicable)

- Date and time of sample analysis

- Results of laboratory verification of field preservation of received samples

- Sample matrix

- DoH ELCP certification number for each laboratory (must be associated with the test results generated by each laboratory analyzing samples under this Grant)

- MDL, Limit of Detection (LOD) or other defined limit of detection

- PQL, Limit of Quantitation (LOQ) or other defined limit of quantification

- Field and laboratory QC blank results:

#### Laboratory QC blank analysis results as required by the method and the NELAC Quality Systems standards (e.g., method blank)

#### Results for trip blanks, field blanks and equipment blanks, as applicable to the project and as specified in the QA Plan (see Section 6)

- Results for field duplicates (or replicates)

- Results for other QC and calibration verification results, as applicable to the specific test methods used for the contracted analyses:

- Results of sample matrix spikes, laboratory duplicates or matrix spike duplicates

- Results of surrogate spike analyses

- Results of laboratory control samples (LCS)

- Results of calibration verifications

- Acceptance criteria used to evaluate each reported quality control measure

  - Unequivocal documentation links between each reported laboratory quality control measure (e.g., QC blanks, matrix spikes, LCS, duplicates, calibration verification) and the associated sample result(s) shall be maintained for all contracted analyses.

  - In addition to any field information provided per Grant deliverable requirements, and Subsections b., e., f. and g., above, the Grantee shall submit any of the field information and/or records associated with the contracted samples as described in this section (Section 4) upon request by DEP, including any of the following:

- Site name and location information

- Field ID for each sample container and the associated analytes (test methods) for which the container was collected

- Date and time of sample collection

- Sample collection depth, if applicable

- Sample collection method identified by the DEP SOP number, where applicable

- If performed, indicate samples that were filtered

- Field test measurement results:

  - DEP SOP number (FT-series), where applicable

  - Parameter name

  - Result

  - Result unit

  - Applicable Data Qualifier Codes per Table 1 of Rule 62-160.700, F.A.C.

- Narrative comments providing explanations, descriptions and/or discussions of: field conditions impacting QC for sample collections, unacceptable field measurements, field-testing meter calibration verification failures, or other problems related to the sampling event, and corrective/preventive actions taken for the items noted (e.g., for blank contamination or meter calibration failure).

  - The Department reserves the right to request some or all of the laboratory or field information in a format as specified in the Grant and/or Scope of Services, and/or as described in the approved QA Plan (see Section 6).

- Audits

  - Technical Audits by the Department – Pursuant to Rule 62-160.650, F.A.C., the Department may conduct audits of field and laboratory activities. In addition to allowing Department representatives to conduct onsite audits of contracted work in the field or at Grantee facilities, upon request by the Department, field and laboratory records pertinent to the contracted research as described per Section 4, above, shall be provided by the Grantee.  If an audit by the Department results in a determination that the reported data are not usable for the purpose(s) of the Grant, do not meet the data quality objectives specified by the Grant, do not meet other applicable Department criteria described in the Grant, its exhibits, the QA Plan (see Section 6) or these QA Requirements, do not meet applicable data validation criteria outlined in Rule 62-160.670, F.A.C., or are not otherwise suitable for the intended use of the data (however applicable), the DEP Grant Manager shall pursue remedies available to the Department pursuant to the terms of the Grant.

  - Planning Review Technical Audits –

    - Initial:  The Grantee shall review the Grant QA Plan (see Section 6) relative to the completed field and laboratory activities to determine if data quality objectives are being met, identify any improvements to be made to project activities, and refine the sampling and/or analytical design or schedule, if applicable.  A summary of the review, including any corrective action plans or amendments to the Grant QA Plan, shall be sent to the DEP Grant Manager, and a copy of all submitted documents shall be maintained with the permanent project records.

    - Ongoing:  Planning reviews as described in subsection (i) above shall occur after the initial planning review audit for the remainder of the Grant, as specified in the Scope of Services.

    - Statements of Usability: Initial and ongoing Planning Review Technical Audits described in (i) and (ii) above shall include statements about data usability relative to the Grant data quality objectives and any data quality indicators that may be specified in the Grant, its exhibits, the QA Plan (see Section 6), or these QA Requirements. This usability determination shall take into account all applicable data quality acceptance and usability criteria for quality control and environmental sample results for the Grant, as specified in the procedures, test methods, QA Plan, Quality Manual(s), other Grant exhibits, or these QA Requirements.

    - Initial and ongoing reviews and summaries shall be completed within timeframes specified in the Grant Scope of Services.

  - Quality Systems Audits – The Grantee shall ensure that any required laboratory and field quality system audits are performed according to the respective Quality Manuals or other relevant internal quality assurance documents for each entity performing work under the Grant.  The results of these audits shall be documented in the Grantee’s records. Copies of the above audit reports or results shall be provided to the DEP Grant Manager upon request. Copies of audit records for internal audits conducted per DEP SOP FA 1000 (subpart FA 4200) or NELAC Quality Systems requirements (NELAC 2003, section 5.4.13) shall be similarly provided upon request.

- Quality Assurance Plan

  - The Grantee shall submit a Quality Assurance (QA) Plan for the Grant to the DEP Grant Manager, as specified in the Grant Scope of Services. The Standard QA Plan Template may be used to capture all required elements in the Plan.

  - The DEP Grant number shall appear on the title page of the submitted QA Plan.  The Department shall review and either approve the QA Plan or provide comments to the Grantee as to why the QA Plan is not approved, within timeframes specified in the Grant Scope of Services.  If further revisions are needed, the Grantee shall respond within timeframes specified in the Grant Scope of Services.  The Department shall respond to all revisions to the QA Plan within timeframes specified in the Grant Scope of Services.

  - Work may not begin for specific Grant tasks until approval (or conditional approval) has been received by the Grantee from the DEP Grant Manager. Sampling and analysis for the Grant may not begin until the QA Plan has been approved (or conditionally approved).

  - Once approved, the Grantee(s) shall follow the procedures and methods described in the approved QA Plan and any other relevant quality assurance documents, including, but not limited to:

- Ensuring that all stated quality control measures are collected, analyzed and evaluated for acceptability;

- Using only the protocols approved in the QA Plan; and

- Using only the equipment approved in the QA Plan.

##### If any significant changes occur in sampling project design, project analyte list, procedures or test methods, equipment, or key personnel, the Grantee shall submit appropriate revisions of the QA Plan to the DEP Grant Manager for review, within timeframes specified in the Grant Scope of Services.  The proposed revisions may not be implemented until they have been approved (or conditionally approved) by the DEP Grant Manager, as documented through written or electronic correspondence.