| SOP Collection | DEP SOP | Edited Sections | Types of Change | Topic Area | Summary of Edits (Effective 4/16/18) |
| --- | --- | --- | --- | --- | --- |
| SOP 001/01 | FA 1000 | all | Clarification | General | Updated DEP program names that have changed. |
| SOP 001/01 | FA 1000 | Appendix: Audit Checklists | Change | Audit Checklists | Updates were made to various audit checklists to correspond with SOP revisions. Added statements that pencil is allowed on waterproof paper, time must be recorded for analytes with a hold time specified in hours, thermal preservation must be documented, specified blank collection for autosamplers, the slope for pH only needs to be checked weekly if applicable to the instrument type. |
| SOP 001/01 | FA 1000 | Appendix: Habitat Assessment Training and Evaluation Checklists | Change | Habitat Assessment Training | This section was deleted because it is redundant with Form FD 9000-34. |
| SOP 001/01 | FA 1000 | FA 1310 | Change | SOP Format List | The name of the flow and continuous monitoring SOPs were updated. |
| SOP 001/01 | FA 1000 | FA 2100, FA 2200 | Change | Alternative Field Methods | FA 2100 - FA 2240: Changes in definitions and procedures for New, Alternative, and Modified field methods to correspond with changes in QA Rule. New method approval process removed, and modified method procedures included. Definitions and procedures for Alternative and Modified field method application and approval expanded. Expanded instructions for applying for an alternative or modified procedure, and expanded acceptance criteria for approval of alternative or modified methods. Replaced "Project-Specific" procedures with "Limited Use" procedures. Added clarification that the number of participants in a statewide use study will depend on the statistical robustness needed and should be determined in collaboration with the department. SOP FS 7000 removed from list of SOPs that can’t have alternatives/modifications. |
| SOP 001/01 | FA 1000 | FA 4200 | Clarification | Audits | Added clarification about the frequency of internal audits. |
| SOP 001/01 | FA 1000 | FA 5720 | Change | Habitat Assessment | The reference to the appendix replaced with a reference to the form FD 9000-34. Form FD 9000-34 was added as a required document to be submitted when testing for habitat assessment. |
| SOP 001/01 | FC 1000 | FC 1000 section 1.4 | Change | Blanks in Toxicity Tests | Added language for determining effectiveness of cleaning for toxicity tests. |
| SOP 001/01 | FC 1000 | Table FC 1000-2 | Change | Cleaning for Toxicity Tests | Removed sodium bicarbonate from the cleaning steps for toxicity to correspond with EPA guidance. |
| SOP 001/01 | FD 1000 | FD 4200 | Change | Documentation for Flow | Added documentation requirements for use of SOP FT 1800 (flow) |
| SOP 001/01 | FD 1000 | FD 5000 | Change | Sampling time documentation | Added requirement to document sample time for all samples if results will be submitted to DEP WIN database. |
| SOP 001/01 | FD 1000 | FD 5000, sec. 1 | Clarification | Unique ID | Clarified requirement for unique field ID, changed requirement for documentation of sample time for short holds from 48 to 72 hours. |
| SOP 001/01 | FD 1000 | FD 5100 | Change | Documentation | Added the method of collection to the required documentation |
| SOP 001/01 | FD 1000 | Various parts of FD 5300 | Clarification | Biological sample preservation | Clarified that preservation should be documented for biological samples |
| SOP 001/01 | FD 1000 | Form FD 9000-1 | Clarification | BioRecon Field Sheet | Revised station information fields of the form to be consistent with other forms. |
| SOP 001/01 | FD 1000 | Form FD 9000-34 | Change | HA Training Checklist | Combined the Training and Evaluation checklists; Reduced the required number of training sites from 12 to 9; Removed requirement for boat site training. Evaluation site requirement is emphasized. |
| SOP 001/01 | FD 1000 | Form FD 9000-35 | Change | SCI Training Checklist | Combined the Training and Evaluation checklists; Reduced the required number of training sites from 12 to 9; Removed requirement for boat site training. Evaluation site requirement is emphasized. |
| SOP 001/01 | FM 1000 | Table FM 1000-1 | Change | Data Qualifier Table | Data qualifier table removed from FM 1000 because it duplicated the Table 1 in Rule 62-160.700, F.A.C. |
| SOP 001/01 | FQ 1000 | FQ 1000 | Clarification | General | Added reference to other SOPs |
| SOP 001/01 | FQ 1000 | FQ 1000, sec. 1 | Clarification | Blanks | Provides examples of tests within analytical groups that do or do not require blank collection, and references Table FA 1000-2 for more details. |
| SOP 001/01 | FQ 1000 | FQ 1200, sec. 4 | Clarification | Sampling operations | Specifies VOC blanks in each cooler where there are VOC samples. |
| SOP 001/01 | FQ 1000 | FQ 1212 section 4.1 | Clarification | Field-Cleaned Equipment Blanks | Equipment blank rinsed with analyte free water 3 times, not site water. |
| SOP 001/01 | FQ 1000 | FQ 1213, sec. 5 | Clarification | Trip Blank | States that all trip blanks must be analyzed by water sample analytical techniques, even the sample collected matrix is non-aqueous. |
| SOP 001/01 | FQ 1000 | FQ 1230, sec. 2.1.3 | Clarification | Mandatory Field Quality Control | Addition and specification of autosampler equipment blank requirements. |
| SOP 001/01 | FS 1000 | FS 1006 Section 4.5 | Clarification | pH Checks | Clarified general language in the guidance for checking pH adjusted preservation frequencies longer than monthly |
| SOP 001/01 | FS 1000 | FS 1009 section 1.3 | Clarification | Field Documentation | Updated the reference for the required and optional documentation forms to FD 9000 |
| SOP 001/01 | FS 1000 | Table FS 1000-1 and Table FS 1000-2 | Clarification | Acceptable construction material for all analyte groups | Added a footnote that states that specific limitations for construction material outlined in Tables FS 1000-3, FS 1000-4, FS 1000-5, and FS 1000-6 supersede the general consideration for an analyte group given in Tables FS 1000-1 and FS 1000-2 |
| SOP 001/01 | FS 1000 | Table FS 1000-11 | Change | SPLP Sample Preservation/HT | Changed the holding time of Volatiles in accordance with DEP Division of Waste Management requirements. |
| SOP 001/01 | FS 1000 | Table FS 1000-2 and Table FS 1000-3 | Clarification | Materials | Clarification that where materials specify Teflon it includes other fluoropolymers. |
| SOP 001/01 | FS 1000 | Table FS 1000-3 | Change | Housing Material for submersible pumps | Added allowances for Polyethylene and polypropylene, and for PVC if permanently installed |
| SOP 001/01 | FS 1000 | Table FS 1000-3 | Change | Surface Water Filtration | Deleted mistaken requirement to use in-line filter with aeration precautions. |
| SOP 001/01 | FS 1000 | Table FS 1000-5 | Change | Sample Containers & Preservation | Added footnote to container for cyanotoxins that states it should be amber glass. |
| SOP 001/01 | FS 1000 | Table FS 1000-7 | Change | All references within the table | Updated to be reference EPA 5035A (SW-846 validated methods) |
| SOP 001/01 | FS 1000 | Table FS 1000-8 | Clarification | Sample pretreatment and preservation | Clarification that sample pretreatment should be conducted before preservation |
| SOP 001/01 | FS 2000 | FS 2000, Section 1.2.1.3 | Clarification | FL-PRO | Added FL-PRO to list of parameters that must be collected as grab samples. |
| SOP 001/01 | FS 2000 | FS 2001, sec. 3; FS 2003, sec. 3; FS 2005 sec. 3; FS 2009 sec. 3 | Clarification | Sample pretreatment and preservation | Clarification that any sample pretreatment should be conducted before acid preservation |
| SOP 001/01 | FS 2000 | FS 2001, sec. 4 | Clarification | pH Checks | Clarified general language in the guidance for checking pH adjusted preservation frequencies longer than monthly |
| SOP 001/01 | FS 2000 | FS 2004, sec. 2 | Clarification | VOC sample collection | Added clarification that VOCs could only be collected as grab samples, and specifies to collect grab samples over time if composites are required, consistent with FS 2006 |
| SOP 001/01 | FS 2000 | FS 2005, sec. 2.4.3 and 2.6.1 | Clarification | Bacteria sampling | Updated the language about disinfection of sample equipment or sampling port to be consistent between these two sections |
| SOP 001/01 | FS 2000 | FS 2005, sec. 4.2 | Change | Holding Time | Changed when the holding time ends from when placed in growth media to when placed in incubator or water bath |
| SOP 001/01 | FS 2000 | FS 2006 Title & sec. 1.1 | Clarification | Hydrocarbon/Oil & Grease sampling | Added FL-PRO as a test to be included in this section with oil and grease and TRPHs |
| SOP 001/01 | FS 2000 | FS 2009, sec. 2 | Clarification | Cyanide Sampling | Added exclusion for use of automatic samplers for cyanide collection, to be consistent with requirements in FS 2000, sec. 1.2.1.3 |
| SOP 001/01 | FS 2100 | FS 2110, sec. 2 | Clarification | VOC Collection with Autosamplers | Updated to correspond with language in FS 2000 that composite samples are not to be collected with autosampler for parameters listed |
| SOP 001/01 | FS 2100 | FS 2110, sec. 2.1.1.2 | Clarification | Equipment Blanks for Autosamplers | Provided language on frequency of blank collection when tubing is not changed at every sampling event. |
| SOP 001/01 | FS 2200 | FS 2211, sections 4-5 | Clarification | Groundwater Purging | Added guidance for finding well depth and calculating volume. |
| SOP 001/01 | FS 2200 | FS 2212, sections 3.5-3.6 | Clarification | Groundwater Purging | Added requirement to monitor all 5 stabilization criteria when using alternative DO and turbidity criteria. Added a statement to record stabilization just before sampling when purging more than 5 well or screen volumes. |
| SOP 001/01 | FS 2200 | FS 2213 | Clarification | Groundwater Purging | Clarified instructions for pump and tubing decontamination in section 2.1. |
| SOP 001/01 | FS 2200 | FS 2215 | Clarification | Groundwater Purging | Edited 1.6, 2.3, 2.6, to be consistent with other SOPs. Made the flow for VOCs the same across SOPs, between 100 and 400 mL/minute. |
| SOP 001/01 | FS 2200 | FS 2217 | Change | Groundwater Purging | Added a new SOP part providing recommendations for purging and sampling artesian (free-flowing) wells. |
| SOP 001/01 | FS 2200 | FS 2220 | Clarification | Groundwater Purging | Fixed the inconsistencies of allowed tubing types. Added Teflon as allowed material for submersible pumps to match FS 1000-3. |
| SOP 001/01 | FS 2200 | Various | Clarification | Equipment for Groundwater Sampling | Clarification that where materials specify Teflon it includes other fluoropolymers. |
| SOP 001/01 | FS 2300 | FS 2310, sec. 1.1.3; FS 2320, sec. 3 | Clarification | Use of Disinfectants | Updated language to be consistent FS 2005 section 2.4.3.1 |
| SOP 001/01 | FS 2400 | FS 2410, sec. 2.2.3, & FS 2430, sec. 2.3.1.1 | Clarification | Preservation temperatures | Replaced specific temperature for sample preservation with more general language that states sample must be cooled |
| SOP 001/01 | FS 2400 | FS 2430, sec. 2.1.1.2 | Clarification | Equipment Blanks for autosamplers | Provided language on frequency of blank collection when tubing is not changed at every sampling event. |
| SOP 001/01 | FS 3000 | FS 3000, sec. 5 & sec. 5.3 | Change | EPA Method number | Updated EPA Method 5035 references to 5035A (SW-846 validated methods) |
| SOP 001/01 | FS 3000 | FS 3000, sec. 5.3 | Change | Sample preservation | Updated sample preservation methods to be consistent with changes in EPA Method 5035A. |
| SOP 001/01 | FS 3000 | FS 3100, sec. 6; FS 3220, sec. 4.9, 5.8, 6.12, &7.7 | Clarification | Sample preservation temperature | Clarification that samples should be stored at the required preservation temperature. |
| SOP 001/01 | FS 7000 | FS 7100, sec. 3.2; FS 7110, sec. 3.5; FS 7210, sec. 3.1; FS 7220, sec. 3; FS 7240, sec. 3.5 | Change | Algal Sample Preservation | Changed the requirement for time to chemical preservation from 24 hours to 36 hours from sampling time. |
| SOP 001/01 | FS 7000 | FS 7320, sec. 3.2, 3.5 | Change | Linear Vegetation Survey | Added allowance for aquatic mosses to be included in LVS; Instructed samplers to properly dispose of voucher specimens of exotic plants. |
| SOP 001/01 | FS 7000 | FS 7320, sec. 3.8 | Clarification | Linear Vegetation Survey | Clarified that the C of C and FLEPPC LVS metric calculations shall only include taxa for which C of C or FLEPPC status is known, respectively. |
| SOP 001/01 | FS 7000 | FS 7430, sec. 4.1; FS 7440, sec. 2 & 4.1; FS 7450, sec. 2 & 4.1; FS 7460, sec. 1 & 3.1; FS 7470, sec. 2, 4.2 & 4.3 | Change | Macroinvertebrate sample preservation | Added allowance for use of non-formalin based fixative that penetrates and stabilizes tissue without compromising analytical capability (e.g. NOTOXhisto®). |
| SOP 001/01 | FT 1000 | FT 1000, sec. 2.1.4 | Clarification | Field meter calibrations | Language clarifies that a chronological bracket means that the meter must be verified before use as well as after use. |
| SOP 001/01 | FT 1000 | FT 1000, sec. 2.2.5.2 | Clarification | Continuing Calibration Verification | Further clarification included regarding the potential need to increase the frequency of CCVs when sampling a range of water quality conditions to avoid qualifying data. |
| SOP 001/01 | FT 1000 | FT 1000, sec. 4.2.4 & 4.2.9 | Clarification | Documentation | Included instructions for recording readings and verifications to an instrument's accuracy; included instructions to qualify data associated with a failed verification. |
| SOP 001/01 | FT 1000 | FT 1000, sec. 1.3.3 | Clarification | Factory-calibrated instruments | Added clarification that factory-calibrated instruments must meet the same calibration/verification requirements as all other instrument designs. |
| SOP 001/01 | FT 1100 | FT 1100, sec. 2.3.2.1 | Clarification | Buffer sequence for calibration | Revised to indicate use of value other than pH=7 for first calibration buffer in sequence, when indicated otherwise by manufacturer calibration instructions. |
| SOP 001/01 | FT 1100 | FT 1100, sec. 2.3.2.4 | Clarification | Verification intervals | Clarified with a reference to FT 1000, sec. 2.2.5.1 & 2.2.5.2. |
| SOP 001/01 | FT 1100 | FT 1100, sec. 2.3.2 | Clarification | Verifications and Bracketing | Clarified that samples must be bracketed by successful verifications. |
| SOP 001/01 | FT 1100 | FT 1100, sec. 4.2.4 & 4.3 | Clarification | Documentation | Included instructions to report readings to the resolution of the instrument; included instruction to qualify data associated with failed verification. |
| SOP 001/01 | FT 1200 | FT 1200, sec. 3.2.1 | Clarification | Calibration and Calibration verification | Clarified that samples must be bracketed by successful verifications. |
| SOP 001/01 | FT 1200 | FT 1200, sec. 3.2.5 | Clarification | Verification intervals | Clarified with a reference to FT 1000, sec. 2.2.5.1 & 2.2.5.2. |
| SOP 001/01 | FT 1200 | FT 1200, sec. 5.2.4 & 5.3 | Clarification | Documentation | Included instruction to report readings to the level of resolution stated by the sensor; included instruction to qualify data associated with failed verifications. |
| SOP 001/01 | FT 1200 | FT 1200, sec. 5.3 | Clarification | Calibration verification | Added reference to section 3.2 for clarification of bracketing requirements for samples <100 umhos/cm when using the J-qualifier. |
| SOP 001/01 | FT 1300 | FT 1300, sec. 3.1.1.3 | Clarification | Calibration and Use | Included reference to FT 1200. |
| SOP 001/01 | FT 1300 | FT 1300, sec. 3.1.1.3 | Change | Calibration and Use | Removed verification with a "zero" standard. |
| SOP 001/01 | FT 1300 | FT 1300, sec. 3.1.1.4 | Clarification | Calibration and Use | Clarified that samples must be bracketed by successful verifications. |
| SOP 001/01 | FT 1300 | FT 1300, sec. 5.2.4 & 5.3 | Clarification | Documentation | Included instruction to report readings to the level of resolution stated by the sensor; included instruction to qualify data associated with failed verifications. |
| SOP 001/01 | FT 1400 | FT 1400, sec. 2.3.4 | Clarification | Measuring samples | Specified that temperature should not be measured from an intermediate container |
| SOP 001/01 | FT 1400 | FT 1400, sec. 4.2.4 & 4.3 | Clarification | Documentation | Included instruction to report readings to the level of resolution stated by the sensor; included instruction to qualify data associated with failed verifications. |
| SOP 001/01 | FT 1500 | FT 1500, sec. 1.2.3 | Change | Standards | The standard for dissolved oxygen should be prepared with tap, distilled, or site water, prior to or during site visit, per USGS documents. Included the ability to use pre-formulated zero-DO solutions. |
| SOP 001/01 | FT 1500 | FT 1500, sec. 2.2 | Clarification | Calibration and use | Clarified that samples must be bracketed by successful verifications. |
| SOP 001/01 | FT 1500 | FT 1500, sec. 2.6 | Clarification | Measuring DO in samples | Do not measure DO from an intermediate container containing static sample. |
| SOP 001/01 | FT 1500 | FT 1500, sec. 4.2.4 & 4.2.9 | Clarification | Documentation | Included instruction to report readings to the level of resolution stated by the sensor; included instruction to qualify data associated with failed calibrations. |
| SOP 001/01 | FT 1500 | FT 1500, Table 1500-1 | Change | Saturation values for temperature | Added a reference to Table FS 2200-2 in footnote 1 of Table 1500-1 for temperature look up in 0.1°C increments. |
| SOP 001/01 | FT 1800 | FT 1800 | Change/New SOP | Flow and Discharge Measurements in Flowing Waters | The FT 1800 Field Measurement of Water Flow and Velocity has been revoked and replaced with FT 1800 Field Measurement of Stage, Surface Water Velocity, and Discharge (Flow) and Computation of Streamflow. The new SOP covers direct and indirect methods for the determination of discharge. Chapters from Techniques of Water Resource Investigations and Techniques & Methods, Book 3, published by the United States Geological Service (USGS) are referenced throughout the document. |
| SOP 001/01 | FT 3000 | FT 3001, sec. 2.4 | Change | HA in deep waters | Removed the instruction to use only visible portions of the 100 m stretch as the denominator in habitat availability calculations for stream and river habitat assessment. |
| SOP 001/01 | FT 3000 | FT 3100, sec. 2.3 | Change | Moss as aquatic habitat | Added clarification to include moss as aquatic macrophytes in stream and river habitat assessments if the predominant length is greater than 15 cm. |
| SOP 002/01 | LT 7000 | LT 7010, sec. 1 & 3.3 | Change | Macroinvertebrate sample preservation | Added allowance for use of non-formalin based fixative that penetrates and stabilizes tissue without compromising analytical capability (e.g. NOTOXhisto®). |
| SOP 002/01 | LT 7000 | LT 7613, sec. 4.4; LT 7623, sec. 4.6 | Change | Macroinvertebrate metric calculations | Replaced the reference to Merritt et al. with reference to the FDEP Statewide Biological Database for taxonomic metric information (scraper and predator identification). |
| SOP 002/01 | LT 7000 | Table LT 7600-1 | Change | Plant taxa lists | Added C of C scores for new plants or those missing scores; updated FLEPPC status as needed. |
| SOP 003/11 | BRN 1000 | BRN 1100, sec. 3 | Change | Sample Collection | Added guidance for sites with five productive habitats. Field sorting now recommended but not required. |
| SOP 003/11 | BRN 1000 | BRN 1110 | Clarification | Documentation | Record of number of dipnet sweeps now required. |
| SOP 003/11 | BRN 1000 | BRN 1200 | Change | Training | Required training is now in Form FD 9000-35 (training checklist) rather than SCI 1000 Appendix. Use of form FD 9000-35 for training documentation is required. |
| SOP 003/11 | BRN 1000 | BRN 1300, sec. 3 | Clarification | Performance Criteria | Organisms left in detrital material after reaching target count will not subtract from sorting efficiency score during audits. |
| SOP 003/11 | BRN 1000 | BRN 2100, sec. 3 | Clarification | BioRecon Determination (Lab) | Clarification to not identify terrestrial or partial organisms in count. |
| SOP 003/11 | BRN 1000 | BRN 2100, sec. 5 | Change | BioRecon Determination (Lab) | Replaced the reference to Merritt et al. with reference to the FDEP Statewide Biological Database for taxonomic metric information (clinger and filterer identification). |
| SOP 003/11 | LVI 1000 | LVI 1100, sec. 3.2.11 & 3.3.3 | Change | Preventing spread of exotics after LVI sampling | Added instruction to rinse boat and trailer thoroughly between sites and dispose of voucher specimens properly to prevent spread of invasive exotic plants. |
| SOP 003/11 | LVI 1000 | LVI Appendix | Change | Plant taxa lists | Added C of C scores for new plants or those missing scores; updated FLEPPC status as needed. |
| SOP 003/11 | SCI 1000 | SCI 1000 Appendix | Change | SCI Training Checklist | Deleted the checklist from the SCI 1000 Appendix because it was redundant with Form FD 9000-35. |
| SOP 003/11 | SCI 1000 | SCI 1100, sec. 1 & 3 | Change | SCI sample preservation | Added allowance for use of non-formalin based fixative that penetrates and stabilizes tissue without compromising analytical capability (e.g. NOTOXhisto®). |
| SOP 003/11 | SCI 1000 | SCI 1100, sec. 2.5 & 2.6.3 | Change | SCI Sampling | Added clarification about how to include moss as habitat in SCI sweeps. |
| SOP 003/11 | SCI 1000 | SCI 1200, sec. 1 | Change | SCI Training Checklist | Replaced the reference to SCI 1000 Appendix training checklist to Form FD 9000-35. Use of form FD 9000-35 for training documentation is required. |
| SOP 003/11 | SCI 1000 | SCI 2200, sec. 5 | Change | SCI score reporting | Removed the requirement to report the SCI score with the "A" qualifier. |
| SOP 003/11 | SCI 1000 | SCI 2200, sec. 5.5 | Change | SCI metric calculations | Replaced the reference to Merritt et al. with reference to the FDEP Statewide Biological Database for taxonomic metric information (clinger and filterer identification). |