ESOC Pilot Study Land Use Review

To determine the extent of the basin areas for the four ESOC pilot study sampling locations, the National Hydrography Dataset (NHD) was traced upstream from each site location using Network Analyst.  These features were copied into a GIS file geodatabase and intersected with the NHD Hydrologic Unit Code (HUC) 8/10/12 digit basins.  The basins were then dissolved to form the drainage areas of the sampling locations.  Lineal and areal measurements of the NHD reaches were performed and a graduated symbol classification based on natural data breaks was completed to classify reaches into major creeks and rivers within the drainage basins.  Below are the lineal and areal measurements for the basin areas.

| ESOC Sampling Location ID | River miles for major rivers and creeks within basin | Basin Area in Square Miles |
| --- | --- | --- |
| FLO571620 | 523 | 352 |
| S296 | 599 | 993 |
| S470 | 9,878 | 17,163 |
| WIT010 | 1,159 | 2,154 |

Research into the land use/ land cover within the four ESOC pilot basin areas was conducted using the National Land Cover Database (NLCD) for 2006.  The NLCD is a 16-class land cover classification system developed by the Multi-Resolution Land Characteristics Consortium which is applied to the conterminous United States at a resolution of 30 meters.  This raster layer was reprocessed in ArcMap to create seven land cover signatures consisting of barren land, cultivated crops, forest, hay/pasture, open water, urban, and wetlands.  The percent land use coverages for the basins are provided below:

| ESOC Sampling Location ID | Barren Land | Cultivated Crops | Forest | Hay/Pasture | Open Water | Urban | Wetlands |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FLO571620 | 6 | 5 | 17 | 9 | 5 | 19 | 39 |
| S296 | <1 | 31 | 36 | 5 | <1 | 7 | 21 |
| S470 | <1 | 12 | 59 | 8 | 2 | 11 | 7 |
| WIT010 | <1 | 30 | 38 | 7 | 1 | 8 | 17 |

Point source dischargers within the ESOC pilot basin areas was also researched.  The analysis of EPA Permit Compliant System (PCS) sites within the basins concentrated on a list of major dischargers to the rivers and creeks within the basin.  A major discharger is a facility that has a design flow greater than one million gallons per day or serves a population greater than 10000, or has a significant impact on water quality.  Facilities not meeting criteria for a major discharger are considered minor dischargers.  PCS dischargers include wastewater treatment facilities, utilities, and industrial facilities which have a outfall or discharge pipe to surface waters.

| ESOC Sampling Location ID | Number of Major Dischargers to Rivers and Creeks Within the Basin Area |
| --- | --- |
| FLO571620 | 15 |
| S296 | 4 |
| S470 | 48 |
| WIT010 | 4 |