Streams associated with livestock operations were sampled by discrete samples (n ¼ 38) or by time-integrated polar organic chemical integrative samplers (POCIS, n ¼ 19). Samples were analyzed for estrogens by gas chromatography-tandem mass spectrometry (GC-MS2) and estrogenic activity was assessed by three bioassays: Yeast Estrogen Screen (YES), T47D-KBluc Assay, MCF-7 Estrogenicity Screen (E-Screen). Water samples were consistently below the United Kingdom proposed Lowest Observable Effect Concentration for 17b-estradiol in fish (10 ng/L) in all watersheds, regardless of land use. Estrogenic activity was often higher in samples during runoff conditions following a period of manure application. Estrone was the most commonly detected estrogen (13 of 38 water samples, mean 1.9, maximum 8.3 ng/L). Because of the T47D-KBluc assay’s sensitivity towards estrone (1.4 times 17b-estradiol) it was the most sensitive method for detecting estrogens, followed by the E-Screen, GC-MS2, and YES. POCIS resulted in more frequent detections of estrogens than discrete water samples across all sites, even when applying the less-sensitive YES bioassay to the POCIS extracts.