Table 1 Average concentrations (±SD) of endocrine active chemicals detected in the Boulder wastewater treatment plant (WWTP) efﬂuent (EFF) and Boulder Creek upstream from the WWTP outfall (REF). Also shown are chemical speciﬁc in vitro and in vivo 17!3-estradiol equivalency factors used to calculate 17!3-estradiol equivalency quotients.

| Compound | In vitro estradiol equivalency factora | In vivo estradiol equivalency factora | Sourceb | 2005 REF, n =6 | SD | 2005 EFF, n =7 | SD | 2006 REF, n =6 | SD | 2006 EFF, n =6 | SD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | (ng L−1) |  | (ng L−1) |  | (ng L−1) |  | (ng L−1) |  |
| Steroids |  |  |  |  |  |  |  |  |  |  |  |
| 17!3-Estradiol | 1 | 1 | 1–3 | <0.2 |  | 2.1 | 1.0 | <0.2 |  | 3.2 | 3.2 |
| 17a-Ethynylestradiol | 1.2 | 25 | 2,3 | <0.2 |  | 1.2 | 0.7 | <0.2 |  | <0.2 |  |
| Estrone | 0.2 | 0.4 | 2,3 | <0.2 |  | 75 | 28 | <0.2 |  | 60 | 75 |
| Estriol |  |  |  | <0.2 |  | 3.4 | 3.2 | <0.2 |  | <0.2 |  |
| 17a-Estradiol |  |  |  | <0.2 |  | 0.1 | 0.3 | <0.2 |  | <0.2 |  |
| Other estrogens |  |  |  |  |  |  |  |  |  |  |  |

4-Nonylphenol                          5.3E−5                    0.003                        3,4              210                230           2000           1300      73                34          240              100

| Bisphenol A |  |  |  | 33 | 46 | 42 | 44 | 18 | 27 | 10 | 5.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1,2-Dichlorobenzene |  |  |  | <10 |  | <10 |  | <10 |  | 120 | 79 |
| 1,4-Dichlorobenzene |  |  |  | 81 | 140 | 2100 | 3000 | <10 |  | 1200 | 360 |
| 4-tert-Butylphenol |  |  |  | <10 |  | 68 | 73 | <10 |  | 47 | 43 |
| 4-tert-Pentylphenol |  |  |  | <10 |  | 65 | 82 | <10 |  | 1.9 | 2.2 |
| 4-Nonylphenolmonoethoxylate |  |  |  | 480 | 1200 | 9800 | 5800 | 68 | 71 | 1500 | 770 |
| 4-Nonylphenoldiethoxylate |  |  |  | 150 | 400 | 5000 | 4000 | 89 | 140 | 440 | 380 |
| 4-Nonylphenoltriethoxylate |  |  |  | 62 | 160 | 2000 | 1800 | <50 |  | 280 | 240 |
| 4-Nonylphenoltetraethoxylate |  |  |  | <50 |  | 450 | 730 | <50 |  | 39 | 31 |
| 4-Nonylphenolmonoethoxycarboxylate |  |  |  | 270 | 330 | 31600 | 9400 | 370 | 220 | 50000 | 6800 |
| 4-Nonylphenoldiethoxycarboxylate |  |  |  | 300 | 510 | 42000 | 7300 | 200 | 110 | 34000 | 13000 |
| 4-Nonylphenoltriethoxycarboxylate |  |  |  | <50 |  | 1500 | 690 | <50 |  | 510 | 240 |
| 4-Nonylphenoltetraethoxycarboxylate |  |  |  | <50 |  | 890 | 740 | <50 |  | 210 | 72 |
| 4-t-Octylphenol |  |  |  | 17 | 29 | 200 | 120 | <10 |  | 30 | 14 |
| 4-t-Octylphenolmonoethoxylate |  |  |  | 17 | 42 | 290 | 260 | <10 |  | 22 | 11 |
| 4-t-Octylphenoldiethoxylate |  |  |  | 82 | 220 | 2100 | 2700 | 8.7 | 11 | 53 | 67 |
| 4-t-Octylphenoltriethoxylate |  |  |  | <10 |  | 260 | 350 | 13 | 33 | <10 |  |
| 4-t-Octylphenoltetraethoxylate |  |  |  | <10 |  | <10 |  | <10 |  | <10 |  |
| Estradiol equivalency quotientc | In vitro |  |  | <1 |  | 13.7 | 10.5 | <1 |  | 12.6 | 10.8 |
| Estradiol equivalency quotientc | In vivo |  |  | <1 |  | 50 | 34 | <1 |  | 23 | 20 |

a In vitro and in vivo estrogen equivalency factors expressed as ng L−1 relative to 17!3-estradiol.

b References for estrogen equivalency factors: (1) Jobling and Sumpter (1993); (2) Soto et al. (1995); (3) Van Den Belt et al. (2004); (4) Preuss et al. (2006).

c Estrogen equivalency quotient in ng 17!3-estradiol L−1 calculated by multiplying concentration of compound by the estradiol equivalency factor of the compound and summing. Concentrations reported are the mean ± SD from 6 to 10 samples collected throughout the experiment.