## A Pilot Study of Emerging Substances of Concern in Florida’s Surface Waters

Cooperative Study—USGS and FDEP

Draft: 1/5/12

## Background:

In 2008 an internal workgroup of the Florida Department of Environmental Protection (FDEP) assessed various strategies to address a new group of contaminants in the environment commonly referred to as emerging substances of concern (ESOCs).   These substances, which have been detected in low concentrations in surface waters throughout the U.S. (Kolpin and others, 2002) and  many other countries (Pal and others, 2010),  include chemicals found in pharmaceuticals and personal care products, flame retardants, pesticides and their degradates, fragrances and flavorants, detergent degradates, plasticizers, solvents, and polycyclic aromatic hydrocarbons.  Several of these contaminants are known endocrine disruptor compounds that are biologically active; however, they are still unregulated or not commonly regulated.  Given that ESOCs are part of the approximately 14 million commercially available compounds in the U.S., it is a daunting task for any regulatory agency to be able to track the occurrence, transport, toxicological effects, and the fate of ESOCs in the environment.  Subsequently, the FDEP workgroup identified several strategies in a report (FDEP, 2008) that involved stakeholder education, assessing ESOC data quality, requests to the U.S. EPA for information on specific ESOC monitoring projects, and improved coordination with federal agencies.

## Problem:

None of the aforementioned FDEP workgroup strategies involve the monitoring of Florida’s surface water or groundwater systems that could be impacted by ESOCs from various waste disposal practices.  Conventional water-quality indicators, such as nitrate, BOD, fecal coliform, are not as effective as certain ESOCs in assessing the influence from wastewater effluent in surface waters (Barber and others, 2006; Froehner and others, 2011).  Without knowing what ESOCs are present in surface water, groundwater, or drinking water, it is not possible to determine impacts on human health and sensitive aquatic organisms.  A recent study that derived provisional drinking water guideline values for 50 selected emerging contaminants based on human toxicological information concluded that for the majority of these compounds the maximum concentrations were well below provisional guideline values (Schriks and others, 2010).  However, the impacts on and risks to aquatic organisms in surface waters have been more pronounced (Smital and others, 2004; Pal and others, 2010).   An association was observed between reproductive endocrine disruption in smallmouth bass in the Potomac River basin and wastewater treatment plant flow, percent of agriculture, total number of animal feeding operations, the number of poultry houses, and animal density within the catchment (Blazer and others, 2011).  Likewise, reproductive disruption in adult male fathead minnows was observed in the Boulder Creek, Colorado, watershed due to endocrine-active chemicals in wastewater treatment plant effluent (Vajda and others, 2011).

## Objective:

The main objective of this project is to conduct a pilot study to determine the occurrence of selected ESOCs in surface water from selected sites in the FDEP Status Monitoring Network of the Watershed Monitoring Program.

## Approach:

We propose to use a stratified sampling approach to determine the occurrence of ESOCs in selected surface water sites in the FDEP Status Monitoring Network of the Watershed Monitoring Program.  Approximately 12-15 surface water sites will be selected that have known or likely sources of wastewater discharges upstream.   (The actual number of sites to be sampled will depend on the final list of analytes to be selected).   Sites that have had elevated levels of bacteria and or nutrients should also be considered for collection of samples for analysis of ESOCs.  At a few of these sites, water samples will be collected upstream from any suspected sources of contamination to provide a baseline for comparison of water quality at the downstream sampling location.

Stringent quality assurance and quality control (QA/QC) protocols will be followed for the collection of samples for ESOCs, so as to not compromise the integrity of the sample.  QA/QC samples will include and environmental blank, a replicate, and a spike.  Blank matrix samples will be processed in the field using laboratory-grade organic-free water that is certified for use in the collection of blank samples for organic compounds.  Replicate samples will provide information on the consistency of sample collection and the precision and variability of the chemical analysis.   Matrix spikes also will be used to assess laboratory analytical recoveries for selected organic compounds in the surface water samples. As part of the QA/QC procedures in the laboratory, blanks, spikes, and surrogate compounds will be used to assess laboratory performance.  Surrogate compounds, which are not normally found in the environment, are used to determine if there are interferences from other chemicals in the sample matrix and to evaluate the efficacy of the analytical methods to detect compounds that are chemically similar.

The USGS has had many years of experience developing and optimizing QA/QC methods that minimize the likelihood of sample contamination.  One of our USGS trained and experienced hydrologic technicians would provide training to FDEP sampling personnel to minimize sample contamination in the field. Samples will be collected using USGS sampling protocol for wastewater, pharmaceutical, and hormone compounds in water (U.S. Geological Survey, Chapter 5.6.1.F).  Field measurements will be made for pH, dissolved oxygen, temperature, and specific conductance. Water samples to be analyzed for ESOCs will be shipped overnight to the USGS National Water Quality Laboratory (NWQL) in Lakewood, Colorado.

Surface water samples will be analyzed for 60 organic wastewater compounds (including flame retardants, fragrances and flavorants, plant and animal steroids, pesticides, detergent degradates, polynuclear aromatic hydrocarbons, pesticides), 48 pharmaceutical compounds (prescription and over-the-counter medications), and 20 hormone compounds (certain known endocrine disruptors).  Laboratory results are expected within 3-4 months after sample collection. See tables 1,2, and 3 for a list of analytes.

## Deliverable:

A summary data report will be prepared that includes tables of the laboratory results for the analyzed ESOCs and a detailed analysis of the QA/QC data. Comparisons will be made for detections of ESOCs in Florida surface waters relative to other studies in the U.S.

Personnel:

Brian Katz will serve as project chief.  Other staff from the USGS Tallahassee office will be used to train and help with field sampling work.

Timeline:

The project will be conducted during a 12-month period, beginning in the summer of 2012.

Costs:

| Salaries--Research Hydrologist (160 hrs) and Hydrologic Technician (160 hrs) |  |  | $35,254 |
| --- | --- | --- | --- |
| Lab Costs | Cost per sample | Number of samples |  |
| Wastewater compounds (Sch1433) | $815 | 15 | 12,218 |
| Pharmaceuticals (48 compounds)(LC8144) | $399 | 15 | 5,985 |
| Hormones (20 compounds) (Sch2434) | $812 | 15 | 12,173 |
| Nitrate isotopes | $270 | 15 | 4,050 |
| Travel |  |  | 1,900 |
| Supplies |  |  | 1,000 |
| Summary data report |  |  | 10,000 |
| Total |  |  | $82,579 |

Selected References:

Barber, L.B., Murphy, S.F., Verplanck, P.L., Sandstrom, M.W., Taylor, H.E., and Furlong, E.T. 2006. Chemical loading into surface water along a hydrological, biogeochemical, and land use gradient: A holistic watershed approach. Environmental Science and Technology, v. 40, pp. 475-486.

Barber, L.B., Brown, G.K., Nettesheim, T.G., Murphy, E.W., Bartell, S.E., Schoenfuss, H.L., 2011. Effects of biologically-active chemical mixtures on fish in a wastewater-impacted urban stream. Science of the Total Environment, .v 409, pp. 4720-4728.

Blazer, V.S., Iwanowicz, L.R., Henderson, H., Mazik, P.M., Jenkins, J.A., Alvarez, D.A., Young, J.A., 2011. Reproductive endocrine disruption in smallmouth bass (Micropterus dolomieu) in the Potomac River basin: spatial and temporal comparisons of biological effects. Environmental Monitoring and Assessment, DOI 10.1007/s10661-011-2266-5.

Froehner, S., Machado, K.S., Falcao, F., Nommich, C., and Bessa, M., 2011. Inputs of domestic and industrial sewage in upper Iguassu, Brazil, indentified by emerging compounds. Water Air Soil Pollution, v. 215, pp. 251-259.

Furlong, E.F., Werner, S.L., Anderson, B.D., and Cahill, J.D., 2008, Methods of Analysis by the U.S. Geological Survey National Water Quality Laboratory- Determination of Human-Health Pharmaceuticals in Filtered Water by Chemically Modified Styrene-Divinylbenzene Resin-Based Solid-Phase Extraction and High-Performance Liquid Chromatography/Mass Spectrometry: U.S. Geological Survey Techniques and Methods, book 5, chap. A9.

Kolpin, D.W., Furlong, E.T., Meyer, M.T., Thurman, M., Zaugg, S.D., Barber, L., B., Buxton, H.T., 2002. Pharmaceuticals, hormones, and other organic wastewater contaminants in U.S. streams, 1999-2000—A national reconnaissance. Environmental Science and Technology, v. 36, pp. 1202-1211.

Pal, A., Yew-Hoong Gin, K., Y-Chen Lin, A., and Reinhard, M., 2010. Impacts of emerging contaminants on freshwater resources: Review of recent occurrences, sources, fate and effects. Science of the Total Environment, v. 408, pp. 6062-6069.

Schriks, M., Heringa, N.B., van der Kooi, M.M.E., de Voogt, P., and van Wezel, A.P., 2010. Toxicological relevance of emerging contaminants for drinking water quality. Water Research, v. 44, pp. 461-476.

Smital, T., Luckenbach, T., Sauerborn, R., Hamdoun, A.M., Vega, R.L., and Epel, D., 2004. Emerging contaminants—pesticides, PPCPs, microbial degradation products and natural substances as inhibitors of multixenobiotic defense in aquatic organisms. Mutation Research, v. 552, pp. 101-117.

U.S. Geological Survey, variously dated, National field manual for the collection of water quality data: U.S.Survey Techniques of Water-Resources Investigations, book 9, chap. A1-A9, accessed March28, 2008, at http://pubs.water.usgs.gov/twri9a

Vajda, A.M., Barber, L.B., Gray, J.L., Lopez, E.M., Bolden, A.M., Schoenfuss, H.L., and Norris, D.O., 2011, Demasculinization of male fish by wastewater treatment plant effluent. Aquatic Toxicology, v. 103, no. 3-4, p. 213-221.

| Table 1. List of organic wastewater compounds (OWCs) compounds and their method reporting levels (0.08 µg/L unless otherwise noted in parentheses in µg/L) that are analyzed by the USGS National Water Quality Laboratory in Denver, Colorado. |  |  |
| --- | --- | --- |
| Fragrances and flavorants | Solvents and Preservatives | Pesticides |
| 3-methyl-1h-indole (skatol) (0.16) | Isophorone | Bromacil (0.4) |
| Acetophenone (0.1) | p-Cresol (0.18) | Carbaryl (1) |
| Tonalide (AHTN) (0.5) | Pentachlorophenol (2) | Carbazole |
| Benzophenone (0.12) | Tetrachloroethylene (PCE) | Chlorpyrifos (0.12) |
| Camphor (0.1) |  | Diazinon |
| Galaxolide (HHCB) (0.5) | Flame Retardants | d-Limonene (0.04) |
| Isoborneol (0.06) | Tri(2-chloroethyl) phosphate (0.1) | Indole (0.14) |
| Isoquinoline (0.2) | Tri(dichloroisopropyl) phosphate (0.12) | Metalaxyl |
| Menthol (0.2) | Tributyl phosphate (0.2) | Metolachlor |
|  |  | N,N-diethyltoluamide (DEET) (0.1) |
| PAHs | Antioxidants | Prometon (0.18) |
| Anthracene | 3-tert-butyl-4-hydroxyanisole (BHA) (0.6) |  |
| Benz[a]pyrene (0.12) | 5-methyl-1h-benzotriazole | Disinfectants |
| Fluoranthene |  | Phenol (0.2) |
| Naphthalene (0.1) | Plant and Animal Steroids | 1,4-dichlorobenzene (deoderizer) |
| Phenanthrene | 3beta-coprostanol (1) | Triclosan (0.2) |
| Pyrene | beta-Sitosterol (1.6) |  |
|  | beta-Stigmastanol (1.2) | Others |
| Fuel-related Compounds | Cholesterol (1.4) | Anthraquinone (manufacturing) (0.16) |
| 1-methylnaphthalene (0.1) |  | Bromoform (disinfection byproduct) |
| 2,6-dimethylnaphthalene (0.12) | Detergent Metabolites | Caffeine (stimulant) (0.1) |
| 2-methylnaphthalene | 4-cumylphenol (0.1) | Cotinine (nicotine metabolite) (0.4) |
| Isopropylbenzene (cumene) (0.1) | 4-n-octylphenol (0.16) | Methyl salicylate (liniment) (0.1) |
|  | 4-tert-octylphenol (1) | Triethyl citrate (cosmetics) (0.2) |
| Plasticizers | Diethoxynonylphenol (total) (5) |  |
| Bisphenol a (0.4) | Diethoxyoctylphenol (1) |  |
| Tri(2-butoxyethyl) phosphate (0.4) | Monoethoxyoctylphenol (1) |  |
| Triphenyl phosphate (0.1) | Para-nonylphenol (total) (1) |  |

| Table 2. List of antibiotics, pharmaceutical, and pharmaceutical degradate compounds. |  |  |
| --- | --- | --- |

| Opiods, barbituates, and benzodiazepines. Codeine. Hydrocodone. Methadone. Oxycodone. Tramadol. Norpropoxyphene. Dihydrocodeine. Butalbital. Pentobarbital. Diazepam. Temazepam. |  |  |
| --- | --- | --- |
| Antiepileptic, muscle relaxants, and antidepressants. Carbamazepine. Oxcarbazepine. Iminostilbene. Phenytoin. Primidone. 2-ethyl-2-phenylmalonamide. Phenobarbital. Venlafaxine. Citalopram. Carisoprodol. Metaxalone Methocarbamol. |  |  |
| Others |  |  |
| Acetominophen Amitriptyline Antipyrine. Meprobamate. Others Chirald Fluconazole. Griseofulvin. Chlorpheniramine. Diphenhydramine. Chloroxylenol. Ticlopidine. Dextromethorphan. Diltiazem. Verapamil. Propofol. Lidocaine. Efavirenz. Celecoxib. Ibuprofen. Piperonyl butoxide. Pentoxifylline. Chloramphenicol. Lincomycin. Ormetoprim. Trimethoprim. |  |  |

| Table 3. Selected hormones, Chemical Abstract Service Registry #s, and reporting levels (ug/L) |  |  |  |
| --- | --- | --- | --- |
| 11-Ketotestosterone | 564-35-2 | 2 |
| 17-alpha-Estradiol | 57-91-0 | 0.8 |
| 17-beta-Estradiol | 50-28-2 | 0.8 |
| 17-alpha-Ethynylestradiol | 57-63-6 | 0.8 |
| Norethindrone | 68-22-4 | 0.8 |
| 4-Androstene-3,17-dione | 63-05-8 | 0.8 |
| cis-Androsterone | 53-41-8 | 0.8 |
| Cholesterol | 57-88-5 | 200 |
| 3-beta-Coprostanol | 360-68-9 | 200 |
| Dihydrotestosterone | 521-18-6 | 4 |
| Bisphenol A | 80-05-7 | 200 |
| Epitestosterone | 481-30-1 | 4 |
| Equilenin | 517-09-9 | 2 |
| Equilin | 474-86-2 | 4 |
| Estriol | 50-27-1 | 2 |
| Estrone | 53-16-7 | 0.8 |
| Mestranol | 72-33-3 | 0.8 |
| Progesterone | 57-83-0 | 8 |
| trans-Diethylstilbestrol | 56-53-1 | 0.8 |
| Testosterone | 58-22-0 | 0.8 |