Hormones

- SGS-AXYS tested for 17 hormone compounds.

- Of those 17 compounds, 15 were from DEP priority list.  (Priority list is 88% of the hormone compounds that SGS-AXYS tested for.)

- 5 hormone  compounds were detected in samples from at least one pilot study site.  Of those, 3 were on DEP priority list.

- For compounds on DEP priority list, data from 2 compounds were qualified due to lab or field blank detections.

- All DEP priority compounds detected were detected using POCIS.  No POCIS data were qualified due to blank detections.

- 2 of the 3 DEP priority compounds detected were detected in the grab samples, but all grab sample data were qualified due to detections in lab or field blanks.

| Sampling Method | # DEP Priority Compounds Detected in Pilot Study | Total # Compounds Detected in Pilot Study |
| --- | --- | --- |
| POCIS & Grab | 2 | 2 |
| POCIS Only | 1 | 3 |
| Grab Only | 0 | 0 |
| Total | 3 | 5 |

- 1 DEP priority compounds were detected at all sites (N = 4) using a single sampling method.

In table below, yellow highlighting indicates compound detected at all site (N=4) using the specified sampling method.  Percentage in parentheses represents portion of data qualified due to blank detections.  V indicates data qualified due to detection in lab blank.  G indicates date qualified due to detection in field blank. Dark gray indicates compound was not detected at any sites using the specified sampling method.

| Compound (* = DEP Priority) | POCIS # Sites with Detects | Grab # Sites with Detects |
| --- | --- | --- |
| Allyl Trenbolone | 2 |  |
| Androstenedione* | 3 |  |
| Androsterone* | 2 | 2 (100% G) |
| Mestranol* | 2 | 4 (100% VG) |
| Testosterone | 1 |  |