Pesticides

- SGS-AXYS tested for 76 pesticide compounds.

- Of those 76 compounds, 72 were from DEP priority list.  (Priority list is 95% of the pesticide compounds that SGS-AXYS tested for.)

- 42 pesticide compounds were detected in samples from at least one pilot study site.  Of those, 39 were on DEP priority list.

- For compounds on DEP priority list, data from 17 compounds were qualified due to lab or field blank detections.  Data from 1 non-priority compound were also qualified due to blank detections.

- A majority of the DEP priority compounds (27 of the 39 that were detected) were detected in the grab samples.  11 of the remaining 12 compounds detected were detected in the SPMD, while only 6 of the remaining 12 compounds were detected in the POCIS.

| Sampling Method | # DEP Priority Compounds Detected in Pilot Study | Total # Compounds Detected in Pilot Study |
| --- | --- | --- |
| SPMD & POCIS & Grab | 17 | 18 |
| SPMD & POCIS | 5 | 6 |
| SPMD & Grab | 6 | 6 |
| POSIC & Grab | 4 | 4 |
| SPMD Only | 6 | 7 |
| POCIS Only | 1 | 1 |
| Grab Only | 0 | 0 |
| Total | 39 | 42 |

- 28 DEP priority compounds were detected at all sites (N = 4) using a single sampling method.  An additional 2 non-priority compounds were detected at all sites (N = 4) using a single sampling method.

In table below, yellow highlighting indicates compound detected at all site (N=4) using the specified sampling method.  Percentage in parentheses represents portion of data qualified due to blank detections.  V indicates data qualified due to detection in lab blank.  G indicates date qualified due to detection in field blank. Dark gray indicates compound was not detected at any sites using the specified sampling method.

| Compound (* = DEP Priority) | SPMD # Sites with Detects | POCIS # Sites with Detects | Grab # Sites with Detects |  | Compound (* = DEP Priority) | SPMD # Sites with Detects | POCIS # Sites with Detects | Grab # Sites with Detects |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2,4'-DDD* | 4 |  |  |  | Endrin Ketone* | 2 |  | 1 (100% G**) |
| 2,4'-DDE* | 3 |  |  |  | Ethalfluralin* | 3 |  |  |
| 4,4'-DDD* | 4 |  |  |  | Flufenacet | 1 |  |  |
| 4,4'-DDE* | 4 |  | 1 |  | Flutriafol* |  | 1 | 1 (100% G) |
| 4,4'-DDT* | 4 |  |  |  | HCH, alpha* | 4 | 2 | 1 |
| Alachlor* | 1 | 1 | 1 (100% VG) |  | HCH, beta* | 4 | 3 | 3 |
| Aldrin* | 4 (75% V) | 1 | 2 |  | HCH, delta* | 2 | 1 |  |
| alpha-Endosulphan* | 4 (100% VG**) | 4 (100% VG**) | 4 (100% VG) |  | HCH, gamma | 4 (100% G) | 3 | 1 |
| Ametryn* |  | 1 |  |  | Heptachlor* | 4 (100% G) | 4 (100% G) |  |
| Atrazine* | 2 | 4 | 4 |  | Heptachlor Epoxide* | 4 (50% V) | 4 (100% G) | 3 (100% VG) |
| beta-Endosulphan* | 4 (100% VG) | 3 (100% VG**) | 4 (100% VG**) |  | Hexachlorobenzene* | 4 (100% G) | 4 (100% VG) | 4 (100% VG) |
| Chlordane, alpha (cis)* | 4 | 2 |  |  | Hexazinone* |  | 1 | 2 |
| Chlordane, gamma (trans)* | 4 (75% G) | 4 (100% G) | 3 (100% V) |  | Metolachlor* | 4 (100% G) | 4 | 4 (25% V) |
| Chlordane, oxy-* | 4 (100% VG) |  | 2 (100% V) |  | Mirex* | 4 (100% VG) | 1 | 3 |
| Chlorpyriphos* | 4 |  | 1 |  | Nonachlor, cis-* | 4 (25% G) |  | 1 (100% G) |
| Desethylatrazine* |  | 4 | 4 |  | Nonachlor, trans-* | 4 | 4 | 2 |
| Diazinon* | 1 | 1 |  |  | Pendimethalin* | 4 |  |  |
| Dieldrin* | 4 | 4 | 4 |  | Quintozene* | 3 (100% G**) | 3 (100% G**) |  |
| Dimethenamid* | 1 | 3 | 3 |  | Simazine* |  | 4 | 4 |
| Endosulphan Sulphate* | 4 | 4 | 3 |  | Tebuconazol* | 3 (100% G) | 4 | 4 |
| Endrin* | 4 (75% V, 100% G) |  | 2 (100% V**) |  | Trifluralin | 4 | 1 |  |

**Presence uncertain due to high analyte concentration in blanks. (Blank value > 90% of sample value for all sites with detections.)