# Chapter 1: Issues of Environmental Interest and Water Quality Initiatives

The Florida Department of Environmental Protection (DEP) works with many different programs and agencies throughout the state to address issues and problems affecting surface water and groundwater quality and quantity. These responsibilities are implemented through a variety of activities, including planning, regulation, watershed management, the assessment and application of water quality standards, nonpoint source pollution management, ambient water quality monitoring, groundwater protection, educational programs and land management. This chapter describes some major issues of environmental interest and ongoing water quality initiatives being undertaken primarily by DEP.

## Monitoring Emerging Contaminants (ECs) and Pesticides—Alicia

There is public concern that unregulated contaminants, and their degradants, may be causing human health and ecological impacts as low levels of these compounds increasingly are being detected in water resources throughout the world. Commonly are referred to as ECs, they include pharmaceuticals, personal care products, hormones, pesticides, detergents, plasticizers, and flame retardants. Some are being introduced into the aquatic environment through treated wastewater, as standard wastewater treatment technologies do not remove many of these types of contaminants.

Based on recommendations from an internal workgroup, in 2009 DEP began developing lab methodologies for ultra-trace level analyses of compounds to be used as indicators for wastewater and pesticides. Compounds include the artificial sweetener sucralose; the pharmaceuticals acetaminophen, carbamazepine, and primidone; and the pesticide imidacloprid. These compounds are hydrophilic, or attracted to water, and therefore may be highly mobile in the freshwater environment.

In 2013, a DEP workgroup recommended a study to determine the potential biological effects of ECs on aquatic organisms by employing screening assays to detect ecological effects, such as estrogenic activity. The priority ECs were selected based on factors such as global presence, exposure to humans and wildlife, bioaccumulative and toxic effects, persistence in the environment, and suspected endocrine disruption. The main objective of the pilot study, completed in 2019, evaluated the efficiency and effectiveness of methods using specific biomarkers to assess waterbody biological health. Site selection was determined based on the frequency and magnitude of sucralose detections at DEAR's Surface Water Trends Monitoring sites in 2012. Three of the river sites chosen were the Apalachicola, the Withlacoochee and the Ochlockonee—all which flow into Florida from Georgia. The fourth site is located on the Alafia River, which flows west into Tampa Bay from the center of the Florida peninsula, with its headwaters near the City of Lakeland.

Polar organic compound integrative samplers (POCIS) and semi-permeable membrane devices (SPMD) were deployed at each river site in March 2017. One-time whole water grab samples were taken when the devices were retrieved 30 to 35 days later. SGS AXYS Laboratories and the DEP Laboratory conducted chemical analyses. These focused on 6 categories of EC: pesticides, pharmaceuticals and personal care products (PPCPs) perfluorinated compounds (PFCs), polybrominated diphenyl ethers (PBDEs), alkylphenols, and hormones. The University of Florida (UF) and U.S. Geological Survey (USGS) conducted biological evaluations. POCIS and SPMD extracts and whole water samples from each site were screened using in vitro receptor assays.

Table 1 lists the chemistry results for the study by EC category and by analytical laboratory. Qualitative results are used so that comparisons can be made between sampling types. All compound categories were detected at all sites in at least one sampling type. Table 2 lists the biology categories by test type and laboratory.

Table 1. Number of compounds analyzed in each category

* The DEP Lab used a reduced analyte list for some sampling devices. POCIS samples were not analyzed for 29 pesticide compounds. SPMD samples were not analyzed for 1 pesticide compound.

**SPMD samples were not analyzed for PPCPs by the SGS AXYS Lab.

| Category | Sampling Devices Used | Number of Compounds Analyzed by SGS AXYS Lab | Number of Compounds Analyzed by DEP Lab |
| --- | --- | --- | --- |
| Pesticides | POCIS, SPMD, and Grab | 76 | 88* |
| PPCPs | POCIS, SPMD,** and Grab | 127 | 4 |
| Hormones | POCIS and Grab | 17 | N/A |
| PFCs | POCIS and Grab | 13 | N/A |
| PBDEs | SPMD and Grab | 8 | N/A |
| Alkylphenols | SPMD and Grab | 4 | N/A |

Table 2. Assay categories with assay name and entity conducting assay

| Assay Category | Assay Name | Entity Conducting Assay |
| --- | --- | --- |
| Receptor Binding Assay | Estrogen and androgen receptor binding by fluorescence polarization | UF |
| Receptor Transactivation Assay | Estrogen and androgen transactivation assay by using both agonist and antagonist modes | UF |
| Receptor Binding Assay | Bioluminescent yeast estrogen and androgen screens (BLYES and BLYAS) | USGS |
| Genotoxicity Assay | SOS Chromotest | USGS |

UF conducted bioanalytical assays (receptor binding and transactivation) on whole water grab samples and extracts from the SPMD and POCIS samplers. For the receptor binding assays, interference from background fluorescence was an issue when analyzing the whole water grab and POCIS extract samples. This interference was likely because of the naturally high total organic carbon (TOC) and humic acids in the water present at the study sites. These preclude any conclusions from being drawn concerning the presence or absence of estrogenic or androgenic compounds in POCIS and grab samples.

However, background fluorescence did not cause notable interference in the interpretation of the binding assay results for the SPMD extracts. For these extracts, on a qualitative basis, there was a positive indication of estrogen receptor and androgen receptor binding chemicals at each of the study sites. The transactivation assays also yielded variable results most likely because of interference. Further analysis of additional river sites and the use of additional sample clean-up steps for dark-water rivers are needed before conclusions about the presence or absence and effects of estrogenic or androgenic compounds can be drawn with confidence.

USGS used the SOS Chromotest to test each of the sample types (POCIS, SPMD, grab) at each river site and on a field blank. All sites were negative for genotoxicity in all sample types, except at the Alafia River site, where the POCIS and SPMD samples tested positive.

USGS also tested samples at all sites and from all sample types using BLYES and BLYAS. The grab samples from the Alafia, Apalachicola, and Withlacoochee River sites were found to be estrogenic. POCIS samples were found to be estrogenic at all sites, while SPMD samples were not estrogenic at any site. No androgenic activity was found in any sample type or at any site.