| Sampling Type | Estrogen Receptor Results | Androgen Receptor Results |
| --- | --- | --- |
| Grab | - Variable results indicated interference* with the assay. | - Variable results indicated interference* with the assay. |
| POCIS | - Variable results indicated interference* with the assay. | No activity |
| SPMD | - Variable results indicated interference* with the assay. | No activity |

Results from the estrogen and androgen transactivation receptor assay.

* Interference from background fluorescence was an issue when analyzing the whole water grab and POCIS extract samples. This interference was likely because of the naturally high total organic carbon (TOC) and humic acids in the water present at the study sites. These preclude any conclusions from being drawn concerning the presence or absence of estrogenic or androgenic compounds in POCIS and grab samples.

Results from the estrogen and androgen binding receptor assay.

* Interference from background fluorescence was an issue when analyzing the whole water grab and POCIS extract samples. This interference was likely because of the naturally high total organic carbon (TOC) and humic acids in the water present at the study sites. These preclude any conclusions from being drawn concerning the presence or absence of estrogenic or androgenic compounds in POCIS and grab samples.

Results from the BLYES, BLYAS, and SOS Chromotest.

| Sampling Type | BLYES | BLYAS | SOS Chromotest |
| --- | --- | --- | --- |
| Grab | Activity present in Alafia, Apalachicola, and Withlacoochee samples. | No activity | No activity |
| POCIS | Activity present in all site samples. | No activity | Presence of DNA damaging compounds in Alafia sample. |
| SPMD | No activity | No activity | Presence of DNA damaging compounds in Alafia sample. |

| Sampling Type | Estrogen Binding Results | Androgen Binding Results |
| --- | --- | --- |
| Grab | Variable results indicated interference* with the assay | Variable results indicated interference* with the assay |
| POCIS | Variable results indicated interference* with the assay. | Variable results indicated interference* with the assay |
| SPMD | Activity present in all site samples. | Activity present in all site samples. |