Abstract

The Florida Department of Environmental Protection (DEP) performed a study to evaluate sampling techniques and technologies, analytical methods, and toxicological assays of emerging contaminants (EC) in Florida’s fresh waters. Four river sites (the Apalachicola, Withlacoochee, Ochlockonee, and Alafia Rivers) were selected based on the sucralose results from previous sampling conducted by DEP in 2012 and 2015. In March 2017, a passive sampler (canister containing polar organic compound integrative samplers (POCIS) and semi-permeable membrane devices (SPMD)) was deployed between 34 and 35 days, and whole water grab samples were collected when the devices were retrieved. The 6 categories of EC that were targeted were: pesticides, pharmaceuticals and personal care products (PPCPs) perfluorinated compounds (PFCs), polybrominated diphenyl ethers (PBDEs), alkylphenols, and hormones. POCIS and SPMD extracts and whole water samples from each site were screened using in vitro receptor assays.

Sampling and chemical analyses for EC are sensitive processes, in part because of the low laboratory detection limits (< 10 ng/L) used to determine the presence of many of these compounds. Because of this, chemical analyses are accompanied by lengthy quality control (QC) reports that must be carefully reviewed to determine data usability.

For passive sampling devices, time-weighted averages of analyte concentrations are sufficient for determining biological exposure rates. To determine time-weighted average concentrations for passive sampling devices, the sampling rates (uptake rate of compound onto passive sampling material adjusted for flow dynamics) of individual compounds should be determined. Furthermore, performance reference compounds (PRCs) should be deployed with the passive samplers so that site-specific uptake distributions can be determined for individual compounds.

In test protocols that measure estrogenicity and androgenicity, the high level of color in Florida waters caused interference in detections. An increased understanding of site-specific sampling rates for estrogenic and androgenic compounds, and further refinement of the laboratory methods used for the determination of estrogenicity and androgenicity are recommended for future studies.

Future studies of EC in Florida’s waterbodies should focus on waterbodies directly affected by known sources of these substances (e.g., river segments continually impacted by wastewater overflows). This approach would reduce the burden of collecting and processing data by focusing on known contaminant sources.