- `Intro/Background

In 2008, a Florida Department of Environmental Protection (FDEP) internal workgroup assessed various strategies to address a group of compounds commonly referred to as emerging contaminants (EC). These substances, which have been detected in low concentrations in surface waters throughout the U.S. and abroad, include chemicals found in pharmaceuticals and personal care products, flame retardants, pesticides and their degradates, fragrances and flavorants, detergent degradates, plasticizers, solvents, and polycyclic aromatic hydrocarbons (Kolpin et al., 2002; Pal et al., 2010). Several of these compounds are known mutagens and endocrine disruptors; however, most them are unregulated (CAS, 2013). The main objective of the study was to evaluate sampling technique and technology, analytical methods, and toxicological assays to assess the biological health of waterbodies. The study tested the ability of the sample collection devices and analytical methods’ effectiveness and efficiency in achieving the objective using a small-scale study design. Chemical screening for the occurrence of ESOCs in Florida’s waterbodies provides information on the fate of these compounds after they are introduced into the environment.

  - Historical context

Due to the source, release, and nature of the compounds, the greatest number of ESOC detections were expected when a combination of sampling methods (POCIS, SPMD, and whole-water grabs) are used (Alvarez, 2010).

Monitoring for chemical tracers in the environment is a powerful tool for characterizing anthropogenic pollutants and identifying potential pollutant sources (i.e. point and non-point sources). The compound sucralose (trade name Splenda) is an effective compound to use as a tracer.  The characteristics that make sucralose useful as a tracer include its presence in domestic wastewater discharge at detectable levels, no natural occurrence, low toxicity, high solubility in water, ineffective metabolization and removal by wastewater treatment processes, and its persistence in the environment (Labare and Alexander 1993; Oppenheimer et al. 2011; Soh et al. 2011; Tollefsen et al. 2012). DEP's monitoring of sucralose has helped identify sites for intensive study and track contaminant migration routes in surface water and groundwater.

To better understand the extent of Florida’s freshwater resources potentially effected by emerging contaminants, DEP's Division of Environmental Assessment and Restoration (DEAR) added wastewater tracers and imidacloprid to its status and trends monitoring networks (FDEP 2017). DEAR sampled statewide for sucralose in both monitoring networks in 2012 (Seal et al. 2016) and for sucralose, the pharmaceuticals acetaminophen, carbamazepine, and primidone, and the neonicotinoid pesticide imidacloprid in both monitoring networks in 2015 (Silvanima et al. 2018). All of these compounds are hydrophilic and therefore may be highly mobile in the freshwater environment (Bonmatin et al. 2015; Jenner and Smithson 1989; Wishart et al. 2006).

Sucralose was commonly detected at ultra-trace concentrations in both networks in 2012 and 2015. Sucralose was found in all freshwater resources monitored, rivers, streams, canals, large and small lakes and unconfined and confined aquifer wells. Statewide status network sucralose detection frequencies were 43.3 and 55.3%, respectively. Status network sampling in 2015 provided statewide ultra-trace detection frequencies of 16.5% for the occurrence of any of the pharmaceuticals, and of 39.4% for imidacloprid. Of the water resources sampled, rivers consistently produced the highest detection frequencies for the three compound classes listed above.

The sucralose results from monthly surface water trend monitoring conducted in 2012 were used to select the sample locations for this pilot study.  Of the 75 stations sampled (refer to fig. X), the four containing the most frequent detections, as determined by using only values greater than the method and quantification limits of DEP’s instrumentation, were selected.

Figure x.  2012 Surface Water Trend Sites Selected for Pilot Study.  Site Ids 3543, 3540, 3539 and 3554 on the Apalachicola, Ochlockonee, Withlacoochee, and the Alafia rivers respectively.

- Study Design

  - Site Selection

Sucralose data were used as a first screening tool for sites where we expect human and animal waste influence and thus the presence of ESOC. Four sites were selected for the 2017 pilot study based on Trend Network river sites that had the highest sucralose concentrations measured with detections (Figure  X). Many cities located in these watersheds use rivers as a method of wastewater treatment plant effluent disposal.

    - Site Characterization

The four sites are river sites, each with a 15-year history of regular monthly sampling by Department staff for the suite of standard water quality analytes (e.g., nutrients, major ions). Three of the rivers, the Apalachicola, the Withlacoochee and the Ochlockonee, flow into Florida from Georgia over the Atlantic Coastal Plain province.  The Ochlockonee and Withlacoochee Rivers each have significant amounts of agricultural and sylvicultural activity in their watersheds. The Apalachicola River, formed by the confluence of the Flint and the Chattahoochee Rivers near Albany Georgia, drains much of the state of Georgia.  It is used for sanitary waste disposal by millions of people in many cities, including metro Atlanta, Griffin and Bainbridge. The Chattahoochee River has three federally maintained water control structures (dams) upstream, which slow flow and pool the river. This provides an opportunity for sedimentation to occur before the water is released further downstream. The fourth site is located on the Alafia River, which flows west into Tampa Bay from the center of the Florida peninsula, with its headwaters near the City of Lakeland.

  - Sampling

    - Technique

The use of passive samplers such as polar organic compound integrative samplers (POCIS) and semipermeable membrane devices (SPMD), as well as whole water grab samples, were considered viable methods to achieve the objective. Three POCIS and three SMPDs contained in one large canister (Figure XX) were deployed at each of the 4 river sites for the study. Whole-water grab samples were collected when the passive devices were retrieved. The devices were deployed at each site for 30 – 35 days. A field blank for each sampling method (POCIS, SPMD, and whole water) was collected at one of the sites.

  - Tests

    - Tables of categories

Many of these compounds are suspected to cause endocrine disruption effects in both aquatic and terrestrial invertebrate and vertebrate species (Blazer, et al. 2011 and 2013, Guy, et al. 2009, Letcher, et al. 2010, Oetken, et al. 2004, Vos, et al. 2000). ECs are categorized based on their manufacturing or commercial use.  Each category of ESOC compounds tends to have similar exposure routes, aquatic life effects, and chemical characteristics (Table 1).

| Category | Primary Routes of Potential Human Exposure | Aquatic Life Effects | Polarity / Water Solubility |
| --- | --- | --- | --- |
| Pesticides | Dermal contact; ingestion (e.g. contaminated food or water); inhalation (e.g. breathing vapors, dust, or spray particles); ocular exposure | Toxic, potential endocrine disruptor (ED) | Most are polar and have high solubility |
| Pharmaceuticals and Personal Care Products (PPCPs) | Ingestion (e.g. oral medications); inhalation (e.g. aerosol hair products, perfumes); dermal contact (e.g. topical medications, deodorants, lotions) | ED | Most are polar and have high solubility |
| Synthetic Hormones | Ingestion (e.g. contaminated food or water, oral medications); inhalation (e.g. dust and particulates); dermal contact | ED | Polar and have high solubility |
| Perfluorinated Compounds (PFCs) | Ingestion (e.g. contaminated food or water, breast milk); hand-to-mouth from surfaces that contain PFCs | Bioaccumulative, ED | Polar and have high solubility |
| Polybrominated Diphenyl Ethers (PBDEs) | Ingestion (e.g. contaminated food, water, breast milk); inhalation; dermal contact | Toxic, Bioaccumulative, ED, Carcinogenic | Nonpolar / low water solubility |
| Alkylphenols | Ingestion (contaminated food or water); contact with some personal care products and detergents; inhalation (primarily indoor air, dusts) | ED | Range of polarities / generally low water solubility |

Table 1. General Categories of ESOC (Adapted from Dellinger et al., 2011)

    - Chem screening

The group of laboratory analyses performed was determined based on the type of sampling device  (Stephanie Table). SGS AXYS Laboratories and Florida Department of Environmental Protection (FDEP) Laboratory conducted the analyses.

Number of Compounds Analyzed in Each Category

| Category | Sampling Devices Used | Study Samples: Number of Compounds Analyzed |  |
| --- | --- | --- | --- |
|  |  | SGS Lab | DEP Lab |
| Pesticides | POCIS, SPMD, & Grab | 76 | 88* |
| Pharmaceuticals and Personal Care Products (PPCPs) | POCIS, SPMD**, & Grab | 127 | 4 |
| Hormones | POCIS & Grab | 17 | N/A |
| Perfluorinated Compounds (PFCs) | POCIS & Grab | 13 | N/A |
| Polybrominated Diphenyl Ethers (PBDEs) | SPMD & Grab | 8 | N/A |
| Alkylphenols | SPMD & Grab | 4 | N/A |

* The DEP lab used a reduced analyte list for some sampling devices. POCIS samples were not analyzed for 29 pesticide compounds.  SPMD samples were not analyzed for 1 pesticide compounds.

**SPMD samples were not analyzed for PPCPs by the SGS lab.

    - Bio screening

Several emerging contaminants are known to be estrogenic and androgenic, such as pesticides and hormones. Estrogenicity and androgenicity in rivers can cause reproductive changes fish and wildlife (USGS, 2019). Other compounds may be found that cause genotoxic effects to the aquatic life, such as Biological response tests were used to measure endocrine activity in cell lines exposed to samples. The biological screening was conducted by University of Florida (UF) and United States Geological Survey (USGS). POCIS dialysis, SPMD extracts, and whole water from each site were screened using the following in Vitro receptor assays.

- Receptor Assay

  - Estrogen and androgen receptor (ER and AR) competitive binding assay (conducted by UF)

- Binding Assays

  - Bioluminescent yeast estrogen and androgen screens (BLYES and BLYAS) (conducted by USGS)

  - Estrogen and androgen receptor (ER and AR) mammalian cell reporter gene assay (conducted by UF)

- Genotoxicity Assay

  - SOS Chromotest (conducted by USGS)

- Results

Pesticides – SGS Lab

We analyzed all samples for 76 pesticide compounds, 72 of which were on the DEP priority list.  Of those 76 compounds, 42 compounds (39  priority) were detected at one or more stations, in at least one sample type (SPMD, POCIS, or  grab samples).  Thirty-eight of the 39 priority compounds detected were found using a combination of SPMD and grab samples.  Thirty compounds (28 priority) were detected at all stations (N = 4) using a single sample type.  Detections for 17 priority and 1 non-priority compounds were qualified due to lab or field blank detections.

Pharmaceuticals and Personal Care Products (PPCPs) – SGS Lab

We analyzed POCIS and grab samples for 127 PPCP compounds, 21 of which were on the DEP priority list.  The 127 compounds were divided into 5 analytical lists. Three lists used acid extraction and positive electrospray ionization (APOS, APOSX and APOSY), one used acid extraction and negative electrospray ionization (ANEG), and one used base extraction and positive electrospray ionization (BPOS). The ANEG list had the most priority compounds on it (8 compounds), while the APOS list had the highest number of priority compounds detected (5 compounds).

Of the 127 compounds analyzed, 39 compounds (15 priority) were detected in at one or more stations, in at least one sample type.  Fourteen of the 15 priority compounds detected were found in POCIS samples, and 10 of the 15 priority compounds detected were found in grab samples.  Seven compounds (2 on priority list) were detected at all stations (N = 4) using a single sample type.  Detections for 3 non-priority compounds were qualified due to lab or field blank detections.

Hormones – SGS Lab

We analyzed POCIS and grab samples for 17 hormone compounds, 15 of which were on the DEP priority list. Of those 17 compounds, 5 compounds (3 priority) were detected at one or more stations, in at least one sample type.  All of the priority compounds detected were found in the POCIS samples, and 2 of the 3 priority compounds detected were found in grab samples.  One priority compound was detected at all stations (N = 4) in grab samples.  All grab sample detections were qualified due to lab or field blank detections, but no data from POCIS samples were qualified due to this reason.

Perfluorinated Compounds (PFCs) – SGS Lab

We analyzed POCIS and grab samples for 13 PFC compounds, 10 of which were on the DEP priority list.  Of those 13 compounds, 10 compounds were detected (8 priority) at one or more stations, in at least one sample type.  All of the priority compounds detected were found in the POCIS samples, and five of the eight priority compounds detected were found in grab samples.  Three priority compounds were detected at all stations (N = 4) in POCIS samples.  No PFC data were qualified due to lab or field blank detections.

Polybrominated diphenyl ethers (PBDEs) – SGS Lab

We analyzed SPMD and grab samples for 8 PBDE compounds, all of which were on the DEP priority list.  All compounds analyzed were detected at one or more stations, in both sample types.  All compounds were also detected at all stations (N = 4) in SPMD samples, and 4 of those compounds were also detected at all stations in grab samples.  All SPMD detections and 92% of grab detections were qualified due to lab or field blank detections.

Alkylphenols – SGS lab

We analyzed SPMD and grab samples for 4 alkylphenol compounds, all of which were on the DEP priority list.  Of those 4 compounds, 2 compounds were detected at one or more stations, in at least one sample type.  Both of the compounds detected were found in the SPMD samples, but only one was found in the grab samples.  Both compounds were detected at all stations (N = 4) in SPMD samples, and one compound was also detected at all stations in grab samples.  All alkyphenol data were qualified due to lab or field blank detections.

Pesticides and PPCPs - DEP lab

We analyzed samples for 92 compounds (88 pesticides and 4 PPCPs).  Grab samples were analyzed for all 92 analytes, SPMDs were analyzed for 91 compounds, and POCIS were analyzed for 63 compounds.  Of the 92 compounds analyzed, 44 were on the DEP priority list (43 pesticides and 1 PPCP).  Thirty-two compounds (29 pesticides and 3 PPCPs) were detected at one or more stations, in at least one sample type.   Of the 32 compounds detected, 13 were on the DEP priority list (12 pesticides and 1 PPCP).  Fourteen compounds (6 on priority list) were detected at all stations (N = 4) using a single sample type.  No data from this laboratory were qualified due to lab or field blank detections.

Comparison of SGS and DEP lab Detections

There were 45 compounds (43 pesticides and 2 PPCPs) analyzed by both labs.  For the pesticides, 32 compounds had similar rates of detection between labs. The remaining 11 pesticide compounds were detected by the SGS lab in at least one sample type, but were not detected by the DEP lab.  For the 2 PPCPs, 1 compound was detected by both labs, while the other was only detected by the DEP lab.

University of Florida Results

……….

USGS Results

………….

- Lessons learned and future recommendations

- Acknowledgements

- References

Bonmatin, J. M., Giorio, C., Girolami, V., Goulson, D., Kreutzweiser, D. P., Krupke, C., Liess, M., Long, E., Marzaro, M., Mitchell, D. A., Simon-Delso, N., & Tapparo, A. (2015). Environmental fate and exposure: neonicotinoids and fipronil. Environmental Science and Pollution Research, 22, 35–67. https://doi.org/10.1007/s11356-014-3332-7.

FDEP (Florida Department of Environmental Protection). (2015). Florida Watershed Monitoring Status and Trend Program design document, technical document, Division of Environmental Assessment and Restoration, Watershed Monitoring Section. http://publicfiles.dep.state.fl.us/dear/DEARweb/WMS/Reports_Docs_SOPs/Design_Docs/WMS-MonitoringDesignDocument2017.pdf. Accessed 15 Feb. 2019.

Jenner,M. R., & Smithson, A. (1989). Physicochemical properties of the sweetener sucralose. Journal of Food Science, 54, 1646–1649. https://doi.org/10.1111/j.1365-2621.1989.tb05179.x.

Labare, M. P., & Alexander, M. (1993). Biodegradation of sucralose, a chlorinated carbohydrate. In Samples of Natural Environments. Environmental Chemistry, 12, 797–804. https://doi.org/10.1002/etc.5620120502.

Oppenheimer, J., Eaton, A., Badruzzaman,M., Haghani, A.W., & Jacangelo, J. G. (2011). Occurrence and suitability of sucralose as an indicator compound of wastewater loading to surface waters in urbanized regions. Water Research, 45, 4019–4027. https://doi.org/10.1016/j.watres.2011.05.014.

Seal, T., Woeber, N. A., & Silvanima, J. (2016). Using tracers to infer potential extent of emerging contaminants in Florida’s ground waters. Florida Scientist, 79, 279 – 289. https://fas.fit.edu/media/site-specific/fasfitedu/wwwfloridaacademyofsciencesorg/wp-content/uploads/2017/05/flsc-79-04-cov.pdf.

Silvanima, J., Woeber, A., Sunderman-Barnes, S. et al. Environ Monit Assess (2018) 190: 435. https://doi.org/10.1007/s10661-018-6782-4

Tollefsen, K. E., Nizzetto, L., & Huggett, D. B. (2012). Presence, fate and effects of the intense sweetener sucralose in the aquatic environment. Science of the Total Environment, 438, 510–516. https://doi.org/10.1016/j.scitotenv.2012.08.060.

U.S Environmental Protection Agency. (2017). Aquatic life benchmarks and ecological risk assessments for registered pesticides. https://www.epa.gov/pesticide-science-and-assessing-pesticide-risks/aquatic-life-benchmarks-and-ecological-risk. Accessed 23 May 2018.Wishart, D. S., Knox, C., Guo, A. C., Shrivastava, S., Hassanali, M., Stothard, P., Chang, Z.,&Woolsey, J. (2006). DrugBank: a comprehensive resource for in silico drug discovery and exploration. Nucleic Acids Research, 34, D668–D672. https://doi.org/10.1093/nar/gkj067.

USGS (United States Geological Survey). (2019). Estrogenicity in Streams of New York State. https://www.usgs.gov/centers/ny-water/science/estrogenicity-streams-new-york-state?qt-science_center_objects=0#qt-science_center_objects. Accessed 5 Apr. 2019.