Exploratory Study of Emerging Contaminants in Select Florida Rivers

Division of Environmental Assessment and Restoration

Florida Department of Environmental Protection

June 28, 2019

Table of Contents

Introduction	4

Section 1: Study Design	6

Site Selection	6

Site Characterization	7

Sampling Technique	7

EC Categories	8

Overview of Laboratory Analysis	9

Section 2: Results	9

Pharmaceuticals and Personal Care Products (PPCPs) – SGS AXYS Lab	10

Hormones – SGS AXYS Lab	11

Perfluorinated Compounds (PFC) – SGS AXYS Lab	11

Polybrominated Diphenyl Ethers (PBDE) – SGS AXYS Lab	11

Alkylphenols – SGS AXYS lab	11

Pesticides and PPCP - FDEP lab	11

University of Florida Results	12

USGS Results	12

Section 3: Lessons Learned and Recommendations	13

Acknowledgements	13

References	14

Appendix A. All Compounds Included in this Study	16

Figure and Tables

Figure 1. Surface Water Trend Sites Selected for Study…………………………………………5

Figure 2. Diagram of the Polar Organic Chemical Integrative Samplers (POCIS) and POCIS holder, Semipermeable Membrane Device (SPMD) and SPMD holder, and a large canister to contain the passive samplers………………………………………………………………………6

Table 1. General Categories of EC………………………………………………………………..8

Table 2. Number of Compounds Analyzed in Each Category………………………………......10

Table 3. Assay Categories with Assay Name and Entity that Conducted Assay………………..10

Table 4. Summary of the compound class detections by sampling method for each of the four study sites………………………………………………………………………………………...11

# Executive Summary

The Florida Department of Environmental Protection performed a study to evaluate sampling techniques and technologies, analytical methods, and toxicological assays to study emerging contaminants in Florida’s freshwaters. The study utilized varying types of sample collection devices and analytical methods using a small-scale study design. Seventy-five surface water trend monitoring sites were sampled monthly during 2012 and 2015 by the Division of Environmental Assessment & Restoration (DEAR) for wastewater tracers, including sucralose... Four of these river sites were selected for this study based on the sucralose results. The sites were on  the Apalachicola, the Withlacoochee, the Ochlockonee, and the Alafia Rivers.

Polar Organic Compound Integrative Samplers (POCIS) and Semi-Permeable Membrane Devices (SPMD) were deployed at each river site in March 2017. One-time whole water grab samples were taken when these devices were retrieved 30-35 days later. SGS AXYS Laboratories and Florida Department of Environmental Protection (FDEP) Laboratory conducted chemical analyses. The analyses focused on 6 categories of emerging contaminants: pesticides, pharmaceuticals and personal care products, perfluorinated compounds, polybrominated diphenyl ethers, alkylphenols, and hormones. The group of laboratory analyses performed was determined based on the type of sampling device. The biological evaluation was conducted by the University of Florida (UF) and the United States Geological Survey (USGS). POCIS  and  SPMD extracts and whole water samples from each site were screened using in Vitro receptor assays.

Outcomes

- Sampling and chemical analyses for emerging contaminants (EC) are sensitive processes. This is in part due to the significantly low laboratory detection limits, commonly less than 10 nanograms per liter, used to determine the presence of these compounds. Because of this, chemical analyses are accompanied by lengthy quality control (QC) reports that must be carefully reviewed for determination of data usability.

- For the passive sampling devices, time-weighted averages of analyte concentration are sufficient for determining biological exposure rates. To determine time-weighted average concentrations for passive sampling devices the sampling rates (uptake rate of compound onto passive sampling material adjusted for flow dynamics) of individual compounds should be determined. Furthermore, performance reference compounds should be deployed with the passive samplers so that site specific uptake distributions can be determined for individual compounds.

- For test protocols that measure estrogenicity and androgenicity, the high level of color in Florida waters caused interference in detections. An increased understanding of site-specific sampling rates for estrogenic and androgenic compounds and further refinement of the laboratory methods used for the determination of estrogenicity and androgenicity are recommended for future studies.

- Future studies of ECs in Florida’s waterbodies should focus on waterbodies directly impacted by known sources of ECs (e.g. river segments continually impacted by wastewater overflows). This would reduce the data burden by focusing on known contaminant sources.

# 1.0 Introduction

In 2008, a Florida Department of Environmental Protection (FDEP) internal workgroup assessed the efficacy of sample collection techniques to determine the bioavailability and occurrence of categories of compounds, commonly referred to as emerging contaminants (EC). These substances, which have been detected in low concentrations in surface waters throughout the U.S. and abroad, include chemicals found in pharmaceuticals and personal care products, flame retardants, pesticides and their degradates, fragrances and flavorants, detergent degradates, plasticizers, solvents, and polycyclic aromatic hydrocarbons (Kolpin et al., 2002; Pal et al., 2010). Several of these compounds are known mutagens and endocrine disruptors; however, most them are unregulated (CAS, 2013). The FDEP workgroup reconvened January 2013 to evaluate how it could respond to the growing problem. The 2013 FDEP workgroup recommended that a study be conducted to investigate likely causes of biological effects by employing screening assays to detect ecological effects, such as estrogenic activity occurring in Florida’s water. By 2016, the workgroup developed a study design that was deployed in April 2017. Site selection for the study was based on the detection frequency of a known wastewater tracer compound (sucralose) coming from monthly sampling done at 78 statewide surface water trend network sites conducted in 2012 and 2015. The main objective of the study was to evaluate novel sampling techniques and technologies, analytical methods, and toxicological assays for the study of emerging contaminants in Florida rivers. The study utilized varying types of sample collection devices and analytical methods to address the objective using a small-scale study design. A wide range of compounds were evaluated because of the ubiquitous nature of EC.

## 1.1 Historical context

Monitoring for chemical tracers in the environment is a powerful tool for characterizing anthropogenic pollutants and identifying potential pollutant sources (i.e. point and non-point sources). The compound sucralose (trade name Splenda) is an effective compound to use as a tracer. The characteristics that make sucralose useful as a tracer include its presence in domestic wastewater discharge at detectable levels, no natural occurrence, low toxicity, high solubility in water, ineffective metabolization and removal by wastewater treatment processes, and its persistence in the environment (Labare and Alexander 1993; Oppenheimer et al. 2011; Soh et al. 2011; Tollefsen et al. 2012). FDEP's monitoring of sucralose has helped identify sites for intensive study and to track contaminant migration routes in surface water and groundwater.

To better understand the extent of Florida’s freshwater resources potentially affected by emerging contaminants, FDEP's Division of Environmental Assessment and Restoration (DEAR) began to collect and analyze wastewater tracers and imidacloprid in its status and trends monitoring networks (FDEP 2015). DEAR sampled statewide for sucralose in both monitoring networks in 2012 (Seal et al. 2016) and for sucralose, the pharmaceuticals acetaminophen, carbamazepine, and primidone, and the neonicotinoid pesticide imidacloprid in both monitoring networks in 2015 (Silvanima et al. 2018). All of these compounds are hydrophilic and therefore may be highly mobile in the freshwater environment (Bonmatin et al. 2015; Jenner and Smithson 1989; Wishart et al. 2006).

Sucralose was commonly detected at ultra-trace levels, with a laboratory method detection limit of 0.01 µg/L, in status and trend networks in 2012 and 2015. Sucralose was found in all freshwater resources monitored: rivers, streams, canals, lakes, and both unconfined and confined aquifers. Statewide status network sucralose detection frequencies for combined water resources were 43.3% (2012) and 55.3% (2015), respectively. Status network sampling in 2015 provided statewide ultra-trace detection frequencies of 16.5% for the occurrence of any of the pharmaceuticals, and of 39.4% for imidacloprid. Of the water resources sampled, rivers consistently produced the highest detection frequencies. Statewide monthly trend network sampling of 75 rivers, streams and canals, provided sucralose detection frequencies of 77.3% (2012) and 86.7% (2015) at the sites sampled. Twenty-eight of these sites had detection frequencies greater than 80% for the combined 24-month period.

# 2.0 Study Design

## 2.1 Site Selection

Of the 75 trend stations sampled (Figure 1), the 4 containing the most frequent sucralose detections, as determined by using only values greater than the method detection and practical quantification limits of FDEP’s instrumentation, were selected for this study.

Figure 1.  Surface Water Trend Sites Selected for Study.

## 2.2 Site Characterization

The 4 sites chosen for the study are rivers, each with at least a 15-year history of regular monthly sampling by Department staff for a suite of standard water quality analytes (e.g., nutrients, major ions). Three of the rivers, the Apalachicola, the Withlacoochee and the Ochlockonee, flow into Florida from Georgia over the Atlantic Coastal Plain province.  The watersheds of the Ochlockonee and Withlacoochee Rivers each have significant amounts of agricultural and sylvicultural activity. The Apalachicola River, formed by the confluence of the Flint and the Chattahoochee Rivers near Albany Georgia, drains much of the state of Georgia.  It is used for sanitary waste disposal by millions of people in many cities, including metro Atlanta, Griffin, Columbus, and Bainbridge. The Chattahoochee River has three federally maintained water control structures (dams) upstream, which slow flow and pool the river. This provides an opportunity for sedimentation to occur before the water is released further downstream. The fourth site is located on the Alafia River, which flows west into Tampa Bay from the center of the Florida peninsula, with its headwaters near the City of Lakeland.

## 2.3 Sampling Technique

The greatest number of EC detections are expected when using a combination of sampling methods, such as polar organic chemical integrative samplers (POCIS), semipermeable membrane devices (SPMD) and whole water grab samples (Alvarez, 2010).  For this study, POCIS, SPMD and grab samples were utilized at each site. Three POCIS and three SMPD contained in one large canister (Figure 2) were deployed at each river site for the study in March 2017. Grab samples were collected when the devices were retrieved 30 -35 days later. A field blank for each sampling method (POCIS, SPMD, and grab) was collected at the Apalachicola River site.

Figure 2. Diagram of the Polar Organic Chemical Integrative Samplers (POCIS) and POCIS holder, Semipermeable Membrane Device (SPMD) and SPMD holder, and a large canister to contain the passive samplers.

## 2.4 EC Categories

Many of the compounds evaluated as part of this study are suspected to cause endocrine disruption effects in both aquatic and terrestrial invertebrate and vertebrate species (Blazer et al. 2011 and 2013, Guy et al. 2009, Letcher et al. 2010, Oetken et al. 2004, Vos et al. 2000). EC are categorized based on their manufacturing or commercial use.  Each category of EC compounds tend to have similar exposure routes, aquatic life effects, and chemical characteristics (Table 1).

| Category | Primary Routes of Potential Human Exposure | Aquatic Life Effects | Polarity / Water Solubility |
| --- | --- | --- | --- |
| Pesticides | Dermal contact; ingestion (e.g. contaminated food or water); inhalation (e.g. breathing vapors, dust, or spray particles); ocular exposure | Toxic, potential endocrine disruptor (ED) | Most are polar and have high solubility |
| Pharmaceuticals and Personal Care Products (PPCP) | Ingestion (e.g. oral medications); inhalation (e.g. aerosol hair products, perfumes); dermal contact (e.g. topical medications, deodorants, lotions) | ED | Most are polar and have high solubility |
| Synthetic Hormones | Ingestion (e.g. contaminated food or water, oral medications); inhalation (e.g. dust and particulates); dermal contact | ED | Polar / high solubility |
| Perfluorinated Compounds (PFC) | Ingestion (e.g. contaminated food or water, breast milk); hand-to-mouth from surfaces that contain PFCs | Bioaccumulative, ED | Polar / high solubility |
| Polybrominated Diphenyl Ethers (PBDE) | Ingestion (e.g. contaminated food, water, breast milk); inhalation; dermal contact | Toxic, Bioaccumulative, ED, Carcinogenic | Nonpolar / low solubility |
| Alkylphenols | Ingestion (contaminated food or water); contact with some personal care products and detergents; inhalation (primarily indoor air, dusts) | ED | Range of polarities / generally low solubility |

Table 1. General Categories of EC (Adapted from Dellinger et al., 2011).

## 2.5 Overview of Laboratory Analysis

Chemical

The group of laboratory analyses performed was determined based on the type of sampling device (Table 2). SGS AXYS Laboratories and Florida Department of Environmental Protection (FDEP) Laboratory conducted the analyses.

Biological

Several emerging contaminants are known to be estrogenic and androgenic, such as pesticides and hormones. Estrogenicity and androgenicity in rivers can cause reproductive changes in fish and wildlife (USGS, 2019). EC may also cause genotoxic effects (DNA damage) to aquatic life.  Biological response tests were used to measure endocrine activity in cell lines exposed to samples. The biological screening was conducted by University of Florida (UF) and United States Geological Survey (USGS). POCIS dialysis, SPMD extracts, and whole water from each site were screened using the in Vitro receptor assays detailed in Table 3 below.

# 3.0 Results

The chemistry results for the study are presented by the EC categories and by analytical laboratory (Table 2). Combined chemistry results per station are listed in Table 4. The biology results are presented by laboratory and test type (Table 3).

## 3.1 Pesticides – SGS AXYS Lab

Analysis was performed on all samples for 76 pesticide compounds; 42 of which were detected at 1 or more stations, in at least 1 sample type (SPMD, POCIS, or grab samples).  Forty-one of the 42 compounds detected were found using a combination of SPMD and grab samples.  Thirty-one compounds were detected at all stations (N = 4).  Detections for 18 compounds were qualified due to lab or field blank detections.

Table 2. Number of Compounds Analyzed in Each Category.

* The FDEP lab used a reduced analyte list for some sampling devices. POCIS samples were not analyzed for 29 pesticide compounds.  SPMD samples were not analyzed for 1 pesticide compounds. **SPMD samples were not analyzed for PPCP by the SGS AXYS lab.

| Category | Sampling Devices Used | Study Samples: Number of Compounds Analyzed |  |
| --- | --- | --- | --- |
|  |  | SGS AXYS Lab | FDEP Lab |
| Pesticides | POCIS, SPMD, & Grab | 76 | 88* |
| Pharmaceuticals and Personal Care Products (PPCP) | POCIS, SPMD**, & Grab | 127 | 4 |
| Hormones | POCIS & Grab | 17 | N/A |
| Perfluorinated Compounds (PFC) | POCIS & Grab | 13 | N/A |
| Polybrominated Diphenyl Ethers (PBDE) | SPMD & Grab | 8 | N/A |
| Alkylphenols | SPMD & Grab | 4 | N/A |

Table 3. Assay Categories with Assay Name and Entity that Conducted Assay.

| Assay Category | Assay Name | Entity that conducted assay |
| --- | --- | --- |
| Receptor Binding Assay | Estrogen and androgen receptor binding by fluorescence polarization | UF |
| Receptor Transactivation Assay | Estrogen and androgen transactivation assay by using both agonist and antagonist modes | UF |
| Receptor Binding Assay | Bioluminescent yeast estrogen and androgen screens (BLYES and BLYAS) | USGS |
| Genotoxicity Assay | SOS Chromotest | USGS |

## 3.2 Pharmaceuticals and Personal Care Products (PPCPs) – SGS AXYS Lab

Analysis was performed on POCIS and grab samples for 127 PPCP compounds; 39 were detected at 1 or more stations, in at least 1 sample type.  Of the 39 compounds detected, 31 compounds were found in POCIS samples and 24 compounds were found in grab samples.  Seven compounds were detected at all stations (N = 4).  Detections for 3 compounds were qualified due to lab or field blank detections.

## 3.3 Hormones – SGS AXYS Lab

Analysis was performed on POCIS and grab samples for 17 hormone compounds; 5 compounds were detected at one or more stations, in at least one sample type.  All compounds detected were found in the POCIS samples, and 2 of the compounds detected were found in grab samples.  One compound was detected at all stations (N = 4).  All grab sample detections were qualified due to lab or field blank detections, but no data from POCIS samples were qualified due to this reason.

## 3.4 Perfluorinated Compounds (PFC) – SGS AXYS Lab

Analysis was performed on POCIS and grab samples for 13 PFC compounds; 10 compounds were detected at one or more stations, in at least one sample type.  Eight of the compounds detected were found in the POCIS samples, and seven of the compounds detected were found in grab samples.  Three compounds were detected at all stations (N = 4).  No PFC data were qualified due to lab or field blank detections.

## 3.5 Polybrominated Diphenyl Ethers (PBDE) – SGS AXYS Lab

Analysis was performed on SPMD and grab samples for 8 PBDE compounds, all of which were detected by both sampling types at one or more sites. For SPMD, all compounds were found at all sites, whereas 4 of the 8 were detected at all sites for grab. All SPMD detections and 92% of grab detections were qualified due to lab or field blank detections.

## 3.6 Alkylphenols – SGS AXYS lab

Analysis was performed on SPMD and grab samples for 4 alkylphenol compounds; 2 compounds were detected at all sites (N=4), in at least one sample type.  Both compounds detected were found in the SPMD samples, but only one was found in the grab samples.  All alkyphenol data were qualified due to lab or field blank detections.

## 3.7 Pesticides and PPCP - FDEP lab

Grab samples were analyzed for all 92 compounds (88 pesticides and 4 PPCPs), SPMDs were analyzed for 91 compounds (87 pesticides and 4 PPCPs), and POCIS were analyzed for 63 compounds (59 pesticides and 4 PPCPs).  Thirty-two compounds (29 pesticides and 3 PPCPs) were detected at one or more stations, in at least one sample type. Fourteen compounds were detected at all stations (N = 4).  No data from this laboratory were qualified due to lab or field blank detections.

Table 4. Summary of the compound class detections by sampling method for each of the four study sites.  Refer to Appendix A for a complete list of compounds detected.

| Category | Apalachicola | Ochlockonee | Withlacoochee | Alafia |
| --- | --- | --- | --- | --- |
| Pesticides | P1,2S1,2G1,2 | P1,2S1,2G1,2 | P1,2S1,2G1,2 | P1,2S1,2G1,2 |
| Pharmaceuticals and Personal Care Products (PPCPs) | P1,2G12 | P1,2G1,2 | P1,2S2G1,2 | P1,2G1,2 |
| Hormones | PG | PG | PG | G |
| Perfluorinated Compounds (PFCs) | PG | PG | P | PG |
| Polybrominated Diphenyl Ethers (PBDEs) | SG | SG | SG | SG |
| Alkylphenols | SG | SG | SG | SG |

P = Detected in POCIS samples, S = Detected in SPMD samples, G = Detected in Grab Samples. For Pesticides and PPCP, 1 = samples analyzed by SGS AXYS laboratory, 2 = samples analyzed by FDEP laboratory.

## 3.8 University of Florida Results

The University of Florida conducted bioanalytical assays (receptor binding and transactivation) on whole water grab samples and extracts from the SPMD and POCIS samplers. For the receptor binding assays, interference from background fluorescence was an issue when analyzing the whole water grab and POCIS extract samples. This interference from background fluorescence was likely due to the naturally high Total Organic Carbon (TOC) and humic acids in the water present at the study sites. The high levels of background fluorescence preclude any conclusions from being drawn concerning the presence or absence of estrogenic or androgenic compounds in POCIS and grab samples. However, background fluorescence did not cause notable interference in the interpretation of the binding assay results for the SPMD extracts. For the SPMD extracts, on a qualitative basis, there was a positive indication of estrogen receptor and androgen receptor binding chemicals at each of the study sites. The transactivation assays also yielded variable results due to interference.  Further analysis of additional river sites and use of additional sample clean-up steps for dark water rivers are needed before conclusions about the presence or absence and effects of estrogenic or androgenic compounds can be drawn with confidence.

## 3.9 USGS Results

USGS used the SOS Chromotest to test each of the sample types (POCIS, SPMD, grab) at each river site and on a field blank. All sites were negative for genotoxicity in all sample types, except at the Alafia River site, where the POCIS and SPMD samples tested positive.

USGS also tested samples at all sample sites and from all sample types using the BLYES and BLYAS. The grab samples from the Alafia, Apalachicola, Withlacoochee River sites were found to be estrogenic. POCIS samples were found to be estrogenic at all sites, while SPMD samples were not estrogenic at any site. No androgenic activity was found in any sample type nor any site.

# 4.0 Lessons Learned and Recommendations

Sampling and chemical analyses for EC is a sensitive process; this is evident by the frequent detections in quality control samples (field and lab blanks). Chemical analyses are accompanied by lengthy QC reports which must be carefully reviewed when determining use of the data. Selecting sampling and analysis methods based on the goals and objectives of the study would reduce the data burden in future studies.

Comparison of the concentrations obtained using the three sampling techniques was not possible because of the difference in measurement units between the passive devices and grab samples. This was due, in part, because sampling rates for these compounds were not determined. Alvarez (2010) defines sampling rate as the rate in liters of water extracted by the passive sampler per day (L/d). Furthermore, Water flow, temperature, and buildup of biofilm on the sampler surfaces can affect sampling rates. Deploying passive samplers containing media spiked with (PRC) at the sample sites is one means to the address these environmental variables.  PRC is a chemical added to the sampler during fabrication. By measuring the amount of PRC loss during deployment in the field, adjustments to the sampling rates of targeted chemicals can be made to reflect the site-specific sampling rates (Alvarez, 2010).

Choosing analytes with known site-specific sampling rates would allow for the determination of time-weighted average concentrations. This would provide the measurement unit conversion necessary for concentration comparisons between samples taken from the three sampling methods and therefore provide additional information on ambient concentrations. For the passive sampling devices, time-weighted averages of analyte concentration are sufficient for determining biological exposure rates.

For test protocols that measure estrogenicity and androgenicity, the high level of color in Florida waters caused interference in detections. An increased understanding of site-specific sampling rates for estrogenic and androgenic compounds and further refinement of the laboratory methods used for the determination of estrogenicity and androgenicity are recommended for future studies.

Future studies of EC in Florida’s waterbodies should focus on waterbodies directly impacted by known sources of EC (e.g. river segments continually impacted by wastewater overflows). This will allow FDEP to tailor the analyte list and sampling methods for known compounds. Additional studies should be used to determine waterbodies for EC sampling (e.g. waterbodies known to have water quality impairments due to biologic metrics for which no known pollutant is identified). To study bioaccumulative effects and address concerns related to public health, a fish and sediment study could provide information on the persistence of EC. These studies could pair whole grab samples with biological community (e.g. fish, invertebrates) monitoring to provide information on the current state of the ecosystem and insight on what concentrations affect a targeted community. Finally, periodic statewide sample surveys for indicators of EC should be conducted so that we can track the extent of waters impacted by these compounds.

Supplemental information including a summary of EC detections is available at the following link.

# Acknowledgements

Florida Department of Environmental Protection wish to thank David Alvarez (USGS) and Terri Spencer (EST, Inc.) who walked us through the world of POCIS and SPMD, Luke Iwanowicz (USGS) for providing us with expertise with the BLYES, BLYAS, and SOS Chromotest, and Nancy Denslow, Tara Sabo-Atwood, and Joseph Bisesi (UF) for guiding us through the transactivation and binding assays results.

# 5.0 References

Alvarez, D.A., 2010. Guidelines for the use of the semipermeable membrane device (SPMD) and the polar organic chemical integrative sampler (POCIS) in environmental monitoring studies: U.S. Geological Survey, Techniques and Methods 1–D4, 28 p.

Blazer, V.S., Iwanowicz, L.R., Henderson, H., Mazik, P.M., Jenkins, J.A., Alvarez, D.A., Young, J.A., 2011. Reproductive endocrine disruption in smallmouth bass (Micropterus dolomieu) in the Potomac River basin: spatial and temporal comparisons of biological effects. Environmental Monitoring and Assessment, DOI 10.1007/s10661-011-2266-5.

Blazer, V.S., Pinkney, A.E., Jenkins, J.A., Iwanowicz, L.R., Minkkinen, S., Draugelis-Dale, R.O., Uphoff, J.H., 2013. Reproductive health of yellow perch Perca flavescens in selected tributaries of the Chesapeake Bay. Science of the Total Environment. Vol. 447, pp. 198 – 209.

Bonmatin, J. M., Giorio, C., Girolami, V., Goulson, D., Kreutzweiser, D. P., Krupke, C., Liess, M., Long, E., Marzaro, M., Mitchell, D. A., Simon-Delso, N., & Tapparo, A. (2015). Environmental fate and exposure: neonicotinoids and fipronil. Environmental Science and Pollution Research, 22, 35–67. https://doi.org/10.1007/s11356-014-3332-7.

Chemical Abstracts Service, 2013.  CAS Content at a Glance.  Accessed July 3, 2013 at http://www.cas.org/content/at-a-glance.

FDEP (Florida Department of Environmental Protection). (2015). Florida Watershed Monitoring Status and Trend Program design document, technical document, Division of Environmental Assessment and Restoration, Watershed Monitoring Section. http://publicfiles.dep.state.fl.us/dear/DEARweb/WMS/Reports_Docs_SOPs/Design_Docs/WMS-MonitoringDesignDocument2017.pdf. Accessed 15 Feb. 2019.

Guy, C.P., A.E. Pinkney, V.S. Blazer, L.R. Iwanowicz, D.A. Alvarez, and J.E. Mullican. 2009. Assessment of endocrine disruption in smallmouth bass (Micropterus dolomieu) and largemouth bass Micropterus salmoides) in the Potomac River Watershed. CBFO-C09-01. U.S. Fish and Wildlife Service, Chesapeake Bay Field Office, Annapolis, MD. http://www.fws.gov/chesapeakebay/envcont.html

Jenner,M. R., & Smithson, A. (1989). Physicochemical properties of the sweetener sucralose. Journal of Food Science, 54, 1646–1649. https://doi.org/10.1111/j.1365-2621.1989.tb05179.x.

Kolpin, D.W., Furlong, E.T., Meyer, M.T., Thurman, M., Zaugg, S.D., Barber, L., B., Buxton, H.T., 2002. Pharmaceuticals, hormones, and other organic wastewater contaminants in U.S. streams, 1999-2000—A national reconnaissance. Environmental Science and Technology, Vol. 36, pp. 1202-1211.

Labare, M. P., & Alexander, M. (1993). Biodegradation of sucralose, a chlorinated carbohydrate. In Samples of Natural Environments. Environmental Chemistry, 12, 797–804. https://doi.org/10.1002/etc.5620120502.

Letcher, Robert J., Jan Ove Bustnes, rune Dietz, Buorn M. Jenssen, Even H. Jorgensen, Christian Sonne, jonathan Verreault, Mathilakath M. Vijayan, geir W. Gabrielsen, 2010. Exposure and effects assessment of persistent organohalogen contaminants in arctic wildlife and fish. Science of the Total Environment, Vol. 408, pp. 2995 – 3043.

Oetken, Matthias, Jean Bachmann, Ulrike Schulte-Oehlmann, and Jorg Oehlmann, 2004. Evidence for Endocrine Disruption in Invertebrates. International review of Cytology, Vol. 236.

Oppenheimer, J., Eaton, A., Badruzzaman,M., Haghani, A.W., & Jacangelo, J. G. (2011). Occurrence and suitability of sucralose as an indicator compound of wastewater loading to surface waters in urbanized regions. Water Research, 45, 4019–4027. https://doi.org/10.1016/j.watres.2011.05.014.

Pal, A., Yew-Hoong Gin, K., Y-Chen Lin, A., and Reinhard, M., 2010. Impacts of emerging contaminants on freshwater resources: Review of recent occurrences, sources, fate and effects. Science of the Total Environment, Vol. 408, pp. 6062-6069.

Seal, T., Woeber, N. A., & Silvanima, J. (2016). Using tracers to infer potential extent of emerging contaminants in Florida’s ground waters. Florida Scientist, 79, 279 – 289. https://fas.fit.edu/media/site-specific/fasfitedu/wwwfloridaacademyofsciencesorg/wp-content/uploads/2017/05/flsc-79-04-cov.pdf.

Silvanima, J., Woeber, A., Sunderman-Barnes, S. et al. Environ Monit Assess (2018) 190: 435. https://doi.org/10.1007/s10661-018-6782-4

Tollefsen, K. E., Nizzetto, L., & Huggett, D. B. (2012). Presence, fate and effects of the intense sweetener sucralose in the aquatic environment. Science of the Total Environment, 438, 510–516. https://doi.org/10.1016/j.scitotenv.2012.08.060.

Wishart, D. S., Knox, C., Guo, A. C., Shrivastava, S., Hassanali, M., Stothard, P., Chang, Z.,&Woolsey, J. (2006).

USGS (United States Geological Survey). (2019). Estrogenicity in Streams of New York State. https://www.usgs.gov/centers/ny-water/science/estrogenicity-streams-new-york-state?qt-science_center_objects=0#qt-science_center_objects. Accessed 5 Apr. 2019.

Vos, J.G., Dybing, E., Helmut, A.G., Ladefoged, O., Lambré, C., Tarazona, J.V., Brandt, I., Vethaak, A.D., 2000. Health Effects of Endocrine-Disrupting Chemicals on Wildlife, with Special Reference to the European Situation. Critical Reviews in Toxicology, Vol. 30(1) pp. 71-133

# Appendix A. All Compounds Included in this Study

Pesticide Compounds Analyzed by SGS AXYS Lab.

- 2,4'-DDD

- 2,4'-DDE

- 2,4'-DDT

- 4,4'-DDD

- 4,4'-DDE

- 4,4'-DDT

- Alachlor

- Aldrin

- alpha-Endosulphan

- Ametryn

- Atrazine

- Azinphos-Methyl

- beta-Endosulphan

- Butralin

- Butylate

- Captan

- Chlordane, alpha (cis)

- Chlordane, gamma (trans)

- Chlordane, oxy-

- Chlorothalonil

- Chlorpyriphos

- Chlorpyriphos-Methyl

- Chlorpyriphos-Oxon

- Cyanazine

- Cypermethrin

- Dacthal

- Desethylatrazine

- Diazinon

- Diazinon-Oxon

- Dieldrin

- Dimethenamid

- Dimethoate

- Disulfoton

- Disulfoton Sulfone

- Endosulphan Sulphate

- Endrin

- Endrin Ketone

- Ethalfluralin

- Ethion

- Fenitrothion

- Flufenacet

- Flutriafol

- Fonofos

- HCH, alpha

- HCH, beta

- HCH, delta

- HCH, gamma

- Heptachlor

- Heptachlor Epoxide

- Hexachlorobenzene

- Hexazinone

- Linuron

- Malathion

- Methoprene

- Methoxychlor

- Metolachlor

- Metribuzin

- Mirex

- Nonachlor, cis-

- Nonachlor, trans-

- Octachlorostyrene

- Parathion-Ethyl

- Parathion-Methyl

- Pendimethalin

- Permethrin

- Perthane

- Phorate

- Phosmet

- Pirimiphos-Methyl

- Quintozene

- Simazine

- Tebuconazol

- Tecnazene

- Terbufos

- Triallate

- Trifluralin

PPCP Compounds Analyzed by SGS AXYS Lab.

- 1,7-Dimethylxanthine

- 10-hydroxy-amitriptyline

- 2-Hydroxy-ibuprofen

- Acetaminophen

- Albuterol

- Alprazolam

- Amitriptyline

- Amlodipine

- Amphetamine

- Amsacrine

- Atenolol

- Atorvastatin

- Azathioprine

- Azithromycin

- Benzoylecgonine

- Benztropine

- Betamethasone

- Bisphenol A

- Busulfan

- Caffeine

- Carbadox

- Carbamazepine

- Cefotaxime

- Cimetidine

- Ciprofloxacin

- Citalopram

- Clarithromycin

- Clinafloxacin

- Clonidine

- Clotrimazole

- Cloxacillin

- Cocaine

- Codeine

- Colchicine

- Cotinine

- Cyclophosphamide

- Daunorubicin

- DEET

- Dehydronifedipine

- Desmethyl-diltiazem

- Diatrizoic acid

- Diazepam

- Digoxigenin

- Digoxin

- Diltiazem

- Diphenhydramine

- Doxorubicin

- Drospirenone

- Enalapril

- Enrofloxacin

- Erythromycin-H2O

- Etoposide

- Flumequine

- Fluocinonide

- Fluoxetine

- Fluticasone propionate

- Furosemide

- Gemfibrozil

- Glipizide

- Glyburide

- Hydrochloro-thiazide

- Hydrocodone

- Hydrocortisone

- Ibuprofen

- Iopamidol

- Lincomycin

- Lomefloxacin

- Medroxy-progesterone Acetate

- Melphalan

- Meprobamate

- Metformin

- Methyl-prednisolone

- Metoprolol

- Metronidazole

- Miconazole

- Moxifloxacin

- Naproxen

- Norfloxacin

- Norfluoxetine

- Norgestimate

- Norverapamil

- Ofloxacin

- Ormetoprim

- Oxacillin

- Oxazepam

- Oxolinic Acid

- Oxycodone

- Paroxetine

- Penicillin G

- Penicillin V

- Prednisolone

- Prednisone

- Promethazine

- Propoxyphene

- Propranolol

- Ranitidine

- Rosuvastatin

- Roxithromycin

- Sarafloxacin

- Sertraline

- Simvastatin

- Sulfachloro-pyridazine

- Sulfadiazine

- Sulfadimethoxine

- Sulfamerazine

- Sulfamethazine

- Sulfamethizole

- Sulfamethoxazole

- Sulfanilamide

- Sulfathiazole

- Tamoxifen

- Teniposide

- Theophylline

- Thiabendazole

- Trenbolone

- Trenbolone acetate

- Triamterene

- Triclocarban

- Triclosan

- Trimethoprim

- Tylosin

- Valsartan

- Venlafaxine

- Verapamil

- Virginiamycin M1

- Warfarin

- Zidovudine

Hormone Compounds Analyzed by SGS AXYS Lab

- 17 alpha-Dihydroequilin

- 17 alpha-Estradiol

- 17 alpha-Ethinyl-Estradiol

- 17 beta-Estradiol

- Allyl Trenbolone

- Androstenedione

- Androsterone

- Desogestrel

- Equilenin

- Equilin

- Estriol

- Estrone

- Mestranol

- Norethindrone

- Norgestrel

- Progesterone

- Testosterone

PFC Compounds Analyzed by SGS AXYS Lab

- Pentafluorobenzoic acid (PFBA)

- Perfluorobutanesulfonic acid (PFBS)

- Perfluorodecanoic acid (PFDA)

- Perfluorododecanoic acid (PFDoA)

- Perfluoroheptanoic acid (PFHpA)

- Perfluorohexanoic acid (PFHxA)

- Perfluorohexanesulfonic acid (PFHxS)

- Perfluorononanoic acid (PFNA)

- Perfluorooctanoic acid (PFOA)

- Perfluorooctanesulfonic acid (PFOS)

- Perfluorooctanesulfonamide (PFOSA)

- Perfluoro-n-pentanoic acid (PFPeA)

- Perfluoroundecanoic acid (PFUnA)

PBDE Compounds Analyzed by SGS AXYS Lab

- 2,2',3,3',4,4',5,5',6,6'-DeBDE

- 2,2',3,4,4',5',6-HpBDE

- 2,2',4,4',5,5'-HxBDE

- 2,2',4,4',5,6'-HxBDE

- 2,2',4,4',5-PeBDE

- 2,2',4,4',6-PeBDE

- 2,2',4,4'-TeBDE

- 2,4,4'-TriBDE

Alkylphenol Compounds Analyzed by SGS AXYS Lab

- 4-n-Octylphenol

- 4-Nonylphenol diethoxylates

- 4-Nonylphenol monoethoxylates

- 4-Nonylphenols

Pesticide Compounds Analyzed by DEP Lab

- 2,4-D

- 3-Hydroxycarbofuran

- Acetochlor

- Alachlor

- Aldicarb

- Aldicarb Sulfone

- Aldicarb Sulfoxide

- Aldrin

- Alpha-BHC

- Alpha-Chlordane

- Ametryn

- Atrazine

- Atrazine Desethyl

- Azinphos Methyl

- Bentazon

- Beta-BHC

- Bromacil

- Butylate

- Carbaryl

- Carbofuran

- Carbophenothion

- Chlordane

- Chlorothalonil

- Chlorpyrifos Ethyl

- Chlorpyrifos Methyl

- Cyanazine

- Cypermethrin

- DDD-p,p'

- DDE-p,p'

- DDT-p,p'

- Delta-BHC

- Demeton

- Diazinon

- Dicofol

- Dieldrin

- Disulfoton

- Diuron

- Endosulfan I

- Endosulfan II

- Endosulfan Sulfate

- Endrin

- Endrin Aldehyde

- Endrin Ketone

- EPTC

- Ethion

- Ethoprop

- Fenamiphos

- Fenuron

- Fipronil

- Fipronil Sulfide

- Fipronil Sulfone

- Fluridone

- Fonofos

- Gamma-BHC

- Gamma-Chlordane

- Glyphosate*

- Heptachlor

- Heptachlor Epoxide

- Hexazinone

- Imidacloprid

- Linuron

- Malathion

- MCPP

- Metalaxyl

- Methiocarb

- Methomyl

- Methoxychlor

- Metolachlor

- Metribuzin

- Mevinphos

- Mirex

- Molinate

- Norflurazon

- Oxamyl

- Parathion Ethyl

- Parathion Methyl

- Pendimethalin

- Permethrin

- Phorate

- Prometon

- Prometryn

- Propoxur

- Simazine

- Terbufos

- Terbuthylazine

- Toxaphene

- Triclopyr

- Trifluralin

PPCP Compounds Analyzed by DEP lab

- Acetaminophen

- Carbamazepine

- Primidone

- Sucralose