Results

- Sucralose levels detected from previous DEP studies determined sampling sites.

- Sampling equipment performance and costs indicates that equipment should be carefully selected for optimum performance. Project purpose, analyte type, and analysis method should be considered prior to selection.

- Samples collected for hormone analysis are easily contaminated and require special handling which may not be feasible for staff.

- Laboratory chemical analyses and biological assays were used to assess the surface water samples.

- All 4 sites sampled had a potential for the presence of estrogenic and androgenic

- Hydrophobic compounds, like PBDEs and alkylphenols, were detected more often in SPMDs than Grab samples. activity (getting confirmation from UF)

- The following sampling devices performed best, based on number of priority compounds detected.

  - A combination of Grab and SPMD performed best for Pesticides.

  - POCIS performed slightly better than Grab for PPCPs, Hormones, and PFCs.

  - SPMD and Grab performed similarly for PBDEs and Alkylphenols.

- Estrogenic activity, as measured by the BLYES, was present in samples from all pilot study stations.

- Genotoxic agents, as measured by the SOS Chromotest, were present at one of the four pilot study