ATTACHMENT __

Scope of Services – Chemical Analysis of Emerging Substances of Concern

Under this contract, the Contractor will provide, on a task assignment basis, laboratory and related services to support certain Department environmental investigations associated with emerging substances of concern (ESOC).  The Contractor’s services on each provided sample shall include chemical analyses as specified in Exhibit A for the associated compounds listed in Exhibits B – G, chemical analyses as specified in Attachment __ for quality assurance purposes, a complete assessment of all reported data, and a determination of data suitability for DEP program needs.  The Contractor shall furnish the necessary personnel, materials, facilities and services necessary for the completion of all assigned tasks.  The Contractor shall assume full responsibility for understanding the Department program objectives associated with assigned tasks, including quality and usability objectives, and shall provide services sufficient to meet or exceed programmatic goals.

The scope of specific service projects will be accomplished through the use of task assignments.  No work on any task shall begin until a task assignment outlining the work has been issued by the Department to the Contractor.  Each task assignment shall specify the approximate number of samples to be analyzed, the category or categories of ESOCs in Exhibit A for which the associated suite of analytes in Exhibits B – G shall be analyzed, and the maximum costs that can be incurred by the Department under the task.  When the Department tasks the Contractor to provide services, the cost of those services shall only be calculated from the rates provided by the Contractor (listed in _______) for the suite of analytes in each ESOC category (listed in Exhibit A) that include the compounds listed in Exhibits B – G, as well as any associated analyses needed for quality assurance purposes.

The analytes requested for chemical analysis are grouped in Exhibit A into the following ESOC categories:

- Polybrominated Flame Retardants (PBDEs)

- Pharmaceuticals And Personal Care Products (PPCPs)

- Hormones

- Nonylphenols And Octylphenols

- Pesticides

- Perfluorinated Compounds (PFCs)

Quality Objective:

These services require the ability, as demonstrated by prior experience, to perform chemical analysis of ESOC categories listed in Exhibit A that include at a minimum, the analytes listed in Exhibits B - G for:

- Environmental samples that are aqueous whole water/effluents, soil, sediment, tissue and biosolids as determined by the analytical method

- Water/effluent for the extracts and dialysis, respectively, from the integrative devices, polar organic compound integrative sampler and semipermeable membrane device as specified in Exhibit A.

Analytical sensitivity, precision, accuracy, consistency and comparability are crucial to the success of the Department’s projects.  It will be incumbent upon the Contractor to fully understand analytical quality expectations associated with assigned projects and to adequately anticipate and respond to those needs.  The Contractor’s quality philosophy must be reflected in their Quality Manual.  The Contractor must have documented performing data quality assessments and product usability assessments as well as mechanisms for performing corrective actions, reporting non-conformances and for handling complaints.  Upon the Department’s request, clear documentation of those protocols must be provided by the Contractor to the Department.

Deliverables:

Unless otherwise specified in a particular task assignment, within 45 business days of receipt of a sample from the Department, the Contractor shall complete all associated laboratory analyses on the sample(s), and have electronically submitted to the Department the analysis results for the applicable parameters in Exhibits B – G, as well as any applicable quality control parameters, for the sample(s) in a Standard AXYS EDD format.

Performance Measures:

The Department’s Contract Manager will review the deliverable(s) to verify that it meets all of the specifications in this Scope of Work and in the associated task assignment.  Upon review and written acceptance of all the task deliverables by the Department’s Contract Manager, the Contractor may proceed with the payment request submittal for costs associated with the task assignment.

Exhibit A

| ESOC Analyte Category | ESOC Analytical Methodology |
| --- | --- |
| Determination of PPCP compounds in environmental samples | Vendor determines PPCP compounds in environmental samples and quality assurance/quality control (QA/QC) blanks by high performance liquid chromatography combined with triple quadrupole mass spectrometry (LC-MS/MS) as described in either EPA 1694 or mod. EPA method 1694 for use with aqueous, sediment, soil, and biosolids matrices, and/or POCIS extracts. The POCIS elution method may be a published in-house method. |
| Determination of hormone compounds in environmental samples | Vendor determines hormone compounds in environmental samples and QA/QC blanks by isotope dilution and internal standard high resolution gas chromatography combined with high resolution mass spectrometry (HRGC/HRMS) as described in either EPA method 1698 or mod. EPA method 1698 for use with aqueous, soil, sediment, and biosolids matrices. By LC-MS/MS that is run in the MRM mode for use with POCIS extracts as determined in a published in-house method. The vendor will perform POCIS extraction by a method that may be a published in-house method. |
| Determination of pesticide compounds in environmental samples | Vendor determines any and/or all of the following by request from the Department: organochlorine, organophosphorus, triazine, and pyrethroid pesticides in environmental samples and QA/QC blanks by HRGC/HRMS using isotope dilution and internal standard quantitation techniques as described in either EPA Method 1699 or mod. EPA Method 1699. EPA Method 1699 or mod. EPA Method 1699 shall be used to determine the organonitrogen pesticides for use with aqueous, solid, tissue and biosolids matrices, POCIS extracts and SPMD dialysis extracts. The vendor will perform POCIS extraction and SPMD dialysis by a method published in house method. |
| Determination of PFC compounds in environmental samples | Vendor determines PFC in environmental samples in aqueous matrices, QA/QC blanks and POCIS extracts by LC-MS/MS combined with a triple quadrupole mass spectrometer. The vendor will perform POCIS extraction by a method that may be a published in-house method. |
| Determination of PBDE compounds in environmental samples | Vendor determines PBDE compounds in environmental samples and QA/QC blanks by isotope dilution and internal standard HRGC/HRMS as described in either EPA method 1614 or mod. EPA Method 1614 for use with aqueous, solid, tissue and multi-phase matrices. Determines brominated diphenyl ethers in SPMD dialysis extracts from EPA method 1614 or mod. EPA Method 1614. The vendor will perform SPMD dialysis by a method that may be a published in-house method |
| Determination of octylphenol and nonylphenol compounds in environmental samples | Vendor determines octylphenol and nonylphenol compounds in environmental samples and QA/QC blanks by capillary gas chromatography column coupled with low resolution mass spectrometry (GCMS) in aqueous, solid matrices and SMPD dialysis extracts. The vendor will perform SPMD dialysis by a method that may be a published in-house method. |

| DEP PPCP list | Instrumentation/Method for environmental samples in aqueous, sediment, soil, and biosolid matrices, POCIS extracts |
| --- | --- |
| 2-Hydroxy-ibuprofen | LC/MC/MS (MRM) triple quad ms (mod.EPA 1694) |
| Atenolol | LC/MC/MS (MRM) triple quad ms (mod.EPA 1694) |
| Bisphenol A | LC/MC/MS (MRM) triple quad ms (mod.EPA 1694) |
| Carbamazepine | LC/MC/MS (MRM) triple quad ms (mod.EPA 1694) |
| Diphenhydramine | LC/MC/MS (MRM) triple quad ms (mod.EPA 1694) |
| Diltiazem | LC/MC/MS (MRM) triple quad ms (mod.EPA 1694) |
| Fluoxetine | LC/MC/MS (MRM) triple quad ms (mod.EPA 1694) |
| Furosemide | LC/MC/MS (MRM) triple quad ms (mod.EPA 1694) |
| Gemfibrozil | LC/MC/MS (MRM) triple quad ms (mod.EPA 1694) |
| Hydrochlorothiazide | LC/MC/MS (MRM) triple quad ms (mod.EPA 1694) |
| Oxycodone | LC/MC/MS (MRM) triple quad ms (mod.EPA 1694) |
| Ibuprofen | LC/MC/MS (MRM) triple quad ms (mod.EPA 1694) |
| Metoprolol | LC/MC/MS (MRM) triple quad ms (mod.EPA 1694) |
| Naproxen | LC/MC/MS (MRM) triple quad ms (mod.EPA 1694) |
| Sertraline | LC/MC/MS (MRM) triple quad ms (mod.EPA 1694) |
| Sulfamethoxozole | LC/MC/MS (MRM) triple quad ms (mod.EPA 1694) |
| Trimethoprim | LC/MC/MS (MRM) triple quad ms (mod.EPA 1694) |
| Triamterene | LC/MC/MS (MRM) triple quad ms (mod.EPA 1694) |
| Triclosan | LC/MC/MS (MRM) triple quad ms (mod.EPA 1694) |
| Valsartan | LC/MC/MS (MRM) triple quad ms (mod.EPA 1694) |
| Verapamil | LC/MC/MS (MRM) triple quad ms (mod.EPA 1694) |
| Analytes on the list for many reasons: subject to public perception, widely used, or have a higher occurrence than others in same usage category (e.g., not all beta blockers will be requested). |  |

Exhibit B

Exhibit C

| DEP PFC List | Acronym | Instrumentation/method for environmental samples in aqueous matrices and POCIS extracts |
| --- | --- | --- |
| Perfluorobutanesulfonic acid | PFBS | HPLC-MS/MS coupled to triple quad ms |
| Perfluorodecanoic acid | PFDA | HPLC-MS/MS coupled to triple quad ms |
| Perfluorododecanoic acid | PFDoA | HPLC-MS/MS coupled to triple quad ms |
| Perfluoroheptanoic acid | PFHpA | HPLC-MS/MS coupled to triple quad ms |
| Perfluorohexanesulfonic acid | PFHxS | HPLC-MS/MS coupled to triple quad ms |
| Perfluorohexanoic acid | PFHxA | HPLC-MS/MS coupled to triple quad ms |
| Perfluorononanoic acid | PFNA | HPLC-MS/MS coupled to triple quad ms |
| Perfluorooctanesulfonic acid | PFOS | HPLC-MS/MS coupled to triple quad ms |
| Perfluorooctanoic acid | PFOA | HPLC-MS/MS coupled to triple quad ms |

Exhibit D

| DEP Hormone list | Instrumentation (method) for environmental samples in aqueous, soil, sediment, and biosolid matrices | Instrumentation (method) for POCIS extraction |
| --- | --- | --- |
| Androstenedione | HRGC/HRMS (EPA 1698 or mod. EPA 1698) | HPLC-MSMS |
| Androsterone | HRGC/HRMS (EPA 1698 or mod. EPA 1698) | HPLC-MSMS |
| Desogestrel | HRGC/HRMS (EPA 1698 or mod. EPA 1698) | HPLC-MSMS |
| 17α-Dihydroequilin | HRGC/HRMS (EPA 1698 or mod. EPA 1698) | HPLC-MSMS |
| Equilenin | HRGC/HRMS (EPA 1698 or mod. EPA 1698) | HPLC-MSMS |
| Equilin | HRGC/HRMS (EPA 1698 or mod. EPA 1698) | HPLC-MSMS |
| 17α-Estradiol | HRGC/HRMS (EPA 1698 or mod. EPA 1698) | HPLC-MSMS |
| 17α-Ethynyl Estradiol | HRGC/HRMS (EPA 1698 or mod. EPA 1698) | HPLC-MSMS |
| 17β-Estradiol | HRGC/HRMS (EPA 1698 or mod. EPA 1698) | HPLC-MSMS |
| Estriol | HRGC/HRMS (EPA 1698 or mod. EPA 1698) | HPLC-MSMS |
| Estrone | HRGC/HRMS (EPA 1698 or mod. EPA 1698) | HPLC-MSMS |
| Mestranol | HRGC/HRMS (EPA 1698 or mod. EPA 1698) | HPLC-MSMS |
| Norethindrone | HRGC/HRMS (EPA 1698 or mod. EPA 1698) | HPLC-MSMS |
| Norgestrel | HRGC/HRMS (EPA 1698 or mod. EPA 1698) | HPLC-MSMS |
| Progesterone | HRGC/HRMS (EPA 1698 or mod. EPA 1698) | HPLC-MSMS |
| HRGC/MS is a more suitable method for hormones in whole water rather than LCMS/MS because HR is 5X lower reporting limits and is therefore more suitable for whole water samples. |  |  |

Exhibit E

| DEP Octylphenols and Nonylphenol list | Instrumentation (method) for environmental samples in aqueous matrices and SPMD dialysis |
| --- | --- |
| Nonylphenol (NP) | Capillary gas chromatography column coupled LRMS |
| 4-Nonylphenol monoethoxylate (NP1EO) | Capillary gas chromatography column coupled LRMS |
| 4-Nonylphenol diethoxylate (NP2EO) | Capillary gas chromatography column coupled LRMS |
| 4-n-Octylphenol | Capillary gas chromatography column coupled LRMS |

Exhibit F

| DEP PBDE analyte list | Congener | Instrumentation (method) for environmental samples in aqueous, solid, and multi-phase matrices and SPMD dialysis |
| --- | --- | --- |
| 2,4,4'-TrBDE | 28 | HRGCMS (EPA 1614 or mod. EPA 1614) |
| 2,2',4,4'-TeBDE | 47 | HRGCMS (EPA 1614 or mod. EPA 1614) |
| 2,2',4,4',5-PeBDE | 99 | HRGCMS (EPA 1614 or mod. EPA 1614) |
| 2,2',4,4',6-PeBDE | 100 | HRGCMS (EPA 1614 or mod. EPA 1614) |
| 2,2',4,4',5,5'-HxBDE | 153 | HRGCMS (EPA 1614 or mod. EPA 1614) |
| 2,2',4,4',5',6-HxBDE | 154 | HRGCMS (EPA 1614 or mod. EPA 1614) |
| 2,2',3,4,4',5',6-HpBDE | 183 | HRGCMS (EPA 1614 or mod. EPA 1614) |
| DeBDE | 209 | HRGCMS (EPA 1614 or mod. EPA 1614) |

Exhibit G

| DEP Pesticide list | Instrumentation/Method for environmental samples in aqueous, sediment, soil, and biosolid matrices, POCIS extracts and SPMD dialysis |
| --- | --- |
| Organochlorine Analytes | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| 2,4'-Ddd 1 | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| 2,4'-Dde 1 | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| 2,4'-Ddt | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| 4,4'-Ddd 1 | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| 4,4'-Dde 1 | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| 4,4'-Ddt | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Aldrin | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Alpha-Endosulphan | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Beta-Endosulphan | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Captan. | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| C-Chlordane | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Chlorothalonil. | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| C-Nonachlor | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Dacthal. | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Dieldrin | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Endosulphan-Sulphate | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Endrin | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Endrin-Ketone | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Hch, Alpha | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Hch, Beta | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Hch, Delta | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Heptachlor | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Heptachlor-Epoxide | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Hexachlorobenzene (Hcb) | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Methoxychlor | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Mirex | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Octachlorostyrene | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Oxychlordane | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Perthane | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Quintozene | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Chlordane | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Tecnazene | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Nonachlor | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Triazine Pesticides | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Ametryn | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Atrazine | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Cyanazine. | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Desethylatrazine | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Hexazinone | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Metribuzin | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Simazine | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Organophosphate Analytes | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Chlorpyriphos | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Chlorpyrifos-Methyl | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Chlorpyrifos-Oxon | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Diazinon | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Diazinon-oxon | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Dimethoate | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Disulfoton Sulfone 3 | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Disulfoton 3 | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Ethion | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Fenitrothion | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Fonofos | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Malathion | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Parathion-Ethyl | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Parathion-Methyl | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Phorate | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Phosmet | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Pirimiphos-Methyl | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Terbufos | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Azinphos-Methyl | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Pyrethroid Analytes | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Total-Permethrins | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Total-Cypermethrins | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Organonitrogen Analytes | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Alachlor | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Butralin | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Butylate | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Dimethenamid | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Ethalfluralin | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Flufenanecet | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Flutriafol | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Linuron | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Methoprene | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Metolachlor | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Pendimethalin | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Tebuconazol | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Triallate | HRGC/MS (EPA 1699 or mod. EPA 1699) |
| Trifluralin | HRGC/MS (EPA 1699 or mod. EPA 1699) |