LAKE ECOSUMMARY

[Lake Name], [WBID], [County], [Sampling dates]

[site picture (optional)]

Figure 1.  [Site Name] in [x] County.  [put figure captions below figures]

DEP conducted water quality and biological sampling at [name of Lake] (STORET xxxxxx, [SBIO Nickname]) in [x] County on [sampling date and date] to assess attainment of designated uses. [This lake was sampled “as part of a strategic monitoring program,” “as part of a statewide random sampling network,” “as part of a reference lake sampling project,” or “in response to a homeowner request”].   Overall, the water quality and plant community data indicated that the lake [met/did not meet/or if, inconclusive, provide further explanation] expectations for a healthy, well-balanced lake.

Background

Healthy, well-balanced lake communities may be maintained with some level of human activity, but excessive human disturbance may result in lake degradation.  Human stressors include increased inputs of nutrients, sediments and/or pesticides from watershed runoff, undesirable removal of native shoreline and/or upland buffer vegetation, and introduction of nuisance (generally exotic) plants and animals.  DEP has methods to evaluate if human activities have resulted in a specific waterbody exceeding water quality criteria (Chapter 62-302, Florida Administrative Code [F.A.C.]), including whether adverse impacts to biological communities have occurred.  DEP water quality standards are designed to protect designated uses of the waters of the state (e.g., recreation, aquatic life support), and exceedances of these standards are associated with interference with the designated use.  DEP assesses the health of plant communities in Florida lakes, as one indication of whether adverse impacts to biological communities have occurred.

Site Description

[Include relevant description of site and location, including map.  Reference Figures 1 and 2 within the text.]

Figure 2.  Location of [Site Name] in [x] County.  [put figure captions below figures]

The predominant land uses surrounding [lake name] are [LU 1], [LU 2], and [LU 3].  This lake is/is not currently listed for [include any listed parameters on most recent verified list] on the Impaired Waters Verified List (date of list).  Additional information about this WBID is available in the TMDL Tracker application (http://webapps.dep.state.fl.us/DearTmdl/tmdlReportAction.do?method=report).

[Insert additional discussion, photos, or anything you think would be informative for the audience. If there is relevant historical information, include it here, e.g., graph of chlorophyll a or LVI over time, discussion of past results, and indication of whether or not the site is on the TMDL list for anything, with a link to TMDL tracker.]

Methods

Water Quality

This lake was sampled on [date and date] by the DEP [Tallahassee office or X Regional Operations Center (ROC)].  Surface water [or other depth] samples were collected from the center of the lake [or other location if different] for analysis of [List analytes here (nutrients, chlorophyll a, color, etc.)] following DEP Standard Operating Procedures (SOPs; see http://dep.state.fl.us/water/sas/sop/sops.htm).  Sampling and analyses met DEP quality assurance/quality control standards.  Results were compared with applicable Class III [typically, but change if it’s another class] water quality criteria contained in 62-302, F.A.C., including nutrients, dissolved oxygen, and other indicators.

Chlorophyll a is a measure of algal biomass in the water column.  Chapter 62-302.531, F.A.C., provides numeric criteria for chlorophyll a, total nitrogen (TN), and total phosphorus (TP) in lakes, and is dependent upon the long term mean color and alkalinity of the lake (Table 1).  If the annual geometric mean chlorophyll a (calculated with at least four samples, representing seasonal variability) does not exceed the chlorophyll a value for the lake type in Table 1, then the TN and TP criteria for that calendar year shall be the annual geometric means of lake TN and TP samples, subject to the minimum and maximum limits in Table 1.  If there are insufficient data to calculate the annual geometric mean chlorophyll a for a given year or the annual geometric mean chlorophyll a exceeds the values in Table 1 for the lake type, then the applicable numeric interpretations for TN and TP shall be the minimum values in Table 1.  Based on the color and alkalinity results presented in this report, Lake [Lake Name] is a low/high color, low/high alkalinity lake.

Table 1. Numeric nutrient criteria in lakes, 62-302.531 (2)(b)(1), F.A.C.  AGM = annual geometric mean

| Long Term Geometric Mean Lake Color and Alkalinity | AGM Chlorophyll a | AGM TP Range | AGM TN Range |
| --- | --- | --- | --- |
| > 40 Platinum Cobalt Units | 20 µg/L | 0.05 to 0.161 mg/L | 1.27 to 2.23 mg/L |
| ≤ 40 Platinum Cobalt Units and > 20 mg/L CaCO3 | 20 µg/L | 0.03 to 0.09 mg/L | 1.05 to 1.91 mg/L |
| ≤ 40 Platinum Cobalt Units and ≤ 20 mg/L CaCO3 | 6 µg/L | 0.01 to 0.03 mg/L | 0.51 to 0.93 mg/L |

1 For lakes with color > 40 PCU in the West Central Region, the maximum TP limit is 0.49 mg/L

Dissolved Oxygen

Rule 62-302.533 (1), F.A.C., states that no more than 10 percent of the daily average percent dissolved oxygen (DO) saturation values shall be below 67 percent in the Panhandle West bioregion, 38 percent in the Peninsula and Everglades bioregions, or 34 percent in the Northeast and Big Bend bioregions. This site is in the [note DO region, which is same as SCI region] region for DO criteria assessment. Percent saturation takes into account factors such as temperature, atmospheric pressure, and salinity. For lakes, the daily average DO level shall be calculated as the average of measurements collected in the upper two meters of the water column at the same location on the same day.

The daily average freshwater DO criteria is preferentially assessed using daily average values calculated from full days of diel (collected over a 24 hour period) monitoring data.  If diel monitoring data are not available [as was the case for this report], instantaneous samples are used to assess the DO criterion by comparing the instantaeous value with a time-of-day-specific translation of the daily average criterion (62-303.420(9), F.A.C.), and a spreadsheet calculator for this purpose is available at:  http://www.dep.state.fl.us/water/wqssp/swq-docs.htm.

[If a report is done for a natural background DO study, include the following:]  Rule 62-302.533(3), F.A.C., states that, “If it is determined that the natural background DO saturation in the waterbody (including values that are naturally low due to vertical stratification) is less than the applicable criteria stated above, the applicable criteria shall be 0.1 mg/L below the DO concentration associated with the natural background DO saturation level.” The FDEP method to demonstrate the appropriateness of a reference site to make a natural background determination includes a demonstration that the proposed reference watershed has been minimally affected by human activities, including point source discharges and nonpoint source inputs.

DEP (with EPA approval) has identified minimally disturbed stream and lake reference sites by application of the following criteria:

- Landscape Development Intensity Index (LDI) (Brown and Vivas 2007) score < 2 for land use within the 100 meter corridor 10 km upstream of the sample site. LDI scores higher than 3 indicate increasing levels of human disturbance;

- Watershed or near-field LDI scores < 3, indicating that that human disturbance is minimal throughout the watershed;

- Presence of relatively benign land uses or human inputs, as discerned by using aerial photographs, field observations, and DEP Best Professional Judgment, that would support the conclusion that the site is minimally disturbed; and

- Characterized by healthy biology (2 temporally independent [sampled at least 3 months apart] LVIs with an average score > 43).

Lake Vegetation Index

The Lake Vegetation Index (LVI) assesses how closely the plant community of a lake resembles a native undisturbed community.  The LVI was sampled and calculated per DEP SOP LVI 1000 and calculated per DEP SOP LVI 2000.   Species lists were developed for four of twelve sections of the lake (Figure 3), and the following information was derived from those lists: percent native species, percent Category 1 invasive exotic species as identified by the Florida Exotic Pest Plant Council, percent sensitive species, and the coefficient of conservatism (C of C; a measure of how tolerant a species is of disturbance) of the dominant or co-dominant species.  Chapter 62-303.330 and 62-303.430, F.A.C., provide that a LVI score of 43 or greater meets the expectation of a healthy, well balanced community, and scores below 43 are considered impaired.

Figure 3. LVI sampling map of [Lake Name].  Sections [1,4,7,10] were sampled for the Lake Vegetation Index.  The water quality sample was collected from the lake center.

Results

Water Quality

The water quality results are shown in Table 2.  [Describe any exceedances of standards and whether they may be due to natural conditions (e.g., naturally low DO) or local human disturbance.  Make sure DO exceedances are based on the time-of-day adjustment calculations.]

Based on these two sampling events, the lake met/exceeded the nutrient thresholds for low/high color, low/high alkalinity lakes; however, the thresholds represent annual geometric mean concentrations (minimum of four samples) not to be exceeded more than once in any three-calendar year period.  DEP’s Watershed Assessment Section will evaluate the complete dataset for this lake to assess compliance with nutrient criteria.

Table 2. Water quality results from [date and date] at [Lake Name].

| Analyte | Result [date] | Result [date] | Applicable Class III Water Quality Criteria |
| --- | --- | --- | --- |
| Field Temperature (°C) |  |  |  |
| Field pH (SU) |  |  | Within 1 SU of natural background |
| Field Dissolved Oxygen (% saturation) |  |  | *[insert criterion from calculator] |
| Field Specific Conductance (µmhos/cm) |  |  | 50% above background or 1275 µmhos/cm, whichever is greater |
| Alkalinity (mg CaCO3/L) |  |  | Shall not be depressed below 20 |
| Color (PCU) |  |  | N/A |
| Chlorophyll a (µg/L) |  |  | ≤ 20 µg/L for colored and alkaline clear lakes |
| Total Phosphorus (mg/L) as P |  |  | [insert applicable range] |
| Nitrate+Nitrite (mg/L) as N |  |  |  |
| Ammonia (mg/L) as N |  |  | **[insert criterion from calculator] |
| Total Kjeldahl Nitrogen (mg/L) as N |  |  | N/A |
| Total Nitrogen (mg/L) as N |  |  | [insert applicable range] |

* Instantaneous DO criterion calculated per DO Saturation Calculator.

** Total ammonia criterion calculated per Total Ammonia Nitrogen Calculator

Phytoplankton [if collected]

The algal community in the water column consisted of x% cyanobacteria (blue-green algae), x% green algae, x% diatoms, etc.  Dominance in algal groups varies naturally throughout the year, but prolonged dominance of cyanobacteria may be associated with a higher risk for harmful algal blooms. [add any specific interpretation of lake results]

Lake Vegetation Index

The LVI score for this lake was [LVI score] out of a possible 100 points, corresponding with a "[Exceptional (≥78), Healthy (43-77), or Not Healthy]" designation.  Table 3 contains the species list and occurrence information for this sampling event.  A total of [number of FLEPPC species] invasive exotic plants were observed in the lake.

Table 3. Species list for the [date] LVI at [Lake Name].  An asterisk (*) indicates an invasive exotic plant species.  P = present, D = dominant, C = codominant.

[See http://www.dep.state.fl.us/water/bioassess/plantid.htm for instructions on how to make Table 3, including linking common names, formatting, and inserting table into report.]

[Insert additional discussion, photos, or anything you think would be informative for the audience. If photos, just be sure to reference them in the text.  If not healthy, mention any planned follow-up to verify condition or mitigate potential sources]

Conclusions

Sampling at [Site Name] [was inconclusive OR] indicates that the lake met/did not meet applicable State Water Quality Criteria.

Met Use Example

Water quality results were well within acceptable limits for chlorophyll a, total nitrogen and total phosphorus on [sampling date(s)].  The Lake Vegetation Index indicated plant communities that are consistent with expectations for a healthy, well-balanced lake.

Not Met Use Example

High nutrient levels measured in [Site Name] were associated with imbalances in the lake’s floral community.  High chlorophyll a measured in [Site Name] indicates that phytoplankton has become the primary consumer of the lake’s elevated nutrients instead of beneficial, native aquatic plants.  Consequently, the LVI score was less than expected for a healthy lake.  [Mention any planned follow-up to verify condition or mitigate potential sources.]

Thank you for your interest in maintaining the water quality of Florida’s lakes. Please contact us if you have any questions.

Contact and resources for more information

[your name, phone number, and e-mail address]

DEP publications on Best Management Practices and Environmental Stewardship and Education:  http://www.dep.state.fl.us/water/nonpoint/pubs.htm

DEP biological assessment resources: http://www.dep.state.fl.us/water/bioassess/index.htm

FWCC Aquatic Plant Management: http://myfwc.com/wildlifehabitats/invasive-plants/

Freshwater Algal Bloom information: http://www.dep.state.fl.us/water/bgalgae/index.htm