STREAM ECOSUMMARY

[Site Name (e.g., “Rice Creek Upstream of SR 100”)], [WBID], [County], [Sampling dates]

[site picture (optional)]

Figure 1. Sampling reach along the [Site Name] in [x] County.

Physical, chemical, and biological sampling was conducted at [site name] (STORET xxxxxx, SBIO Nickname) in [x] County on [sampling date and date] by [samplers] of the DEP [Tallahassee or x DEAR Field] Office.  This stream was sampled [ “as part of strategic monitoring program” or “in response to a homeowner request,” “NPDES assessment”, “to determine if the observed low dissolved oxygen values are natural and result in full support of aquatic life uses” etc.].  Overall, the water quality and macroinvertebrate community data indicated that the stream [met/did not meet/or, if inconclusive, provide further explanation] expectations for a healthy, well-balanced stream.

Background

Healthy, well-balanced stream and river communities may be maintained with some level of human activity, but excessive human disturbance may result in waterbody degradation.  Human stressors may include increased inputs of nutrients, sediments, and/or other contaminants from watershed runoff, adverse hydrologic alterations, undesirable removal of habitat or riparian buffer vegetation, and introduction of nuisance (generally exotic) plants and animals.  DEP evaluates if human activities have resulted in the condition where a waterbody has exceeded water quality criteria (Chapter 62-302, Florida Administrative Code), including whether adverse impacts to biological communities have occurred.  DEP water quality standards are designed to protect designated uses of the waters of the state (e.g., recreation, aquatic life use support), and exceedances of these standards is associated with interference with the designated use.  Additionally, DEP evaluates the health of floral (plants and algae) and faunal (animal) communities and conducts habitat assessments to diagnose potential physical impacts.

Site Description

[Include relevant description of site and location, including map.  Suggested text below.]

The [site name] sampling location (Figure 1) is located [location description] in the [watershed name] Watershed (WBID [x], Figure 2).  It is a [first, second, etc] order stream draining the Gulf Coast Flatwoods, a subecoregion of the Southern Coastal Plain ecoregion.  The [site name] is a tributary of [receiving water] that flows into [receiving estuary or larger water body] and has a Landscape Development Intensity (LDI) Index score of [site LDI score].  The LDI quantifies the intensity of human activity in the surrounding area based on land use and land cover, and is normalized on a scale from 1 to 10, where a natural land use has a coefficient of 1.0 and a central business district has a coefficient of 10.0.  This site’s score of [site LDI score] suggests [minimal, moderate, heavy] human activity in the 100m buffer surrounding the stream.  The WBID is currently listed for [include any listed parameters on most recent verified list] on the Impaired Waters Verified List (date of list).  Additional information about this WBID is available in the TMDL Tracker application  (http://webapps.dep.state.fl.us/DearTmdl/tmdlReportAction.do?method=report).

Figure 2.  Location of [Site Name] in [x] County.  [put figure captions below figures]

Methods

Water Quality

Surface water [or other depth] samples were collected for analysis of [nutrients, chlorophyll a, color, etc.] following DEP Standard Operating Procedures (SOPs, see http://www.dep.state.fl.us/water/sas/sop/index.htm for details) and met the requirements of the FDEP Quality Assurance Rule, Chapter 62-160, F.A.C. (https://www.flrules.org/gateway/ChapterHome.asp?Chapter=62-160).  Results were compared with applicable Class III [typically, but change if it’s another class] water quality criteria contained in 62-302, F.A.C., including nutrients, dissolved oxygen, and other indicators.

Chapter 62-302.531, F.A.C., provides that Numeric Nutrient Criteria are achieved in streams if: Information on chlorophyll a levels, algal mats or blooms, nuisance macrophyte growth, and changes in algal species composition do not indicate an imbalance in flora or fauna; AND EITHER

The average score of at least two temporally independent Stream Condition Indices (SCIs) is 40 or higher, with neither of the two most recent SCI scores less than 35, OR

The Nutrient Thresholds (expressed as annual geometric means) are not exceeded more than once in a three year period.  The nutrient thresholds are region-specific.  This sampling site is in the [note nutrient region, NOT same as SCI region] nutrient region.  The floral and faunal measures are described below.

Dissolved Oxygen

Rule 62-302.533 (1), F.A.C., states that no more than 10 percent of the daily average percent dissolved oxygen (DO) saturation values shall be below 67 percent in the Panhandle West bioregion, 38 percent in the Peninsula and Everglades bioregions, or 34 percent in the Northeast and Big Bend bioregions. This site is in the [note DO region, which is same as SCI region] region for DO criteria assessment.  Percent saturation takes into account factors such as temperature, atmospheric pressure, and salinity.  [Include the following if the study was conducted in the following areas:]  Site specific DO criteria have been established for the Suwannee, Santa Fe, north Withlacoochee, and portions of the St. Johns Rivers.  These criteria are in Appendix I of Technical Support Document: Derivation of Dissolved Oxygen Criteria to Protect Aquatic Life in Florida’s Fresh and Marine Waters, (DEP-SAS-001/13).

The daily average freshwater DO criteria is preferentially assessed using daily average values calculated from full days of diel (collected over a 24 hour period) monitoring data.  If diel monitoring data are not available (as was the case for this report), instantaneous samples are used to assess the DO criterion by comparing the instantaeous value with a time-of-day-specific translation of the daily average criterion (62-303.420(9), F.A.C.).

[If report is done for a natural background DO study, include the following:]  Rule 62-302.533(3), F.A.C., states that, “If it is determined that the natural background DO saturation in the waterbody (including values that are naturally low due to vertical stratification) is less than the applicable criteria stated above, the applicable criteria shall be 0.1 mg/L below the DO concentration associated with the natural background DO saturation level”.  The FDEP method to demonstrate the appropriateness of a reference site to make a natural background determination includes a demonstration that the proposed reference watershed has been minimally affected by human activities, including point source discharges and nonpoint source inputs.

DEP (with EPA approval) has identified minimally disturbed stream and lake reference sites by application of the following criteria:

- Landscape Development Intensity Index (LDI) (Brown and Vivas 2007) score < 2 for land use within the 100 meter corridor 10 km upstream of the sample site. LDI scores higher than 3 indicate increasing levels of human disturbance;

- Watershed or near-field LDI scores < 3, indicating that that human disturbance is minimal throughout the watershed;

- Presence of relatively benign land uses or human inputs, as discerned by using aerial photographs, field observations, and DEP Best Professional Judgment, that would support the conclusion that the site is minimally disturbed; and

- Characterized by healthy biology (SCI scores > 40 or LVI scores > 43).

Habitat Assessment

The Stream and River Habitat Assessment was conducted per DEP SOP FT 3100.  The Habitat Assessment is a rapid field method in which a sampler scores eight in-stream and riparian components to estimate the influence of habitat factors on the resident aquatic organisms.  Habitat Assessment scores range from 11-160 and overall habitat quality is assigned to one of four categories:  Optimal (120-160 points), Suboptimal (80-119 points), Marginal (40-79 points), and Poor (11-39 points).

Floral Assessment

Evaluations of stream flora include the Rapid Periphyton Survey (RPS, DEP SOP FS 7230), community composition information associated with dominant algal taxa (where applicable), Linear Vegetation Survey (LVS, DEP SOP 7320), and chlorophyll a.  The RPS assesses the extent and thickness of algae in the stream reach, while the LVS assesses how closely the in-stream plant community resembles a minimally disturbed condition.  Results are assessed as described beginning on page 8 of “Implementation of Florida’s Numeric Nutrient Standards,” dated April 2013, using evidentiary thresholds derived from a population of minimally disturbed reference sites.

Faunal Assessment

The Stream Condition Index (SCI) assesses how closely the macroinvertebrate community (e.g., aquatic insects, clams, crayfish) of a stream resembles the macroinvertebrate community of an undisturbed or “reference” condition.   The SCI was sampled and calculated per DEP SOP SCI 1000 [BioRecon BRN 1000].  For the SCI, ten measures of macroinvertebrate health are calculated and summed to yield a final score from 0-100.  Average SCI scores ≥ 40 are considered “Healthy” (as long as neither of the latest two SCI scores were < 35) with scores ≥ 64 considered “Exceptional.”  The SCI is part of the NNC evaluation for streams, as described above, and also serves as a biological health assessment tool in 62-303, F.A.C., DEP’s Impaired Waters Rule.

Evaluation of Stream Exclusions for Applying Nutrient Criteria

When assessing a stream for compliance with numeric nutrient criteria (NNC) in Rule 62-302.531, F.A.C., it is important to note that several waterbody types are excluded from the stream definition in Rule 62-302.200(36), F.A.C.  These include lake-like portions of rivers (where the lake NNC apply) and ditches/canals used for water management purposes, non-perennial streams, wetlands, and tidally fluctuating areas (where the narrative criterion, “no imbalances in natural populations of aquatic flora or fauna,” continues to apply).  The NNC in Rule 62-302.531, F.A.C., were/were not applicable to site name.

Results

Water Quality

The water quality results are shown in Table 1.  [Describe any exceedances of standards and whether they may be due to natural conditions (e.g., DO, low pH) or local human disturbance.  Make sure DO exceedances are based on the time-of-day adjustment calculations.]

NNC Threshold Assessment

Based on these two sampling events, the stream [met/exceeded] the numeric nutrient thresholds for the [Bioregion]; however, the thresholds represent annual geometric mean concentrations (minimum of four samples) not to be exceeded more than once in any three calendar year period, and are used in combination with floral and faunal information.  Therefore, even though the thresholds were/were not exceeded, additional nutrient sampling may be needed to meet data sufficiency requirements to calculate and assess attainment of the water quality standard.

Floral Assessment

Results of the floral metrics show that this site is/is not within the reference stream distribution and therefore attains/does not attain the floral component of the NNC (Table 2).  All floral measures must pass for the stream to attain the NNC.

[Following the Implementation Document, describe any exceedances of the floral thresholds, whether they may be due to natural conditions (low flow, changes in grazers, etc.) as compared with contemporaneously sampled reference sites, or if they represent imbalances of flora (and then state that Rule 62-302.531(2)(c) is not achieved)].

Table 1. Water quality results from [date] and [date] at [site Name].  [include remark codes in the result column and define them below the table, as needed]

| Analyte | Result [date] | Result [date] | Applicable Class III Water Quality Criteria |
| --- | --- | --- | --- |
| Field Temperature (°C) |  |  |  |
| Field pH (SU) |  |  | Within 1 SU of natural background |
| Field Dissolved Oxygen (% saturation) |  |  | * Panhandle West: 67%; Peninsula and Everglades: 38%; Big Bend and Northeast: 34%. |
| Field Specific Conductance (µmhos/cm) |  |  | 50% above background or 1275 µmhos/cm, whichever is greater |
| Alkalinity (mg CaCO3/L) |  |  | Shall not be depressed below 20 |
| Color (PCU) |  |  | N/A |
| Total Phosphorus (mg/L) as P |  |  | **Panhandle West: 0.06 mg/L; Panhandle East: 0.18 mg/L; North Central 0.30 mg/L; Peninsula: 0.12 mg/L; West Central: 0.49 mg/L |
| Nitrate+Nitrite (mg/L) as N |  |  | 0.35 mg/L for spring vents only |
| Ammonia (mg/L) as N |  |  | *** |
| Total Kjeldahl Nitrogen (mg/L) as N |  |  | N/A |
| Total Nitrogen (mg/L) as N |  |  | **Panhandle West: 0.67 mg/L; Panhandle East: 1.03 mg/L; North Central 1.87 mg/L; Peninsula: 1.54 mg/L; West Central: 1.65 mg/L |

* Instantaneous DO criterion calculated per DO Saturation Calculator.

** Annual geometric numeric nutrient threshold for [x] nutrient region, used in combination with information on flora and fauna.

*** Total ammonia criterion calculated per Total Ammonia Nitrogen Calculator

Table 2.  Floral and faunal assessment results from [dates for Sample 1 and 2] at [site Name]

| Floral/Faunal Metric | Evidentiary Threshold of No Imbalances | Result [Date] Sample [SBIO#] | Result [Date] Sample [SBIO#] |
| --- | --- | --- | --- |
| LVS C of C | Site average > 2.5 |  |  |
| LVS FLEPPC (%) | Site average ≤ 25% |  |  |
| RPS (%) | ≤ 25% rank 4-6 (algae > 6 mm thick) coverage. If 20 to 25 % rank 4-6 coverage, evaluate algal species |  |  |
| Chlorophyll a (corrected; µg/L) | < 3.2 ug/L. If 3.2 to 20 ug/L, site specific evaluation |  |  |
| Algal Community Composition | No adverse shifts in dominant taxa |  |  |
| SCI | Average > 40 |  |  |

Habitat Assessment

The stream and river habitat assessment scores were [score] and [score], which are in the [optimal, suboptimal, marginal, poor] range.  The typical velocity measured was [0.xx] m/s, which resulted in an [optimal, suboptimal, marginal, poor] score for stream flow. Fish and aquatic insects require sufficient water velocity to maintain high dissolved oxygen concentrations in the stream; in this case the dissolved oxygen was xx% saturation. The average width within the assessment stretch was [x] m. The banks were [describe – examples include: well-armored, but relatively steep, potentially leading to instability during high flow events; low with a gradual slope, providing a good connection with the floodplain].  The bank stability scores, as a result, were in the marginal range. [Two, Three, Four, etc] productive habitats were noted: [woody debris, leaf packs, rocks, and roots,] which comprised roughly x% of the stream area.  [Edit as needed to provide brief description of which habitat components were good, bad, etc. and why habitat may be related to the other results] [If there is other relevant historical information related to the HA, include it here, e.g., graph of habitat assessment over time, discussion of past results.  Insert additional discussion, photos, or anything you think would be informative for the audience.]

Faunal Assessment (SCI)

The average SCI score for this site was [SCI score] out of a possible 100 points, corresponding with a [“Exceptional” (≥64), “Healthy” (40-63), “Impaired”] designation (Table 2).  These results are/are not within the range of previous scores for this site, which ranged from x to x (most recent version of SCI only, from 200x-20xx). [Describe why the system did or did not meet a “healthy” expectation (e.g., smothering, habitat, hydrology, algal mats, etc.).  If there was an issue with the SCI sampling (inappropriate flows, etc.), do not report SCI score, and state that the SCI sample is not reported because it did not meet SOP requirements].

[If there is other relevant historical information related to the SCI, include it here, e.g., graph of SCI scores over time for current SCI tool, discussion of past results.  Insert additional discussion, photos, or anything you think would be informative for the audience.]

[If the site may qualify for the Managed Conveyance exclusion to the streams criteria, use the following text (modify as needed).]  A waterbody does not meet the stream definition for NNC assessment if: it is man-made or predominantly channelized for physically altered; it is primarily used for water management purposes, such as flood protection, stormwater management, irrigation, or water supply; and it has marginal or poor stream habitat components, specifically habitat availability and diversity, and artificial channelization.  To fall under this exclusion, there must be evidence that this waterbody is man-made or has been physically altered, has poor habitat, and that the primary use is water management.  This site [here you should insert information about dredging, plant control, other maintenance for conveyance that satisfies requirements].  The habitat availability and diversity scores are in the [poor, marginal] category and/or there is potential for low scores due to herbicide maintenance.

Conclusions

Sampling at [Site name] was inconclusive OR indicates that the stream met/did not meet applicable State Water Quality Criteria.

Inconclusive Example

Evidence suggests the stream dries up periodically, and it was not definitively established that the creek had flowing water for a minimum of six months prior to sampling (as specified in the SOP).  It is possible that the low SCI score can be attributed to natural desiccation. Etc.

Met Use Example

Habitat Assessment data indicated a near optimal physical condition of the creek with little disturbance in the watershed.  Stream Condition Index data indicated the presence of a “healthy” macroinvertebrate community with sensitive and long-lived taxa that are indicative of good water quality conditions. The Linear Vegetation Survey and Rapid Periphyton Survey indicated vascular plant and algal communities, respectively, that are consistent with expectations for minimally disturbed sites.

Not Met Use Example

The SCI results indicated that [stream x] was in the impaired category. The site was characterized by significant algal smothering, as demonstrated through the RPS results.  Taxonomic identification showed the algae community to be dominated by Lyngbya, a known nuisance species.  Although the vascular plant community, as assessed using the LVS, was within the range of reference streams, and chlorophyll a was non-problematic, the algal growth resulted in aquatic habitat smothering, which likely led to the failing SCI score.  The Department concluded that the RPS results, coupled with a poor habitat smothering score, was evidence that the stream has impaired flora, which in turn caused impaired fauna. Mention any planned follow-up to verify condition or mitigate potential sources.

DO Example

FDEP determined that XX Creek is minimally disturbed by humans, and that the low DO is a natural condition.  Evidence of benign land uses included the watershed Landscape Development Intensity Index (LDI) score of XX and the 100 m riparian buffer LDI score XX, which were confirmed by direct observations.  The biological community was found to be healthy, indicating that the existing DO regime fully supports aquatic life uses.  Therefore, in accordance with Rule 62-302.533(3), F.A.C., the evidence indicates that the DO standard is achieved.

Thank you for your interest in maintaining the water quality of Florida’s aquatic resources. Please contact us if you have any questions.

Contact and resources for more information

[your name, phone number, and e-mail address]

DEP biological assessment resources: http://www.dep.state.fl.us/water/bioassess/index.htm