Site-Specific Information 
in Support of 
Establishing Numeric Nutrient Criteria for the Florida Keys

Florida Department of Environmental Protection

Tallahassee, FL 32399

September 2011

Table of Contents

Executive Summary	1

Geographic and Physical Description	5

Sources and Fates of Nutrients	7

Nutrient Sources and Loading Estimates	7

Biological Summary	13

Seagrasses	13

Coral Reefs	19

Additional Biological Resources	25

Water Quality Studies	27

Waters on the 303(d) List	33

Numeric Nutrient Criteria Recommendations	37

FKRAD recommendations	38

FDEP Nutrient Criteria Approach	40

Proposed Numeric Criteria	44

Magnitude	44

Frequency and Duration	44

Summary of the Proposed Criteria	45

Comparison of FKRAD and FDEP approaches	47

References	51

Appendix A.  Permitted facilities and landfills in the Florida Keys basin by planning unit (FDEP 2008b).	55

List of Tables

Table 1.  Checklist of nutrient enrichment symptoms in the Florida Keys.	3

Table 2.  State parks and aquatic preserves.	6

Table 3. Estimated annual nutrient loadings in 1999 (lbs year-1) for the different Keys regions (FDEP 2008a).	8

Table 4. Comparison of total 1999 estimated annual anthropogenic land based (wastewater and stormwater) nutrient loadings to the Keys (FDEP 2008a) with some estimated farfield loadings from Florida Bay passes and Atlantic upwelling (Gibson et al. 2008) and atmospheric deposition (calculated using atmospheric loading estimates in Rudnick et al. 1999)  in metric tons year-1.	13

Table 5.  Catch statistics for the 15 more abundant reef fish species observed during the Florida Keys visual sampling, 1999–2006.	26

Table 6.  Water quality variables measured in the FKNMS, March 1995–December 2008 (Boyer and Briceño 2009).	30

Table 7. Verified impairments in the Florida Keys basin.	34

Table 8.  Natural background conditions (modeled) and water quality targets for the northern Keys (FDEP 2008a).	38

Table 9.  Natural background conditions (modeled) and water quality targets for the central Keys (FDEP 2008a).	38

Table 10.  Natural background conditions (modeled) and water quality targets for the south-central Keys (FDEP 2008a).	39

Table 11.  Natural background conditions (modeled) and water quality targets for the southern Keys (FDEP 2008a).	39

Table 12. 1985 FDEP OFW water quality data (FDEP 2008a).	40

Table 13. Range and means of modeled TN and TP targets for bayside and oceanside bubble WBIDs (FKRAD 2008).	40

Table 14.  Proposed regionalization for the Florida Keys.	41

Table 15.  Proposed numeric nutrient criteria for all segments of the Florida Keys including TP, TN, and chlorophyll a.  For assessment purposes, the average of all stations in a segment shall not exceed the network average more than once in a 3-year period.	47

Table 16.  Comparison of TN (µg l-1) targets for halo zone modeled WBIDs and OFW targets (from the FKRAD) with nearshore FDEP criteria proposed for sub-basins using FIU long-term water quality data.  For maps showing the locations of WBIDs and sub-basins, see Figures 24 and 25.	49

Table 17.  Comparison of TP (µg l-1) targets for halo zone modeled WBIDs and OFW targets (from the FKRAD) with nearshore criteria proposed for sub-basins using FIU long-term water quality data.  For maps showing the locations of WBIDs and sub-basins, see Figures 24 and 25.	50

List of Figures

Figure 1.  Map of the Florida Keys and FKNMS boundary.	5

Figure 2.  Regional circulation patterns in the Florida Keys (Lee et al. 2007).	7

Figure 3.  Nitrogen loading to Florida Bay and the Florida Keys (Gibson et al. 2008; Key for sources in figure: a. Gibson et al. 2008, b. Boyer and Keller 2007, c. Rudnick et al. 1999, d. Kruczynski and McMannus 2002, e. Leichter et al. 2003).	11

Figure 4.  Phosphorus loading to Florida Bay and the Florida Keys (Gibson et al. 2008; Key for sources in figure: a. Gibson et al. 2008, b. Boyer and Keller 2007, c. Rudnick et al. 1999, d. Kruczynski and McMannus 2002, e. Leichter et al. 2003).	12

Figure 5.  Conceptual model of the relationship between seagrass leaf nutrient content and nutrient availability in south Florida (Fourqurean and Rutten 2003).	15

Figure 6.  Conceptual model relating nutrient availability (trophic state) to the abundance of primary producers in nearshore marine waters of south Florida (Fourqurean and Rutten 2003).	16

Figure 7.  EI status of seagrasses in the FKNMS for 2006, 2007, and 2008 compared with the long-term mean EI (dashed line) for 1995 to 2005 (Fourqurean and Escorcia 2009).	17

Figure 8.  SCI status of seagrasses in the FKNMS for 2006, 2007, and 2008 compared with the long-term mean SCI (dashed line) for 1995 to 2005 (Fourqurean and Escorcia 2009).	17

Figure 9.  Seasonality of epiphyte load, averaged across all 30 Level 1 sites, 2001–09 (Fourqurean et al., in press).	18

Figure 10.  Line graph of mean annual percent cover for the four major benthic taxa recorded in CREMP image analysis (CREMP 2010).  Mean percent cover is pooled from 97 stations in the Florida Keys.	20

Figure 11. Line graph of the mean annual percent cover for six species of stony corals.  Mean percent cover is pooled from 97 stations (CREMP 2010).	21

Figure 12.  Long term trends of percent cover from 1999-2008 at CREMP sites for a) stony corals and b) octocorals (CREMP 2010).	22

Figure 13.  Number of stations with stony coral disease recorded (Callahan et al. 2007).	23

Figure 14.  Percent cover comparisons between summer 2009 and February 2010 for a) stony corals and b) octocorals at eight stations (CREMP 2010).	24

Figure 15.  Mean fish density by year (2000–06) as recorded during visual surveys in the Florida Keys (Donahue et al. 2008).	26

Figure 16.  Map of study area (Boyer 2010).	28

Figure 17.  Florida International University (FIU) Southeast Environmental Research Center (SERC) Water Quality Monitoring Network.  Stations in red are sampled as part of the FKNMS monitoring program (Boyer and Briceño 2009).	29

Figure 18.  Median salinity, 1995–2009 (Boyer and Briceño 2009).	31

Figure 19.  Dissolved inorganic nitrogen (micromolar[µM]) median, 1995–2007 (Boyer and Briceño 2009).	31

Figure 20.  TP (µM) median, 1995–2007 (Boyer and Briceño 2009).	32

Figure 21.  Chlorophyll a (µg L-1) median, 1995–2007 (Boyer and Briceño 2009).	32

Figure 22.  Dissolved inorganic nitrogen (µM) (Boyer and Briceño 2009).	33

Figure 23. Map of the Florida Keys with WBIDS and regions (north, central, southcentral, and southern) identified (FDEP 2008a).	36

Figure 24.  WBID models in the Florida Keys (McClelland 2010).	37

Figure 25.  Proposed Marine Nutrient Regions for South Florida.	42

Figure 26.  Proposed Marine Nutrient Regions for the Florida Keys	43

Executive Summary

This report was prepared by the Florida Department of Environmental Protection (FDEP), in cooperation with the U.S. Environmental Protection Agency (EPA) and local scientists, to support the development of numeric nutrient criteria for the Florida Keys.  The primary purpose of the proposed numeric nutrient criteria is to protect healthy, well-balanced natural populations of flora and fauna from the effects of excess nutrient enrichment.  This report presents approaches for setting criteria for the Florida Keys in both the halo zone (within 500 meters [m] from shore) and nearshore zone waters (state waters beyond 500 m).

The Florida Keys are an archipelago of islands in Monroe County, Florida, that extend from the southern tip of Florida over 354 kilometers (km) to the southwest (Collado-Vides et al. 2007; Figure 1).  Consisting of 822 islands, of which about 30 are inhabited, the Keys are traversed by U.S. Highway 1 (also known as U.S. 1 or Overseas Highway) with 19 miles of bridges (FDEP 2008).  The Florida Keys National Marine Sanctuary (FKNMS),  consisting of 9,500 square kilometers (km2) of coastal and oceanic waters and submerged lands that surround the entire Florida Keys island chain, was established in 1990 (Figure 1).  The Keys are generally divided into three geographic areas:  the Upper, Middle, and Lower Keys (Figure 2).  The three regions are further divided into oceanside and bayside.  The bayside regions of the Lower and Middle Keys are known as Backcountry and Sluiceway, respectively.

There are both anthropogenic and natural sources of local and external nutrient loads to the Keys.  The Florida Keys are directly influenced by the Florida Current, the Gulf of Mexico Loop Current, inshore currents of the Southwest Florida Shelf (Shelf), discharge from the Everglades through Shark River Slough, and tidal exchange with both Florida Bay and Biscayne Bay (Lee et al. 2002).  FDEP recognizes that there are land-based sources of nutrients in the Keys, such as wastewater and stormwater, which may contribute to the nutrient regime in waters of the Keys. The Florida Keys Reasonable Assurance Document (FKRAD) (FDEP 2008) identified local nutrient sources in the Keys, including wastewater and stormwater originating from unincorporated Monroe County in the Key Largo area; the Villages of Islamorada, Marathon, Key Colony Beach, and Layton; the City of Key West; the Florida Department of Transportation (FDOT); and the Florida State Park Service.  Additionally, in the south-central and southern Keys, stormwater and wastewater originate from the U.S. Navy facilities in Boca Chico and those adjacent to the City of Key West.

The three dominant habitats that make up the coastal marine ecosystems of the Florida Keys are seagrass meadows, coral reefs, and mangroves.  Seagrasses are the principal component of benthic marine habitats in the Keys, occupying over 70% of the FKNMS (Fourqurean et al. 2003).  Coral reefs, patch reefs, and hardbottom habitats cover approximately 7%, 1%, and 19% of the FKNMS, respectively.  Mangroves are a dominant feature of the land-sea margin throughout the Florida Keys.

Long-term monitoring of water quality indicates that nitrate and total organic carbon (TOC) follow a concentration gradient, with higher levels in the nearshore area and declining concentrations away from shore, implying a land-based source (Boyer and Briceño 2009).  However, neither total phosphorus (TP) nor chlorophyll a concentrations show a trend with distance from land.  The Keys are an extremely oligotrophic system and anthropogenic nutrient loads may not be immediately observed in the water column due to rapid assimilation by seagrasses and other benthic flora.

Florida Keys waters were listed as impaired for nutrients on the 1998 303(d) list.  However, as provided for in the Impaired Surface Waters Rule (Chapter 62-303, Florida Administrative Code [F.A.C.]), the Keys have not been placed on the verified list of impaired waters because local stakeholders provided documentation that nutrient loads from local sources will be reduced as part of a Keys restoration process (see the Florida Keys Reasonable Assurance Document [FKRAD] (FDEP 2008a)).

For the purposes of nutrient criteria development, FDEP has considered the halo zone (within 500 m) and nearshore zone (state waters beyond 500 m) areas of the Florida Keys separately and is applying two different approaches for establishing numeric nutrient criteria.

For the halo zone area, FDEP has yet to develop numeric nutrient criteria, but plans to apply a “response based” approach that is based on the FKRAD document.  As indicated in the FKRAD, existing canal nutrient levels and loading from land based sources of nutrients may contribute to the designated uses not being supported in the canals (in addition to physical constraints).  The FKRAD was developed to set forth and accelerate actions that have been taken or are planned to be taken to reduce nutrient loadings throughout the Florida Keys so that water quality standards are met and beneficial uses are restored (FDEP 2008a).  The FKRAD is based on improving water quality to targets developed using a land-use model.  The water quality targets would be assessed at the boundary of the halo zone, which is defined as 500 m from shore.  Although the FKRAD does not specifically address criteria for canals, FDEP supports the outlined load reductions in order to improve water quality at 500 m and help ensure that criteria are met in nearshore waters.  The FKRAD targets for the halo zone boundary are presented in Tables 8-11 in this report.

For the nearshore waters, FDEP is applying a “maintain existing healthy conditions” approach.  FDEP’s evaluation of the biological resources indicates that the waters support a healthy well-balanced community, are attaining the designated use, and have not shown adverse effects from nutrient enrichment in the nearshore area.   Although there have been changes in some communities (e.g., coral loss) (Table 1), evidence suggests that causes other than nutrients are responsible for the responses.   Waters in the nearshore area were subdivided into segments of similar water quality and criteria established to maintain the current nutrient data distributions, which would continue to support the existing healthy condition.   For each segment, the proposed criteria are expressed as an annual geometric mean nutrient concentration target not to be exceeded more than once in a 3-year period. These criteria are presented in Table 15 in this report.

Table 1.  Checklist of nutrient enrichment symptoms in the Florida Keys.

- = Empty cell/no data

| Response variable | Observed historically or currently? | Explanation | Source |
| --- | --- | --- | --- |
| Low dissolved oxygen (DO) (hypoxia/anoxia) | Yes | Low DO concentrations have been observed in canals in the Keys and were observed at several nearshore stations during sampling in 1985 for the OFW designation. Diel low DO in SAV beds is a natural phenomenon resulting from SAV respiration. | FDEP (2008) |
| Reduced clarity | Yes | There has been increased turbidity in nearshore areas, likely due to resuspension of sediments by wind and wave action. However, light penetration to SAV beds generally exceeds 40% of surface values, meaning light is not a limiting factor for SAV growth throughout the Keys. | Boyer and Jones (2002) |
| Increased chlorophyll a concentrations | No | Chlorophyll a has not increased over time nor does it vary as a function of distance from land. Chlorophyll a median concentrations are extremely low (0.19 micrograms per liter [µg l-1]) for the monitoring period between 1995 and 2008. | Boyer and Briceño (2009) |
| Phytoplankton blooms (nuisance or toxic) | Yes | Periodic algae blooms originate on the Gulf side of the Keys in Florida Bay or along the southwest Shelf, and are not associated with human activities. The Blackwater event in 2002, resulting from natural tannic inputs into west Florida coastal waters, included a nontoxic algal bloom that was carried south through the Keys. More recently, in 2005 and 2006, blooms associated with highway development and hurricane activity were restricted to Barnes Sound and Lake Surprise. | - |
| Problematic epiphyte growth | No | Epiphyte growth is highly seasonal, and recent experimental studies have not found increased epiphytes associated with nutrient additions due to the activity of grazers. | e.g., Fourqurean et al. (in press) |
| Problematic macroalgal growth | No | Abundance of calcareous macroalgae in the FKNMS has increased over time, however, calcareous macroalgae is considered a viable benthic habitat. Sites closest to land and with the highest water column nitrogen (N) concentrations have the highest calcareous algae abundance. | e.g. Lapointe et al. (2004), Collado-Vides et al. (2005) and (2007), Fourqurean et al. (in press) |
| Submerged aquatic vegetation (SAV) community changes or loss | No | Data from the FKNMS monitoring program’s permanent stations indicate no significant sanctuary-wide decrease in SAV coverage or adverse changes in community composition and show no clear spatial pattern that human activities have resulted in adverse eutrophication effects. Monitoring results indicate that there are early signs of eutrophication within the 500 m halo zone, but those eutrophication effects have not manifested themselves to where there is imbalance of flora and therefore have not exceeded the narrative criteria. Thalassia tissue nitrogen:phosphorus (N:P) ratios are being investigated to determine if this metric may be indicative of early signs of eutrophication. | e.g., Fourqurean and Escorcia (2009) |
| Emergent or shoreline vegetation community changes or loss | Yes | Shoreline vegetation has changed in many areas where human development has occurred due to physical (not nutrient) disturbance. | - |
| Coral/hardbottom community changes or loss | Yes | Coral cover has significantly declined, and there have been periodic outbreaks of coral diseases (black band and other diseases) in the Keys and throughout the Caribbean. Overall mean percent stony coral cover has been stable since 1999. Changes in coral health (cover and condition) has not been directly related to anthropogenic land-based sources of nutrients. | Donahue et al. (2008), Callahan et al. (2007) LIrman and Fong (2007) |
| Impacts to benthic community | - | Covered above. | - |
| Fish kills | Yes | Periodic fish kills have been attributed to factors other than nutrient enrichment (e.g., low DO, red tide, cold, and other factors). | Florida Fish and Wildlife Research Institute (FWRI) |

Geographic and Physical Description

The Florida Keys are an archipelago of islands in Monroe County, Florida, that extend from the southern tip of Florida over 354 km to the southwest (Collado-Vides et al. 2007; Figure 1).  About 30 of the 822 islands are inhabited and the Keys are traversed by U.S. Highway 1 with 19 miles of bridges (FDEP 2008).  Approximately 34% of the land area in the Keys portion of Monroe County is conserved  (Monroe County 2010).

The Florida Keys was designated an Outstanding Florida Water (OFW) in 1985, and in 1990, the the entire Florida Keys island chain and the 9,500 km2 of coastal and oceanic waters and submerged lands that surround it were established as the Florida Keys National Marine Sanctuary (Figure 1).  The Keys are bordered to the north and northwest by Biscayne National Park and Everglades National Park.  There are also 2 aquatic preserves and 10 state parks located in the Keys (Table 2).

The Florida Keys region is underlain by the extremely permeable Key Largo Formation limestone, and the Keys have little vertical relief (5 meter maximum at Windley Key).  Soils quickly infiltrate runoff and its pollutants are transported rapidly to nearshore waters with little or no nutrient entrapment in the soil matrix (FDEP 2008).  The islands function as a discontinuous, semipermeable dam that separates Florida Bay from the Atlantic Ocean (Reich et al. 2002).

The Keys are generally divided into three geographic areas:  the Upper, Middle, and Lower Keys (Figure 2).  The Lower Keys are most influenced by cyclonic gyres that spin off the Florida Current, the Middle Keys by exchange with Florida Bay, and the Upper Keys by Florida Current frontal eddies and somewhat by exchange with Biscayne Bay (Figure 2; Boyer and Briceño 2008).  The three regions are further divided into oceanside and bayside.  The bayside regions of the Lower and Middle Keys are known as Backcountry and Sluiceway, respectively.

Figure 1.  Map of the Florida Keys and FKNMS boundary.

Table 2.  State parks and aquatic preserves.

| Name | Description |
| --- | --- |
| Coupon Bight Aquatic Preserve | - 4,600 acres of seagrass meadows, hard bottom communities, mangrove wetlands, and coral reefs - The patch reef community includes sponges, soft and hard corals, and algae - Home to rare wading birds and the endangered Key deer |
| Lignumvitae Key Aquatic Preserve | - 7,000 acres of seagrass meadows, deep water channels, hard-bottom communities and mangrove wetlands - Lignumvitae Key Botanical State Park overlaps with much of the preserve |
| Dagny Johnson Key Largo Hammock Botanical State Park | - Contains one of the largest tracts of West Indian tropical hardwood hammock in the United States - Home to 84 protected species of plants and animals |
| John Pennekamp Coral Reef State Park | - First underwater park in the United States - 70 nautical square miles with coral reefs and associated marine life - Upland area includes mangrove swamps and tropical hammocks |
| Windley Key Fossil Reef Geological State Park | - Formed of Key Largo limestone, fossilized coral - Historically was a quarry used for stone to build the Flagler Railroad |
| Indian Key Historic State Park | - Tiny island accessible only by canoe or kayak |
| San Pedro Underwater Archaeological Preserve State Park | - Underwater archeological preserve that features a submerged shipwreck |
| Lignumvitae Key Botanical State Park | - Virgin tropical hardwood hammock only accessible by boat |
| Long Key State Park | - Profusion of plant and animal life - Known for bonefishing |
| Curry Hammock State Park | - Group of islands - Hardwood hammocks support a large population of thatch pines - Other communities include mangrove swamps, seagrass beds, and wetlands |
| Bahia Honda State Park | - Site of hawksbill turtle nesting |
| Fort Zachary Taylor Historic State Park | - Fort was constructed in the mid-1800s to defend the nation’s southern coastline |

Figure 2.  Regional circulation patterns in the Florida Keys (Lee et al. 2007).

Sources and Fates of Nutrients

Nutrient Sources and Loading Estimates

The water quality of the FKNMS can be directly affected both by local nutrient loading sources and external nutrient transport (Boyer and Briceño 2009).  There are both anthropogenic and natural sources for the local and external nutrient loads.  The FKNMS is directly influenced by the Florida Current, the Gulf of Mexico Loop Current, inshore currents of the Shelf, discharge from the Everglades through Shark River Slough, and tidal exchange with both Florida Bay and Biscayne Bay (Lee et al. 2002).  Advection from these external sources significantly affects the physical, chemical, and biological composition of waters within the FKNMS, as does internal nutrient loading and freshwater runoff from the Keys themselves (Boyer and Jones 2002).

Local anthropogenic nutrient sources within the Keys include point source discharges (domestic wastewater) and nonpoint source inputs, such as septic tanks and stormwater.  The FKRAD (FDEP 2008a) identified local nutrient sources in the Keys, including wastewater and stormwater originating in unincorporated Monroe County in the Key Largo area; the Villages of Islamorada, Marathon, Key Colony Beach, and Layton; the City of Key West; the Florida Department of Transportation (FDOT); and the Florida State Park Service.  Additionally, in the south-central and southern Keys, stormwater and wastewater originate in the U.S. Navy facilities both in Boca Chico and those adjacent to the City of Key West.   Table 3 provides a summary of the estimated contribution of nutrients from wastewater and stormwater to WBIDS in the north, central, south-central, and southern regions of the Keys for the baseline time period of 1999.  Load estimates were calculated as part of the Florida Keys Carrying Capacity Study based on the number of equivalent dwelling units (EDUs), their estimated daily flows and the effluent characteristics of their wastewater treatment methods.  Table 4 provides load estimate totals by regions converted to metric tons per year (m tons year-1) for comparison with estimates of farfield nutrient sources.

The 2008 Water Quality Assessment Report for the Florida Keys provides the following information on permitted discharges within each planning unit (FDEP 2008b; a full list of discharging facilities is provided in Appendix A).  In the Upper Keys planning unit there are 147 permitted wastewater facilities, of which 139 are domestic waste facilities, five are industrial waste facilities, two are industrial stormwater discharges, and one is an underground injection control facility.  Of these facilities, only two discharge to state surface waters, and those are the stormwater discharges.  There are 91 permitted wastewater facilities in the Middle Keys planning unit, of which 83 are domestic waste facilities, six are industrial waste facilities, and two are industrial stormwater discharges.  Of these facilities, two discharge to state surface waters, and those are the stormwater discharges. There are 57 permitted wastewater facilities in the Lower Keys Planning Unit, of which 51 are domestic waste facilities, five are industrial waste facilities, and one is an industrial stormwater discharge.  Three of these facilities discharge to state surface waters: one domestic waste facility, one industrial waste facility, and one stormwater discharge.   There are two inactive solid waste landfills in the Upper Keys planning unit, in the Middle Keys there are two solid waste landfills, one active and one inactive; and in the Lower Keys there are four landfills, two active and two inactive; however, the solid waste is currently sent to the mainland for disposal.   There are no hazardous waste sites, Superfund sites, state-funded waste cleanup sites, or delineated ground water contamination areas in the Upper Keys, Middle Keys, or Lower Keys Planning Units.

Table 3. Estimated annual nutrient loadings in 1999 (lbs year-1) for the different Keys regions (FDEP 2008a).

| WBID | Wastewater |  | Stormwater |  |
| --- | --- | --- | --- | --- |
|  | TN | TP | TN | TP |
| Northern |  |  |  |  |
| 6006A-N | 66,869 | 19,324 | 21,469 | 3,310 |
| 6006A-S | 67,601 | 19,643 | 28,338 | 4,442 |
| 6006B-N | 585 | 167 | 19,985 | 3,257 |
| 6006B-S | 1,847 | 497 | 4,489 | 721 |
| 6006C-N | 12,488 | 3,747 | 4,477 | 748 |
| 6006C-S | 6,522 | 1,956 | 5,247 | 810 |
| 6009-N | 30,541 | 8,829 | 8,988 | 1,288 |
| 6009-S | 23,975 | 6,899 | 4,557 | 634 |
| 6017-N | 12,643 | 3,709 | 3,035 | 423 |
| 6017-S | 16,149 | 4,823 | 3,058 | 398 |
| 6019-N | 7,920 | 2,315 | 3,496 | 574 |
| 6019-S | 4,462 | 1,413 | 1,402 | 227 |
| Northern - Total | 251,573 | 73,346 | 108,842 | 16,832 |
| Central |  |  |  |  |
| 6010-N | 5,180 | 1,546 | 1,740 | 251.18 |
| 6010-S | 1,357 | 407 | 2,935 | 462.49 |
| 6011A-N | 5,180 | 1,546 | 1,740 | 251.18 |
| 6011A-S | 1,357 | 407 | 2,935 | 462.49 |
| 6011B-S | 38,089 | 9,665 | 9,433 | 1,502.94 |
| 6011C-N | 9,043 | 2,688 | 2,514 | 385.03 |
| 6011C-S | 4,146 | 1,200 | 1,558 | 238.10 |
| 6016-N | 802 | 233 | 142 | 20.30 |
| 6016-S | 7,492 | 2,248 | 1,958 | 281.17 |
| Central - Total | 129,856 | 35,288 | 34,848 | 5,529 |
| Southcentral |  |  |  |  |
| 6012A | 30,009 | 8,943 | 21,362 | 3,353 |
| 6012C | 193 | 58 | 2,842 | 460 |
| 6012D | 346 | 104 | 1,280 | 205 |
| 6012E | 1,175 | 343 | 3,113 | 511 |
| 6013A | 11,971 | 3,558 | 22,724 | 3,806 |
| 6013B | 632 | 188 | 843 | 132 |
| 6013C | 28,183 | 8,162 | 15,223 | 2,449 |
| 6013D | 82 | 25 | 787 | 128 |
| 6014C | 25,498 | 7,532 | 23,910 | 4,566 |
| 6018 | 2,388 | 716 | 3,085 | 480 |
| Southcentral - Total | 100,477 | 29,629 | 95,169 | 15,650 |
| Southern |  |  |  |  |
| 6014A-N | 114,276 | 34,283 | 12,491 | 1,730 |
| 6014A-S | 62,651 | 18,797 | 10,516 | 1,603 |
| 6014B-N | 9,191 | 2,754 | 3,289 | 474 |
| 6014B-S | 23,100 | 8,775 | 4,349 | 619 |
| Southern - Total | 209,218 | 64,609 | 30,645 | 4,427 |

Submarine ground water discharge is also a component of nutrient input in the Keys (Lapointe et al. 1990, Lapointe and Matzie 1996).  Lapointe et al. (1990) assessed the degree of nutrient enrichment of limestone ground waters by onsite sewage disposal systems (septic tanks) and evaluated seasonal relationships between nutrient-enriched ground waters and adjacent surface waters.  They observed that ground waters associated with septic tanks had elevated nutrient concentrations compared with controls.

Reich et al. (2002) verified the natural gradient flow of ground water from Florida Bay beneath Key Largo and ultimately into the Atlantic Ocean using multi-depth wells and the injection of fluorescein and rhodamine dyes as tracers.  The study included clusters of deep (13.6 m) and shallow (6.1 m) wells on the bayside and oceanside of an undeveloped area of Key Largo.  They observed that the greatest forcing factor in ground water movement is the difference in water level between Florida Bay and the Atlantic Ocean.  For over 6 months, Florida Bay was on average 12 to 15 centimeters (cm) higher than the Atlantic side.  There was both horizontal and vertical flow of ground water, with the predominant direction of horizontal flow to the southeast, towards the reef tract.  Their results indicate that a component of vertical flow likely contributes ground water seepage up into overlying surface waters and could transport nutrients and pollutants into nearshore waters as well as to the reef tract.

External sources of nutrients to the Keys include the natural and regulated discharges from the Everglades to Florida Bay, regulated discharges from Lake Okeechobee through the Peace River, Caloosahatchee River and Mississippi River discharges to the Gulf of Mexico, Atlantic upwelling, and atmospheric deposition (FDEP 2008a; Lapointe et al. 2004).

Lapointe et al. (2004) quantified the role of physical forcing and water management in south Florida on nutrient enrichment and blooms of phytoplankton, macroalgae, and seagrass epiphytes in the Keys.  Results suggested that regional-scale agricultural runoff (from the mainland Everglades watersheds) and local sewage discharges from the Keys are both significant N sources.   However, Gibson et al. (2008) determined that while the Middle Keys passes were a significant outflow from Florida Bay towards the reef tract, inputs of cool, nutrient rich waters via offshore tidal bores contributed far greater amounts of biologically available N directly to the reef system.  For P, fluxes across the open western boundary of Florida Bay and input from tidal bores are also important nutrient sources to the reef system.  Gibson et al. (2008) quantified the exchange of water and nutrients through Long Key Channel using long-term, high-resolution, mass flow measurements, combined with periodic tidal-scale observations of nutrient concentrations.  Loads were compared with other published values of regional nutrient loads and fluxes (Figures 3 and 4).

Figure 3.  Nitrogen loading to Florida Bay and the Florida Keys (Gibson et al. 2008; Key for sources in figure: a. Gibson et al. 2008, b. Boyer and Keller 2007, c. Rudnick et al. 1999, d. Kruczynski and McMannus 2002, e. Leichter et al. 2003).

Figure 4.  Phosphorus loading to Florida Bay and the Florida Keys (Gibson et al. 2008; Key for sources in figure: a. Gibson et al. 2008, b. Boyer and Keller 2007, c. Rudnick et al. 1999, d. Kruczynski and McMannus 2002, e. Leichter et al. 2003).

Table 4 shows a comparison of the 1999 anthropogenic land based nutrient loads (Table 3 totals for stormwater and wastewater; FDEP 2008a) converted to metric tons per year to compare with some farfield estimates of nutrient loads to the region from Gibson et al. (2008) and Rudnick et al. (1999).  Land based nutrient sources are a small component of the total nutrient inputs to the Keys.

Table 4. Comparison of total 1999 estimated annual anthropogenic land based (wastewater and stormwater) nutrient loadings to the Keys (FDEP 2008a) with some estimated farfield loadings from Florida Bay passes and Atlantic upwelling (Gibson et al. 2008) and atmospheric deposition (calculated using atmospheric loading estimates in Rudnick et al. 1999)  in metric tons year-1.

- = Empty cell/no data

| Source | TN (metric tons year-1) | TP (metric tons year-1) |
| --- | --- | --- |
| Keys wastewater and stormwater | 438 | 111.8 |
| Atlantic upwelling | 5190 | 861 |
| Florida Bay (through Middle Keys passes) | 5500 | 90 |
| Atmospheric deposition | 3040 | 161.5 |

Biological Summary

The three dominant habitats that make up the coastal marine ecosystems of the Florida Keys are seagrass meadows, coral reefs, and mangroves.  Seagrasses are the principal component of benthic marine habitats in the Keys, occupying over 70% of the FKNMS (Fourqurean et al. 2003).  Coral reefs, patch reefs, and hardbottom habitats cover approximately 7%, 1%, and 19% of the FKNMS, respectively.  Mangroves are a dominant feature of the land-sea margin throughout the Florida Keys.

Seagrasses

The dominant primary producers in the Keys are seagrasses and macroalgae.  One of the world’s most extensive seagrass beds, approximately 14,000 km2, surrounds the Florida Keys (Fourqurean and Zieman 2002).  The structure and distribution of seagrass communities varies throughout the Keys.  Seagrasses form extensive meadows and patchy beds, and are also found scattered throughout other habitats (Fourqurean et al. 2001).  The distribution and species composition of seagrasses in south Florida results from many factors, with the most important being water depth, water clarity, and nutrient availability (Fourqurean et al. 2002).

Seagrass beds are important habitat for other species, including pink shrimp (Farfantepenaeus), spiny lobster (Panulirus argus), queen conch (Strombus gigas), and spotted seatrout (Cynoscion nebulosus) (Fourqurean et al. 2001).  Lapointe et al. (1994) note that, since the densities of benthic invertebrates and fishes that inhabit seagrass meadows are positively correlated with seagrass density and productivity, the eutrophication of seagrass meadows could also lead to reductions in secondary productivity.

Fourqurean et al. (2001) documented the distribution and abundance of seagrasses in south Florida in a larger study area that included the FKNMS from 1995 to 1998 and found T. testudinum to be the most common seagrass at 67.2% of the sites.  Other seagrasses observed included Syringodium filiforme at 38.9% of the sites and Halodule wrightii and Halophila decipens at about 15% at each site.   A survey of seagrass species composition and density in the FKNMS from 1996 to 1997 found water depth related to the density of all observed seagrass species except for Halophila engelmanni.  The densities of 
T. testudinum, H. wrightii, and Ruppia maritime were higher in shallow water, while S. filiforme and 
H. decipens density were higher in deeper water (Fourqurean et al. 2002).

There is typically a gradient of water column and sediment nutrient availability, with low N offshore and low P nearshore (Boyer and Jones 2002; Szmant and Forrester 1996).  In seagrass-dominated systems, it can be difficult to determine the actual nutrient status of the water column because of the rapid uptake of soluble nutrients (Tomasko and Lapointe 1991).  The ratio of N:P in seagrass leaf tissues has been used to infer the relative availability of nutrients and spatial patterns in limiting nutrients (Fourqurean and Zieman 2002; Burkholder et al. 2007).  Fourqurean and Zieman (2002) sampled T. testudinum in the Keys and observed strong spatial patterns in the relative amount of N and P in tissues.  On the Gulf side of the Keys, the N:P ratios indicated P limitation, except at the western margin of Florida Bay.  There was also a signal of P-limitation in the nearshore area along the Atlantic side of the Keys; however, farther offshore the ratios indicated N limitation.  It is important to note that factors other than nutrient availability, such as variation in light availability or any other factor that limits growth rate, can affect the nutrient content of seagrass tissue.  The spatial patterns of N:P ratios in this study are consistent with estimates of nutrient loading in different areas and sediment nutrient ratios.

The results of a 14-month fertilization experiment in the FKNMS indicated that there are taxon-specific responses to nutrient enrichment and that nutrient limitation differs for nearshore and offshore sites and for seagrass and algae (Ferdie and Fourqurean 2004).  The authors measured the response of seagrasses, macroalgae, epiphytes, and microalgae to additions of N and P fertilizer to assess gradients of nutrient limitation nearshore and offshore in the Florida Keys.  Sites were located on the east side of the upper Keys between the Keys and the reef tract.  Fertilization increased both sediment nutrient content and plant nutrient content.  There was rapid uptake of N by T. testudinum at both nearshore and offshore sites, but compared with N, P uptake was delayed.  There was an increase in T. testudinum leaf length, standing crop, and growth rates in response to N, but not P, at offshore sites, implying that seagrasses in offshore habitats in the upper Keys are N limited.  However, there was no clear increase in biomass or primary productivity of seagrasses at nearshore sites in response to added P.  Overall, nutrient enrichment resulted in increased seagrass biomass at offshore sites and increased macroalgal and sediment microalgal biomass at nearshore sites.

Seagrasses have been monitored in the FKNMS since 1995 (Fourqurean and Escorcia 2009).  The FKNMS seagrass monitoring program collects data on the distribution and abundance of seagrasses and other benthic plants and animals, seagrass tissue nutrient concentration and isotopic composition, and water quality (Fourqurean and Escorcia 2009).  The general objective of the seagrass monitoring is to measure the status and trends of seagrass communities to evaluate progress toward protection and restoration of the living marine resources (Fourqurean and Escorcia 2009).

The FKNMS uses two indicator criteria to assess status and trends in the seagrass communities.  The first criterion, called the elemental indicator (EI), is based on a conceptual model that predicts that T. testudinum is likely to be replaced by faster-growing competitors if nutrient availability is such that leaf tissue concentration is near the critical N:P  ratio of approximately 30:1.  An N:P ratio of 30:1 in seagrass leaves hypothetically indicates a balance in the availability of N and P, and monitoring deviations from this ratio can be used to infer whether N or P are limiting this species’ growth.  This indicator is based on the hypothesis that change in the N:P ratio over time, from an N or P limited ratio to a value closer to 30:1, could indicate eutrophication (Figure 5).

EI is calculated from the mean absolute difference in N:P from the ideal 30:1 for the 30 permanent monitoring sites based on the 10-year (1995–2005) mean N:P ratio at each of those sites.

Figure 5.  Conceptual model of the relationship between seagrass leaf nutrient content and nutrient availability in south Florida (Fourqurean and Rutten 2003).

The second indicator criterion, the Species Composition Indicator (SCI), measures whether the abundance of the slowest-growing seagrass species, T. testudinum, has changed compared with the abundance of other seagrass species.  The SCI is based on a dominance-eutrophication gradient conceptual model, where each of the six seagrass species can potentially dominate over a range of nutrient availability (Figure 6).  The conceptual model predicts that species dominance will change as nutrient availability changes.  However, field evidence suggests that species replacements may take place over a decade or more, making it difficult to relate the model prediction to observed response (Fourqurean and Escorcia 2009).  The model predicts that the first responses to eutrophication would be evidenced by an increase in the relative abundance of faster-growing seagrass species (H. wrightii and S. filiforme) at the expense of the now-dominant, slow-growing T. testudinum.  At later stages of eutrophication, macroalgae and microalgae would become the dominant primary producers.

Figure 6.  Conceptual model relating nutrient availability (trophic state) to the abundance of primary producers in nearshore marine waters of south Florida (Fourqurean and Rutten 2003).

The change in T. testudinum abundance compared with other species at a site is defined as “SLOW.”  The distribution and abundance of seagrasses are measured using rapid assessment Braun-Blanquet surveys.  The SCI is defined as the mean of SLOW over the 30 monitoring sites, where:

ATt = long-term mean abundance of T. testudinum

ASf = long-term mean abundance of S. filiforme

AHw = long-term mean abundance of H. wrightti

Amacroalgae = long-term mean abundance of macroalgal taxa

The long-term (1995–2005) EI is 8.28 ± 1.47 (± 1 standard error) and the long-term SCI is 0.48 ± 0.04 (± 1 standard error).  Despite a slight decreasing trend in the mean values for both the EI and SCI for 2007 and 2008, the means were all within the confidence estimate of the long-term averages for 1995 to 2005 (Figures 7 and 8).  Since the EI and SCI are not significantly different from the long-term mean, these indices suggest that there have been no significant adverse seagrass responses to nutrients for 2006-2008 across the FKNMS.

Figure 7.  EI status of seagrasses in the FKNMS for 2006, 2007, and 2008 compared with the long-term mean EI (dashed line) for 1995 to 2005 (Fourqurean and Escorcia 2009).

Figure 8.  SCI status of seagrasses in the FKNMS for 2006, 2007, and 2008 compared with the long-term mean SCI (dashed line) for 1995 to 2005 (Fourqurean and Escorcia 2009).

Increased macroalgal abundance, coincident with increased N concentrations, and in some cases, increased epiphyte productivity has been observed in nearshore areas of the Keys (Lapointe et al. 1994; Collado-Vides et al. 2005; Collado-Vides et al. 2007).  Lapointe et al. (1994) observed higher epiphyte loads on T.  testudinum and H. wrightii at sites closer to shore than at more oligotrophic offshore sites.  Collado-Vides et al. (2005) sampled seagrass and macroalgal abundance quarterly from 1996 to 2002 at 28 of the 30 permanent sites in the FKNMS and found the only significant gradient was with distance from land, with sites closer to land showing higher values.  The principal species of calcareous green algae in the seagrass meadows were Halimeda incrassata, H. monile, H. tuna, H. opuntia, Penicillus capitatus, P. dumetosus, P. pyryformis, Rhipocephalus phoenix, Udotea cyathiformis, and U. flabellum.  Each genus had a unique distribution and seasonal pattern of abundance, with lower abundance in winter and higher abundance in summer.  In addition, the 4 genera studied increased in abundance at most sites over the 7-year study period.  Regional-scale gradients in environmental conditions, such as salinity and water temperature, did not appear to influence abundance.  The higher values of the long term trend closer to land could suggest that nutrient influx from land.

Collado-Vides et al. (2007) used data from the long-term seagrass monitoring program and the water quality monitoring program in the FKNMS to detect the long-term abundance trends of macroalgae groups and their correlations with median nutrient concentrations for the 30 permanent monitoring  sites between March 1996 and May 2003. Their analyses suggest that the abundance and long-term increases in abundance of major macroalgal groups in the FKNMS were highest in areas with the highest availability of N.  The dominant group in the seagrass beds was calcareous green algae, dominated by Halimeda, followed by the “red other” group, dominated by Laurencia.  Results indicated that the abundance of almost all macroalgae groups was increasing over the course of the study, suggesting a “slow and steady” increase of slow-growing calcareous green algae, as opposed to bloom condition.

Elevated epiphyte loads on seagrass have been observed under eutrophic conditions and in response to N and P fertilization (Tomasko and Lapointe 1991; Lapointe et al. 1994; Ferdie and Fourqurean 2004).  Epiphyte loads on seagrasses in the FKNMS are highly seasonal, with peaks in early spring and minimum abundance in early fall (Figure 9; Fourqurean and Escorcia 2009; Fourqurean et al. in press; Lapointe et al. 1994).  However, Fourqurean et al. (in press) did not find a correlation between epiphyte load on turtle grass, T. testudinum, and water column or sediment nutrient concentrations.  Fourqurean et al. (in press) propose that the top-down grazer removal of epiphytes may obscure the influence of nutrients on the epiphyte community.

Figure 9.  Seasonality of epiphyte load, averaged across all 30 Level 1 sites, 2001–09 (Fourqurean et al., in press).

As noted previously, the seagrass beds in the Florida Keys are part of one of the most extensive SAV beds in the world.  Indices used for the seagrass monitoring program in the FKNMS indicate no decreased coverage or abundance of seagrass throughout the sanctuary.  The various indicators used in the FKNMS seagrass monitoring program show no statistically significant change in N:P ratios or community composition, when comparing the 2006 to 2008 period with 1995 to 2005 conditions (Fourqurean and Escorcia 2009).  Although there have been observations of Increased calcareous macroalgal growth, it is limited to nearshore areas and is correlated with areas that receive increased N loading (e.g. Collado-Vides et al. 2007). Note that calcareous algae are considered a viable benthic habitat.  Additionally, despite the slow increase in macroalgal abundance at nearshore sites, the community composition and coverage of seagrass overall in the FKNMS has not changed since monitoring began in 1995 (Collado-Vides et al. 2007; Fourqurean and Escorcia 2009).

Coral Reefs

The information in the following section is summarized from the State of Coral Reef Ecosystems of the Florida Keys report (Donahue et al. 2008), the 2006 Executive Summary of the Coral Reef Evaluation and Monitoring Project (CREMP) (Callahan et al. 2007), the 2010 US EPA Water Quality Protection Program Steering Committee Presentation (Ruznicka et al. 2010) and the CREMP EPA Steering Committee Meeting Presentation Summary (CREMP 2010).

There have been numerous studies that report loss of coral in recent decades.  For example, Chiappone and Sullivan (1994) reported changes observed after 4 years of monitoring, from 1989-1992, at two hardbottom sites in the Keys.  The Craig Key site (CKS) is located about 100 m offshore on the Atlantic side of Craig Key.  The Fiesta Key site (FKS) is located about 100 m from the shore on the FL Bay side of Fiesta Key.  At each site, benthic organsims were estimated visually in 1 m2 quadrats and included algae, sponges, octocorals, and stony corals.  During the study period, there were reductions in species numbers and abundance at the FKS.  At the CKS, there were increases in abundance of sessile benthic invertebrates, with the exception of a loss of coral coverage during 1990-91 due to the mortality of Millepora alcicornis (encrusting fire coral) colonies.

The Florida Fish and Wildlife Research Institute collects annual data on the status of coral habitats in the Florida reef tract through the Coral Reef Evaluation and Monitoring Project (CREMP), which began monitoring with 40 sites in 1996 and has since expanded to include sites in the Dry Tortugas and along the SE coast (Ruznicka et al. 2010, Donahue et al. 2008; Callahan et al. 2007).  CREMP sites encompass four reef habitat categories:  hardbottom, patch reef, and offshore deep and shallow reefs.   The following summarizes the main long-term trends in benthic cover from 1999-2008 and recent changes from 2008 to 2009 (CREMP 2010, Ruznicka et al. 2010).

Overall trends in percent cover from 1999 to 2008 were determined from a generalized mixed model regression (Figure 10).  Stony coral and sponge were both decreasing (p<0.001), octocoral was increasing (p<0.001), and there was no change in macroalgae percent cover (p<0.05) (CREMP 2010).  Macroalgae cover has been highly variable, but mean cover throughout the project is ~11% (Figure 10).    However, a significant increase in stony coral was observed between 2008 and 2009, for the first time in CREMP’s monitoring history.

Figure 10.  Line graph of mean annual percent cover for the four major benthic taxa recorded in CREMP image analysis (CREMP 2010).  Mean percent cover is pooled from 97 stations in the Florida Keys.

Five of the six most spatially dominant species of stony corals showed a significant decreasing trend in percent cover over the period 1996 to 2009 (Figure 11).  The only exception is Siderastrea siderea, which does not show any significant change.  Decreasing trends were observed for Monastraea annularis (p<0.001), Monastraea cavernosa (p<0.001), Porites astreoides (p<0.001) and Colpophyllia natans (p<0.001) while there was no change in Siderastrea sidereal (p>0.05).

The negative trends for the M. annularis complex are most evident in the Lower Keys and Dry Tortugas.  Overall, 43% (16 of 37) of all CREMP sites showed a negative trend and 69% (11 of 16) of the sites showing the negative trend are located in the Lower Keys and Tortugas regions (Figure 12).

Figure 11. Line graph of the mean annual percent cover for six species of stony corals.  Mean percent cover is pooled from 97 stations (CREMP 2010).

Figure 12.  Long term trends of percent cover from 1999-2008 at CREMP sites for a) stony corals and b) octocorals (CREMP 2010).

Soft coral, or octocoral, cover has significantly increased during the past 10 years after reaching a low point in 1999.  Cover of soft corals declined during the initial four years of the project, but has increased every year from 1999 through 2004 (Figure 12).  Although cover dipped again due to hurricane damage between 2005 and 2007, it was 14.8% in 2009, the highest value ever reported by CREMP.  The pattern of increased octocoral cover is most evident at the shallow forereef sites and coincides with the loss of Acropora palmata and Millepora complanata from these habitats during the mass bleaching and disease outbreaks that occurred from 1997-2000 (CREMP 2010).

The incidence of black band disease peaked in 1998, and this disease has not been recorded at more than 10 stations since then (Figure 13).  White diseases and “other” disease conditions have generally declined, although there were peaks in 1998-1999 and 2001-2002.  Long spined urchins (Diadema antillarum) are herbivores that help keep corals free of algal overgrowth.  Observations of D. antillarum densities have fluctuated, with highs reported in 2002 and 2005 and a general increasing trend since 1996.  There are three species of clionid sponges (Cliona delitrix, C. lampa, and C. caribbaea) monitored by CREMP.  These taxa are monitored because they can infest and damage coral colonies.  By habitat, clionid area increased between 2002 and 2006 at hardbottom sites.  At shallow, offshore, and patch reef sites, the populations have had minor fluctuations in mean clionid area through time.  In general, the patch reef sites have had higher clionid area than shallow or deep reef sites.

Figure 13.  Number of stations with stony coral disease recorded (Callahan et al. 2007).

In January 2010 Florida experienced one of its coldest 12-day periods on record.  Water temperatures were depressed below the lethal limit for corals (~16°C) for four to five days.  The coldest temperatures were observed at exchanges between Florida Bay and The Atlantic.  The most significant coral mortality was observed at sites closest to shore (Figure 14).  Mortality varied by station due to species composition.  Some stations which were primarily composed by species like M. annularis complex and M. cavernosa experienced a greater than 50% reduction in coral cover.  In many instances declines in octocoral cover exceeded those of stony corals (Figure 14).

Figure 14.  Percent cover comparisons between summer 2009 and February 2010 for a) stony corals and b) octocorals at eight stations (CREMP 2010).

Swart et al. (2005) reported temporal and spatial variability in δ13C and δ15N of the tissues and zooxanthellae of corals from nearshore and offshore reefs over a 2 year period, from 1995-1996.  There were no significant seasonal variations in the δ15N of coral or zooxanthellae at any of the sites, but there was a significant decrease in δ15N over the course of the study.  The decrease was greater in the zooxanthellae than the coral tissue and correlated with a decrease in the concentration of NH4+ and NO2- in the water column.  The δ13C values of the coral tissues did show seasonal patterns at some, but not all, sites.  The authors note that in the literature there is a discrepancy in interpretation of δ15N values in marine organisms.  In this study, there was no difference in δ15N in coral tissues and zooxanthellae from nearshore and offshore environments.  Assuming that the δ15N can be considered an indicator of anthropogenic influence, these results suggest that the Florida reefs are not affected by anthropogenic sources of N.  Similar to the N results, they also found no differences in δ13C between the inner and outer reefs.

Furman and Heck (2008) tested the single and interactive effects of the urchin (Diadema antillarum) grazing and effects of experimental nutrient enrichment on macroalgal community dynamics at a site in the lower Keys in the summer of 2004.  The objective of their study was to assess the relative importance of top-down and bottom-up processes in controlling macroalgal recruitment in a sub-tropical coral reef system.  Although algal succession differed from what had been expected, the results demonstrated the important role of herbivory in controlling benthic primary production, especially related to nutrient enrichment.  Results of fish counts and grazing assays suggested that piscine grazing was dominant as opposed to the expected echinoderm grazing.  Total fish abundance and herbivore density was consistently greater at enriched plots, leading to no significant increase in algal biomass at the nutrient enriched sites.

Nutrient enrichment is considered to be one of several factors potentially contributing to coral declines.  Lirman and Fong (2007) evaluated whether proximity to shore and connections to coastal bays (two measures of potential land-based sources of disturbance) adversely influenced coral community and population structure, and the abundance, distribution, and condition of corals within the Florida Reef Tract.  They found that spatial patterns in coral indicators do not appear to be directly related to water quality gradients influenced by land-based sources of disturbance.  Szmant (2002) evaluated reports based on the supporting evidence for nutrient enrichment and other factors that could simultaneously affect coral reefs.  She proposed that many factors, other than nutrient enrichment, can all be significant causes of coral death and also can cause changes in algal cover. These factors include:  decreased abundance of grazing fishes caused by over fishing, decreases in grazing urchins caused by disease, grazing preferences of the remaining grazers, lethal temperature stress, sedimentation, coral diseases, and outbreaks of coral predators and sea urchins (possibly as secondary effects of over fishing).  In the Florida Keys, Szmant (2002) found no reports to indicate that there is a clear connection between present water quality conditions and coral health that explain the declines in the reefs.  In fact, higher algal cover and lower adult coral cover and recruitment has been observed on outer reefs when compared with those reefs closer to shore (Chiappone 1996 and Miller et al. 2000 in Szmant 2002).  Note that the nearshore reefs are directly adjacent to potential sources of land-based nutrients, temperature stress, and sedimentation. Since nearshore reefs actually fared better than their offshore counterparts, this also suggests that land based sources of pollution, including nutrients, are not directly responsible for coral declines.

Additional Biological Resources

There are recreational, commercial, and “headboat” fisheries in the Florida Keys, with fishers including both residents and visitors (Donahue et al. 2008).   The Keys have been heavily fished, and fishing pressure has greatly increased due to augmented fishing power per vessel and a much larger recreational fishery (Ault et al. 1997).  Estimated fishery exploitation rates suggest that many of the Florida Keys reef fish stocks are overfished according to definitions for U.S. fisheries, and results show the classic pattern of serial overfishing where more vulnerable species are progressively depleted.  Data from Ault et al. (1997) also suggest that the biomass composition and abundance of the reef fish community may have changed over the past several decades.  In 1985, fisheries for conch (Strombas gigas) and Goliath and Nassau groupers were closed.  In response to overfishing, the FKNMS established a series of Sanctuary Preservation Areas (SPAs) in 1997.

Bohnsack et al. (2004) reported trends in reef fish landings from 1981 to 1992 for the Florida Keys.  Depending on the year, recreational landings comprised between 40% and 66% of total landings.  Reef fish accounted for 58% of total fish landings, 69% of recreational landings, and 16% of commercial landings.  Commercial landings were dominated by invertebrates (spiny lobster, shrimp, and stone crabs), which comprised 63% of total landings.

The Florida Fish and Wildlife Conservation Commission (FWC) conducts visual censuses between April and October to monitor finfish populations along the Atlantic margin of the Florida Keys in the FKNMS (Donahue et al. 2008).  The goal is to evaluate the relative abundance, size structure, and habitat utilization of the reef fish species that comprise local, commercial, and recreational fisheries (Figure 15; Table 5).

The FWC began a spiny lobster monitoring program in 1997 to test the effectiveness of no-take zones (Donahue et al. 2008).  Since protection, mean lobster size in reserves has been larger than legal size, but in exploited areas it has remained below the legal limit most of the time.  In most years, the abundance of lobsters declined in both reserves and exploited areas during the open season, but the declines were lower in the reserves.  The decline in abundance inside reserves during the open season indicates that the reserves are too small to adequately protect the lobsters from harvest.

The FWC also monitors the recovery of the queen conch population (Donahue et al. 2008).  Since the fishery was closed in 1985, there have been signs of recovery.

Figure 15.  Mean fish density by year (2000–06) as recorded during visual surveys in the Florida Keys (Donahue et al. 2008).

Table 5.  Catch statistics for the 15 more abundant reef fish species observed during the Florida Keys visual sampling, 1999–2006.

Note:  Percent (%) is the percentage of the total observations represented by that species; percent occurrence (% Occur) is the percentage of samples in which the species was observed; CV is the coefficient of variation.  Taxa are ranked in order of decreasing mean density (reference in Donahue et al. 2008).

Water Quality Studies

Inshore-to-offshore trends have been observed in many investigations of water quality in the Keys (e.g. Lapointe and Clark 1992; Boyer and Jones 2002; Boyer and Briceño 2009).  Lapointe and Clark (1992) characterized nutrient concentrations at 30 sites along a gradient from the watershed-coastal zone to offshore regions, including sites in Florida Bay and along the reef tract.  They observed elevated ammonia (NH4+), nitrate (NO3-), and particulate nitrogen (PN) concentrations in canal and seagrass meadows compared with patch and bank reef stations.  Soluble reactive phosphorus (SRP) and particulate phosphorus (PP) concentrations were significantly higher in canal systems compared with seagrass, patch reef, and bank reef stations.  Chlorophyll a was also higher in nearshore canal and seagrass systems compared with reef stations.

Boyer and Jones (2002) also reported declining inshore-to-offshore trends for NO3-, NH4+, TOC, total organic nitrogen (TON), and turbidity.  The water quality sampling was conducted quarterly at over 200 stations in the FKNMS and Shelf, and was conducted monthly in Florida Bay, Biscayne Bay, and the southwest coast mangrove estuaries.  Sampling stations were stratified according to depth, geography, distance from shore, proximity to tidal passes, and influence of shelf waters.  In the Lower Keys, inorganic N and TP were elevated in the Backcountry area.  TP concentrations in the Lower Keys decreased with distance offshore, but increased along transects in the Upper Keys (Boyer and Jones 2002).  The Upper Keys generally had lower nutrient concentrations than the Lower and Middle Keys.  Ambient water quality in the Lower Keys and Marquesas is most influenced by the Shelf; the Middle Keys is influenced by the Shelf and Florida Bay; the Backcountry is influenced by internal nutrient sources; and the Upper Keys water quality is influenced by intrusion of the Florida Current (Boyer and Jones 2002).

Water quality monitoring in the Florida Keys is collected as part of the larger Southeast Environmental Research Center (SERC) water quality monitoring network in south Florida.  Stations in the FKNMS are sampled quarterly and samples have been collected since it was established in 1995.  The information below is selected from the 2008 Annual Report of the Water Quality Monitoring Project for the FKNMS water quality protection program (Boyer and Briceño 2009).  The report includes information on data collected between March 1995 and December 2008.  Figure 16 shows the study area and subregions, and Figure 17 shows the location of the monitoring sites (Boyer and Briceño 2009).  The 2008 report includes data from 50 quarterly sampling events at 154 stations within the FKNMS.

Figure 16.  Map of study area (Boyer 2010).

Figure 18 shows the distribution of salinity across the region based on median salinities for each station.  The major freshwater sources to the region are the Shark River/Slough system on the southwest coast of the Florida peninsula and the Taylor Slough/C-111 Basin in eastern Florida Bay.  Southerly currents along the southwest coast and Shelf move water through the Keys passes and may impact the reef tract.

The distribution of dissolved NO3- and NH4+ are very different than that for salinity (Figure 19), with highest values in nearshore areas.  This implies that factors other than mixing of freshwater affect their distributions, such as phytoplankton and seagrass uptake as well as N2 fixation and benthic remineralization.

In contrast, TP distributions are very similar to salinity patterns on the west coast (Figures 17 and 20).  This implies that the source of P on the Shelf is partly terrestrial and partly from the southward transport of coastal waters from above Cape Romano.  It is important to note that the chlorophyll a concentrations are tightly coupled to TP availability (Figures 20 and 21).

Figure 17.  Florida International University (FIU) Southeast Environmental Research Center (SERC) Water Quality Monitoring Network.  Stations in red are sampled as part of the FKNMS monitoring program (Boyer and Briceño 2009).

Table 6 provides median values for water quality parameters collected as part of the monitoring program (Boyer and Briceño 2009).  TN values ranged from 0.0546 to 0.692 and 0.044 to 1.358 mg L-1 for surface and bottom samples respectively.  TP values ranged from 0.003 to 0.0441 and 0.0027 to 0.177 mg L-1 for surface and bottom samples respectively.  Median values were similar for surface and bottom values for both TN and TP.  Chlorophyll a ranged from 0 to 9.18 µg L-1 with a median of 0.19 µg L-1.  Surface and bottom DO concentrations were similar with median values of 5.26 and 4.88 respectively.

Table 6.  Water quality variables measured in the FKNMS, March 1995–December 2008 (Boyer and Briceño 2009).

1 mg L-1 = Milligrams per liter

NTU = Nephelometric Turbidity Unit

SU = Standard Unit

g L-1 = Micrograms per liter

| Variable1 | Depth | Median | Minimum | Maximum |
| --- | --- | --- | --- | --- |
| NO3- (mg L-1) | Surface | 0.00252 | 0 | 0.03066 |
| NO3- (mg L-1) | Bottom | 0.00168 | 0 | 0.02576 |
| NO2- (mg L-1) | Surface | 0.00056 | 0 | 0.00462 |
| NO2- (mg L-1) | Bottom | 0.00042 | 0 | 0.00266 |
| NH4+ (mg L-1) | Surface | 0.00686 | 0.00042 | 0.10598 |
| NH4+ (mg L-1) | Bottom | 0.0042 | 0.00014 | 0.04676 |
| TN (mg L-1) | Surface | 0.1435 | 0.0546 | 0.69188 |
| TN (mg L-1) | Bottom | 0.1211 | 0.04438 | 1.35772 |
| TON (mg L-1) | Surface | 0.12362 | 0 | 0.68754 |
| TON (mg L-1) | Bottom | 0.1106 | 0.02688 | 1.3566 |
| TP (mg L-1) | Surface | 0.0066 | 0.003 | 0.0441 |
| TP (mg L-1) | Bottom | 0.006 | 0.0027 | 0.0177 |
| SRP (mg L-1) | Surface | 0.0009 | -0.0003 | 0.0234 |
| SRP (mg L-1) | Bottom | 0.0009 | -0.0009 | 0.0204 |
| Turbidity (NTU) | Surface | 0.51 | 0 | 10.73 |
| Turbidity (NTU) | Bottom | 0.37 | 0.01 | 5.78 |
| Salinity (SU) | Surface | 36.25 | 23.49 | 47.3 |
| Salinity (SU) | Bottom | 36.37 | 33.68 | 38.9 |
| Temperature (°C) | Surface | 26.96 | 18.87 | 33.2 |
| Temperature (°C) | Bottom | 26.75 | 18.8 | 33 |
| DO (mg L-1) | Surface | 5.26 | 2.07 | 9.5 |
| DO (mg L-1) | Bottom | 4.88 | 2.97 | 8 |
| DO Saturation (%) | Surface | 80.05 | 0 | 134.7 |
| DO Saturation (%) | Bottom | 73.34 | 0 | 119.3 |
| Chlorophyll a (µg L-1) | Surface | 0.19 | 0 | 9.18 |

Figure 18.  Median salinity, 1995–2009 (Boyer and Briceño 2009).

Figure 19.  Dissolved inorganic nitrogen (micromolar[µM]) median, 1995–2007 (Boyer and Briceño 2009).

Figure 20.  TP (µM) median, 1995–2007 (Boyer and Briceño 2009).

Figure 21.  Chlorophyll a (µg L-1) median, 1995–2007 (Boyer and Briceño 2009).

The monitoring project resulted in several important findings.  First, the study documented elevated nitrate in the inshore waters of the Keys (Figure 22).  This result was evident from the first sampling event in 1995 and continues to be a characteristic of the ecosystem.  Interestingly, this gradient was not observed in a comparison transect from the Tortugas (no human impact).  This type of distribution implies an inshore source that is diluted by low-nutrient Atlantic Ocean waters.  The presence of a similar gradient in TOC and decreased variability in salinity from land to reef also support this concept. There were no trends in either TP or chlorophyll a with distance from land.

Second, the study showed that the highest chlorophyll a concentrations are seen on the Shelf, with a strong gradient towards the Marquesas and Tortugas (Figure 21).  This is due to the higher TP concentrations on the Shelf as a result of the southerly advection of water along the coast.

Figure 22.  Dissolved inorganic nitrogen (µM) (Boyer and Briceño 2009).

Waters on the 303(d) List

The halo zone waters of the Keys were listed as impaired on the 1998 303(d) list, however they have yet to be verified as impaired under the Impaired Waters Rule (IWR), pending a decision by EPA on the FKRAD.  For the purposes of the FKRAD, the halo zone, or bubble WBIDS, waters are defined as extending from the coastline out to 500 m.  Nearshore waters extend from 500 m to 12,100 m and farfield waters, which are assumed to have water quality controlled by outside influences as opposed to land based nutrient sources, are beyond the 12,100 m.  Figure 23 shows a map of the Keys with the WBIDS and regions identified.

In addition to nutrients, waters in the Florida Keys are listed as impaired for several other parameters:  mercury (in fish tissue), bacteria, and fecal coliform bacteria (Table 7).  Mercury impairments are not related to anthropogenic nutrient inputs.  The relationship between bacteria and anthropogenic nutrients is more complex, because although potential sources of bacteria may also be sources of nutrients, the presence of fecal coliform or enterococci bacteria does not necessarily indicate the presence of anthropogenic nutrients.

Table 7. Verified impairments in the Florida Keys basin.

| Basin Group Name | Planning Unit | Waterbody ID (WBID) | Water Segment Name | Waterbody Type | Waterbody Class1 | Parameters Assessed Using the 2001 Impaired Surface Waters Rule (IWR) | Concentration Causing Impairment2 | Priority for TMDL Development3 | Comments (# Exceedances/# Samples) pp = Planning Period vp = Verified Period4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Florida Keys | Upper Keys | 6005EB | JOHN PENNEKAMP STATE PARK | BEACH | 3M | Bacteria | Beach Advisories > 21 days/yr | High | Beach advisories > 21 days/yr in 2002, 2003, 2004, 2005, and 2006. |
| Florida Keys | Lower Keys | 8073C | SIMONTON STREET BEACH (KEY WEST) | BEACH | 3M | Bacteria | Beach Advisories > 21 days/yr | High | Beach advisories > 21 days/yr in 2004, 2005, and 2006. |
| Florida Keys | Lower Keys | 8073E | SOUTH BEACH (KEY WEST) | BEACH | 3M | Bacteria | Beach Advisories > 21 days/yr | High | Beach advisories > 21 days/yr in 2002, 2003, 2004, 2005, and 2006. |
| Florida Keys | Lower Keys | 8073F | HIGGS BEACH | BEACH | 3M | Bacteria | Beach Advisories > 21 days/yr | High | Beach advisories > 21 days/yr in 2001, 2002, 2003, 2004, 2005, and 2006. |
| Florida Keys | Lower Keys | 8073G | REST BEACH (KEY WEST) | BEACH | 3M | Bacteria | Beach Advisories > 21 days/yr | High | Beach advisories > 21 days/yr in 2001. (No data available for 2003, 2004, 2005, and 2006.) |
| Florida Keys | Lower Keys | 8073H | SMATHERS BEACH | BEACH | 3M | Bacteria | Beach Advisories > 21 days/yr | High | Beach advisories > 21 days/yr in 2002, 2003, 2005, and 2006. |
| Florida Keys | Middle Keys | 8076A | VETERAN'S BEACH | BEACH | 3M | Bacteria | Beach Advisories > 21 days/yr | High | Beach advisories > 21 days/yr in 2004 and 2005 |
| Florida Keys | Upper Keys | 8078A | HARRY HARRIS COUNTY PARK | BEACH | 3M | Bacteria | Beach Advisories > 21 days/yr | High | Beach advisories > 21 days/yr in 2002, 2004, 2005, and 2006. |
| Florida Keys | Upper Keys | 8078B | ISLAMORADA LIBRARY BEACH | BEACH | 3M | Bacteria | Beach Advisories > 21 days/yr | High | Beach advisories > 21 days/yr in 2004, 2005, and 2006. |
| Florida Keys | Upper Keys | 8078C | FOUNDER | BEACH | 3M | Bacteria | Beach Advisories > 21 days/yr | High | Beach advisories > 21 days/yr in 2002, 2004, 2005, and 2006. |
| Florida Keys | Lower Keys | 8080A | BAHIA HONDA SANDSPUR | BEACH | 3M | Bacteria | Beach Advisories > 21 days/yr | High | Beach advisories > 21 days/yr in 2005 and 2006 |
| Florida Keys | Lower Keys | 8080B | BAHIA HONDA OCEANSIDE | BEACH | 3M | Bacteria | Beach Advisories > 21 days/yr | High | Beach advisories > 21 days/yr in 2001, 2005, and 2006. |
| Florida Keys | Middle Keys | 8081A | COCO PLUM BEACH | BEACH | 3M | Bacteria | Beach Advisories > 21 days/yr | High | Beach advisories > 21 days/yr in 2001, 2004, 2005, and 2006. |
| Florida Keys | Upper Keys | 6009 | PLANTATION KEY | COASTAL | 3M | Copper | > 3.7 µg/L | Medium | PP = No Data; VP = 21 / 29 |
| Basin Group Name | Planning Unit | Waterbody ID (WBID) | Water Segment Name | Waterbody Type | Waterbody Class1 | Parameters Assessed Using the 2001 Impaired Surface Waters Rule (IWR) | Concentration Causing Impairment2 | Priority for TMDL Development3 | Comments (# Exceedances/# Samples) pp = Planning Period vp = Verified Period4 |
| Florida Keys | Upper Keys | 6019 | LOWER MATECUMBE KEY | COASTAL | 3M | Copper | > 3.7 µg/L | Medium | PP = No Data; VP = 8 / 13 |
| Florida Keys | Lower Keys | 8073 | KEY WEST AND OUTLYING ISLANDS | COASTAL | 3M | Fecal Coliform | > 400 colonies/100 ml | Low | pp = 325/2262; vp = 362/3036 |
| Florida Keys | Upper Keys | 6006A | SOUTH KEY LARGO | COASTAL | 3M | Copper | > 3.7 µg/L | Medium | PP = No Data; VP = 42 / 86 |
| Florida Keys | Upper Keys | 6006C | NORTH KEY LARGO | COASTAL | 3M | Copper | > 3.7 µg/L | Medium | PP = No Data; VP = 5 / 8. Assessment based on Period of Record data |
| Florida Keys | Middle Keys | 6011A | VACA KEY | COASTAL | 3M | Copper | > 3.7 µg/L | Medium | PP = No Data; VP = 19 / 47 |
| Florida Keys | Lower Keys | 6012A | BIG PINE KEY | COASTAL | 3M | Copper | > 3.7 µg/L | Medium | PP = No Data; VP = 8 / 21 |
| Florida Keys | Lower Keys | 6013C | CUDJOE KEY | COASTAL | 3M | Copper | > 3.7 µg/L | Medium | PP = No Data; VP = 5 / 19 |
| Florida Keys | Lower Keys | 6014A | KEY WEST | COASTAL | 3M | Fecal Coliform | > 400 colonies/100 ml | Low | pp = 418/2256; vp = 459/2573 |
| Florida Keys | Lower Keys | 6014A | KEY WEST | COASTAL | 3M | Copper | > 3.7 µg/L | Medium | PP = No Data; VP = 9 / 31 |
| Florida Keys | Lower Keys | 6014B | STOCK ISLAND | COASTAL | 3M | Copper | > 3.7 µg/L | Medium | PP = No Data; VP = 7 / 14 |

Figure 23. Map of the Florida Keys with WBIDS and regions (north, central, southcentral, and southern) identified (FDEP 2008a).

Figure 24.  WBID models in the Florida Keys (McClelland 2010).

Numeric Nutrient Criteria Recommendations

FDEP’s evaluation of the biological resources of the Florida Keys indicates that the waters in the nearshore zone support a healthy well-balanced community, are attaining the designated use, and have not shown adverse effects from nutrient enrichment in the nearshore area (state waters beyond 500 m).   Although there have been changes in some communities (e.g., coral loss) (Table 1), evidence suggests that causes other than nutrients are responsible for the responses.   Additionally, long term monitoring of water quality data indicates consistently low water column nutrient concentrations over time.   For the halo zone area, however, current nutrient conditions in the canals may not be protective of canals in the Keys.   While nutrient criteria for the canals and halo zone still need to be determined, FDEP supports the nutrient load reductions proposed in the FKRAD in order to improve water quality in the halo zone and help ensure that criteria are met in nearshore.

FDEP is considering two methods for establishing numeric nutrient criteria in the Keys.  The first consists of adopting the criteria established in the FKRAD, which are based on improving water quality in the inshore halo zone to modeled targets (FDEP 2008a).  The second approach is to subdivide the nearshore waters (state waters beyond 500 m) into zones of similar water quality and establish criteria to maintain the current nutrient data distributions, which would continue to support the existing healthy condition.

FKRAD recommendations

The FKRAD used two approaches to determine their proposed nutrient targets.  One set of targets were developed based on an insignificant increase in natural background TN and TP levels at 500 m from shore.  An increase of 10 µg l-1 TN or 2 µg l-1 TP was considered an insignificant increase at 500 m from the shore.  Natural background is the predicted model result in the Halo Zone with all of the urban land uses changed to natural conditions (Figure 24).  The model is described in more detail in the FKRAD (FDEP 2008a).  According to the model, the targets, which FDEP interprets as long term averages, are defined as follows for each area of the Keys (Tables 8-11):

Table 8.  Natural background conditions (modeled) and water quality targets for the northern Keys (FDEP 2008a).

Table 9.  Natural background conditions (modeled) and water quality targets for the central Keys (FDEP 2008a).

Table 10.  Natural background conditions (modeled) and water quality targets for the south-central Keys (FDEP 2008a).

Table 11.  Natural background conditions (modeled) and water quality targets for the southern Keys (FDEP 2008a).

The second approach outlined in the FKRAD sets the target nutrient concentrations at the concentrations measured during the 1985 designation as an OFW.  Data from the 1985 designation are provided below – the data and ranges represent nutrient thresholds (FDEP 2008a).

Table 12. 1985 FDEP OFW water quality data (FDEP 2008a).

For comparison, Table 13 provides the range and means of the modeled targets for both TN and TP.  Minimum and maximum values are the modeled values for the individual bubble WBIDs and the average values are the mean of modeled WBID criteria for either the bayside (N) or Oceanside (S) of the Keys.   The modeled criteria and the OFW values for bayside TN are comparable, however the modeled maximum values for TN for the oceanside WBIDs are lower and result in a lower average value.  For TP, the range of the modeled values is much less than range of the OFW values and the average of the modeled targets are lower for both bayside and oceanside  areas.  FDEP does not suggest using the OFW numbers as targets for water quality criteria because the values are based on a limited number of samples taken during a short sampling period.  The OFW values are also considerably higher than the modeled recommendations outlined in the FKRAD as well as current data on Florida Keys water quality.

Table 13. Range and means of modeled TN and TP targets for bayside and oceanside bubble WBIDs (FKRAD 2008).

| Modeled water quality targets |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | TN µg l-1 |  |  | TP µg l-1 |  |  |
|  | Average | Minimum | Maximum | Average | Minimum | Maximum |
| Bayside | 353 | 179 | 764 | 10.9 | 8 | 13 |
| Oceanside | 133 | 123 | 142 | 7.7 | 7 | 8 |

FDEP Nutrient Criteria Approach

The second approach to setting numeric nutrient criteria is to use water quality data collected by FIU SERC as part of the FKNMS water quality monitoring network to establish nutrient criteria that would maintain nutrients at the current concentrations that are fully protective of aquatic life.  Under this approach, TN and TP criteria would be established for sub-basins having similar water quality outside of the 500 m limit of the FKRAD criteria.

Regionalization

FIU has utilized the data collected as part of their Coastal Water Quality Monitoring Network initiated in 1992 to spatially aggregate monitoring sites located in south Florida based on similar water quality characteristics (Boyer and Briceño 2008).  Based on the unique water quality and geologic characteristics observed in south Florida, Everglades National Park (ENP) and Florida International University (FIU) have proposed that the estuarine and coastal waters of south Florida be divided into sub-basins for the purpose of deriving nutrient criteria.  On behalf of ENP, FIU has recently completed an extensive statistical characterization of coastal waters from Biscayne Bay to Dry Tortugas to Pine Island Sound.  The recent analysis was similar to the earlier spatial regionalization method used by FIU, but utilized a longer period of record and a modified set of parameters.  The analysis used a combination of Principal Component Analysis and Hierarchical Clustering of multiple (from 8 to 13) parameters, produced a division of the bays as shown in the figure and table below.  More detail concerning the analysis conducted by Henry Briceño, Joe Boyer, and Peter Harlem from FIU can be found in Appendix A.  Additionally, the GIS layers with information on the sub-basins as well as FIU’s working files with data and statistical analysis are available at a FIU’s ftp site provided earlier.

This regionalization scheme will be the basis for proposing numeric criteria for each sub-basin. A final step in the process will be to evaluate the similarity of the derived criteria with the purpose of combining sub-basins with similar criteria (i.e., criteria that are not statistically different) for total phosphorus, total nitrogen, and chlorophyll-a.

On the basis of the statistical analysis performed, the south Florida bays and coastal waters and the Florida Keys, were divided as detailed in Table 14 and illustrated in Figures 25 and 26, respectively.

Table 14.  Proposed regionalization for the Florida Keys.

| Sub-Basin Map Code | Sub-Basin Name |
| --- | --- |
| BKB | Back Bay |
| BKS | Backshelf |
| LK | Lower Keys |
| MAR | Marquesas |
| MK | Middle Keys |
| OCEAN | Oceanside |
| UK | Upper Keys |
| BKB | Back Bay |

Figure 25.  Proposed Marine Nutrient Regions for South Florida.

77

Figure 26.  Proposed Marine Nutrient Regions for the Florida Keys.

Proposed Numeric Criteria

To be applied consistently and to provide an appropriate level of protection, water quality criteria need to include magnitude, frequency, and duration components.  The magnitude is a measure of how much of a pollutant may be present in the water without an unacceptable adverse effect.  Duration is a measure of how long a pollutant may be above the magnitude, and frequency relates to how often the magnitude may be exceeded without adverse effects.  It is preferable to derive the magnitude component of a criterion through a cause-effect relationship (such as that measured through toxicity testing).  The magnitude would then be set at a level that would protect a majority of the sensitive aquatic organisms inhabiting the system.  Absent sufficient data to demonstrate a cause-effect relationship, the magnitude may be set at a level designed to maintain the current data distribution, accounting for natural temporal variability, assuming the current conditions are protective of the designated uses of the waterbody.  Since a criterion derived based on the existing data distribution has no direct link to any observed cause-and effect relationship, it is assumed that maintaining the current data distribution will preserve the uses associated with that distribution.

The frequency and duration components of the criteria are best established as additional descriptors of the reference condition data distribution.  Specifically, these components should be part of a statistical test designed to determine whether the long-term distribution of data has shifted upward from the reference distribution.  This test would then be used to determine whether future monitoring data are consistent with the magnitude (long-term average) defined by the reference dataset.  It is critical to account for the natural variability surrounding the magnitude expression and to control for statistical errors.  The magnitude component can be set at the long-term central tendency (geometric mean) of the distribution, while the frequency and duration components describe how often and by how much nutrient concentrations can be above the central tendency while still being consistent with the reference distribution.  The derivation of the magnitude, frequency and duration components of numeric nutrient criteria for the Southwest Coastal area is described briefly below.  More details concerning the statistical approaches used can be found in the document, Overview of FDEP Approaches for Nutrient Criteria Development in Marine Waters (Appendix B).

Magnitude

The magnitude component represents a level of nutrients demonstrated to be protective of the designated use.  For the “healthy existing conditions” approach, the magnitude can be interpreted as the central tendency of the baseline distribution and may be set at a level that represents a long-term average condition of that distribution.  For the “healthy existing conditions” approach, the Department proposes establishing the magnitude as an annual geometric mean, not to be exceeded more than once over a five- year period.

The objective of this magnitude component is to maintain the long-term average concentration at the level observed in the baseline data set.  Exceedance of the one magnitude component more than once in a five-year period would provide strong evidence that the waterbody nutrient levels had increased above the baseline distribution.

Frequency and Duration

To provide a consistent and appropriate level of protection, the duration and frequency components of the criteria must be consistent with the derivation of the magnitude component.  While the magnitude component of the criteria is being expressed as a long-term geometric mean concentration, it is not practical to assess compliance with the criteria on the same long-term basis.  Instead, a statistical test can be developed to allow the application of the criteria on a shorter-term basis.  For the criteria to be protective, the duration component of the criteria (e.g., single sample maximum, annual geometric mean) must be linked to the response time frame of the sensitive endpoint.   Short-term averaging periods (e.g., 1 to 30 days) would be appropriate for nutrient criteria where a sufficiently robust cause-effect relationship has demonstrated that a eutrophic response occurs over such time frames.  If, however, such a short-term response cannot be demonstrated, or there is no indication of use impairment, then longer averaging periods should be considered.

For example, since the relationship between nutrient and chlorophyll a response in Florida lakes was extremely weak, with a much more robust relationship found when data were evaluated based on annually averaged log-transformed data, FDEP and EPA used an averaging period of a year to assess the enrichment in Florida lakes with the criteria being expressed as an annual geometric mean.  Likewise, the nutrient criteria for estuaries will be assessed annually.  Since the duration and frequency components of the criteria must be consistent with the derivation of the magnitude component to provide a consistent and appropriate level of protection, the long-term geometric mean target cannot simply be applied as an annual mean.  Doing so would result in unacceptably high Type I failure rate (identifying a healthy system as being impaired), since approximately 50% of the individual years can be expected to be above the long-term mean.  Therefore, the long-term target must be adjusted to allow for the application to a shorter duration with an acceptable Type I error rate of no more than 10%.  This assessment of the Type I error rate is related only to addressing the null hypothesis that future monitoring data are equivalent to the baseline distribution.  This Type I error does not take into account the possibility that a higher nutrient threshold would be fully protective of the use.  The Type I error rate, for the current application, may be defined as the rate of incorrectly concluding that the mean of (future) monitoring data is greater than the baseline or reference long-term mean condition identifying.  Type I statistical errors result in the management decision error to incorrectly list a healthy waterbody as impaired.

An annual target concentration with an approximate 10% Type I error rate for a given frequency can be derived by appropriately accounting for the annual variability above the mean.  This annual target concentration can be derived as an upper percentile of the distribution of the annual geometric mean concentrations.  Previous proposals by EPA have used 3-year assessment periods to express the magnitude and duration nutrient criteria components.  Assuming a 3-year assessment period, it can be statistically determined that using the 80th percentile of the annual geometric means from the long-term dataset with a frequency and duration of no more than once during the 3-year period will achieve the targeted 10% error rate.  Therefore the proposed criteria will be applied such that the 80th percentile of the annual geometric mean concentrations cannot be exceeded in more than 1 out of 3 years.

Summary of the Proposed Criteria

FDEP proposes that the magnitude component of protective nutrient criteria be expressed as a long- For a “healthy existing conditions” dataset, the Department is considered several potential ways to express the NNC.  The Department’s proposed approach is to set the magnitude as an annual geometric mean limit established at the upper 80 percent prediction limit of the spatially averaged annual geometric means, with a frequency and duration of no more than 1 annual geometric mean exceeding the limit in a 3-year period.

For example, the TP criterion for the Lower Keys would then be expressed as follows:  “the annual geometric mean TP shall not surpass 0.008 mg/L more than once a year in a 3-year period.”  This establishes the magnitude components as an annual geometric mean of a network of sites, with the frequency and duration components used to assess whether the inter-annual variability is consistent with the maintenance of the long-term geometric mean (0.006 mg/L), considering natural variability around that average.  Duration is expressed as a 1- year periods.  The proposed annual limits for the protection of a healthy, well-balanced aquatic community in the Florida Keys, for each segment, are provided in Table 15.

Table 15.  Proposed numeric nutrient criteria for all segments of the Florida Keys including TP, TN, and chlorophyll a.  For assessment purposes, the average of all stations in a segment shall not exceed the network average more than once in a 3-year period.

| Total Phosphorus (mg∙ l-1) |  |  |
| --- | --- | --- |
|  | Existing Long Term Geometric Mean | Maximum Annual Geometric Mean (1 of 3-year exceedance rate) |
| Back Bay | 0.007 | 0.009 |
| Backshelf | 0.010 | 0.011 |
| Lower Keys | 0.006 | 0.008 |
| Marquesas | 0.007 | 0.008 |
| Middle Keys | 0.006 | 0.007 |
| Oceanside | 0.006 | 0.007 |
| Upper Keys | 0.005 | 0.007 |
| Total Nitrogen (mg∙ l-1) |  |  |
|  | Existing Long Term Geometric Mean | Maximum Annual Geometric Mean (1 of 3-year exceedance rate) |
| Back Bay | 0.20 | 0.25 |
| Backshelf | 0.19 | 0.23 |
| Lower Keys | 0.17 | 0.21 |
| Marquesas | 0.15 | 0.21 |
| Middle Keys | 0.17 | 0.22 |
| Oceanside | 0.13 | 0.17 |
| Upper Keys | 0.15 | 0.18 |
| Chlorophyll a (µg∙ l-1) |  |  |
|  | Existing Long Term Geometric Mean | Maximum Annual Geometric Mean (1 of 3-year exceedance rate) |
| Back Bay | 0.22 | 0.3 |
| Backshelf | 0.52 | 0.7 |
| Lower Keys | 0.26 | 0.3 |
| Marquesas | 0.45 | 0.6 |
| Middle Keys | 0.18 | 0.3 |
| Oceanside | 0.20 | 0.3 |
| Upper Keys | 0.17 | 0.2 |

Comparison of FKRAD and FDEP approaches

Tables 16 and 17 compare the FKRAD targets for the halo zone with the FDEP proposed criteria for the nearshore zone for each of the sub-basins.  For both TN and TP, the OFW targets are higher than both the modeled targets and the criteria proposed for the sub-basins in the nearshore zone, with the exception of the modeled targets for WBIDs 8N and 9N on the bay-side of the upper Keys.  In most cases, the TP targets in the FKRAD agree with concurrent sub-basin two out of five year annual limits calculated by FDEP.  There are several TN WBID targets on the bay side of the Keys, however, that have considerably higher target concentrations in the FKRAD than those calculated for adjacent sub-basins by FDEP in Florida Bay.  FDEP is seeking input on the proposed approaches.

Table 16.  Comparison of TN (µg l-1) targets for halo zone modeled WBIDs and OFW targets (from the FKRAD) with nearshore FDEP criteria proposed for sub-basins using FIU long-term water quality data.  For maps showing the locations of WBIDs and sub-basins, see Figures 24 and 25.

| Halo Zone (500 m) targets (FKRAD) |  |  |  | FDEP Criteria |  |
| --- | --- | --- | --- | --- | --- |
| Modeled bubble WBIDs |  | OFW |  | Proposed criteria by sub-basin |  |
| WBID model ID | Target |  | Target | Sub-basin name | Long term limit (1 out of 3-year limit) |
| 1N | 179 | Bayside | 370 | Lower Keys | 170 (210) |
| 1N | 179 | Bayside | 370 | Back Bay | 200 (250) |
| 2N | 205 | Bayside | 370 | Back Bay | 200 (250) |
| 3N | 215 | Bayside | 370 | Back Bay | 200 (250) |
| 4N | 221 | Bayside | 370 | Back Bay | 200 (250) |
| 5N | 225 | Bayside | 370 | Back Bay | 200 (250) |
| 6N | 248 | Bayside | 370 | Back Bay | 200 (250) |
| 7N | 399 | Bayside | 370 | Back Bay | 200 (250) |
| 7N | 399 | Bayside | 370 | West Florida Bay* | 300 (370) |
| 8N | 764 | Bayside | 370 | West Florida Bay* | 300 (370) |
| 8N | 764 | Bayside | 370 | South Florida Bay* | 490 (640) |
| 9N | 739 | Bayside | 370 | South Florida Bay* | 490 (640) |
| 9N | 739 | Bayside | 370 | East Central Florida Bay* | 500 (650) |
| 10N | 334 | Bayside | 370 | East Central Florida Bay* | 500 (650) |
| 10N | 334 | Bayside | 370 | Manatee Bay-Barnes* Sound* | 470 (580) |
| 10N | 334 | Bayside | 370 | Card Sound* | 260 (330) |
| 1S | 142 | Oceanside | 288 | Lower Keys | 170 (210) |
| 2S | 135 | Oceanside | 288 | Lower Keys | 170 (210) |
| 3S | 138 | Oceanside | 288 | Lower Keys | 170 (210)) |
| 4S | 141 | Oceanside | 288 | Lower Keys | 170 (210)) |
| 5S | 135 | Oceanside | 288 | Lower Keys | 170 (210) |
| 6S | 135 | Oceanside | 288 | Lower Keys | 170 (210) |
| 7S | 125 | Oceanside | 288 | Middle Keys | 170 (220) |
| 8S | 124 | Oceanside | 288 | Middle Keys | 170 (220) |
| 9S | 123 | Oceanside | 288 | Upper Keys | 150 (180) |
| 10S | 128 | Oceanside | 288 | Upper Keys | 150 (180) |
|  |  |  |  | Marquesas | 150 (210) |

*Three sub-basins from Florida Bay and two from Biscayne Bay are included for comparison because they are adjacent to halo zone WBIDs on the bay-side of the upper Keys.

Table 17.  Comparison of TP (µg l-1) targets for halo zone modeled WBIDs and OFW targets (from the FKRAD) with nearshore criteria proposed for sub-basins using FIU long-term water quality data.  For maps showing the locations of WBIDs and sub-basins, see Figures 24 and 25.

| Halo Zone (500 m) targets (FKRAD) |  |  |  | FDEP Criteria |  |
| --- | --- | --- | --- | --- | --- |
| Modeled bubble WBIDs |  | OFW |  | Proposed criteria by sub-basin |  |
| WBID model ID | Target | Location | Target | Sub-basin name | Long term limit (1 out of 3-year limit) |
| 1N | 11 | Bayside | 14 | Lower Keys | 6 (8) |
| 1N | 11 | Bayside | 14 | Back Bay | 7 (9) |
| 2N | 12 | Bayside | 14 | Back Bay | 7 (9) |
| 3N | 12 | Bayside | 14 | Back Bay | 7 (9) |
| 4N | 12 | Bayside | 14 | Back Bay | 7 (9) |
| 5N | 10 | Bayside | 14 | Back Bay | 7 (9) |
| 6N | 10 | Bayside | 14 | Back Bay | 7 (9) |
| 7N | 13 | Bayside | 14 | Back Bay | 7 (9) |
| 7N | 13 | Bayside | 14 | West Florida Bay* | 12 (15) |
| 8N | 11 | Bayside | 14 | West Florida Bay* | 12 (15) |
| 8N | 11 | Bayside | 14 | South Florida Bay* | 7 (9) |
| 9N | 10 | Bayside | 14 | South Florida Bay* | 7 (9) |
| 9N | 10 | Bayside | 14 | East Central Florida Bay* | 6 (7) |
| 10N | 8 | Bayside | 14 | East Central Florida Bay* | 6 (7) |
| 10N | 8 | Bayside | 14 | Manatee Bay-Barnes* Sound* | 6 (7) |
| 10N | 8 | Bayside | 14 | Card Sound* | 6 (8) |
| 1S | 8 | Oceanside | 15 | Lower Keys | 6 (8) |
| 2S | 8 | Oceanside | 15 | Lower Keys | 6 (8) |
| 3S | 8 | Oceanside | 15 | Lower Keys | 6 (8) |
| 4S | 7 | Oceanside | 15 | Lower Keys | 6 (8) |
| 5S | 8 | Oceanside | 15 | Lower Keys | 6 (8) |
| 6S | 7 | Oceanside | 15 | Lower Keys | 6 (8) |
| 7S | 7 | Oceanside | 15 | Middle Keys | 6 (7) |
| 8S | 8 | Oceanside | 15 | Middle Keys | 6 (7) |
| 9S | 8 | Oceanside | 15 | Upper Keys | 5 (7) |
| 10S | 8 | Oceanside | 15 | Upper Keys | 5 (7) |
|  |  |  |  | Marquesas | 7 (8) |

*Three sub-basins from Florida Bay and two from Biscayne Bay are included for comparison because they are adjacent to halo zone WBIDs on the bay-side of the upper Keys.

References

Ault, J.S., J.A. Bohnsack, and G.A. Meester.  1997.  A retrospective (1979–1996) multispecies assessment of coral reef fish stocks in the Florida Keys.  Fishery Bulletin 96(3): 395–414.

Bohnsack, J.A., D.E. Harper, and D.B. McClellan. 1994.   Fisheries trends from Monroe County, Florida.  Bulletin of Marine Science 54(3): 982–1018.

Boyer, J.N.  2010.  Nutrient criteria in the Florida Keys National Marine Sanctuary and Dry Tortugas National Park.  Presented at the FDEP Estuary and Coastal Numeric Nutrient Criteria Development Meeting, Miami, FL, March 3, 2010.  Available:  http://www.dep.state.fl.us/water/
wqssp/nutrients/docs/estuarine/miami/fknms_tortugas_nutrient_criteria.pdf.

Boyer, J.N., and H.O. Briceño.  2009.  2008 Annual report of the Water Quality Monitoring Project for the Florida Keys National Marine Sanctuary.  Southeast Environmental Research Center Technical Report T-437.  Miami, FL:  Florida International University.

Boyer, J.N., and R.D. Jones.  2002.  A view from the bridge:  External and internal forces affecting the ambient water quality of the Florida Keys National Marine Sanctuary.  In:  J.W. Porter and K.G. Porter (Eds.), The Everglades, Florida Bay and coral reefs of the Florida Keys (Boca Raton: CRC Press, pp. 609–640).

Boyer, J.N., C.R. Kelble, P.B.Ortner, and D.T Rudnick.  2009.  Phytoplankton bloom status: Chlorophyll a biomass as an indicator of water quality condition in the southern estuaries of Florida, USA.  Ecological Indicators 9S: S56–S67.

Callahan, M., J. Wheaton, C. Beaver, S. Brooke, D. Johnson, J. Kidney, S. Kupfner, J.W. Porter, M. Meyers, S. Wade, M. Colella, and M. Bertin.  2007.  2006 Executive summary:  EPA Steering Committee Meeting.  Coral Reef Evaluation and Monitoring Project.  Florida Fish and Wildlife Conservation Commission and the University of Georgia.

Campbell, J.E., and J.W. Fourqurean.  2009.  Interspecific variation in the elemental and stable isotope content of seagrasses in South Florida.  Marine Ecology Progress Series 387: 109–123.

Chiappone, M.  1996.  Coral Watch Program summary:  A report on volunteer and scientific efforts to document the status of reefs in the Florida Keys National Marine Sanctuary.  Summerland Key, FL:  The Nature Conservancy.

Chiappone, M., and K.M. Sullivan.  1994.  Ecological structure and dynamics of nearshore hard-bottom communities in the Florida Keys.  Bulletin of Marine Science 54(3): 747–756.

Collado-Vides, L., V.G. Caccia, J.N. Boyer, and J.W. Fourqurean.  2007.  Tropical seagrass-associated macroalgae distributions and trends relative to water quality.  Estuarine, Coastal and Shelf Science 73: 680–694.

Collado-Vides, L., L.M. Rutten, and J.W. Fourqurean.  2005.  Spatiotemporal variation of the abundance of calcareous green macroalgae in the Florida Keys:  A study of synchrony within a macroalgal functional-form group.  Journal of Phycology 41: 742–752.

Donahue, S., A. Acosta, L.  Akins, J. Ault, J. Bohnsack, and J. Boyer et al.  2008.  The state of coral reef ecosystems of the Florida Keys.  In:  J.E. Waddell and A.M. Clarke (Eds.), The state of coral reef ecosystems of the United States and Pacific freely associated states  (Silver Spring, MD:  NOAA Technical Memorandum NOS NCCOS 73, NOAA/NCCOS Center for Coastal Monitoring and Assessment’s Biogeography Team, pp. 161–187).

Ferdie, M., and J.W. Fourqurean.  2004.  Responses of seagrass communities to fertilization along a gradient of relative availability of nitrogen and phosphorus in a carbonate environment.  Limnology and Oceanography 49(6): 2082–2094.

FDEP (Florida Department of Environmental Protection).  2008a.  Florida Keys reasonable assurance document.  Available:  http://www.dep.state.fl.us/Water/watersheds/bmap.htm.

FDEP, Division of Environmental Assessment and Restoration. 2008b. Florida Keys Water Quality Assessment Report.  http://tlhdwf2.dep.state.fl.us/basin411/keys/assessment/G5AS-Florida_Keys-LORES_Merged.pdf

Fourqurean, J.W., L.M. Rutten, and T. Philippi.  2003.  Florida Keys carrying capacity study:  Assessment of nearshore benthic communities of the Florida Keys.  Miami, FL:  Seagrass Ecosystem Research Lab, Southeast Environmental Center, Florida International University.

Fourqurean, J.W., and S.P. Escorcia.  2009.  Seagrass monitoring in the Florida Keys National Marine Sanctuary:  FY2008 annual report, executive summary.  Southeast Environmental Research Center Technical Report.  Miami, FL:  Florida International University.

Fourqurean, J.W., and J.C. Zieman.  2002.  Nutrient content of the seagrass Thalassia testudinum reveals regional patterns of relative availability of nitrogen and phosphorus in the Florida Keys USA.  Biogeochemistry 61: 229–245.

Fourqurean, J.W., M.J. Durako, M.O. Hall, and L.N. Hefty.  2002.  Seagrass distribution in south Florida: A multi-agency coordinated monitoring program.  In:  J.W. Porter and K.G. Porter (Eds.), The Everglades, Florida Bay and coral reefs of the Florida Keys (Boca Raton, FL: CRC Press, pp. 609–640).

Fourqurean, J.W., M.F. Muth, and J.N. Boyer.  In press.  Epiphyte loads on seagrasses and microphytobenthos abundance are not reliable indicators of nutrient availability in oligotrophic coastal ecosystems.  Marine Pollution Bulletin.

Fourqurean, J.W., A. Willsie, C.D. Rose, and L.M. Rutten.  2001.  Spatial and temporal pattern in seagrass community composition and productivity in south Florida.  Marine Biology 138: 341–354.

Furman, B.T., and K.L. Heck, Jr.  2008.  Effects of nutrient enrichment and grazers on coral reefs:  An experimental assessment.  Marine Ecology Progress Series 363: 89–101.

Gibson, P.J., J.N. Boyer, and N.P. Smith.  2008.  Nutrient mass flux between Florida Bay and the Florida Keys National Marine Sanctuary.  Estuaries and Coasts 31: 21–32.

Kruczynski, W.L., and F. McManus.  2002.  Water quality concerns in the Florida Keys:  Sources, effects, and solutions.  In: J.W. Porter and K.G. Porter (Eds.), The Everglades, Florida Bay and coral reefs of the Florida Keys (Boca Raton, FL:  CRC Press, pp. 827–882).

Lapointe, B.E., and M.W. Clark.  1992.  Nutrient inputs from the watershed and coastal eutrophication in the Florida Keys.  Estuaries 15(4): 465–476.

Lapointe, B.E., and W.R. Matzie.  1996.  Effects of stormwater nutrient discharges on eutrophication processes in nearshore waters of the Florida Keys.  Estuaries 19(2B): 422–435.

Lapointe, B.E., P.J. Barile, and W.R. Matzie.  2004.  Anthropogenic nutrient enrichment of seagrass and reef communities in the lower Florida Keys:  Discrimination of local versus regional nitrogen sources.  Journal of Experimental Marine Biology and Ecology 308: 23–58.

Lapointe, B.E., J.D. O’Connell, and G.S. Garrett.  1990.  Nutrient couplings between on-site sewage disposal systems, groundwaters, and nearshore surface waters of the Florida Keys.  Biogeochemistry 10: 289–307.

Lapointe, B.E., D.A. Tomasko, and W.R. Matzie. 1994.  Eutrophication and trophic state classification of seagrass communities in the Florida Keys.  Bulletin of Marine Science 54(3): 696–717.

Lee, T.N., E. Williams, E. Johns, D. Wilson, and N.P. Smith.  2002.  Transport processes linking South Florida coastal ecosystems.  In:  J.W. Porter and K.G. Porter (Eds.), The Everglades, Florida Bay and coral reefs of the Florida Keys (Boca Raton, FL:  CRC Press, pp. 309–342).

McClelland, S.  2010.  Florida Keys reasonable assurance demonstration:  Florida Keys nutrient targets.  Presented at the FDEP Estuary and Coastal Numeric Nutrient Criteria Development Meeting,  Miami, FL, March 3, 2010.  Available: http://www.dep.state.fl.us/water/wqssp/nutrients/docs/
estuarine/miami/florida_keys_reasonable_assurance_nutrients.pdf.

Miller, M.W., E. Weil, and A.M. Szmant.  2000.  Coral recruitment and juvenile mortality as structuring factors for reef benthic communities in Biscayne National Park, USA.  Coral Reefs 19: 115–123.

Monroe County.  2010.  2010 Comprehensive Plan.  Monroe County, FL.

NOAA.  1996.  Florida Keys National Marine Sanctuary final management plan/environmental impact statement.  Volume I.

Odum, W.E., and McIvor, C.C.  1990.  Chapter 15:  Mangroves.  In:  R.L. Meyers, and J.J. Ewel (Eds.), Ecosystems of Florida (Orlando, FL:  University of Central Florida Press, pp. 517–548).

Porter, J.W., V. Kosmynin, K.L. Patterson, K.G. Porter, W.C. Jaap, and J.L. Wheaton et al.  2002.  Detection of coral reef change by the Florida Keys Coral Reef Monitoring Project.  In:  J.W. Porter and K.G. Porter (Eds.), The Everglades, Florida Bay and coral reefs of the Florida Keys (Boca Raton, FL:  CRC Press, pp. 749–769).

Reich, C.D., E.A. Shinn, T.D. Hickey, and A.B. Tihansky.  2002.  Tidal and meteorological influences on shallow marine groundwater flow in the Upper Florida Keys.  In:  J.W. Porter and K.G. Porter (Eds.), The Everglades, Florida Bay and coral reefs of the Florida Keys (Boca Raton, FL:  CRC Press, pp. 659–676).

Smith, T.B., J. Purcell, and J.F. Barimo.  2007.  The rocky intertidal biota of the Florida Keys:  Fifty-two years of change after Stephenson and Stephenson (1950).  Bulletin of Marine Science 80(1): 1–19.

Swart, P.K., A. Saied, and K. Lamb.  2005.  Temporal and spatial variation in the δ15N and δ13C of coral tissue and zooxanthellae in Montastraea faveolata collected from the Florida reef tract.  Limnology and Oceanography 50(4): 1049–1058.

Szmant, A.M.  2002.  Nutrient enrichment on coral reefs: Is it a major cause of coral reef decline? Estuaries 25(4): 743–766.

Tomasko, D.A., and B.E. Lapointe.  1991.  Productivity and biomass of Thalassia testudinum as related to water column nutrient availability and epiphyte levels:  Field observations and experimental studies.  Marine Ecology Progress Series 75: 9–17.

Appendix A.  Permitted facilities and landfills in the Florida Keys basin by planning unit (FDEP 2008b).