Alexander Spring

Lake County, Florida

-----------------------------------------------------------------------------------------------

Water Quality

Alexander Spring is a first-magnitude spring, discharging ground water from the upper Floridan aquifer system. It has been sampled for major ions and nutrients as far back as 1956 by the U.S. Geological Survey (USGS) and quarterly by the Florida Department of Environmental Protection (FDEP) since 2002 for a larger number of analytes, including major ions, nutrients, field analytes and salinity indicators. Table 1 summarizes these results for selected analytes.

Unlike many Florida springs, nutrient levels in Alexander Spring have remained very low, with values for both total nitrate + nitrite (measured as nitrogen) and dissolved orthophosphate (measured as phosphorus) generally at or below 0.05 mg/L. Based on the data presented here (Figure 1), as well as historic and recent data collected by the USGS and St. Johns River Water Management District, no upward trend in macronutrient concentrations has been detected throughout the entire period of record. A large portion of the springshed for Alexander Spring lies within the Ocala National Forest, and land use patterns there (primarily silviculture) likely do not contribute significant nutrients from surface sources.

Salinity indicators (sodium, chloride, sulfate and specific conductance) have been slowly increasing over time. Chloride and sulfate concentrations have been increasing since about 1990, and the trends have continued to the present (Figure 2). Specific conductance trends have historically shown a slow increase since the mid-1950’s (based on available USGS data), with mean values of 941 µmhos/cm in the 1960’s (4 values), 1050 µmhos/cm in the 1970’s (2 values), 1090 µmhos/cm in the 1980’s (11 values), and 1132 µmhos/cm in the 1990’s (5 values). These results should be considered tentative, due to the small number of measurements made in the years prior to the late 1990’s. Table 1 shows that the mean value from 2002-2012, based on FDEP data, is 1101 µmhos/cm, a decrease from mean values measured in the 1990’s.

The measured increases in salinity indicators reflect potential upconing of deeper, more saline ground water due to increasing fresh ground water withdrawals from shallower portions of the Floridan aquifer system, decreasing precipitation patterns, or a combination of these two causes. Area long-term precipitation data for the period from 1950 – 2012 shows that there was a slightly increasing trend at DeLand of about 0.125 inches/year (1950-2008), a slightly decreasing trend at Gainesville of about 0.16 inches/year (1950-2012), and virtually no trend at Orlando (1950-2012) (Florida Climate Center, 2012).

Dissolved oxygen levels are important for fish and other biota, and are generally measured at levels below 5 mg/L in fresh ground water. The levels measured in Alexander Spring (Table 1) are within this normal ground water range.

Boron, not known to occur naturally in high concentrations in fresh Floridan aquifer system ground water, has recently been sampled as a possible wastewater tracer in wells and springs, due to its widespread use in laundry detergents. Elevated boron concentrations were detected in Alexander Spring (Table 1); however, when the boron/chloride ratio is calculated for the spring, it is very close to the ratio found from measurements made in North Atlantic Ocean seawater (approximately 0.00025 – Rakestraw et al, 1935). Therefore, it is likely that the measured boron concentrations in Alexander Spring are due to elevated salinity indicators, resulting from relict seawater trapped within the Floridan aquifer system during the last interglacial high sea stand (mid-Pleistocene, about 120,000 years ago - Muhs et al, 2009). Most low-elevation springs discharging into the St. Johns River Basin have been shown to have elevated salinity indicators when compared to other non-coastal Florida springs.

Sucralose is used as an artificial sweetener. Because it passes through water treatment systems largely intact, it has recently used as a potential wastewater tracer. No sucralose has been detected in Alexander Spring.

Ecological Health

The FDEP Biology Section has conducted a number of bioassessments within Alexander Spring Creek from 2002 through 2006. In 2002, this spring run was determined to be healthy but by late 2003, assessments showed the run to be impaired

The periphyton community was dominated by diatoms,

with some blue-green and green algal taxa. In conclusion,

macroinvertebrate community health and smothering of

macroinvertebrate habitats were the parameters of

concern in Alexander Springs.

In conclusion,

macroinvertebrate community health and smothering of

macroinvertebrate habitats continued to be the

parameters of concern in Alexander Springs

The distribution of submerged aquatic vegetation in Alexander Spring and in Alexander Springs Creek below the spring vent shows a dichotomy of species, with patches of healthy native Vallisnaria growing around the main spring vent (Figure 3). Away from the vent, the bottom of Alexander Spring Creek is covered by various benthic algal species, with large areas of exposed mud and limestone (Figure 4). Water velocities are relatively high in the vicinity of the spring vent, but drop considerably in Alexander Spring Creek, which is wide (about 250 feet across) and shallow.

References:

Florida Climate Center, Florida State University; Products and Services – Data: Long-Term Precipitation Data through 2012. Online at: http://climatecenter.fsu.edu/products-services/data

Muhs, Daniel R., J.F. Wehmiller, K.R. Simmons and L.L. York, 2009, Quaternary sea-level history of the United States: Developments In Quaternary Science, Volume 1; Elsevier; pp. 147-183. Online at: http://www.clim-past-discuss.net/5/C628/2009/cpd-5-C628-2009-supplement.pdf

Rakestraw, Noris W. and Henry E. Mahncke, 1935, Boron content of sea water of the North Atlantic Coast: Industrial Analytical Chemistry – Analytical Edition, Volume 7, Number 6, p. 425. Online at: http://pubs.acs.org/doi/abs/10.1021/ac50098a026

Scott, T.M., et al,  2004, Springs of Florida; Florida Geological Survey Bulletin No. 66; 377 p.

SJRWMD, Water bodies, watersheds and storm water: Alexander Springs; SJRWMD website:  http://floridaswater.com/springs/lake/alexander.html

U.S. Environmental Protection Agency, STORET/WQX: EPA’s repository and framework for sharing water monitoring data; Online at: http://www.epa.gov/storet/

For more information, contact:

Gary Maddox, FDEP Ground Water Management Section, 2600 Blair Stone Rd; Tallahassee, FL 32399; (850)245-8511; Gary.Maddox@dep.state.fl.us

Figure 1: Macronutrient trends in Alexander Spring: 2002 - 2012

Figure 2: Salinity indicator trends in Alexander Spring: 2002 - 2012

Figure 3: Healthy Vallisnaria surrounding Alexander Spring vent in June, 2009

(Gary Maddox – FDEP)

Figure 4: Algae covering bottom of Alexander Run downstream from spring vent in June, 2009

(Gary Maddox – FDEP)

Table 1: Summary of selected water quality results for Alexander Spring