Blue Spring - Volusia County

“Blue” is the most common name given to springs in Florida, due to their naturally clear blue water. Blue Spring in Volusia County (also known as Volusia Blue) is the largest of the state’s springs named “blue.”

Spring Location and Characteristics

Volusia Blue Spring lies six miles southwest of DeLand, in Volusia County.  It is located in Blue Spring State Park, which covers 2,600 acres.  This first-magnitude spring and the largest spring on the St. Johns River, Volusia Blue Spring discharges an historical average of 1,654 million gallons of water each day.  The spring is about 20 feet deep over its single vent.  The spring pool is nearly circular, with a visible boil in the center.  It is flanked by dramatically steep banks on three sides, with an overlook constructed near the vent. The water is clear and blue with a greenish tinge, and algae are ubiquitous in both the spring and spring run. Both the spring pool and spring run bottoms are mostly limestone and sand, with limited vegetation along the shoreline. The spring run has steep, sandy banks and flows almost half a mile to the St. Johns River through a thick native forest of cabbage palms and oaks.

An underwater cave system has been mapped at Volusia Blue Spring, but it is very difficult and dangerous for divers to enter the cave through the spring due to the strong outward flow of ground water.

For centuries, the spring area was home to Native Americans; it contains a prehistoric shell midden that was used as a house site by European settlers in the late 1800s.

In 1776, the explorer and botanist John Bartram wrote that the water in the spring run “smelled like bilge water, tasting sweetish and loathsome, warm and very clear . . . [T]he springhead is about 30 yards broad, and boils up from the bottom like a pot  . . .” Other early visitors also described the water as sulfurous. Scientists now understand that Volusia Blue Spring, along with other springs on the St. Johns River, is fed by relict seawater in the region’s ground water. The resulting water discharging from the spring vent is a combination of relatively young fresh water and older salt water; current water quality may thus be affected by water that entered the aquifer as much as 100 years earlier.

Biology

Volusia Blue Spring is the only natural winter refuge for the federally endangered West Indian manatee on Florida’s east coast and has been designated by the U.S. Fish and Wildlife Service as a critical habitat for the species (SJWMD, 2010).

The manatees congregate in large numbers in the spring and spring run during the winter months, when the spring water is considerably warmer than the water of the St. Johns River. These huge, gentle animals cannot survive for extended periods in water colder than about 63°F. From mid-November to mid-March, the spring and run are closed to swimming and boating. The boardwalk that parallels the spring run provides one of the best views in Florida of manatees in their natural habitat. Since the mid-1980s, the winter population of manatees increased almost fivefold, from less than 60 to more than 250 individuals in both the 2005–06 and the 2006–07 winter seasons. The manatee count reached as high as 371 individuals during the 2010 hard freeze, giving park visitors a spectacular viewing experience.

In addition, two species of aquatic snails (the Blue Spring silt snail) found in the spring and spring run are endemic, meaning that they exist nowhere else in the world.

Nutrient enrichment in the spring and spring run have led to eutrophic conditions, or accelerated aging. Filamentous algae such as the blue-green cyanobacteria Lyngyba have replaced a majority of the native submerged aquatic vegetation. These organisms can release toxic substances, deter grazing, and smother vegetation and macroinvertebrate habitat. Overall habitat assessment scores for the spring and spring run are suboptimal, and the Stream Condition Index score, a measure of biological health, ranges from “poor” to “very poor.”

Land Use

The springshed has seen substantial changes in land use in the last 50 years, from a predominantly rural and agricultural area to one with increasing high-intensity development. The human population has grown about 25% since 2000 and is currently about 155,000.

In 2000, almost 60% of the approximately 100-square-mile springshed that provides the ground water for Volusia Blue Spring was urban, with forested areas covering about 24%.  Concurrently with the shift in land use, the spring has experienced an overall decrease in spring flow, increased nitrate concentrations, diminished water clarity, and degraded aquatic habitat.

In 2010, the swimming area at Blue Spring State Park was closed for almost two weeks because of elevated levels of E. coli and total coliform bacteria.  The closure—the first in five years—was thought to be caused by rainfall flushing bacteria from the springshed into the swimming area.

Restoration Efforts

Given the importance of this site for both human and nonhuman visitors, especially the manatee, efforts are under way to protect and restore both the quality and quantity of water discharging from the spring. The St. Johns River Water Management District formed the Blue Spring Minimum Flow Interagency Working Group, which in 2006 developed a minimum flow regime for Volusia Blue Spring designed to protect and preserve the amount of water needed for manatees to shelter during cold spells.  Long-term average spring flows are being raised at five-year intervals to accommodate an increase in the manatee population of about 7% each year, until a goal of 157 cubic feet per second in spring discharge is reached in 2024.

As part of the implementation of minimum flows, the water management district is carrying out a number of different activities to inventory ecological resources in the spring and spring run, to better understand the dynamics of the manatee population, and to document the effects of human land uses and other factors influencing water quality and quantity.

Because the minimum flow regime limits the amount of ground water that can be used in the springshed, residents and businesses alike are looking at alternative sources of water and at increased water conservation.  As part of this effort, Volusia County’s water authority is planning to construct at least one surface water plant on the St. Johns River to meet new water supply needs.  It will also enforce consumptive use permit rules to ensure that minimum flows are maintained in the spring and spring run.

The Florida’s Wildlife Legacy Initiative of the Florida Fish and Wildlife Conservation Commission formed the Blue Spring Working Group in 2008.  Local governments and other stakeholders in the springshed come together quarterly to address the degradation of habitats for fish and wildlife and to work together on projects to conserve and restore the springs and spring run.

In 2009, the U.S. Environmental Protection Agency verified that Volusia Blue Spring was impaired for nutrients—meaning that increased nutrient concentrations were causing an imbalance in natural populations of aquatic plants and animals.  The EPA established a Total Maximum Daily Load for the spring in the form of a 91% reduction in nutrients.  A TMDL is the maximum amount of a given pollutant that a waterbody can assimilate and still meet water quality standards.  The restoration of ecological health in the spring and spring run depends heavily on the active participation of stakeholders in the springshed, who are required to develop projects to reduce nutrient concentrations.

To implement the TMDL, the Florida Department of Environmental Protection has developed a Basin Management Action Plan for Blue Spring that includes nutrient reduction allocations among stakeholders in the springshed, projects to achieve specific reductions, timelines for initiating and completing projects, identification of funding opportunities, agreements, local ordinances, local water quality standards and permits, and follow-up monitoring. LL note:  I could not locate an actual copy of the BMAP—do you know if it was developed by FDEP, or the EPA?

In 2004, the Florida Springs Initiative replaced the septic system at the park ranger residence with a connection to the sewer system in order to reduce nutrient contamination to the springs.  Other recent Springs Initiative activities include educational brochures, videos, and sponsored working group programs.

Sources:

2006 Volusia Blue Spring Minimum Flow Regime Action Plan (SJRWMD)

Volusia Blue Spring—A Hydrological Treasure (USGS)

http://www.floridastateparks.org/bluespring/

http://www.dep.state.fl.us/geology/geologictopics/springs/bulletin66.htm

http://www.epa.gov/waters/tmdldocs/22542_FINAL%20TMDL%20FL22542.pdf

http://share1.myfwc.com/VBSWG/Meeting%20Presentations/Volusia-Blue%20Spring%20Working%20Group/2009-09-16%20Chris%20Ferraro.pdf

http://www.floridaswater.com/minimumflowsandlevels/bluespring/FAQs.html

http://www.floridaspringsinstitute.org/Resources/Documents/Volusia_Blue_Hall.pdf