Sampling Log 9/19/2022 Lake Powell

Upon a public request about a petroleum smell in the western arm of Lake Powell we collected water and sediment samples at 3 locations on Lake Powell (see Figure 1).

Sample Location G3NW0243 - Cove east of Origins 8. Sample time 1510.

Water did not present a sheen, no unusual smell, tannic (brown color)

Secchi Disk – approximately half (1.2m) the total depth (2.7m)

Sediment – Fine soft muck.  Anaerobic smell (sulfur, rotten eggs)

Sample Location G3NW0242 – Western most cove near YOLO Park SE of Splash Drive. Sample time 1540.

Water did not present a sheen, no unusual smell, tannic (brown color)

Secchi Disk – approximately half (1.5m) the total depth (2.5m)

Sediment – Fine soft muck.  Strong Anaerobic smell (sulfur, rotten eggs)

Sample Location G3NW0244 – Southern area of Phillips Inlet. Sample time 1625.

Water did not present a sheen, no unusual smell, tannic (brown color)

Secchi Disk – Visible to the bottom, the total depth (0.7m)

Sediment - Coarse sand, no smell.