Integrated Water Quality Assessment for Florida:

2010 305(b) Report

and 303(d) List Update

APPENDICES

September 2010

Florida Department of Environmental Protection

Division of Environmental Assessment and Restoration

Tallahassee, Florida

Florida Department of Environmental Protection

Division of Water Resource Management

Bureau of Watershed Management

2600 Blair Stone Road

Tallahassee, Florida 32399-2400

www2.dep.state.fl.us/water/

Table of Contents

APPENDIX A:  Maps and Tables with 2004–08 Status Network Surfacae Water Assessment Results for Large Lakes, Small Lakes, Rivers, and Streams	1

APPENDIX B:  Discussion of Status Network Surface Water Indicators for Rivers, Streams, and Lakes, and Ground Water Indicators for Confined and Unconfined Aquifers	50

Surface Water Indicators for Rivers, Streams, and Lakes	50

Fecal Coliform Bacteria	50

Dissolved Oxygen	50

Un-ionized Ammonia	51

Chlorophyll a	51

Trophic State Index	51

Ground Water Indicators	52

Total Coliform Bacteria	52

Fecal Coliform Bacteria	53

Arsenic	53

Cadmium	53

Chromium	54

Fluoride	54

Lead	54

Nitrate-Nitrite	54

Sodium	55

APPENDIX C:  Maps and Tables with 2004–08 Probabilistic Ground Water Assessment Results for Confined Aquifers and Unconfined Aquifers	56

APPENDIX D.  Impaired Surface Waters Rule Methodology for Evaluating Impairment for the Basin Assessments	111

Aquatic Life Based Attainment	111

Exceedances of Numeric Water Quality Criteria	111

Exceedances of Nutrient Thresholds	112

Exceedances of Biological Thresholds	113

Lake Condition Index	114

Stream Condition Index	114

BioReconnaissance	115

Biological Integrity Standard	115

Evaluation of Toxicity Data	115

Primary Contact and Recreation Attainment	116

Fish and Shellfish Consumption Attainment	116

Drinking Water Attainment and Protection of Human Health	116

Appendix E:  Impaired Lakes in Florida,  Group 1–5 Basins	117

List of Tables

Table A.1.  Percentage of Rivers Meeting Threshold Value for DO Calculated Using Probabilistic Monitoring Design	3

Table A.2.  Percentage of Rivers Meeting Threshold Value for Fecal Coliform Calculated Using Probabilistic Monitoring Design	6

Table A.3.  Percentage of Rivers Meeting Threshold Value for Un-ionized Ammonia Calculated Using Probabilistic Monitoring Design	9

Table A.4.  Percentage of Rivers Meeting Threshold Value for Chlorophyll a Calculated Using Probabilistic Monitoring Design	12

Table A.5.  Percentage of Streams Meeting Threshold Value for DO Calculated Using Probabilistic Monitoring Design	15

Table A.6.  Percentage of Streams Meeting Threshold Value for Fecal Coliform Calculated Using Probabilistic Monitoring Design	18

Table A.7.  Percentage of Streams Meeting Threshold Value for Un-ionized Ammonia Calculated Using Probabilistic Monitoring Design	21

Table A.8.  Percentage of Streams Meeting Threshold Value for Chlorophyll a Calculated Using Probabilistic Monitoring Design	24

Table A.9.  Percentage of Large Lakes Meeting Threshold Value for DO Calculated Using Probabilistic Monitoring Design	27

Table A.10.  Percentage of Large Lakes Meeting Threshold Value for Fecal Coliform Calculated Using Probabilistic Monitoring Design	30

Table A.11.  Percentage of Large Lakes Meeting Threshold Value for Un-ionized Ammonia Calculated Using Probabilistic Monitoring Design	33

Table A.12.  Percentage of Large Lakes Meeting Threshold Value for TSI Calculated Using Probabilistic Monitoring Design	36

Table A.13.  Percentage of Small Lakes Meeting Threshold Value for DO Calculated Using Probabilistic Monitoring Design	39

Table A.14.  Percentage of Small Lakes Meeting Threshold Value for Fecal Coliform Calculated Using Probabilistic Monitoring Design	42

Table A.15.  Percentage of Small Lakes Meeting Threshold Value for Un-ionized Ammonia Calculated Using Probabilistic Monitoring Design	45

Table A.16.  Percentage of Small Lakes Meeting Threshold Value for TSI Calculated Using Probabilistic Monitoring Design	48

Table B.1.  Status Monitoring Network Water Quality Standards for Surface Water	50

Table B.2.  Status Monitoring Network Water Quality Standards for Ground Water	52

Table C.1.  Status Monitoring Network Water Quality Criteria/Thresholds for Ground Water	56

Table C.1.  Percentage of Confined Aquifers Meeting Threshold Value for Arsenic Calculated Using Probabilistic Monitoring Design	58

Table C.2.  Percentage of Confined Aquifers Meeting Threshold Value for Cadmium Calculated Using Probabilistic Monitoring Design	61

Table C.3.  Percentage of Confined Aquifers Meeting Threshold Value for Chromium Calculated Using Probabilistic Monitoring Design	64

Table C.4.  Percentage of Confined Aquifers Meeting Threshold Value for Fluoride Calculated Using Probabilistic Monitoring Design	67

Table C.5.  Percentage of Confined Aquifers Meeting Threshold Value for Lead Calculated Using Probabilistic Monitoring Design	70

Table C.6.  Percentage of Confined Aquifers Meeting Threshold Value for Nitrate + Nitrite Calculated Using Probabilistic Monitoring Design	73

Table C.7.  Percentage of Confined Aquifers Meeting Threshold Value for Sodium Calculated Using Probabilistic Monitoring Design	76

Table C.8.  Percentage of Confined Aquifers Meeting Threshold Value for Fecal Coliform Calculated Using Probabilistic Monitoring Design	79

Table C.9.  Percentage of Confined Aquifers Meeting Threshold Value for Total Coliform Calculated Using Probabilistic Monitoring Design	82

Table C.10.  Percentage of Unconfined Aquifers Meeting Threshold Value for Arsenic Calculated Using Probabilistic Monitoring Design	85

Table C.11.  Percentage of Unconfined Aquifers Meeting Threshold Value for Cadmium Calculated Using Probabilistic Monitoring Design	88

Table C.12.  Percentage of Unconfined Aquifers Meeting Threshold Value for Chromium Calculated Using Probabilistic Monitoring Design	91

Table C.13.  Percentage of Unconfined Aquifers Meeting Threshold Value for Fluoride Calculated Using Probabilistic Monitoring Design	94

Table C.14.  Percentage of Unconfined Aquifers Meeting Threshold Value for Lead Calculated Using Probabilistic Monitoring Design	97

Table C.15.  Percentage of Unconfined Aquifers Meeting Threshold Value for Nitrate + Nitrite Calculated Using Probabilistic Monitoring Design	100

Table C.16.  Percentage of Unconfined Aquifers Meeting Threshold Value for Sodium Calculated Using Probabilistic Monitoring Design	103

Table C.17.  Percentage of Unconfined Aquifers Meeting Threshold Value for Fecal Coliform Calculated Using Probabilistic Monitoring Design	106

Table C.18.  Percentage of Unconfined Aquifers Meeting Threshold Value for Total Coliform Calculated Using Probabilistic Monitoring Design	109

Table D.1.  SCI Metrics for the Northeast, Panhandle, and Peninsula Regions of Florida	114

Table D.2.  BioRecon Metrics for the Northeast, Panhandle, and Peninsula Regions of Florida	115

Table D.3.  BioRecon Sample Size and Index Range	115

List of Figures

Figure A.1.  Summary of DO Assessment for Rivers	2

Figure A.2.  Summary of Fecal Coliform Assessment for Rivers	5

Figure A.3.  Summary of Un-ionized Ammonia Assessment for Rivers	8

Figure A.4.  Summary of Chlorophyll a Assessment for Rivers	11

Figure A.5.  Summary of DO Assessment for Streams	14

Figure A.6.  Summary of Fecal Coliform Assessment for Streams	17

Figure A.7.  Summary of Un-ionized Ammonia Assessment for Streams	20

Figure A.8.  Summary of Chlorophyll a Assessment for Streams	23

Figure A.9.  Summary of DO Assessment for Large Lakes	26

Figure A.10.  Summary of Fecal Coliform Assessment for Large Lakes	29

Figure A.11.  Summary of Un-ionized Ammonia Assessment for Large Lakes	32

Figure A.12.  Summary of TSI Assessment for Large Lakes	35

Figure A.13.  Summary of DO Assessment for Small Lakes	38

Figure A.14.  Summary of Fecal Coliform Assessment for Small Lakes	41

Figure A.15.  Summary of Un-ionized Ammonia Assessment for Small Lakes	44

Figure A.16.  Summary of TSI Assessment for Small Lakes	47

Figure C.1.  Summary of Arsenic Assessment for Confined Aquifers	57

Figure C.2.  Summary of Cadmium Assessment for Confined Aquifers	60

Figure C.3.  Summary of Chromium Assessment for Confined Aquifers	63

Figure C.4.  Summary of Fluoride Assessment for Confined Aquifers	66

Figure C.5.  Summary of Lead Assessment for Confined Aquifers	69

Figure C.6.  Summary of Nitrate + Nitrite Assessment for Confined Aquifers	72

Figure C.7.  Summary of Sodium Assessment for Confined Aquifers	75

Figure C.8.  Summary of Fecal Coliform Assessment for Confined Aquifers	78

Figure C.9.  Summary of Total Coliform Assessment for Confined Aquifers	81

Figure C.10.  Summary of Arsenic Assessment for Unconfined Aquifers	84

Figure C.11.  Summary of Cadmium Assessment for Unconfined Aquifers	87

Figure C.12.  Summary of Chromium Assessment for Unconfined Aquifers	90

Figure C.13.  Summary of Fluoride Assessment for Unconfined Aquifers	93

Figure C.14.  Summary of Lead Assessment for Unconfined Aquifers	96

Figure C.15.  Summary of Nitrate + Nitrite Assessment for Unconfined Aquifers	99

Figure C.16.  Summary of Sodium Assessment for Unconfined Aquifers	102

Figure C.17.  Summary of Fecal Coliform Assessment for Unconfined Aquifers	105

Figure C.18.  Summary of Total Coliform Assessment for Unconfined Aquifers	108

# APPENDIX A:  Maps and Tables with 2004–08 Status Network Surface Water Assessment Results for Large Lakes, Small Lakes, Rivers, and Streams

The Status Network design focuses on the following four surface water resource types:

Rivers are major rivers of the state;

Streams are the remaining streams;

Large Lakes are 25 acres or greater; and

Small Lakes are 2.5 to less than 25 acres in size.

Status Network indicators include the following:

Rivers and Streams:

DO

Fecal coliform

Un-ionized ammonia (calculated)

Chlorophyll a

Large and Small Lakes:

DO

Fecal coliform

Un-ionized ammonia (calculated)

TSI

The threshold criteria for these indicators are as follows:

DO: 5.0 mg/L or above.

Fecal coliform: less than or equal to 400 counts/100mL.

Un-ionized ammonia: less than 0.02 mg/L.

Chlorophyll a: less than or equal to 20 g/L.

TSI:  For samples with color less than or equal to 40 PCUs, the threshold is less than or equal to 40.  For samples with color greater than 40 PCUs, the threshold is less than or equal to 60.

The light blue portion of each individual pie chart is the portion that meets the threshold for each indicator.

Note:  Appendix B provides additional information on whether the thresholds listed in the tables in this appendix are water quality standards or screening levels.

Figure A.1.  Summary of DO Assessment for Rivers

Table A.1.  Percentage of Rivers Meeting Threshold Value for DO Calculated Using Probabilistic Monitoring Design

Status Network		Designated Use:  Recreation and Aquatic Life 		Units:  Miles

This is a 10-column table.  Column 1 lists the basin, Column 2 lists the target population, Column 3 lists the sampleable miles, Column 4 lists the dry miles, Column 5 lists the inaccessible miles, Column 6 lists the number of samples, Column 7 lists the percent meeting threshold, Column 8 lists the 95% confidence bounds, Column 9 lists the percent not meeting threshold, and Column 10 lists the assessment period.

1 No large rivers in basin

2 N/A = Not applicable

| Basin | Target Population | Sampleable | Dry | Inaccessible | Number of Samples | % Meeting Threshold | 95% Confidence Bounds (% Meeting Threshold) | % Not Meeting Threshold | Assessment Period |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NWFWMD1 Ochlockonee–St. Marks | 143 | 116 | 0 | 27 | 30 | 76.7 | 65.4-87.9 | 23.3 | Aug–Oct 2004 |
| SFWMD1 Everglades–West Coast | 255 | 207 | 0 | 48 | 30 | 76.7 | 62.6-90.7 | 23.3 | May–June 2004 |
| SFWMD6 Lake Okeechobee | 23 | 14 | 0 | 9 | 30 | 60.0 | 47.8-72.2 | 40.0 | May–June 2004 |
| SJRWMD1 Ocklawaha | 62 | 54 | 0 | 8 | 30 | 60.00 | 47.8-72.2 | 40.0 | May–June 2004 |
| SRWMD1 Suwannee | 333 | 294 | 20 | 19 | 30 | 66.7 | 53.5-79.8 | 33.3 | Aug–Sept 2004 |
| SWFWMD1 Tampa Bay1 | 0 | 0 | 0 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| NWFWMD2 Apalachicola–Chipola | 184 | 178 | 0 | 6 | 30 | 100.0 | 100.0 | 0.0 | Aug-Sept 2005 |
| SFWMD2 St Lucie–Loxahatchee | 128 | 128 | 0 | 0 | 30 | 40.0 | 29.4-50.6 | 60.0 | May-July 2005 |
| SFWMD7 Charlotte Harbor1 | 0 | 0 | 0 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD2 Lower St. Johns | 130 | 95 | 6 | 29 | 30 | 93.3 | 85.6-100.0 | 6.7 | May-June 2005 |
| SJRWMD6 Middle St. Johns | 124 | 95 | 0 | 29 | 30 | 63.3 | 52.0-74.7 | 36.7 | May-July 2005 |
| SWFWMD2 Tampa Bay Tributaries | 127 | 69 | 0 | 58 | 30 | 80.0 | 68.3-91.7 | 20.0 | May-Dec 2005 |
| NWFWMD3 Choctawhatchee– St. Andrews | 169 | 127 | 17 | 25 | 30 | 100.0 | 100.0 | 0.0 | Aug-Oct 2006 |
| SFWMD3 Caloosahatchee | 44 | 44 | 0 | 0 | 30 | 70.0 | 65.0-75.0 | 30.0 | May-June 2006 |
| SFWMD8 Lake Worth Lagoon– Palm Beach Coast | 87 | 87 | 0 | 0 | 30 | 83.3 | 71.5-95.2 | 16.7 | May 2006 |
| SJRWMD3 Upper St. Johns | 62 | 62 | 0 | 0 | 30 | 63.3 | 50.2-76.5 | 36.7 | May-June 2006 |
| SWFWMD3 Sarasota Bay–Peace–Myakka | 136 | 95 | 0 | 41 | 30 | 63.3 | 52.1-74.6 | 36.7 | June-July 2006 |
| NWFWMD4 Pensacola | 243 | 235 | 0 | 8 | 30 | 100.0 | 100.0 | 0.0 | Aug-Sept 2007 |
| SFWMD4 Fisheating Creek | 105 | 58 | 0 | 47 | 27 | 59.3 | 48.4-70.1 | 40.7 | July 2007 |
| SFWMD9 Kissimmee River | 177 | 82 | 8 | 87 | 30 | 53.3 | 39.0-67.7 | 46.7 | May-July 2007 |
| SFWMD10 Southeast Coast– Biscayne Bay | 179 | 179 | 0 | 0 | 27 | 59.3 | 45.0-73.5 | 40.7 | May-July 2007 |
| SJRWMD4 Nassau–St. Marys | 116 | 53 | 0 | 63 | 30 | 80.0 | 69.5-90.5 | 20.0 | May-June 2007 |
| SWFWMD4 Withlacoochee | 88 | 42 | 23 | 23 | 30 | 96.7 | 91.1-100.0 | 3.3 | March-May 2007 |
| NWFWMD5 Perdido | 22 | 19 | 0 | 3 | 30 | 100.0 | 100.0 | 0.0 | Aug-Sept 2008 |
| SFWMD5 Everglades | 313 | 313 | 0 | 0 | 30 | 36.7 | 24.3-49.0 | 63.3 | May-July 2008 |
| SFWMD11 Florida Keys1 | 0 | 0 | 0 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD5 Indian River Lagoon1 | 0 | 0 | 0 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD7 Upper East Coast | 12 | 4 | 5 | 3 | 30 | 36.7 | 27.0-46.4 | 63.3 | May-June 2008 |
| SWFWMD5 Springs Coast | 20 | 14 | 5 | 1 | 30 | 53.3 | 39.2-67.4 | 46.7 | June-July 2008 |
| Summary of All Basins | 3,282 | 2,664 | 84 | 534 | 744 | 71.8 | 69.1-74.5 | 28.2 | 2004–08 |

Figure A.2.  Summary of Fecal Coliform Assessment for Rivers

Table A.2.  Percentage of Rivers Meeting Threshold Value for Fecal Coliform Calculated Using Probabilistic Monitoring Design

Status Network		Designated Use:  Recreation and Aquatic Life 		Units:  Miles

This is a 10-column table.  Column 1 lists the basin, Column 2 lists the target population, Column 3 lists the sampleable miles, Column 4 lists the dry miles, Column 5 lists the inaccessible miles, Column 6 lists the number of samples, Column 7 lists the percent meeting threshold, Column 8 lists the 95% confidence bounds, Column 9 lists the percent not meeting threshold, and Column 10 lists the assessment period.

1 No large rivers in basin

2 N/A = Not applicable

| Basin | Target Population | Sampleable | Dry | Inaccessible | Number of Samples | % Meeting Threshold | 95% Confidence Bounds (% Meeting Threshold) | % Not Meeting Threshold | Assessment Period |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NWFWMD1 Ochlockonee–St. Marks | 143 | 116 | 0 | 27 | 30 | 96.7 | 91.1-100.0 | 3.3 | Aug–Oct 2004 |
| SFWMD1 Everglades–West Coast | 255 | 207 | 0 | 48 | 30 | 100.0 | 100.0 | 0.0 | May–June 2004 |
| SFWMD6 Lake Okeechobee | 23 | 14 | 0 | 9 | 30 | 93.3 | 86.4-100.0 | 6.7 | May–June 2004 |
| SJRWMD1 Ocklawaha | 62 | 54 | 0 | 8 | 28 | 100.0 | 100.0 | 0.0 | May–June 2004 |
| SRWMD1 Suwannee | 333 | 294 | 20 | 19 | 29 | 93.1 | 85.2-100.0 | 6.9 | Aug–Sept 2004 |
| SWFWMD1 Tampa Bay1 | 0 | 0 | 0 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| NWFWMD2 Apalachicola–Chipola | 184 | 178 | 0 | 6 | 30 | 100.0 | 100.0 | 0.0 | Aug-Sept 2005 |
| SFWMD2 St Lucie–Loxahatchee | 128 | 128 | 0 | 0 | 28 | 92.9 | 84.4-100.0 | 7.1 | May-July 2005 |
| SFWMD7 Charlotte Harbor1 | 0 | 0 | 0 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD2 Lower St. Johns | 130 | 95 | 6 | 29 | 26 | 100.0 | 100.0 | 0.0 | May-June 2005 |
| SJRWMD6 Middle St. Johns | 124 | 95 | 0 | 29 | 14 | 50.0 | 27.0-73.0 | 50.0 | May-July 2005 |
| SWFWMD2 Tampa Bay Tributaries | 127 | 69 | 0 | 58 | 30 | 80.0 | 70.0-90.0 | 20.0 | May-Dec 2005 |
| NWFWMD3 Choctawhatchee– St. Andrews | 169 | 127 | 17 | 25 | 30 | 96.7 | 90.8-100.0 | 3.3 | Aug-Oct 2006 |
| SFWMD3 Caloosahatchee | 44 | 44 | 0 | 0 | 30 | 96.7 | 91.3-100.0 | 3.3 | May-June 2006 |
| SFWMD8 Lake Worth Lagoon– Palm Beach Coast | 87 | 87 | 0 | 0 | 30 | 96.7 | 91.3-100.0 | 3.3 | May 2006 |
| SJRWMD3 Upper St. Johns | 62 | 62 | 0 | 0 | 29 | 96.6 | 90.9-100.0 | 3.4 | May-June 2006 |
| SWFWMD3 Sarasota Bay–Peace–Myakka | 136 | 95 | 0 | 41 | 2 | 100.0 | 100.0 | 0.0 | June-July 2006 |
| NWFWMD4 Pensacola | 243 | 235 | 0 | 8 | 30 | 96.7 | 91.0-100.0 | 3.3 | Aug-Sept 2007 |
| SFWMD4 Fisheating Creek | 105 | 58 | 0 | 47 | 27 | 96.3 | 90.0-100.0 | 3.7 | July 2007 |
| SFWMD9 Kissimmee River | 177 | 82 | 8 | 87 | 27 | 88.9 | 79.7-98.1 | 11.1 | May-July 2007 |
| SFWMD10 Southeast Coast– Biscayne Bay | 179 | 179 | 0 | 0 | 28 | 71.4 | 59.2-83.7 | 28.6 | May-July 2007 |
| SJRWMD4 Nassau–St. Marys | 116 | 53 | 0 | 63 | 28 | 100.0 | 100.0 | 0.0 | May-June 2007 |
| SWFWMD4 Withlacoochee | 88 | 42 | 23 | 23 | 30 | 93.3 | 85.6-100.0 | 6.7 | May-July 2007 |
| NWFWMD5 Perdido | 22 | 19 | 0 | 3 | 30 | 93.3 | 87.5-99.1 | 6.7 | Aug-Sept 2008 |
| SFWMD5 Everglades | 313 | 313 | 0 | 0 | 30 | 93.3 | 86.4-100.0 | 6.7 | May-July 2008 |
| SFWMD11 Florida Keys1 | 0 | 0 | 0 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD5 Indian River Lagoon1 | 0 | 0 | 0 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD7 Upper East Coast | 12 | 4 | 5 | 3 | 30 | 100.0 | 100.0 | 0.0 | May-June 2008 |
| SWFWMD5 Springs Coast | 20 | 14 | 5 | 1 | 30 | 73.3 | 63.0-83.7 | 26.7 | June-July 2008 |
| Summary of All Basins | 3,282 | 2,664 | 84 | 534 | 686 | 89.2 | 87.5-90.8 | 10.8 | 2004–08 |

Figure A.3.  Summary of Un-ionized Ammonia Assessment for Rivers

Table A.3.  Percentage of Rivers Meeting Threshold Value for Un-ionized Ammonia Calculated Using Probabilistic Monitoring Design

Status Network		Designated Use:  Recreation and Aquatic Life 		Units:  Miles

This is a 10-column table.  Column 1 lists the basin, Column 2 lists the target population, Column 3 lists the sampleable miles, Column 4 lists the dry miles, Column 5 lists the inaccessible miles, Column 6 lists the number of samples, Column 7 lists the percent meeting threshold, Column 8 lists the 95% confidence bounds, Column 9 lists the percent not meeting threshold, and Column 10 lists the assessment period.

1 No large rivers in basin

2 N/A = Not applicable

| Basin | Target Population | Sampleable | Dry | Inaccessible | Number of Samples | % Meeting Threshold | 95% Confidence Bounds (% Meeting Threshold) | % Not Meeting Threshold | Assessment Period |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NWFWMD1 Ochlockonee–St. Marks | 143 | 116 | 0 | 27 | 30 | 100.0 | 100.0 | 0.0 | Aug–Oct 2004 |
| SFWMD1 Everglades–West Coast | 255 | 207 | 0 | 48 | 30 | 100.0 | 100.0 | 0.0 | May–June 2004 |
| SFWMD6 Lake Okeechobee | 23 | 14 | 0 | 9 | 30 | 96.7 | 91.0-100.0 | 3.3 | May–June 2004 |
| SJRWMD1 Ocklawaha | 62 | 54 | 0 | 8 | 30 | 100.0 | 100.0 | 0.0 | May–June 2004 |
| SRWMD1 Suwannee | 333 | 294 | 20 | 19 | 30 | 100.0 | 100.0 | 0.0 | Aug–Sept 2004 |
| SWFWMD1 Tampa Bay1 | 0 | 0 | 0 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| NWFWMD2 Apalachicola–Chipola | 184 | 178 | 0 | 6 | 30 | 100.0 | 100.0 | 0.0 | Aug-Sept 2005 |
| SFWMD2 St Lucie–Loxahatchee | 128 | 128 | 0 | 0 | 30 | 100.0 | 100.0 | 0.0 | May-July 2005 |
| SFWMD7 Charlotte Harbor1 | 0 | 0 | 0 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD2 Lower St. Johns | 130 | 95 | 6 | 29 | 30 | 100.0 | 100.0 | 0.0 | May-June 2005 |
| SJRWMD6 Middle St. Johns | 124 | 95 | 0 | 29 | 30 | 100.0 | 100.0 | 0.0 | May-July 2005 |
| SWFWMD2 Tampa Bay Tributaries | 127 | 69 | 0 | 58 | 30 | 100.0 | 100.0 | 0.0 | May-Dec 2005 |
| NWFWMD3 Choctawhatchee– St. Andrews | 169 | 127 | 17 | 25 | 30 | 100.0 | 100.0 | 0.0 | Aug-Oct 2006 |
| SFWMD3 Caloosahatchee | 44 | 44 | 0 | 0 | 30 | 100.0 | 100.0 | 0.0 | May-June 2006 |
| SFWMD8 Lake Worth Lagoon– Palm Beach Coast | 87 | 87 | 0 | 0 | 30 | 100.0 | 100.0 | 0.0 | May 2006 |
| SJRWMD3 Upper St. Johns | 62 | 62 | 0 | 0 | 30 | 100.0 | 100.0 | 0.0 | May-June 2006 |
| SWFWMD3 Sarasota Bay–Peace–Myakka | 136 | 95 | 0 | 41 | 30 | 90.0 | 82.7-97.3 | 10.0 | June-July 2006 |
| NWFWMD4 Pensacola | 243 | 235 | 0 | 8 | 30 | 100.0 | 100.0 | 0.0 | Aug-Sept 2007 |
| SFWMD4 Fisheating Creek | 105 | 58 | 0 | 47 | 27 | 100.0 | 100.0 | 0.0 | July 2007 |
| SFWMD9 Kissimmee River | 177 | 82 | 8 | 87 | 30 | 100.0 | 100.0 | 0.0 | May-July 2007 |
| SFWMD10 Southeast Coast– Biscayne Bay | 179 | 179 | 0 | 0 | 29 | 100.0 | 100.0 | 0.0 | May-July 2007 |
| SJRWMD4 Nassau–St. Marys | 116 | 53 | 0 | 63 | 30 | 100.0 | 100.0 | 0.0 | May-June 2007 |
| SWFWMD4 Withlacoochee | 88 | 42 | 23 | 23 | 30 | 96.7 | 91.1-100.0 | 3.3 | May-July 2007 |
| NWFWMD5 Perdido | 22 | 19 | 0 | 3 | 30 | 100.0 | 100.0 | 0.0 | Aug-Sept 2008 |
| SFWMD5 Everglades | 313 | 313 | 0 | 0 | 30 | 100.0 | 100.0 | 0.0 | May-July 2008 |
| SFWMD11 Florida Keys1 | 0 | 0 | 0 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD5 Indian River Lagoon1 | 0 | 0 | 0 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD7 Upper East Coast | 12 | 4 | 5 | 3 | 30 | 100.0 | 100.0 | 0.0 | May-June 2008 |
| SWFWMD5 Springs Coast | 20 | 14 | 5 | 1 | 30 | 100.0 | 100.0 | 0.0 | June-July 2008 |
| Summary of All Basins | 3,282 | 2,664 | 84 | 534 | 746 | 99.4 | 99.1–99.8 | 0.6 | 2004–08 |

Figure A.4.  Summary of Chlorophyll a Assessment for Rivers

Table A.4.  Percentage of Rivers Meeting Threshold Value for Chlorophyll a Calculated Using Probabilistic Monitoring Design

Status Network		Designated Use:  Recreation and Aquatic Life 		Units:  Miles

This is a 10-column table.  Column 1 lists the basin, Column 2 lists the target population, Column 3 lists the sampleable miles, Column 4 lists the dry miles, Column 5 lists the inaccessible miles, Column 6 lists the number of samples, Column 7 lists the percent meeting threshold, Column 8 lists the 95% confidence bounds, Column 9 lists the percent not meeting threshold, and Column 10 lists the assessment period.

1 No large rivers in basin

2 N/A = Not applicable

| Basin | Target Population | Sampleable | Dry | Inaccessible | Number of Samples | % Meeting Threshold | 95% Confidence Bounds (% Meeting Threshold) | % Not Meeting Threshold | Assessment Period |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NWFWMD1 Ochlockonee–St. Marks | 143 | 116 | 0 | 27 | 30 | 100.0 | 100.0 | 0.0 | Aug–Oct 2004 |
| SFWMD1 Everglades–West Coast | 255 | 207 | 0 | 48 | 30 | 56.7 | 42.6 - 70.8 | 43.3 | May–June 2004 |
| SFWMD6 Lake Okeechobee | 23 | 14 | 0 | 9 | 30 | 33.3 | 23.6 - 43.1 | 66.7 | May–June 2004 |
| SJRWMD1 Ocklawaha | 62 | 54 | 0 | 8 | 29 | 82.6 | 72.5 – 93.0 | 17.4 | May–June 2004 |
| SRWMD1 Suwannee | 333 | 294 | 20 | 19 | 30 | 100.0 | 100.0 | 0.0 | Aug–Sept 2004 |
| SWFWMD1 Tampa Bay1 | 0 | 0 | 0 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| NWFWMD2 Apalachicola–Chipola | 184 | 178 | 0 | 6 | 30 | 100.0 | 100.0 | 0.0 | Aug-Sept 2005 |
| SFWMD2 St Lucie–Loxahatchee | 128 | 128 | 0 | 0 | 30 | 76.7 | 63.7-89.7 | 23.3 | May-July 2005 |
| SFWMD7 Charlotte Harbor1 | 0 | 0 | 0 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD2 Lower St. Johns | 130 | 95 | 6 | 29 | 29 | 93.1 | 84.7-100.0 | 6.9 | May-June 2005 |
| SJRWMD6 Middle St. Johns | 124 | 95 | 0 | 29 | 30 | 83.3 | 73.3-93.4 | 16.7 | May-July 2005 |
| SWFWMD2 Tampa Bay Tributaries | 127 | 69 | 0 | 58 | 30 | 96.7 | 91.0-100.0 | 3.3 | May-Dec 2005 |
| NWFWMD3 Choctawhatchee– St. Andrews | 169 | 127 | 17 | 25 | 30 | 100.0 | 100.0 | 0.0 | Aug-Oct 2006 |
| SFWMD3 Caloosahatchee | 44 | 44 | 0 | 0 | 30 | 100.0 | 100.0 | 0.0 | May-June 2006 |
| SFWMD8 Lake Worth Lagoon– Palm Beach Coast | 87 | 87 | 0 | 0 | 30 | 66.7 | 54.3-79.0 | 33.3 | May 2006 |
| SJRWMD3 Upper St. Johns | 62 | 62 | 0 | 0 | 30 | 93.3 | 85.7-100.0 | 6.7 | May-June 2006 |
| SWFWMD3 Sarasota Bay–Peace–Myakka | 136 | 95 | 0 | 41 | 30 | 83.3 | 74.1-92.6 | 16.7 | June-July 2006 |
| NWFWMD4 Pensacola | 243 | 235 | 0 | 8 | 30 | 100.0 | 100.0 | 0.0 | Aug-Sept 2007 |
| SFWMD4 Fisheating Creek | 105 | 58 | 0 | 47 | 27 | 33.3 | 24.2-42.5 | 66.7 | July 2007 |
| SFWMD9 Kissimmee River | 177 | 82 | 8 | 87 | 27 | 66.7 | 52.1-81.3 | 33.3 | May-July 2007 |
| SFWMD10 Southeast Coast– Biscayne Bay | 179 | 179 | 0 | 0 | 29 | 96.0 | 89.6-100.0 | 4.0 | May-July 2007 |
| SJRWMD4 Nassau–St. Marys | 116 | 53 | 0 | 63 | 29 | 96.6 | 91.0-100.0 | 3.4 | May-June 2007 |
| SWFWMD4 Withlacoochee | 88 | 42 | 23 | 23 | 30 | 86.7 | 82.3-91.1 | 13.3 | March-May 2007 |
| NWFWMD5 Perdido | 22 | 19 | 0 | 3 | 30 | 100.0 | 100.0 | 0.0 | Aug-Sept 2008 |
| SFWMD5 Everglades | 313 | 313 | 0 | 0 | 30 | 80.0 | 70.9-89.1 | 20.0 | May-July 2008 |
| SFWMD11 Florida Keys1 | 0 | 0 | 0 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD5 Indian River Lagoon1 | 0 | 0 | 0 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD7 Upper East Coast | 12 | 4 | 5 | 3 | 30 | 63.3 | 49.2-77.4 | 36.7 | May-June 2008 |
| SWFWMD5 Springs Coast | 20 | 14 | 5 | 1 | 30 | 90.0 | 81.0-99.0 | 10.0 | June-July 2008 |
| Summary of All Basins | 3,282 | 2,664 | 84 | 534 | 740 | 85.6 | 83.6-87.5 | 14.4 | 2004–08 |

Figure A.5.  Summary of DO Assessment for Streams

Table A.5.  Percentage of Streams Meeting Threshold Value for DO Calculated Using Probabilistic Monitoring Design

Status Network		Designated Use:  Recreation and Aquatic Life 		Units:  Miles

This is a 10-column table.  Column 1 lists the basin, Column 2 lists the target population, Column 3 lists the sampleable miles, Column 4 lists the dry miles, Column 5 lists the inaccessible miles, Column 6 lists the number of samples, Column 7 lists the percent meeting threshold, Column 8 lists the 95% confidence bounds, Column 9 lists the percent not meeting threshold, and Column 10 lists the assessment period.

1 No small streams in basin

2 N/A = Not applicable

| Basin | Target Population | Sampleable | Dry | Inaccessible | Number of Samples | % Meeting Threshold | 95% Confidence Bounds (% Meeting Threshold) | % Not Meeting Threshold | Assessment Period |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NWFWMD1 Ochlockonee–St. Marks | 1,267 | 535 | 482 | 250 | 30 | 90.0 | 80.3-99.7 | 10.0 | April-June 2004 |
| SFWMD1 Everglades–West Coast | 902 | 173 | 77 | 652 | 8 | 37.5 | 11.0-89.0 | 62.5 | Oct 2004 |
| SFWMD6 Lake Okeechobee | 648 | 490 | 70 | 88 | 27 | 25.9 | 11.4-40.5 | 74.1 | Oct 2004 |
| SJRWMD1 Ocklawaha | 320 | 125 | 35 | 160 | 27 | 44.4 | 28.4-60.5 | 55.6 | Aug-Dec 2004 |
| SRWMD1 Suwannee | 1,794 | 233 | 686 | 875 | 19 | 63.2 | 46.2-80.1 | 36.8 | April-June 2004 |
| SWFWMD1 Tampa Bay | 212 | 116 | 12 | 84 | 30 | 43.3 | 27.3-59.4 | 56.7 | Aug-Sept 2004 |
| NWFWMD2 Apalachicola–Chipola | 1,353 | 643 | 133 | 577 | 29 | 75.9 | 64.0-87.7 | 24.1 | May-June 2005 |
| SFWMD2 St Lucie–Loxahatchee | 3,984 | 1,784 | 178 | 2,022 | 29 | 17.2 | 6.4-28.1 | 82.8 | Aug-Oct 2005 |
| SFWMD7 Charlotte Harbor | 108 | 87 | 6 | 15 | 28 | 60.7 | 45.9-75.5 | 39.3 | Aug-Oct 2005 |
| SJRWMD2 Lower St. Johns | 1,597 | 622 | 249 | 726 | 29 | 24.1 | 11.9-36.4 | 75.9 | Aug-Oct 2005 |
| SJRWMD6 Middle St. Johns | 615 | 303 | 40 | 272 | 29 | 10.3 | 0.3-20.4 | 89.7 | Aug-Oct 2005 |
| SWFWMD2 Tampa Bay Tributaries | 931 | 405 | 94 | 432 | 30 | 50.0 | 34.3-65.7 | 50.0 | Aug-Sept 2005 |
| NWFWMD3 Choctawhatchee– St. Andrews | 2,142 | 918 | 704 | 520 | 30 | 96.7 | 91.2-100.0 | 3.3 | April-June 2006 |
| SFWMD3 Caloosahatchee | 1,585 | 754 | 76 | 755 | 30 | 26.7 | 13.5-39.8 | 73.3 | Aug-Sept 2006 |
| SFWMD8 Lake Worth Lagoon– Palm Beach Coast | 695 | 579 | 0 | 116 | 30 | 30.0 | 15.9-44.1 | 70.0 | Aug 2006 |
| SJRWMD3 Upper St. Johns | 2,578 | 600 | 240 | 1,738 | 26 | 19.2 | 5.2-33.2 | 81.8 | Aug-Sept 2006 |
| SWFWMD3 Sarasota Bay–Peace–Myakka | 2,880 | 640 | 320 | 1,920 | 30 | 41.7 | 26.1-57.4 | 58.3 | Aug-Oct 2006 |
| NWFWMD4 Pensacola | 1,703 | 1,135 | 341 | 227 | 30 | 93.3 | 85.6-100.0 | 6.7 | April-June 2007 |
| SFWMD4 Fisheating Creek | 1,123 | 290 | 252 | 581 | 30 | 56.7 | 40.8-72.5 | 43.3 | Sept-Oct 2007 |
| SFWMD9 Kissimmee River | 2,323 | 1,096 | 526 | 701 | 25 | 64.0 | 48.0-80.0 | 36.0 | Oct-Nov 2007 |
| SFWMD10 Southeast Coast– Biscayne Bay | 662 | 567 | 0 | 95 | 30 | 56.7 | 43.7-69.7 | 43.3 | Sept-Oct 2007 |
| SJRWMD4 Nassau–St. Marys | 605 | 123 | 45 | 437 | 30 | 13.3 | 2.6-24.1 | 86.7 | Aug-Oct 2007 |
| SWFWMD4 Withlacoochee | 422 | 56 | 321 | 45 | 21 | 38.1 | 21.9-54.3 | 61.9 | Aug-Oct 2007 |
| NWFWMD5 Perdido | 223 | 93 | 62 | 68 | 30 | 80.0 | 67.0-93.0 | 20.0 | April-June 2008 |
| SFWMD5 Everglades | 3,258 | 1,086 | 36 | 2,136 | 30 | 33.3 | 18.1-48.5 | 66.7 | Aug-Oct 2008 |
| SFWMD11 Florida Keys1 | 0 | 0 | 0 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD5 Indian River Lagoon | 166 | 83 | 52 | 31 | 19 | 42.1 | 21.9-62.3 | 57.9 | Sept-Oct 2008 |
| SJRWMD7 Upper East Coast | 144 | 58 | 16 | 70 | 29 | 17.2 | 5.5-29.0 | 82.8 | Aug-Sept 2008 |
| SWFWMD5 Springs Coast | 150 | 58 | 37 | 55 | 30 | 60.0 | 46.4-73.6 | 40.0 | Sept-Oct 2008 |
| Summary of All Basins | 34,390 | 13,652 | 5,090 | 15,648 | 765 | 45.6 | 42.0-49.2 | 54.4 | 2004–08 |

Figure A.6.  Summary of Fecal Coliform Assessment for Streams

Table A.6.  Percentage of Streams Meeting Threshold Value for Fecal Coliform Calculated Using Probabilistic Monitoring Design

Status Network		Designated Use:  Recreation and Aquatic Life 		Units:  Miles

This is a 10-column table.  Column 1 lists the basin, Column 2 lists the target population, Column 3 lists the sampleable miles, Column 4 lists the dry miles, Column 5 lists the inaccessible miles, Column 6 lists the number of samples, Column 7 lists the percent meeting threshold, Column 8 lists the 95% confidence bounds, Column 9 lists the percent not meeting threshold, and Column 10 lists the assessment period.

1 No small streams in basin

2 N/A = Not applicable

| Basin | Target Population | Sampleable | Dry | Inaccessible | Number of Samples | % Meeting Threshold | 95% Confidence Bounds (% Meeting Threshold) | % Not Meeting Threshold | Assessment Period |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NWFWMD1 Ochlockonee–St. Marks | 1,267 | 535 | 482 | 250 | 30 | 76.7 | 63.2-90.1 | 23.3 | April-June 2004 |
| SFWMD1 Everglades–West Coast | 902 | 173 | 77 | 652 | 8 | 87.5 | 66.3-100.0 | 12.5 | Oct 2004 |
| SFWMD6 Lake Okeechobee | 648 | 490 | 70 | 88 | 26 | 80.8 | 66.7-94.8 | 19.2 | Oct 2004 |
| SJRWMD1 Ocklawaha | 320 | 125 | 35 | 160 | 27 | 77.8 | 65.3-90.2 | 22.2 | Aug-Dec 2004 |
| SRWMD1 Suwannee | 1,794 | 233 | 686 | 875 | 21 | 85.7 | 71.9-99.5 | 14.3 | April-June 2004 |
| SWFWMD1 Tampa Bay | 212 | 116 | 12 | 84 | 30 | 30.0 | 16.5-43.5 | 70.0 | Aug-Sept 2004 |
| NWFWMD2 Apalachicola–Chipola | 1,353 | 643 | 133 | 577 | 29 | 82.8 | 71.2-94.3 | 17.2 | May-June 2005 |
| SFWMD2 St Lucie–Loxahatchee | 3,984 | 1,784 | 178 | 2,022 | 29 | 75.9 | 62.8-88.9 | 24.1 | Aug-Oct 2005 |
| SFWMD7 Charlotte Harbor | 108 | 87 | 6 | 15 | 28 | 92.9 | 84.3-100.0 | 7.1 | Aug-Oct 2005 |
| SJRWMD2 Lower St. Johns | 1,597 | 622 | 249 | 726 | 29 | 55.2 | 38.5-71.9 | 44.8 | Aug-Oct 2005 |
| SJRWMD6 Middle St. Johns | 615 | 303 | 40 | 272 | 29 | 79.3 | 66.6-92.1 | 20.7 | Aug-Oct 2005 |
| SWFWMD2 Tampa Bay Tributaries | 931 | 405 | 94 | 432 | 30 | 40.0 | 24.5-55.5 | 60.0 | Aug-Sept 2005 |
| NWFWMD3 Choctawhatchee– St. Andrews | 2,142 | 918 | 704 | 520 | 30 | 90.0 | 81.5-98.5 | 10.0 | April-June 2006 |
| SFWMD3 Caloosahatchee | 1,585 | 754 | 76 | 755 | 30 | 70.0 | 56.5-83.5 | 30.0 | Aug-Sept 2006 |
| SFWMD8 Lake Worth Lagoon– Palm Beach Coast | 695 | 579 | 0 | 116 | 30 | 60.0 | 43.5-76.5 | 40.0 | Aug 2006 |
| SJRWMD3 Upper St. Johns | 2,578 | 600 | 240 | 1,738 | 29 | 48.3 | 33.5-63.0 | 51.7 | Aug-Sept 2006 |
| SWFWMD3 Sarasota Bay–Peace–Myakka | 2,880 | 640 | 320 | 1,920 | 30 | 70.9 | 56.7-85.0 | 29.1 | Aug-Oct 2006 |
| NWFWMD4 Pensacola | 1,703 | 1,135 | 341 | 227 | 24 | 100.0 | 100.0 | 0.0 | April-June 2007 |
| SFWMD4 Fisheating Creek | 1,123 | 290 | 252 | 581 | 30 | 50.0 | 36.7-63.2 | 50.0 | Sept-Oct 2007 |
| SFWMD9 Kissimmee River | 2,323 | 1,096 | 526 | 701 | 25 | 70.8 | 55.6-86.0 | 29.2 | Oct-Nov 2007 |
| SFWMD10 Southeast Coast– Biscayne Bay | 662 | 567 | 0 | 95 | 30 | 63.0 | 47.0-79.0 | 37.0 | Sept-Oct 2007 |
| SJRWMD4 Nassau–St. Marys | 605 | 123 | 45 | 437 | 30 | 83.3 | 73.0-93.6 | 16.7 | Aug-Oct 2007 |
| SWFWMD4 Withlacoochee | 422 | 56 | 321 | 45 | 21 | 88.9 | 77.2-100.0 | 11.1 | Aug-Oct 2007 |
| NWFWMD5 Perdido | 223 | 93 | 62 | 68 | 30 | 83.3 | 71.5-95.2 | 16.7 | April-June 2008 |
| SFWMD5 Everglades | 3,258 | 1,086 | 36 | 2,136 | 30 | 60.0 | 45.4-74.6 | 40.0 | Aug-Oct 2008 |
| SFWMD11 Florida Keys1 | 0 | 0 | 0 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD5 Indian River Lagoon | 166 | 83 | 52 | 31 | 19 | 47.4 | 29.9-64.8 | 52.6 | Sept-Oct 2008 |
| SJRWMD7 Upper East Coast | 144 | 58 | 16 | 70 | 29 | 51.7 | 36.0-67.4 | 48.3 | Aug-Sept 2008 |
| SWFWMD5 Springs Coast | 150 | 58 | 37 | 55 | 30 | 53.3 | 43.5-63.1 | 46.7 | Sept-Oct 2008 |
| Summary of All Basins | 34,390 | 13,652 | 5,090 | 15,648 | 763 | 71.6 | 68.2-75.0 | 28.4 | 2004–08 |

Figure A.7.  Summary of Un-ionized Ammonia Assessment for Streams

Table A.7.  Percentage of Streams Meeting Threshold Value for Un-ionized Ammonia Calculated Using Probabilistic Monitoring Design

Status Network		Designated Use:  Recreation and Aquatic Life 		Units:  Miles

This is a 10-column table.  Column 1 lists the basin, Column 2 lists the target population, Column 3 lists the sampleable miles, Column 4 lists the dry miles, Column 5 lists the inaccessible miles, Column 6 lists the number of samples, Column 7 lists the percent meeting threshold, Column 8 lists the 95% confidence bounds, Column 9 lists the percent not meeting threshold, and Column 10 lists the assessment period.

1 No small streams in basin

2 N/A = Not applicable

| Basin | Target Population | Sampleable | Dry | Inaccessible | Number of Samples | % Meeting Threshold | 95% Confidence Bounds (%Meeting Threshold) | % Not Meeting Threshold | Assessment Period |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NWFWMD1 Ochlockonee–St. Marks | 1,267 | 535 | 482 | 250 | 30 | 100.0 | 100.0 | 0.0 | April-June 2004 |
| SFWMD1 Everglades–West Coast | 902 | 173 | 77 | 652 | 8 | 100.0 | 100.0 | 0.0 | Oct 2004 |
| SFWMD6 Lake Okeechobee | 648 | 490 | 70 | 88 | 28 | 96.4 | 90.2-100.0 | 3.6 | Oct 2004 |
| SJRWMD1 Ocklawaha | 320 | 125 | 35 | 160 | 29 | 100.0 | 100.0 | 0.0 | Aug-Dec 2004 |
| SRWMD1 Suwannee | 1,794 | 233 | 686 | 875 | 21 | 95.2 | 86.8-100.0 | 4.8 | April-June 2004 |
| SWFWMD1 Tampa Bay | 212 | 116 | 12 | 84 | 30 | 100.0 | 100.0 | 0.0 | Aug-Sept 2004 |
| NWFWMD2 Apalachicola–Chipola | 1,353 | 643 | 133 | 577 | 29 | 100.0 | 100.0 | 0.0 | May-June 2005 |
| SFWMD2 St Lucie–Loxahatchee | 3,984 | 1,784 | 178 | 2,022 | 30 | 100.0 | 100.0 | 0.0 | Aug-Oct 2005 |
| SFWMD7 Charlotte Harbor | 108 | 87 | 6 | 15 | 30 | 100.0 | 100.0 | 0.0 | Aug-Oct 2005 |
| SJRWMD2 Lower St. Johns | 1,597 | 622 | 249 | 726 | 30 | 96.7 | 90.8-100.0 | 3.3 | Aug-Oct 2005 |
| SJRWMD6 Middle St. Johns | 615 | 303 | 40 | 272 | 30 | 100.0 | 100.0 | 0.0 | Aug-Oct 2005 |
| SWFWMD2 Tampa Bay Tributaries | 931 | 405 | 94 | 432 | 28 | 100.0 | 100.0 | 0.0 | Aug-Sept 2005 |
| NWFWMD3 Choctawhatchee– St. Andrews | 2,142 | 918 | 704 | 520 | 30 | 100.0 | 100.0 | 0.0 | April-June 2006 |
| SFWMD3 Caloosahatchee | 1,585 | 754 | 76 | 755 | 30 | 100.0 | 100.0 | 0.0 | Aug-Sept 2006 |
| SFWMD8 Lake Worth Lagoon– Palm Beach Coast | 695 | 579 | 0 | 116 | 30 | 100.0 | 100.0 | 0.0 | Aug 2006 |
| SJRWMD3 Upper St. Johns | 2,578 | 600 | 240 | 1,738 | 30 | 100.0 | 100.0 | 0.0 | Aug-Sept 2006 |
| SWFWMD3 Sarasota Bay–Peace–Myakka | 2,880 | 640 | 320 | 1,920 | 30 | 96.5 | 90.5-100.0 | 3.5 | Aug-Oct 2006 |
| NWFWMD4 Pensacola | 1,703 | 1,135 | 341 | 227 | 2 | 100.0 | 100.0 | 0.0 | April-June 2007 |
| SFWMD4 Fisheating Creek | 1,123 | 290 | 252 | 581 | 30 | 100.0 | 100.0 | 0.0 | Sept-Oct 2007 |
| SFWMD9 Kissimmee River | 2,323 | 1,096 | 526 | 701 | 25 | 88.0 | 78.3-97.7 | 12.0 | Oct-Nov 2007 |
| SFWMD10 Southeast Coast– Biscayne Bay | 662 | 567 | 0 | 95 | 30 | 96.7 | 91.2-100.0 | 3.3 | Sept-Oct 2007 |
| SJRWMD4 Nassau–St. Marys | 605 | 123 | 45 | 437 | 30 | 100.0 | 100.0 | 0.0 | Aug-Oct 2007 |
| SWFWMD4 Withlacoochee | 422 | 56 | 321 | 45 | 21 | 100.0 | 100.0 | 0.0 | Aug-Oct 2007 |
| NWFWMD5 Perdido | 223 | 93 | 62 | 68 | 30 | 100.0 | 100.0 | 0.0 | April-June 2008 |
| SFWMD5 Everglades | 3,258 | 1,086 | 36 | 2,136 | 30 | 96.7 | 90.8-100.0 | 3.3 | Aug-Oct 2008 |
| SFWMD11 Florida Keys1 | 0 | 0 | 0 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD5 Indian River Lagoon | 166 | 83 | 52 | 31 | 19 | 100.0 | 100.0 | 0.0 | Sept-Oct 2008 |
| SJRWMD7 Upper East Coast | 144 | 58 | 16 | 70 | 30 | 100.0 | 100.0 | 0.0 | Aug-Sept 2008 |
| SWFWMD5 Springs Coast | 150 | 58 | 37 | 55 | 30 | 100.0 | 100.0 | 0.0 | Sept-Oct 2008 |
| Summary of All Basins | 34,390 | 13,652 | 5,090 | 15,648 | 750 | 98.4 | 97.4-99.3 | 1.6 | 2004–08 |

Figure A.8.  Summary of Chlorophyll a Assessment for Streams

Table A.8.  Percentage of Streams Meeting Threshold Value for Chlorophyll a Calculated Using Probabilistic Monitoring Design

Status Network		Designated Use:  Recreation and Aquatic Life 		Units:  Miles

This is a 10-column table.  Column 1 lists the basin, Column 2 lists the target population, Column 3 lists the sampleable miles, Column 4 lists the dry miles, Column 5 lists the inaccessible miles, Column 6 lists the number of samples, Column 7 lists the percent meeting threshold, Column 8 lists the 95% confidence bounds, Column 9 lists the percent not meeting threshold, and Column 10 lists the assessment period.

1 No small streams in basin

2 N/A = Not applicable

| Basin | Target Population | Sampleable | Dry | Inaccessible | Number of Samples | % Meeting Threshold | 95% Confidence Bounds (% Meeting Threshold) | % Not Meeting Threshold | Assessment Period |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NWFWMD1 Ochlockonee–St. Marks | 1,267 | 535 | 482 | 250 | 30 | 100.0 | 100.0 | 0.0 | April-June 2004 |
| SFWMD1 Everglades–West Coast | 902 | 173 | 77 | 652 | 8 | 62.5 | 31.7-93.3 | 37.5 | Oct 2004 |
| SFWMD6 Lake Okeechobee | 648 | 490 | 70 | 88 | 27 | 63.0 | 46.3-79.6 | 37.0 | Oct 2004 |
| SJRWMD1 Ocklawaha | 320 | 125 | 35 | 160 | 27 | 85.2 | 76.4-94.0 | 14.8 | Aug-Dec 2004 |
| SRWMD1 Suwannee | 1,794 | 233 | 686 | 875 | 21 | 90.5 | 79.5-100.0 | 9.5 | April-June 2004 |
| SWFWMD1 Tampa Bay | 212 | 116 | 12 | 84 | 30 | 86.7 | 76.0-97.3 | 13.3 | Aug-Sept 2004 |
| NWFWMD2 Apalachicola–Chipola | 1,353 | 643 | 133 | 577 | 29 | 100.0 | 100.0 | 0.0 | May-June 2005 |
| SFWMD2 St Lucie–Loxahatchee | 3,984 | 1,784 | 178 | 2,022 | 29 | 69.0 | 54.5-83.4 | 31.0 | Aug-Oct 2005 |
| SFWMD7 Charlotte Harbor | 108 | 87 | 6 | 15 | 28 | 67.9 | 53.5-82.2 | 32.1 | Aug-Oct 2005 |
| SJRWMD2 Lower St. Johns | 1,597 | 622 | 249 | 726 | 29 | 82.8 | 71.7-93.8 | 17.2 | Aug-Oct 2005 |
| SJRWMD6 Middle St. Johns | 615 | 303 | 40 | 272 | 29 | 100.0 | 100.0 | 0.0 | Aug-Oct 2005 |
| SWFWMD2 Tampa Bay Tributaries | 931 | 405 | 94 | 432 | 30 | 86.7 | 76.0-97.3 | 13.3 | Aug-Sept 2005 |
| NWFWMD3 Choctawhatchee– St. Andrews | 2,142 | 918 | 704 | 520 | 30 | 100.0 | 100.0 | 0.0 | April-June 2006 |
| SFWMD3 Caloosahatchee | 1,585 | 754 | 76 | 755 | 30 | 76.7 | 64.3-89.1 | 23.3 | Aug-Sept 2006 |
| SFWMD8 Lake Worth Lagoon– Palm Beach Coast | 695 | 579 | 0 | 116 | 30 | 43.3 | 27.4-59.3 | 56.7 | Aug 2006 |
| SJRWMD3 Upper St. Johns | 2,578 | 600 | 240 | 1,738 | 29 | 62.1 | 47.0-77.1 | 37.9 | Aug-Sept 2006 |
| SWFWMD3 Sarasota Bay–Peace–Myakka | 2,880 | 640 | 320 | 1,920 | 30 | 79.1 | 65.9-92.3 | 20.9 | Aug-Oct 2006 |
| NWFWMD4 Pensacola | 1,703 | 1,135 | 341 | 227 | 24 | 100.0 | 100.0 | 0.0 | April-June 2007 |
| SFWMD4 Fisheating Creek | 1,123 | 290 | 252 | 581 | 30 | 50.0 | 35.1-64.9 | 50.0 | Sept-Oct 2007 |
| SFWMD9 Kissimmee River | 2,323 | 1,096 | 526 | 701 | 25 | 64.0 | 46.6-81.4 | 36.0 | Oct-Nov 2007 |
| SFWMD10 Southeast Coast– Biscayne Bay | 662 | 567 | 0 | 95 | 27 | 81.5 | 69.8-93.1 | 18.5 | Sept-Oct 2007 |
| SJRWMD4 Nassau–St. Marys | 605 | 123 | 45 | 437 | 30 | 90.0 | 81.4-98.6 | 10.0 | Aug-Oct 2007 |
| SWFWMD4 Withlacoochee | 422 | 56 | 321 | 45 | 21 | 71.4 | 53.9-89.0 | 28.6 | Aug-Oct 2007 |
| NWFWMD5 Perdido | 223 | 93 | 62 | 68 | 30 | 100.0 | 100.0 | 0.0 | April-June 2008 |
| SFWMD5 Everglades | 3,258 | 1,086 | 36 | 2,136 | 30 | 46.7 | 33.0-60.3 | 53.3 | Aug-Oct 2008 |
| SFWMD11 Florida Keys1 | 0 | 0 | 0 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD5 Indian River Lagoon | 166 | 83 | 52 | 31 | 19 | 78.9 | 63.9-93.9 | 21.1 | Sept-Oct 2008 |
| SJRWMD7 Upper East Coast | 144 | 58 | 16 | 70 | 29 | 90.0 | 80.0-99.4 | 10.0 | Aug-Sept 2008 |
| SWFWMD5 Springs Coast | 150 | 58 | 37 | 55 | 30 | 76.7 | 62.7-90.6 | 23.3 | Sept-Oct 2008 |
| Summary of All Basins | 34,390 | 13,652 | 5,090 | 15,648 | 761 | 75.2 | 71.5-78.9 | 24.8 | 2004–008 |

Figure A.9.  Summary of DO Assessment for Large Lakes

Table A.9.  Percentage of Large Lakes Meeting Threshold Value for DO Calculated Using Probabilistic Monitoring Design

Status Network		Designated Use:  Recreation and Aquatic Life 		Units:  Acres

This is a 10-column table.  Column 1 lists the basin, Column 2 lists the target population, Column 3 lists the sampleable acres, Column 4 lists the dry miles, Column 5 lists the inaccessible acres, Column 6 lists the number of samples, Column 7 lists the percent meeting threshold, Column 8 lists the 95% confidence bounds, Column 9 lists the percent not meeting threshold, and Column 10 lists the assessment period.

1 No large lakes in basin

2 N/A = Not applicable

| Basin | Target Population | Sampleable | Dry | Inaccessible | Number of Samples | % Meeting Threshold | 95% Confidence Bounds (% Meeting Threshold) | % Not Meeting Threshold | Assessment Period |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NWFWMD1 Ochlockonee–St. Marks | 30,107 | 17,042 | 1,704 | 11,361 | 30 | 53.3 | 42.2-64.4 | 46.7 | June-Aug 2004 |
| SFWMD1 Everglades–West Coast | 1,524 | 1,426 | 0 | 98 | 29 | 100.0 | 100.0 | 0.0 | Oct-Dec 2004 |
| SFWMD6 Lake Okeechobee | 340,652 | 234,933 | 0 | 105,719 | 23 | 100.0 | 100.0 | 0.0 | Nov-Dec 2004 |
| SJRWMD1 Ocklawaha | 142,923 | 119,103 | 0 | 23,820 | 30 | 90.0 | 81.2-98.8 | 10.0 | Oct-Dec 2004 |
| SRWMD1 Suwannee | 24,566 | 14,932 | 4,335 | 5,299 | 31 | 87.1 | 78.0-96.2 | 12.9 | July 2004 |
| SWFWMD1 Tampa Bay | 6,191 | 6,191 | 0 | 0 | 30 | 86.7 | 78.3-95.0 | 13.3 | Oct-Nov 2004 |
| NWFWMD2 Apalachicola–Chipola | 15,723 | 13,873 | 0 | 1,850 | 30 | 50.0 | 37.9-62.1 | 50.0 | July 2005 |
| SFWMD2 St Lucie–Loxahatchee | 626 | 470 | 0 | 156 | 30 | 86.7 | 76.4-97.0 | 13.3 | Nov-Dec 2005 |
| SFWMD7 Charlotte Harbor | 10 | 0 | 0 | 10 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD2 Lower St. Johns | 37,417 | 30,338 | 0 | 7,079 | 30 | 96.7 | 91.3-100.0 | 3.3 | Oct-Dec 2005 |
| SJRWMD6 Middle St. Johns | 116,082 | 94,121 | 6,275 | 15,686 | 30 | 96.7 | 91.1-100.0 | 3.3 | Nov-Dec 2005 |
| SWFWMD2 Tampa Bay Tributaries | 6,473 | 4,979 | 0 | 1,494 | 30 | 100.0 | 100.0 | 0.0 | Oct-Nov 2005 |
| NWFWMD3 Choctawhatchee– St. Andrews | 14,871 | 8,747 | 5,832 | 292 | 30 | 96.7 | 91.2-100.0 | 3.3 | Jun-Aug 2006 |
| SFWMD3 Caloosahatchee | 203 | 0 | 0 | 203 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SFWMD8 Lake Worth Lagoon– Palm Beach Coast | 1,490 | 1,490 | 0 | 0 | 30 | 96.7 | 91.4-100.0 | 3.3 | Oct-Nov 2006 |
| SJRWMD3 Upper St. Johns | 20,908 | 16,952 | 565 | 3,391 | 30 | 83.3 | 73.8-92.9 | 16.7 | Oct-Nov 2006 |
| SWFWMD3 Sarasota Bay–Peace–Myakka | 36,927 | 31,652 | 0 | 5,275 | 30 | 100.0 | 100.0 | 0.0 | Nov-Dec 2006 |
| NWFWMD4 Pensacola | 1,688 | 1,298 | 216 | 174 | 30 | 96.7 | 91.3-100.0 | 3.3 | June-July 2007 |
| SFWMD4 Fisheating Creek | 419 | 185 | 0 | 234 | 30 | 96.7 | 91.3-100.0 | 3.3 | Nov-Dec 2007 |
| SFWMD9 Kissimmee River | 192,758 | 192,758 | 0 | 0 | 27 | 100.0 | 100.0 | 0.0 | Nov-Dec 2007 |
| SFWMD10 Southeast Coast– Biscayne Bay1 | 0 | 0 | 0 | 0 | 0 | N/A2 | N/A2 | N/A2 | Nov-Dec 2007 |
| SJRWMD4 Nassau–St. Marys | 2,874 | 2,874 | 0 | 0 | 30 | 93.3 | 85.4-100.0 | 6.7 | Oct 2007 |
| SWFWMD4 Withlacoochee | 38,016 | 17,022 | 2,837 | 18,157 | 30 | 96.7 | 91.2-100.0 | 3.3 | Nov-Dec 2007 |
| NWFWMD5 Perdido | 69 | 69 | 0 | 0 | 8 | 100.0 | 100.0 | 0.0 | June 2008 |
| SFWMD5 Everglades1 | 0 | 0 | 0 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SFWMD11 Florida Keys1 | 0 | 0 | 0 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD5 Indian River Lagoon | 90 | 0 | 90 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD7 Upper East Coast | 288 | 0 | 288 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SWFWMD5 Springs Coast | 6,004 | 1,396 | 4,421 | 187 | 30 | 100.0 | 100.0 | 0.0 | Nov-Dec 2008 |
| Summary of All Basins | 1,038,899 | 811,851 | 26,563 | 200,485 | 628 | 95.0 | 93.4-96.7 | 5.0 | 2004–08 |

Figure A.10.  Summary of Fecal Coliform Assessment for Large Lakes

Table A.10.  Percentage of Large Lakes Meeting Threshold Value for Fecal Coliform Calculated Using Probabilistic Monitoring Design

Status Network		Designated Use:  Recreation and Aquatic Life 		Units:  Acres

This is a 10-column table.  Column 1 lists the basin, Column 2 lists the target population, Column 3 lists the sampleable acres, Column 4 lists the dry miles, Column 5 lists the inaccessible acres, Column 6 lists the number of samples, Column 7 lists the percent meeting threshold, Column 8 lists the 95% confidence bounds, Column 9 lists the percent not meeting threshold, and Column 10 lists the assessment period.

1 No large lakes in basin

2 N/A = Not applicable

| Basin | Target Population | Sampleable | Dry | Inaccessible | Number of Samples | % Meeting Threshold | 95% Confidence Bounds (% Meeting Threshold) | % Not Meeting Threshold | Assessment Period |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NWFWMD1 Ochlockonee–St. Marks | 30,107 | 17,042 | 1,704 | 11,361 | 30 | 96.7 | 91.0-100.0 | 3.3 | June-Aug 2004 |
| SFWMD1 Everglades–West Coast | 1,524 | 1,426 | 0 | 98 | 29 | 100.0 | 100.0 | 0.0 | Oct-Dec 2004 |
| SFWMD6 Lake Okeechobee | 340,652 | 234,933 | 0 | 105,719 | 17 | 100.0 | 100.0 | 0.0 | Nov-Dec 2004 |
| SJRWMD1 Ocklawaha | 142,923 | 119,103 | 0 | 23,820 | 29 | 100.0 | 100.0 | 0.0 | Oct-Dec 2004 |
| SRWMD1 Suwannee | 24,566 | 14,932 | 4,335 | 5,299 | 31 | 93.5 | 86.0-100.0 | 6.5 | July 2004 |
| SWFWMD1 Tampa Bay | 6,191 | 6,191 | 0 | 0 | 30 | 96.7 | 91.0-100.0 | 3.3 | Oct-Nov 2004 |
| NWFWMD2 Apalachicola–Chipola | 15,723 | 13,873 | 0 | 1,850 | 30 | 96.7 | 91.0-100.0 | 3.3 | July 2005 |
| SFWMD2 St Lucie–Loxahatchee | 626 | 470 | 0 | 156 | 28 | 100.0 | 100.0 | 0.0 | Nov-Dec 2005 |
| SFWMD7 Charlotte Harbor | 10 | 0 | 0 | 10 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD2 Lower St. Johns | 37,417 | 30,338 | 0 | 7,079 | 29 | 100.0 | 100.0 | 0.0 | Oct-Dec 2005 |
| SJRWMD6 Middle St. Johns | 116,082 | 94,121 | 6,275 | 15,686 | 27 | 100.0 | 100.0 | 0.0 | Nov-Dec 2005 |
| SWFWMD2 Tampa Bay Tributaries | 6,473 | 4,979 | 0 | 1,494 | 30 | 100.0 | 100.0 | 0.0 | Oct-Nov 2005 |
| NWFWMD3 Choctawhatchee– St. Andrews | 14,871 | 8,747 | 5,832 | 292 | 30 | 100.0 | 100.0 | 0.0 | Jun-Aug 2006 |
| SFWMD3 Caloosahatchee | 203 | 0 | 0 | 203 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SFWMD8 Lake Worth Lagoon– Palm Beach Coast | 1,490 | 1,490 | 0 | 0 | 29 | 96.6 | 90.9-100.0 | 3.4 | Oct-Nov 2006 |
| SJRWMD3 Upper St. Johns | 20,908 | 16,952 | 565 | 3,391 | 30 | 100.0 | 100.0 | 0.0 | Oct-Nov 2006 |
| SWFWMD3 Sarasota Bay–Peace–Myakka | 36,927 | 31,652 | 0 | 5,275 | 30 | 100.0 | 100.0 | 0.0 | Nov-Dec 2006 |
| NWFWMD4 Pensacola | 1,688 | 1,298 | 216 | 174 | 30 | 100.0 | 100.0 | 0.0 | June-July 2007 |
| SFWMD4 Fisheating Creek | 419 | 185 | 0 | 234 | 30 | 100.0 | 100.0 | 0.0 | Nov-Dec 2007 |
| SFWMD9 Kissimmee River | 192,758 | 192,758 | 0 | 0 | 29 | 100.0 | 100.0 | 0.0 | Nov-Dec 2007 |
| SFWMD10 Southeast Coast– Biscayne Bay1 | 0 | 0 | 0 | 0 | 0 | N/A2 | N/A2 | N/A2 | Nov-Dec 2007 |
| SJRWMD4 Nassau-St. Marys | 2,874 | 2,874 | 0 | 0 | 30 | 100.0 | 100.0 | 0.0 | Oct 2007 |
| SWFWMD4 Withlacoochee | 38,016 | 17,022 | 2,837 | 18,157 | 30 | 100.0 | 100.0 | 0.0 | Nov-Dec 2007 |
| NWFWMD5 Perdido | 69 | 69 | 0 | 0 | 8 | 100.0 | 100.0 | 0.0 | June 2008 |
| SFWMD5 Everglades1 | 0 | 0 | 0 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SFWMD11 Florida Keys1 | 0 | 0 | 0 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD5 Indian River Lagoon | 90 | 0 | 90 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD7 Upper East Coast | 288 | 0 | 288 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SWFWMD5 Springs Coast | 6,004 | 1,396 | 4,421 | 187 | 30 | 96.7 | 91.0-100.0 | 3.3 | Nov-Dec 2008 |
| Summary of All Basins | 1,038,899 | 811,851 | 26,563 | 200,485 | 616 | 99.7 | 99.5-99.9 | 0.3 | 2004–08 |

Figure A.11.  Summary of Un-ionized Ammonia Assessment for Large Lakes

Table A.11.  Percentage of Large Lakes Meeting Threshold Value for Un-ionized Ammonia Calculated Using Probabilistic Monitoring Design

Status Network		Designated Use:  Recreation and Aquatic Life 		Units:  Acres

This is a 10-column table.  Column 1 lists the basin, Column 2 lists the target population, Column 3 lists the sampleable acres, Column 4 lists the dry miles, Column 5 lists the inaccessible acres, Column 6 lists the number of samples, Column 7 lists the percent meeting threshold, Column 8 lists the 95% confidence bounds, Column 9 lists the percent not meeting threshold, and Column 10 lists the assessment period.

1 No large lakes in basin

2 N/A = Not applicable

| Basin | Target Population | Sampleable | Dry | Inaccessible | Number of Samples | % Meeting Threshold | 95% Confidence Bounds (% Meeting Threshold) | % Not Meeting Threshold | Assessment Period |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NWFWMD1 Ochlockonee–St. Marks | 30,107 | 17,042 | 1,704 | 11,361 | 30 | 100.0 | 100.0 | 0.0 | June-Aug 2004 |
| SFWMD1 Everglades–West Coast | 1,524 | 1,426 | 0 | 98 | 29 | 89.7 | 79.9-99.4 | 10.3 | Oct-Dec 2004 |
| SFWMD6 Lake Okeechobee | 340,652 | 234,933 | 0 | 105,719 | 19 | 100.0 | 100.0 | 0.0 | Nov-Dec 2004 |
| SJRWMD1 Ocklawaha | 142,923 | 119,103 | 0 | 23,820 | 30 | 80.0 | 71.4-88.6 | 20.0 | Oct-Dec 2004 |
| SRWMD1 Suwannee | 24,566 | 14,932 | 4,335 | 5,299 | 31 | 100.0 | 100.0 | 0.0 | July 2004 |
| SWFWMD1 Tampa Bay | 6,191 | 6,191 | 0 | 0 | 30 | 100.0 | 100.0 | 0.0 | Oct-Nov 2004 |
| NWFWMD2 Apalachicola–Chipola | 15,723 | 13,873 | 0 | 1,850 | 30 | 100.0 | 100.0 | 0.0 | July 2005 |
| SFWMD2 St Lucie–Loxahatchee | 626 | 470 | 0 | 156 | 29 | 100.0 | 100.0 | 0.0 | Nov-Dec 2005 |
| SFWMD7 Charlotte Harbor | 10 | 0 | 0 | 10 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD2 Lower St. Johns | 37,417 | 30,338 | 0 | 7,079 | 30 | 100.0 | 100.0 | 0.0 | Oct-Dec 2005 |
| SJRWMD6 Middle St. Johns | 116,082 | 94,121 | 6,275 | 15,686 | 30 | 100.0 | 100.0 | 0.0 | Nov-Dec 2005 |
| SWFWMD2 Tampa Bay Tributaries | 6,473 | 4,979 | 0 | 1,494 | 30 | 100.0 | 100.0 | 0.0 | Oct-Nov 2005 |
| NWFWMD3 Choctawhatchee– St. Andrews | 14,871 | 8,747 | 5,832 | 292 | 30 | 100.0 | 100.0 | 0.0 | Jun-Aug 2006 |
| SFWMD3 Caloosahatchee | 203 | 0 | 0 | 203 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SFWMD8 Lake Worth Lagoon– Palm Beach Coast | 1,490 | 1,490 | 0 | 0 | 30 | 100.0 | 100.0 | 0.0 | Oct-Nov 2006 |
| SJRWMD3 Upper St. Johns | 20,908 | 16,952 | 565 | 3,391 | 30 | 100.0 | 100.0 | 0.0 | Oct-Nov 2006 |
| SWFWMD3 Sarasota Bay–Peace–Myakka | 36,927 | 31,652 | 0 | 5,275 | 30 | 100.0 | 100.0 | 0.0 | Nov-Dec 2006 |
| NWFWMD4 Pensacola | 1,688 | 1,298 | 216 | 174 | 30 | 93.3 | 87.3-99.3 | 6.7 | June-July 2007 |
| SFWMD4 Fisheating Creek | 419 | 185 | 0 | 234 | 30 | 100.0 | 100.0 | 0.0 | Nov-Dec 2007 |
| SFWMD9 Kissimmee River | 192,758 | 192,758 | 0 | 0 | 30 | 100.0 | 100.0 | 0.0 | Nov-Dec 2007 |
| SFWMD10 Southeast Coast– Biscayne Bay1 | 0 | 0 | 0 | 0 | 0 | N/A2 | N/A2 | N/A2 | Nov-Dec 2007 |
| SJRWMD4 Nassau-St. Marys | 2,874 | 2,874 | 0 | 0 | 30 | 100.0 | 100.0 | 0.0 | Oct 2007 |
| SWFWMD4 Withlacoochee | 38,016 | 17,022 | 2,837 | 18,157 | 30 | 100.0 | 100.0 | 0.0 | Nov-Dec 2007 |
| NWFWMD5 Perdido | 69 | 69 | 0 | 0 | 8 | 100.0 | 100.0 | 0.0 | June 2008 |
| SFWMD5 Everglades1 | 0 | 0 | 0 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SFWMD11 Florida Keys1 | 0 | 0 | 0 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD5 Indian River Lagoon | 90 | 0 | 90 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD7 Upper East Coast | 288 | 0 | 288 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SWFWMD5 Springs Coast | 6,004 | 1,396 | 4,421 | 187 | 30 | 100.0 | 100.0 | 0.0 | Nov-Dec 2008 |
| Summary of All Basins | 1,038,899 | 811,851 | 26,563 | 200,485 | 626 | 96.6 | 95.2-98.1 | 3.4 | 2004–08 |

Figure A.12.  Summary of TSI Assessment for Large Lakes

Table A.12.  Percentage of Large Lakes Meeting Threshold Value for TSI Calculated Using Probabilistic Monitoring Design

Status Network		Designated Use:  Recreation and Aquatic Life 		Units:  Acres

This is a 10-column table.  Column 1 lists the basin, Column 2 lists the target population, Column 3 lists the sampleable acres, Column 4 lists the dry miles, Column 5 lists the inaccessible acres, Column 6 lists the number of samples, Column 7 lists the percent meeting threshold, Column 8 lists the 95% confidence bounds, Column 9 lists the percent not meeting threshold, and Column 10 lists the assessment period.

1 No large lakes in basin

2 N/A = Not applicable

| Basin | Target Population | Sampleable | Dry | Inaccessible | Number of Samples | % Meeting Threshold | 95% Confidence Bounds (% Meeting Threshold) | % Not Meeting Threshold | Assessment Period |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NWFWMD1 Ochlockonee–St. Marks | 30,107 | 17,042 | 1,704 | 11,361 | 30 | 96.7 | 90.9-100.0 | 3.3 | June-Aug 2004 |
| SFWMD1 Everglades–West Coast | 1,524 | 1,426 | 0 | 98 | 29 | 79.3 | 66.8-91.8 | 20.7 | Oct-Dec 2004 |
| SFWMD6 Lake Okeechobee | 340,652 | 234,933 | 0 | 105,719 | 19 | 94.7 | 86.1-100.0 | 5.3 | Nov-Dec 2004 |
| SJRWMD1 Ocklawaha | 142,923 | 119,103 | 0 | 23,820 | 30 | 90.0 | 80.7-99.3 | 10.0 | Oct-Dec 2004 |
| SRWMD1 Suwannee | 24,566 | 14,932 | 4,335 | 5,299 | 31 | 93.5 | 86.1-100.0 | 6.5 | July 2004 |
| SWFWMD1 Tampa Bay | 6,191 | 6,191 | 0 | 0 | 30 | 93.3 | 87.7-98.9 | 6.7 | Oct-Nov 2004 |
| NWFWMD2 Apalachicola–Chipola | 15,723 | 13,873 | 0 | 1,850 | 30 | 93.3 | 85.5-100.0 | 6.7 | July 2005 |
| SFWMD2 St Lucie–Loxahatchee | 626 | 470 | 0 | 156 | 30 | 100.0 | 100.0 | 0.0 | Nov-Dec 2005 |
| SFWMD7 Charlotte Harbor | 10 | 0 | 0 | 10 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD2 Lower St. Johns | 37,417 | 30,338 | 0 | 7,079 | 30 | 100.0 | 100.0 | 0.0 | Oct-Dec 2005 |
| SJRWMD6 Middle St. Johns | 116,082 | 94,121 | 6,275 | 15,686 | 30 | 100.0 | 100.0 | 0.0 | Nov-Dec 2005 |
| SWFWMD2 Tampa Bay Tributaries | 6,473 | 4,979 | 0 | 1,494 | 30 | 93.3 | 87.4-99.3 | 6.7 | Oct-Nov 2005 |
| NWFWMD3 Choctawhatchee– St. Andrews | 14,871 | 8,747 | 5,832 | 292 | 30 | 100.0 | 100.0 | 0.0 | Jun-Aug 2006 |
| SFWMD3 Caloosahatchee | 203 | 0 | 0 | 203 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SFWMD8 Lake Worth Lagoon– Palm Beach Coast | 1,490 | 1,490 | 0 | 0 | 30 | 100.0 | 100.0 | 0.0 | Oct-Nov 2006 |
| SJRWMD3 Upper St. Johns | 20,908 | 16,952 | 565 | 3,391 | 30 | 100.0 | 100.0 | 0.0 | Oct-Nov 2006 |
| SWFWMD3 Sarasota Bay–Peace–Myakka | 36,927 | 31,652 | 0 | 5,275 | 30 | 73.3 | 60.3-86.3 | 26.7 | Nov-Dec 2006 |
| NWFWMD4 Pensacola | 1,688 | 1,298 | 216 | 174 | 30 | 90.0 | 81.5-98.5 | 10.0 | June-July 2007 |
| SFWMD4 Fisheating Creek | 419 | 185 | 0 | 234 | 30 | 93.3 | 86.8-99.8 | 6.7 | Nov-Dec 2007 |
| SFWMD9 Kissimmee River | 192,758 | 192,758 | 0 | 0 | 30 | 93.3 | 85.6-100.0 | 6.7 | Nov-Dec 2007 |
| SFWMD10 Southeast Coast– Biscayne Bay1 | 0 | 0 | 0 | 0 | 0 | N/A2 | N/A2 | N/A2 | Nov-Dec 2007 |
| SJRWMD4 Nassau–St. Marys | 2,874 | 2,874 | 0 | 0 | 30 | 96.7 | 90.8-100.0 | 3.3 | Oct 2007 |
| SWFWMD4 Withlacoochee | 38,016 | 17,022 | 2,837 | 18,157 | 30 | 96.7 | 90.8-100.0 | 3.3 | Nov-Dec 2007 |
| NWFWMD5 Perdido | 69 | 69 | 0 | 0 | 8 | 100.0 | 100.0 | 0.0 | June 2008 |
| SFWMD5 Everglades1 | 0 | 0 | 0 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SFWMD11 Florida Keys1 | 0 | 0 | 0 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD5 Indian River Lagoon | 90 | 0 | 90 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD7 Upper East Coast | 288 | 0 | 288 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SWFWMD5 Springs Coast | 6,004 | 1,396 | 4,421 | 187 | 30 | 90.0 | 80.7-99.3 | 10.0 | Nov-Dec 2008 |
| Summary of All Basins | 1,038,899 | 811,851 | 26,563 | 200,485 | 6327 | 94.0 | 90.75-97.6 | 6.0 | 2004–08 |

Figure A.13.  Summary of DO Assessment for Small Lakes

Table A.13.  Percentage of Small Lakes Meeting Threshold Value for DO Calculated Using Probabilistic Monitoring Design

Status Network		Designated Use:  Recreation and Aquatic Life 		Units:  Lakes

This is a 10-column table.  Column 1 lists the basin, Column 2 lists the target population, Column 3 lists the sampleable lakes, Column 4 lists the dry miles, Column 5 lists the inaccessible lakes, Column 6 lists the number of samples, Column 7 lists the percent meeting threshold, Column 8 lists the 95% confidence bounds, Column 9 lists the percent not meeting threshold, and Column 10 lists the assessment period.

1 No small lakes in basin

2 N/A = Not applicable

| Basin | Target Population | Sampleable | Dry | Inaccessible | Number of Samples | % Meeting Threshold | 95% Confidence Bounds (% Meeting Threshold) | % Not Meeting Threshold | Assessment Period |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NWFWMD1 Ochlockonee–St. Marks | 301 | 161 | 32 | 108 | 30 | 86.7 | 77.6-95.8 | 13.3 | March-April 2004 |
| SFWMD1 Everglades–West Coast | 3 | 0 | 3 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SFWMD6 Lake Okeechobee1 | 0 | 0 | 0 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD1 Ocklawaha | 552 | 224 | 75 | 253 | 30 | 53.3 | 37.1-69.6 | 46.7 | July-Sept 2004 |
| SRWMD1 Suwannee | 994 | 215 | 382 | 397 | 27 | 88.9 | 78.6-99.2 | 11.1 | March-April 2004 |
| SWFWMD1 Tampa Bay | 216 | 175 | 6 | 35 | 30 | 63.3 | 49.3-77.4 | 36.7 | July-Aug 2004 |
| NWFWMD2 Apalachicola–Chipola | 353 | 76 | 76 | 201 | 30 | 73.3 | 61.2-85.4 | 26.7 | March-May 2005 |
| SFWMD2 St Lucie–Loxahatchee | 18 | 11 | 0 | 7 | 11 | 27.3 | 2.2-52.4 | 72.7 | July-Aug 2005 |
| SFWMD7 Charlotte Harbor | 19 | 0 | 6 | 13 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD2 Lower St. Johns | 180 | 84 | 6 | 90 | 30 | 66.7 | 52.8-80.5 | 33.3 | July 2005 |
| SJRWMD6 Middle St. Johns | 499 | 272 | 9 | 218 | 30 | 36.7 | 22.0-51.3 | 63.3 | July-Aug 2005 |
| SWFWMD2 Tampa Bay Tributaries | 248 | 105 | 24 | 119 | 31 | 41.9 | 27.5-56.4 | 58.1 | July-Aug 2005 |
| NWFWMD3 Choctawhatchee– St. Andrews | 298 | 142 | 19 | 137 | 30 | 100.0 | 100.0 | 0.0 | March-May 2006 |
| SFWMD3 Caloosahatchee | 44 | 6 | 0 | 38 | 4 | 50.0 | 2.0-98.0 | 50.0 | July 2006 |
| SFWMD8 Lake Worth Lagoon– Palm Beach Coast | 13 | 10 | 0 | 3 | 10 | 100.0 | 100.0 | 0.0 | July 2006 |
| SJRWMD3 Upper St. Johns | 105 | 16 | 12 | 77 | 4 | 75.0 | 33.4-100.0 | 25.0 | July-Sept 2006 |
| SWFWMD3 Sarasota Bay–Peace–Myakka | 252 | 115 | 9 | 128 | 27 | 81.5 | 68.0-95.0 | 18.5 | Aug-Sept 2006 |
| NWFWMD4 Pensacola | 245 | 119 | 63 | 63 | 30 | 100.0 | 100.0 | 0.0 | April-May 2007 |
| SFWMD4 Fisheating Creek | 19 | 6 | 0 | 13 | 6 | 100.0 | 100.0 | 0.0 | Aug 2007 |
| SFWMD9 Kissimmee River | 260 | 145 | 58 | 57 | 30 | 93.3 | 85.6-100.0 | 6.7 | July-Aug 2007 |
| SFWMD10 Southeast Coast– Biscayne Bay | 16 | 14 | 0 | 2 | 7 | 100.0 | 100.0 | 0.0 | July 2007 |
| SJRWMD4 Nassau–St. Marys | 3 | 2 | 0 | 1 | 2 | 50.0 | 0.0-100.0 | 50.0 | July 2007 |
| SWFWMD4 Withlacoochee | 788 | 115 | 563 | 110 | 24 | 41.7 | 24.6-58.8 | 58.3 | Aug-Sept 2007 |
| NWFWMD5 Perdido | 13 | 10 | 0 | 3 | 10 | 100.0 | 100.0 | 0.0 | March 2008 |
| SFWMD5 Everglades1 | 0 | 0 | 0 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SFWMD11 Florida Keys1 | 0 | 0 | 0 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD5 Indian River Lagoon | 32 | 1 | 26 | 5 | 0 | N/A2 | N/A2 | N/A2 | July 2008 |
| SJRWMD7 Upper East Coast | 7 | 2 | 2 | 3 | 2 | 100.0 | 100.0 | 0.0 | July 2008 |
| SWFWMD5 Springs Coast | 404 | 121 | 145 | 138 | 30 | 73.3 | 58.9-87.7 | 26.7 | July-Sept 2008 |
| Summary of All Basins | 5,882 | 2,147 | 1,516 | 2,219 | 465 | 71.2 | 66.7-75.6 | 28.8 | 2004–08 |

Figure A.14.  Summary of Fecal Coliform Assessment for Small Lakes

Table A.14.  Percentage of Small Lakes Meeting Threshold Value for Fecal Coliform Calculated Using Probabilistic Monitoring Design

Status Network		Designated Use:  Recreation and Aquatic Life 		Units:  Lakes

This is a 10-column table.  Column 1 lists the basin, Column 2 lists the target population, Column 3 lists the sampleable lakes, Column 4 lists the dry miles, Column 5 lists the inaccessible lakes, Column 6 lists the number of samples, Column 7 lists the percent meeting threshold, Column 8 lists the 95% confidence bounds, Column 9 lists the percent not meeting threshold, and Column 10 lists the assessment period.

1 No small lakes in basin

2 N/A = Not applicable

| Basin | Target Population | Sampleable | Dry | Inaccessible | Number of Samples | % Meeting Threshold | 95% Confidence Bounds (% Meeting Threshold ) | % Not Meeting Threshold | Assessment Period |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NWFWMD1 Ochlockonee–St. Marks | 301 | 161 | 32 | 108 | 30 | 96.7 | 91.2-100.0 | 3.3 | March-April 2004 |
| SFWMD1 Everglades–West Coast | 3 | 0 | 3 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SFWMD6 Lake Okeechobee1 | 0 | 0 | 0 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD1 Ocklawaha | 552 | 224 | 75 | 253 | 30 | 93.3 | 87.8-98.8 | 6.7 | July-Sept 2004 |
| SRWMD1 Suwannee | 994 | 215 | 382 | 397 | 25 | 100.0 | 100.0 | 0.0 | March-April 2004 |
| SWFWMD1 Tampa Bay | 216 | 175 | 6 | 35 | 30 | 83.3 | 72.8-93.9 | 16.7 | July-Aug 2004 |
| NWFWMD2 Apalachicola–Chipola | 353 | 76 | 76 | 201 | 30 | 96.7 | 91.0-100.0 | 3.3 | March-May 2005 |
| SFWMD2 St Lucie–Loxahatchee | 18 | 11 | 0 | 7 | 8 | 87.5 | 67.2-100.0 | 12.5 | July-Aug 2005 |
| SFWMD7 Charlotte Harbor | 19 | 0 | 6 | 13 | 0 | N/A2 | N/A2 | N/A2 | July-Aug 2005 |
| SJRWMD2 Lower St. Johns | 180 | 84 | 6 | 90 | 30 | 96.7 | 91.0-100.0 | 3.3 | July 2005 |
| SJRWMD6 Middle St. Johns | 499 | 272 | 9 | 218 | 30 | 96.7 | 91.0-100.0 | 3.3 | July-Aug 2005 |
| SWFWMD2 Tampa Bay Tributaries | 248 | 105 | 24 | 119 | 30 | 100.0 | 100.0 | 0.0 | July-Aug 2005 |
| NWFWMD3 Choctawhatchee- St. Andrews | 298 | 142 | 19 | 137 | 30 | 100.0 | 100.0 | 0.0 | March-May 2006 |
| SFWMD3 Caloosahatchee | 44 | 6 | 0 | 38 | 4 | 100.0 | 100.0 | 0.0 | July 2006 |
| SFWMD8 Lake Worth Lagoon– Palm Beach Coast | 13 | 10 | 0 | 3 | 10 | 90.0 | 73.7-100.0 | 10.0 | July 2006 |
| SJRWMD3 Upper St. Johns | 105 | 16 | 12 | 77 | 2 | 50.0 | 0.0-100.0 | 50.0 | July-Sept 2006 |
| SWFWMD3 Sarasota Bay–Peace–Myakka | 252 | 115 | 9 | 128 | 27 | 77.8 | 64.7-90.8 | 22.2 | Aug-Sept 2006 |
| NWFWMD4 Pensacola | 245 | 119 | 63 | 63 | 30 | 100.0 | 100.0 | 0.0 | April-May 2007 |
| SFWMD4 Fisheating Creek | 19 | 6 | 0 | 13 | 6 | 50.0 | 10.8-89.2 | 50.0 | Aug 2007 |
| SFWMD9 Kissimmee River | 260 | 145 | 58 | 57 | 30 | 100.0 | 100.0 | 0.0 | July-Aug 2007 |
| SFWMD10 Southeast Coast– Biscayne Bay | 16 | 14 | 0 | 2 | 7 | 85.7 | 62.0-100.0 | 14.3 | July 2007 |
| SJRWMD4 Nassau–St. Marys | 3 | 2 | 0 | 1 | 2 | 100.0 | 100.0 | 0.0 | July 2007 |
| SWFWMD4 Withlacoochee | 788 | 115 | 563 | 110 | 24 | 95.8 | 88.8-100.0 | 4.2 | Aug-Sept 2007 |
| NWFWMD5 Perdido | 13 | 10 | 0 | 3 | 10 | 90.0 | 73.8-100.0 | 10.0 | March 2008 |
| SFWMD5 Everglades1 | 0 | 0 | 0 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SFWMD11 Florida Keys1 | 0 | 0 | 0 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD5 Indian River Lagoon | 32 | 1 | 26 | 5 | 0 | N/A2 | N/A2 | N/A2 | July 2008 |
| SJRWMD7 Upper East Coast | 7 | 2 | 2 | 3 | 2 | 100.0 | 100.0 | 0.0 | July 2008 |
| SWFWMD5 Springs Coast | 404 | 121 | 145 | 138 | 29 | 75.9 | 62.5-89.2 | 24.1 | July-Sept 2008 |
| Summary of All Basins | 5882 | 2147 | 1516 | 2219 | 456 | 90.9 | 84.1-97.7 | 9.1 | 2004–08 |

Figure A.15.  Summary of Un-ionized Ammonia Assessment for Small Lakes

Table A.15.  Percentage of Small Lakes Meeting Threshold Value for Un-ionized Ammonia Calculated Using Probabilistic Monitoring Design

Status Network		Designated Use:  Recreation and Aquatic Life 		Units:  Lakes

This is a 10-column table.  Column 1 lists the basin, Column 2 lists the target population, Column 3 lists the sampleable lakes, Column 4 lists the dry miles, Column 5 lists the inaccessible lakes, Column 6 lists the number of samples, Column 7 lists the percent meeting threshold, Column 8 lists the 95% confidence bounds, Column 9 lists the percent not meeting threshold, and Column 10 lists the assessment period.

1 No small lakes in basin

2 N/A = Not applicable

| Basin | Target Population | Sampleable | Dry | Inaccessible | Number of Samples | % Meeting Threshold | 95% Confidence Bounds (% Meeting Threshold) | % Not Meeting Threshold | Assessment Period |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NWFWMD1 Ochlockonee–St. Marks | 301 | 161 | 32 | 108 | 30 | 100.0 | 100.0 | 0.0 | March-April 2004 |
| SFWMD1 Everglades–West Coast | 3 | 0 | 3 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SFWMD6 Lake Okeechobee1 | 0 | 0 | 0 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD1 Ocklawaha | 552 | 224 | 75 | 253 | 30 | 100.0 | 100.0 | 0.0 | July-Sept 2004 |
| SRWMD1 Suwannee | 994 | 215 | 382 | 397 | 27 | 96.3 | 90.3-100.0 | 3.7 | March-April 2004 |
| SWFWMD1 Tampa Bay | 216 | 175 | 6 | 35 | 30 | 96.7 | 90.9-100.0 | 3.3 | July-Aug 2004 |
| NWFWMD2 Apalachicola–Chipola | 353 | 76 | 76 | 201 | 30 | 100.0 | 100.0 | 0.0 | March-May 2005 |
| SFWMD2 St Lucie–Loxahatchee | 18 | 11 | 0 | 7 | 11 | 100.0 | 100.0 | 0.0 | July-Aug 2005 |
| SFWMD7 Charlotte Harbor | 19 | 0 | 6 | 13 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD2 Lower St. Johns | 180 | 84 | 6 | 90 | 30 | 100.0 | 100.0 | 0.0 | July 2005 |
| SJRWMD6 Middle St. Johns | 499 | 272 | 9 | 218 | 30 | 100.0 | 100.0 | 0.0 | July-Aug 2005 |
| SWFWMD2 Tampa Bay Tributaries | 248 | 105 | 24 | 119 | 31 | 100.0 | 100.0 | 0.0 | July-Aug 2005 |
| NWFWMD3 Choctawhatchee– St. Andrews | 298 | 142 | 19 | 137 | 30 | 100.0 | 100.0 | 0.0 | March-May 2006 |
| SFWMD3 Caloosahatchee | 44 | 6 | 0 | 38 | 4 | 100.0 | 100.0 | 0.0 | July 2006 |
| SFWMD8 Lake Worth Lagoon- Palm Beach Coast | 13 | 10 | 0 | 3 | 10 | 100.0 | 100.0 | 0.0 | July 2006 |
| SJRWMD3 Upper St. Johns | 105 | 16 | 12 | 77 | 4 | 100.0 | 100.0 | 0.0 | July-Sept 2006 |
| SWFWMD3 Sarasota Bay–Peace–Myakka | 252 | 115 | 9 | 128 | 27 | 100.0 | 100.0 | 0.0 | Aug-Sept 2006 |
| NWFWMD4 Pensacola | 245 | 119 | 63 | 63 | 29 | 100.0 | 100.0 | 0.0 | April-May 2007 |
| SFWMD4 Fisheating Creek | 19 | 6 | 0 | 13 | 6 | 100.0 | 100.0 | 0.0 | Aug 2007 |
| SFWMD9 Kissimmee River | 260 | 145 | 58 | 57 | 30 | 100.0 | 100.0 | 0.0 | July-Aug 2007 |
| SFWMD10 Southeast Coast– Biscayne Bay | 16 | 14 | 0 | 2 | 7 | 100.0 | 100.0 | 0.0 | July 2007 |
| SJRWMD4 Nassau–St. Marys | 3 | 2 | 0 | 1 | 2 | 100.0 | 100.0 | 0.0 | July 2007 |
| SWFWMD4 Withlacoochee | 788 | 115 | 563 | 110 | 24 | 100.0 | 100.0 | 0.0 | Aug-Sept 2007 |
| NWFWMD5 Perdido | 13 | 10 | 0 | 3 | 10 | 100.0 | 100.0 | 0.0 | March 2008 |
| SFWMD5 Everglades1 | 0 | 0 | 0 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SFWMD11 Florida Keys1 | 0 | 0 | 0 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD5 Indian River Lagoon | 32 | 1 | 26 | 5 | 0 | N/A2 | N/A2 | N/A2 | July 2008 |
| SJRWMD7 Upper East Coast | 7 | 2 | 2 | 3 | 2 | 100.0 | 100.0 | 0.0 | July 2008 |
| SWFWMD5 Springs Coast | 404 | 121 | 145 | 138 | 30 | 96.7 | 91.3-100.0 | 3.3 | July-Sept 2008 |
| Summary of All Basins | 5,882 | 2,147 | 1,516 | 2,219 | 464 | 99.3 | 98.5-100.0 | 0.7 | 2004–08 |

Figure A.16.  Summary of TSI Assessment for Small Lakes

Table A.16.  Percentage of Small Lakes Meeting Threshold Value for TSI Calculated Using Probabilistic Monitoring Design

Status Network		Designated Use:  Recreation and Aquatic Life 		Units:  Lakes

This is a 10-column table.  Column 1 lists the basin, Column 2 lists the target population, Column 3 lists the sampleable lakes, Column 4 lists the dry miles, Column 5 lists the inaccessible lakes, Column 6 lists the number of samples, Column 7 lists the percent meeting threshold, Column 8 lists the 95% confidence bounds, Column 9 lists the percent not meeting threshold, and Column 10 lists the assessment period.

1 No small lakes in basin

2 N/A = Not applicable

| Basin | Target Population | Sampleable | Dry | Inaccessible | Number of Samples | % Meeting Threshold | 95% Confidence Bounds (% Meeting Threshold) | % Not Meeting Threshold | Assessment Period |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NWFWMD1 Ochlockonee–St. Marks | 301 | 161 | 32 | 108 | 30 | 93.3 | 85.7-100.0 | 6.7 | March-April 2004 |
| SFWMD1 Everglades–West Coast | 3 | 0 | 3 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SFWMD6 Lake Okeechobee1 | 0 | 0 | 0 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD1 Ocklawaha | 552 | 224 | 75 | 253 | 30 | 93.3 | 85.6-100.0 | 6.7 | July-Sept 2004 |
| SRWMD1 Suwannee | 994 | 215 | 382 | 397 | 27 | 100.0 | 100.0 | 0.0 | March-April 2004 |
| SWFWMD1 Tampa Bay | 216 | 175 | 6 | 35 | 30 | 83.3 | 73.2-93.5 | 16.7 | July-Aug 2004 |
| NWFWMD2 Apalachicola–Chipola | 353 | 76 | 76 | 201 | 30 | 96.7 | 91.0-100.0 | 3.3 | March-May 2005 |
| SFWMD2 St Lucie–Loxahatchee | 18 | 11 | 0 | 7 | 11 | 100.0 | 100.0 | 0.0 | July-Aug 2005 |
| SFWMD7 Charlotte Harbor | 19 | 0 | 6 | 13 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD2 Lower St. Johns | 180 | 84 | 6 | 90 | 30 | 93.3 | 85.3-100.0 | 6.7 | July 2005 |
| SJRWMD6 Middle St. Johns | 499 | 272 | 9 | 218 | 30 | 90.0 | 80.1-99.9 | 10.0 | July-Aug 2005 |
| SWFWMD2 Tampa Bay Tributaries | 248 | 105 | 24 | 119 | 31 | 87.1 | 77.9-96.3 | 12.9 | July-Aug 2005 |
| NWFWMD3 Choctawhatchee– St. Andrews | 298 | 142 | 19 | 137 | 30 | 100.0 | 100.0 | 0.0 | March-May 2006 |
| SFWMD3 Caloosahatchee | 44 | 6 | 0 | 38 | 4 | 100.0 | 100.0 | 0.0 | July 2006 |
| SFWMD8 Lake Worth Lagoon– Palm Beach Coast | 13 | 10 | 0 | 3 | 10 | 90.0 | 72.7-100.0 | 10.0 | July 2006 |
| SJRWMD3 Upper St. Johns | 105 | 16 | 12 | 77 | 4 | 75.0 | 33.1-100.0 | 25.0 | July-Sept 2006 |
| SWFWMD3 Sarasota Bay–Peace–Myakka | 252 | 115 | 9 | 128 | 27 | 88.9 | 78.4-99.4 | 11.1 | Aug-Sept 2006 |
| NWFWMD4 Pensacola | 245 | 119 | 63 | 63 | 30 | 93.3 | 85.4-100.0 | 6.7 | April-May 2007 |
| SFWMD4 Fisheating Creek | 19 | 6 | 0 | 13 | 6 | 100.0 | 100.0 | 0.0 | Aug 2007 |
| SFWMD9 Kissimmee River | 260 | 145 | 58 | 57 | 30 | 80.0 | 68.7-91.3 | 20.0 | July-Aug 2007 |
| SFWMD10 Southeast Coast– Biscayne Bay | 16 | 14 | 0 | 2 | 7 | 100.0 | 100.0 | 0.0 | July 2007 |
| SJRWMD4 Nassau–St. Marys | 3 | 2 | 0 | 1 | 2 | 100.0 | 100.0 | 0.0 | July 2007 |
| SWFWMD4 Withlacoochee | 788 | 115 | 563 | 110 | 24 | 83.3 | 70.6-96.1 | 16.7 | Aug-Sept 2007 |
| NWFWMD5 Perdido | 13 | 10 | 0 | 3 | 10 | 100.0 | 100.0 | 0.0 | March 2008 |
| SFWMD5 Everglades1 | 0 | 0 | 0 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SFWMD11 Florida Keys1 | 0 | 0 | 0 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD5 Indian River Lagoon | 32 | 1 | 26 | 5 | 0 | N/A2 | N/A2 | N/A2 | July 2008 |
| SJRWMD7 Upper East Coast | 7 | 2 | 2 | 3 | 2 | 100.0 | 100.0 | 0.0 | July 2008 |
| SWFWMD5 Springs Coast | 404 | 121 | 145 | 138 | 30 | 76.7 | 63.4-89.9 | 23.3 | July-Sept 2008 |
| Summary of All Basins | 5,882 | 2,147 | 1,516 | 2,219 | 465 | 90.3 | 86.8-93.8 | 9.7 | 2004–08 |

# APPENDIX B:  Discussion of Status Network Surface Water Indicators for Rivers, Streams, and Lakes, and Ground Water Indicators for Confined and Unconfined Aquifers

### Surface Water Indicators for Rivers, Streams, and Lakes

Table B.1 contains the list of surface water analytes, with the associated criterion for each analyte.

Table B.1.  Status Monitoring Network Water Quality Standards for Surface Water

This is a three-column table.  Column 1 lists the indicator, Column 2 lists the criterion/threshold, and Column 3 lists the designated use.

| Indicators | Criterion/Threshold1 | Designated Use |
| --- | --- | --- |
| Fecal Coliform Bacteria | < 400 counts/100mL | Recreation |
| DO | ≥ 5 mg/L | Aquatic Life |
| Un-ionized Ammonia | ≤ 0.02 mg/L | Aquatic Life |
| Chlorophyll a | ≤ 20 μg/L | Aquatic Life |
| TSI | Color ≤ 40 PCUs, then TSI ≤ 40 Color > 40 PCUs, then TSI ≤ 60 | Aquatic Life |

1 mL – milliliters; mg/L – milligrams per liter; μg/L – micrograms per liter; PCUs – platinum cobalt units

#### Fecal Coliform Bacteria

The threshold for fecal coliform bacteria is 400 colonies per 100 mL of water.  Additionally, twice that number (800), as cited in Rule 62-302, F.A.C., indicates a highly contaminated result, and is used for regulatory purposes.  The presence of these bacteria can indicate the contamination of a waterway or well and the possible presence of other pathogenic organisms.

Fecal coliform bacteria can enter water through the discharge of waste from mammals and birds, agricultural and stormwater runoff, and untreated human sewage.  Septic tanks for individual homes can become overloaded during the rainy season and allow untreated human wastes to flow into drainage ditches and nearby waters.  Agricultural practices that fail to contain animal wastes during the rainy season, as well as spreading manure and fertilizer on fields during rainy periods, and allowing livestock access to streams can all contribute fecal coliform contamination.

#### Dissolved Oxygen

The state criterion for DO is greater than or equal to 5 mg/L.  DO is a measure of water quality indicating free oxygen dissolved in water.  Oxygen is measured in its dissolved form.  If more oxygen is consumed than is produced, DO levels decline and some sensitive aquatic animals may move away, weaken, or die.  Levels vary with water temperature; therefore, cold water holds more oxygen than warm water.

Surface water gains oxygen from the atmosphere and plants as a result of photosynthesis.  Running water contains more oxygen than still water because of its flow.  Respiration by aquatic animals, decomposition, and various chemical reactions consume oxygen.

Wastewater from sewage treatment plants often contains organic materials that are decomposed by microorganisms, which use oxygen in the process.  Other sources of oxygen-consuming waste include stormwater runoff from farmland or urban streets, feedlots, and failing septic systems.  Ground water is naturally low in DO.  Surface water contact with ground water seepage or upwelling can cause a natural lowering of DO levels.

#### Un-ionized Ammonia

The threshold for un-ionized ammonia is ≤0.02 mg/L as ammonia and is calculated using temperature, salinity, ammonia, and pH.  This criterion applies to predominantly fresh waters in Florida.  In water, ammonia occurs in two forms, which together are called total ammonia nitrogen, or TAN.  Chemically, these two forms are represented as NH4+ and NH3.  NH4+ is called ionized ammonia because it has a positive electrical charge, and NH3 is called un-ionized ammonia as it has no charge.  Un-ionized ammonia (abbreviated as UIA), is the form that is toxic to fish and invertebrates.  Water temperature and pH affect the form of ammonia that is predominant at any given time in an aquatic system.

#### Chlorophyll a

The threshold for chlorophyll a is ≤ 20 g/L.  This threshold is applied to the rivers and streams resources in the Status Monitoring Network.  This is not a criterion under Rule 62-302, F.A.C.; rather, it is listed as a measure to identify impairment in surface waters in Section 62-303.351, F.A.C., which describes the assessment of nutrients in streams.  Chlorophyll is the pigment that allows plants—including algae—to convert sunlight into organic compounds during the process of photosynthesis.  Chlorophyll a is the predominant type found in algae and cyanobacteria (blue-green algae), and its abundance is a measurable indicator of the amount of algae present in a surface waterbody.

Excessive quantities of chlorophyll a can indicate the presence of algal blooms.  These usually consist of species undesirable for fish and other predators to consume.  Unconsumed algae sink to the bottom and decay, using up the oxygen required by other plants and benthic organisms to survive.  The presence of too many nutrients, such as nitrogen and phosphorus, can stimulate algal blooms and reduce water clarity.

Chlorophyll a also plays a direct role in reducing the amount of light available to plants in shallow-water habitats.  Like their terrestrial cousins, these plants need sunlight to grow.  As chlorophyll a levels increase, the amount of sunlight reaching underwater plants declines.

#### Trophic State Index

TSI and chlorophyll a are the primary measures used to assess nutrient impairment in waterbodies.  TSI is measured using chlorophyll, nitrogen, and phosphorus concentrations.  There are two thresholds for TSI that are based on the color of a lake.  Dark-water lakes with a mean color greater than 40 PCUs are impaired when their annual mean TSI exceeds 60.  Clear and low-color lakes with a mean color less than or equal to 40 PCUs are impaired when their annual mean TSI exceeds 40.  A 10-unit increase or decrease in the index represents a doubling or halving, respectively, of the number of algal cells present

Note:  Both chlorophyll a and TSI are not standards, but thresholds used to estimate the condition of state waters.  These thresholds are used in the analysis of Status Monitoring Network data based on single samples in a basin during a predetermined index period.  The analysis and representation of these data are not intended to infer the verification of impairment in these waters, as defined in Rule 62-303, F.A.C.

### Ground Water Indicators

Analytes with primary drinking water standards have been added to measure the condition of Florida’s aquifers.  Table B.2 contains the list of ground water analytes with the associated criterion for each analyte.  Primary standards mean that the criterion for an analyte is based on human health effects.

Key indicator contaminants for ground water (e.g., chloride, nitrate, metals, and bacteria) serve to assess its general suitability for drinking water purposes.  Aquifer samples collected for the Status Monitoring Network between 1999 and 2003 (Cycle 1) were filtered to mitigate well construction factors, and the analytes were measured as dissolved constituents.  Aquifer samples collected for the Status Monitoring Network between 2004 and 2009 (Cycle 2) were not filtered and were analyzed as total constituents.  These samples represent more closely the conditions of water as it comes directly from the aquifer.  Additionally, standards are measured using unfiltered water, and so unfiltered samples allow consistency with standards.

Table B.2.  Status Monitoring Network Water Quality Standards for Ground Water

This is a three-column table.  Column 1 lists the indicator, Column 2 lists the criterion/threshold, and Column 3 lists the designated use.

1 The MDL for fecal coliform, per FDEP SOP MB-X, is 2 counts/100mL.

| Indicators | Criterion/Threshold | Designated Use |
| --- | --- | --- |
| Arsenic | ≤ 10 μg/L | Potable Water (drinking water) |
| Cadmium | ≤ 5 μg/L | Potable Water (drinking water) |
| Chromium | ≤ 100 μg/L | Potable Water (drinking water) |
| Lead | ≤ 15 μg/L | Potable Water (drinking water) |
| Nitrate–Nitrite | ≤ 10 mg/L | Potable Water (drinking water) |
| Sodium | ≤ 160 mg/L | Potable Water (drinking water) |
| Fluoride | ≤ 4 mg/L | Potable Water (drinking water) |
| Total Coliform Bacteria (counts/100mL) | ≤ 4 (sample maximum) | Potable Water (drinking water) |
| Fecal Coliform Bacteria (counts/100mL) | < 2 (sample maximum)1 | Potable Water (drinking water) |

#### Total Coliform Bacteria

The EPA has determined that the presence of total coliform is a possible health concern.  Total coliform bacteria are common in the environment and are generally not harmful themselves.  The presence of these bacteria in drinking water, however, is a result of a problem with water treatment or the pipes that distribute the water, and indicates that the water may be contaminated with organisms that can cause disease.

The EPA and the state have set an enforceable drinking water standard for total coliform of 4 counts per 100 mL to reduce the risk of adverse health effects.  Drinking water that meets this standard is usually not associated with a health risk from disease-causing bacteria and should be considered safe.

#### Fecal Coliform Bacteria

The EPA has determined that the presence of fecal coliform bacteria is a possible health concern because they are usually associated with sewage or animal wastes.  Their presence in drinking water generally results from a problem with water treatment or pipes that distribute the water, and indicates that the water may be contaminated with organisms that can cause disease.  In addition, ground water can become contaminated with fecal coliform from surface water interactions in karst (limestone) terrains, such as those found in Florida.

The EPA and the state have set an enforceable drinking water standard for fecal coliform to reduce the risk of adverse health effects.  Under this standard, all drinking water samples must be free of fecal coliform bacteria.  The FDEP Bureau of Laboratories has a Minimum Detection Limit (MDL) per sample of 2/100mL, and so the presence of any detectable fecal coliform is considered an exceedance.  Drinking water that meets this standard is associated with little or no health risk, and should be considered safe.

#### Arsenic

Arsenic, a naturally occurring element, is widely distributed in the earth’s crust.  Two main categories are found, inorganic and organic arsenic.  Inorganic arsenic compounds are mainly used to preserve wood.  Organic arsenic compounds, found in animals and plants, occur when arsenic combines with carbon and hydrogen.  Organic arsenic compounds are used as pesticides.

Many arsenic compounds can dissolve in water, and can be transported into ground water.  Arsenic can affect human health.  Several studies have shown that inorganic arsenic can increase the risk of lung, skin, bladder, liver, kidney, and prostate cancers.  The World Health Organization (WHO), the U.S. Department of Health and Human Services (DHHS), and the EPA have determined that inorganic arsenic is a human carcinogen.  For this reason, the EPA and FDEP have set an enforceable drinking water standard of 10 parts per billion (ppb) for arsenic.

Organic arsenic compounds are less toxic than inorganic arsenic compounds.  However, exposure to high levels of some organic arsenic compounds may cause similar effects to those of inorganic arsenic.

#### Cadmium

The EPA and FDEP set the drinking water standard for cadmium at 5 ppb to protect against the risk of adverse health effects.  Cadmium, a naturally occurring heavy metal whose chemical properties are similar to those of zinc, does not occur uncombined in nature.  A byproduct of smelting and refining zinc and lead ores, it is used for its anticorrosive properties in the electroplating of steel, in its sulfide form in the manufacture of paint pigments, and in the manufacture of batteries and other electrical components.  Cadmium also occurs as a byproduct in many chemical fertilizers that are produced from phosphate ores.  Cadmium enters the ambient air primarily from local smelting operations, it enters soil from local mining operations and from chemical fertilizers, and it enters water from fertilizer runoff and/or industrial wastewater.

This inorganic metal is a contaminant in the metals used to galvanize pipe.  It generally enters water by the corrosion of galvanized pipes or by improper waste disposal.  Drinking water that meets the EPA standard is associated with little to none of this risk and is considered safe with respect to cadmium.

#### Chromium

This inorganic metal, which occurs naturally in the ground, is often used in the electroplating of metals.  It generally enters water from runoff from old mining operations and improper waste disposal from plating operations.  Some humans exposed to high levels of chromium have suffered liver and kidney damage, dermatitis, and respiratory problems.  The EPA has set the drinking water standard for chromium at 100 ppb to protect against the risk of adverse health effects.  Drinking water that meets the EPA standard is associated with little to none of this risk and is considered safe with respect to chromium.

#### Fluoride

EPA regulations require that fluoride, which occurs naturally in some water supplies, not exceed a concentration of 4.0 mg/L in drinking water.  Extended exposure to drinking water levels above 4.0 mg/L may result in crippling skeletal fluorosis, a serious bone disorder.

State regulations require notification of the public when monitoring indicates that the fluoride in a drinking water system exceeds 2.0 mg/L.  This is intended to alert families about dental problems that might affect children under 9 years of age.  Fluoride in children's drinking water at levels of approximately 1 mg/L reduces the number of dental cavities.  However, some children exposed to levels of fluoride greater than about 2.0 mg/L may develop dental fluorosis.  In its moderate and severe forms, this is a brown staining and/or pitting of the permanent teeth.

Because dental fluorosis occurs only when developing teeth (before they erupt from the gums) are exposed to elevated fluoride levels, households without children are not expected to be affected by this level of fluoride.  Families with children under the age of nine are encouraged to seek other sources of drinking water for their children to avoid the possibility of staining and pitting.

#### Lead

The EPA and FDEP set the drinking water standard for lead at 15 ppb to protect against the risk of adverse health effects.  Lead toxicity affects the nervous system, blood, kidney, heart, and reproductive system.  Infants and young children whose nervous and circulatory systems are not fully developed are more susceptible to the adverse health effects from lead exposure.  Irreversible learning difficulties, mental retardation, and delayed neurological and physical development can occur from long-term exposure to even low levels of lead.

Materials that contain lead have frequently been used in the construction of water supply distribution systems, and in plumbing systems in private homes and other buildings.  The most commonly found materials include service lines, pipes, brass and bronze fixtures, and solders and fluxes.  Lead in these materials can contaminate drinking water as a result of the corrosion that takes place when water comes into contact with those materials.

#### Nitrate-Nitrite

The EPA has set the drinking water standard at 10 ppm for nitrate to protect against the risk of adverse effects.  Excessive levels of nitrate in drinking water have caused serious illness and sometimes death in infants less than 6 months of age.  Nitrate is used in fertilizer and is found in sewage and wastes from human and/or farm animals; it generally enters drinking water from these activities.

The EPA has also set a drinking water standard for nitrite at 1 part per million (ppm).  To allow for the fact that the toxicity of nitrate and nitrite are additive, the EPA has established a standard for the sum of nitrate and nitrite at 10 ppm.  Drinking water that meets the EPA standard is associated with little to none of this risk and is considered safe with respect to nitrate.

#### Sodium

The EPA has set the drinking water standard for sodium (salt) at 160 ppm to protect individuals who are susceptible to sodium-sensitive hypertension or diseases that cause difficulty in regulating body fluid volume.  Sodium is monitored so that individuals on sodium-restricted diets may take the sodium in their water into account.  Sodium naturally occurs in food and drinking water.  Food is the common source of sodium.  Drinking water contributes only a small fraction (less than 10%) of an individual’s overall sodium intake.

# APPENDIX C:  Maps and Tables with 2004–08 Probabilistic Ground Water Assessment Results for Confined Aquifers and Unconfined Aquifers

The Status Network design focuses on the following two ground water resource types:

Confined Aquifers

Unconfined Aquifers

This appendix contains information on the following indicators for Confined and Unconfined Aquifers for the Status Network:

Arsenic

Cadmium

Chromium

Fluoride

Lead

Nitrate + nitrite

Sodium

Fecal coliform

Total coliform

Table C.1 lists the threshold criteria for these ground water analytes, with the associated criterion for each analyte.

Table C.1.  Status Monitoring Network Water Quality Criteria/Thresholds for Ground Water

This is a two-column table.  Column 1 lists the indicator, and Column 2 lists the criterion/threshold.

Note:  Appendix B provides additional information on whether the thresholds listed in the tables in this appendix  are water quality standards or screening levels.

| Indicator | Criterion/Threshold1 |
| --- | --- |
| Arsenic | ≤ 10 μg/L |
| Cadmium | ≤ 5 μg/L |
| Chromium | ≤ 100 μg/L |
| Lead | ≤ 15 μg/L |
| Nitrate–Nitrite | ≤ 10 mg/L |
| Sodium | ≤ 160 mg/L |
| Fluoride | ≤ 4 mg/L |
| Fecal Coliform Bacteria (counts/100mL) | ≤ 2 (sample maximum) |
| Total Coliform Bacteria (counts/100mL) | ≤ 4 (sample maximum) |

Figure C.1.  Summary of Arsenic Assessment for Confined Aquifers

Table C.1.  Percentage of Confined Aquifers Meeting Threshold Value for Arsenic Calculated Using Probabilistic Monitoring Design

Status Network		Designated Use:  Primary Drinking Water Standards 		Units:  Number of wells in list frame

This is a nine-column table.  Column 1 lists the basin, Column 2 lists the target population, Column 3 lists the number of sampleable wells, Column 4 lists the number of inaccessible wells, Column 5 lists the number of samples, Column 6 lists the percent meeting threshold, Column 7 lists the 95% confidence bounds, Column 8 lists the percent not meeting threshold, and Column 9 lists the assessment period.

1 No confined aquifer wells in basin

2 N/A = Not applicable

| Basin | Target Population | Sampleable | Inaccessible | Number of Samples | % Meeting Threshold | 95% Confidence Bounds (% Meeting Threshold) | % Not Meeting Threshold | Assessment Period |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NWFWMD1 Ochlockonee–St. Marks | 1,600 | 1,024 | 576 | 30 | 92.9 | 84.5-100.0 | 7.1 | Jan-Mar 2004 |
| SFWMD1 Everglades–West Coast | 80 | 63 | 17 | 30 | 96.5 | 90.7-100.0 | 3.5 | Jan-Mar 2004 |
| SFWMD6 Lake Okeechobee | 8 | 4 | 4 | 4 | 100.0 | 100.0 | 0.0 | Jan-Mar 2004 |
| SJRWMD1 Ocklawaha | 167 | 116 | 51 | 30 | 100.0 | 100.0 | 0.0 | Jan-Mar 2004 |
| SRWMD1 Suwannee | 376 | 260 | 116 | 30 | 100.0 | 100.0 | 0.0 | Jan-Mar 2004 |
| SWFWMD1 Tampa Bay | 113 | 72 | 41 | 30 | 96.9 | 91.8-100.0 | 3.1 | Jan-Mar 2004 |
| NWFWMD2 Apalachicola–Chipola | 194 | 89 | 105 | 30 | 100.0 | 100.0 | 0.0 | Jan-Feb 2005 |
| SFWMD2 St Lucie–Loxahatchee | 12 | 9 | 3 | 9 | 100.0 | 100.0 | 0.0 | Jan-Feb 2005 |
| SFWMD7 Charlotte Harbor | 54 | 33 | 21 | 30 | 100.0 | 100.0 | 0.0 | Feb-Mar 2005 |
| SJRWMD2 Lower St. Johns | 259 | 161 | 98 | 21 | 100.0 | 100.0 | 0.0 | Jan-Mar 2005 |
| SJRWMD6 Middle St. Johns | 304 | 188 | 116 | 30 | 100.0 | 100.0 | 0.0 | Jan-Mar 2005 |
| SWFWMD2 Tampa Bay Tributaries | 172 | 57 | 115 | 19 | 100.0 | 100.0 | 0.0 | Jan-Feb 2005 |
| NWFWMD3 Choctawhatchee– St. Andrews | 1,823 | 820 | 1,003 | 30 | 100.0 | 100.0 | 0.0 | Jan-Feb 2006 |
| SFWMD3 Caloosahatchee | 57 | 43 | 14 | 30 | 100.0 | 100.0 | 0.0 | Jan-Feb 2006 |
| SFWMD8 Lake Worth Lagoon– Palm Beach Coast1 | 0 | 0 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD3 Upper St. Johns | 54 | 31 | 23 | 28 | 100.0 | 100.0 | 0.0 | Jan-Feb 2006 |
| SWFWMD3 Sarasota Bay–Peace–Myakka | 460 | 179 | 281 | 30 | 100.0 | 100.0 | 0.0 | Jan-Mar 2006 |
| NWFWMD4 Pensacola | 149 | 75 | 74 | 30 | 100.0 | 100.0 | 0.0 | Jan-Feb 2007 |
| SFWMD4 Fisheating Creek | 15 | 6 | 9 | 6 | 100.0 | 100.0 | 0.0 | Jan-Feb 2007 |
| SFWMD9 Kissimmee River | 131 | 56 | 75 | 29 | 100.0 | 100.0 | 0.0 | Jan-Mar 2007 |
| SFWMD10 Southeast Coast– Biscayne Bay | 2 | 0 | 2 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD4 Nassau–St. Marys | 78 | 28 | 50 | 28 | 100.0 | 100.0 | 0.0 | Jan-Mar 2007 |
| SWFWMD4 Withlacoochee | 48 | 23 | 25 | 23 | 100.0 | 100.0 | 0.0 | Feb-Mar 2007 |
| NWFWMD5 Perdido | 1 | 0 | 1 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SFWMD5 Everglades | 4 | 4 | 0 | 4 | 100.0 | 100.0 | 0.0 | Feb 2008 |
| SFWMD11 Florida Keys1 | 0 | 0 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD5 Indian River Lagoon | 45 | 21 | 24 | 21 | 100.0 | 100.0 | 0.0 | Jan-Mar 2008 |
| SJRWMD7 Upper East Coast | 62 | 26 | 36 | 26 | 100.0 | 100.0 | 0.0 | Feb-Mar 2008 |
| SWFWMD5 Springs Coast | 87 | 51 | 36 | 30 | 86.7 | 77.2-96.2 | 13.3 | Jan-Mar 2008 |
| Summary of All Basins | 6,355 | 3,439 | 2,916 | 608 | 98.5 | 96.9-100.0 | 1.5 | 2004–08 |

Figure C.2.  Summary of Cadmium Assessment for Confined Aquifers

Table C.2.  Percentage of Confined Aquifers Meeting Threshold Value for Cadmium Calculated Using Probabilistic Monitoring Design

Status Network		Designated Use:  Primary Drinking Water Standards 		Units:  Number of wells in list frame

This is a nine-column table.  Column 1 lists the basin, Column 2 lists the target population, Column 3 lists the number of sampleable wells, Column 4 lists the number of inaccessible wells, Column 5 lists the number of samples, Column 6 lists the percent meeting threshold, Column 7 lists the 95% confidence bounds, Column 8 lists the percent not meeting threshold, and Column 9 lists the assessment period.

1 No confined aquifer wells in basin

2 N/A = Not applicable

| Basin | Target Population | Sampleable | Inaccessible | Number of Samples | % Meeting Threshold | 95% Confidence Bounds (% Meeting Threshold) | % Not Meeting Threshold | Assessment Period |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NWFWMD1 Ochlockonee–St. Marks | 1,600 | 1,024 | 576 | 30 | 100.0 | 100.0 | 0.0 | Jan-Mar 2004 |
| SFWMD1 Everglades–West Coast | 80 | 63 | 17 | 30 | 100.0 | 100.0 | 0.0 | Jan-Mar 2004 |
| SFWMD6 Lake Okeechobee | 8 | 4 | 4 | 4 | 100.0 | 100.0 | 0.0 | Jan-Mar 2004 |
| SJRWMD1 Ocklawaha | 167 | 116 | 51 | 30 | 100.0 | 100.0 | 0.0 | Jan-Mar 2004 |
| SRWMD1 Suwannee | 376 | 260 | 116 | 30 | 100.0 | 100.0 | 0.0 | Jan-Mar 2004 |
| SWFWMD1 Tampa Bay | 113 | 72 | 41 | 30 | 100.0 | 100.0 | 0.0 | Jan-Mar 2004 |
| NWFWMD2 Apalachicola–Chipola | 194 | 89 | 105 | 30 | 100.0 | 100.0 | 0.0 | Jan-Feb 2005 |
| SFWMD2 St Lucie–Loxahatchee | 12 | 9 | 3 | 9 | 100.0 | 100.0 | 0.0 | Jan-Feb 2005 |
| SFWMD7 Charlotte Harbor | 54 | 33 | 21 | 30 | 100.0 | 100.0 | 0.0 | Feb-Mar 2005 |
| SJRWMD2 Lower St. Johns | 259 | 161 | 98 | 30 | 100.0 | 100.0 | 0.0 | Jan-Mar 2005 |
| SJRWMD6 Middle St. Johns | 304 | 188 | 116 | 30 | 100.0 | 100.0 | 0.0 | Jan-Mar 2005 |
| SWFWMD2 Tampa Bay Tributaries | 172 | 57 | 115 | 30 | 100.0 | 100.0 | 0.0 | Jan-Feb 2005 |
| NWFWMD3 Choctawhatchee- St. Andrews | 1,823 | 820 | 1,003 | 30 | 100.0 | 100.0 | 0.0 | Jan-Feb 2006 |
| SFWMD3 Caloosahatchee | 57 | 43 | 14 | 30 | 100.0 | 100.0 | 0.0 | Jan-Feb 2006 |
| SFWMD8 Lake Worth Lagoon– Palm Beach Coast1 | 0 | 0 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD3 Upper St. Johns | 54 | 31 | 23 | 28 | 100.0 | 100.0 | 0.0 | Jan-Feb 2006 |
| SWFWMD3 Sarasota Bay–Peace–Myakka | 460 | 179 | 281 | 30 | 100.0 | 100.0 | 0.0 | Jan-Mar 2006 |
| NWFWMD4 Pensacola | 149 | 75 | 74 | 30 | 100.0 | 100.0 | 0.0 | Jan-Feb 2007 |
| SFWMD4 Fisheating Creek | 15 | 6 | 9 | 6 | 100.0 | 100.0 | 0.0 | Jan-Feb 2007 |
| SFWMD9 Kissimmee River | 131 | 56 | 75 | 29 | 100.0 | 100.0 | 0.0 | Jan-Mar 2007 |
| SFWMD10 Southeast Coast– Biscayne Bay | 2 | 0 | 2 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD4 Nassau–St. Marys | 78 | 28 | 50 | 28 | 100.0 | 100.0 | 0.0 | Jan-Mar 2007 |
| SWFWMD4 Withlacoochee | 48 | 23 | 25 | 23 | 100.0 | 100.0 | 0.0 | Feb-Mar 2007 |
| NWFWMD5 Perdido | 1 | 0 | 1 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SFWMD5 Everglades | 4 | 4 | 0 | 4 | 100.0 | 100.0 | 0.0 | Feb 2008 |
| SFWMD11 Florida Keys1 | 0 | 0 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD5 Indian River Lagoon | 45 | 21 | 24 | 21 | 100.0 | 100.0 | 0.0 | Jan-Mar 2008 |
| SJRWMD7 Upper East Coast | 62 | 26 | 36 | 26 | 100.0 | 100.0 | 0.0 | Feb-Mar 2008 |
| SWFWMD5 Springs Coast | 87 | 51 | 36 | 30 | 100.0 | 100.0 | 0.0 | Jan-Mar 2008 |
| Summary of All Basins | 6,355 | 3,439 | 2,916 | 628 | 100.0 | 100.0 | 0.0 | 2004–08 |

Figure C.3.  Summary of Chromium Assessment for Confined Aquifers

Table C.3.  Percentage of Confined Aquifers Meeting Threshold Value for Chromium Calculated Using Probabilistic Monitoring Design

Status Network		Designated Use:  Primary Drinking Water Standards 		Units:  Number of wells in list frame

This is a nine-column table.  Column 1 lists the basin, Column 2 lists the target population, Column 3 lists the number of sampleable wells, Column 4 lists the number of inaccessible wells, Column 5 lists the number of samples, Column 6 lists the percent meeting threshold, Column 7 lists the 95% confidence bounds, Column 8 lists the percent not meeting threshold, and Column 9 lists the assessment period.

1 No confined aquifer wells in basin

2 N/A = Not applicable

| Basin | Target Population | Sampleable | Inaccessible | Number of Samples | % Meeting Threshold | 95% Confidence Bounds (% Meeting Threshold) | % Not Meeting Threshold | Assessment Period |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NWFWMD1 Ochlockonee–St. Marks | 1,600 | 1,024 | 576 | 30 | 100.0 | 100.0 | 0.0 | Jan-Mar 2004 |
| SFWMD1 Everglades–West Coast | 80 | 63 | 17 | 30 | 100.0 | 100.0 | 0.0 | Jan-Mar 2004 |
| SFWMD6 Lake Okeechobee | 8 | 4 | 4 | 4 | 100.0 | 100.0 | 0.0 | Jan-Mar 2004 |
| SJRWMD1 Ocklawaha | 167 | 116 | 51 | 30 | 100.0 | 100.0 | 0.0 | Jan-Mar 2004 |
| SRWMD1 Suwannee | 376 | 260 | 116 | 30 | 100.0 | 100.0 | 0.0 | Jan-Mar 2004 |
| SWFWMD1 Tampa Bay | 113 | 72 | 41 | 30 | 100.0 | 100.0 | 0.0 | Jan-Mar 2004 |
| NWFWMD2 Apalachicola–Chipola | 194 | 89 | 105 | 30 | 100.0 | 100.0 | 0.0 | Jan-Feb 2005 |
| SFWMD2 St Lucie–Loxahatchee | 12 | 9 | 3 | 9 | 100.0 | 100.0 | 0.0 | Jan-Feb 2005 |
| SFWMD7 Charlotte Harbor | 54 | 33 | 21 | 30 | 100.0 | 100.0 | 0.0 | Feb-Mar 2005 |
| SJRWMD2 Lower St. Johns | 259 | 161 | 98 | 30 | 100.0 | 100.0 | 0.0 | Jan-Mar 2005 |
| SJRWMD6 Middle St. Johns | 304 | 188 | 116 | 30 | 100.0 | 100.0 | 0.0 | Jan-Mar 2005 |
| SWFWMD2 Tampa Bay Tributaries | 172 | 57 | 115 | 30 | 100.0 | 100.0 | 0.0 | Jan-Feb 2005 |
| NWFWMD3 Choctawhatchee– St. Andrews | 1,823 | 820 | 1,003 | 30 | 100.0 | 100.0 | 0.0 | Jan-Feb 2006 |
| SFWMD3 Caloosahatchee | 57 | 43 | 14 | 30 | 100.0 | 100.0 | 0.0 | Jan-Feb 2006 |
| SFWMD8 Lake Worth Lagoon- Palm Beach Coast1 | 0 | 0 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD3 Upper St. Johns | 54 | 31 | 23 | 28 | 100.0 | 100.0 | 0.0 | Jan-Feb 2006 |
| SWFWMD3 Sarasota Bay–Peace–Myakka | 460 | 179 | 281 | 30 | 100.0 | 100.0 | 0.0 | Jan-Mar 2006 |
| NWFWMD4 Pensacola | 149 | 75 | 74 | 30 | 100.0 | 100.0 | 0.0 | Jan-Feb 2007 |
| SFWMD4 Fisheating Creek | 15 | 6 | 9 | 6 | 100.0 | 100.0 | 0.0 | Jan-Feb 2007 |
| SFWMD9 Kissimmee River | 131 | 56 | 75 | 29 | 100.0 | 100.0 | 0.0 | Jan-Mar 2007 |
| SFWMD10 Southeast Coast– Biscayne Bay | 2 | 0 | 2 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD4 Nassau–St. Marys | 78 | 28 | 50 | 28 | 100.0 | 100.0 | 0.0 | Jan-Mar 2007 |
| SWFWMD4 Withlacoochee | 48 | 23 | 25 | 23 | 100.0 | 100.0 | 0.0 | Feb-Mar 2007 |
| NWFWMD5 Perdido | 1 | 0 | 1 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SFWMD5 Everglades | 4 | 4 | 0 | 4 | 100.0 | 100.0 | 0.0 | Feb 2008 |
| SFWMD11 Florida Keys1 | 0 | 0 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD5 Indian River Lagoon | 45 | 21 | 24 | 21 | 100.0 | 100.0 | 0.0 | Jan-Mar 2008 |
| SJRWMD7 Upper East Coast | 62 | 26 | 36 | 26 | 100.0 | 100.0 | 0.0 | Feb-Mar 2008 |
| SWFWMD5 Springs Coast | 87 | 51 | 36 | 30 | 100.0 | 100.0 | 0.0 | Jan-Mar 2008 |
| Summary of All Basins | 6,355 | 3,439 | 2,916 | 628 | 100.0 | 100.0 | 0.0 | 2004–08 |

Figure C.4.  Summary of Fluoride Assessment for Confined Aquifers

Table C.4.  Percentage of Confined Aquifers Meeting Threshold Value for Fluoride Calculated Using Probabilistic Monitoring Design

Status Network		Designated Use:  Primary Drinking Water Standards 		Units:  Number of wells in list frame

This is a nine-column table.  Column 1 lists the basin, Column 2 lists the target population, Column 3 lists the number of sampleable wells, Column 4 lists the number of inaccessible wells, Column 5 lists the number of samples, Column 6 lists the percent meeting threshold, Column 7 lists the 95% confidence bounds, Column 8 lists the percent not meeting threshold, and Column 9 lists the assessment period.

1 No confined aquifer wells in basin

2 N/A = Not applicable

| Basin | Target Population | Sampleable | Inaccessible | Number of Samples | % Meeting Threshold | 95% Confidence Bounds (% Meeting Threshold) | % Not Meeting Threshold | Assessment Period |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NWFWMD1 Ochlockonee–St. Marks | 1,600 | 1,024 | 576 | 30 | 100.0 | 100.0 | 0.0 | Jan-Mar 2004 |
| SFWMD1 Everglades–West Coast | 80 | 63 | 17 | 30 | 100.0 | 100.0 | 0.0 | Jan-Mar 2004 |
| SFWMD6 Lake Okeechobee | 8 | 4 | 4 | 4 | 100.0 | 100.0 | 0.0 | Jan-Mar 2004 |
| SJRWMD1 Ocklawaha | 167 | 116 | 51 | 30 | 100.0 | 100.0 | 0.0 | Jan-Mar 2004 |
| SRWMD1 Suwannee | 376 | 260 | 116 | 30 | 100.0 | 100.0 | 0.0 | Jan-Mar 2004 |
| SWFWMD1 Tampa Bay | 113 | 72 | 41 | 30 | 97.6 | 93.6-100.0 | 2.4 | Jan-Mar 2004 |
| NWFWMD2 Apalachicola–Chipola | 194 | 89 | 105 | 30 | 100.0 | 100.0 | 0.0 | Jan-Feb 2005 |
| SFWMD2 St Lucie–Loxahatchee | 12 | 9 | 3 | 9 | 100.0 | 100.0 | 0.0 | Jan-Feb 2005 |
| SFWMD7 Charlotte Harbor | 54 | 33 | 21 | 29 | 100.0 | 100.0 | 0.0 | Feb-Mar 2005 |
| SJRWMD2 Lower St. Johns | 259 | 161 | 98 | 30 | 100.0 | 100.0 | 0.0 | Jan-Mar 2005 |
| SJRWMD6 Middle St. Johns | 304 | 188 | 116 | 30 | 100.0 | 100.0 | 0.0 | Jan-Mar 2005 |
| SWFWMD2 Tampa Bay Tributaries | 172 | 57 | 115 | 30 | 100.0 | 100.0 | 0.0 | Jan-Feb 2005 |
| NWFWMD3 Choctawhatchee– St. Andrews | 1,823 | 820 | 1,003 | 30 | 100.0 | 100.0 | 0.0 | Jan-Feb 2006 |
| SFWMD3 Caloosahatchee | 57 | 43 | 14 | 30 | 100.0 | 100.0 | 0.0 | Jan-Feb 2006 |
| SFWMD8 Lake Worth Lagoon– Palm Beach Coast1 | 0 | 0 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD3 Upper St. Johns | 54 | 31 | 23 | 28 | 100.0 | 100.0 | 0.0 | Jan-Feb 2006 |
| SWFWMD3 Sarasota Bay–Peace–Myakka | 460 | 179 | 281 | 30 | 100.0 | 100.0 | 0.0 | Jan-Mar 2006 |
| NWFWMD4 Pensacola | 149 | 75 | 74 | 30 | 100.0 | 100.0 | 0.0 | Jan-Feb 2007 |
| SFWMD4 Fisheating Creek | 15 | 6 | 9 | 6 | 100.0 | 100.0 | 0.0 | Jan-Feb 2007 |
| SFWMD9 Kissimmee River | 131 | 56 | 75 | 29 | 100.0 | 100.0 | 0.0 | Jan-Mar 2007 |
| SFWMD10 Southeast Coast– Biscayne Bay | 2 | 0 | 2 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD4 Nassau–St. Marys | 78 | 28 | 50 | 28 | 100.0 | 100.0 | 0.0 | Jan-Mar 2007 |
| SWFWMD4 Withlacoochee | 48 | 23 | 25 | 23 | 100.0 | 100.0 | 0.0 | Feb-Mar 2007 |
| NWFWMD5 Perdido | 1 | 0 | 1 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SFWMD5 Everglades | 4 | 4 | 0 | 4 | 100.0 | 100.0 | 0.0 | Feb 2008 |
| SFWMD11 Florida Keys1 | 0 | 0 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD5 Indian River Lagoon | 45 | 21 | 24 | 21 | 100.0 | 100.0 | 0.0 | Jan-Mar 2008 |
| SJRWMD7 Upper East Coast | 62 | 26 | 36 | 26 | 100.0 | 100.0 | 0.0 | Feb-Mar 2008 |
| SWFWMD5 Springs Coast | 87 | 51 | 36 | 30 | 100.0 | 100.0 | 0.0 | Jan-Mar 2008 |
| Summary of All Basins | 6,355 | 3,439 | 2,916 | 627 | 99.97 | 99.9-100.0 | 0.03 | 2004–08 |

Figure C.5.  Summary of Lead Assessment for Confined Aquifers

Table C.5.  Percentage of Confined Aquifers Meeting Threshold Value for Lead Calculated Using Probabilistic Monitoring Design

Status Network		Designated Use:  Primary Drinking Water Standards 		Units:  Number of wells in list frame

This is a nine-column table.  Column 1 lists the basin, Column 2 lists the target population, Column 3 lists the number of sampleable wells, Column 4 lists the number of inaccessible wells, Column 5 lists the number of samples, Column 6 lists the percent meeting threshold, Column 7 lists the 95% confidence bounds, Column 8 lists the percent not meeting threshold, and Column 9 lists the assessment period.

1 No confined aquifer wells in basin

2 N/A = Not applicable

| Basin | Target Population | Sampleable | Inaccessible | Number of Samples | % Meeting Threshold | 95% Confidence Bounds (% Meeting Threshold) | % Not Meeting Threshold | Assessment Period |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NWFWMD1 Ochlockonee–St. Marks | 1,600 | 1,024 | 576 | 30 | 100.0 | 100.0 | 0.0 | Jan-Mar 2004 |
| SFWMD1 Everglades–West Coast | 80 | 63 | 17 | 30 | 74.5 | 61.8-87.3 | 25.45 | Jan-Mar 2004 |
| SFWMD6 Lake Okeechobee | 8 | 4 | 4 | 4 | 100.0 | 100.0 | 0.0 | Jan-Mar 2004 |
| SJRWMD1 Ocklawaha | 167 | 116 | 51 | 30 | 100.0 | 100.0 | 0.0 | Jan-Mar 2004 |
| SRWMD1 Suwannee | 376 | 260 | 116 | 30 | 95.1 | 86.7-100.0 | 4.9 | Jan-Mar 2004 |
| SWFWMD1 Tampa Bay | 113 | 72 | 41 | 24 | 100.0 | 100.0 | 0.0 | Jan-Mar 2004 |
| NWFWMD2 Apalachicola–Chipola | 194 | 89 | 105 | 30 | 100.0 | 100.0 | 0.0 | Jan-Feb 2005 |
| SFWMD2 St Lucie–Loxahatchee | 12 | 9 | 3 | 9 | 100.0 | 100.0 | 0.0 | Jan-Feb 2005 |
| SFWMD7 Charlotte Harbor | 54 | 33 | 21 | 30 | 96.7 | 91.4-100.0 | 3.3 | Feb-Mar 2005 |
| SJRWMD2 Lower St. Johns | 259 | 161 | 98 | 30 | 100.0 | 100.0 | 0.0 | Jan-Mar 2005 |
| SJRWMD6 Middle St. Johns | 304 | 188 | 116 | 30 | 97.0 | 91.9-100.0 | 2.8 | Jan-Mar 2005 |
| SWFWMD2 Tampa Bay Tributaries | 172 | 57 | 115 | 26 | 100.0 | 100.0 | 0.0 | Jan-Feb 2005 |
| NWFWMD3 Choctawhatchee– St. Andrews | 1,823 | 820 | 1,003 | 30 | 100.0 | 100.0 | 0.0 | Jan-Feb 2006 |
| SFWMD3 Caloosahatchee | 57 | 43 | 14 | 30 | 100.0 | 100.0 | 0.0 | Jan-Feb 2006 |
| SFWMD8 Lake Worth Lagoon– Palm Beach Coast1 | 0 | 0 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD3 Upper St. Johns | 54 | 31 | 23 | 28 | 100.0 | 100.0 | 0.0 | Jan-Feb 2006 |
| SWFWMD3 Sarasota Bay–Peace–Myakka | 460 | 179 | 281 | 30 | 100.0 | 100.0 | 0.0 | Jan-Mar 2006 |
| NWFWMD4 Pensacola | 149 | 75 | 74 | 30 | 100.0 | 100.0 | 0.0 | Jan-Feb 2007 |
| SFWMD4 Fisheating Creek | 15 | 6 | 9 | 6 | 100.0 | 100.0 | 0.0 | Jan-Feb 2007 |
| SFWMD9 Kissimmee River | 131 | 56 | 75 | 29 | 100.0 | 100.0 | 0.0 | Jan-Mar 2007 |
| SFWMD10 Southeast Coast– Biscayne Bay | 2 | 0 | 2 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD4 Nassau–St. Marys | 78 | 28 | 50 | 28 | 100.0 | 100.0 | 0.0 | Jan-Mar 2007 |
| SWFWMD4 Withlacoochee | 48 | 23 | 25 | 22 | 100.0 | 100.0 | 0.0 | Feb-Mar 2007 |
| NWFWMD5 Perdido | 1 | 0 | 1 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SFWMD5 Everglades | 4 | 4 | 0 | 4 | 100.0 | 100.0 | 0.0 | Feb 2008 |
| SFWMD11 Florida Keys1 | 0 | 0 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD5 Indian River Lagoon | 45 | 21 | 24 | 21 | 100.0 | 100.0 | 0.0 | Jan-Mar 2008 |
| SJRWMD7 Upper East Coast | 62 | 26 | 36 | 26 | 100.0 | 100.0 | 0.0 | Feb-Mar 2008 |
| SWFWMD5 Springs Coast | 87 | 51 | 36 | 30 | 100.0 | 100.0 | 0.0 | Jan-Mar 2008 |
| Summary of All Basins | 6,355 | 3,439 | 2,916 | 615 | 99.4 | 99.0-99.8 | 0.6 | 2004–08 |

Figure C.6.  Summary of Nitrate + Nitrite Assessment for Confined Aquifers

Table C.6.  Percentage of Confined Aquifers Meeting Threshold Value for Nitrate + Nitrite Calculated Using Probabilistic Monitoring Design

Status Network		Designated Use:  Primary Drinking Water Standards 		Units:  Number of wells in list frame

This is a nine-column table.  Column 1 lists the basin, Column 2 lists the target population, Column 3 lists the number of sampleable wells, Column 4 lists the number of inaccessible wells, Column 5 lists the number of samples, Column 6 lists the percent meeting threshold, Column 7 lists the 95% confidence bounds, Column 8 lists the percent not meeting threshold, and Column 9 lists the assessment period.

1 No confined aquifer wells in basin

2 N/A = Not applicable

| Basin | Target Population | Sampleable | Inaccessible | Number of Samples | % Meeting Threshold | 95% Confidence Bounds (% Meeting Threshold) | % Not Meeting Threshold | Assessment Period |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NWFWMD1 Ochlockonee–St. Marks | 1,600 | 1,024 | 576 | 21 | 100.0 | 100.0 | 0.0 | Jan-Mar 2004 |
| SFWMD1 Everglades–West Coast | 80 | 63 | 17 | 30 | 100.0 | 100.0 | 0.0 | Jan-Mar 2004 |
| SFWMD6 Lake Okeechobee | 8 | 4 | 4 | 4 | 100.0 | 100.0 | 0.0 | Jan-Mar 2004 |
| SJRWMD1 Ocklawaha | 167 | 116 | 51 | 30 | 100.0 | 100.0 | 0.0 | Jan-Mar 2004 |
| SRWMD1 Suwannee | 376 | 260 | 116 | 30 | 100.0 | 100.0 | 0.0 | Jan-Mar 2004 |
| SWFWMD1 Tampa Bay | 113 | 72 | 41 | 29 | 100.0 | 100.0 | 0.0 | Jan-Mar 2004 |
| NWFWMD2 Apalachicola–Chipola | 194 | 89 | 105 | 30 | 100.0 | 100.0 | 0.0 | Jan-Feb 2005 |
| SFWMD2 St Lucie–Loxahatchee | 12 | 9 | 3 | 9 | 100.0 | 100.0 | 0.0 | Jan-Feb 2005 |
| SFWMD7 Charlotte Harbor | 54 | 33 | 21 | 29 | 100.0 | 100.0 | 0.0 | Feb-Mar 2005 |
| SJRWMD2 Lower St. Johns | 259 | 161 | 98 | 30 | 100.0 | 100.0 | 0.0 | Jan-Mar 2005 |
| SJRWMD6 Middle St. Johns | 304 | 188 | 116 | 30 | 100.0 | 100.0 | 0.0 | Jan-Mar 2005 |
| SWFWMD2 Tampa Bay Tributaries | 172 | 57 | 115 | 28 | 100.0 | 100.0 | 0.0 | Jan-Feb 2005 |
| NWFWMD3 Choctawhatchee– St. Andrews | 1,823 | 820 | 1,003 | 30 | 100.0 | 100.0 | 0.0 | Jan-Feb 2006 |
| SFWMD3 Caloosahatchee | 57 | 43 | 14 | 30 | 100.0 | 100.0 | 0.0 | Jan-Feb 2006 |
| SFWMD8 Lake Worth Lagoon– Palm Beach Coast1 | 0 | 0 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD3 Upper St. Johns | 54 | 31 | 23 | 28 | 100.0 | 100.0 | 0.0 | Jan-Feb 2006 |
| SWFWMD3 Sarasota Bay–Peace–Myakka | 460 | 179 | 281 | 30 | 100.0 | 100.0 | 0.0 | Jan-Mar 2006 |
| NWFWMD4 Pensacola | 149 | 75 | 74 | 30 | 100.0 | 100.0 | 0.0 | Jan-Feb 2007 |
| SFWMD4 Fisheating Creek | 15 | 6 | 9 | 6 | 100.0 | 100.0 | 0.0 | Jan-Feb 2007 |
| SFWMD9 Kissimmee River | 131 | 56 | 75 | 27 | 100.0 | 100.0 | 0.0 | Jan-Mar 2007 |
| SFWMD10 Southeast Coast– Biscayne Bay | 2 | 0 | 2 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD4 Nassau–St. Marys | 78 | 28 | 50 | 28 | 100.0 | 100.0 | 0.0 | Jan-Mar 2007 |
| SWFWMD4 Withlacoochee | 48 | 23 | 25 | 23 | 100.0 | 100.0 | 0.0 | Feb-Mar 2007 |
| NWFWMD5 Perdido | 1 | 0 | 1 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SFWMD5 Everglades | 4 | 4 | 0 | 4 | 100.0 | 100.0 | 0.0 | Feb 2008 |
| SFWMD11 Florida Keys1 | 0 | 0 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD5 Indian River Lagoon | 45 | 21 | 24 | 17 | 100.0 | 100.0 | 0.0 | Jan-Mar 2008 |
| SJRWMD7 Upper East Coast | 62 | 26 | 36 | 26 | 100.0 | 100.0 | 0.0 | Feb-Mar 2008 |
| SWFWMD5 Springs Coast | 87 | 51 | 36 | 30 | 100.0 | 100.0 | 0.0 | Jan-Mar 2008 |
| Summary of All Basins | 6,355 | 3,439 | 2,916 | 609 | 100.0 | 100.0 | 0.0 | 2004–08 |

Figure C.7.  Summary of Sodium Assessment for Confined Aquifers

Table C.7.  Percentage of Confined Aquifers Meeting Threshold Value for Sodium Calculated Using Probabilistic Monitoring Design

Status Network		Designated Use:  Primary Drinking Water Standards 		Units:  Number of wells in list frame

This is a nine-column table.  Column 1 lists the basin, Column 2 lists the target population, Column 3 lists the number of sampleable wells, Column 4 lists the number of inaccessible wells, Column 5 lists the number of samples, Column 6 lists the percent meeting threshold, Column 7 lists the 95% confidence bounds, Column 8 lists the percent not meeting threshold, and Column 9 lists the assessment period.

1 No confined aquifer wells in basin

2 N/A = Not applicable

| Basin | Target Population | Sampleable | Inaccessible | Number of Samples | % Meeting Threshold | 95% Confidence Bounds (% Meeting Threshold) | % Not Meeting Threshold | Assessment Period |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NWFWMD1 Ochlockonee–St. Marks | 1,600 | 1,024 | 576 | 30 | 97.9 | 94.3-100.0 | 2.1 | Jan-Mar 2004 |
| SFWMD1 Everglades–West Coast | 80 | 63 | 17 | 30 | 59.5 | 43.0-76.0 | 40.5 | Jan-Mar 2004 |
| SFWMD6 Lake Okeechobee | 8 | 4 | 4 | 4 | 23.3 | 0.0-62.1 | 76.7 | Jan-Mar 2004 |
| SJRWMD1 Ocklawaha | 167 | 116 | 51 | 30 | 96.9 | 91.7-100.0 | 3.1 | Jan-Mar 2004 |
| SRWMD1 Suwannee | 376 | 260 | 116 | 30 | 100.0 | 100.0 | 0.0 | Jan-Mar 2004 |
| SWFWMD1 Tampa Bay | 113 | 72 | 41 | 30 | 72.8 | 59.3-86.3 | 27.2 | Jan-Mar 2004 |
| NWFWMD2 Apalachicola–Chipola | 194 | 89 | 105 | 30 | 100.0 | 100.0 | 0.0 | Jan-Feb 2005 |
| SFWMD2 St Lucie–Loxahatchee | 12 | 9 | 3 | 9 | 44.4 | 20.3-68.6 | 55.6 | Jan-Feb 2005 |
| SFWMD7 Charlotte Harbor | 54 | 33 | 21 | 30 | 33.3 | 22.0-44.7 | 66.7 | Feb-Mar 2005 |
| SJRWMD2 Lower St. Johns | 259 | 161 | 98 | 30 | 96.4 | 90.3-100.0 | 3.6 | Jan-Mar 2005 |
| SJRWMD6 Middle St. Johns | 304 | 188 | 116 | 30 | 87.3 | 78.1-96.6 | 12.7 | Jan-Mar 2005 |
| SWFWMD2 Tampa Bay Tributaries | 172 | 57 | 115 | 30 | 93.2 | 86.9-99.5 | 6.8 | Jan-Feb 2005 |
| NWFWMD3 Choctawhatchee– St. Andrews | 1,823 | 820 | 1,003 | 30 | 97.6 | 93.5-100.0 | 2.4 | Jan-Feb 2006 |
| SFWMD3 Caloosahatchee | 57 | 43 | 14 | 30 | 50.0 | 35.6-64.4 | 50.0 | Jan-Feb 2006 |
| SFWMD8 Lake Worth Lagoon– Palm Beach Coast1 | 0 | 0 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD3 Upper St. Johns | 54 | 31 | 23 | 28 | 78.6 | 71.4-85.8 | 21.4 | Jan-Feb 2006 |
| SWFWMD3 Sarasota Bay–Peace–Myakka | 460 | 179 | 281 | 30 | 87.9 | 79.7-96.0 | 12.1 | Jan-Mar 2006 |
| NWFWMD4 Pensacola | 149 | 75 | 74 | 30 | 100.0 | 100.0 | 0.0 | Jan-Feb 2007 |
| SFWMD4 Fisheating Creek | 15 | 6 | 9 | 6 | 83.3 | 57.0-100.0 | 16.7 | Jan-Feb 2007 |
| SFWMD9 Kissimmee River | 131 | 56 | 75 | 29 | 97.9 | 94.4-100.0 | 2.1 | Jan-Mar 2007 |
| SFWMD10 Southeast Coast– Biscayne Bay | 2 | 0 | 2 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD4 Nassau–St. Marys | 78 | 28 | 50 | 28 | 96.4 | 90.6-100.0 | 3.6 | Jan-Mar 2007 |
| SWFWMD4 Withlacoochee | 48 | 23 | 25 | 23 | 100.0 | 100.0 | 0.0 | Feb-Mar 2007 |
| NWFWMD5 Perdido | 1 | 0 | 1 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SFWMD5 Everglades | 4 | 4 | 0 | 4 | 100.0 | 100.0 | 0.0 | Feb 2008 |
| SFWMD11 Florida Keys1 | 0 | 0 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD5 Indian River Lagoon | 45 | 21 | 24 | 21 | 57.1 | 42.0-72.3 | 42.9 | Jan-Mar 2008 |
| SJRWMD7 Upper East Coast | 62 | 26 | 36 | 26 | 57.7 | 47.0-68.4 | 42.3 | Feb-Mar 2008 |
| SWFWMD5 Springs Coast | 87 | 51 | 36 | 30 | 73.3 | 59.9-86.7 | 26.7 | Jan-Mar 2008 |
| Summary of All Basins | 6,355 | 3,439 | 2,916 | 628 | 94.9 | 93.7-96.2 | 5.1 | 2004–08 |

Figure C.8.  Summary of Fecal Coliform Assessment for Confined Aquifers

Table C.8.  Percentage of Confined Aquifers Meeting Threshold Value for Fecal Coliform Calculated Using Probabilistic Monitoring Design

Status Network		Designated Use:  Primary Drinking Water Standards 		Units:  Number of wells in list frame

This is a nine-column table.  Column 1 lists the basin, Column 2 lists the target population, Column 3 lists the number of sampleable wells, Column 4 lists the number of inaccessible wells, Column 5 lists the number of samples, Column 6 lists the percent meeting threshold, Column 7 lists the 95% confidence bounds, Column 8 lists the percent not meeting threshold, and Column 9 lists the assessment period.

1 No confined aquifer wells in basin

2 N/A = Not applicable

| Basin | Target Population | Sampleable | Inaccessible | Number of Samples | % Meeting Threshold | 95% Confidence Bounds (% Meeting Threshold) | % Not Meeting Threshold | Assessment Period |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NWFWMD1 Ochlockonee–St. Marks | 1,600 | 1,024 | 576 | 30 | 100.0 | 100.0 | 0.0 | Jan-Mar 2004 |
| SFWMD1 Everglades–West Coast | 80 | 63 | 17 | 30 | 93.0 | 84.5-100.0 | 7.0 | Jan-Mar 2004 |
| SFWMD6 Lake Okeechobee | 8 | 4 | 4 | 4 | 100.0 | 100.0 | 0.0 | Jan-Mar 2004 |
| SJRWMD1 Ocklawaha | 167 | 116 | 51 | 30 | 96.5 | 90.7-100.0 | 3.5 | Jan-Mar 2004 |
| SRWMD1 Suwannee | 376 | 260 | 116 | 30 | 100.0 | 100.0 | 0.0 | Jan-Mar 2004 |
| SWFWMD1 Tampa Bay | 113 | 72 | 41 | 30 | 92.9 | 84.6-100.0 | 7.1 | Jan-Mar 2004 |
| NWFWMD2 Apalachicola–Chipola | 194 | 89 | 105 | 29 | 97.8 | 94.2-100.0 | 2.2 | Jan-Feb 2005 |
| SFWMD2 St Lucie–Loxahatchee | 12 | 9 | 3 | 9 | 100.0 | 100.0 | 0.0 | Jan-Feb 2005 |
| SFWMD7 Charlotte Harbor | 54 | 33 | 21 | 30 | 100.0 | 100.0 | 0.0 | Feb-Mar 2005 |
| SJRWMD2 Lower St. Johns | 259 | 161 | 98 | 30 | 100.0 | 100.0 | 0.0 | Jan-Mar 2005 |
| SJRWMD6 Middle St. Johns | 304 | 188 | 116 | 30 | 89.4 | 77.2-100.0 | 10.3 | Jan-Mar 2005 |
| SWFWMD2 Tampa Bay Tributaries | 172 | 57 | 115 | 26 | 100.0 | 100.0 | 0.0 | Jan-Feb 2005 |
| NWFWMD3 Choctawhatchee– St. Andrews | 1,823 | 820 | 1,003 | 30 | 100.0 | 100.0 | 0.0 | Jan-Feb 2006 |
| SFWMD3 Caloosahatchee | 57 | 43 | 14 | 30 | 96.7 | 91.4-100.0 | 3.3 | Jan-Feb 2006 |
| SFWMD8 Lake Worth Lagoon– Palm Beach Coast1 | 0 | 0 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD3 Upper St. Johns | 54 | 31 | 23 | 28 | 100.0 | 100.0 | 0.0 | Jan-Feb 2006 |
| SWFWMD3 Sarasota Bay–Peace–Myakka | 460 | 179 | 281 | 30 | 100.0 | 100.0 | 0.0 | Jan-Mar 2006 |
| NWFWMD4 Pensacola | 149 | 75 | 74 | 30 | 100.0 | 100.0 | 0.0 | Jan-Feb 2007 |
| SFWMD4 Fisheating Creek | 15 | 6 | 9 | 6 | 100.0 | 100.0 | 0.0 | Jan-Feb 2007 |
| SFWMD9 Kissimmee River | 131 | 56 | 75 | 28 | 100.0 | 100.0 | 0.0 | Jan-Mar 2007 |
| SFWMD10 Southeast Coast– Biscayne Bay | 2 | 0 | 2 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD4 Nassau–St. Marys | 78 | 28 | 50 | 28 | 100.0 | 100.0 | 0.0 | Jan-Mar 2007 |
| SWFWMD4 Withlacoochee | 48 | 23 | 25 | 23 | 100.0 | 100.0 | 0.0 | Feb-Mar 2007 |
| NWFWMD5 Perdido | 1 | 0 | 1 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SFWMD5 Everglades | 4 | 4 | 0 | 4 | 100.0 | 100.0 | 0.0 | Feb 2008 |
| SFWMD11 Florida Keys1 | 0 | 0 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD5 Indian River Lagoon | 45 | 21 | 24 | 21 | 100.0 | 100.0 | 0.0 | Jan-Mar 2008 |
| SJRWMD7 Upper East Coast | 62 | 26 | 36 | 26 | 100.0 | 100.0 | 0.0 | Feb-Mar 2008 |
| SWFWMD5 Springs Coast | 87 | 51 | 36 | 30 | 93.3 | 87.0-99.7 | 6.7 | Jan-Mar 2008 |
| Summary of All Basins | 6,355 | 3,439 | 2,916 | 622 | 98.7 | 97.5-99.8 | 1.3 | 2004–08 |

Figure C.9.  Summary of Total Coliform Assessment for Confined Aquifers

Table C.9.  Percentage of Confined Aquifers Meeting Threshold Value for Total Coliform Calculated Using Probabilistic Monitoring Design

Status Network		Designated Use:  Primary Drinking Water Standards 		Units:  Number of wells in list frame

This is a nine-column table.  Column 1 lists the basin, Column 2 lists the target population, Column 3 lists the number of sampleable wells, Column 4 lists the number of inaccessible wells, Column 5 lists the number of samples, Column 6 lists the percent meeting threshold, Column 7 lists the 95% confidence bounds, Column 8 lists the percent not meeting threshold, and Column 9 lists the assessment period.

1 No confined aquifer wells in basin

2 N/A = Not applicable

| Basin | Target Population | Sampleable | Inaccessible | Number of Samples | % Meeting Threshold | 95% Confidence Bounds (% Meeting Threshold) | % Not Meeting Threshold | Assessment Period |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NWFWMD1 Ochlockonee–St. Marks | 1,600 | 1,024 | 576 | 30 | 87.4 | 72.2-100.0 | 12.6 | Jan-Mar 2004 |
| SFWMD1 Everglades–West Coast | 80 | 63 | 17 | 28 | 80.8 | 68.7-92.9 | 19.2 | Jan-Mar 2004 |
| SFWMD6 Lake Okeechobee | 8 | 4 | 4 | 4 | 100.0 | 100.0 | 0.0 | Jan-Mar 2004 |
| SJRWMD1 Ocklawaha | 167 | 116 | 51 | 30 | 83.2 | 71.9-94.5 | 16.8 | Jan-Mar 2004 |
| SRWMD1 Suwannee | 376 | 260 | 116 | 30 | 100.0 | 100.0 | 0.0 | Jan-Mar 2004 |
| SWFWMD1 Tampa Bay | 113 | 72 | 41 | 30 | 86.6 | 75.1-98.0 | 13.4 | Jan-Mar 2004 |
| NWFWMD2 Apalachicola–Chipola | 194 | 89 | 105 | 29 | 94.9 | 86.0-100.0 | 5.1 | Jan-Feb 2005 |
| SFWMD2 St Lucie–Loxahatchee | 12 | 9 | 3 | 9 | 88.9 | 70.0-100.0 | 11.1 | Jan-Feb 2005 |
| SFWMD7 Charlotte Harbor | 54 | 33 | 21 | 30 | 96.7 | 90.8-100.0 | 3.3 | Feb-Mar 2005 |
| SJRWMD2 Lower St. Johns | 259 | 161 | 98 | 27 | 89.6 | 78.9-100.0 | 10.6 | Jan-Mar 2005 |
| SJRWMD6 Middle St. Johns | 304 | 188 | 116 | 30 | 77.3 | 61.8-92.8 | 22.7 | Jan-Mar 2005 |
| SWFWMD2 Tampa Bay Tributaries | 172 | 57 | 115 | 27 | 69.6 | 54.2-84.9 | 30.4 | Jan-Feb 2005 |
| NWFWMD3 Choctawhatchee– St. Andrews | 1,823 | 820 | 1,003 | 30 | 92.3 | 83.1-100.0 | 7.7 | Jan-Feb 2006 |
| SFWMD3 Caloosahatchee | 57 | 43 | 14 | 30 | 46.7 | 32.9-60.5 | 53.3 | Jan-Feb 2006 |
| SFWMD8 Lake Worth Lagoon– Palm Beach Coast1 | 0 | 0 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD3 Upper St. Johns | 54 | 31 | 23 | 28 | 96.4 | 90.6-100.0 | 3.6 | Jan-Feb 2006 |
| SWFWMD3 Sarasota Bay–Peace–Myakka | 460 | 179 | 281 | 30 | 78.9 | 65.9-91.9 | 21.1 | Jan-Mar 2006 |
| NWFWMD4 Pensacola | 149 | 75 | 74 | 30 | 92.9 | 84.3-100.0 | 7.1 | Jan-Feb 2007 |
| SFWMD4 Fisheating Creek | 15 | 6 | 9 | 6 | 100.0 | 100.0 | 0.0 | Jan-Feb 2007 |
| SFWMD9 Kissimmee River | 131 | 56 | 75 | 28 | 94.0 | 86.6-100.0 | 6.0 | Jan-Mar 2007 |
| SFWMD10 Southeast Coast– Biscayne Bay | 2 | 0 | 2 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD4 Nassau–St. Marys | 78 | 28 | 50 | 28 | 96.4 | 90.0-100.0 | 3.6 | Jan-Mar 2007 |
| SWFWMD4 Withlacoochee | 48 | 23 | 25 | 23 | 95.6 | 88.1-100.0 | 4.4 | Feb-Mar 2007 |
| NWFWMD5 Perdido | 1 | 0 | 1 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SFWMD5 Everglades | 4 | 4 | 0 | 4 | 100.0 | 100.0 | 0.0 | Feb 2008 |
| SFWMD11 Florida Keys1 | 0 | 0 | 0 | 0 | N/A2 | N/A2 | N/A2 | N/A2 |
| SJRWMD5 Indian River Lagoon | 45 | 21 | 24 | 21 | 90.5 | 79.2-100.0 | 9.5 | Jan-Mar 2008 |
| SJRWMD7 Upper East Coast | 62 | 26 | 36 | 26 | 80.8 | 68.6-92.9 | 19.2 | Feb-Mar 2008 |
| SWFWMD5 Springs Coast | 87 | 51 | 36 | 18 | 77.8 | 61.9-93.6 | 22.2 | Jan-Mar 2008 |
| Summary of All Basins | 6,355 | 3,439 | 2,916 | 606 | 90.0 | 85.6-94.4 | 10.0 | 2004–08 |

Figure C.10.  Summary of Arsenic Assessment for Unconfined Aquifers

Table C.10.  Percentage of Unconfined Aquifers Meeting Threshold Value for Arsenic Calculated Using Probabilistic Monitoring Design

Status Network		Designated Use:  Primary Drinking Water Standards 		Units:  Number of wells in list frame

This is a nine-column table.  Column 1 lists the basin, Column 2 lists the target population, Column 3 lists the number of sampleable wells, Column 4 lists the number of inaccessible wells, Column 5 lists the number of samples, Column 6 lists the percent meeting threshold, Column 7 lists the 95% confidence bounds, Column 8 lists the percent not meeting threshold, and Column 9 lists the assessment period.

1 No confined aquifer wells in basin

2 N/A = Not applicable

| Basin | Target Population | Sampleable | Inaccessible | Number of Samples | % Meeting Threshold | 95% Confidence Bounds (% Meeting Threshold) | % Not Meeting Threshold | Assessment Period |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NWFWMD1 Ochlockonee–St. Marks | 542 | 303 | 239 | 30 | 100.0 | 100.0 | 0.0 | June-July 2004 |
| SFWMD1 Everglades–West Coast | 240 | 133 | 107 | 30 | 82.8 | 65.2-100.0 | 17.2 | April-May 2004 |
| SFWMD6 Lake Okeechobee | 103 | 74 | 29 | 30 | 100.0 | 100.0 | 0.0 | April-May 2004 |
| SJRWMD1 Ocklawaha | 273 | 115 | 158 | 30 | 92.1 | 82.8-100.0 | 7.9 | April-May 2004 |
| SRWMD1 Suwannee | 560 | 198 | 362 | 29 | 93.3 | 86.2-100.0 | 6.7 | June-July 2004 |
| SWFWMD1 Tampa Bay | 125 | 92 | 33 | 30 | 92.2 | 83.5-100.0 | 7.8 | April-May 2004 |
| NWFWMD2 Apalachicola–Chipola | 74 | 44 | 30 | 30 | 100.0 | 100.0 | 0.0 | Aug 2005 |
| SFWMD2 St Lucie–Loxahatchee | 339 | 217 | 122 | 30 | 100.0 | 100.0 | 0.0 | April-May 2005 |
| SFWMD7 Charlotte Harbor | 52 | 42 | 10 | 29 | 89.7 | 80.0-99.3 | 10.3 | April-May 2005 |
| SJRWMD2 Lower St. Johns | 177 | 133 | 44 | 30 | 98.1 | 94.9-100.0 | 1.9 | April-June 2005 |
| SJRWMD6 Middle St. Johns | 293 | 173 | 120 | 30 | 100.0 | 100.0 | 0.0 | April-June 2005 |
| SWFWMD2 Tampa Bay Tributaries | 74 | 34 | 40 | 30 | 79.6 | 62.1-97.0 | 20.4 | April-May 2005 |
| NWFWMD3 Choctawhatchee– St. Andrews | 455 | 264 | 191 | 30 | 100.0 | 100.0 | 0.0 | June-Aug 2006 |
| SFWMD3 Caloosahatchee | 102 | 60 | 42 | 30 | 86.7 | 77.3-96.0 | 13.3 | April-May 2006 |
| SFWMD8 Lake Worth Lagoon– Palm Beach Coast | 343 | 106 | 237 | 30 | 90.7 | 82.3-99.1 | 9.3 | April-May 2006 |
| SJRWMD3 Upper St. Johns | 121 | 31 | 90 | 29 | 96.3 | 90.3-100.0 | 3.7 | April-June 2006 |
| SWFWMD3 Sarasota Bay–Peace–Myakka | 279 | 95 | 184 | 30 | 92.9 | 84.5-100.0 | 7.1 | April-June 2006 |
| NWFWMD4 Pensacola | 1,015 | 508 | 507 | 30 | 100.0 | 100.0 | 0.0 | June-Aug 2007 |
| SFWMD4 Fisheating Creek | 34 | 13 | 21 | 13 | 100.0 | 100.0 | 0.0 | April-May 2007 |
| SFWMD9 Kissimmee River | 406 | 57 | 349 | 25 | 100.0 | 100.0 | 0.0 | May-June 2007 |
| SFWMD10 Southeast Coast– Biscayne Bay | 590 | 171 | 419 | 30 | 100.0 | 100.0 | 0.0 | April-June 2007 |
| SJRWMD4 Nassau–St. Marys | 68 | 34 | 34 | 29 | 96.6 | 90.6-100.0 | 3.4 | April-May 2007 |
| SWFWMD4 Withlacoochee | 304 | 100 | 204 | 30 | 97.7 | 93.8-100.0 | 2.3 | April-June 2007 |
| NWFWMD5 Perdido | 629 | 262 | 367 | 30 | 100.0 | 100.0 | 0.0 | June-Aug 2008 |
| SFWMD5 Everglades | 212 | 56 | 156 | 30 | 100.0 | 100.0 | 0.0 | April-June 2008 |
| SFWMD11 Florida Keys | 15 | 8 | 7 | 8 | 87.5 | 67.6-100.0 | 12.5 | May-June 2008 |
| SJRWMD5 Indian River Lagoon | 264 | 152 | 112 | 30 | 94.5 | 87.5-100.0 | 5.5 | April-June 2008 |
| SJRWMD7 Upper East Coast | 117 | 50 | 67 | 29 | 100.0 | 100.0 | 0.0 | April-June 2008 |
| SWFWMD5 Springs Coast | 397 | 173 | 224 | 30 | 91.4 | 83.6-99.3 | 8.6 | April-May 2008 |
| Summary of All Basins | 8,203 | 3,698 | 4,505 | 821 | 96.9 | 95.9-97.9 | 3.1 | 2004–08 |

Figure C.11.  Summary of Cadmium Assessment for Unconfined Aquifers

Table C.11.  Percentage of Unconfined Aquifers Meeting Threshold Value for Cadmium Calculated Using Probabilistic Monitoring Design

Status Network		Designated Use:  Primary Drinking Water Standards 		Units:  Number of wells in list frame

This is a nine-column table.  Column 1 lists the basin, Column 2 lists the target population, Column 3 lists the number of sampleable wells, Column 4 lists the number of inaccessible wells, Column 5 lists the number of samples, Column 6 lists the percent meeting threshold, Column 7 lists the 95% confidence bounds, Column 8 lists the percent not meeting threshold, and Column 9 lists the assessment period.

1 No confined aquifer wells in basin

2 N/A = Not applicable

| Basin | Target Population | Sampleable | Inaccessible | Number of Samples | % Meeting Threshold | 95% Confidence Bounds (% Meeting Threshold) | % Not Meeting Threshold | Assessment Period |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NWFWMD1 Ochlockonee–St. Marks | 542 | 303 | 239 | 30 | 100.0 | 100.0 | 0.0 | June-July 2004 |
| SFWMD1 Everglades–West Coast | 240 | 133 | 107 | 30 | 100.0 | 100.0 | 0.0 | April-May 2004 |
| SFWMD6 Lake Okeechobee | 103 | 74 | 29 | 30 | 100.0 | 100.0 | 0.0 | April-May 2004 |
| SJRWMD1 Ocklawaha | 273 | 115 | 158 | 30 | 100.0 | 100.0 | 0.0 | April-May 2004 |
| SRWMD1 Suwannee | 560 | 198 | 362 | 29 | 100.0 | 100.0 | 0.0 | June-July 2004 |
| SWFWMD1 Tampa Bay | 125 | 92 | 33 | 30 | 100.0 | 100.0 | 0.0 | April-May 2004 |
| NWFWMD2 Apalachicola–Chipola | 74 | 44 | 30 | 30 | 100.0 | 100.0 | 0.0 | Aug 2005 |
| SFWMD2 St Lucie–Loxahatchee | 339 | 217 | 122 | 30 | 97.8 | 94.0-100.0 | 2.2 | April-May 2005 |
| SFWMD7 Charlotte Harbor | 52 | 42 | 10 | 29 | 100.0 | 100.0 | 0.0 | April-May 2005 |
| SJRWMD2 Lower St. Johns | 177 | 133 | 44 | 30 | 100.0 | 100.0 | 0.0 | April-June 2005 |
| SJRWMD6 Middle St. Johns | 293 | 173 | 120 | 30 | 100.0 | 100.0 | 0.0 | April-June 2005 |
| SWFWMD2 Tampa Bay Tributaries | 74 | 34 | 40 | 30 | 100.0 | 100.0 | 0.0 | April-May 2005 |
| NWFWMD3 Choctawhatchee– St. Andrews | 455 | 264 | 191 | 30 | 100.0 | 100.0 | 0.0 | June-Aug 2006 |
| SFWMD3 Caloosahatchee | 102 | 60 | 42 | 30 | 100.0 | 100.0 | 0.0 | April-May 2006 |
| SFWMD8 Lake Worth Lagoon– Palm Beach Coast | 343 | 106 | 237 | 30 | 100.0 | 100.0 | 0.0 | April-May 2006 |
| SJRWMD3 Upper St. Johns | 121 | 31 | 90 | 29 | 100.0 | 100.0 | 0.0 | April.-June 2006 |
| SWFWMD3 Sarasota Bay–Peace–Myakka | 279 | 95 | 184 | 30 | 100.0 | 100.0 | 0.0 | April-June 2006 |
| NWFWMD4 Pensacola | 1,015 | 508 | 507 | 30 | 100.0 | 100.0 | 0.0 | June-Aug 2007 |
| SFWMD4 Fisheating Creek | 34 | 13 | 21 | 13 | 100.0 | 100.0 | 0.0 | April-May 2007 |
| SFWMD9 Kissimmee River | 406 | 57 | 349 | 12 | 100.0 | 100.0 | 0.0 | May-June 2007 |
| SFWMD10 Southeast Coast– Biscayne Bay | 590 | 171 | 419 | 30 | 100.0 | 100.0 | 0.0 | April-June 2007 |
| SJRWMD4 Nassau–St. Marys | 68 | 34 | 34 | 29 | 100.0 | 100.0 | 0.0 | April-May 2007 |
| SWFWMD4 Withlacoochee | 304 | 100 | 204 | 30 | 100.0 | 100.0 | 0.0 | April-June 2007 |
| NWFWMD5 Perdido | 629 | 262 | 367 | 30 | 100.0 | 100.0 | 0.0 | June-Aug 2008 |
| SFWMD5 Everglades | 212 | 56 | 156 | 30 | 97.8 | 94.0-100.0 | 2.2 | April-June 2008 |
| SFWMD11 Florida Keys | 15 | 8 | 7 | 8 | 100.0 | 100.0 | 0.0 | May-June 2008 |
| SJRWMD5 Indian River Lagoon | 264 | 152 | 112 | 30 | 100.0 | 100.0 | 0.0 | April-June 2008 |
| SJRWMD7 Upper East Coast | 117 | 50 | 67 | 29 | 100.0 | 100.0 | 0.0 | April-June 2008 |
| SWFWMD5 Springs Coast | 397 | 173 | 224 | 30 | 100.0 | 100.0 | 0.0 | April-May 2008 |
| Summary of All Basins | 8,203 | 3,698 | 4,505 | 808 | 99.9 | 99.7-100.0 | 0.1 | 2004–08 |

Figure C.12.  Summary of Chromium Assessment for Unconfined Aquifers

Table C.12.  Percentage of Unconfined Aquifers Meeting Threshold Value for Chromium Calculated Using Probabilistic Monitoring Design

Status Network		Designated Use:  Primary Drinking Water Standards 		Units:  Number of wells in list frame

This is a nine-column table.  Column 1 lists the basin, Column 2 lists the target population, Column 3 lists the number of sampleable wells, Column 4 lists the number of inaccessible wells, Column 5 lists the number of samples, Column 6 lists the percent meeting threshold, Column 7 lists the 95% confidence bounds, Column 8 lists the percent not meeting threshold, and Column 9 lists the assessment period.

1 No confined aquifer wells in basin

2 N/A = Not applicable

| Basin | Target Population | Sampleable | Inaccessible | Number of Samples | % Meeting Threshold | 95% Confidence Bounds (% Meeting Threshold) | % Not Meeting Threshold | Assessment Period |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NWFWMD1 Ochlockonee–St. Marks | 542 | 303 | 239 | 30 | 100.0 | 100.0 | 0.0 | June-July 2004 |
| SFWMD1 Everglades–West Coast | 240 | 133 | 107 | 30 | 100.0 | 100.0 | 0.0 | April-May 2004 |
| SFWMD6 Lake Okeechobee | 103 | 74 | 29 | 30 | 100.0 | 100.0 | 0.0 | April-May 2004 |
| SJRWMD1 Ocklawaha | 273 | 115 | 158 | 30 | 100.0 | 100.0 | 0.0 | April-May 2004 |
| SRWMD1 Suwannee | 560 | 198 | 362 | 29 | 100.0 | 100.0 | 0.0 | June-July 2004 |
| SWFWMD1 Tampa Bay | 125 | 92 | 33 | 30 | 100.0 | 100.0 | 0.0 | April-May 2004 |
| NWFWMD2 Apalachicola–Chipola | 74 | 44 | 30 | 30 | 100.0 | 100.0 | 0.0 | Aug 2005 |
| SFWMD2 St Lucie–Loxahatchee | 339 | 217 | 122 | 30 | 100.0 | 100.0 | 0.0 | April-May 2005 |
| SFWMD7 Charlotte Harbor | 52 | 42 | 10 | 29 | 100.0 | 100.0 | 0.0 | April-May 2005 |
| SJRWMD2 Lower St. Johns | 177 | 133 | 44 | 30 | 100.0 | 100.0 | 0.0 | April-June 2005 |
| SJRWMD6 Middle St. Johns | 293 | 173 | 120 | 30 | 100.0 | 100.0 | 0.0 | April-June 2005 |
| SWFWMD2 Tampa Bay Tributaries | 74 | 34 | 40 | 30 | 100.0 | 100.0 | 0.0 | April-May 2005 |
| NWFWMD3 Choctawhatchee– St. Andrews | 455 | 264 | 191 | 11 | 100.0 | 100.0 | 0.0 | June-Aug 2006 |
| SFWMD3 Caloosahatchee | 102 | 60 | 42 | 30 | 100.0 | 100.0 | 0.0 | April-May 2006 |
| SFWMD8 Lake Worth Lagoon– Palm Beach Coast | 343 | 106 | 237 | 30 | 100.0 | 100.0 | 0.0 | April-May 2006 |
| SJRWMD3 Upper St. Johns | 121 | 31 | 90 | 29 | 100.0 | 100.0 | 0.0 | April-June 2006 |
| SWFWMD3 Sarasota Bay–Peace–Myakka | 279 | 95 | 184 | 30 | 100.0 | 100.0 | 0.0 | April-June 2006 |
| NWFWMD4 Pensacola | 1,015 | 508 | 507 | 30 | 100.0 | 100.0 | 0.0 | June-Aug 2007 |
| SFWMD4 Fisheating Creek | 34 | 13 | 21 | 13 | 100.0 | 100.0 | 0.0 | April-May 2007 |
| SFWMD9 Kissimmee River | 406 | 57 | 349 | 6 | 100.0 | 100.0 | 0.0 | May-June 2007 |
| SFWMD10 Southeast Coast– Biscayne Bay | 590 | 171 | 419 | 30 | 100.0 | 100.0 | 0.0 | April-June 2007 |
| SJRWMD4 Nassau–St. Marys | 68 | 34 | 34 | 29 | 96.6 | 90.9-100.0 | 3.4 | April-May 2007 |
| SWFWMD4 Withlacoochee | 304 | 100 | 204 | 30 | 100.0 | 100.0 | 0.0 | April-June 2007 |
| NWFWMD5 Perdido | 629 | 262 | 367 | 30 | 100.0 | 100.0 | 0.0 | June-Aug 2008 |
| SFWMD5 Everglades | 212 | 56 | 156 | 30 | 100.0 | 100.0 | 0.0 | April-June 2008 |
| SFWMD11 Florida Keys | 15 | 8 | 7 | 8 | 100.0 | 100.0 | 0.0 | May-June 2008 |
| SJRWMD5 Indian River Lagoon | 264 | 152 | 112 | 30 | 100.0 | 100.0 | 0.0 | April-June 2008 |
| SJRWMD7 Upper East Coast | 117 | 50 | 67 | 29 | 100.0 | 100.0 | 0.0 | April-June 2008 |
| SWFWMD5 Springs Coast | 397 | 173 | 224 | 30 | 100.0 | 100.0 | 0.0 | April-May 2008 |
| Summary of All Basins | 8,203 | 3,698 | 4,505 | 783 | 99.98 | 99.9-100.0 | 0.02 | 2004–08 |

Figure C.13.  Summary of Fluoride Assessment for Unconfined Aquifers

Table C.13.  Percentage of Unconfined Aquifers Meeting Threshold Value for Fluoride Calculated Using Probabilistic Monitoring Design

Status Network		Designated Use:  Primary Drinking Water Standards 		Units:  Number of wells in list frame

This is a nine-column table.  Column 1 lists the basin, Column 2 lists the target population, Column 3 lists the number of sampleable wells, Column 4 lists the number of inaccessible wells, Column 5 lists the number of samples, Column 6 lists the percent meeting threshold, Column 7 lists the 95% confidence bounds, Column 8 lists the percent not meeting threshold, and Column 9 lists the assessment period.

1 No confined aquifer wells in basin

2 N/A = Not applicable

| Basin | Target Population | Sampleable | Inaccessible | Number of Samples | % Meeting Threshold | 95% Confidence Bounds (% Meeting Threshold) | % Not Meeting Threshold | Assessment Period |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NWFWMD1 Ochlockonee–St. Marks | 542 | 303 | 239 | 30 | 100.0 | 100.0 | 0.0 | June-July 2004 |
| SFWMD1 Everglades–West Coast | 240 | 133 | 107 | 30 | 100.0 | 100.0 | 0.0 | April-May 2004 |
| SFWMD6 Lake Okeechobee | 103 | 74 | 29 | 30 | 100.0 | 100.0 | 0.0 | April-May 2004 |
| SJRWMD1 Ocklawaha | 273 | 115 | 158 | 30 | 100.0 | 100.0 | 0.0 | April-May 2004 |
| SRWMD1 Suwannee | 560 | 198 | 362 | 30 | 100.0 | 100.0 | 0.0 | June-July 2004 |
| SWFWMD1 Tampa Bay | 125 | 92 | 33 | 30 | 97.6 | 93.6-100.0 | 2.4 | April-May 2004 |
| NWFWMD2 Apalachicola–Chipola | 74 | 44 | 30 | 30 | 100.0 | 100.0 | 0.0 | Aug 2005 |
| SFWMD2 St Lucie–Loxahatchee | 339 | 217 | 122 | 30 | 100.0 | 100.0 | 0.0 | April-May 2005 |
| SFWMD7 Charlotte Harbor | 52 | 42 | 10 | 29 | 100.0 | 100.0 | 0.0 | April-May 2005 |
| SJRWMD2 Lower St. Johns | 177 | 133 | 44 | 30 | 100.0 | 100.0 | 0.0 | April-June 2005 |
| SJRWMD6 Middle St. Johns | 293 | 173 | 120 | 30 | 100.0 | 100.0 | 0.0 | April-June 2005 |
| SWFWMD2 Tampa Bay Tributaries | 74 | 34 | 40 | 30 | 100.0 | 100.0 | 0.0 | April-May 2005 |
| NWFWMD3 Choctawhatchee– St. Andrews | 455 | 264 | 191 | 30 | 100.0 | 100.0 | 0.0 | June-Aug 2006 |
| SFWMD3 Caloosahatchee | 102 | 60 | 42 | 30 | 100.0 | 100.0 | 0.0 | April-May 2006 |
| SFWMD8 Lake Worth Lagoon– Palm Beach Coast | 343 | 106 | 237 | 30 | 100.0 | 100.0 | 0.0 | April-May 2006 |
| SJRWMD3 Upper St. Johns | 121 | 31 | 90 | 29 | 100.0 | 100.0 | 0.0 | April-June 2006 |
| SWFWMD3 Sarasota Bay–Peace–Myakka | 279 | 95 | 184 | 30 | 100.0 | 100.0 | 0.0 | April-June 2006 |
| NWFWMD4 Pensacola | 1,015 | 508 | 507 | 30 | 93.6 | 82.8-100.0 | 6.4 | June-Aug 2007 |
| SFWMD4 Fisheating Creek | 34 | 13 | 21 | 13 | 100.0 | 100.0 | 0.0 | April-May 2007 |
| SFWMD9 Kissimmee River | 406 | 57 | 349 | 25 | 100.0 | 100.0 | 0.0 | May-June 2007 |
| SFWMD10 Southeast Coast– Biscayne Bay | 590 | 171 | 419 | 30 | 100.0 | 100.0 | 0.0 | April-June 2007 |
| SJRWMD4 Nassau–St. Marys | 68 | 34 | 34 | 29 | 100.0 | 100.0 | 0.0 | April-May 2007 |
| SWFWMD4 Withlacoochee | 304 | 100 | 204 | 30 | 100.0 | 100.0 | 0.0 | April-June 2007 |
| NWFWMD5 Perdido | 629 | 262 | 367 | 30 | 100.0 | 100.0 | 0.0 | June-Aug 2008 |
| SFWMD5 Everglades | 212 | 56 | 156 | 30 | 100.0 | 100.0 | 0.0 | April-June 2008 |
| SFWMD11 Florida Keys | 15 | 8 | 7 | 8 | 100.0 | 100.0 | 0.0 | May-June 2008 |
| SJRWMD5 Indian River Lagoon | 264 | 152 | 112 | 30 | 100.0 | 100.0 | 0.0 | April-June 2008 |
| SJRWMD7 Upper East Coast | 117 | 50 | 67 | 29 | 100.0 | 100.0 | 0.0 | April-June 2008 |
| SWFWMD5 Springs Coast | 397 | 173 | 224 | 30 | 100.0 | 100.0 | 0.0 | April-May 2008 |
| Summary of All Basins | 8,203 | 3,698 | 4,505 | 822 | 99.3 | 98.2-100.0 | 0.7 | 2004–08 |

Figure C.14.  Summary of Lead Assessment for Unconfined Aquifers

Table C.14.  Percentage of Unconfined Aquifers Meeting Threshold Value for Lead Calculated Using Probabilistic Monitoring Design

Status Network		Designated Use:  Primary Drinking Water Standards 		Units:  Number of wells in list frame

This is a nine-column table.  Column 1 lists the basin, Column 2 lists the target population, Column 3 lists the number of sampleable wells, Column 4 lists the number of inaccessible wells, Column 5 lists the number of samples, Column 6 lists the percent meeting threshold, Column 7 lists the 95% confidence bounds, Column 8 lists the percent not meeting threshold, and Column 9 lists the assessment period.

1 No confined aquifer wells in basin

2 N/A = Not applicable

| Basin | Target Population | Sampleable | Inaccessible | Number of Samples | % Meeting Threshold | 95% Confidence Bounds (% Meeting Threshold) | % Not Meeting Threshold | Assessment Period |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NWFWMD1 Ochlockonee–St. Marks | 542 | 303 | 239 | 30 | 100.0 | 100.0 | 0.0 | June-July 2004 |
| SFWMD1 Everglades–West Coast | 240 | 133 | 107 | 30 | 100.0 | 100.0 | 0.0 | April-May 2004 |
| SFWMD6 Lake Okeechobee | 103 | 74 | 29 | 30 | 97.4 | 92.9-100.0 | 2.6 | April-May 2004 |
| SJRWMD1 Ocklawaha | 273 | 115 | 158 | 27 | 89.1 | 75.9-100.0 | 10.9 | April-May 2004 |
| SRWMD1 Suwannee | 560 | 198 | 362 | 29 | 100.0 | 100.0 | 0.0 | June-July 2004 |
| SWFWMD1 Tampa Bay | 125 | 92 | 33 | 30 | 94.0 | 86.4-100.0 | 6.0 | April-May 2004 |
| NWFWMD2 Apalachicola–Chipola | 74 | 44 | 30 | 30 | 100.0 | 100.0 | 0.0 | Aug 2005 |
| SFWMD2 St Lucie–Loxahatchee | 339 | 217 | 122 | 30 | 95.1 | 89.5-100.0 | 4.9 | April-May 2005 |
| SFWMD7 Charlotte Harbor | 52 | 42 | 10 | 26 | 96.2 | 90.0-100.0 | 3.8 | April-May 2005 |
| SJRWMD2 Lower St. Johns | 177 | 133 | 44 | 30 | 100.0 | 100.0 | 0.0 | April-June 2005 |
| SJRWMD6 Middle St. Johns | 293 | 173 | 120 | 30 | 97.8 | 94.1-100.0 | 2.2 | April-June 2005 |
| SWFWMD2 Tampa Bay Tributaries | 74 | 34 | 40 | 30 | 97.3 | 92.9-100.0 | 2.7 | April-May 2005 |
| NWFWMD3 Choctawhatchee– St. Andrews | 455 | 264 | 191 | 30 | 100.0 | 100.0 | 0.0 | June-Aug 2006 |
| SFWMD3 Caloosahatchee | 102 | 60 | 42 | 30 | 96.7 | 91.1-100.0 | 3.3 | April-May 2006 |
| SFWMD8 Lake Worth Lagoon– Palm Beach Coast | 343 | 106 | 237 | 30 | 89.8 | 81.1-98.6 | 10.2 | April-May 2006 |
| SJRWMD3 Upper St. Johns | 121 | 31 | 90 | 29 | 91.8 | 82.1-100.0 | 8.2 | April-June 2006 |
| SWFWMD3 Sarasota Bay–Peace–Myakka | 279 | 95 | 184 | 30 | 100.0 | 100.0 | 0.0 | April-June 2006 |
| NWFWMD4 Pensacola | 1,015 | 508 | 507 | 30 | 94.8 | 86.4-100.0 | 5.2 | June-Aug 2007 |
| SFWMD4 Fisheating Creek | 34 | 13 | 21 | 8 | 100.0 | 100.0 | 0.0 | April-May 2007 |
| SFWMD9 Kissimmee River | 406 | 57 | 349 | 25 | 100.0 | 100.0 | 0.0 | May-June 2007 |
| SFWMD10 Southeast Coast– Biscayne Bay | 590 | 171 | 419 | 30 | 95.2 | 87.9-100.0 | 4.8 | April-June 2007 |
| SJRWMD4 Nassau–St. Marys | 68 | 34 | 34 | 20 | 90.0 | 78.3-100.0 | 10.0 | April-May 2007 |
| SWFWMD4 Withlacoochee | 304 | 100 | 204 | 30 | 93.3 | 85.7-100.0 | 6.7 | April-June 2007 |
| NWFWMD5 Perdido | 629 | 262 | 367 | 30 | 96.2 | 89.5-100.0 | 3.8 | June-Aug 2008 |
| SFWMD5 Everglades | 212 | 56 | 156 | 15 | 100.0 | 100.0 | 0.0 | April-June 2008 |
| SFWMD11 Florida Keys | 15 | 8 | 7 | 8 | 100.0 | 100.0 | 0.0 | May-June 2008 |
| SJRWMD5 Indian River Lagoon | 264 | 152 | 112 | 30 | 98.0 | 94.8-100.0 | 2.0 | April-June 2008 |
| SJRWMD7 Upper East Coast | 117 | 50 | 67 | 29 | 100.0 | 100.0 | 0.0 | April-June 2008 |
| SWFWMD5 Springs Coast | 397 | 173 | 224 | 30 | 93.9 | 86.5-100.0 | 6.1 | April-May 2008 |
| Summary of All Basins | 8,203 | 3,698 | 4,505 | 786 | 97.5 | 96.3-98.7 | 2.5 | 2004–08 |

Figure C.15.  Summary of Nitrate + Nitrite Assessment for Unconfined Aquifers

Table C.15.  Percentage of Unconfined Aquifers Meeting Threshold Value for Nitrate + Nitrite Calculated Using Probabilistic Monitoring Design

Status Network		Designated Use:  Primary Drinking Water Standards 		Units:  Number of wells in list frame

This is a nine-column table.  Column 1 lists the basin, Column 2 lists the target population, Column 3 lists the number of sampleable wells, Column 4 lists the number of inaccessible wells, Column 5 lists the number of samples, Column 6 lists the percent meeting threshold, Column 7 lists the 95% confidence bounds, Column 8 lists the percent not meeting threshold, and Column 9 lists the assessment period.

1 No confined aquifer wells in basin

2 N/A = Not applicable

| Basin | Target Population | Sampleable | Inaccessible | Number of Samples | % Meeting Threshold | 95% Confidence Bounds (% Meeting Threshold) | % Not Meeting Threshold | Assessment Period |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NWFWMD1 Ochlockonee–St. Marks | 542 | 303 | 239 | 30 | 100.0 | 100.0 | 0.0 | June-July 2004 |
| SFWMD1 Everglades–West Coast | 240 | 133 | 107 | 30 | 100.0 | 100.0 | 0.0 | April-May 2004 |
| SFWMD6 Lake Okeechobee | 103 | 74 | 29 | 30 | 100.0 | 100.0 | 0.0 | April-May 2004 |
| SJRWMD1 Ocklawaha | 273 | 115 | 158 | 30 | 100.0 | 100.0 | 0.0 | April-May 2004 |
| SRWMD1 Suwannee | 560 | 198 | 362 | 30 | 98.4 | 95.7-100.0 | 1.6 | June-July 2004 |
| SWFWMD1 Tampa Bay | 125 | 92 | 33 | 30 | 100.0 | 100.0 | 0.0 | April-May 2004 |
| NWFWMD2 Apalachicola–Chipola | 74 | 44 | 30 | 30 | 100.0 | 100.0 | 0.0 | Aug 2005 |
| SFWMD2 St Lucie–Loxahatchee | 339 | 217 | 122 | 30 | 100.0 | 100.0 | 0.0 | April-May 2005 |
| SFWMD7 Charlotte Harbor | 52 | 42 | 10 | 29 | 100.0 | 100.0 | 0.0 | April-May 2005 |
| SJRWMD2 Lower St. Johns | 177 | 133 | 44 | 30 | 100.0 | 100.0 | 0.0 | April-June 2005 |
| SJRWMD6 Middle St. Johns | 293 | 173 | 120 | 30 | 88.3 | 75.4-100.0 | 11.7 | April-June 2005 |
| SWFWMD2 Tampa Bay Tributaries | 74 | 34 | 40 | 30 | 100.0 | 100.0 | 0.0 | April-May 2005 |
| NWFWMD3 Choctawhatchee– St. Andrews | 455 | 264 | 191 | 30 | 100.0 | 100.0 | 0.0 | June-Aug 2006 |
| SFWMD3 Caloosahatchee | 102 | 60 | 42 | 28 | 100.0 | 100.0 | 0.0 | April-May 2006 |
| SFWMD8 Lake Worth Lagoon– Palm Beach Coast | 343 | 106 | 237 | 27 | 100.0 | 100.0 | 0.0 | April-May 2006 |
| SJRWMD3 Upper St. Johns | 121 | 31 | 90 | 25 | 100.0 | 100.0 | 0.0 | April-June 2006 |
| SWFWMD3 Sarasota Bay–Peace–Myakka | 279 | 95 | 184 | 29 | 100.0 | 100.0 | 0.0 | April-June 2006 |
| NWFWMD4 Pensacola | 1,015 | 508 | 507 | 30 | 100.0 | 100.0 | 0.0 | June-Aug 2007 |
| SFWMD4 Fisheating Creek | 34 | 13 | 21 | 6 | 100.0 | 100.0 | 0.0 | April-May 2007 |
| SFWMD9 Kissimmee River | 406 | 57 | 349 | 19 | 65.2 | 46.4-84.0 | 34.8 | May-June 2007 |
| SFWMD10 Southeast Coast– Biscayne Bay | 590 | 171 | 419 | 30 | 100.0 | 100.0 | 0.0 | April-June 2007 |
| SJRWMD4 Nassau–St. Marys | 68 | 34 | 34 | 14 | 100.0 | 100.0 | 0.0 | April-May 2007 |
| SWFWMD4 Withlacoochee | 304 | 100 | 204 | 23 | 92.0 | 81.9-100.0 | 5.7 | April-June 2007 |
| NWFWMD5 Perdido | 629 | 262 | 367 | 28 | 100.0 | 100.0 | 0.0 | June-Aug 2008 |
| SFWMD5 Everglades | 212 | 56 | 156 | 30 | 97.4 | 92.8-100.0 | 2.6 | April-June 2008 |
| SFWMD11 Florida Keys | 15 | 8 | 7 | 8 | 100.0 | 100.0 | 0.0 | May-June 2008 |
| SJRWMD5 Indian River Lagoon | 264 | 152 | 112 | 30 | 94.0 | 86.0-100.0 | 6.0 | April-June 2008 |
| SJRWMD7 Upper East Coast | 117 | 50 | 67 | 24 | 100.0 | 100.0 | 0.0 | April-June 2008 |
| SWFWMD5 Springs Coast | 397 | 173 | 224 | 26 | 100.0 | 100.0 | 0.0 | April-May 2008 |
| Summary of All Basins | 8,203 | 3,698 | 4,505 | 766 | 97.7 | 96.8-98.7 | 2.3 | 2004–08 |

Figure C.16.  Summary of Sodium Assessment for Unconfined Aquifers

Table C.16.  Percentage of Unconfined Aquifers Meeting Threshold Value for Sodium Calculated Using Probabilistic Monitoring Design

Status Network		Designated Use:  Primary Drinking Water Standards 		Units:  Number of wells in list frame

This is a nine-column table.  Column 1 lists the basin, Column 2 lists the target population, Column 3 lists the number of sampleable wells, Column 4 lists the number of inaccessible wells, Column 5 lists the number of samples, Column 6 lists the percent meeting threshold, Column 7 lists the 95% confidence bounds, Column 8 lists the percent not meeting threshold, and Column 9 lists the assessment period.

1 No confined aquifer wells in basin

2 N/A = Not applicable

| Basin | Target Population | Sampleable | Inaccessible | Number of Samples | % Meeting Threshold | 95% Confidence Bounds (% Meeting Threshold) | % Not Meeting Threshold | Assessment Period |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NWFWMD1 Ochlockonee–St. Marks | 542 | 303 | 239 | 30 | 100.0 | 100.0 | 0.0 | June-July 2004 |
| SFWMD1 Everglades–West Coast | 240 | 133 | 107 | 30 | 80.2 | 61.9-98.4 | 19.8 | April-May 2004 |
| SFWMD6 Lake Okeechobee | 103 | 74 | 29 | 30 | 84.7 | 72.8-96.6 | 15.3 | April-May 2004 |
| SJRWMD1 Ocklawaha | 273 | 115 | 158 | 30 | 100.0 | 100.0 | 0.0 | April-May 2004 |
| SRWMD1 Suwannee | 560 | 198 | 362 | 30 | 94.6 | 88.9-100.0 | 5.4 | June-July 2004 |
| SWFWMD1 Tampa Bay | 125 | 92 | 33 | 30 | 96.9 | 91.9-100.0 | 3.1 | April-May 2004 |
| NWFWMD2 Apalachicola–Chipola | 74 | 44 | 30 | 30 | 100.0 | 100.0 | 0.0 | Aug 2005 |
| SFWMD2 St Lucie–Loxahatchee | 339 | 217 | 122 | 30 | 97.8 | 94.1-100.0 | 2.2 | April-May 2005 |
| SFWMD7 Charlotte Harbor | 52 | 42 | 10 | 29 | 82.8 | 71.4-94.1 | 17.2 | April-May 2005 |
| SJRWMD2 Lower St. Johns | 177 | 133 | 44 | 30 | 86.1 | 65.4-100.0 | 13.9 | April-June 2005 |
| SJRWMD6 Middle St. Johns | 293 | 173 | 120 | 30 | 100.0 | 100.0 | 0.0 | April-June 2005 |
| SWFWMD2 Tampa Bay Tributaries | 74 | 34 | 40 | 30 | 96.8 | 91.3-100.0 | 3.2 | April-May 2005 |
| NWFWMD3 Choctawhatchee– St. Andrews | 455 | 264 | 191 | 30 | 100.0 | 100.0 | 0.0 | June-Aug 2006 |
| SFWMD3 Caloosahatchee | 102 | 60 | 42 | 30 | 90.0 | 81.3-98.7 | 10.0 | April-May 2006 |
| SFWMD8 Lake Worth Lagoon– Palm Beach Coast | 343 | 106 | 237 | 30 | 100.0 | 100.0 | 0.0 | April-May 2006 |
| SJRWMD3 Upper St. Johns | 121 | 31 | 90 | 29 | 92.7 | 84.7-100.0 | 7.3 | April-June 2006 |
| SWFWMD3 Sarasota Bay–Peace–Myakka | 279 | 95 | 184 | 30 | 97.3 | 92.8-100.0 | 2.7 | April-June 2006 |
| NWFWMD4 Pensacola | 1,015 | 508 | 507 | 30 | 96.5 | 90.9-100.0 | 3.5 | June-Aug 2007 |
| SFWMD4 Fisheating Creek | 34 | 13 | 21 | 13 | 92.3 | 79.9-100.0 | 7.7 | April-May 2007 |
| SFWMD9 Kissimmee River | 406 | 57 | 349 | 25 | 100.0 | 100.0 | 0.0 | May-June 2007 |
| SFWMD10 Southeast Coast– Biscayne Bay | 590 | 171 | 419 | 30 | 92.3 | 82.9-100.0 | 7.7 | April-June 2007 |
| SJRWMD4 Nassau–St. Marys | 68 | 34 | 34 | 29 | 100.0 | 100.0 | 0.0 | April-May 2007 |
| SWFWMD4 Withlacoochee | 304 | 100 | 204 | 30 | 100.0 | 100.0 | 0.0 | April-June 2007 |
| NWFWMD5 Perdido | 629 | 262 | 367 | 30 | 97.7 | 93.9-100.0 | 2.3 | June-Aug 2008 |
| SFWMD5 Everglades | 212 | 56 | 156 | 30 | 84.2 | 76.9-91.6 | 15.8 | April-June 2008 |
| SFWMD11 Florida Keys | 15 | 8 | 7 | 8 | 100.0 | 100.0 | 0.0 | May-June 2008 |
| SJRWMD5 Indian River Lagoon | 264 | 152 | 112 | 30 | 80.2 | 66.6-93.7 | 19.8 | April-June 2008 |
| SJRWMD7 Upper East Coast | 117 | 50 | 67 | 29 | 93.1 | 84.9-100.0 | 6.9 | April-June 2008 |
| SWFWMD5 Springs Coast | 397 | 173 | 224 | 30 | 77.8 | 63.6-92.1 | 22.2 | April-May 2008 |
| Summary of All Basins | 8,203 | 3,698 | 4,505 | 822 | 95.7 | 94.5-96.9 | 4.3 | 2004–08 |

Figure C.17.  Summary of Fecal Coliform Assessment for Unconfined Aquifers

Table C.17.  Percentage of Unconfined Aquifers Meeting Threshold Value for Fecal Coliform Calculated Using Probabilistic Monitoring Design

Status Network		Designated Use:  Primary Drinking Water Standards 		Units:  Number of wells in list frame

This is a nine-column table.  Column 1 lists the basin, Column 2 lists the target population, Column 3 lists the number of sampleable wells, Column 4 lists the number of inaccessible wells, Column 5 lists the number of samples, Column 6 lists the percent meeting threshold, Column 7 lists the 95% confidence bounds, Column 8 lists the percent not meeting threshold, and Column 9 lists the assessment period.

1 No confined aquifer wells in basin

2 N/A = Not applicable

| Basin | Target Population | Sampleable | Inaccessible | Number of Samples | % Meeting Threshold | 95% Confidence Bounds (% Meeting Threshold) | % Not Meeting Threshold | Assessment Period |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NWFWMD1 Ochlockonee–St. Marks | 542 | 303 | 239 | 30 | 98.0 | 95.1-100.0 | 2.0 | June-July 2004 |
| SFWMD1 Everglades–West Coast | 240 | 133 | 107 | 30 | 96.5 | 90.6-100.0 | 3.5 | April-May 2004 |
| SFWMD6 Lake Okeechobee | 103 | 74 | 29 | 30 | 100.0 | 100.0 | 0.0 | April-May 2004 |
| SJRWMD1 Ocklawaha | 273 | 115 | 158 | 30 | 93.2 | 83.3-100.0 | 6.8 | April-May 2004 |
| SRWMD1 Suwannee | 560 | 198 | 362 | 29 | 100.0 | 100.0 | 0.0 | June-July 2004 |
| SWFWMD1 Tampa Bay | 125 | 92 | 33 | 30 | 94.3 | 84.9-100.0 | 5.7 | April-May 2004 |
| NWFWMD2 Apalachicola–Chipola | 74 | 44 | 30 | 30 | 95.7 | 88.1-100.0 | 4.3 | Aug 2005 |
| SFWMD2 St Lucie–Loxahatchee | 339 | 217 | 122 | 25 | 94.0 | 84.2-100.0 | 6.0 | April-May 2005 |
| SFWMD7 Charlotte Harbor | 52 | 42 | 10 | 28 | 92.9 | 84.9-100.0 | 7.1 | April-May 2005 |
| SJRWMD2 Lower St. Johns | 177 | 133 | 44 | 30 | 96.1 | 90.2-100.0 | 3.9 | April-June 2005 |
| SJRWMD6 Middle St. Johns | 293 | 173 | 120 | 30 | 97.8 | 94.1-100.0 | 2.2 | April-June 2005 |
| SWFWMD2 Tampa Bay Tributaries | 74 | 34 | 40 | 29 | 100.0 | 100.0 | 0.0 | April-May 2005 |
| NWFWMD3 Choctawhatchee– St. Andrews | 455 | 264 | 191 | 30 | 100.0 | 100.0 | 0.0 | June-Aug 2006 |
| SFWMD3 Caloosahatchee | 102 | 60 | 42 | 29 | 96.6 | 90.8-100.0 | 3.4 | April-May 2006 |
| SFWMD8 Lake Worth Lagoon– Palm Beach Coast | 343 | 106 | 237 | 30 | 97.5 | 93.0-100.0 | 2.5 | April-May 2006 |
| SJRWMD3 Upper St. Johns | 121 | 31 | 90 | 29 | 97.5 | 93.2-100.0 | 2.5 | April-June 2006 |
| SWFWMD3 Sarasota Bay–Peace–Myakka | 279 | 95 | 184 | 30 | 82.9 | 71.7-94.1 | 17.1 | April-June 2006 |
| NWFWMD4 Pensacola | 1,015 | 508 | 507 | 30 | 100.0 | 100.0 | 0.0 | June-Aug 2007 |
| SFWMD4 Fisheating Creek | 34 | 13 | 21 | 13 | 100.0 | 100.0 | 0.0 | April-May 2007 |
| SFWMD9 Kissimmee River | 406 | 57 | 349 | 25 | 87.3 | 74.8-99.8 | 12.7 | May-June 2007 |
| SFWMD10 Southeast Coast– Biscayne Bay | 590 | 171 | 419 | 30 | 86.9 | 76.1-97.6 | 13.1 | April-June 2007 |
| SJRWMD4 Nassau–St. Marys | 68 | 34 | 34 | 28 | 92.9 | 84.5-100.0 | 7.1 | April-May 2007 |
| SWFWMD4 Withlacoochee | 304 | 100 | 204 | 31 | 94.3 | 87.1-100.0 | 5.7 | April-June 2007 |
| NWFWMD5 Perdido | 629 | 262 | 367 | 30 | 96.2 | 89.9-100.0 | 3.8 | June-Aug 2008 |
| SFWMD5 Everglades | 212 | 56 | 156 | 30 | 89.0 | 80.5-97.6 | 11.0 | April-June 2008 |
| SFWMD11 Florida Keys | 15 | 8 | 7 | 8 | 100.0 | 100.0 | 0.0 | May-June 2008 |
| SJRWMD5 Indian River Lagoon | 264 | 152 | 112 | 30 | 96.5 | 90.4-100.0 | 3.5 | April-June 2008 |
| SJRWMD7 Upper East Coast | 117 | 50 | 67 | 29 | 100.0 | 100.0 | 0.0 | April-June 2008 |
| SWFWMD5 Springs Coast | 397 | 173 | 224 | 30 | 100.0 | 100.0 | 0.0 | April-May 2008 |
| Summary of All Basins | 8,203 | 3,698 | 4,505 | 813 | 96.2 | 94.2-98.2 | 3.8 | 2004–08 |

Figure C.18.  Summary of Total Coliform Assessment for Unconfined Aquifers

Table C.18.  Percentage of Unconfined Aquifers Meeting Threshold Value for Total Coliform Calculated Using Probabilistic Monitoring Design

Status Network		Designated Use:  Primary Drinking Water Standards 		Units:  Number of wells in list frame

This is a nine-column table.  Column 1 lists the basin, Column 2 lists the target population, Column 3 lists the number of sampleable wells, Column 4 lists the number of inaccessible wells, Column 5 lists the number of samples, Column 6 lists the percent meeting threshold, Column 7 lists the 95% confidence bounds, Column 8 lists the percent not meeting threshold, and Column 9 lists the assessment period.

1 No confined aquifer wells in basin

2 N/A = Not applicable

| Basin | Target Population | Sampleable | Inaccessible | Number of Samples | % Meeting Threshold | 95% Confidence Bounds (% Meeting Threshold) | % Not Meeting Threshold | Assessment Period |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NWFWMD1 Ochlockonee–St. Marks | 542 | 303 | 239 | 30 | 55.1 | 28.5-81.7 | 44.9 | June-July 2004 |
| SFWMD1 Everglades–West Coast | 240 | 133 | 107 | 30 | 89.8 | 82.4-97.2 | 10.2 | April-May 2004 |
| SFWMD6 Lake Okeechobee | 103 | 74 | 29 | 30 | 94.7 | 88.4-100.0 | 5.3 | April-May 2004 |
| SJRWMD1 Ocklawaha | 273 | 115 | 158 | 30 | 69.3 | 52.6-86.0 | 30.7 | April-May 2004 |
| SRWMD1 Suwannee | 560 | 198 | 362 | 28 | 82.1 | 66.2-98.1 | 17.9 | June-July 2004 |
| SWFWMD1 Tampa Bay | 125 | 92 | 33 | 30 | 83.3 | 71.7-94.9 | 16.7 | April-May 2004 |
| NWFWMD2 Apalachicola–Chipola | 74 | 44 | 30 | 30 | 89.6 | 76.8-100.0 | 10.4 | Aug 2005 |
| SFWMD2 St Lucie–Loxahatchee | 339 | 217 | 122 | 25 | 82.5 | 67.6-97.4 | 17.5 | April-May 2005 |
| SFWMD7 Charlotte Harbor | 52 | 42 | 10 | 26 | 73.1 | 59.6-86.5 | 26.9 | April-May 2005 |
| SJRWMD2 Lower St. Johns | 177 | 133 | 44 | 21 | 100.0 | 100.0 | 0.0 | April-June 2005 |
| SJRWMD6 Middle St. Johns | 293 | 173 | 120 | 30 | 74.3 | 58.5-90.0 | 25.7 | April-June 2005 |
| SWFWMD2 Tampa Bay Tributaries | 74 | 34 | 40 | 29 | 86.2 | 76.4-96.1 | 13.8 | April-May 2005 |
| NWFWMD3 Choctawhatchee– St. Andrews | 455 | 264 | 191 | 30 | 92.9 | 85.7-100.0 | 7.1 | June-Aug 2006 |
| SFWMD3 Caloosahatchee | 102 | 60 | 42 | 27 | 85.2 | 75.8-94.6 | 14.8 | April-May 2006 |
| SFWMD8 Lake Worth Lagoon– Palm Beach Coast | 343 | 106 | 237 | 30 | 72.2 | 58.8-85.7 | 27.8 | April-May 2006 |
| SJRWMD3 Upper St. Johns | 121 | 31 | 90 | 29 | 95.0 | 89.7-100.0 | 5.0 | April-June 2006 |
| SWFWMD3 Sarasota Bay–Peace–Myakka | 279 | 95 | 184 | 30 | 55.2 | 39.8-70.6 | 44.8 | April-June 2006 |
| NWFWMD4 Pensacola | 1,015 | 508 | 507 | 30 | 93.8 | 85.6-100.0 | 6.2 | June-Aug 2007 |
| SFWMD4 Fisheating Creek | 34 | 13 | 21 | 13 | 92.3 | 78.4-100.0 | 7.7 | April-May 2007 |
| SFWMD9 Kissimmee River | 406 | 57 | 349 | 25 | 70.6 | 54.1-87.0 | 29.4 | May-June 2007 |
| SFWMD10 Southeast Coast– Biscayne Bay | 590 | 171 | 419 | 30 | 64.4 | 47.9-81.0 | 35.6 | April-June 2007 |
| SJRWMD4 Nassau–St. Marys | 68 | 34 | 34 | 3 | 100.0 | 100.0 | 0.0 | April-May 2007 |
| SWFWMD4 Withlacoochee | 304 | 100 | 204 | 31 | 84.9 | 75.3-94.4 | 15.1 | April-June 2007 |
| NWFWMD5 Perdido | 629 | 262 | 367 | 30 | 93.6 | 83.2-100.0 | 6.4 | June-Aug 2008 |
| SFWMD5 Everglades | 212 | 56 | 156 | 30 | 55.9 | 38.0-73.9 | 44.1 | April-June 2008 |
| SFWMD11 Florida Keys | 15 | 8 | 7 | 8 | 75.0 | 47.0-100.0 | 25.0 | May-June 2008 |
| SJRWMD5 Indian River Lagoon | 264 | 152 | 112 | 30 | 79.9 | 65.0-94.9 | 20.1 | April-June 2008 |
| SJRWMD7 Upper East Coast | 117 | 50 | 67 | 29 | 86.2 | 76.4-96.0 | 13.8 | April-June 2008 |
| SWFWMD5 Springs Coast | 397 | 173 | 224 | 30 | 89.7 | 79.3-100.0 | 10.3 | April-May 2008 |
| Summary of All Basins | 8,203 | 3,698 | 4,505 | 774 | 82.6 | 78.8-86.5 | 17.4 | 2004–08 |

# APPENDIX D.  Impaired Surface Waters Rule Methodology for Evaluating Impairment for the Basin Assessments

To determine the status of surface water quality in individual WBIDs in Florida, three categories of data—chemistry data, biological data, and fish consumption advisories—are evaluated using the IWR methodology to determine potential impairments for four designated use attainment categories:  aquatic life, primary contact and recreation, fish and shellfish consumption, drinking water use, and protection of human health.

## Aquatic Life Based Attainment

The IWR follows the principle of independent applicability.  A waterbody is listed for potential impairment of aquatic life use support based on exceedances of any one of four types of water quality indicators (numeric water quality criteria, nutrient thresholds, biological thresholds, and toxicity data).

### Exceedances of Numeric Water Quality Criteria

The water quality data from STORET used in evaluating impairment were also used in making attainment determinations.  The assessment of impairment included only ambient surface water quality stations.  Water quality information from point sources or wells was excluded.  Data that were determined to be of insufficient quality for TMDL data quality objectives were also excluded from the Verified List assessment.  Monitoring stations were classified as one of five waterbody types—spring, stream, lake, estuary, or blackwater—based on specific criteria.  The assessments included the following parameters:

Metals	Arsenic, aluminum, cadmium, chromium VI, chromium III, copper, iron, lead, mercury, nickel, selenium, silver, thallium, and zinc

Nutrients	Chlorophyll a for streams and estuaries, and TSI (chlorophyll a, total nitrogen, and total phosphorus) for lakes

Conventionals	DO, fecal coliform, pH, unionized ammonia

The requirements for placing waters on the Planning List of potentially impaired waters included a minimum of 10 temporally independent samples from the 10-year period of record shown in Tables 13a and 13b (in Chapter 3), unless there were 3 exceedances of water quality or 1 exceedance of an acute toxicity criterion in a 3-year period.  The screening methodology for the Verified List of impaired waters requires at least 20 samples from the last 7.5 years preceding the Planning List assessment.  An exceedance, meaning that water quality criteria or standards are not met, is recorded any time the criterion is exceeded by any amount.  An exceedance for DO, however, means that a waterbody does not meet the DO criterion, rather than an actual exceedance of the criterion.

To determine if a water should be placed on the Planning List for each parameter, the chemical data were analyzed using a computer program written to assess the data, based on criteria established in the IWR, with two exceptions.  First, un-ionized ammonia data were not analyzed by the program, but rather with an Excel spreadsheet.  Second, because the full complexity of the pH criterion could not be programmed, the incomplete listings for pH were not included, but were further examined while additional data were collected during Phase 2 of the watershed management cycle.  Data analysis and statistical summaries of waterbody identification numbers (WBIDs), waterbody types, and parameters obtained from the STORET Database were conducted using Access, SAS statistical software, and ArcView GIS applications

The data for metals and conventional parameters were compared with the state surface water quality criteria in Section 62-302.530, F.A.C. (IWR).  The rule contains a table of sample numbers versus exceedances.  A waterbody was placed on the Planning List if there was at least 80% confidence that the actual criteria exceedance rate was greater than or equal to 10%.  To be placed on the Verified List, at least a 90% confidence rate was required.

### Exceedances of Nutrient Thresholds

The state currently has a narrative nutrient criterion instead of a numeric value for nutrient thresholds.  The narrative criterion states, “In no case shall nutrient concentrations of a body of water be altered so as to cause an imbalance in natural populations of aquatic flora or fauna.”  The IWR provides an interpretation of the narrative nutrient criterion.  In general, the TSI and the annual mean chlorophyll a values are the primary means for assessing whether a waterbody should be assessed further for nutrient impairment.

The rule also considers other information that might indicate an imbalance in flora or fauna due to nutrient enrichment, such as algal blooms, excessive macrophyte growth, a decrease in the distribution (either in density or aerial coverage) of seagrasses or other submerged aquatic vegetation, changes in algal species richness, and excessive diel oxygen swings.

Potential nutrient impairment was evaluated by calculating annual mean chlorophyll a values for estuaries and streams and the TSI for lakes.  For lakes, the TSI was calculated using chlorophyll a, total phosphorus, and total nitrogen measurements.  Direct evidence of imbalances of flora and fauna in waterbodies was also considered in the evaluation of nutrient impairments.

In estuarine areas, a waterbody was considered nutrient enriched if the annual mean chlorophyll a values were greater than 11 g/L, or if annual mean chlorophyll a values increased by more than 50% over historical values for at least 2 consecutive years.  For streams, a waterbody was considered nutrient enriched if the annual mean chlorophyll a values were greater than 20 g/L, or if the annual mean increased by more than 50% over historical values for at least 2 consecutive years.

A lake with a mean color greater than 40 PCUs was considered nutrient enriched if the annual mean TSI exceeded 60.  A lake with a mean color less than or equal to 40 PCUs was considered nutrient enriched if the annual mean TSI exceeded 40.  In addition, a lake was considered nutrient enriched if there was a statistically significant increase in TSI over the last 5-year period, or if TSI measurements were 10 units higher than historical values.

### Exceedances of Biological Thresholds

Bioassessments were carried out for streams, lakes, canals, and rivers using the IWR as guidance and following FDEP’s standard operating procedures, which provide definitions and specific methods for the generation and analysis of bioassessment data.  These are referenced in the individual bioassessment data tables contained in the Water Quality Status Reports (available at http://www.dep.state.fl.us/water/basin411/default.htm).  The purpose behind using a bioassessment methodology in surface water characterizations is that biological components of the environment manifest long-term water quality conditions and thus provide a better indication of a waterbody’s true health than discrete chemical or physical measurements alone.  Biological communities also respond to factors other than water quality, such as habitat disruption and hydrologic disturbances.  Therefore, to use bioassessment data to list a waterbody, FDEP must reasonably demonstrate the pollutant responsible for the observed degradation.  Recent recalibrations of the SCI and the BioRecon method involved the use of the Human Disturbance Gradient (HDG).  The HDG ranked sites based on independent assessments of habitat quality, the degree of hydrologic disturbance, water quality, and human land use intensity.  Biological measures (metrics) associated with minimally affected waters in a given region were used to set ecoregional expectations.

A metric is a precise measure of biological community structure or function that responds in a predictable manner to human disturbance.  Metrics (e.g., number of long-lived taxa, number of sensitive taxa, % filter feeders, % clingers) were calculated and aggregated into a dimensionless, multimetric index.  Indices have advantages over individual metrics in that they can integrate several nonredundant metrics into a single score that reflects a wider range of biological variables.  In Florida, multimetric bioassessment indices have been developed for vascular plants (aquatic trees, shrubs, and herbaceous macrophytes) and benthic macroinvertebrates (animals without a backbone, such as aquatic insects, crayfish, snails, and mussels).  Efforts to create multimetric indices for periphyton (attached algae) and phytoplankton (drifting algae) are currently under way.

Only macroinvertebrate data from ambient sites in state surface waters were used in the bioassessments analyzed for the Planning and Verified Lists.  The data included sites designated as test and background sites for NPDES fifth-year inspections, but excluded data from effluent outfalls from discharging facilities, or data from monitoring sites not clearly established to collect ambient water quality data.  Because site-specific habitat and physicochemical assessment information (e.g., % suitable macroinvertebrate habitat, water velocities, extent of sand or silt smothering, and riparian [or streamside] buffer zone widths) was not available at the time of reporting, it was not included.  However, this information is important for identifying the stressors responsible for a failed bioassessment and will be included in future reporting.  Waterbodies that are adversely affected only by pollution (e.g., a lack of habitat or hydrologic disruption) but not a pollutant (a water quality exceedance) are not placed on the Verified List.

The data used to develop the Planning and Verified Lists were obtained from FDEP’s Biological Database (SBIO) and the EPA’s STORET Water Quality Database, where it could be substantiated that the data were generated in compliance with the bioassessment standard operating procedures referenced in the IWR (Section 62-303.330, F.A.C.).  The data from these databases were used without regard to the randomness of sample site selection.

### Lake Condition Index

The scoring of the individual metrics of the Lake Condition Index (LCI), except % Diptera, was performed according to the following formula:

100(B/A) where A = the 95 percentile of the reference population and B = observed value.

For % Diptera, the following formula was used:

100 (100-B)/(100-A) where A = the 95 percentile of the reference population and B = observed value.

An average LCI score was calculated by averaging the scores of the 6 metrics in the method:  total number of taxa; total number of taxa belonging to the orders Ephemeroptera, Odonata, and Trichoptera (EOT taxa); % EOT taxa; Shannon-Wiener Diversity Index score; Hulbert Index score; and % Dipteran individuals.  LCI calculations were only provided for noncolored lakes 
(< 20 PCUs).  As macroinvertebrate-based indices have not been shown to assess colored lakes in Florida accurately (> 20 PCUs), they have been excluded from bioassessments.  A poor or very poor rating based on the average score constituted a failed bioassessment, based on the IWR.

### Stream Condition Index

A total SCI score was calculated by averaging the scores of the 10 metrics in the method:  total number of taxa; total number of taxa belonging to the order Ephemeroptera, total taxa of the order Trichoptera, % filter feeders, % long-lived taxa, clinger taxa, % dominant taxa, % taxa in the Tanytarsini, % sensitive taxa, and % very tolerant taxa (see Table D.1 for calculations).  A poor or very poor rating based on the total score constituted a failed bioassessment, based on the IWR.  The Basin Status Reports contain definitions and specific methods for the generation and analysis of bioassessment data.

Table D.1.  SCI Metrics for the Northeast, Panhandle, and Peninsula Regions of Florida

This is a four-column table.  Column 1 lists the SCI metric, Column 2 lists the calculation for northeast Florida, Column 3 lists the calculation for the Panhandle region, and Column 4 lists the calculation for the Peninsula region.

| SCI Metric | Northeast | Panhandle | Peninsula |
| --- | --- | --- | --- |
| Total taxa | 10 * (X–16)/26 | 10 * (X–16)/33 | 10 * (X–16)/25 |
| Ephemeroptera taxa | 10 * X /3.5 | 10 * X /6 | 10 * X /5 |
| Trichoptera taxa | 10 * X /6.5 | 10 * X /7 | 10 * X /7 |
| % filterer | 10 * (X–1)/41 | 10 * (X–1)/44 | 10 * (X–1)/39 |
| Long-lived taxa | 10 * X /3 | 10 * X /5 | 10 * X /4 |
| Clinger taxa | 10 * X /9 | 10 * X /15.5 | 10 * X /8 |
| % dominance | 10 – ( 10 * [ ( X–10)/44 ] ) | 10 – ( 10 * [ ( X–10)/33 ] ) | 10 – ( 10 * [ ( X–10)/44 ] ) |
| % Tanytarsini | 10 * [ ln( X + 1) /3.3] | 10 * [ ln( X + 1) /3.3] | 10 * [ ln( X + 1) /3.3] |
| Sensitive taxa | 10 * X /11 | 10 * X /19 | 10 * X /9 |
| % Very tolerant | 10 – (10 * [ ln( X + 1)/4.4 ] ) | 10 – (10 * [ ln( X + 1)/3.6 ] ) | 10 – (10 * [ ln( X + 1)/4.1 ] ) |

### BioReconnaissance

To establish an impairment rating based on BioRecon data, the six metrics as calculated in Table D.2 were used.

Table D.2.  BioRecon Metrics for the Northeast, Panhandle, and Peninsula Regions of Florida

This is a four-column table.  Column 1 lists the BioRecon metric, Column 2 lists the calculation for northeast Florida, Column 3 lists the calculation for the Panhandle region, and Column 4 lists the calculation for the Peninsula region.

| BioRecon Metric | Northeast | Panhandle | Peninsula |
| --- | --- | --- | --- |
| Total taxa | (X–14)/23 | (X–16)/33 | (X–11)/25 |
| Ephemeroptera taxa | X /3.5 | X /12 | X /5 |
| Trichoptera taxa | X /6.5 | X /7 | X /7 |
| Long–lived taxa | X /6 | X /10 | X /7 |
| Clinger taxa | X /7 | X /15.5 | X /8 |
| Sensitive taxa | X /11 | X /19 | X /9 |

A poor score, based on two BioRecon failures, warrants inclusion on the Verified List (Table D.3).

Table D.3.  BioRecon Sample Size and Index Range

This is a two-column table.  Column 1 lists the BioRecon sample size and score, and Column 2 lists the index range.

| BioRecon | Index Range |
| --- | --- |
| 1 sample: Pass | (6–10) |
| 1 sample: Fail | (0–6) |
| 2 samples: Good | (7–10) |
| 2 samples: Fair | (4–7) |
| 2 samples: Poor | (0–4) |

### Biological Integrity Standard

Quantitative data, generated through the use of Hester-Dendy artificial substrate samplers, were used to calculate Shannon-Wiener Diversity Index scores for paired background and test sites, as specified in the Biological Integrity Standard of Subsection 62-302.530(11), F.A.C.  One failure of the standard meant that a waterbody segment was listed as potentially impaired.

### Evaluation of Toxicity Data

Although the IWR describes the use of toxicity data for the assessment of aquatic life-based attainment, no ambient toxicity data were available for assessment and this metric was not used.

## Primary Contact and Recreation Attainment

For Class I, II, or III waters, a waterbody was potentially impaired if the following criteria were met:

The waterbody segment did not meet the applicable water quality criteria for bacteriological quality,

The waterbody segment included a bathing area that was closed by a local health department or county government for more than 1 week or more than once during a calendar year based on bacteriological data,

The waterbody segment included a bathing area for which a local health department or county government issued closures, advisories, or warnings totaling 21 days or more during a calendar year based on bacteriological data, or

The waterbody segment included a bathing area that was closed or had advisories or warnings for more than 12 weeks during a calendar year based on previous bacteriological data or on derived relationships between bacteria levels and rainfall or flow.

## Fish and Shellfish Consumption Attainment

For Class I, II, or III waters, a waterbody was potentially impaired if it did not meet the applicable Class II water quality criteria for bacteriological quality, or if a fish consumption advisory had been issued.  Fish consumption advisories were based on FDOH’s “limited consumption” or “no consumption” advisories for surface waters because of high levels of mercury in fish tissue.  In addition, for Class II waters, waterbody segments that had been approved for shellfish harvesting but were downgraded to a more restrictive classification were listed as potentially impaired.

## Drinking Water Attainment and Protection of Human Health

For Class I waters, a waterbody was potentially impaired if it did not meet the applicable Class I water quality criteria.

# Appendix E:  Impaired Lakes in Florida, 
Group 1–5 Basins

This is a five-column table.  Column 1 lists each impaired lake, Column 2 lists the WBID number, Column 3 lists the parameter causing impairment, Column 4 lists the basin group, and Column 5 lists the basin group name.

Note:  The most up-to-date list of verified waters, by basin group, is available at http://www.dep.state.fl.us/water/tmdl/.

| Lake | WBID | Parameter Causing Impairment | Basin Group | Basin Group Name |
| --- | --- | --- | --- | --- |
| Alachua Sink | 2720A | Fecal Coliform | Group 1 | Ocklawaha |
| Alford Arm | 647 | DO | Group 1 | Ochlockonee–St. Marks |
| Alligator Lake | 1574A | DO | Group 1 | Tampa Bay |
| Alligator Lake | 3176 | Fish | Group 4 | Kissimmee River |
| Alligator Lake | 3516A | DO | Group 1 | Suwannee |
| Alligator Lake | 3516A | TSI | Group 1 | Suwannee |
| Alligator Lake | Alligator Lake | Alligator Lake | Alligator Lake | Alligator Lake |
| Asher Lake | 3004O | TSI | Group 2 | Middle St. Johns |
| Banana Lake | 1549B | TSI | Group 3 | Sarasota Bay–Peace–Myakka |
| Bear Gulley Lake | 3009 | TSI | Group 2 | Middle St. Johns |
| Bear Lake | 3004A | Fish | Group 2 | Middle St. Johns |
| Bear Lake | 3004A | TSI | Group 2 | Middle St. Johns |
| Bellows Lake | 1579A | TSI | Group 1 | Tampa Bay |
| Bevens Arm | 2718B | Historic TSI | Group 1 | Ocklawaha |
| Bevens Arm | 2718B | TSI | Group 1 | Ocklawaha |
| Bevens Arm | 2718B | Turbidity | Group 1 | Ocklawaha |
| Big Redfish Lake | 959I | DO | Group 3 | Choctawhatchee–St. Andrew |
| Blue Cypress Lake | 2893V | Fish | Group 3 | Upper St. Johns |
| Blue Cypress Lake | 2893V | TSI Trend | Group 3 | Upper St. Johns |
| Boyd Lake | 2917 | Fish | Group 2 | Middle St. Johns |
| Brick Lake | 3177A | Fish | Group 4 | Kissimmee River |
| Broken Arrow Lake | 2953A | Historic TSI | Group 2 | Middle St. Johns |
| Buck Lake | 1493E | TSI | Group 1 | Tampa Bay |
| Buck Lake | 2966A | Fish | Group 3 | Upper St. Johns |
| Buck Lake | 2991B | TSI | Group 2 | Middle St. Johns |
| Calm Lake | 1473Y | Historic TSI | Group 1 | Tampa Bay |
| Camp Creek Lake | 1027A | DO | Group 3 | Choctawhatchee–St. Andrew |
| Cane Lake | 3169J | TSI | Group 4 | Kissimmee River |
| Cascade Lake | 878D | DO | Group 1 | Ochlockonee–St. Marks |
| Clark Lake | 1971 | TSI | Group 3 | Sarasota Bay–Peace–Myakka |
| Clear Lake | 3169G | TSI | Group 4 | Kissimmee River |
| Clear Lake | 3245C2 | TSI | Group 3 | Lake Worth Lagoon–Palm Beach Coast |
| Cowpen Lake | 2723A | Fish | Group 1 | Ocklawaha |
| Crescent Lake | 1474V | TSI | Group 1 | Tampa Bay |
| Crescent Lake | 1700A | DO | Group 1 | Tampa Bay |
| Crescent Lake | 2606B | Fish | Group 2 | Lower St. Johns |
| Crescent Lake | 2606B | TSI | Group 2 | Lower St. Johns |
| Crystal Lake | 1497A | TSI | Group 3 | Sarasota Bay–Peace–Myakka |
| Cub Lake | 3004P | TSI | Group 2 | Middle St. Johns |
| Cue Lake | 2575 | Fish | Group 2 | Lower St. Johns |
| Davis Lake | 2593A | DO | Group 2 | Lower St. Johns |
| Dead Lake | 2615A | Fish | Group 2 | Lower St. Johns |
| Dead Lake | 51A | Fish | Group 2 | Apalachicola–Chipola |
| Deer Lake | 1521P | TSI | Group 3 | Sarasota Bay–Peace–Myakka |
| Deerpoint Lake | 553A | Fish | Group 3 | Choctawhatchee–St. Andrew |
| Doctors Lake | 2389 | TSI | Group 2 | Lower St. Johns |
| Double Pond | 210A | Fish | Group 3 | Choctawhatchee–St. Andrew |
| Draper Lake | 959D | DO | Group 3 | Choctawhatchee–St. Andrew |
| Eagle Lake | 1623M | TSI | Group 3 | Sarasota Bay–Peace–Myakka |
| Eagle Run | 2308 | DO | Group 2 | Lower St. Johns |
| Eagle Run | 2308 | Fecal Coliform | Group 2 | Lower St. Johns |
| East Lake Tohopekaliga | 3172 | Fish | Group 4 | Kissimmee River |
| Eastern Lake | 1037 | DO | Group 3 | Choctawhatchee–St. Andrew |
| Ella Lake | 2797A | DO | Group 1 | Ocklawaha |
| Ella Lake | 2797A | Fish | Group 1 | Ocklawaha |
| Fairy Lake | 2994C | TSI | Group 2 | Middle St. Johns |
| Fox Lake | 3008A | DO | Group 3 | Upper St. Johns |
| Fox Lake | 3008A | Fish | Group 3 | Upper St. Johns |
| Fruitwood Lake | 2994Y | Historic TSI | Group 2 | Middle St. Johns |
| Fruitwood Lake | 2994Y | TSI | Group 2 | Middle St. Johns |
| Fuller Lake | 959G | DO | Group 3 | Choctawhatchee–St. Andrew |
| Georges Lake | 2541 | Fish | Group 2 | Lower St. Johns |
| Georges Lake | 2541 | Historic TSI | Group 2 | Lower St. Johns |
| Georges Lake | 2541 | TSI Trend | Group 2 | Lower St. Johns |
| Grassy Lake | 1623M1 | TSI | Group 3 | Sarasota Bay–Peace–Myakka |
| Hampton Lake | 3635A | Fish | Group 1 | Suwannee |
| Hillsborough Reservoir | 1443E1 | DO | Group 2 | Tampa Bay Tributaries |
| Hillsborough Reservoir | 1443E1 | Fish | Group 2 | Tampa Bay Tributaries |
| Hillsborough Reservoir | 1443E1 | TSI | Group 2 | Tampa Bay Tributaries |
| Hollingsworth Lake | 1549X | TSI | Group 3 | Sarasota Bay–Peace–Myakka |
| Holly Lake | 2803A | DO | Group 1 | Ocklawaha |
| Holly Lake | 2803A | Fish | Group 1 | Ocklawaha |
| Horseshoe Lake | 2991D | DO | Group 2 | Middle St. Johns |
| Howell Lake | 2997B | Historic TSI | Group 2 | Middle St. Johns |
| Howell Lake | 2997B | TSI | Group 2 | Middle St. Johns |
| Huckleberry Lake | 1893 | TSI | Group 4 | Kissimmee River |
| Island Lake | 2994D | TSI | Group 2 | Middle St. Johns |
| Johns Lake | 2873C | Fish | Group 1 | Ocklawaha |
| Johns Lake | 2873C | Historic TSI | Group 1 | Ocklawaha |
| Johns Lake | 2873C | TSI | Group 1 | Ocklawaha |
| King Lake–Open Water | 1451G | TSI | Group 2 | Tampa Bay Tributaries |
| Kingsley Lake | 2476B | DO | Group 2 | Lower St. Johns |
| Kingsley Lake | 2476B | Historic TSI | Group 2 | Lower St. Johns |
| Kingsley Lake | 2476B | TSI Trend | Group 2 | Lower St. Johns |
| Konomac Lake Reservoir | 2954 | Fish | Group 2 | Middle St. Johns |
| Lake Adair | 2997R | TSI | Group 2 | Middle St. Johns |
| Lake Alfred | 1488D | TSI | Group 3 | Sarasota Bay–Peace–Myakka |
| Lake Altho | 3605H | Fish | Group 1 | Suwannee |
| Lake Ann | 2997B1 | TSI Trend | Group 2 | Middle St. Johns |
| Lake Ann | 2997B1 | TSI | Group 2 | Middle St. Johns |
| Lake Annie | 1539C | TSI | Group 3 | Sarasota Bay–Peace–Myakka |
| Lake Arbuckle | 1685A | Fish | Group 4 | Kissimmee River |
| Lake Arianna (North) | 1501B | TSI | Group 3 | Sarasota Bay–Peace–Myakka |
| Lake Ashby | 2925A | Fish | Group 2 | Middle St. Johns |
| Lake Ashby | 2925A | TSI Trend | Group 2 | Middle St. Johns |
| Lake Barton | 3023E | TSI | Group 2 | Middle St. Johns |
| Lake Blue | 1521Q | TSI | Group 3 | Sarasota Bay–Peace–Myakka |
| Lake Bonny | 1497E | TSI | Group 3 | Sarasota Bay–Peace–Myakka |
| Lake Broward | 2617A | Fish | Group 2 | Lower St. Johns |
| Lake Bryant | 2782C | Fish | Group 1 | Ocklawaha |
| Lake Bryant | 2782C | TSI | Group 1 | Ocklawaha |
| Lake Buckeye | 1488S | TSI | Group 3 | Sarasota Bay–Peace–Myakka |
| Lake Buffum | 1677C | Fish | Group 3 | Sarasota Bay–Peace–Myakka |
| Lake Burkett | 3009C | TSI | Group 2 | Middle St. Johns |
| Lake Butler | 3170Q | Fish | Group 4 | Kissimmee River |
| Lake Butler | 3170Q | Historic TSI | Group 4 | Kissimmee River |
| Lake Catherine | 3169P | DO | Group 4 | Kissimmee River |
| Lake Catherine | 3169P | TSI | Group 4 | Kissimmee River |
| Lake Chautauqua | 1603D | DO | Group 1 | Tampa Bay |
| Lake Cherry | 2839D | DO | Group 1 | Ocklawaha |
| Lake Cherry | 2839D | Fish | Group 1 | Ocklawaha |
| Lake Cherry | 2839D | Historic TSI | Group 1 | Ocklawaha |
| Lake Cherry | 3322A | Fish | Group 1 | Suwannee |
| Lake Clarke | 3245B | Fecal Coliform | Group 3 | Lake Worth Lagoon– Palm Beach Coast |
| Lake Clinch | 1706 | Fish | Group 4 | Kissimmee River |
| Lake Cone at Seminole | 28932 | Fish | Group 3 | Upper St. Johns |
| Lake Confusion | 15003 | TSI | Group 3 | Sarasota Bay–Peace–Myakka |
| Lake Conine | 1488U | TSI | Group 3 | Sarasota Bay–Peace–Myakka |
| Lake Copeland | 3168M | TSI | Group 4 | Kissimmee River |
| Lake Crosby | 3593A | Fish | Group 1 | Suwannee |
| Lake Cypress | 3180A | Fish | Group 4 | Kissimmee River |
| Lake Cypress | 3180A | TSI | Group 4 | Kissimmee River |
| Lake Daisy | 1539R | TSI | Group 3 | Sarasota Bay–Peace–Myakka |
| Lake Daniel | 3004E | TSI | Group 2 | Middle St. Johns |
| Lake Daugharty | 2671A | Fish | Group 2 | Lower St. Johns |
| Lake Davenport | 1436A | DO | Group 4 | Kissimmee River |
| Lake Dead Lady | 1474W | TSI | Group 1 | Tampa Bay |
| Lake Delancey | 2894 | Fish | Group 2 | Middle St. Johns |
| Lake Denham | 2832A | DO | Group 1 | Ocklawaha |
| Lake Denham | 2832A | TSI | Group 1 | Ocklawaha |
| Lake Dexter | 1539P | Fish | Group 3 | Sarasota Bay–Peace–Myakka |
| Lake Dexter | 2921C | Fish | Group 2 | Middle St. Johns |
| Lake Dias | 2667A | TSI | Group 2 | Lower St. Johns |
| Lake Disston | 2630B | Fish | Group 2 | Lower St. Johns |
| Lake Disston | 2630B | Lead | Group 2 | Lower St. Johns |
| Lake Dorr | 2929C | Fish | Group 2 | Middle St. Johns |
| Lake Dot | 2997Q | Fecal Coliform | Group 2 | Middle St. Johns |
| Lake Dot | 2997Q | Historic TSI | Group 2 | Middle St. Johns |
| Lake Dot | 2997Q | TSI | Group 2 | Middle St. Johns |
| Lake Eaton | 2771A | DO | Group 1 | Ocklawaha |
| Lake Eaton | 2771A | Fish | Group 1 | Ocklawaha |
| Lake Echo | 1488Z | TSI | Group 3 | Sarasota Bay–Peace–Myakka |
| Lake Effie | 1617A | DO | Group 3 | Sarasota Bay–Peace–Myakka |
| Lake Elbert | 1548 | TSI | Group 3 | Sarasota Bay–Peace–Myakka |
| Lake Eloise | 1521B | TSI | Group 3 | Sarasota Bay–Peace–Myakka |
| Lake Emporia | 2912A | Historic TSI | Group 2 | Middle St. Johns |
| Lake Emporia | 2912A | TSI | Group 2 | Middle St. Johns |
| Lake Erie | 971A | DO | Group 1 | Ochlockonee–St. Marks |
| Lake Estelle | 2997K | TSI | Group 2 | Middle St. Johns |
| Lake Eustis | 2817B | DO | Group 1 | Ocklawaha |
| Lake Eva | 15101 | TSI | Group 3 | Sarasota Bay–Peace–Myakka |
| Lake Fairview | 3004B | TSI | Group 2 | Middle St. Johns |
| Lake Fairview | 3004N | TSI | Group 2 | Middle St. Johns |
| Lake Formosa | 2997M | TSI | Group 2 | Middle St. Johns |
| Lake Frederica | 3036 | Fish | Group 2 | Middle St. Johns |
| Lake Gandy | 3004J | Historic TSI | Group 2 | Middle St. Johns |
| Lake Gandy | 3004J | TSI | Group 2 | Middle St. Johns |
| Lake Gear | 3023D | TSI | Group 2 | Middle St. Johns |
| Lake Geneva | 2509 | Historic TSI | Group 2 | Lower St. Johns |
| Lake Geneva | 2509 | Lead | Group 2 | Lower St. Johns |
| Lake George | 2893A | Fish | Group 2 | Middle St. Johns |
| Lake George | 2893A | TSI | Group 2 | Middle St. Johns |
| Lake George | 3009E | Historic TSI | Group 2 | Middle St. Johns |
| Lake Georgia | 3009E | TSI | Group 2 | Middle St. Johns |
| Lake Gibson | 1497D | TSI | Group 3 | Sarasota Bay–Peace–Myakka |
| Lake Haines | 1488C | TSI | Group 3 | Sarasota Bay–Peace–Myakka |
| Lake Hall | 689B | DO | Group 1 | Ochlockonee–St. Marks |
| Lake Hamilton | 15041 | Fish | Group 3 | Sarasota Bay–Peace–Myakka |
| Lake Hancock | 1392B | DO | Group 5 | Springs Coast |
| Lake Hancock | 1623L | DO | Group 3 | Sarasota Bay–Peace–Myakka |
| Lake Hancock | 1623L | TSI | Group 3 | Sarasota Bay–Peace–Myakka |
| Lake Harney | 2964A | DO | Group 2 | Middle St. Johns |
| Lake Harney | 2964A | Fish | Group 2 | Middle St. Johns |
| Lake Harney | 2964A | TSI | Group 2 | Middle St. Johns |
| Lake Harriet | 3000A | DO | Group 2 | Middle St. Johns |
| Lake Harriet | 3000A | Fecal Coliform | Group 2 | Middle St. Johns |
| Lake Hart | 3171 | Fish | Group 4 | Kissimmee River |
| Lake Hatchineha | 1472B | Fish | Group 4 | Kissimmee River |
| Lake Hayes | 2999A | TSI | Group 2 | Middle St. Johns |
| Lake Helen Blazes | 2893Q | Fish | Group 3 | Upper St. Johns |
| Lake Hiawassa | 878C | DO | Group 1 | Ochlockonee–St. Marks |
| Lake Hiawassee | 3002J | TSI | Group 2 | Middle St. Johns |
| Lake Hiawatha | 2839B | DO | Group 1 | Ocklawaha |
| Lake Hicpochee | 3237C | DO | Group 3 | Caloosahatchee |
| Lake Hicpochee | 3237C | DO | Group 3 | Caloosahatchee |
| Lake Holden | 3168H | TSI | Group 4 | Kissimmee River |
| Lake Ida | 3262A | TSI | Group 3 | Lake Worth Lagoon– Palm Beach Coast |
| Lake in the Woods | 29977 | TSI | Group 2 | Middle St. Johns |
| Lake Istokpoga | 1856B | Historic TSI | Group 4 | Kissimmee River |
| Lake Istokpoga | 1856B | TSI | Group 4 | Kissimmee River |
| Lake Jackson (Osceola County) | 3183G | DO | Group 4 | Kissimmee River |
| Lake Jackson (Osceola County) | 3183G | TSI | Group 4 | Kissimmee River |
| Lake Joanna | 2821B | Historic TSI | Group 1 | Ocklawaha |
| Lake Josephine | 1860B | Fish | Group 4 | Kissimmee River |
| Lake Josephine | 1860B | TSI | Group 4 | Kissimmee River |
| Lake Juanita | 1473W | Historic TSI | Group 1 | Tampa Bay |
| Lake June in Winter | 1938A | Fish | Group 4 | Kissimmee River |
| Lake Juniper | 283 | Fish | Group 3 | Choctawhatchee–St. Andrew |
| Lake Kerr | 2899B | Fish | Group 2 | Middle St. Johns |
| Lake Kerr | 2899B | TSI Trend | Group 2 | Middle St. Johns |
| Lake Killarney | 2997X | TSI | Group 2 | Middle St. Johns |
| Lake Killarney | 647C | DO | Group 1 | Ochlockonee–St. Marks |
| Lake Kissimmee | 3183B | Fish | Group 4 | Kissimmee River |
| Lake Kissimmee | 3183B | TSI | Group 4 | Kissimmee River |
| Lake Lafayette (Lower Segment) | 756C | DO | Group 1 | Ochlockonee–St. Marks |
| Lake Lafayette (Lower Segment) | 756C | TSI | Group 1 | Ochlockonee–St. Marks |
| Lake Lafayette (Upper Segment) | 756A | DO | Group 1 | Ochlockonee–St. Marks |
| Lake Lafayette (Upper Segment) | 756A | Fecal Coliform | Group 1 | Ochlockonee–St. Marks |
| Lake Lena | 1501 | TSI | Group 3 | Sarasota Bay–Peace–Myakka |
| Lake Lorna Doone | 3169H | TSI | Group 4 | Kissimmee River |
| Lake Louisa | 2839M | DO | Group 1 | Ocklawaha |
| Lake Louisa | 2839M | Fish | Group 1 | Ocklawaha |
| Lake Lucien | 3011C | Fish | Group 2 | Middle St. Johns |
| Lake Madelene | 1516B | TSI | Group 1 | Tampa Bay |
| Lake Maggiore | 1731A | DO | Group 1 | Tampa Bay |
| Lake Maggiore | 1731A | TSI | Group 1 | Tampa Bay |
| Lake Maggiore | 1731A | Unionized Ammonia | Group 1 | Tampa Bay |
| Lake Manatee Reservoir | 1807B | DO | Group 2 | Tampa Bay Tributaries |
| Lake Manatee Reservoir | 1807B | Fecal Coliform | Group 2 | Tampa Bay Tributaries |
| Lake Manatee Reservoir | 1807B | TSI | Group 2 | Tampa Bay Tributaries |
| Lake Mann | 3169I | TSI | Group 4 | Kissimmee River |
| Lake Margaret | 2892 | Fish | Group 2 | Middle St. Johns |
| Lake Marian | 3184 | TSI | Group 4 | Kissimmee River |
| Lake Marianna | 1521L | TSI | Group 3 | Sarasota Bay–Peace–Myakka |
| Lake Marion | 3731A | DO | Group 1 | Suwannee |
| Lake Martha | 1488P | TSI | Group 3 | Sarasota Bay–Peace–Myakka |
| Lake Mary Jane | 3171A | Fish | Group 4 | Kissimmee River |
| Lake Mary Jane | 3171A | Lead | Group 4 | Kissimmee River |
| Lake Maude | 1488Q | TSI | Group 3 | Sarasota Bay–Peace–Myakka |
| Lake Mcleod | 1588A | TSI | Group 3 | Sarasota Bay–Peace–Myakka |
| Lake Menzie | 1539Z | TSI | Group 3 | Sarasota Bay–Peace–Myakka |
| Lake Miccosukee | 791N | DO | Group 1 | Ochlockonee–St. Marks |
| Lake Miccosukee | 791N | TSI | Group 1 | Ochlockonee–St. Marks |
| Lake Minnehaha | 2839N | DO | Group 1 | Ocklawaha |
| Lake Minnehaha | 2839N | Fish | Group 1 | Ocklawaha |
| Lake Minneola | 2839A | Fish | Group 1 | Ocklawaha |
| Lake Molly | 2680A | TSI | Group 2 | Lower St. Johns |
| Lake Monroe | 2893D | DO | Group 2 | Middle St. Johns |
| Lake Monroe | 2893D | Fish | Group 2 | Middle St. Johns |
| Lake Monroe | 2893D | TSI | Group 2 | Middle St. Johns |
| Lake Munson | 807C | DO | Group 1 | Ochlockonee–St. Marks |
| Lake Munson | 807C | TSI | Group 1 | Ochlockonee–St. Marks |
| Lake Munson | 807C | Turbidity | Group 1 | Ochlockonee–St. Marks |
| Lake Myakka Lower Segment) | 1981 | Fish | Group 3 | Sarasota Bay–Peace–Myakka |
| Lake Myakka (Upper Segment) | 1981C | Fish | Group 3 | Sarasota Bay–Peace–Myakka |
| Lake Myakka (Upper Segment) | 1981C | TSI | Group 3 | Sarasota Bay–Peace–Myakka |
| Lake Myrtle | 2986B | DO | Group 2 | Middle St. Johns |
| Lake Mystic | 926A1 | Fish | Group 2 | Apalachicola–Chipola |
| Lake Nash | 1450B | Fish | Group 5 | Springs Coast |
| Lake Ned | 1539Q | TSI | Group 3 | Sarasota Bay–Peace–Myakka |
| Lake Norris | 2929B | Fish | Group 2 | Middle St. Johns |
| Lake Ocklawaha | 2740B | Fish | Group 1 | Ocklawaha |
| Lake Octahatchee | 3321A | Fish | Group 1 | Suwannee |
| Lake Okeechobee | 3212A | Iron | Group 1 | Lake Okeechobee |
| Lake Okeechobee | 3212A | Fish | Group 1 | Lake Okeechobee |
| Lake Okeechobee | 3212B | Fish | Group 1 | Lake Okeechobee |
| Lake Okeechobee | 3212C | Iron | Group 1 | Lake Okeechobee |
| Lake Okeechobee | 3212C | Fish | Group 1 | Lake Okeechobee |
| Lake Okeechobee | 3212D | Iron | Group 1 | Lake Okeechobee |
| Lake Okeechobee | 3212D | Fish | Group 1 | Lake Okeechobee |
| Lake Okeechobee | 3212D | pH | Group 1 | Lake Okeechobee |
| Lake Okeechobee | 3212E | Alkalinity | Group 1 | Lake Okeechobee |
| Lake Okeechobee | 3212E | Iron | Group 1 | Lake Okeechobee |
| Lake Okeechobee | 3212E | Fish | Group 1 | Lake Okeechobee |
| Lake Okeechobee | 3212F | Iron | Group 1 | Lake Okeechobee |
| Lake Okeechobee | 3212F | Fish | Group 1 | Lake Okeechobee |
| Lake Okeechobee | 3212F | pH | Group 1 | Lake Okeechobee |
| Lake Okeechobee | 3212G | Fish | Group 1 | Lake Okeechobee |
| Lake Okeechobee | 3212H | Iron | Group 1 | Lake Okeechobee |
| Lake Okeechobee | 3212H | Fish | Group 1 | Lake Okeechobee |
| Lake Okeechobee | 3212H | pH | Group 1 | Lake Okeechobee |
| Lake Okeechobee | 3212I | Fish | Group 1 | Lake Okeechobee |
| Lake Okeechobee | 3212I | pH | Group 1 | Lake Okeechobee |
| Lake Olive | 3168N | TSI | Group 4 | Kissimmee River |
| Lake Osborne | 3256A | DO | Group 3 | Lake Worth Lagoon– Palm Beach Coast |
| Lake Overstreet | 689A | DO | Group 1 | Ochlockonee–St. Marks |
| Lake Panasoffkee | 1351B | DO | Group 4 | Withlacoochee |
| Lake Panasoffkee | 1351B | Historic TSI | Group 4 | Withlacoochee |
| Lake Panasoffkee | 1351B | TSI | Group 4 | Withlacoochee |
| Lake Pansy | 1488Y | TSI | Group 3 | Sarasota Bay–Peace–Myakka |
| Lake Park | 2997U | TSI | Group 2 | Middle St. Johns |
| Lake Parker | 1497B | TSI | Group 3 | Sarasota Bay–Peace–Myakka |
| Lake Pearl | 3000 | TSI | Group 2 | Middle St. Johns |
| Lake Piney Z | 756B | DO | Group 1 | Ochlockonee–St. Marks |
| Lake Piney Z | 756B | TSI | Group 1 | Ochlockonee–St. Marks |
| Lake Placid | 1938C | Fish | Group 4 | Kissimmee River |
| Lake Poinsett | 2893K | DO | Group 3 | Upper St. Johns |
| Lake Poinsett | 2893K | Fish | Group 3 | Upper St. Johns |
| Lake Poinsett | 2893K | Historic TSI | Group 3 | Upper St. Johns |
| Lake Powell | 1055A | DO | Group 3 | Choctawhatchee–St. Andrew |
| Lake Reinheimer | 1478H | TSI | Group 1 | Tampa Bay |
| Lake Rochelle | 1488B | TSI | Group 3 | Sarasota Bay–Peace–Myakka |
| Lake Ross | 2543F | Lead | Group 2 | Lower St. Johns |
| Lake Ross | 2543F | TSI | Group 2 | Lower St. Johns |
| Lake Rowena | 2997J | TSI | Group 2 | Middle St. Johns |
| Lake Russell | 3170B | Fish | Group 4 | Kissimmee River |
| Lake Sampson | 3598D | Fish | Group 1 | Suwannee |
| Lake Sarah | 3004F | TSI | Group 2 | Middle St. Johns |
| Lake Sebring | 1842 | Fish | Group 4 | Kissimmee River |
| Lake Seminole | 60 | TSI | Group 2 | Apalachicola–Chipola |
| Lake Shadow | 3011B | TSI | Group 2 | Middle St. Johns |
| Lake Sheelar | 2528B | DO | Group 2 | Lower St. Johns |
| Lake Sheelar | 2528B | Historic TSI | Group 2 | Lower St. Johns |
| Lake Silver | 1488G | TSI | Group 3 | Sarasota Bay–Peace–Myakka |
| Lake Smart | 1488A | TSI | Group 3 | Sarasota Bay–Peace–Myakka |
| Lake Spring | 2997S | TSI | Group 2 | Middle St. Johns |
| Lake Stahl | 1549B1 | DO | Group 3 | Sarasota Bay–Peace–Myakka |
| Lake Stahl | 1549B1 | TSI | Group 3 | Sarasota Bay–Peace–Myakka |
| Lake Sue | 2997I | TSI | Group 2 | Middle St. Johns |
| Lake Susannah | 3023C | TSI Trend | Group 2 | Middle St. Johns |
| Lake Swoope | 1488V | TSI | Group 3 | Sarasota Bay–Peace–Myakka |
| Lake Sybella | 29975 | TSI | Group 2 | Middle St. Johns |
| Lake Sylvan | 2961 | Fish | Group 2 | Middle St. Johns |
| Lake Talquin | 1297C | DO | Group 1 | Ochlockonee–St. Marks |
| Lake Talquin | 1297C | Fish | Group 1 | Ochlockonee–St. Marks |
| Lake Talquin | 1297C | TSI | Group 1 | Ochlockonee–St. Marks |
| Lake Talquin | 1297D | DO | Group 1 | Ochlockonee–St. Marks |
| Lake Talquin | 1297D | Fish | Group 1 | Ochlockonee–St. Marks |
| Lake Talquin | 1297D | TSI | Group 1 | Ochlockonee–St. Marks |
| Lake Tarpon | 1486A | DO | Group 1 | Tampa Bay |
| Lake Tarpon | 1486A | Historic TSI | Group 1 | Tampa Bay |
| Lake Tenoroc | 1497C | TSI | Group 3 | Sarasota Bay–Peace–Myakka |
| Lake Thonotosassa | 1522B | DO | Group 2 | Tampa Bay Tributaries |
| Lake Thonotosassa | 1522B | Historic TSI | Group 2 | Tampa Bay Tributaries |
| Lake Thonotosassa | 1522B | TSI | Group 2 | Tampa Bay Tributaries |
| Lake Thonotosassa | 1522B | Unionized Ammonia | Group 2 | Tampa Bay Tributaries |
| Lake Tohopekaliga | 3173A | Fish | Group 4 | Kissimmee River |
| Lake Tony | 2994Y1 | TSI | Group 2 | Middle St. Johns |
| Lake Tracy | 14921 | TSI | Group 3 | Sarasota Bay–Peace–Myakka |
| Lake Trafford | 3259W | DO | Group 1 | Everglades West Coast |
| Lake Trafford | 3259W | TSI | Group 1 | Everglades West Coast |
| Lake Trafford | 3259W | Unionized Ammonia | Group 1 | Everglades West Coast |
| Lake Underhille | 3168G | TSI | Group 4 | Kissimmee River |
| Lake Washington | 2893O | DO | Group 3 | Upper St. Johns |
| Lake Washington | 2893O | Fish | Group 3 | Upper St. Johns |
| Lake Washington | 2893O | Historic TSI | Group 3 | Upper St. Johns |
| Lake Weeks | 971B | DO | Group 1 | Ochlockonee–St. Marks |
| Lake Weir | 2790A | TSI | Group 1 | Ocklawaha |
| Lake Wekiva (Orlando) | 3004K | TSI | Group 2 | Middle St. Johns |
| Lake Weston | 3011A | TSI | Group 2 | Middle St. Johns |
| Lake Wilson | 2839C | DO | Group 1 | Ocklawaha |
| Lake Winder | 2893Y | DO | Group 3 | Upper St. Johns |
| Lake Winder | 2893Y | Fish | Group 3 | Upper St. Johns |
| Lake Winder | 2893Y | TSI | Group 3 | Upper St. Johns |
| Lake Winnemissett | 2931 | Historic TSI | Group 2 | Middle St. Johns |
| Lake Winnemissett | 2931 | TSI Trend | Group 2 | Middle St. Johns |
| Lake Winona | 2659A | Historic TSI | Group 2 | Lower St. Johns |
| Lake Winona | 2659A | TSI Trend | Group 2 | Lower St. Johns |
| Lake Wire | 1537 | Lead | Group 2 | Tampa Bay Tributaries |
| Lake Wire | 1537 | TSI | Group 2 | Tampa Bay Tributaries |
| Lake Woodruff | 2921 | Fish | Group 2 | Middle St. Johns |
| Leftover Lake Ivanhoe | 29971 | TSI | Group 2 | Middle St. Johns |
| Lily Lake | 2509H | Lead | Group 2 | Lower St. Johns |
| Linden Lake | 2956A1 | DO | Group 2 | Middle St. Johns |
| Little Hickory Bay | 3259Z | Fish | Group 1 | Everglades West Coast |
| Little Lake Hamilton | 15001 | TSI | Group 3 | Sarasota Bay–Peace–Myakka |
| Little Lake Howell | 2994X | TSI | Group 2 | Middle St. Johns |
| Little Lake Wilson | 1463M | Fecal Coliform | Group 1 | Tampa Bay |
| Little Lake Wilson | 1463M | TSI | Group 1 | Tampa Bay |
| Little Redfish Lake | 959J | DO | Group 3 | Choctawhatchee–St. Andrew |
| Livingston Lake | 1730B | Fish | Group 4 | Kissimmee River |
| Lochloosa Lake | 2738A | Historic TSI | Group 1 | Ocklawaha |
| Lochloosa Lake | 2738A | TSI | Group 1 | Ocklawaha |
| Low Lake | 3496A | DO | Group 1 | Suwannee |
| Merritts Mill Pond | 180A | Chlorophyll | Group 2 | Apalachicola–Chipola |
| Mill Dam Lake | 2779A | Fish | Group 1 | Ocklawaha |
| Moccasin Creek | 1530A | DO | Group 1 | Tampa Bay |
| Moccasin Creek | 1530A | Fecal Coliform | Group 1 | Tampa Bay |
| Moccasin Creek | 1530A | TSI | Group 1 | Tampa Bay |
| Moore Lake | 889A | DO | Group 1 | Ochlockonee–St. Marks |
| Moore Lake | 889A | Fish | Group 1 | Ochlockonee–St. Marks |
| Morris Lake | 959 | DO | Group 3 | Choctawhatchee–St. Andrew |
| Mound Lake | 1473X | Historic TSI | Group 1 | Tampa Bay |
| Mud Lake | 1467 | TSI | Group 4 | Withlacoochee |
| Mud Lake | 2893J | Fish | Group 2 | Middle St. Johns |
| Mullet Lake | 2893H | Fish | Group 2 | Middle St. Johns |
| Newnans Lake | 2705B | DO | Group 1 | Ocklawaha |
| Ocean Pond | 2339 | Fish | Group 4 | Nassau–St. Marys |
| Ocheesee Pond | 344 | DO | Group 2 | Apalachicola–Chipola |
| Orange Lake | 2749A | DO | Group 1 | Ocklawaha |
| Orange Lake Reach | 2749 | DO | Group 1 | Ocklawaha |
| Park Lake | 2997O | TSI | Group 2 | Middle St. Johns |
| Parker Creek | 1141A | DO | Group 3 | Choctawhatchee–St. Andrew |
| Peacock Lake | 3438A | DO | Group 1 | Suwannee |
| Persimmon Lake | 1938E | TSI | Group 4 | Kissimmee River |
| Pine Lake | 3245C4 | DO | Group 3 | Lake Worth Lagoon– Palm Beach Coast |
| Pine Lake | 3245C4 | Fecal Coliform | Group 3 | Lake Worth Lagoon– Palm Beach Coast |
| Pine Lake | 3245C4 | TSI | Group 3 | Lake Worth Lagoon– Palm Beach Coast |
| Pineloch | 3168I | TSI | Group 4 | Kissimmee River |
| Puzzle Lake | 2964B | DO | Group 3 | Upper St. Johns |
| Puzzle Lake | 2964B | Fish | Group 3 | Upper St. Johns |
| Red Bug Lake | 2994E | TSI | Group 2 | Middle St. Johns |
| Red Lake | 3171C | Copper | Group 4 | Kissimmee River |
| Rock Lake | 3169Q | TSI | Group 4 | Kissimmee River |
| Ruth Lake | 2964C | Fish | Group 3 | Upper St. Johns |
| Ruth Lake | 2964C | TSI | Group 3 | Upper St. Johns |
| Sand Hammock Pond | 61A | Fish | Group 3 | Choctawhatchee–St. Andrew |
| Sand Lake | 2956E | TSI | Group 2 | Middle St. Johns |
| Santa Fe Lake | 3605G | DO | Group 1 | Suwannee |
| Santa Fe Lake | 3605G | Fish | Group 1 | Suwannee |
| Savannas | 3194C | Copper | Group 2 | St. Lucie–Loxahatchee |
| Savannas | 3194C | DO | Group 2 | St. Lucie–Loxahatchee |
| Sawgrass Lake | 28931 | DO | Group 3 | Upper St. Johns |
| Sawgrass Lake | 28931 | Fish | Group 3 | Upper St. Johns |
| Sawgrass Lake | 28931 | Historic TSI | Group 3 | Upper St. Johns |
| Sawgrass Lake | 28931 | TSI | Group 3 | Upper St. Johns |
| Saxon Lake | 1451W | TSI | Group 2 | Tampa Bay Tributaries |
| Sears Lake | 1501W | TSI | Group 3 | Sarasota Bay–Peace–Myakka |
| Shell Creek Reservoir (Hamilton Reservoir) | 2041B | DO | Group 3 | Sarasota Bay–Peace–Myakka |
| Silver Lake | 2825A | TSI | Group 1 | Ocklawaha |
| Smith Lake | 2785A | Fish | Group 1 | Ocklawaha |
| South Grasshopper Lake | 2916B | Fish | Group 2 | Middle St. Johns |
| South Lake | 3008B | Fish | Group 3 | Upper St. Johns |
| St. Johns River above Black Creek | 2213I | Silver | Group 2 | Lower St. Johns |
| St. Johns River above Black Creek | 2213I | Fish | Group 2 | Lower St. Johns |
| St. Johns River above Doctors Lake | 2213G | Fish | Group 2 | Lower St. Johns |
| St. Johns River above Doctors Lake | 2213G | Thallium | Group 2 | Lower St. Johns |
| St. Johns River above Federal Point | 2213L | DO | Group 2 | Lower St. Johns |
| St. Johns River above Federal Point | 2213L | Fish | Group 2 | Lower St. Johns |
| St. Johns River above Julington Creek | 2213H | Fish | Group 2 | Lower St. Johns |
| St. Johns River above Palmo Creek | 2213J | Fish | Group 2 | Lower St. Johns |
| St. Johns River above Tocoi | 2213K | Fish | Group 2 | Lower St. Johns |
| Sunset Lake | 1496A | TSI | Group 1 | Tampa Bay |
| Tallavanna Lake | 540A | TSI | Group 1 | Ochlockonee–St. Marks |
| Thompson Pond | 272 | TSI | Group 2 | Apalachicola–Chipola |
| Trout Lake | 2819A | DO | Group 1 | Ocklawaha |
| Watermelon Pond | 3703A | DO | Group 1 | Suwannee |
| Western Lake | 1009A | DO | Group 3 | Choctawhatchee–St. Andrew |
| Wildcat Lake | 2905C | Fish | Group 2 | Middle St. Johns |