LAKE ECOSUMMARY

[Lake Name], [WBID], [County], [Sampling dates]

[site picture (optional)]

DEP conducted water quality and biological sampling at [name of Lake] (STORET xxxxxx, [SBIO Nickname]) in [x] County on [sampling date and date] to assess attainment of designated uses. [This lake was sampled “as part of strategic monitoring program” or “as part of a statewide random sampling network” or “in response to a homeowner request,” etc.].   Overall, the water quality and plant community data indicated that the lake [met/did not meet/or if, inconclusive, provide further explanation] expectations for a healthy, well-balanced lake.

Background

Healthy, well-balanced lake communities may be maintained with some level of human activities, but excessive human disturbance may result in lake degradation.  Human stressors include increased inputs of nutrients, sediments and/or pesticides from watershed runoff, undesirable removal of native shoreline and/or upland buffer vegetation, and introduction of nuisance (generally exotic) plants and animals.  DEP has methods to evaluate if human activities have resulted in a particular waterbody exceeding water quality criteria (Chapter 62-302, Florida Administrative Code [F.A.C.]), including whether adverse impacts to biological communities have occurred.  DEP water quality standards are designed to protect designated uses of the waters of the state (e.g., recreation, aquatic life support), and exceedances of these standards are associated with interference with the designated use.  DEP assesses the health of plant communities in Florida lakes, as one indication of whether adverse impacts to biological communities have occurred.

Site Description

[Include relevant description of site and location, including map.]

Figure 1.  Location of [Site Name] in [x] County.  [put figure captions below figures]

Site History

[If there is relevant historical information, include it here, e.g., graph of chlorophyll a or LVI over time, discussion of past results, and indication of whether or not the site is on the TMDL list for anything, with a link to TMDL tracker]

[Insert additional discussion, photos, or anything you think would be informative for the audience]

Methods

Water Quality

This lake was sampled on [date and date] by the DEP [Tallahassee or x District] Office.  Surface water [or other depth] samples were collected for analysis of [nutrients, chlorophyll a, color, etc.] following DEP Standard Operating Procedures (SOPs; see http://www.dep.state.fl.us/water/sas/qa/sops.htm).  Sampling and analyses met DEP quality assurance/quality control standards (see http://www.dep.state.fl.us/water /sas/qa/index.htm ).

Chlorophyll a is a measure of algal biomass in the water column.  Chapter 62-302.531, F.A.C., provides numeric criteria for chlorophyll a, total nitrogen (TN), and total phosphorus (TP) in lakes (Table 1).  If the annual geometric mean chlorophyll a (calculated with at least four samples, representing seasonal variability) does not exceed the chlorophyll a value for the lake type in Table 1, then the TN and TP criteria for that calendar year shall be the annual geometric means of lake TN and TP samples, subject to the minimum and maximum limits in Table 1.  If there are insufficient data to calculate the annual geometric mean chlorophyll a for a given year or the annual geometric mean chlorophyll a exceeds the values in Table 1 for the lake type, then the applicable numeric interpretations for TN and TP shall be the minimum values in Table 1.

Table 1. Numeric nutrient criteria in lakes, 62-302.531 (2)(b)(1), F.A.C.  AGM = annual geometric mean

| Long Term Geometric Mean Lake Color and Alkalinity | AGM Chlorophyll a | AGM TP Range | AGM TN Range |
| --- | --- | --- | --- |
| > 40 Platinum Cobalt Units | 20 µg/L | 0.05 to 0.161 mg/L | 1.27 to 2.23 mg/L |
| ≤ 40 Platinum Cobalt Units and > 20 mg/L CaCO3 | 20 µg/L | 0.03 to 0.09 mg/L | 1.05 to 1.91 mg/L |
| ≤ 40 Platinum Cobalt Units and ≤ 20 mg/L CaCO3 | 6 µg/L | 0.01 to 0.03 mg/L | 0.51 to 0.93 mg/L |

1 For lakes with color > 40 PCU in the West Central Region, the maximum TP limit is 0.49 mg/L

Dissolved Oxygen

Rule 62-302.533 (1), F.A.C., states that no more than 10 percent of the daily average percent dissolved oxygen (DO) saturation values shall be below 67 percent in the Panhandle West bioregion, 38 percent in the Peninsula and Everglades bioregions, or 34 percent in the Northeast and Big Bend bioregions. Percent saturation takes into account factors such as temperature, atmospheric pressure, and salinity. For lakes, the daily average DO level shall be calculated as the average of measurements collected in the upper two meters of the water column at the same location on the same day.

The daily average freshwater DO criterion is preferentially assessed using daily average values calculated from full days of diel (collected over a 24 hour period) monitoring data.  If diel monitoring data are not available (as was the case for this report), instantaneous samples are used to assess the DO criterion by comparing the instantaeous value with a time-of-day-specific translation of the daily average criterion.  The equations to calculate the time-of-day-specific translation of the daily average criterion are cited in 62-303.420(9), F.A.C., and a spreadsheet calculator for this purpose is available at:  http://www.dep.state.fl.us/water/wqssp/dissoxy.htm.

If a report is done for a natural background DO study, include the following:  Rule 62-302.533(3), F.A.C., states that, “If it is determined that the natural background DO saturation in the waterbody (including values that are naturally low due to vertical stratification) is less than the applicable criteria stated above, the applicable criteria shall be 0.1 mg/L below the DO concentration associated with the natural background DO saturation level”. The FDEP method to demonstrate the appropriateness of a reference site to make a natural background determination includes a demonstration that the proposed reference watershed has been minimally affected by human activities, including point source discharges and nonpoint source inputs.

DEP (with EPA approval) has identified minimally disturbed stream and lake reference sites by application of the following criteria:

- Landscape Development Intensity Index (LDI) (Brown and Vivas 2007) score < 2 for land use within the 100 meter corridor 10 km upstream of the sample site. LDI scores higher than 3 indicate increasing levels of human disturbance;

- Watershed or near-field LDI scores < 3, indicating that that human disturbance is minimal throughout the watershed;

- Presence of relatively benign land uses or human inputs, as discerned by using aerial photographs, field observations, and DEP Best Professional Judgment, that would support the conclusion that the site is minimally disturbed; and

- Characterized by healthy biology (2 SCI scores sampled at least 3 months apart with an average score > 40 and no single score < 35 or 2 LVIs with an average score > 43).

Lake Vegetation Index

The Lake Vegetation Index (LVI) assesses how closely the plant community of a lake resembles a native undisturbed community.  The LVI was sampled and calculated per DEP SOP LVI 1000 and calculated per DEP SOP LVI 2000.   Species lists were developed for four of twelve sections of the lake (Figure 2), and the following information was derived from those lists: percent native species, percent invasive exotic species, percent sensitive species, and the coefficient of conservatism (C of C; a measure of how tolerant a species is of disturbance) of the dominant species.  Chapter 62-303.330 and 62-303.430, F.A.C., provide that a LVI score of 43 or greater meets the expectation of a healthy, well balanced community, and scores below 43 are considered impaired.

Figure 2. Sampling map of [Lake Name].  Sections [1,4,7,10] were sampled for the Lake Vegetation Index.  The water quality sample was collected from the lake center.

Figure 3. Photo of [Lake Name] on [Sampling Date].

Results

Water Quality

The water quality results are shown in Table 2.  [Describe any exceedances of standards and whether they may be due to natural conditions (e.g., DO) or local human disturbance.  Make sure DO exceedances are based on the time-of-day adjustment calculations.]

Based on these two sampling events, the lake [met/exceeded] the nutrient thresholds for the [alkalinity and color category]; however the thresholds represent annual geometric mean concentrations (minimum of four samples) not to be exceeded more than once in any three calendar year period.  Additional sampling is needed to fully assess compliance with nutrient criteria.

Table 2. Water quality results from [date and date] at [Lake Name].

| Analyte | Result [date] | Result [date] | Applicable Class III Water Quality Criteria |
| --- | --- | --- | --- |
| Field Temperature (°C) |  |  |  |
| Field pH (SU) |  |  |  |
| Field Dissolved Oxygen (% saturation) |  |  | Panhandle West: 67%; Peninsula and Everglades: 38%; Big Bend and Northeast: 34%. |
| Field Specific Conductance (µmhos/cm) |  |  | 50% above background or 1275 µmhos/cm, whichever is greater |
| Alkalinity (mg CaCO3/L) |  |  |  |
| Color (PCU) |  |  | N/A |
| Chlorophyll a (µg/L) |  |  | ≤ 20 µg/L for colored and alkaline clear lakes |
| Total Phosphorus (mg/L) as P |  |  | [insert applicable range] |
| Nitrate+Nitrite (mg/L) as N |  |  |  |
| Ammonia (mg/L) as N |  |  | 0.02 as unionized ammonia |
| Total Kjeldahl Nitrogen (mg/L) as N |  |  | N/A |
| Total Nitrogen (mg/L) as N |  |  | [insert applicable range] |

Phytoplankton [if collected]

The algal community in the water column consisted of x% cyanobacteria (blue-green algae), x% green algae, x% diatoms, etc.  Dominance in algal groups varies naturally throughout the year, but prolonged dominance of cyanobacteria may be associated with a higher risk for harmful algal blooms. [specific interpretation of lake results]

Lake Vegetation Index

The LVI score for this lake was [LVI score] out of a possible 100 points, corresponding with a "[Exceptional, Healthy, or Not Healthy]" designation.  Table 3 contains the species list and occurrence information for this sampling event.  A total of [number of FLEPPC species] invasive exotic plants were observed in the lake.

Table 3. Species list for the [date] LVI at [Lake Name].  An asterisk (*) indicates an invasive exotic plant species.  P = present, D = dominant, C = codominant.

[See http://www.dep.state.fl.us/water/bioassess/plantid.htm for instructions on how to make Table 2, including linking common names, formatting, and inserting table into report. ]

[Insert additional discussion, photos, or anything you think would be informative for the audience.  If not healthy, mention any planned follow-up to verify condition or mitigate potential sources]

Thank you for your interest in maintaining the water quality of Florida’s lakes. Please contact us if you have any questions.

Contact and resources for more information

[your name, phone number, and e-mail address]

DEP publications on Best Management Practices and Environmental Stewardship and Education:  http://www.dep.state.fl.us/water/nonpoint/pubs.htm

DEP biological assessment resources: http://www.dep.state.fl.us/water/bioassess/index.htm

FWCC Aquatic Plant Management: http://myfwc.com/wildlifehabitats/habitat/invasive-plants/aquatic-plant/

Freshwater Algal Bloom information: http://www.dep.state.fl.us/labs/biology/hab/index.htm